TeleReplica Instruction Manual

0.    To Get TeleReplica Up and Running Quickly...

If you will be using modems then read the modem setup suggestions at the

start of appendix A. Then read whichever of the following sections most

closely matches your own application:

A1  Consultant/Client via Modem

One consultant wishing to connect via modem to many different

clients in order to take over the clients' computers.

A2  Remote/Shared via Modem

One remote access PC to be taken over by many different users via

modem.

A3  Laptop/Desktop via Direct Cable

A simple direct connection with no modems.

A4  Remote/Shared via Common Mini-computer

Instead of a direct link between the two PCs, each PC will be

logged into a common UNIX system.

A5  Home/Office via Modem

A dedicated office PC to be taken over from a home PC.

Note that A5 also contains some advice about configuring high speed

V32bis/V42bis modems (USR Courier V32bis or similar).

1.    What TeleReplica Will Do

1.1   TeleReplica will allow you to connect two IBM PC-compatible computers

together (via telephone lines and modems, or via a direct cable, or via a

common mini- or main-frame computer), and arrange for input to be

accepted from either keyboard, and for both displays to appear identical.

In this document the computer which is running normal programs is called

the host computer, and the other computer is called the replica computer,

since its display is a replica of the host computer's display.

1.2   TeleReplica will do a reasonable job of optimizing the screen changes

transmitted, so that (for example) moving a highlighted menu bar will

appear almost immediately even at 1200 bps (provided the call is

relatively local).

1.3   TeleReplica always updates the replica computer's screen to match the

current host screen. This means that if several commands are issued

without waiting for the display to catch up, then the replica screen will

be updated to show the final host screen, without wasting time

transmitting the intermediate screens.

1.4   TeleReplica uses a password of up to 39 characters, which means an

intruder would take an astronomical time to guess a well chosen password!

Since this password is transmitted automatically, there is no need to

keep it short and easy to remember - once TeleReplica is installed, you

will never need to enter the password manually.

1.5   TeleReplica will allow the host site to call the replica site, or the

other way around. For consultants this means you can make your clients

pay the 'phone bills!

1.6   TeleReplica will automatically detect the "baud" rate being used by the

replica computer, and set the host computer's "baud" rate to match. This

means that users with different speed modems can all access a single

host.

1.7   TeleReplica uses just over 10K of memory on the host computer, which

means there should be enough memory left to run most applications.

1.8   TeleReplica can be installed to "type" a command in response to the

'phone ringing, which allows better control than merely using the modem's

auto-answer capability. It can also send a command directly to the modem.

1.9   TeleReplica will trap anything printed to any printer on the host PC, and

redirect it to the chosen printer at the replica PC. This redirection

takes place only when a session is active, so at other times the host can

be used to print normally even if TRHOST is still loaded.

1.10  TeleReplica will allow the host PC to run with screen blanked and

keyboard dead for improved privacy.

1.11  TeleReplica will support file transfers using external protocol programs

such as DSZ, which provide crash recovery and cope better with poor lines

than TeleReplica's built-in protocol.

1.12  TeleReplica can limit the duration of a single session, to prevent one

user from "hogging" the line.

1.13  TeleReplica can automatically locate the COM ports (including

non-standard IRQ lines) currently available in a PC, including IRQ8-15.

1.14  TR is "DESQview-Aware". See Appendix D for details.

1.15  Full source code is available at a small cost. See REGISTER.DOC for

details.

2.    What TeleReplica Will NOT Do

2.1   TeleReplica will not replicate graphics displays. If the user at the

replica computer runs a program that displays graphics, then the replica

display may show (textual) garbage. Keystrokes will still be transmitted

to the host computer, but the user must remember which keys are necessary

to get out of graphics mode again! TeleReplica gives a warning that

graphics mode has been entered, but by then it is too late to avoid it.

However, in an emergency TR can instruct the host computer to reboot, so

provided TRHOST is run from AUTOEXEC.BAT this provides a means of

recovery.

2.2   TeleReplica will only replicate an 80 column by 25 line display.

2.3   TeleReplica will only work with the standard keyboard buffer (starting at

the address in segment 0040h specified at offset 0080h, ending at the

address specified at offset 0082h, with head pointer stored at offset

001Ah, and tail pointer stored at offset 001Ch).

2.4   Where TeleReplica is being used with a modem, it will work best if the

modem and modem cable provide carrier detect, data terminal ready and

ring indicator signals in addition to the minimum transmit data, receive

data and ground signals.

2.5   TeleReplica will cause bad "snowing" on some older CGA display cards. No

attempt has been made to use the retrace intervals for screen memory

access.

2.6   TeleReplica will slow down the normal operation of the host computer. On

a 4.77MHz PC performance can be halved; on a faster host the difference

will not be so noticeable.

2.7   TeleReplica requires DOS 3.0 or later.

2.8   TeleReplica will not work particularly well on a noisy 'phone line. File

transfers are error-checked, but normal screen updates are not. Normally

this will not be a problem, since a corrupted screen will be obvious to

the user and in any case TR will usually detect the problem and

automatically request a full refresh. However, if errors happen very

frequently then TR can spend all its time continually re-refreshing the

display! Printer capture also is not error-checked, so line noise will

garble the printout. [If this proves to be a problem then the best

solution would be to use error-correcting (MNP4 or V42) modems.]

3.    Why Use TeleReplica?

3.1   Access to Local Area Network from Home

TeleReplica will allow you to connect into your office computer from

home. This means you do not need to take all your data files home with

you, and that you do not need to license additional copies of your

software (since your home computer is not running the software, only

replicating the screen of your office computer). If your office computer

is part of a local area network, then you automatically have access to

all the network facilities from home.

We regularly use TeleReplica with such programs as WordPerfect,

WordPerfect Office (especially mail), WordStar, FoxBASE, Lotus-style

spreadsheets, all the Novell NetWare utilities, and our local area

network is regularly backed up after hours via TeleReplica! The only

programs we have come across that could not be used remotely either:

a)  used graphical displays (e.g. WordPerfect preview); or

b)  took over interrupts needed by TRHOST (e.g. comms programs);

3.2   Software Support

TeleReplica will allow you to take over your client's computer, so that

you can see what is going wrong. In many cases this will avoid the need

for an expensive trip to the client's site.

3.3   Library CD-ROM Access

TeleReplica is becoming quite popular for allowing remote (dial in)

access to CD-ROM databases running on PCs in libraries. The chief

attraction is that the library only needs to register once for each

CD-ROM PC that will be simultaneously available, and the users do not

need to register at all; the library can give them the software freely.

However, another significant factor is that the CD-ROM database software

often requires a very large amount of memory, and TeleReplica uses very

little memory on the host PC.

For details of these and other applications, see the scenarios in

Appendix A.

4.    TeleReplica Host Installation

To install TeleReplica at the host computer, first copy the files:

TRHOSTIN.EXE, TRHOSTC.EXE, TRHOST.EXE, TRPUT.EXE, TRGET.EXE, TRTALK.EXE,

TRCHECK.EXE, TRASK.EXE, TRDIAL.EXE, TREXTR.EXE, TREXTS.EXE, TRMODSET.EXE,

TRRZ.BAT and TRSZ.BAT into a directory (say \TR) on the host computer.

Then enter the command:

TRHOSTIN

Your display should show a menu similar to:

������������������������������������������������������������������ͻ

� TeleReplica Host 4.23 Installation Copyright (c) Douglas Thomson �

�    A. Select serial port (currently COM1)                        �

�    B. Change password (currently "password")                     �

�    C. Change modem initialization string (currently:             �

�       "\rat &c1 &d2\r")                                          �

�    D. Change modem dialling format string (currently:            �

�       "atdt %s\r")                                               �

�    E. Select maximum "baud" rate (currently 2400)                �

�    F. Use modem CARRIER DETECT signal (currently YES)            �

�    G. Flow control (currently NONE)                              �

�    H. Generate INT 9 on each key (currently NO)                  �

�    I. Respond to modem RING signal (currently NO)                �

�    J. Reboot on loss of carrier (currently NO)                   �

�    K. Synchronize "baud" rate (currently YES)                    �

�    L. Send wakeup every 2 minutes (currently NO)                 �

�    M. Private mode (currently NO)                                �

�    N. Printer redirection (currently HOST)                       �

�    O. Maximum call length (currently unlimited)                  �

�    P. Create consistent .TR file                                 �

�    Q. Abandon changes                                            �

�    S. Save changes and leave installation program                �

� Selection:                                                       �

������������������������������������������������������������������ͼ

Consider each of these options in turn:

4.1   Select Serial Port

Another menu will appear:

����������������������������������������������������������ͻ

� TeleReplica Host Program Serial Port Selection           �

�                                                          �

�    0. Completed, no (further) change                     �

� => 1. COM1 port (base 0x03F8, interrupt IRQ4)            �

�    2. COM2 port (base 0x02F8, interrupt IRQ3)            �

�    A. Custom base (currently 0x03F8)                     �

�    B. Custom interrupt (currently IRQ4)                  �

�    X. Search for hardware                                �

�                                                          �

� Selection:                                               �

����������������������������������������������������������ͼ

Initially, the ports listed will be those reported by the computer's BIOS

during its initialization at power-up. Since the BIOS provides no

information about interrupt numbers, the IRQ numbers will simply be the

most common (IRQ4 for COM1 and COM3, IRQ3 for COM2 and COM4). If you only

use COM1 and COM2 then this may be all you need.

However, if you use COM3 or COM4, especially with non-standard IRQ

numbers, you should select option X to have TRHOSTIN examine your

hardware and see what ports are really there. [This is not done

automatically since the searching process could conceivably crash the

computer - although it has not done so on any of the computers I have

tested it with.] After selecting X, the menu will be modified to reflect

any additional ports discovered; for example:

����������������������������������������������������������ͻ

� TeleReplica Host Program Serial Port Selection           �

�                                                          �

�    0. Completed, no (further) change                     �

� => 1. COM1 port (base 0x03F8, interrupt IRQ4)            �

�    2. COM2 port (base 0x02F8, interrupt IRQ3)            �

�    3. COM3 port (base 0x03E8, interrupt IRQ5)            �

�    A. Custom base (currently 0x03F8)                     �

�    B. Custom interrupt (currently IRQ4)                  �

�    X. Search for hardware                                �

�                                                          �

� Selection:                                               �

����������������������������������������������������������ͼ

All the existing ports are now listed. To select one of these ports, just

type the port number. If you have a non-standard base address (one that

is neither in the BIOS table nor one of the common "standard" values),

then use A and B to specify exactly what you want [it may be worth

selecting the base address, and then choosing X again, so that TRHOSTIN

will check for the right IRQ line for that base and add it to the menu].

When you are happy with the settings, select 0 to return to the main

menu.

Please note that I do not have access to any system which uses the AT IRQ

numbers (8-15), so I cannot guarantee the performance of TeleReplica when

these 16-bit cards are used. Also, you should be careful to avoid having

two programs simultaneously active on ports which are listed with the

same IRQ number - for example a mouse on COM1/IRQ4 and a modem on

COM3/IRQ4.

4.2   Change Password

The string editing menu will appear:

��������������������������������������������������������������������ͻ

� TeleReplica Host Program Password Selection                        �

�                                                                    �

� Edit existing string:                                              �

� password                                                           �

��������������������������������������������������������������������ͼ

Whenever you see the "Edit existing string:" prompt, you can use the left

and right arrow keys to move the cursor through the string, and the

backspace and delete keys to delete characters to the left of or under

the cursor respectively. Any normal characters you type will be inserted

at the cursor position. If the first key you press is a normal printable

character then the string will be cleared automatically.

You will not be able to enter more than the maximum allowed number of

characters for any string.

Since you will only need to enter the password once during installation,

and never in normal use, it is recommended that you make the password

near the maximum length and quite random. The main thing is that no-one

should have any chance of guessing the password.

4.3   Change Modem Initialization String

The string editing menu will appear again, this time asking for the modem

initialization string. This string is transmitted to the modem at the

maximum "baud" rate when TRHOST runs for the first time. If you are using

an automatic power switch that powers up the host computer when the

'phone starts ringing, then the initialization string may need to answer

the 'phone (see scenario A2). There may also be various operating modes

that cannot be stored permanently in your modem's non-volatile memory,

and which must therefore be established each time TRHOST starts. In many

applications, where nothing needs to be changed, this string may be

empty.

You will probably need to refer to the instruction manual for the host

modem to work out what if anything is required.

As well as normal characters, you may need to include some special

characters such as carriage return. Carriage return can be encoded as \r

(see the main menu at the start of section 4). Alternatively, any

character can be entered in hexadecimal as \0x followed by two hex digits

(for example, the escape key is \0x1B). To get a \ character, use \\.

It may also be necessary to include some kind of delay. For example, some

modems go off into a world of their own for a second or so after a reset

command. Delays are entered in milliseconds as \D followed by the delay

(for example, \D1000. [1 second] or \D500. [half a second]). Delays are

only accurate to the nearest 55 ms, so the number you enter may be

modified slightly to reflect the nearest possible actual delay.

4.4   Change Modem Dialling Format String

This string determines what TRDIAL will send to the modem to dial a

number. The usual use of TRDIAL will be something like TRDIAL 221166, so

normally the dialling format string should contain a %s to insert the

number. For example, if the dialling string is "atdt %s\r" and the user

enters the previous dialling command, then what will be sent to the modem

is:

atdt 221166

followed by a carriage return.

Alternatively, if only one number is required, then the dialling format

string may contain the actual number, in which case the user will only

need to enter TRDIAL by itself.

Note that (when CARRIER DETECT is being used) TRDIAL will only send

anything to the modem if there is NO carrier detected - once a carrier

had been established the modem would not act on the command in any case,

but would send it through to the other end. When TRHOST is installed to

ignore CARRIER DETECT, TRDIAL will send data to the modem regardless.

4.5   Select Maximum "Baud" Rate

Another window will appear:

�������������������������������������������������������������������ͻ

� TeleReplica Host Program Maximum "Baud" Rate Selection            �

�                                                                   �

� Enter "baud" rate:                                                �

�������������������������������������������������������������������ͼ

You should enter the maximum "baud" rate that the host modem is capable

of using. Usually this will be 2400 or 19200. Note that TRHOST can

automatically adjust to half this speed, a quarter of this speed, or one

eighth of this speed. Thus if you choose 19200 bps, TRHOST will be able

to work at 19200, 9600, 4800 or 2400 bps (provided the host modem can

handle these speeds).

If you will be using a direct cable between two PCs, then 19200 should

work on all systems and 38400 will work on faster computers. If your

computers have NS16550AFN UART chips instead of the standard 8250 chips

then speeds of 57600 or even 115200 should work.

4.6   Use modem CARRIER DETECT signal

Selecting this option will determine whether or not TRHOST will use the

carrier detect signal provided by the modem. If you are using a real

modem, then you should set this to YES and make sure the modem is

initialized so that it reports the true CD status from the 'phone line.

If you are using a direct cable between two PCs, or connecting PCs via a

mini-computer, then set this option to NO.

4.7   Flow control

Selecting this option will bring up the following menu:

����������������������������������������������������������ͻ

� TeleReplica Host Program Flow Control Selection          �

�                                                          �

�    0. Completed, no (further) change                     �

� => 1. No flow control                                    �

�    2. Hardware (RTS/CTS) flow control                    �

�    3. Software (XON/XOFF) flow control                   �

�    4. Both hardware and software flow control            �

�    5. Set inter-character gap time (currently 330 ms)    �

�                                                          �

� Selection:                                               �

����������������������������������������������������������ͼ

4.7.1 TeleReplica can use the following flow control systems:

NONE     No flow control. Use this with a direct cable or with modems

that require no speed adjustment.

CTS      Hardware flow control using RTS (request to send) and CTS (clear

to send). Use this in preference to software flow control if it

is available.

XON      Software flow control using XON and XOFF (^S/^Q). Use this if

flow control is needed and hardware flow control is not

available.

BOTH     Use both software and hardware flow control. I can't imagine why

anyone would want this...

4.7.2 Normally TeleReplica will not need to use any flow control even when it

is available. However, some modems communicate with the PC at a different

speed than that at which they communicate with other modems, so that it

is possible for the PC to "get ahead" of the modem and hence lose data.

In these cases it is necessary for the modem to tell the PC when to wait

a bit before sending the next byte.

4.7.3 A side effect of software flow control is that TeleReplica will not use

ANY control characters: it will not send any, and it will ignore all

control characters received apart from XON and XOFF. This may be useful

for networks that respond to certain control characters; however, binary

file transfers will be slightly slower since 64 characters (control

characters with and without the parity bit) are not used. Note that both

TR and TRHOST must therefore either use or not use software flow control

- if one end uses software flow control and the other does not, then no

communication will be possible!

4.7.4 When TRHOST is receiving characters (normally keys that have been pressed

at the remote PC - each key is encoded as two or three characters) it

waits a certain length of time after receiving the first character before

giving up and ignoring the key. This is important, since otherwise a

single byte introduced by noise on the line could cause all subsequent

keys to be misinterpreted (the second character of one key being treated

as the first character in the next key and so on). TR always sends all

the characters which constitute one key as close together as the

communication speed will allow, so usually a gap of even 0.1 seconds

means that something has gone wrong and that TRHOST should start looking

for a new key again. However, if the link between TR and TRHOST uses

packets, or for any other reason introduces significant gaps between two

characters that were sent together, then TRHOST may need to be adjusted

to tolerate longer gaps between characters which should be together.

Selecting option 5 will bring up the following screen:

��������������������������������������������������������������������ͻ

� TeleReplica Host Program Inter-character Gap Time Selection        �

�                                                                    �

� Longest gap to be tolerated (ms): 330                              �

��������������������������������������������������������������������ͼ

Any time up to about 10 seconds (10000 ms) can be entered. Be aware that

if this time is set to 10 seconds, and line noise does introduce a stray

character, then you will need to wait 10 seconds without typing anything

at the remote PC to get TRHOST back into synchronization!

4.8   Generate INT 9 on each key

When TRHOST receives a key from TR, it will usually just place it in the

keyboard buffer to be found by any program that wants to read a key.

However, some applications monitor the keyboard interrupt (INT 9) and

only read the keyboard input buffer after there has been a keyboard

interrupt! To keep such programs happy, TRHOST can generate an INT 9

after it has placed a key into the buffer. However, this can cause

problems for other programs, particularly when the host and remote

keyboards may both be in use at the same time (e.g. TRTALK). Leave this

option OFF unless it is really necessary.

4.9   Respond to modem RING signal

Selecting this option will determine whether or not TRHOST will respond

to the RING signal which the modem provides to tell the PC that the

'phone is ringing. If this option is turned on, then the menu will be

extended with the following sub-options:

G. Respond to modem RING signal (currently YES)����� 1. too short  220 ms

�� 2. too long  5000 ms

�� 3. delay    10000 ms

�� 4. reset    10000 ms

�� 5. "type" response

�� 6. response Ŀ

"\0x1Btrauto\r"

4.9.1 If the 'phone only rings for a very short time, you may wish to ignore

the ringing completely - otherwise just knocking the 'phone off the hook

can trigger a TRHOST response!

4.9.2 If the 'phone rings for a long time, then this may indicate that the

caller is a person who should not be greeted by an ear full of modem! If

TRHOST is intended to respond before the 'phone stops ringing, then this

time should be set the same as the delay below.

4.9.3 After a certain length of time (measured from when the 'phone starts

ringing) TRHOST will check that the amount of ringing was within the

specified range, and if so, will respond to it (see point 5).

4.9.4 If the 'phone rang for too long, then TRHOST must know when to start

looking for another call. If the 'phone has not rung for a certain length

of time then TRHOST assumes any ringing must be a new call.

4.9.5 The response string may be either "typed" (that is, inserted into the

keyboard buffer as if a user at the host PC had just typed it in), or it

can be sent direct to the modem. The advantage of "typing" is that you

can run a batch file to check times and do all sorts of other things. The

advantage of sending direct to the modem is that the entire operation can

take place in the background while an application continues running.

4.9.6 You may specify a response of up to 39 characters. If you expect the host

PC to be at the DOS prompt when a call comes in, then it is a good idea

to start with an escape key, so that DOS discards anything already typed

and treats what follows as a new command.

If all this sounds too complicated, but you still think you want to

enable ring response, then have a look at the configuration scenario

called "Home/Office via Modem" in section A5.

4.10  Reboot on loss of carrier

Selecting this option will determine whether or not the computer will

reboot when the modem reports that the carrier has been lost (presumably

because the other end hung up). See scenario A5 for an example of an

application taking advantage of this feature.

The PC will not reboot unless there has been a TeleReplica session active

since TRHOST was loaded.

4.11  Synchronize "baud" rate

Selecting this option will determine whether or not TRHOST will attempt

to match the "baud" rate being used by TR at the other end. This is

useful if several users will all dial into the same host, but some users

have slower modems than others. See scenario A2 for an example of this.

4.12  Send wakeup every 2 minutes

Selecting this option will determine whether or not TRHOST will send a

NULL character whenever two minutes has passed with no other activity. A

single NULL is ignored by TR, but may serve to keep a connection to a

mini-computer link. See scenario A4 for an example of this.

4.13  Private mode

In private mode, the screen of the host PC will be blank, and the

keyboard of the host PC will be inactive (including CTRL-ALT-DEL). This

is useful to enhance privacy when the host PC is in a public location.

Note that the screen blanking has only been tested on VGA, EGA, CGA and

Hercules display cards, and only in text modes. It has worked correctly

with all the applications tested, but you should test it with your own

applications and display cards before relying on it!

4.14  Printer redirection

When a TeleReplica session is active, it is possible to redirect printer

output (which would otherwise be printed at the host PC) so that it is

printed at the replica PC instead. When no session is active, the host

will print normally even though TRHOST is still loaded.

4.15  Maximum call length

In some cases, perhaps where multiple users are competing for a limited

number of phone lines or a toll-free number is being used, it will be

desirable to enforce some upper limit on the connection time. After this

time, TRHOST will drop DTR for 2 seconds, which will cause a (correctly

configured) modem to hang up.

One minute before the timeout TRHOST sends a warning message to TR, so

that the user will have a minute in which to exit gracefully from

whatever is happening. Note that this will not happen during a file

transfer, so you should not start a file transfer unless you are sure

there will be time to complete it before the host hangs up!

Due to the way this timer is implemented, there is a limit of 3598

seconds (just under an hour). If I get enough people complaining that

they really need limited length sessions, but the limit needs to be over

an hour, I will consider changing the implementation...

Note that DTR will also be dropped before rebooting if a maximum call

length has been specified, so there is no (known) way to defeat the

mechanism. However, it might be a good idea to restrict access to

programs like TRHOSTIN and TRHOSTC which could be used to modify the time

limit!

4.16  Create consistent .TR file

Once TRHOST has been installed correctly, TRHOSTIN can create a

configuration file for TR that is consistent (same "baud" rate, password,

use of carrier detect etc). It will normally still be necessary to edit

this file to do some fine tuning, but the work required will be less than

if you started with the vanilla TR.TR file distributed with TeleReplica.

4.17  Abandon Changes

If you have made a mistake, or if you only wanted to see what the current

settings were, then this option allows you to quit without making any

changes.

4.18  Save Changes and Leave Installation Program

This is the normal way to exit this program when you have made all the

required changes. The relevant files (TRHOST.EXE, TRGET.EXE, TRPUT.EXE

TRTALK.EXE, TRDIAL.EXE, TREXTR.EXE, TREXTS.EXE and TRMODSET.EXE) will all

be modified appropriately.

Note that the executable files are actually modified, so beware of virus

protection programs which compute checksums for executable files - they

are likely to report possible virus infections after installation! The

alternative (loading configuration information from a disk file) was

rejected due to the additional memory required for the file processing

code...

5.    Replica Installation

To install TeleReplica at the replica computer, first copy the files:

TR.EXE, TR.HLP, and TR.TR (or better still the .TR file that was created

during host installation) into a directory (say \TR). Then use a text

editor to edit the .TR file. This file should look something like:

��������������������������������������������������������������������ͻ

�    TeleReplica Configuration File                                  �

�    Do NOT change the order of these parameters or delete lines!    �

� "COM1"       Communications port ("COMn" or base/interrupt)        �

� 2400         "Baud" rate (typically 2400 or 19200)                 �

� "COMSPEC"    Path for DOS Command Interpreter (or "COMSPEC")       �

� 07           Attribute for normal text (such as help screen)       �

� 47/70        Attribute for file transfer monitoring                �

� 4F/0F        Attribute for pop-up windows                          �

� "trput %s\r" Host command to send (download) files                 �

� "trget\r"    Host command to receive (upload) files                �

� 0            Printer (normally 0, 1 or 2)                          �

� 440          Frequency of "bell" tone (Hz)                         �

� 100          Duration of "bell" tone (ms)                          �

� 256          Block size for file upload (1-256)                    �

� 8            Number of block buffers for upload (1-8)              �

� 111          Length of "break" (ms)                                �

� 3000         Time to wait for first char before timeout (ms)       �

� 500          Time to wait for subsequent chars in block (ms)       �

� 56           Time to wait between "typing" keys (ms)               �

� "password"   Password to log in to host computer                   �

� "MODEM"      Type of link (MODEM, DIRECT, or OTHER)                �

� "NONE"       Flow control (CTS, XON, BOTH or NONE)                 �

� "YES"        Send BREAKs to host computer                          �

� "YES"        Use BIOS for keyboard input                           �

� "[TR] %s"    DOS shell prompt format string                        �

� "LRCA"       Local command shifts (Left, Right, Ctrl, Alt)         �

� ""           Connect command for modem                             �

� ""           Disconnect command for modem                          �

�                                                                    �

�    Each of the remaining lines must contain 3 strings and a number.�

�    The first string is the trigger string to search for.           �

�    The second string is the response to be sent after the trigger. �

�    The third string is the response to be sent after timeout.      �

�    The number is the time to wait before timeout (in ms).          �

��������������������������������������������������������������������ͼ

Most of these parameters will not need to be changed.

5.1   The serial port may be one of the standard ones ("COM1", "COM2", "COM3",

or "COM4") or you may specify the base address and interrupt number [e.g.

COM1 would be 03F8/IRQ4]. For compatibility with previous versions of

TeleReplica, you may also use a hex interrupt vector number, so that a

third way to specify COM1 is 03F8/0C.

5.2   The "baud" rate should be set appropriately for the modem.

5.3   The DOS command interpreter should normally be taken from the COMSPEC

environment variable. However, if this does not work (or does not select

the desired command interpreter) the full path name of a command

interpreter can be specified (e.g. "C:/DOS/COMMAND.COM" - or

"C:\\DOS\\COMMAND.COM" - the \\ is needed since \ introduces an escape

sequence).

5.4   Video attributes are two hexadecimal digits: the first is the background

color, and the second is the foreground color. Optionally a second

attribute may be specified (e.g. 4F/70) - the first attribute is used for

color displays and the second is used for monochrome systems (this allows

a single control file to work without change with different displays).

5.5   When the user running TR requests a file transfer, TR "types" the

necessary command at the host PC. These two lines should not be changed

except possibly to fill in a full path name if TRGET.EXE and TRPUT.EXE

are not in the path of the host PC.

5.6   When host printing is being redirected to the replica computer's printer,

TR needs to know which printer to use. LPT1: is printer 0, LPT2: is

printer 1, and LPT3: is printer 2. Serial printers are not directly

supported.

Note that it is not necessary or possible to specify which host printer

is to be redirected; anything sent to LPT1:, LPT2: or LPT3: will be

redirected. This was done to save the user the trouble of working out

which printer the host was using.

5.7   The frequency (or pitch) and the duration of the "bell" can be modified

to suit your personal preferences (and the volume of your PC's speaker!).

The "bell" is used to indicate that some corruption of the data being

received by TR has occurred.

5.8   The size of file transfer blocks and the number of buffers should not

need to be changed, except on a very poor 'phone line (in which case

smaller blocks and fewer buffers may improve matters). However, if the

line is this bad then TeleReplica will probably be unusable anyway...

5.9   The length of "break" required is not likely to need to be changed -

unless perhaps a mini-computer requires longer?

5.10  During a file transfer, TR will wait a certain time for the start of each

block. If nothing has been received within the time, then TR will request

the block be sent again (since presumably it got lost).

5.11  Once the first character in a block has been received, the rest should

follow immediately - any gap probably means problems. However, when using

an overloaded mini-computer link (or using a packet network that may

introduce delays between packets, or using a modem with error correction,

compression or bps translation) this parameter could need to be longer.

5.12  Since on some PCs (mainly ones with CGA displays) interrupts are disabled

for LONG periods when writing to the screen, it may be necessary to make

TR pretend to type quite slowly, especially when sending things like file

transfer commands. If you find you are losing characters under these

circumstances, then make this delay longer; if you have a smart BIOS

and/or version of DOS and/or command interpreter and/or display card then

you may be able to get away with a shorter delay (even 0 ms). On all the

systems I have used, the time taken to send two characters at 2400 bps or

below is sufficient, so this delay is probably only needed above 2400

bps.

5.13  The password must match the password installed in TRHOST at the host

computer.

5.14  There are three kinds of link that may be used between two PCs:

MODEM    Use this if you are working with a modem that provides CARRIER

DETECT (note that you will require an appropriate cable [see

Appendix C1] and possibly some initialization of the modem [see

the start of Appendix A]).

You can also use this if you are using a null modem cable that

provides a CD line (see Appendix C3).

DIRECT   Use this if you are working with a three-wire null modem cable,

or if you are using a modem/cable that cannot provide accurate

CD information.

OTHER    Use this if you are working with a mini-computer link (see

scenario A4), or for initial modem setup (see the start of

Appendix A).

5.15  TR can use the following flow control systems:

NONE     No flow control. Use this with a direct cable or with modems

that require no speed adjustment.

CTS      Hardware flow control using RTS (request to send) and CTS (clear

to send). Use this in preference to software flow control if it

is available.

XON      Software flow control using XON and XOFF (^S/^Q). Use this if

flow control is needed and hardware flow control is not

available.

BOTH     Use both software and hardware flow control.

See the discussion of flow control under host installation for further

details.

5.16  Normally TR should send BREAKs to TRHOST, but see scenario A4 for use

with a mini-computer link.

5.17  Normally TR will use the BIOS for keyboard input. However, many BIOSes

filter out keys from enhanced keyboards (e.g. <F11>), and if use of these

keys is important then it may be necessary to NOT use the BIOS, but

rather to use the keyboard input buffer directly. Note however that some

keyboard enhancing programs (such as Newkey) will only work with the BIOS

setting. This is because they intercept the keyboard interrupt and store

keys away in some place other than the standard buffer area, so that TR

cannot find them!

5.18  The DOS prompt for when you are in a TR shell can be edited to suit your

individual preferences - the %s inserts the original prompt.

5.19  If you don't like the default <Ctrl-Alt-LeftShift-RightShift> key

combination to access local commands in TR, then you can change this to

any other combination of these four main shift keys ("CAL" would mean

<Ctrl-Alt-LeftShift> for example) - see scenario A3 for an application of

this).

5.20  The connect command for the modem is sent when TR starts running (this

will often be a dial command). The modem command strings here have the

same format as those described above for the host computer installation

(with the addition of a \B escape sequence that causes TR to transmit a

BREAK). Make sure you leave the double quotes (") around each string!

5.21  The disconnect command is sent when TR terminates (using exit option 3

only).

5.22  The automatic response strings at the end of the file can be used to

respond to modem status messages. For example, if the replica computer's

modem is not set to auto-answer, then TR may need to respond to a "RING"

message from the modem by answering the 'phone - see scenario A5 for an

application. These strings can also be used to achieve an automatic login

to a mini-computer - see scenario A4 for an example. The trigger strings

are only active before a TR-TRHOST link is established. They are also

disabled once the final trigger string has been detected.

Please note that since this file contains the password, anyone who

obtains a copy of this file can potentially access the host computer!

6.    How to Use TeleReplica

The following instructions assume that TeleReplica has been correctly

installed (see sections 4 and 5). Only general guidelines are given here;

more details are provided in the scenarios in Appendix A.

6.1   Host Computer

First run the TRHOST program. If the replica computer is to originate the

'phone call, then this may be all that is necessary.

If the host computer is to originate the call, then use the command:

TRDIAL

or

TRDIAL number

depending on how the dialling format string was installed.

Once TRHOST is resident and the call is established, the host computer

can be used normally. The only difference is that everything that appears

on the screen is also being replicated on the replica computer's screen.

The best way to remove TRHOST from memory is to run TRREMOVE. It is also

possible to use utilities such as marknet/relnet. Note that some common

mark/release utilities can NOT be used, because they do not restore the

interrupt mask register.

6.2   Replica Computer

Use the command:

TR                     [use TR.TR configuration file]

or

TR filename            [use filename.TR configuration file]

If the replica computer was to originate the call, then the modem

initialization string in the configuration file will usually include a

dial command to call the host computer.

If the host computer was to originate the call, then the initialization

string may contain an answer 'phone command. Note that in this case it is

important that the TR program not be run until after the 'phone starts

ringing! (Alternatively, there may be no initialization string, but

instead there may be a trigger string to respond to RING from the modem -

see scenario A5.)

If the configuration file contains no modem initialization string, then

the user at the remote computer can enter modem commands manually. This

may be convenient for a consultant wishing to dial many different

clients. (Before a carrier is detected, TR will behave much like a normal

communications program.)

Once both TR and TRHOST are running and connected to each other, they

will automatically adjust the host computer's "baud" rate (if this

feature is enabled) and the host computer will check the password

provided by the replica computer. Provided the password is correct, the

replica computer will effectively be able to take over the host computer,

or alternatively to observe whatever the host computer is doing. Any key

typed on the replica computer will be acted upon by the host computer,

and anything that appears on the host computer's screen will also appear

on the replica computer's screen.

It is sometimes necessary to execute a command on the replica computer,

rather than transmitting the keystroke to the host computer. This is

achieved by holding down some combination of left shift, right shift,

ctrl and alt all at the same time (the exact combination is specified in

the .TR file currently in use). When you do this a list of all the

possible commands will be displayed, and you can then type the required

command key:

F2      simulate a carrier signal (useful if the modem does not provide

this information) [link type should be OTHER to use this command]

Ctrl-F2 simulate NO carrier signal (opposite of F2)

F6      send break, restart session [use this if TeleReplica locks up -

very rare!]

F8      reboot the host computer (useful if host computer "hangs" for any

reason, but make sure TRHOST is run from AUTOEXEC.BAT!)

F9      shell directly to DOS, return to TR later by typing "exit"

F10     exit TeleReplica [optionally shell out and return later]

Home    refresh display from HOST [use this if a bad 'phone line messes

up the screen]

PgUp    upload file using DMP protocol [transfer file(s) from replica to

host]

PgDn    download file using DMP protocol [transfer file(s) from host to

replica]

Note that before performing a file transfer, the host computer must be at

the DOS prompt, and TRGET.EXE and TRPUT.EXE must be accessible in the

current path (or alternatively the .TR file must contain full paths, not

just (say) TRGET but rather C:/TR/TRGET - note forward slashes!). If

TRGET/TRPUT cannot be found, then the host will display a "command not

found" error message, but TR will go on waiting for a response from

TRGET/TRPUT! Pressing any key will terminate this waiting and bring up an

appropriate error message.

If a file transfer needs to access a file in another directory, it may be

convenient to shell out of TR, change directories, and then exit from DOS

back into TR to start the file transfer. TR will transmit any path that

you specify, so if the two PCs have equivalent directory structures it is

easy to transfer files to the corresponding directory on the other PC

(see Appendix B for a sample file transfer session).

7.    TeleReplica Utility Programs

7.1   TRREMOVE

This will search for a copy of TRHOST loaded in memory. If one is found,

then it is checked to see whether it is currently in use (if so, TRREMOVE

will ask for confirmation before removing it). If it is not in use, or if

it is to be removed anyway, all interrupts, the interrupt mask register

and the serial chip state are restored to their initial state when TRHOST

started running, and TRHOST's memory is freed.

Please note that as with most TSR programs, this may not work unless

TRHOST was the most recent TSR to be loaded. In general TSR programs

should be unloaded in the reverse order they were loaded. In this case

"not working" means that any TSR loaded after TRHOST may cease to

function. If this is not possible then the use of marknet/relnet (or some

other mark/release utility that preserves the interrupt mask register as

well as the vectors) is recommended.

It is sometimes convenient to be able to keep a modem connection even

though TRHOST is being unloaded. Normally, TRREMOVE will restore the

serial port state to its original state when TRHOST was first run.

However, it is possible to force the DTR line to be left active by

entering:

TRREMOVE ON

and DTR can be forced off by entering:

TRREMOVE OFF

If either ON or OFF is specified, then TRREMOVE assumes you know what you

are doing and does not ask for confirmation even if a link is active,

allowing TRREMOVE to be used in a batch file on the host PC without

pausing for input.

7.2   TRHOSTC

This program works much like TRHOSTIN, except that instead of modifying

files on disk it modifies the copy of TRHOST currently active in memory.

Not all parameters may be altered "on the fly" - for example, the COM

port cannot be altered mid-session. The anticipated use for this program

is to do things like turning private mode on or off for just part of a

session...

Note that the copy of TRHOST.EXE on disk is not modified.

7.3   TRTALK

TRTALK will allow the users at the host and replica computers to "talk"

to each other. Anything that is typed at the host computer is displayed

in the top half of the screen, and anything that is typed at the replica

computer is displayed in the bottom half of the screen.

TRTALK sends a special command to TR to tell TR that talk mode is

required, so that TR will send keys in a way that can be distinguished

from the host keys. If this command is lost, then what is typed at either

end will appear in the top window! If this happens, simply exit TRTALK

(Alt-X) and run it again.

TRTALK also sends a special command to TR to tell TR to end talk mode. If

this command is lost, then TR will behave normally unless a program that

uses shift-key status is run, which will cause problems because in talk

mode TR does not send shift status information! To correct this problem,

use local TR command <F2> (simulate carrier), since a side effect of this

command is to end talk mode.

7.4   TREXTS and TREXTR

These programs are normally called from TRSZ.BAT and TRRZ.BAT

respectively. They allow a clean interface between TeleReplica and

external file transfer programs such as DSZ.

These programs are an exception to the rule that TRHOSTIN installs only

programs used at the host end; TREXTS.EXE and TREXTR.EXE both need to run

at both ends, so they must be installed for the correct modem port etc on

the replica as well as the host.

Once TRHOSTIN has installed these programs correctly on both the host and

the replica, and TRRZ.BAT, TRSZ.BAT and the external protocol program

(such as DSZ.EXE) are in some directory on the path of both the host and

replica, file transfer is quite simple:

7.4.1 To download a file from the host PC to the replica PC, simply enter the

following command on the host PC:

trsz <filename> ...

and TR will automatically run TRRZ on the replica without you doing

anything!

7.4.2 To upload a file from the replica PC to the host PC, first ensure that

the host PC is at the DOS command prompt. Then shell out of TR (using

local F9). Finally enter the following command on the replica PC:

trsz <filename> ...

and TRHOST will automatically run TRRZ on the host.

TRRZ.BAT, TRSZ.BAT, TREXTS.EXE and TREXTR.EXE all pass parameters on to

the external protocol program, so parameters other than file names can be

used if desired.

TREXTS.EXE and TREXTR.EXE recognize one special parameter:

#portx#

which will substitute in the DSZ portx <base>,<IRQ> command. This allows

TRRZ.BAT and TRSZ.BAT to remain unchanged even when TeleReplica is

installed for a different port. See the supplied TRRZ.BAT and TRSZ.BAT

for examples of this.

7.5   TRMODSET

Usage: TRMODSET [-fr] [filename]

TRMODSET will send all the commands in TRMODSET.DAT (or in filename.DAT

if specified) to the modem, waiting for an "OK" response to each command.

This is useful to set up very complicated parameters necessary for some

of the more sophisticated modems. It assumes a modem that uses the Hayes

AT command set (at least at&f to restore factory defaults and OK in reply

to a good command).

The -r command line option will cause TRMODSET to start with an "at&f"

command to reset the modem to factory defaults, just in case a previous

program has left the modem in a peculiar state. The "at&f" is sent before

TRMODSET starts monitoring for command echo or "OK" responses.

If a carrier is present when TRMODSET runs then it will exit without

doing anything unless the "-f" (for "force") command line argument is

present. This allows TRMODSET to be run from AUTOEXEC.BAT without

disrupting an existing connection if the host PC needs to be rebooted.

Note that TRMODSET is installed by TRHOSTIN, so if you want to use

TRMODSET on the replica PC you will need to install the host programs on

the replica PC (once installed, the other host programs may be deleted,

but they must be present for TRHOSTIN to run).

Note also that you cannot use TRMODSET to set the modem to "no echo" or

"quiet" modes, since TRMODSET monitors that commands are being echoed and

that the modem is responding with "OK" to each command.

7.6   TRCHECK

TRCHECK checks to see if the current time is within a specified range,

and returns the DOS errorlevel set to 0 if within the range and 1 if

outside the range. The following code might appear in an AUTOEXEC.BAT

file on a host computer:

trcheck 9:00:00 17:00:00

if not errorlevel 1 goto done

trhost

:done

The effect of this is to first check to see if the time is between 9AM

and 5PM. If so, then errorlevel is set to 0, and nothing further happens.

However, if the time is not between 9 and 5, then trhost will be run.

This has proved valuable when TRHOST must run on an office computer

outside normal working hours, but the computer must be used normally

during the day. See also scenario A5 for a further application.

7.7   TRASK

Usage: TRASK <timeout> <prompt> <answers>

TRASK works much like a normal batch file enhancing ASK program, except

that it has a timeout. This is useful when a computer will usually be

used as a TeleReplica host, but sometimes must be used locally. Rather

than having to boot from a different disk, the following code might

appear in an AUTOEXEC.BAT file on a host computer:

@echo off

path c:\tr

trask 5 "Do you want to load TRHOST? (y/n)\a\a: " "yn"

if errorlevel 1 goto no

trhost

:no

The effect of this is to run TRHOST if the user presses the 'y' key or if

no key has been pressed after 5 seconds. If a user is at the computer and

answers 'n' then TRHOST will not be loaded. See scenario A5 for an

example of this.

TRASK returns an errorlevel that depends on the <answers> string: the

first letter in this string (or a timeout) returns error level 0, the

second returns error level 1 and so on. Case is not significant unless

the <answers> string contains the same letter in both upper and lower

case.

TRASK is a public domain program and may be distributed separately.

APPENDIX A - Configuration Scenarios

The following are specific situations in which TeleReplica has been used.

The descriptions of the situations, and the resulting configurations of

TeleReplica, are presented in detail. It is hoped that seeing the way

TeleReplica was installed in some typical applications will help when it

comes to installing TR for your own situation.

Note that unless otherwise stated, all modems are assumed initially to be

in the TeleReplica default state, to be connected to the PC via a 25-way

straight-through modem cable, and to be Hayes-AT-command-compatible. To

get a Hayes-AT-command-compatible modem into the TeleReplica default

state the following commands should be sent to the modem using any normal

communications program (or using TRMODSET):

at &f      restore modem to "factory default"

at &c1     modem should report true carrier status

at &d2     modem should take notice of DTR to hang up 'phone

at x4      (optional) give detailed status messages etc

at &w      save current state for future use (the state is saved in

non-volatile RAM, so that this setup only needs to be done

once unless some other program interferes)

To ensure that TeleReplica is talking to the modem, I recommend the

following sequence (at both the host and the replica PC):

1.  Run TRHOSTIN and select the COM port and IRQ line that are

connected to your modem (use the X option first to have TRHOSTIN

search your hardware for available ports and active IRQ lines).

2.  Set the "baud" rate to the appropriate speed for your modem.

3.  Save the configuration.

4.  Run TRMODSET. This not only sets the modem into a reasonable state

for TeleReplica, but in doing so it will confirm (if everything is

OK) or deny (if it reports an ERROR) that you have indeed selected

the right address and interrupt for your modem. There is no point

in proceeding further until TRMODSET will run and report

everything OK.

A1.   Consultant/Client via Modem

A1.1  Here the consultant is running a 2400 bps modem connected to COM1, and

the client (whom we will call Fred) is running a similar modem also

connected to COM1. Both computers are equipped with EGA color display

cards. The consultant will be performing the TRHOST installation for the

client, and then sending the client the installed version ready to run.

When the client wants help, he will call the consultant (a normal voice

call). When both parties are ready TeleReplica must take over the call.

A1.2  First the consultant must run TRHOSTIN, and make the changes necessary to

achieve the following display:

������������������������������������������������������������������ͻ

� TeleReplica Host 4.23 Installation Copyright (c) Douglas Thomson �

�    A. Select serial port (currently COM1)                        �

�    B. Change password (currently "Fred's password")              �

�    C. Change modem initialization string (currently:             �

�       "")                                                        �

�    D. Change modem dialling format string (currently:            �

�       "ata\r")                                                   �

�    E. Select maximum "baud" rate (currently 2400)                �

�    F. Use modem CARRIER DETECT signal (currently YES)            �

�    G. Flow control (currently NONE)                              �

�    H. Generate INT 9 on each key (currently NO)                  �

�    I. Respond to modem RING signal (currently NO)                �

�    J. Reboot on loss of carrier (currently NO)                   �

�    K. Synchronize "baud" rate (currently YES)                    �

�    L. Send wakeup every 2 minutes (currently NO)                 �

�    M. Private mode (currently NO)                                �

�    N. Printer redirection (currently HOST)                       �

�    O. Maximum call length (currently unlimited)                  �

�    P. Create consistent .TR file                                 �

�    Q. Abandon changes                                            �

�    S. Save changes and leave installation program                �

� Selection:                                                       �

������������������������������������������������������������������ͼ

Refer to section 4 for the meaning of each option. Since both modems run

at the same speed it does not matter whether "baud" rates get

synchronized or not. Since Fred will only run TRHOST when he wishes to

get help from the consultant, the password is not important. The modem

dialling format string simply answers the 'phone - or more accurately,

since the 'phone will already be in use for the voice call, it causes the

modem to take over the call in "answer" mode.

Next the consultant selects option P from the main menu, and enters the

name of the configuration file he wishes to use for Fred:

������������������������������������������������������������������ͻ

� TeleReplica Host Program .TR File Name Selection                 �

�                                                                  �

� Edit existing string:                                            �

� fred                                                             �

������������������������������������������������������������������ͼ

Finally the consultant exits TRHOSTIN by selecting option S to save the

changes. The consultant can now send Fred a disk with all the installed

TeleReplica files on it (note that the consultant should also send Fred

an unaltered original copy of the TeleReplica files in case Fred wants to

pass them on to anyone - see the LICENCE.DOC file for details).

A1.3  Now the consultant needs to edit the file named FRED.TR to make a few

alterations (TRHOSTIN only provides a starting point...). In particular,

the following line must be changed:

""                Connect command for modem

to:

"at d\r"          Connect command for modem

This command tells the modem to go "off hook" and take over the call in

"originate" mode.

This completes the installation stage.

A1.4  When Fred wants some help, he will run TRHOST and then 'phone the

consultant and explain the problem. When the consultant is ready, he will

enter the command:

TR FRED

and then instruct Fred to enter the command:

TRDIAL

and hang up the 'phone as soon as he (Fred) hears his modem making a

"screaming" noise. After giving these instructions (and thus while Fred

is still typing) the consultant will hang up his 'phone. Fred will finish

entering the TRDIAL command, and hang up when he hears the screaming

noise. TeleReplica will then take over the call, synchronize "baud"

rates, exchange passwords, and give the consultant control of Fred's

computer.

A1.5  When the consultant is finished, he will exit TR using option 3, which

will cause the modem to hang up and terminate the 'phone call.

Fred can either leave TRHOST running (it will do no harm, and uses only a

little over 10K of memory), or else use TRREMOVE to uninstall TRHOST.

A2.   Remote/Shared via Modem

A2.1  Here a remote user wishes to connect into a local area network, and one

node in the LAN is dedicated to this purpose. The main problem is that

since the LAN node will be unattended, all it takes is one user to run a

program which "hangs" the computer, and then no-one will be able to dial

in until someone pushes the reset button on the node PC! However, in this

scenario there is a modem-controlled switch box which turns on power to

the node PC as soon as the 'phone starts ringing, and leaves the power on

until either a minute or so has elapsed with no carrier being

established, or until the carrier is lost (which will happen when the

remote user hangs up). [These switch boxes are (I believe) commercially

available, and indeed the electronics required is trivial to someone who

feels at home with circuit diagrams and a soldering iron. We have several

home-made ones in use here at MUCG.] This means there is no problem if a

user "hangs" the computer; just hang up and then dial in again, and the

computer will have been powered down and come up again clean!

Since many users must share the remote access PC, the duration of a

session is to be limited so that one user cannot tie up the line for more

than 30 minutes.

The node PC has a 2400 bps modem connected to COM2, and the remote user

is running a 1200 bps modem connected to COM1. The node PC is equipped

with a CGA color display card (with no monitor required). The remote PC

is equipped with a Hercules monochrome display.

The LAN manager will be performing the TRHOST installation for the node

PC, and then providing all the remote users with a disk that is almost

ready to go.

The 'phone number of the node PC is 123 4567.

A2.2  First the LAN manager must run TRHOSTIN, and make the changes necessary

to achieve something like the following display:

������������������������������������������������������������������ͻ

� TeleReplica Host 4.23 Installation Copyright (c) Douglas Thomson �

�    A. Select serial port (currently COM2)                        �

�    B. Change password (currently "sdggsdjgxvwept9m,.mvikjdsdh")  �

�    C. Change modem initialization string (currently:             �

�       "\rata\r")                                                 �

�    D. Change modem dialling format string (currently:            �

�       "\r%s\r")                                                  �

�    E. Select maximum "baud" rate (currently 2400)                �

�    F. Use modem CARRIER DETECT signal (currently YES)            �

�    G. Flow control (currently NONE)                              �

�    H. Generate INT 9 on each key (currently NO)                  �

�    I. Respond to modem RING signal (currently NO)                �

�    J. Reboot on loss of carrier (currently NO)                   �

�    K. Synchronize "baud" rate (currently YES)                    �

�    L. Send wakeup every 2 minutes (currently NO)                 �

�    M. Private mode (currently YES)                               �

�    N. Printer redirection (currently REPLICA)                    �

�    O. Maximum call length (currently 1800 s)                     �

�    P. Create consistent .TR file                                 �

�    Q. Abandon changes                                            �

�    S. Save changes and leave installation program                �

� Selection:                                                       �

������������������������������������������������������������������ͼ

Refer to section 4 for the meaning of each option. Note that the modem

initialization string answers the 'phone - this is nice in the case of

timed call charges, since the remote user does not pay for the time it

takes the PC to boot. Since some remote users will dial in at 2400 bps,

TRHOST must be installed for 2400; but since other remote users will dial

in at 1200 bps it is essential that "baud" rates get synchronized. Since

anyone could try to get into the LAN via the node PC the password is very

important - notice that it is both long and quite random. The LAN node PC

is in a public location, so the screen is blanked and keyboard locked for

privacy. Printer output is sent back home to the users. The maximum

session length allowed is set to 1800 seconds (30 minutes).

Next the LAN manager selects option P from the main menu, and enters the

name of the configuration file he wishes to use for the node PC:

������������������������������������������������������������������ͻ

� TeleReplica Host Program .TR File Name Selection                 �

�                                                                  �

� Edit existing string:                                            �

� lan1                                                             �

������������������������������������������������������������������ͼ

Finally the LAN manager exits TRHOSTIN by selecting option S to save the

changes. He can now make up a disk with all the installed TeleReplica

files on it to give to the remote users, but first he must edit the

LAN1.TR file to install the node PC's 'phone number. This means changing

the line:

""                Connect command for modem

to:

"at d 123 4567\r" Connect command for modem

so that when remote users run TR the number will be dialled

automatically.

Also, since modems more often use COM1 than COM2, the LAN manager should

probably edit the line:

02F8/IRQ3         Communications port (COMn or base/interrupt)

to become:

"COM1"            Communications port (COMn or base/interrupt)

A2.3  Now the remote user needs to edit his personal copy of the file named

LAN1.TR to work at 1200 bps. The line containing:

2400              "Baud" rate (typically 2400 or 19200)

must be changed to:

1200              "Baud" rate (typically 2400 or 19200)

This completes the installation stage.

A2.4  When the remote user wishes to connect into the LAN, he will enter the

command:

TR LAN1

This will cause the modem to dial the node PC, which will cause the modem

switch box to turn on the power to the node PC, which will boot and then

run TRHOST, which will make the node PC's modem answer the call.

TeleReplica will then synchronize "baud" rates, exchange passwords, and

give the remote user control of the node PC.

A2.5  When the remote user is finished, he will exit TR using option 3, which

will cause the modem to hang up and terminate the 'phone call. The node

PC will then be powered down until the next call.

A3.   Laptop/Desktop via Direct Cable

A3.1  Here someone wishes to connect a laptop computer to a desktop computer

via a three-wire null modem cable. The reason for this is that the

desktop only has a 5.25" disk, and the laptop only has a 3.5" disk, and

it is necessary to transfer files between the two systems. The desktop

computer is a 20MHz 80386, and the laptop is a 12MHz 80286. The cable is

connected from COM1 on the desktop to COM1 on the laptop. The desktop has

a VGA color display; the laptop has a gas plasma display that emulates

EGA.

It does not matter which PC is the host, but for the sake of this example

we will assume the desk PC will be the host.

The laptop computer has its <Del> key right next to <RightShift>, so the

user does not wish to be using <Ctrl-Alt-LeftShift-RightShift> for fear

of accidentally rebooting the computer. <Ctrl-Alt-LeftShift> seems a

safer combination.

A3.2  First the user must run TRHOSTIN, and make the changes necessary to

achieve something like the following display:

������������������������������������������������������������������ͻ

� TeleReplica Host 4.23 Installation Copyright (c) Douglas Thomson �

�    A. Select serial port (currently COM1)                        �

�    B. Change password (currently "password")                     �

�    C. Change modem initialization string (currently:             �

�       "")                                                        �

�    D. Change modem dialling format string (currently:            �

�       "")                                                        �

�    E. Select maximum "baud" rate (currently 38400)               �

�    F. Use modem CARRIER DETECT signal (currently NO)             �

�    G. Flow control (currently NONE)                              �

�    H. Generate INT 9 on each key (currently NO)                  �

�    I. Respond to modem RING signal (currently NO)                �

�    J. Reboot on loss of carrier (currently NO)                   �

�    K. Synchronize "baud" rate (currently NO)                     �

�    L. Send wakeup every 2 minutes (currently NO)                 �

�    M. Private mode (currently NO)                                �

�    N. Printer redirection (currently HOST)                       �

�    O. Maximum call length (currently unlimited)                  �

�    P. Create consistent .TR file                                 �

�    Q. Abandon changes                                            �

�    S. Save changes and leave installation program                �

� Selection:                                                       �

������������������������������������������������������������������ͼ

Refer to section 4 for the meaning of each option. Notice that the "baud"

rate is set to 38400 for fast file transfers (this works well on my test

system, but slower speeds may be needed on other systems - experiment!).

Obviously here the password is not important. Since the three-wire cable

will not provide any carrier information, it is essential that carrier be

ignored.

Next the user selects option P from the main menu, and enters the name of

the configuration file he wishes to use for the desktop:

������������������������������������������������������������������ͻ

� TeleReplica Host Program .TR File Name Selection                 �

�                                                                  �

� Edit existing string:                                            �

� desk                                                             �

������������������������������������������������������������������ͼ

Finally the user exits TRHOSTIN by selecting option S to save the

changes.

A3.3  Next the user needs to edit the file named DESK.TR to use the shift key

combination required for the laptop. The line containing:

"LRCA"            Local command shifts (Left, Right, Ctrl, Alt)

must be changed to:

"LCA"             Local command shifts (Left, not Right, Ctrl, Alt)

The user can now copy the installed files onto a 5.25" floppy disk, take

the disk to a computer that has both 5.25" and 3.5" inch drives, and copy

the file onto a 3.5" disk for the laptop.

This completes the installation stage.

A3.4  When the user wishes to connect the two computers, he will enter the

command:

TRHOST

on the desktop, and the command:

TR DESK

on the laptop. This will cause TeleReplica to exchange passwords and give

the user control of the desktop computer from the laptop. Files can now

be transfered at almost 3.8K per second!

A3.5  When the user is finished, he will exit TR using option 3, which will

take the laptop back to normal, but leave TRHOST in the desktop's memory

(see A1.5 for further discussion of this).

A4.   Remote/Shared via Common Mini-computer

A4.1  Here a remote user wishes to connect into a local area network, and one

node in the LAN is dedicated to this purpose. There is no direct link

between the PCs; instead, both the node PC and the remote user must log

into the same mini-computer and run a link program (such as trfifo) to

establish a connection via the mini-computer.

One problem is that since the LAN node will be unattended, all it takes

is one user to run a program which "hangs" the computer, and then no more

remote users will be able to connect to the LAN until someone pushes the

reset button on the node PC! (Note that TeleReplica does support

rebooting the host PC, but this can only work if interrupts are not

disabled and if the relevant code has not been overwritten. As a rule,

TeleReplica can get out of any situation that <Ctrl-Alt-Del> could get

out of, but if the hardware reset button was needed TR would be

helpless.)

Another potential problem is that if one user exits TR while still logged

in to the LAN, then the next user could pick up the session! This is

avoided by having the link program send a "reboot" command to the node PC

when the link is broken.

Trfifo cannot pass BREAKs from one end to the other; instead, BREAK is

used to terminate the execution of trfifo. This means that TR must not

send a BREAK unless it wants to kill the link.

To make life complicated, the mini-computer will automatically disconnect

any terminal that does no input or output for 5 minutes. However, the

node PC must remain connected to the mini-computer all day waiting for

remote users to connect in.

The mini-computer links are set at 9600 bps, and cannot be changed. Both

computers are connected to the mini-computer via COM1, and both computers

are running color displays (however, the dedicated node PC requires no

monitor).

The LAN manager will be performing the TRHOST installation for the node

PC, and then providing all the remote users with a disk that is almost

ready to go.

A4.2  First the LAN manager must run TRHOSTIN, and make the changes necessary

to achieve something like the following display:

������������������������������������������������������������������ͻ

� TeleReplica Host 4.23 Installation Copyright (c) Douglas Thomson �

�    A. Select serial port (currently COM1)                        �

�    B. Change password (currently "sdfddffgjkiou77hgsduh875sdh")  �

�    C. Change modem initialization string (currently:             �

�       "")                                                        �

�    D. Change modem dialling format string (currently:            �

�       "")                                                        �

�    E. Select maximum "baud" rate (currently 9600)                �

�    F. Use modem CARRIER DETECT signal (currently NO)             �

�    G. Flow control (currently NONE)                              �

�    H. Generate INT 9 on each key (currently NO)                  �

�    I. Respond to modem RING signal (currently NO)                �

�    J. Reboot on loss of carrier (currently NO)                   �

�    K. Synchronize "baud" rate (currently NO)                     �

�    L. Send wakeup every 2 minutes (currently YES)                �

�    M. Private mode (currently YES)                               �

�    N. Printer redirection (currently REPLICA)                    �

�    O. Maximum call length (currently unlimited)                  �

�    P. Create consistent .TR file                                 �

�    Q. Abandon changes                                            �

�    S. Save changes and leave installation program                �

� Selection:                                                       �

������������������������������������������������������������������ͼ

Refer to section 4 for the meaning of each option. The password is not so

critical here, since a user must be able to log into the mini-computer

before getting through to the node PC. The mini-computer line will not

provide any carrier information, so this must be ignored. "Baud" rates

must NOT be synchronized, since a NULL character transmitted at 2400 bps

would be received as a BREAK by the mini-computer at 9600 bps! To avoid

the node PC getting logged out, TRHOST must send a NULL character to

"wake up" the mini-computer every few minutes. This will be ignored by

TR. Printer output is redirected back to the users. The LAN PC is run

with screen blank and keyboard dead for added privacy and security.

Next the LAN manager selects option P from the main menu, and enters the

name of the configuration file he wishes to use for the node PC:

������������������������������������������������������������������ͻ

� TeleReplica Host Program .TR File Name Selection                 �

�                                                                  �

� Edit existing string:                                            �

� lan2                                                             �

������������������������������������������������������������������ͼ

Finally the LAN manager exits TRHOSTIN by selecting option S to save the

changes. He can now make up a disk with all the installed TeleReplica

files on it to give to the remote users.

A4.3  Due to the way trfifo buffers input (or possibly some other peculiarity

of our mini-computer?) the delay between sending keys needs to be 165 ms

to avoid losing characters! Therefore, the line containing:

56                Time to wait between "typing" keys (ms)

must be changed to:

165               Time to wait between "typing" keys (ms)

Also, since we are using a mini-computer link, the line containing:

"DIRECT"          Type of link (MODEM, DIRECT, or OTHER)

must be changed to:

"OTHER"           Type of link (MODEM, DIRECT, or OTHER)

Finally, TR is capable of automating the mini-computer login process. If

this is desired, then something like the following lines must be added at

the end of the file:

"DISCONNECTED"      "\D250\r"            "\r"    5000

"Enter RESOURCE ? " "\D100G9000\r"       "\r"    5000

"CONNECTED TO"      ""                   ""      5000

"login: "           "\D100doug\r"        "\B\r"   500

"Password:"         "\D100password\r"    "\r"   10000

"Selection: "       "\D100d\r"           ""     40000

"/users/doug>"      "\D100trfifo lan2\r" ""     40000

"and run TR"        "\D1000"             ""     40000

The exact strings will depend on the mini-computer. This example (apart

from the password) works for the author.

This completes the installation stage.

A4.4  When the remote user wishes to connect into the LAN, he will run TR:

TR LAN2

which will automatically log into the mini-computer and run trfifo to

connect to the node PC (in this case lan2).

TeleReplica will then exchange passwords and give the remote user control

of the node PC.

A4.5  When the remote user is finished, he will exit TR using option 3, which

will send a BREAK to the mini-computer. This will terminate the trfifo

program which the remote user was running. The trfifo program that the

node PC was running will detect that this has happened, and send a

TeleReplica reboot command to the node PC to ensure that the next user

cannot pick up anything the previous user left running. The node PC's

trfifo program will then prepare for the next remote user. The node PC

will complete rebooting, and run TRHOST from AUTOEXEC.BAT ready for the

next remote login.

[All this sounds complicated, but we used this system daily here at MUCG

for some staff who had mini-computer lines but were too far away for LAN

connections.]

A5.   Home/Office via Modem

A5.1  Here a user wishes to dial in from home and take over the PC on his desk

at work. The office PC has a USR Courier V32bis/V42bis modem connected to

COM2, and the home PC has an identical modem connected to COM1. The

office PC is equipped with an EGA color display, and the home PC is

equipped with a Hercules monochrome display.

One problem is that since the office PC will be unattended, all it takes

is for the user to run a program which "hangs" the computer, and that

will be the end of the office PC until the user goes back to work and

presses the reset button! (Note that TeleReplica does support rebooting

the host PC, but this can only work if interrupts are not disabled and if

the relevant code has not been overwritten. As a rule, TeleReplica can

get out of any situation that <Ctrl-Alt-Del> could get out of, but if the

hardware reset button was needed TR would be helpless.)

Since the user is putting in overtime working at night without getting

paid for it, it seems reasonable that the office 'phone should at least

pay for the call. However, how can the office computer know when to call

the user at home? One possibility would be to set up a batch file to run

TRDIAL at a predetermined time. However, it would be better if the user

could choose any convenient time. TeleReplica can achieve this by

monitoring the RING signal from the modem, and responding appropriately.

The basic idea is that the user will dial the office number, and then

hang up as soon as the 'phone starts ringing. TeleReplica will detect the

'phone ringing, and respond by calling the user at home.

Since the user does not want to be bothered by nuisance calls from the

office computer, TeleReplica needs to be quite selective about the ring

patterns it will respond to. Too short a ring may mean a telephone

company routine line test. Too long a ring will mean a person trying to

ring the office.

The 'phone number of the office PC is 123 4567.

The 'phone number of the home PC is 765 4321.

A5.2  First the user must run TRHOSTIN, and make the changes necessary to

achieve something like the following display:

�����������������������������������������������������������������������ͻ

�plica Host 4.23 Installation Copyright (c) 1992 Douglas Thomson        �

�Select serial port (currently COM2)                                    �

�Change password (currently "fhkjeuyibn45938eh8fdfff631wjds874dffd")    �

�Change modem initialization string (currently:                         �

�"at &B1 &H1 S0=0\r")                                                   �

�Change modem dialling format string (currently:                        �

�"\r%s\r")                                                              �

�Select maximum "baud" rate (currently 38400)                           �

�Use modem CARRIER DETECT signal (currently YES)                        �

�Flow control (currently CTS)                                           �

�Generate INT 9 on each key (currently NO)                              �

�Respond to modem RING signal (currently YES)����� 1. too short  220 ms �

�Reboot on loss of carrier (currently YES)      �� 2. too long  5000 ms �

�Synchronize "baud" rate (currently NO)         �� 3. delay    10000 ms �

�Send wakeup every 2 minutes (currently NO)     �� 4. reset    10000 ms �

�Private mode (currently YES)                   �� 5. send to modem     �

�Printer redirection (currently REPLICA)        �� 6. response Ŀ       �

�Maximum call length (currently unlimited)            "atdt 765 4321\r" �

�Create consistent .TR file                                             �

�Abandon changes                                                        �

�Save changes and leave installation program                            �

�ion:                                                                   �

�����������������������������������������������������������������������ͼ

Refer to section 4 for the meaning of each option.

Since anyone could try to get into the office PC the password is

important - notice that it is both long and quite random.

The modem initialisation string changes a USR V32bis modem from its

factory default settings to the following:

&B1   Fixed DTE/DCE rate. This means that the modem will always talk

to the PC at 38400 bps, regardless of what kind of telephone

connection is made.

&H1   Hardware flow control when transmitting data to the modem (no

flow control is needed in the other direction, since

TeleReplica can keep up with data arriving at 38400 bps).

S0=0  The office phone should never answer calls.

Since the modem can transmit 1700 bytes per second, and V42bis

compression may improve this to around 3400 bytes per second for things

like screen updates, the "baud" rate should be at least 34000 bps, and

38400 is the lowest speed over 34000 that the modems can accommodate.

It is essential that TRHOST takes notice of the CD signal, since the host

computer will be automatically rebooted if the carrier is lost (see

below).

Flow control is essential, since the PC will be sending data to the modem

at 38400 bps and the modem can only guarantee to get rid of it at about

17000 bps! Hardware flow control (CTS) is selected to match the modem

configuration.

The inter-character gap should be set to a fairly long time; the modems

support error correction, so TRHOST should be patient and give the modems

time to sort out problems introduced by line noise. 5-10 seconds is a

reasonable time:

����������������������������������������������������������ͻ

� TeleReplica Host Program Flow Control Selection          �

�                                                          �

�    0. Completed, no (further) change                     �

�    1. No flow control                                    �

� => 2. Hardware (RTS/CTS) flow control                    �

�    3. Software (XON/XOFF) flow control                   �

�    4. Both hardware and software flow control            �

�    5. Set inter-character gap time (currently 5000 ms)   �

�                                                          �

� Selection:                                               �

����������������������������������������������������������ͼ

The host computer is to monitor the 'phone ringing and respond

accordingly (again, see below).

"Baud" rate synchronization is not appropriate here, since the DTE/DCE

speed is locked.

The host is set in PRIVATE mode so that no-one walking through the office

can read what is on the screen or type commands on the keyboard (very

useful if the user will be logging into a LAN as SUPERVISOR!).

Printer output is redirected to the replica PC, so that any printout

comes out at home rather than in the office.

Next the user selects option P from the main menu, and enters the name of

the configuration file he wishes to use for the office:

������������������������������������������������������������������ͻ

� TeleReplica Host Program .TR File Name Selection                 �

�                                                                  �

� Edit existing string:                                            �

� office                                                           �

������������������������������������������������������������������ͼ

Finally the user exits TRHOSTIN by selecting option S to save the

changes. He can now make up a disk with all the installed TeleReplica

files on it to take home.

Before leaving the office, the user must ensure that AUTOEXEC.BAT has the

directory containing all the TeleReplica files on the path, and that

TRHOST is run whenever the system boots. However, when the user is at the

office he will not want TRHOST loaded, so AUTOEXEC.BAT should contain

something like:

prompt $p$g

path c:\tr

trask 5 "Do you want to load TRHOST? (y/n): \a\a" "yn"

if errorlevel 1 goto no

trhost

:no

This will ask the user at the host PC whether or not to load TRHOST (the

\a makes TRASK beep), and if the user does not type anything within 5

seconds TRHOST will be loaded anyway. If the user is at the office, then

typing 'n' will prevent TRHOST from loading.

A5.3  At home the user needs to edit the file named OFFICE.TR to install the

office PC's 'phone number. This means changing the line:

""                Connect command for modem

to:

"at d 123 4567\r" Connect command for modem

so that when the user runs TR the number will be dialled automatically.

Next, since the home modem is on COM1 the line:

02F8/IRQ3         Communications port (COMn or base/interrupt)

must become:

"COM1"            Communications port (COMn or base/interrupt)

Also, since the modems support error correction, and since line noise can

introduce a significant gap in the transmitted data at any time, the

timeouts should be made fairly generous by changing the lines:

3000              Time to wait for first char before timeout (ms)

500               Time to wait for subsequent chars in block (ms)

to:

10000             Time to wait for first char before timeout (ms)

10000             Time to wait for subsequent chars in block (ms)

Finally, TR should respond to the modem ringing by answering the 'phone.

This means adding an extra line at the end of the file containing:

"RING"        "\D250ata\r"    ""         40000

(Note the \D250 to wait a little bit for the modem to finish sending the

carriage return and line feed after RING before sending the answer

command. With some modems this may not be necessary; with others, a

different delay may work better. If the answer command is ignored, then

try a different delay.)

This completes the installation stage.

A5.4  Before leaving the office, the user must remember to reboot the PC (thus

loading TRHOST) and leave it turned on! However, it would be a good idea

to switch the monitor off.

When the user is ready to dial into the office PC from home, he will

enter the command:

TR OFFICE

This will cause the modem to dial the office PC. The user must then

listen to the modem and wait until he hears the 'phone ring once. As soon

as he hears this, he must press any key (say the space bar) to terminate

the call. TRHOST will start a timer going when the 'phone rings, and then

wait 10 seconds, keeping track of the total time the 'phone has spent

ringing. If, during the 10 seconds, the 'phone was ringing for more than

0.2 seconds but less than 5 seconds, then TRHOST will send the dial

command (atdt 765 4321) to the modem, causing the office modem to ring

home. When the user's 'phone rings, the modem will send the message:

RING

to the home PC, and TR will trigger on this to respond to the modem with:

ata

which will answer the 'phone. TeleReplica will then exchange passwords

and give the user control of the office PC.

A5.5  When the user is finished, he will exit TR using option 3, which will

cause the modem to hang up and terminate the 'phone call. The office PC

will then automatically reboot, ready for the user to call in again.

[This is probably the most complicated set up of all, but it is the one

the author has found most useful, and is in daily use by a number of

staff at MUCG.]

APPENDIX B - Sample File Transfer Session

B1.   In this example, we will assume that a good link has been established as

described in scenario A2. The user has logged into the LAN from home, and

wishes to transfer all the C source files from the LAN directories:

F:\USERDATA\FRED\TEST

F:\USERDATA\FRED\UTILS

F:\USERDATA\FRED\PROBLEM

to the corresponding directories on his home PC:

C:\PROGRAMS\TEST

C:\PROGRAMS\UTILS

C:\PROGRAMS\PROBLEM

The current working directory on the LAN PC is F:\USERDATA\FRED. The

current working directory on the home PC is C:\TR.

First, the user needs to make C:\PROGRAMS the current working directory

on his home PC. To do this, he must hold down Ctrl, Alt, LeftShift and

RightShift all at the same time! This will bring up a screen similar to:

������������������������������������������������������������������ͻ

� TeleReplica Copyright (c) 1992 Douglas Thomson                   �

�                                                                  �

�  Key         Effect                                              �

�  ===         ======                                              �

�  <F2>      - simulate carrier - not used with MODEM link         �

�  <Ctrl-F2> - simulate NO carrier - not used with MODEM link      �

�  <F6>      - send break, restart session                         �

�  <F8>      - perform warm boot on host                           �

�  <F9>      - shell to DOS                                        �

�  <F10>     - exit TeleReplica                                    �

�  <Alt-X>   - exit TeleReplica                                    �

�  <Home>    - refresh display from HOST                           �

�  <PgUp>    - upload file using DMP protocol                      �

�  <PgDn>    - download file using DMP protocol                    �

�                                                                  �

�  <ESC>     - do nothing                                          �

�                                                     Press key:   �

������������������������������������������������������������������ͼ

Then he should press <F9> to shell to DOS, and enter the command:

CD \PROGRAMS

to make C:\PROGRAMS the current working directory. Then he can enter the

command:

EXIT

to return to TR.

Next he needs to start the download. To do this he must again hold down

Ctrl, Alt, LeftShift and RightShift, and then press <PgDn>. Another box

will appear:

�����������������������������������������������������������������������ͻ

� Name(s) of file(s) to be downloaded (use DOS wildcards):              �

� WARNING: Files with the same names on this PC will be overwritten!    �

�                                                                       �

�����������������������������������������������������������������������ͼ

and he must enter the relative paths and wildcards:

�����������������������������������������������������������������������ͻ

� Name(s) of file(s) to be downloaded (use DOS wildcards):              �

� WARNING: Files with the same names on this PC will be overwritten!    �

� TEST\*.C TEST\*.H UTILS\*.C UTILS\*.H PROBLEM\*.C PROBLEM\*.H         �

�����������������������������������������������������������������������ͼ

If there were some problem (for example, if TRPUT.EXE were not on the

path of the LAN PC), then the following could be displayed:

������������������������������������������������������������������ͻ

� TeleReplica Error:                                               �

� no ready marker from host                                        �

� Type <ESC> to continue:                                          �

������������������������������������������������������������������ͼ

However, normally the download will then commence, leaving the following

display:

���������������������������������������������ͻ

� File Transfer in Progress...                �

�                                             �

� File:       test\xyz.c                      �

�                                             �

� Error:                                      �

�                                             �

� Throughput: 233.6 cps                       �

�                                             �

� You may:                                    �

�    1. Abort the file transfer               �

�    2. Shell to DOS                          �

�                                             �

� Selection:                                  �

���������������������������������������������ͼ

The number of bytes remaining to be transfered is displayed in the top

right corner of the screen. The most recent error will be reported in the

appropriate line of the box, and the throughput of the transfer (in

characters per second) may help to relieve the boredom while waiting for

the transfer to complete! The box also indicates which file is currently

being transfered.

At this stage, the user can either wait for the transfer to finish, or

else select option 2 from the menu and shell to DOS. In this case, he can

use his home PC for other jobs while the transfer takes place in the

background. TR will continue to display the number of bytes remaining in

the top right corner until the transfer is completed, at which time the

number in the top right corner will become the string "Completed" (or,

possibly, "Doubtful" if one or more files could not be created on the

home PC). However, the user must be careful not to do things like delete

the file that is being downloaded!

While shelled out to DOS in this way, the user may exit back into TR at

any time. If the file transfer is still in progress, then the file

transfer window will still be there; otherwise, the exit will leave TR

displaying the normal replica of the host PC's display.

APPENDIX C - Modem Cables

C1.   Minimum Standard Modem Cable

Although a full 25-wire straight-through male-to-female RS232 cable is

recommended for TeleReplica (since there is less room for error in the

cable construction/selection), in fact the following signals are the only

ones ever required or used:

MALE (modem)                            FEMALE (PC)

=============                           ========

25D RS232 pin     signal       signal   25D   9D (as in AT)

=============     ======       ======   ===   ==

2     TD  <------- TD         2    3

3     RD  -------> RD         3    2

4     RTS <------- RTS        4    7

5     CTS -------> CTS        5    8

7     GND <------> GND        7    5

8     CD  -------> CD         8    1

20     DTR <------- DTR       20    4

22     RI  -------> RI        22    9

The following sections describe the kind of null modem cables that can be

used with TeleReplica.

C2.   Simplest Possible 3-wire Null Modem Cable

This is the most common kind of null modem cable, but TR and TRHOST must

be set to ignore carrier detect, since the cable does not provide this

information.

MALE                                    FEMALE

=============                           ========

25D RS232 pin     signal       signal   25D   9D (as in AT)

=============     ======       ======   ===   ==

7     GND <------> GND        7    5

3     RD  <------- TD         2    3

2     TD  -------> RD         3    2

C3.   Full TeleReplica Null Modem Cable

If you can build or get hold of one of these, then TeleReplica can use

carrier detect correctly. Note that CTS is not provided, and that

TeleReplica must therefore be installed not to use it (there would be no

point is using flow control with a direct cable in any case)!

MALE                                    FEMALE

=============                           ========

25D RS232 pin     signal       signal   25D   9D (as in AT)

=============     ======       ======   ===   ==

7     GND <------> GND        7    5

3     RD  <------- TD         2    3

2     TD  -------> RD         3    2

20     DTR -------> CD         8    1

8     CD  <------- DTR       20    4

C4.   Complete Null Modem Cable

If you really want a complete null modem cable, which should work with

ANY communications program, then I would suggest:

MALE                                    FEMALE

=============                           ========

25D RS232 pin     signal       signal   25D   9D (as in AT)

=============     ======       ======   ===   ==

7     GND <------> GND        7    5

3     RD  <------- TD         2    3

2     TD  -------> RD         3    2

5     CTS <------- RTS        4    7

4     RTS -------> CTS        5    8

20     DTR ----+--> CD         8    1

�

+--> DSR        6    6

8     CD  <--+---- DTR       20    4

�

6     DSR <--+

APPENDIX D - DESQview Compatibility

D1.   TR is "DESQview-Aware" and runs nicely in a small DESQview window. TRHOST

needs to put keys into the BIOS keyboard buffer, and this means that what

is typed at the remote end cannot be distinguished by DESQview from what

is typed at the real keyboard at the host end. The net effect is that

there is no point in trying to run TRHOST in a DESQview window, and so

TRHOST makes no attempt to be "DESQview-Aware". Also, DESQview is

activated by "tapping" the ALT key, but TRHOST has no way to make this

appear to happen, hence DESQview cannot be activated remotely. TRHOST and

DESQview really don't mix!

D2.   For anyone wishing to set up DESQview for TR, these are the settings that

I am currently using successfully:

�1��Change�a�Program���������������������������������������������������ͻ

�                            Change a Program                           �

�                                                                       �

� Program Name............: TR                                          �

�                                                                       �

� Keys to Use on Open Menu: TR                  Memory Size (in K):  70 �

�����������������������������������������������������������������������ĺ

� Program...: C:\TR\TR                                                  �

�                                                                       �

� Parameters: REMOTE                                                    �

�                                                                       �

� Directory.: \TR                                                       �

�����������������������������������������������������������������������ĺ

� Options:                                                              �

�               Writes text directly to screen.......: [N]              �

�               Displays graphics information........: [N]              �

�               Virtualize text/graphics (Y,N,T).....: [N]              �

�               Uses serial ports (Y,N,1,2)..........: [1]              �

�               Requires floppy diskette.............: [N]              �

�                                                                       �

�  Press F1 for advanced options           Press <�� when you are DONE  �

�����������������������������������������������������������������������ͼ

and advanced options:

�1��Change�a�Program���������������������������������������������������ͻ

�                   Change a Program Advanced Options                   �

�                                                                       �

� System Memory (in K):  0    Maximum Program Memory Size (in K)..:  70 �

�                                                                       �

� Script Buffer Size:  100    Maximum Expanded Memory Size (in K):    0 �

�                                                                       �

� Text Pages: 1  Graphics Pages: 0  Initial Mode:  Interrupts: 00 to 2F �

�����������������������������������������������������������������������ĺ

� Window Position:                                                      �

�    Maximum Height: 25     Starting Height: 25     Starting Row...:  1 �

�    Maximum Width.: 80     Starting Width.: 80     Starting Column:  1 �

�����������������������������������������������������������������������ĺ

�                            Shared Program                             �

� Pathname..:                                                           �

� Data......:                                                           �

�����������������������������������������������������������������������ĺ

� Close on exit (Y,N,blank)..: [Y]  Uses its own colors...........: [Y] �

� Allow Close Window command.: [N]  Runs in background (Y,N,blank): [Y] �

� Uses math coprocessor......: [N]  Keyboard conflict (0-4).......: [0] �

� Share CPU when foreground..: [Y]  Share EGA when foreground/....: [Y] �

� Can be swapped out (Y,N, ).: [N]  Protection level (0-3)........: [0] �

�                                                                       �

�  Press F1 for standard options           Press <�� when you are DONE  �

�����������������������������������������������������������������������ͼ

D3.   When running under DESQview, the "shell to DOS" option during a file

transfer is disabled, since DESQview does not permit interrupt handlers

to call DOS and BIOS functions (necessary for reading and writing files).

However, this is not a serious limitation under DESQview, since DESQview

will allow you to run multiple simultaneous programs.

D4.   The normal "shell to DOS" option may be used under DESQview, but in this

case more than 70K of memory will need to be allocated - with 4dos, it

has been reported that at least 150K is required!

APPENDIX E - Acknowledgments

E1.   I would like to thank all the users of TeleReplica who have helped me to

improve the package by suggesting new features and reporting

difficulties. The following individuals in particular have had a

significant impact on the current version:

Mr Lindsay Smith, Monash University College, Gippsland, Australia

Prof. Timo Salmi, University of Vaasa, Finland