INPRT.DOC File

(c)  Hewlett-Packard Company, 1992.

Overview

--------

The INPRT application reads infrared (IR) printer output from different

Hewlett-Packard calculators into the HP 48 stack.  INPRT assumes that the

incoming bytes would print with the Roman 8 character set on an HP 82240

printer, and it remaps these bytes to the HP 48 character set (ISO 8859

Latin 1).

Installation

------------

Transfer the INPRT file from your computer to the HP 48.  (Use binary if

you're transferring INPRT from the HP 48.)  The |INPRT| menu label will

show up in your HP 48 VAR menu.  (If your calculator has a double-spaced

print mode, set it to single-spaced print.)

Procedure

---------

These are the steps for transferring IR printer output to the HP 48:

1.  Line up the calculators.  (See "Alignment Instructions" below.)

2.  Press |INPRT| on the HP 48.  (INPRT terminates in 10 seconds if no IR

output is received.)

3.  Execute the print command on the other calculator.

4.  Repeat steps 2 through 4 as needed to transfer additional output.

Results

-------

When the transfer is complete, the HP 48 returns the received data to the

stack in the form of a string to level 2 and a flag to level 1.  If the

flag is 0, uncorrectable transfer errors were detected; if the flag is 1,

there still may be missing bytes. The 1 means that each byte that was

detected was received correctly.  You should still check the received data

since there may be a linefeed in strings, names, expressions, etc. more

than 24 characters long. If the flag is 0, each byte that was not

correctable is a gray blot (character number 127) in the level 2 string.

Alignment Instructions

----------------------

The calculators should be no more than 0.5 inches (1.3 cm) apart for the

transmission.  Also, other alignment issues apply to several calculators:

o For the HP 41:  The IR printer module should be in one of the top ports,

and the shiny "IR cone" in the center of the module should be aligned

between the E and T of HEWLETT on the HP 48's Hewlett-Packard logo.

o For the HP 27S, HP 17B, and HP 42S:  The calculator must be raised 0.3

inches (0.75 cm). The center of the LED (which is located at the top end

of the calculator in line between the E and W of HEWLETT) should be

aligned between the E and T of HEWLETT on the HP 48's Hewlett-Packard

logo.

o For the HP 28, HP 18C, and HP 19B:  Just line up the calculator's LED

between the E and T of HEWLETT on the HP 48's Hewlett-Packard logo.

---------------------------------------------------------------------------

V 1.0 04/26/92 04:27 PM