PCLPRINT.DOC File

(c)  Hewlett-Packard Company, 1992.

Overview

--------

The PCLPRINT library enables you to print a graphics object on a PCL

printer, such as the HP ThinkJet, HP DeskJet, and HP LaserJet printers.

When attached to the HOME directory, the library is named HPPRT.  (An

|HPPRT| menu label is visible in the LIBRARY menu).  Two commands in the

library, HPON and MAG, add an extra sublist of parameters as the fifth

element in the HP 48 reserved variable PRTPAR.  These parameters store

information needed by your HP 48 to print graphics objects to the PCL

printer.  In order of occurrence in the sublist, the parameters are as

follows:

o The real number 0, which indicates PCL printer type.

o A real number, which represents the magnification.  This is controlled

by the MAG command and has a default setting of 2, which tells the

library to expand each pixel in the graphics object to a 2x2 square on

the printer.

o A string containing the escape sequence sent before a graphics object is

printed.  The default begin sequence is "<esc>*r0A", which tells the

printer to start printing raster graphics at the left margin.  The only

way to change this is to edit PRTPAR.

o A string containing the escape sequence sent after a graphics object is

printed.  The default end sequence is "<esc>*rB<cr><lf>", which tells

the printer to end raster graphics and execute a carriage-return and

linefeed.  The only way to change this is to edit PRTPAR.

Installation

------------

1.  Transfer the PCLPRINT.LIB file from the computer to your calculator.

The menu label |PCLPR| will appear in your HP 48 VAR menu.

2.  Attach the library to your HOME directory:

A.  Press the VAR menu label |PCLPR| to recall the library to the

stack.

B.  Enter the port number where you want the library to reside--0 for

the HP 48S and 0, 1, or 2 for the HP 48SX.

C.  Execute STO.

D.  Turn the HP 48 off and then on again.  |HPPRT| attaches itself and

appears as a library in the LIBRARY menu.

E.  Purge the original copy of PCLPR in the VAR menu.

Operation

---------

Here are the steps to follow to print an HP 48 graphics object on a PCL

printer:

1.  Using the serial cable 25-pin adapter and a male-male gender converter

(not included), connect the HP 48 to the PCL printer.

2.  Set the HP 48 baud (using the I/O SETUP menu) and the printer baud to

match each other. Set the printer to use XON/XOFF handshaking.

3.  Execute HPON (press [LS]-[LIBRARY] |HPPRT| |HPON|).

4.  Optionally execute MAG and DPI. (See descriptions of these commands

below.)

5.  Execute one of the HP 48 print commands for printing graphics objects--

PRLCD, PR1, or PRVAR. ([ON]-[PRINT] does not work and should be

avoided.)

6.  If you wish to switch back to printing on the HP 82240 Infrared

Printer, execute HPOFF (press [LS]-[LIBRARY] |HPPRT| |HPOFF|).

Command Summary

---------------

These commands are contained in the PCL library:

HPON      Enables special graphics printing and adds the previously

explained sublist to PRTPAR.  Also sets flag -34 and clears flag

-33 so that printing is directed to the wired serial port, and

modifies IOPAR to enable transmit pacing (XON/XOFF) and to set

parity to "none."

HPOFF     Disables special graphics printing and clears flag -34 (so that

printing will be directed to an HP 82240B printer).  HPOFF does

not change the sublist in PRTPAR--the magnification setting and

other changes are preserved for the next time you execute HPON.

MAG       Takes a real number from level 1 and puts it in the magnification

position in the sublist in PRTPAR.  A magnification of N causes

one pixel in the graphics object to be expanded to an NxN square

on the printer.

(MAG 0 effectively disables printing.)

DPI       Takes a real number from level one and sends an escape sequence

to set the printer to that number of dots per inch.  This command

only works for the HP DeskJet and HP LaserJet printers.

Special Note For The HP LaserJet Printer

----------------------------------------

Since the HP LaserJet printer won't print until it gets a whole page of

data, you need to send a formfeed character (character number 12) to print

a graphics object of less than one page.  You can do this by putting a

string containing a formfeed character in level 1 and executing PR1.  (You

can also edit the sublist in PRTPAR so that the end-sequence string

contains a formfeed.  This would cause each graphics object to be printed

on its own page.)

---------------------------------------------------------------------------

V 1.0 04/26/92 04:29 PM