*  L S i m  *

The Loudspeaker Simulator

Version 2.1r2 - 31 March 1992

User Manual

Please read the LICENSE.ME file first!

About LSim

LSim is a revolutionary software for the speaker designer.  With LSim and a

PC, you can simulate the amplitude and phase response of a speaker model in

anechoic and room conditions before building a model.  By speaker model, I

mean any drivers mounted on a baffle of known dimensions.  The time delay

made by the displacement between the drivers and the diffraction loss made

by the finite baffle dimensions are taken into account by LSim and the

frequency and phase response graph of the speaker model is generated.

Please note that the graph is not the response of a single driver but the

complete model.

LSim runs on a so called IBM PC or compatible PC (including the PS/1 and

PS/2 series) running the MS/PC-DOS 3.1 or later version.

LSim is fast!  Even on the original IBM PC (a 4.77 MHz 8088), its response

time is respectable; but try LSim on an 80386 machine for a real treat (or

one with a faster CPU).  A PC with a 25 MHz 80486 solves and displays the

graph of a three way model before you realize it!

The LSim Concept

The algorithms developed for LSim is based on a driver's inherent band-pass

characteristics.  LSim models a driver as a band-pass filter.  It then

cascades this driver to its crossover filter and adds the refraction effect

of the baffle which the driver is mounted.  Lastly, a model is built with

parallel cascaded cross-over, driver and baffle groups, as it is the case

in multi-way speakers.  But, there is nothing to stop you to model a one-

way speaker!  This filter theory approach is what gives LSim its speed.  If

I had used the network theory approach, LSim would have been hardly usable

on a 80286 and useless on a 8088.

Building a speaker model starts with defining the driver (s) of your

choice.  You must first define the low-pass, high-pass frequencies and

their Q values and the high-pass slope of a driver by interpreting the

manufacturer's data or your own measurements.  You then define the baffle

you wish to mount this driver on.

Once you reached a match; you can tweak your guessed values until you get a

good match between the LSim generated and measured graphs.  Once a good

match is reached, you can save this data to a driver data file for future

use.

Although this may seem like a long and tedious process, in reality it is

not.  But, please remember: the results will only be as accurate as the

care you take in defining your driver (s).

Running LSim

For PCs with IBM compatible display cards (i.e. MDA, CGA, EGA, VGA or

MCGA), start LSim by entering LSIM at the DOS prompt.  For PCs with

Hercules or compatible display card, you should first install the Hercules

video support routines by running MSHERC.COM (a TSR program) BEFORE running

LSIM.EXE.  LSim senses the display card of your PC and configures itself at

start up.  You can also force LSim to configure manually with the below

command line options.

LSIM /C   Colour CGA

LSIM /E   Colour EGA or VGA

LSIM /H   Hercules (you must first run MSHERC.COM)

LSIM /W   Black on white display (good for laptops)

Note: you cannot display or print graphs with the MDA card.  But, you can

load, save and modify a model or driver data file.

LSim displays its model screen in colour and its graphic output screen in

monochrome.  For monochrome monitors and the LCD displays in the laptop

PCs, this color scheme may make the model display unreadable.  Then, you

may start LSim with the /W option.  This switch makes a black & white

display instead of the default colour one.

Because of the large variety of VGA compatibles, you may have difficulty in

using LSim with a VGA card.  Try removing or inserting your card's video

BIOS.SYS into your CONFIG.SYS file.  Difficulties are usually in cursor

appearances.

You can also force LSim to load a model file upon start by adding the model

file name in the command line as shown in below examples.

LSIM TEST.LSM

LSIM D:\MODELS\TEST.LSM /W

Please note that the file name must precede the option letter and the

option letter must start with a stroke (/).  As in the second examples, you

may include the path to the file name.

LSim supports Microsoft or PC Mouse and its compatibles.  I highly

recommend you to use a mouse.  Please refer to your mouse user manual for

installing the mouse and its driver.

Using LSim

Once you have called up LSim, the pull-down menu options can be seen by

pressing the first letter of the menu at the top of the screen.  Some menu

options have keyboard equivalents (usually a function key).  If you have a

mouse you can activate the menus and select options with the left mouse

key.

You must first select a new model to work for.  Pull down the Model menu

(press M) and select the New option.  A line of a graphic showing one way

(as in three way) of the speaker model is displayed.  Each box shows a part

of the speaker model.  From left to right these are: Source, Connections

(On/Off and Normal/Inverted connection),  Cross over (X'Over), Driver,

Baffle and Output.  You will now notice that the driver box is highlighted

(i.e. is made of thicker lines).  This means it is selected.  You can

select any box in the model with the arrow (cursor) keys.  Select the Show

Data...  option from the Driver menu or use the keyboard equivalent for it

(the F3 short-cut key).  Or you can select the box with the left mouse key

and show its data with the right mouse key.  You can now enter your driver

data and then using similar methods, the baffle data (note the following

paragraph) that you intend to use in your model.  You then choose a cross-

over type (or leave as no cross-over) for each driver from the X'Over menu.

There are full compliments of high-pass; low-pass and band-pass types of

1st to 4th order and a special notch filter.  After choosing your cross-

over you can enter the element values, decide whether to invert the driver

or not and lastly display the response graph of your model.  The

technically well-informed designer may also define the crossover transfer

function directly; either as factored or as polynomial.  This way any

filter can be defined, including active filters.

When entering displacement values in the baffle section, assume a 0 cm

depth displacement for the tweeter and measure the displacement of the

other drivers relative to the tweeter and in millimeters.  Use a positive

displacement for a driver which is in front of the tweeter (i.e. its

diaphragm is in front of the tweeter diaphragm) and a negative displacement

for a driver which is behind the tweeter.  Always try to use positive

values as usually this is the way to a flat response.  When working out the

values, middle of the cone should be considered as the centre of driver

(for this is about where the acoustic centre lies).

This ends the data entry.  Now select (highlight) the output block (the

right-most rectangle) and press the F8 key (or point the output block and

click the right mouse key).  Right in front of your eyes the response graph

of your proposed speaker model starts to form.

You have not yet built anything and it have not cost you a penny other than

the donation that you have made (have you honored my request?) and the few

minutes you have spent so far.  If you do not like what you see on the

screen you are free to tweak the cross-over values, apply or remove

polarity inversion (with the F6 key and if you use a mouse by clicking with

the left mouse key) until you reach the target magnitude and/or phase

response in either anechoic or room condition.  After reaching a good

compromise (well, it is always a compromise isn't it?) you can save your

model or driver(s) for future referral. (Data file format is pure ASCII

text and is in public domain. Please refer to the F-FORMAT.DOC file for

detailed explanation of their formats.)

To have the graph of the original driver alone, select the driver block

instead of the output block.  If you wish to see the effect of the cross-

over on that driver, switch off other drivers and plot the graph in the

usual way from the output block.  Unless you have not cleared the graph

(with the F10 key) you can superimpose unlimited number of graphs.  You can

change the amplitude axis to any standard value.  But, please note that

axis value change removes the previous graphs but the last one.  You can

also change the amplitude axis to read either relative (+-dB) or absolute

(dB SPL) with the F4 key (toggle action).  By using the left/right arrow

keys you can move the frequency cursor and read the corresponding amplitude

and phase values.  If you use a mouse you can move the frequency cursor by

clicking to the left or right of the frequency cursor with your left mouse

key.  The 0 dB level can be adjusted with the up/down arrow keys or set to

any value with the 0 key.  If you use a mouse you ca n click to the above

or below of the 0 dB line with the right mouse key and increase or decrease

the 0 dB value.

The response calculations can be done in 1, 1/3 or 1/6 octave increments.

If you have a slow PC, selecting the 1 octave response speeds the

calculation as there are less points to calculate.  You can also speed the

calculation by selecting a narrower frequency range from the Graph menu.

You can get quality prints of your models, drivers and graphs with LSim.

What you need is a printer in IBM mode.  Nearly all printers have an IBM

mode, consult your printer manual and find out the DIP switch settings.  It

is an easy job meant for the average PC user.  But, it is not as easy as

driving the Apollo Lunar Module!  If you are not content with the graphs of

LSim, you can export the graph data to a file and import it to a dedicated

graphing program or a spreadsheet for presentation quality output.  You may

also import the graph data to a computer based acoustic measurement system

and compare your design with the resulting prototype.

You can create (New), Save and rename (Save As...) data files of both your

models and drivers for future referral (you can enter the full path with

the file name while creating or renaming a data file).  Once a model data

file is created you no longer need the driver data files that you have used

in that model.  You can create a model data file without creating a driver

data file.  But, creating a driver data file makes your life easier later,

since you can later insert the driver data from its file to a future model.

This is by far easier than entering the data manually.

As everything in LSim is pull down menu driven, you do not have to memorize

any commands.  Also, all short-cut keys (i.e. F3, F8, etc.) are labeled

besides the menu options.  You do not even need this manual!

Follow these lines for creating the data files

1.  Create a new model by selecting New Model from the Model menu.

2.  Create a new driver by selecting New Driver from the Driver menu.

3.  Enter the driver parameter values deduced from the manufacturer's

graphs.  Be attentive to the baffle size used by the manufacturer.

This data is usually very well-hidden!

4.  Fine tune values to match LSim's and manufacturer's graphs.  Important!

This is the most (read only) critical step for defining data for LSim.

5.  Select Save Driver As... from the Driver menu, give a name and save

this data to a driver file.  Use the driver model number if possible,

this makes recalling easy (e.g. B110B.DRV, 2215.DRV, etc).

6.  Change the low frequency cut-off values of the drivers to their

corresponding in box values.  These new values can be calculated by the

usual Small-Thiele methods.

7.  Change the baffle data to correspond with your proposed speaker box

(usually you do not use the same baffle used for measuring).

8.  Change the sensitivity to correspond with your proposed speaker (you

may be using an attenuator).  Note that you should add the series

resistance of this attenuator to the driver impedance.

9.  Select Save As... from the Model menu, give a name and save this data

to a driver file.  But, this time to a Model file (if you have already

given a name to this file select Save instead of Save As...).

10. Add drivers as you wish by selecting Add line from the Model menu and

loop to item 2.

Examples

With the LSim diskette I included some driver data files.  Please load them

to LSim, generate their graphs and compare these graphs with the

manufacturer's published graphs.  Also, included are some model data that

have very good magnitude and phase responses.  Be attentive to the non-

orthodox polarity selections, cross-over types and values.  With LSim you

usually start with a textbook solution but end up with a design that looks

very unusual to a well-informed (!) speaker designer.  No wonder there

ain't many good sounding speakers around.

LSim Files

LSim is distributed as a single, self-extracting file named LSIMXX.EXE (XX

shows the version number).  This distribution file is created by the LHA

utility of Harayasu Yoshizaki (Yoshi) and it encloses the program, utility,

document and sample files.  This way the total size (and the needed time to

down/upload it) is reduced and the number of files to deal with are reduced

are reduced to one.  You can extract the LSim files packed within this

archive file by the simply copying the above file to your preferred

directory (C:\LSIM for example) and running it from there.

Below is a list of the files packed within the distribution file.

LICENSE.ME     Registration form and general information

LSIM.EXE       LSim run file

LSIM.DOC       LSim user manual text file

LSIM-FF.DOC    LSim data file format information

MSHERC.COM     Hercules video support routines (a TSR program)

XXXXXXXX.DRV   Various example driver data files

XXXXXXXX.MOD   Various example model data files

The extensions "DRV" and "MOD" in these files are for convenience only.

You may use any legal DOS name for either type of data file.  I suggest you

create two directories named as DRIVERS and MODELS and put all driver and

model data files to these named directories.  Unlike other DOS programs,

changing directories with LSim is extremely easy!

Some notes on LSim

I want to point your attention that as LSim is a simulator, it calculates

neither the cross-over element nor box values.  LSim will only show you the

expected response of your proposed model before you build it.  There are

excellent programs available for cross-over or box calculations.  I just

wanted to add another tool to the speaker designer�s tool-box.  My belief

is that, a designer must have all the necessary tools to work with and mix

and match them as she or he likes it.  Had I designed a complete speaker

design program it would be perfect for me but perfectly terrible for

another designer!

LSim is similar to Ralph Gonzales's LMP programs.  I have heard about LMP

after writing LSim One and since then became friends with him.  It seems

that we have both developed the algorithms at about the same time but

without each of us knowing the other.  LSim and LMP gives the same results,

but I think my LSim is easier to use then LMP and has better output.

Ralph's LMP is distributed by the Speaker Builder (SB) magazine and is

available in many incarnations, even one for the Mac.

(SB is published by Audio Amateur Publications, PO Box 576, Peterborough,

NH 03458-0576.  I strongly suggest you to subscribe to SB as it is THE

source for loudspeaker designers and DON'T be put off by the amateur in the

name, it is a serious journal.  They also publish Voice Coil, a

professional newsletter, edited by Vance Dickason.)

You can open LSim files with LMP and LMP can read LSim files.  But, please

note that LMP has three incarnations; LMP, LMP2, LMP3, their LMPG versions

(they differ in cross-over definition methods) and SD-LMP (a souped-up LMPG

by Sitting Duck Software).  LSim model file format is compatible ONLY with

LMP, LMPG and SD-LMP model file formats.

LSim is backward compatible with LSim One data files.  While loading an old

type model data file (called LSim 1.0 files) you will be notified.  You

than have the possibility to save your data in either the 1.0 or 2.0

format.

As a final note I want to point you out that, LSim is a vertical market

software which is not used by the millions.  During its use you may find

some quirks in it which are not intended.  Please write any such anomalies

(or suggestions) to me.  Although I hate bugs; I love suggestions! And

don't forget that, I am an amateur speaker designer myself.

A Zillion Thanks to:

*   Hasan Basri Buguner, the coder of LSim that does not know "no" for an

answer.  I would have never made LSim without him.

*   I. Ethem Cetintas of Silver Ltd., London, U.K.  Ethem was the

distributor of the previous and commercial versions of LSim.  He is my

best bug finder (the guinea-pig), best friend and best supporter in

this project.  Most of the user friendly futures of LSim are his ideas.

*   Dr. Richard Small for sending me his Ph.D. thesis, "Direct-Radiator

Electrodynamic Loudspeaker Systems," upon my request.  This was in

1978, the year the speaker designer in me was born!

*   My friend Siegfried Linkwitz who patiantly answers all the questions!

A Thousand Thanks to:

*   Interface for the QuickWindows library.  They sure must have used the

Macintosh GUI as their model (Mac is my preferred platform and I use

Macs daily, but unfortunately, I am not literate enough to write for

it).  QuickWindows is the shortest way to a decent BASIC application in

the DOS world.

*   Crescent for QuickPak and ProPak libraries.  My only critic is that

there is too much code in them!  But, the tricky routine that you had

been trying to write for the last weekend is definitely there

somewhere.

*   Microsoft for the QuickBasic and lately the Professional Development

System v7.  For the record: both I and Hasan know only BASIC!

For the lawyers

All trademarks and copyrights are acknowledged. Don't pin me!

Revision History

Some notes on the version number scheme: Version number is formed as X.YaZ.

X is the major and Y is the minor revision number where Z is maintanence

release number. Letter is 'b' for BETA and 'r' for release versions.

LSim v1.0b2   First version distributed to beta sites.  This version had no

printing facility.  LSim Demo was made from this version.

The only difference was that the demo could not save a data

file.  Created with QuickBasic v2.0.

LSim v1.0b3   First version distributed commercially.  It was labeled as

LSim One.  Printing facilities added.  Created with

QuickBasic v4.0.

LSim v1.0b4   Room response option is added.  Minor changes to source code

to fit QuickBasic v4.5.  This is the last commercial version.

LSim v2.0b8   This was the beta version of the first major overhaul.  It

was distributed only in Turkey.  Pull down menus added for

the output window.  Graph display and printing is improved.

Model data file format was changed to be compatible with LMP.

Created with Professional Development System v7.

LSim v2.0b15  The first ShareWare version that was distributed on a global

scale.  All registered LSim v1.0b4 or earlier users were sent

a free of charge copy of this version.  Because of a bug, the

Model data file format is changed and the format of v2.0b8 is

no more supported!

LSim v2.0r2   The first release version which is not regarded as build! The

$50 ShareWare fee request is lifted.  Instead, the request

for donation to an audio education fund is implemented.  /M

switch is removed.  /W switch now handles EGA and VGA cards

including the CGA card and it makes a black & white display

instead of the default colour one, useful for LCD displays.

VGA is now supported on VGA mode, earlier, VGA screens were

supported on EGA mode only.  A bug making system halt while

loading or saving a driver when the cursor was on the

"Output" block, is removed.

LSim v2.1r2   Export graph data option added.  LSIM.EXE file is compacted

with the LZEXE utility by F. Bellard.  Also, distribution

file is now a self extracting EXE file created by the LHA

utility of Yoshi, instead of a ZIP file which required

PKWare's PKUNZIP utility. FPU (80X87 Arithmetic co-processor)

support is lifted.

I know my bugs!

Below is a list of the documented anomalies (bugs?).

*   LSim version 2.X does not work with an on-board FPU (80X87).

*   LSim cannot display the footer window on a NCR PC4i PC.

*   Command line model file loading (ie. LSIM DEMO.MOD) does not work if

the model file resides in the root directory. It works in any BUT the

root directory! You may open such a file with-in LSim with the Model

Open... menu.

*   If you cancel a calculation (press <Escape>) the first time you

calculate a model, you can not move the frequency cursor with the

cursor keys. After a full calculation this problem is cleared.

Please direct all enquiries concerning LSim to me, at the below addresses.

I can react fastest via CompuServe.

Riza Nur Pacalioglu

27A Eskibuyukdere, 4. Levent

80650 Istanbul, Turkey

Fax:   90-1-268 6901

Tel:   90-1-268 6900 (office hours, +2 hours GMT)

Telex: 39084 cctv tr

Email: CompuServe 70242,145

AppleLink  BES.TURK

Internet   70242.145@compuserve.com

bes.turk@applelink.apple.com

Happy loudspeakering...and do support audio education!

Appendix

Please read the LSIM-FF.DOC file for information on LSim data file

formats.

<end>