Multimedia Windows Drivers for the Pro AudioSpectrum

Version 1.30

copyright 1991,1992, MEDIA VISION

(415) 770-8600

This release (2/03/92) includes several bug fixes to the drivers

on the Media Vision Multimedia CD.

Media Vision will continue to improve and enhance our drivers in

order to continue to be the best source of Multimedia Windows

products for your PC.

Don't forget to check the NEWS section.

Here is a description of the files on this disk:

OEMSETUP.INF    This file allows you to add drivers to Windows using

the drivers applet in the control panel.  If you

use the drivers applet, you will not have to

manually edit your system.ini file.

MIX.EXE         This program allows you to modify those controls

on the mixer that you'll want to change most often.

Its main difference from promix is that it doesn't

use as much screen area.

Put this program in your \windows directory.

There are no changes to this version.

PROMIX.EXE      ProMix can be launced from the WIN.INI load line and it

will come up minimized.

Put this program in your \windows directory.

Here's a nifty new feature:  Now when running

Windows, you can use your familiar CTRL-ALT-U,

CTRL-ALT-D, and CTRL-ALT-M sequences.

The system menu contains a new feature: a pair of

DB meters.  The DB meters are active only during record

and playback of waveform data.

MMMIXER.DLL     Put this DLL in your \WINDOWS\SYSTEM directory.

MMMIXER.DLL is like MMSYSTEM.DLL in a couple of ways.

First, it provides a driver-independent layer for all

mixer API's.  Second, it loads any mixer device drivers

in the system.

If you build an application that makes use of our

mixer API's, this DLL handles the call.  If there are

no mixer device drivers in the system, the mixer

API's will simply return an error.  This means you

can utilize the Pro AudioSpectrum's mixer capabilities

without requiring the user to have our hardware.

No changes this release.

The mixer DLL has two BIG selling points:

1) Interrupt-driven timed volume fades

If you're a programmer, investigate

the mixSetState function as defined

in the MIXSPEC directory.

2) Startup volume initialization

The first time the MIXER driver is loaded

it reads WIN.INI for volume setting and

patch information.  When Windows shuts

down, the current settings are saved.

Here's a sample WIN.INI entry:

/////////////////////////////////////////////////////

[mvmixer.drv]

LineIn1=SYNTH     MUS]  PLAY     L=100% R=100%  @=0000

LineIn2=MIXER     MIX]  PLAY     L= 72% R= 72%  @=0000

LineIn3=AUX       AUX|  PLAY     L=100% R=100%  @=0000

LineIn4=CD        CDA|  PLAY     L=  0% R=  0%  @=0000

LineIn5=MIC       MIC|  PLAY     L=  0% R=  0%  @=0000

LineIn6=DIGITAL   WAV]  RECORD   L=  0% R=  0%  @=0000

LineIn7=SPKR      SPK]  PLAY     L=100% R=100%  @=0000

LineOut1=MASTER   AMP|  PLAY     L= 79% R= 79%  @=0000

StereoEnhance=ON

Loudness=ON

Bass=50

Treble=78

SaveSettingsOnWindowsExit=YES

LINE FIELD:

There are 7 input Lines and 1 output line.

PATCH NAME:

The field immediately after the '=' is the patch name.

These names (ie. SYNTH) appear in promix.exe at the

bottom of each slider pair.

DEVICE TYPE

The next field is the device type field.  The device

type determines the type of icon displayed in MIX.EXE

Each device type has a standard 3-letter mnemonic name

that can be obtained by calling mixGetDeviceName.  The

current list of types is:

N/C   No Connection             SPK        PC speaker

CDA   Compact Disk Player       AAT        Analog Audio Tape

AUX   Auxillary                 DAT        Digital Audio Tape

MIC   Microphone                TEL        Telephone

VTR   Video Tape Recorder       MIX        Mixer

VDP   Video Disc Player         EQU        Equalizer

MUS   Musical Instrument        AMP        Amplifier

WAV   Digital Waveform Player   LPA        Phonograph Audio

RAD   Radio                     TVA        Television Audio

UNK   Unknown Device Type

The fourth character indicates whether the user is

responsible for making the physical device connection.

Think of the character as the mounting bracket of an audio

card.  A hole, the '|' character, represents a mini

phone jack.  If the fourth character is ']', the connection

is made by the driver.  The '|' character indicates a jack

or connector must be hooked up before audio can be

heard.  User-friendly applications may use this information

to remind the user to connect a particular device.

OUTPUT CONNECTION

The next field corresponds to the internal mixer

connection info.  The word PLAY indicates that input

is routed to the PLAY output (LineOut1).  RECORD

means the input is routed to the ADC circuitry.

VOLUMES

If it isn't obvious, the L= and R= fields specify the

volumes of the left and right channels at start-up.

ASSOCIATION

This field is used to save information that specifies

a relationship between a mixer input and some other

device in the system.  For example, if there are two

CD's in the system and one is connected to LineIn4

and the other is connected to LineIn3, the

dwAssociation can be used to determine which MCI CD

device is on which input.

MISC

The other lines are self explanatory.  In the future

we will let users change the settings through control

panel's driver setup.

CHANGING WIN.INI

As an example, if you connect your Walkman to LineIn3

(AUX), you can replace the word AUX with RADIO.

Then, change the device type field to RAD|

/////////////////////////////////////////////////////

This DLL was developed to provide MultiMedia developers

better control over audio mixing functionality than

is afforded by the current volume API's.  Every

attempt was made in the API design to avoid its being

biased toward any specific mixer hardware.

Furthermore, the driver design is modeled after Microsoft's

Multimedia Windows driver design in order to facilitate

its acceptance.

We are offering this DLL to Microsoft and the MPC

community as a proposed standard for Multimedia Windows

volume control.  There is no license required to use

MMMIXER.DLL and you may freely distribute it with your

products.

Hardware manufacturers are encouraged to develop there own

drivers to work with MMMIXER.DLL.  Media Vision does

not yet offer a DDK for mixer drivers.  Parties

interested in the source to mvmixer.drv should

contact Media Vision's Jim Gifford at (415) 770-8600.

See the MIXSPEC directory for detailed information on the

Multimedia Mixer API.

MVMIXER.DRV     Put this file in your \WINDOWS\SYSTEM directory.

This is Media Vision's Mixer driver for the Pro

AudioSpectrum.  In the [drivers] section of your system.ini

file, you must add the line:

mixer=mvmixer.drv

Changes this release: none

There is a known bug with this driver.  Do not attempt

to unload and reload it from the drivers applet.

Your system will likely crash during reboot.

MCIMIXER.DRV    Put this file in your \WINDOWS\SYSTEM directory.

This driver provides a set of extended MCI commands

for accessing mixer capabiities.

Under the [mci] section of your system.ini file, you

must add the line:

mixer=mcimixer.drv

Developers may ship this driver, license free, with

their products.

This version has some minor bug fixes.

See the MIXSPEC directory for detailed information on the

Multimedia Mixer MCI command set (MCIMIXER.DOC).

MVAUX.DRV       OBSOLETE. This file has been replaced and superceded

by MVPROAUD.drv.  The old mvwave.drv, mvaux.drv and

mvmidi.drv have been combined to solve a problem

caused by the fact that they shared the same interrupt.

This change will result in a more efficient handling

of interrupts.

When our aux driver is the only one in the drivers

section, a call to auxGetNumDevs returns a value

indicating 4 aux devices are in the system:

Device 0 is internal CD-ROM volume

Device 1 is auxillary (line in) volume

Device 2 is microphone volume

Device 3 is master volume

While we provide a Multimedia Mixer library, we still

support the existing volume control calls, auxGetVolume

and auxSetVolume, when this driver is installed.

In the [drivers] section of your system.ini file, you

must add the line:

aux=mvaux.drv

MVPROAUD.DRV	Put this file in your \WINDOWS\SYSTEM directory.

This is the Pro AudioSpectrum Multimedia Windows Wave

Driver.  This file supercedes and replaces mvwave.drv

mvaux.drv and mvmidi.drv.  It will now look for

the [mvproaud.drv] entry in system.ini for configuration

information.  If it fails to find that entry, it will

check to see if MVSOUND.SYS is loaded and use it's

settings.  Failing that, it will look for the

outmoded [mvwave.drv] and [mvmidi.drv] entries.

As a last resort, the driver will default to IRQ=7

and DMA=1.

MVWAVE.DRV      OBSOLETE.  This file has been replaced and superceded

by MVPROAUD.drv.  The old mvwave.drv, mvaux.drv and

mvmidi.drv have been combined to solve a problem

caused by the fact that they shared the same interrupt.

This change will result in a more efficient handling

of interrupts.

Using the Control Panel's drivers applet, remove

the wave driver, MIDI driver and Auxillary volume

driver.  You may wish to make archive copies of

mvwave.drv, mvmidi.drv and mvaux.drv in the event

you have some problem with MVPROAUD.DRV.

This release incorporates several bug fixes.

One major bug occurred when using sound record to

record.  In the past you would have encountered

a noisy tone imposed upon the recording.  This

feedback was caused by having the recorded signal

routed through two channels of the mixer.

You will notice that the volume for the WAVE input

to the mixer is temporarily set to 0 during

recordings and restored afterward.  This will make

a big difference in the quality of your recordings.

Note:  If you are using a Beta version of Win 3.1 and

the retail MDK, do not use the Win 3.1 version of

MCIWAVE.DRV.  This will cause WaveEdit to fail.

Use the version of MCIWAVE.drv that came with the

Multimedia Extensions.

We now also added support two more wave out API's:

waveOutGetPlaybackRate  and

waveOutSetPlaybackRate

If you're a developer you can use these API's to

modify the playback speed, making voices sound

like chipmunks or giants.

In the [drivers] section of system.ini you must add:

wave=mvwave.drv

Also in system.ini you must indicate the current

jumper settings of your card.  For example:

[mvwave.drv]

dma=1

irq=7

Note: If you have used our drivers before, remove all

references to mvsound.drv in system.ini.

MVFM.DRV       	We have redesigned the FM driver and offer this to

you as a STEREO driver for the Pro AudioSpectrum

onboard synthesizers.

In the [drivers] section of system.ini, add the line

midi=mvfm.drv

ADLIB.DRV       OBSOLETE. MVFM.drv replaces this driver. Remove any

reference to ADLIB.DRV from your system.ini file.

MVMIDI.DRV      OBSOLETE.  The MIDI driver has been merged with

MVWAVE.DRV to create MVPROAUD.DRV.   Remove any

reference to MVMIDI.DRV from your system.ini file.

VADMAD.386      Put this file in the \WINDOWS\SYSTEM directory.

VADMAD stands for "virtual audio DMA device".  If you

run Windows in 386 enhanced mode, you must add

device=vadmad.386

to the [386Enh] section of system.ini.  This device

driver allows our wave driver to use auto-initialized

DMA mode.  When this file is not present, you will

hear a PC-speaker beep as Windows boots and you will

not be able to play any digital audio sounds.

If you are using Windows 3.1.55 or later, you do not

need this file.

MUSICBOX.INI	Music Box has the capability of remembering the

names of tracks on a CD.  This file should be

appended to your \windows\musicbox.ini file.  It

defines the tracks for Media Vision's Multimedia

Windows CD.

[SCSI]          The latest version of TSLCDR.SYS is here.

By the way, Trantor Systems at (510) 770-1400 can

provide you with a driver for your SCSI hard disk.

Device=c:\tscsi\tslcdr.sys /d:TSLCD /r /s:31 /P:3

The '/R' parameter causes a SCSI bus reset when the

device driver loads.

Experiment with adding the /s:31 parameter, it can

speed up your SCSI performance.

The /P command enables seek commands and provides

some additional buffering.

[CDSPEED]       Some of you have CDSPEED, a program that tests

the transfer and burst rates of a CD-ROM interface.

In this directory you will find our preliminary test

results for the Pro AudioSpectrum SCSI interface.

We are working on further optimizations to increase

our SCSI burst rate.

[MIXSPEC]       Contains up-to-date documentation on Media Vision's

proposal for a standard MPC mixer driver API.

The mixGetPatchCaps and mixGetLineCaps functions have

been changed to mixGetPatchInfo and mixGetLineInfo to

better reflect their actual functionality.

[DEVEL]         If you plan to write code that uses the MultiMedia

Mixer library routines, the files in this directory

will get you started.  MIX.ZIP contains the source

to the Media Vision mixer program.

_____________________________________________________________________

SAMPLE SYSTEM.INI FILE DRIVERS SECTION

[drivers]

timer=timer.drv

Wave=mvproaud.drv

Aux=mvproaud.drv

MIDI=mvfm.drv

MIDI1=mvproaud.drv

mixer=mvmixer.drv

[mci]

CDAudio=mcicda.drv

WaveAudio=mciwave.drv 4

Sequencer=mciseq.drv

animation=mcimmp.drv

mixer=mcimixer.drv

_____________________________________________________________________

MEDIA VISION NEWS:

PRO AUDIOSPECTRUM SCSI:

Media Vision had discovered that with slight modification, your

Pro AudioSpectrum SCSI interface's data transfer rate can be

increased up to 33 percent.  It requires the change of one of

the chips on the PAS, so if you want the enhancement, you'll have

to send in your board.  If you don't need faster SCSI right away,

it would be best to wait:  Media Vision will be offering an

upgrade to our next product, and it will have MUCH faster SCSI.

Don't call to ask about the next product, though.  It's still

a secret.

THUNDERBOARD FIX:

The first week of ThunderBoards that shipped had a problem when

digital audio was being played concurrently with synth sounds.

The result was a lot of noise.  If you have one of these bad cards,

contact Media Vision for a FREE replacement.

Installation Notes

------------------

The sub-mini jacks at the back of the board are:

Top:    External Input

Middle: Amplified Output

Bottom: Microphone input (MONO)

The four pin header connector is for the PC-speaker input.

The five pin header connector is for the CD-Audio internal input.

If you have bugs to report, technical questions or feature requests

regarding this software, please contact Media Vision Technical Support

at phone (510) 770-9905 and FAX (510) 770-9592.