Slickware is Copy(c)right 1989,1992 by Gary M. Raymond, New Orleans, La.

Slick AT Modem Driver Program V1.2 by Gary M. Raymond

Simple <Software> Company

HOME of SlickWare

Gary M. Raymond

P.O.Box 8184

New Orleans, La.  70182

(504) 288-6550

Compuserve 70613,3165

====================================================================

Files Provided:

AT.COM        Slick DOS Command Modem Driver

AT.DOC        This document.

Why AT?  Occasionally the need arrises to send your modem a command

via a batch file or the DOS command prompt. Having the need myself,

I tried several similar utilities already on the shareware market and

discovered several things. Either they were too involved, requiring

the poking of even more information into my already crowded DOS

environment (in order to configure the utility), or required the

selection of one out of many separately pre configured files to

use on my particular port. Finally, most are written in high level

languages that produce code that is simple inelegant and requires

too many CPU cycles to do a job that can be done in less.

If you type AT (alone) at the DOS prompt, you will see the following

help screen.

AT Modem DOS Command Utility Ver 1.1

The Correct Syntax is AT #+STRING

# = 1,2,3 or 4  (COM1 thru COM4)

Examples:

AT 1DT288-6550 = Tone Dial (COM1)

AT 2M2 = Turn ON Speaker   (COM2)

AT 3Z  = Reset Defaults    (COM3)

AT 4H1 = Phone OFF Hook    (COM4)

Right off you will notice that this one utility will run any one of

the four standard IBM port addresses by simply including the port

number as the leading numeral on the command string. These default

ports are COM1 = 3F8, COM2 = 2F8, COM3 = 3E8, COM4 = 2E8. This

utility will even overcome some of the limitations of DOS when it

comes to operating ports 3 or 4.

If you attempt to use a port number out of range, AT simply will not

load and inform you of the reason. If you choose a port that is not

active, AT will also attempt to inform you of that as well.

To make AT as useful as possible over a wide range of CPU and modem

configurations (that all have different UART responce times) it has

been coded with fixed timing constraints based on CPU clock ticks.

For an additional five dollar fee I will customize this utility

for any non standard serial port addresses you might require. Leave

E-Mail here or on Compuserve.

Meanwhile, for reference, I have listed below the majority of the

Hayes string switches. Please note that every modem is somewhat a

different animal and may not utilize all these switches.

AT        ATTENTION COMMAND

A/        REPEAT COMMAND

A         ANSWER MODE COMMAND

B/B0      SELECT CCITT V.22 IN 1200 BPS

B1        SELECT BELL 212A

Dn        DIAL COMMAND (n = DIAL STRING, 0-9, *, #, !, W, R, AND,

E         DISABLE ECHO

E1        ENABLE ECHO

H         ON HOOK (HANG UP)

H1        OFF HOOK

I/I0      IDENTIFY

I1        DISPLAY 000

I2        DISPLAY OK

I3        DISPLAY MESSAGE PROGRAM ROM

L/L0      LOW VOLUME

L1        LOW VOLUME

L2        MEDIUM VOLUME

L3        MAXIMUM SPEAKER VOLUME

M/M0      DISABLE SPEAKER

M1        SPEAKER ON UNTIL CARRIER

M2        SPEAKER ALWAYS ON

M3        DISABLE SPEAKER WHEN DIALING OR CARRIER

O         GO ON LINE

O1        RETRAIN (2400 BPS MODE ONLY)

Q/Q0      ENABLE RESULT CODES

Q1        DISABLE RESULT CODES

Sn=X      S REGISTER WRITE (n = VALID S REG, X = ALLOWED VALUE SEE

&F COMMAND)

Sn?       READ SPECIFIED S REGISTER (n = ANY VALID S REGISTER)

V0        SELECT DIGIT RESULT CODE

V1        SELECT WORD RESULT CODE

X0        BLIND DIAL, DISPLAY "CONNECT"

X1        BLIND DIAL, DISPLAY "CONNECT 1200/2400"

X2        DETECT DIAL TONE, DISPLAY "CONNECT 1200/2400"

X3        DETECT BUSY TONE, DISPLAY "CONNECT 1200/2400"

X4        DIAL AND BUSY TONE DETECT, DISPLAY "CONNECT 1200/2400"

Y0        DISABLE LONG SPACE DISCONNECT

Y1        ENABLE LONG SPACE DISCONNECT

Z         SOFTWARE RESET

+++       ESCAPE CODE (GO TO COMMAND MODE IF ON LINE)

&C0       DCD ALWAYS ON

&C1       DCD FOLLOWS REMOTE CARRIER STATUS

&D0       IGNORE DTR

&D1       GO TO COMMAND WHEN DTR ON TO OFF

&D2       HANGUP AND GO TO COMMAND WHEN DTR ON TO OFF

&D3       POWER UP RESET WHEN DTR ON TO OFF

&F        FETCH FACTORY CONFIGURATION for S REGESTERS

&G0       NO GUARD TONE

&G1       550 HZ GUARD TONE

&G2       1800 HZ GUARD TONE

&J0       SELECT RJ-11/RJ-41S/RJ-45S

&J1       SELECT RJ-12/RJ-13

&M0       SELECT ASYNCHRONOUS MODE

&P0       MAKE BREAK RATIO FOR US (40/60)

&P1       MAKE BREAK RATIO FOR FOREIGN STANDARD (33/66)

&R0       CTS TRACKS RTS

&R1       CTS ON WHEN MODEM READY IN DATA MODE

&S0       DSR ALWAYS ON

&S1       DSR FOLLOWS RE232 SPECIFICATION

&T0       STOP TEST

&T1       LOCAL ALB TEST

&T3       LOCAL DLB

&T        ENABLE DLB REQUEST

&T5       DISABLE DLB REQUEST

&T6       REMOTE DLB

&T7       REMOTE DLB AND SELF TEST

&T8       LOCAL ALB AND SELF TEST

&V        VIEW ACTIVE PROFILE

====================================================================

V1.2  Corrects bug in COM4 (geezzzs you guys report fast!)

====================================================================

V1.1  First release.......................................

====================================================================

���������Ŀ

� MEMBER  �   Society of

�  ���������Ŀ Independent

�  �         �   Shareware

��Ĵ    �    �     Authors

�    �    �

�����������

This program is produced by a member of the Society of Independent

Shareware Authors (SISA). The Society wants to ensure that all

valid shareware principle actually work for you and SISA members.

The principle behind shareware distribution is simple; try before

you buy. Society members agree to license all shareware for a minimum

of 10 days, free of charge, to first time users as an evaluation period.

After 10 days, buyers are then obligated to license their copy with

the Society member. Society members are obligated to provide high

quality, useful shareware, but, are free to choose whatever marketing

methods suit their specific needs. SISA sanctioned marketing methods

include: demonstration versions; providing printed documentation

after purchase; registration keys that unlock additional features

not necessary to determine basic usefulness; and, providing bug

fixes free of charge. Any Shareware author may become a member of

SISA without cost by simply agreeing to the above conditions and

displaying, at their option, this logo in their documentation.

====================================================================

WARRANTY:

Software:

Simple Software warrants that the software contained herein will

perform in substantial compliance with the documentation

accompanying the software. If you report, in writing, a significant

defect to us, and we are unable to correct it within 90 days of the

date you report the defect, you may return the software and

accompanying materials, and we will refund the purchase price.

Diskette's and Documentation:

Simple Software, warrants all diskette's and documentation to be

free of defects in materials for a period of 30 days from the date

of purchase. In the event of notification within the warranty period

of defects in any materials, Simple Software will replace the

defective diskette or documentation.

Remedies:

The remedy for breach of the warranty shall be limited to

replacement and shall not encompass any other damages, including but

not limited to loss of profit, special, incidental, consequential,

or similar damages, losses, or claims.

DISCLAIMER:

Simple Software specifically disclaims all other warranties,

expressed or implied, including but not limited to, implied

warranties of merchantability and fitness for a particular purpose

with respect to defects in the diskette and documentation, and the

program license granted herein, in particular, and without limiting

operation of the program license with respect to any particular

application, use, or purpose.  In no event shall Simple Software be

liable for any loss of profit or any other commercial damage,

including but not limited to special, incidental, consequential or

other damages.

GOVERNING LAW:

This statement shall be construed, interpreted, and governed by the

laws of the State of Louisiana.

====================================================================

Registering your copy will help continue the competitive advantages

of providing economical shareware. Upon receipt of your payment I

will provide a registration code number which will allow you to self

register and personalize your shareware copy. Upon receipt of your

code  type SLICK REGISTER and enter your first and last name followed

by the code number. Code numbers may be issued by voice phone if you

are willing to accept a collect call, otherwise it will be sent to

you via first class US mail service.

Yes Gary, I enjoy your program and would like to register and

obtain the latest version.

Name_____________________________________________________________

Mailing Address__________________________________________________

City & State ___________________________________________________

ZIP _____________________________ Phone _________________________

Send $4 + $1 (ship & hand) check or money order to:

Gary M. Raymond

P.O.Box 8184

New Orleans, La. 70182

504-288-6550

Compuserve 70613,3165

====================================================================

EOF