�������������������������������������������������������������������������ͻ

�                                                                         �

�           Intel External Modem Serial Port Pin Assignments              �

�                                                                         �

�������������������������������������������������������������������������ͼ

GENERAL INFORMATION

Intel external modems use a DB25S (D-shell, 25-pin female) connector

for the RS-232 serial port.  The connector on the modem's end of the

cable must be a DB25P (D-shell, 25-pin male) connector.  The connector

on the opposite end must mate to the serial port on your computer or

terminal.  Consult the device's user guide for more information on its

connector.

ASYNCHRONOUS CABLE CONSTRUCTION

This diagram shows you how to build a cable good for synchronous and

asynchronous cummunications.  Most PCs, however, use RS-232

asynchronous serial ports.  For synchronous communications, you must

use a synchronous serial port.

NOTE:   You must use shielded cable and shielded connector backshells

when building the cable.

�����������������������������������������������������������ͻ

�Modem End � Signal                 �Signal   � DB25 � DB9  �

�Pin Number� Name                   �Direction� Pin #� Pin #�

�����������������������������������������������������������͹

�     1    �Case Ground             �   N/A   �   1  �      �

�����������������������������������������������������������Ķ

�     2    �Transmitted Data (TD)   � <�����  �   2  �   3  �

�����������������������������������������������������������Ķ

�     3    �Received Data    (RD)   � �����>  �   3  �   2  �

�����������������������������������������������������������Ķ

�     4    �Request to Send (RTS)   � <�����  �   4  �   7  �

�����������������������������������������������������������Ķ

�     5    �Clear to Send   (CTS)   � �����>  �   5  �   8  �

�����������������������������������������������������������Ķ

�     6    �Data Set Ready  (DSR)   � �����>  �   6  �   6  �

�����������������������������������������������������������Ķ

�     7    �Signal Ground           �   N/A   �   7  �   5  �

�����������������������������������������������������������Ķ

�     8    �Carrier Detect   (CD)   � �����>  �   8  �   1  �

�����������������������������������������������������������Ķ

�    15    �Transmit Data Clock     � �����>  �  15  �      �

�����������������������������������������������������������Ķ

�    17    �Receive Data Clock      � �����>  �  17  �      �

�����������������������������������������������������������Ķ

�    20    �Computer Ready  (DTR)   � <�����  �  20  �   4  �

�����������������������������������������������������������Ķ

�    22    �Ring Indicator   (RI)   � �����>  �  22  �   9  �

�����������������������������������������������������������Ķ

�    24    �External Transmit  Clock� �����>  �  24  �      �

�����������������������������������������������������������ͼ

���������������������������������������������������������������������������

March 2, 1992             Intel FaxBACK # 4300                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�           Intel External Modem Serial Port Pin Assignments              �

�                                                                         �

������������������������������ (continued) ������������������������������ͼ

2400EX/2400EX MNP CUSTOM CABLE DIAGRAM

Cable specification for connecting 2400EX to _________________________.

������������������������������������������������������������ͻ

� Modem End �  Signal               � Signal    � Other End  �

�Pin Number �  Name                 � Direction � Pin Number �

������������������������������������������������������������͹

�     1     � Case Ground           �    N/A    �            �

������������������������������������������������������������Ķ

�     2     � Transmitted Data (TD) �  <�����   �            �

������������������������������������������������������������Ķ

�     3     � Received Data    (RD) �  �����>   �            �

������������������������������������������������������������Ķ

�     4     � Request to Send (RTS) �  <�����   �            �

������������������������������������������������������������Ķ

�     5     � Clear to Send   (CTS) �  �����>   �            �

������������������������������������������������������������Ķ

�     6     � Data Set Ready  (DSR) �  �����>   �            �

������������������������������������������������������������Ķ

�     7     � Signal Ground         �    N/A    �            �

������������������������������������������������������������Ķ

�     8     � Carrier Detect   (CD) �  �����>   �            �

������������������������������������������������������������Ķ

�    20     � Computer Ready  (DTR) �  <�����   �            �

������������������������������������������������������������Ķ

�    22     � Ring Indicator   (RI) �  �����>   �            �

������������������������������������������������������������ͼ

���������������������������������������������������������������������������

March 2, 1992             Intel FaxBACK # 4300                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�           Intel External Modem Serial Port Pin Assignments              �

�                                                                         �

������������������������������ (continued) ������������������������������ͼ

14.4EX/9600EX CUSTOM CABLE DIAGRAM

Cable specification for connecting 14.4EX/9600EX to

______________________.

NOTE:   You must use shielded cable and shielded connector backshells

when building cables for this modem.

��������������������������������������������������������������ͻ

� Modem End �  Signal                 � Signal    � Other End  �

�Pin Number �  Name                   � Direction � Pin Number �

��������������������������������������������������������������͹

�     1     � Case Ground             �    N/A    �            �

��������������������������������������������������������������Ķ

�     2     � Transmitted Data (TD)   �  <�����   �            �

��������������������������������������������������������������Ķ

�     3     � Received Data    (RD)   �  �����>   �            �

��������������������������������������������������������������Ķ

�     4     � Request to Send (RTS)   �  <�����   �            �

��������������������������������������������������������������Ķ

�     5     � Clear to Send   (CTS)   �  �����>   �            �

��������������������������������������������������������������Ķ

�     6     � Data Set Ready  (DSR)   �  �����>   �            �

��������������������������������������������������������������Ķ

�     7     � Signal Ground           �    N/A    �            �

��������������������������������������������������������������Ķ

�     8     � Carrier Detect   (CD)   �  �����>   �            �

��������������������������������������������������������������Ķ

�    12     � Speed Indicator         �  �����>   �            �

��������������������������������������������������������������Ķ

�    15     � Transmit Data Clock     �  �����>   �            �

��������������������������������������������������������������Ķ

�    17     � Receive Data Clock      �  �����>   �            �

��������������������������������������������������������������Ķ

�    20     � Computer Ready  (DTR)   �  <�����   �            �

��������������������������������������������������������������Ķ

�    22     � Ring Indicator   (RI)   �  �����>   �            �

��������������������������������������������������������������Ķ

�    24     � External Transmit  Clock�  �����>   �            �

��������������������������������������������������������������ͼ

���������������������������������������������������������������������������

March 2, 1992             Intel FaxBACK # 4300                       Page 3

�������������������������������������������������������������������������ͻ

�                                                                         �

�           Intel External Modem Serial Port Pin Assignments              �

�                                                                         �

������������������������������ (continued) ������������������������������ͼ

MACINTOSH 8-PIN SCC MODEM CABLE

Cable specification for connecting an Intel external modem to an Apple

Macintosh.

NOTE:   You must use shielded cable and shielded connector backshells

when building cables for this modem.

������������������������������������������������������������ͻ

� Modem End �  Signal               � Signal    � Macintosh  �

�Pin Number �  Name                 � Direction � Pin Number �

������������������������������������������������������������͹

�  �� 1     � Case Ground           �    N/A    �            �

��ĳ��������������������������������������������������������Ķ

�  �  2     � Transmitted Data (TD) �  <�����   �    3  (TX-)�

��ĳ��������������������������������������������������������Ķ

�  �  3     � Received Data    (RD) �  �����>   �    5  (RX-)�

��ĳ��������������������������������������������������������Ķ

�  �  4     � Request to Send (RTS) �  <�����   �    1  (HSO)�

��ĳ��������������������������������������������������������Ķ

�  �  5     � Clear to Send   (CTS) �  �����>   �    2  (HSI)�

��ĳ��������������������������������������������������������Ķ

�  �  6 Ŀ  � Data Set Ready  (DSR) �  �����>   �            �

��ĳ����ĳ��������������������������������������������������Ķ

�  �� 7  �  � Signal Ground         �    N/A    � 8 & 4 (GND)�

��������ĳ��������������������������������������������������Ķ

�     8  �  � Carrier Detect   (CD) �  �����>   �            �

��������ĳ��������������������������������������������������Ķ

�    20 ��  � Computer Ready  (DTR) �  <�����   �            �

������������������������������������������������������������Ķ

�    22     � Ring Indicator   (RI) �  �����>   �            �

������������������������������������������������������������ͼ

NOTE:   On the Intel modem end (DB25 connector), short pins 1 and 7

together in addition to shorting pins 6 and 20.  On the

Macintosh end (SCC 8), short pins 8 and 4 together.

If you look at the pins of the 8-pin SCC connector, they are numbered

like this (the block at the top represents the key which prevents you

from incorrectly inserting the connector):

�

6 7 8

3 4   5

1 2

���������������������������������������������������������������������������

March 2, 1992             Intel FaxBACK # 4300                       Page 4