�������������������������������������������������������������������������ͻ

�                                                                         �

�              Setting Up a Modem in a UNIX/XENIX System                  �

�                                                                         �

�������������������������������������������������������������������������ͼ

UNIX/XENIX BASED OPERATING SYSTEMS

UNIX, XENIX, VMS and ULTRIX

Setting up a modem for an operating system other than DOS can be

tricky. There are so many different varieties of each operating system

that there is no one setup that will work with them all.  This document

describes the general configuration requirements of most non-DOS

operating systems.  Consult your operating system's documentation for

more specific information.

To use a modem in a non-DOS operating system, you must:

1.  Configure the computer's serial port for the highest baud rate the

modem supports.

2.  Connect the modem to the port.

3.  Configure the modem.

CONFIGURE THE SERIAL PORT

The first step is to configure the serial port you want to use for the

modem.  You can download instructions for configuring SCO Xenix from

Intel's BBS. Download 9600XENX.TXT from the "Modem" section of the

"Files Download" menu. For other operating systems, read the manuals or

contact the manufacturer for serial port configuration information.

Since calling modems can never connect at a speed faster the the modem-

to-computer connection, always configure the serial port for the

maximum modem through-put speed.  For example, if a 9600EX is connected

to a computer at 4800 baud, 4800 is the fastest rate at which any modem

can connect.  If the serial port cannot match the maximum modem

through-put, select the next highest available speed.  Here are the

recommended settings for Intel modems:

Maximum

Modem                Through-put

-----------------    -----------

SatisFAXtion/400E       57600

14.4EX                  57600

9600EX                  38400

2400EX MNP               4800

2400EX                   2400

Also, you must enable CTS/RTS flow control on the serial port if you

want to allow "Reliable" connections.  Reliable connections use MNP or

CCITT error correction and data compression protocols to ensure

accurate data transfer and maximum through-put.

CONNECT THE MODEM TO THE PORT

Once the serial port is configured, you need the correct cable to

connect the serial port to the modem.  FaxBACK document 4300 shows the

correct cable configuration.

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4350                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�              Setting Up a Modem in a UNIX/XENIX System                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

CONFIGURE THE MODEM

Lastly, you must configure the modem correctly.  There are many

possible configurations, but Unix/Xenix documentation often suggests

this string:

ATE0Q1S0=1&C1&D0\J0\N3\Q3&W

Intel does not guarantee this command will work with all operating

systems, but you can use it as a basis to begin modem configuration.

Here is a description of the commands in the configuration string:

Modem Command    Function

-------------    --------

AT            Attention code.  Must be first in line

E0            Modem doesn't echo

Q1            Modem doesn't display result codes

S0=1          Modem answers after 1 ring

&C1           Carrier Detect signal always on

&D0           Ignore DTR signal from mainframe

\J0           Disable serial baud rate adjust

\N3           Auto-reliable mode (MNP active)

\Q3           Bi-directional CTS/RTS flow control

&W            Save configuration to non-volatile RAM

If you're receiving data from a main frame and losing characters,

there's a good chance that the cable and/or the operating system

doesn't have CTS/RTS flow control enabled. Refer to your reference

manual for the operating system, contact your system administrator, or

contact the manufacturer of the operating system to find out how to

enable CTS/RTS.

Some operating systems require that the Data Carrier Detect (DCD or CD)

signal remain "ON" at all times. Issue the command  AT&C0&W  to turn on

DCD permanently.  Make sure your serial cable has a wire connecting the

modem's DCD signal (pin 8) to the computer's DCD signal.

If you still have problems using the modem after trying these

suggestions, contact the manufacturer of the operating system and ask

them how to install a Hayes 2400 baud modem.  Follow the instructions

except, if you're using a 9600EX, set the serial port baud rate to

38,400bps.

When the modem is correctly connected and configured, these LEDs on the

front panel should be lighted.

AA             Auto Answer

TR   Terminal Ready

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4350                       Page 2