�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: E-P                  �

�                                                                         �

�������������������������������������������������������������������������ͼ

CROSSTALK COMMUNICATOR

Crosstalk Communicator comes free with every Intel 14.4EX modem.  It

offers 13 different terminal emulations and 8 file transfer protocols

including ZMODEM.  The Intel version of Crosstalk Communicator contains

set up selections for all Intel modems.  The off-the-shelf version from

DCA must be configured specially for your modem.  The table shows the

settings that are different  from modem to modem.  The list shows

settings that are the same for all Intel modems.

����������������������������������������Ŀ

�  Table of Unique Intel Modem Settings  �

����������������������������������������������������������������

�                           �Max   �Lock �Modem

Modem        � Modem Init                �Speed �Speed�Flow

����������������������������������������������������������������

2400B         �                           �      �     �

2400EX        � ATE0Q0V1X4^M              �  2400� off � off

2400B Modem 2 �                           �      �     �

����������������������������������������������������������������

2400B MNP     �                           �      �     �

2400EX MNP    � ATE0Q0X4\J0\N3\Q3\V1^M    �  4800�  on � rts/cts

SatisFAX.     �                           �      �     �

����������������������������������������������������������������

14.4EX        � ATE0Q0V1X4\J0\N3\Q3\V1^M  � 38400� on  � rts/cts

����������������������������������������������������������������

9600EX        �^M~ATE0Q0V1X4\J0\N3\Q3\V1^M� 38400� on  � rts/cts

����������������������������������������������������������������

����������������������������������������Ŀ

�  List of Common Intel Modem Settings   �

����������������������������������������������������������������

Dial Prefix: ATDT            �       Connect: CONNECT^M^J

Dial Suffix: ^M              � Connect Speed: CONNECT %^M^J

Modem Init: {see table}     �  Connect Rel.: CONNECT %/REL^M^J

Answer Setup: ^M~ATS0=1^M     �    No Connect: NO CARRIER^M^J

Hang Command: +++~~~ATH^M     �     Line Busy: BUSY^M^J

Max Speed: {see table}     �    Command OK: OK^M^J

Lock Speed: {see table}     � Command Error: ERROR^M^J

Results Only: off             �     Dial Tone:

Modem Flow: {see table}     �  No Dial Tone: NO DIALTONE^M^J

Address:{3F8,2F8,3E8,2E8}�          Ring: RING^M^J

IRQ: {4, 3, 5, 2}    �

����������������������������������������������������������������

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4354                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: E-P                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

LOTUS EXPRESS

* Customer report *   One user reports that Lotus Express works with an

Intel 9600EX when you use this command in the  Connection

Initialization string:

ATV1X1DT@N|{CONNECT}@M

MTE/MTEZ

All of Intel's modems are defined in MTE and MTEZ's setup, and no extra

setup is required.

PC ANYWHERE VERSION 3.X

This program is compatible with Intel modems.  The PC Anywhere file

AWMODEMX.CNF contains a list of supported modems.  If Intel modems are

not in this list, you must download the latest  AWMODEMX.CNF file from

DMA's bulletin board system.  All of Intel's modems are listed in the

latest version of this file.

DMA's BBS number is (516) 736-1494.

PC ANYWHERE VERSION IV

This program is compatible with Intel modems.  The PC Anywhere IV

AWMODEM.CNF file lists all Intel 2400bps modems.  If you want to use a

14.4EX or 9600EX and it isn't listed,  you must download the latest

AWMODEMX.CNF file from DMA's BBS.

DMA's BBS is (516) 736-1494.

Special notes about SatisFAXtion and PC Anywhere:

It is possible to use PC Anywhere IV to set up remote Faxing on a

computer with an Intel SatisFAXtion Board. The SatisFAXtion fax and

modem MUST be set to answer on the same number of rings. The fax answer

portion can be set in the SETUP program under Advanced Setup/Set Up

Technical Software Options/Rings to Answer.

The rings until answer option on the modem is set in PC Anywhere IV in

the "PC Anywhere Hardware Configuration". You must also use "EXIT" and

select "Allow incoming calls" option when starting PC Anywhere 4 HOST.

Using any other HOST mode option will cause the computer to lock up if

a fax comes in.

Set the time on the remote PC to your time, or at least note any time

difference between that PC and your time.

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4354                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: E-P                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

Ensure that a fax gets scheduled for 5 or 10 minutes in the future. If

this is not done, the fax board won't accept modem calls from that

point on.

Disconnect from the Host PC using the EXIT, ALLOW INCOMING CALLS option

of PC Anywhere 4. This will allow the HOST PC to release the modem and

allow it to send the fax.

PROCOMM 2.4.2 AND 2.4.4

This program is compatible with Intel modems.  Here's how to configure

Procomm.

NOTE:   This version doesn't support CTS/RTS flow control - only

XON/XOFF!

After entering Procomm, press any key to go directly into terminal

mode.

�   Press <Alt-P> to access the "Line Settings" menu

-   Select the Port that the modem is using

-   Select the "Baud" rate for your modem:

9600EX/14.4EX               : 38400

All 2400 Baud MNP modems    :  4800

All 2400 Baud non-MNP modems:  2400

-   Type 24 and press <ENTER> to save these settings

�   Press <ALT-S> to access the "Setup Menu"

-   Press 1 then <ENTER> to select "Modem Setup"

-   Press 1 then <ENTER> to select "Modem Init String"

-   Type ATZ then <ENTER.

-   Press <ESC>

-   Press 2 then <ENTER> to select "Terminal Options"

-   Press 3 then <ENTER> to select "Flow Control"

-   Press <SPACE> until "XON/XOFF" appears then <ENTER>

-   Press <ESC>

-   Press S then <ENTER> to save "Save Setup to Disk"

-   Press <ESC>

Now configure your modem using the appropriate command.

9600EX/14.4EX               : AT&F\G1\Q1

All 2400 Baud MNP modems    : AT&C1&D2\G1\J0\\N3\Q0\V1

All 2400 Baud non-MNP modems: AT&C1&D2

That's all there is to it!

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4354                       Page 3

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: E-P                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

PROCOMM PLUS V1.0, 1.0B

This program is compatible with Intel modems.  Here's how to configure

Procomm Plus.

�   Start Procomm Plus then, press any key to go directly to Terminal

mode.

�   Press <ALT-P> to access the "Line Settings" menu.

-   Select the "Port" on which you installed your modem.

-   Select the correct "Baud" rate for your modem:

9600EX/14.4EX               : 38400

All 2400 Baud MNP modems    :  4800

All 2400 Baud non-MNP modems:  2400

-   Press <ALT-S> to save the setup

�   Press <alt-S> to access the "Setup Utility" "Main Menu"

-   Select "Modem Options" then "General Options"

-   Change "Initialization Command" to "ATZ^M"

-   Press <ESC> twice

-   Select "Terminal Options" then "General Options"

-   Set "Software Flow control" and "Hardware Flow Control" for

your modem:

Modem     Hardware Flow Control   Software flow Control

-----     ---------------------   ---------------------

non-MNP              OFF                       ON

MNP                  ON                        OFF

9600EX/14.4EX        ON                        OFF

-   Press <ESC>

�   Select "Save Setup Options" then <ENTER>

�   Press <ESC>

Now configure your modem.  You only need to do this once.

�   Enter the corrent command for your modem.

9600EX/14.4EX               :  AT&F&W<ENTER>

All 2400 Baud MNP modems    :  AT&F\N3\J0\Q3\V1&W <ENTER>

All 2400 baud non-MNP modems:  AT&C1&D2&W <ENTER>

All done!

NOTE:   Before operating in HOST mode, go into the setup menus and make

sure AUTO-BAUD DETECT is set to OFF.

PROCOMM PLUS V2.0 AND 2.01

This program is compatible with Intel modems.  Procomm Plus V2.0 and

2.01 have all of Intel's modems built in to their setup program.  Run

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4354                       Page 4

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: E-P                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

the PCINSTAL.EXE program in the Procomm subdirectory and select the

proper Intel modem.

NOTE:   Before operating in HOST mode, go into the setup menus and make

sure AUTO-BAUD DETECT is set to OFF.

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4354                       Page 5