�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: A-D                  �

�                                                                         �

�������������������������������������������������������������������������ͼ

CARBON COPY +

SatisFAXtion/100:

Intel's lab tested this software (version 5.0 & 6.0) with the

SatisFAXtion/100 board, and it is compatible.

Intel Modems:

This program is compatible with Intel modems.

Configuring the Program:

Here's how to configure Carbon Copy +.

�   Run CCINSTAL and choose the communications port for the modem.

Also set these options as indicated:

Your Intel Modem             CC+ Modem Choice    Port rate

����������������������       ����������������    ���������

Intel V.42 modems (1)     :   Intel 9600EX          38400

SatisFAXtion/200          :   Intel 9600EX           9600

Intel 2400B MNP modems (2):   Intel 2400EX MNP       4800

Intel non-MNP modems (3)  :   Hayes 2400             2400

That's all there is to it!

CLOSE UP

* Customer report *   Customers report that this program works well

with the 9600EX and SatisFAXtion modem.  Intel recommends setting

AUTOBAUD=OFF when using Close Up with Intel V.42 modems (1) or 2400B

MNP modems (2). You can select AUTOBAUD=ON when using an Intel non-MNP

modem (3).

COMPUSERVE INFORMATION MANAGER

Intel Modems:

Intel modems are compatible with this program.

Configuring the Program:

Here's how to configure Compuserve Information Manager:

�   Run Compuserve Information Manager

�   Press Alt-P to get to the special screen

�   Select "Session Settings" and press <ENTER>

�   Fill out all personal information.

-   Press ALT-B to set the  "BAUD RATE".

Intel V.42 modems (1)      :  9600

Intel 2400B MNP modems (2) :  4800

Intel non-MNP modems (3)   :  2400

-   Press ALT-C to set the COM.

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4352                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: A-D                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

�   Press ALT-M to get into the modem setup screen

Initialize           : ATS0=0 Q0 V1 S12=100^M

Prefix               : AT

Reset                :   Z

Dial Tone            :   DT

Dial Pulse           :   DP

Hang up              :   H

Suffix               : ^M

Escape               : +++

Modem acknowledgement: OK

Connect Message      : CONNECT

Connect failure      : NO CARRIER

Logon Parameters     : <leave blank>

Pause Time           : 10

Match Time           : 10

�   Press ALT-O twice.

You're done!

CROSSTALK XVI

SatisFAXtion/100:

Intel's lab tested this software (version 3.7) with the

SatisFAXtion/100 board, and it is compatible.

Intel Modems:

This program is compatible with Intel modems.

Configuring the Program:

Here's how to configure Crosstalk XVI.

�   Start up Crosstalk XVI. At the Status Screen, press <ENTER> to get

into command mode:

�   Set the correct serial port.

-   Type:  PO x <ENTER>  (where "x" is the serial port that your

Intel modem is using).

�   Enter the appropriate command to set the correct baud rate for your

modem:

Intel V.42 modems (1)     : SP 38400

SatisFAXtion/200          : SP 9600

Intel 2400B MNP modems (2): SP 4800

Intel non-MNP modems (3)  : SP 2400

�   Set the flow control for your modem

Hardware flow   Software flow

-------------   -------------

Intel V.42 Modems (1)     :      ON              OFF

Intel 2400B MNP modems (2):      ON              OFF

Intel non-MNP modems (3)  :      OFF             ON

-   To view the Miscellaneous Parameters screen, type: LIT <ENTER>

-   To turn Software Flow Control OFF, type: FLOW- <ENTER>

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4352                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: A-D                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

To turn Software Flow Control  ON, type: <Ctrl-S>

-   To turn on Hardware Flow Control, type : HA ON <ENTER>

To turn off Hardware Flow Control, type: HA OFF <ENTER>

�   Go into terminal mode and issue the appropriate command for your

modem.

-   Type GO LO <ENTER>

-   Enter the correct command for your modem.

Intel V.42 modems (1)     :  AT&F&W <ENTER>

Intel 2400B MNP modems (2):  AT&C1&D2\N3\J0\Q3\V1&W

<ENTER>

Intel non-MNP modems (3)  :  AT&C1&D2&W <ENTER>

You're done!

CROSSTALK MARK 4

SatisFAXtion/100:

Intel's lab tested this software (version 1.01A) with the

SatisFAXtion/100 board, and it is compatible.

Intel Modems:

This program is compatible with Intel modems.

Configuring the Program:

Here's how to configure Crosstalk Mark 4.

With AUTOEXEM enabled, you will see a menu after starting Crosstalk

Mark 4. Press <ESCAPE> to get to the command prompt. With AUTOEXEM

disabled, you will immediately see the command prompt.

�   Set the serial port.  Type: PORT <ENTER>

-   Highlight the port on which you installed your modem and press

<ENTER>.

�   Select the correct baud rate for your modem. Type: SPEED <ENTER>

Intel V.42 modems (1)     : 38400 <ENTER>

Intel 2400B MNP modems (2):  4800 <ENTER>

Intel non-MNP modems (3)  :  2400 <ENTER>

�   Select flow control for your modem. Type: FLOW <ENTER>

Intel V.42 modems (1)     : RTS/CTS <ENTER>

Intel 2400B MNP modems (2): RTS/CTS <ENTER>

Intel non-MNP modems (3)  : XON/XOFF <ENTER>

�   Select "modem init" string in Crosstalk Mark 4.

-   Type: MODEMINIT <ENTER>

-   Type: |ATZ|  <ENTER>

�   Go into terminal mode and configure the modem.

-   Type: LOCAL ON <ENTER>

-   Type: GO <ENTER>

-   Enter the correct init string for your modem:

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4352                       Page 3

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: A-D                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

Intel V.42 modems (1)     : AT&F&W <ENTER>

Intel 2400B MNP modems (2): AT&F&C1&D2\N3\J0\Q3\V1&W <ENTER>

Intel non-MNP modems (3)  : AT&F&C1&D2&W <ENTER>

You're done!

DOW JONES 2.03

Intel Modems:

This program is compatible with Intel modems.

Configuring the Program:

To configure the program, run the MAP program then:

1.  In the Data Maintenance window, select Setup System.

2.  Press Y in answer to the question, "Do you want to make any changes

now?"

3.  Under Communications Parameters:

a.  Highlight Com Port and select the port to which the modem is

attached.  P ress 1 for COM1, 2 for COM2)

b.  Highlight Modem Type and select the correct modem type for your

modem:

Intel V.42 modems (1)     : press O to select Other.

Intel 2400B MNP modems (2): press O to select Other.

Intel non-MNP modems (3)  : press H to select Hayes.

c.  Highlight Baud Rate and press 2 to select 2400.

4.  If you have a 2400 Baud non-MNP modem, you done.  If you have a

14.4EX, 9600EX, or 2400 Baud MNP modem, press ESC to configure

modem command strings.

a.  Highlight Modem Reset and enter the correct reset string for

your modem:

Intel V.42 modems (1)     : ,+++,AT&F\J1\N0\Q0\V0"H0-J0[13]

Intel 2400B MNP modems (2): ,+++,AT&F\J1\N0\Q0\V0[13]

b.  Press ENTER.  Just press ENTER at the Modem Dial, Acknowledge,

or Hang up strings to leave them unchanged.

CROSSTALK FOR WINDOWS V1.1

When you run Crosstalk for Windows the first time, it offers the option

to "Run Config". Select "Cancel" to get to the "Crosstalk for Windows"

screen. After the first time, Crosstalk displays its screen

immediately. Consult your Windows or Crosstalk manuals if you don't

know how to "select" an option.

If you have an Intel V.42 modem (1) or 2400B MNP modem (2), follow

these instructions:

�   Select the "Setup" then select "Device".

-   Select the "Port" on which you installed your modem.

-   Select the "Speed" for your modem:

Intel V.42 Modems (1)     :  19200

Intel 2400B MNP modems (2):  4800

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4352                       Page 4

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: A-D                  �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

-   Select the "Flow Control" option "RTS/CTS".

-   Select "OK"

�   Select the "Setup" menu then select "Modem".

-   Select "Custom Modem".

-   Select "Setting".

-   Set "Dial Prefix" to:    ATDT

-   Set "Dial Suffix" to:    ^M

-   Set "Answer Setup" to:   ATS0=1^M

-   Set "Initialization" for your modem:

Intel V.42 modems (1)     : ^M~ATE0V1X4\J0\N3\Q3\V1^M

Intel 2400B MNP modems (2): ^M~ATE0\J0\N3\Q3\V1^M

-   Set "Disconnect" to:        ~~+++~~ATH0^M

-   Set "Command OK" to:        OK^M^J

-   Set "Command Error" to:     ERROR^M^J

-   Set "No Carrier" to:        NO CARRIER^M^J

-   Set "Connect" to:           CONNECT^M^J

-   Set "Connect Speed" to:     CONNECT %^M^J

-   Set "Connect ARQ" to:       CONNECT %/REL^M^J

-   Set "Dial Tone" to:         TONE^M^J

-   Set "No Dial Tone" to:      NO DIALTONE^M^J

-   Set "Ringing" to:           RING^M^J

-   Set "Busy" to:              BUSY^M^J

-   Set "Automatically Select Speed" to:  No (Remove X)

-   Select "OK"

�   Select the "File" menu then select "Save As".

-   Type:  NORMAL

-   Select "OK".

If you have an Intel non-MNP modem (3), follow these instructions:

�   Select the "Setup" menu then select "Device".

-   Select the "Port" on which you installed your modem.

-   Select the "Speed" option "2400".

-   Select "OK" (leave other options as they are).

�   Select "Setup" menu then select "Modem".

-   Select "Hayes 2400".

-   Select "OK".

�   Select the "File" menu then select "Save As".

-   Type:  NORMAL

-   Select "OK".

���������������������������������������������������������������������������

June 4, 1992              Intel FaxBACK # 4352                       Page 5