�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: Q-Z                  �

�                                                                         �

�������������������������������������������������������������������������ͼ

QMODEM

This program is compatible with Intel modems.  The first time you run

Qmodem it automatically takes you to the "Setup Menu".  After the first

time, use <ALT-N> to call up the "Setup Menu".

�   Press M to select the Modem menu

-   Press C to select "Communications Parameters"

-   Press A to select "Default baud rate"

-   Highlight the baud rate for your modem then press <ENTER>

14.4EX                      :  57600

9600EX                      :  38400

All 2400 Baud MNP modems    :  4800

All 2400 Baud non-MNP modems:  2400

-   Press J to select "Serial Port"

-   Type the number of the COM port of your modem and press <ENTER>

-   Press <ESCAPE>

�   Press M to select "Modem Commands"

-   Press I to select "Initialization Command"

-   Type  ATZ^M then press <ENTER>.

-   Press <ESCAPE>

NOTE:   QMODEM Comes with COM1 and COM2 defined, but COM3 and COM4 are

undefined. If you installed your modem on COM3 or COM4, use the

following commands to define the address and IRQ for COM3 or

COM4. If you installed your modem on COM1 or COM2, skip over

this section to the next.

�   If you installed your modem on COM3 or COM4, press P to select

"Port Addresses" then follow these instructions:

-   Highlight the "Base" address of your modem's COM port then

<ENTER>

-   Type 0, your modem's 3 digit address (2E8 or 3E8), then press

<ENTER>

-   Highlight the "IRQ" of your modem's COM port then <ENTER>

-   Type your modem's IRQ number then <ENTER>

-   Press <ESCAPE> twice

�   Press H to select "Host" options menu.

-   Press I to select "Init Command".

-   Type  ATZ^M (delete any extra characters) then <ENTER>

-   Press <ESCAPE>

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4356                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: Q-Z                  �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

�   Press O to select "Options"

-   Press T to select "Toggles"

-   Set the following option for your modem:

All 2400   All 2400   9600EX

non-MNP    MNP        14.4EX

--------   --------   ------

[XON/XOFF] Flow ctrl.    ON         OFF       OFF

CTS/RTS Flow ctrl.       OFF        ON        ON

CTS When writing         OFF        OFF       OFF

Redial sets baud rate    ON         OFF       OFF

-   Press <ESCAPE> twice

�   Press E to select "Exit"

-   Press S to select "Save changes"

�   Now configure your modem.

-   In terminal mode, enter the correct command for your modem:

9600EX                      : AT&F&W <ENTER>

All 2400 Baud MNP modems    : AT&F\N3\Q3\J0&W <ENTER>

All 2400 Baud non-MNP modems: AT&F&C1&D2&W<ENTER>

That's all there is to it!

QUICK LINK II

This modem is compatible with Intel modems.  Here's how to configure

Quick Link II.

�   Run Quick Link

�   Select "Modem Setup" from the "Setup"/"Modem Setup" menu

�   Set the "Init string" to  AT&F&C1&D2

�   Set "Dial prefix" to  ATDT

�   Set "Hangup string" to  ATH

�   Set "Modem port" to the phone icon

�   Select "Terminal options" from the "Setup"/"Terminal Options"menu.

9600EX   All 2400   All 2400

MNP        non-MNP

------   --------   --------

XON/XOFF flow control  OFF       OFF        ON

CTS/RTS flow control   ON        ON         OFF

Baud rate             19200      4800       2400

(38400 if possible)

That's all there is to it!

SMARTCOM II

Intel has tested this software and version 2.1 is compatible with all

Intel modems.  Smartcom II is designed for use with Hayes 2400 modems

and any deviation can cause problems.  So before using Smartcom II with

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4356                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: Q-Z                  �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

any Intel MNP modem (SatisFAXtion and 9600EX included), you must

configure them identically to the Hayes.  Use the "Terminal emulation"

option in Intel's TESTCOM program to send the command

AT\J1\N0\Q0\V0&Wto the modem.

Before using Smartcom II v3.0 with the SatisFAXtion or Connection

CoProcessor (CCP) modem, you must remove the fax program from memory.

If you don't, your system will lockup when you load Smartcom.  For

SatisFAXtion, enter  CASMGR /U.  For CCP, enter  XCAM.

Special notes about Smartcom II Version 3.0:

�   Version 3.0A is compatible with all Intel modems.

�   Version 3.0B is unknown.

�   Version 3.0C is incompatible because it checks for a Hayes ROM in

the modem.  If the modem ROM's checksum doesn't exactly match

Hayes, the program doesn't work with that modem.

SMARTCOM III

This program is compatible with all Intel modems.  Smartcom III is

designed for use with Hayes modems and any deviation can cause

problems.  So before using Smartcom III with any Intel MNP modem

(SatisFAXtion and 9600EX included), you must configure them identically

to the Hayes.  Use the "Terminal emulation" option in Intel's TESTCOM

program to send the command  AT\J1\N0\Q0\V0&Wto the modem.

TELIX/TELIX SE

This program is compatible with Intel modems.  After entering Telix,

configure the program and modem using these instructions.

�   Press <ALT-O> to access the "Configure Telix" menu.

�   Press T to select "Terminal Options".

-   Press A to select "Baud" rate.

-   Highlight the correct baud rate for your modem then press

<ENTER>

9600EX               : 38400

All 2400 Baud MNP    :  4800

All 2400 Baud non-MNP:  2400

-   Select the "Data Bits" and "Stop Bits" to match the systems you

call most often.  If it doubt, set "Data Bits" to 8 and "Stop

Bits" to 1.

-   Press B to set the "COM port".

-   Highlight the "COM port" on which you installed your modem then

press <Enter>.

-   Set f low control for your modem:

Modem                      XON/XOFF   CTS/RTS

��������������������       ��������   �������

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4356                       Page 3

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Configuring Communications Software: Q-Z                  �

�                                                                         �

��������������������������������� (continued) ���������������������������ͼ

9600EX                        OFF       ON

2400B MNP/2400EX MNP          OFF       ON

2400B Classic/2400EX          ON        OFF

-   Press <Enter>

�   Press M to select the "Modem & Dialing" menu.

-   Press  A  to select "Init String".

-   Type:  ATZ^M  (delete any extra characters) then press <Enter>.

-   Press <Enter> again.

�   Press W to "Write and Save".

�   Enter the correct command for your modem:

9600EX                      :  AT&F\N3\Q3\J0\V1&W

All 2400 Baud MNP modems    :  AT&F\N3\Q3\J0\V1&W

All 2400 Baud non-MNP modems:  AT&C1&D2&W

That's all there is to it!

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4356                       Page 4