�������������������������������������������������������������������������ͻ

�                                                                         �

�               Troubleshooting the Intel 2400B Modem 2                   �

�                                                                         �

�������������������������������������������������������������������������ͼ

CARRIER ALREADY ESTABLISHED MESSAGE

Carrier already established/Already on line

This error can happen when the modem is on line when entering a

communications package OR our 2400bps (non-MNP) modems are set to

factory configurations. When shipped from the factory, carrier detect

is on at all times, just like a Hayes 2400. To correct this problem,

issue:

AT&C1&D2&W

This causes carrier detect to represent the true state of what the

connection status is. The &D2 will cause the modem to hang up the phone

when DTR is dropped, which is also desired for most communication

packages.

USING COM3/COM4 UNDER WINDOWS 3.0

If you have problems using the your modem on COM3 or COM4 under Windows

3.0, exit Windows and use Intel's TESTCOM.EXE program (which comes with

all Intel modems) to test your modem.  TESTCOM performs several

diagnostic tests on the modem and helps you verify the phone line

connection.  SatisFAXtion board users who need a copy of TESTCOM can

order document 0018 from Intel's FaxBACK system.  Modem users can

download TESTCOM from the Fax/Modem section of Intel's BBS.

Once you know the modem works on COM3 or COM4 outside of Windows, you

can test the modem with your Windows program.  If you run Windows in

Enhanced mode, you need edit the Windows SYS.INI file.  Under the

[386enh] symbol, add these lines:

COM1BASE=3F8h

COM2BASE=2F8h

COM3BASE=3E8h

COM4BASE=2E8h

Editing your INI files is risky business.  Carefully read your Windows

documentation or call Microsoft Microsoft Technical Support if you

questions or problems.

Microsoft Tech Support:  206-454-2030

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4360                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Troubleshooting the Intel 2400B Modem 2                   �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

DETECTING PORTS WITH DEBUG

By using debug you can determine what COM ports are in the system. KEEP

IN MIND THAT THIS WILL ONLY SHOW THE PRESENCE OF COM 1 AND COM 2:

debug

-d40:0

0040:0000  F8 03 F8 02 00 00 00 00-BC 03 00 00 00 00 00 00 00 00

This display indicates the presence of a COM 1 (03F8) and a COM 2

(02F8). Note that the address bytes are reversed!

If you want to know what COM ports are in the system (including COM

3&4), use TESTCOM Version 4.0 or newer (available at FSI). Issue

"TESTCOM /S" at the DOS prompt.

DO YOU HAVE A DIGITAL PHONE LINE?

Conventional modems, including Intel's, will NOT work with digital

phone systems. Digital phone systems (Rolm, Aspect, etc...) convert

digital signals back to sound and from sound into digital for clearer

communication. All of this conversion is done in each end of the

telephone which is not something that is present in a data modem. All

digital phone systems are different. To use a modem on a digital line,

an A to D & D to A converter is needed. Each A to D converter is

specific to each phone system. Have your customer contact their phone

company for more details.A good way to identify a digital phone system

is to look on the phone for things like "Voice mail" buttons and

"Message waiting" lights. Most of the time these features are only

available in digital phone systems.

ERRATIC RESPONSE TO AT COMMANDS

Erratic response to AT commands can be attributed to two things: IRQ

Conflicts and/or COM port overlaps.  You can use the  TESTCOM.EXE

program can verify modem operation and detect conflicts.

First ensure that no other devices are using the same IRQ as the modem.

Modems use IRQs to signal the computer that data has been received or

sent. For an internal modem, make sure that the COM port it's set to

does not conflict with a COM port already in the system (whether used

or not). When in doubt, try another COM port setting.

Port   I/O   IRQ

����   ���   ���

COM1   3E8    4

COM2   2E8    3

COM3   3F8    4

COM4   2F8    3

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4360                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Troubleshooting the Intel 2400B Modem 2                   �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

For instance, consider a situation where you add an internal modem to a

system which has COM1 and COM2 built in. There's a mouse on COM1 and

nothing on COM2.  You must either:

�   Disable the built-in COM2 and address the modem as COM2 (prefered)

�   Leave the mouse on COM1 and address the modem as COM4

�   Move the mouse to COM2 and address the modem as COM3

INTERMITTENT SWITCH TO COMMAND MODE

If your modem intermittently switches to command mode while on line,

change the DTR option:

AT&D2&W

LEASED LINE SETUP

If modems fail to communicate over leased lines, but work fine on

standard lines, issue "AT&L1" to the modem. The default is &L0. Refer

to the modem guide for more information.

In addition, if the leased line is a 4 wire line rather than a 2 wire

line, a 4 wire to 2 wire converter will be required. This converter can

be obtained from their phone company.

MAKE IT A HAYES WHEN IT DOESN'T WORK!

If you are having problems with an MNP based modem that seem to be

unresolvable, turn off all of the extra features in our MNP modems.

Issue "AT&F\N0\Q0\V0\J1&W". This will make the modem look exactly like

a Hayes 2400.

MANUAL DIALING (EXISTING CONNECTION)

If someone tries to use the manual dialing method explained in the

manual they will receive a message of "NO DIALTONE". Use the following

instead:

o           Get into the terminal mode of your communications

package.

o           Dial the number and wait for the phone to start ringing.

-OR-

o Finish talking to the other person.

o Type ATH1 to have the modem pick up the phone

o Type ATO (letter O, not ZERO!) to instruct the modem to go

on line.

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4360                       Page 3

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Troubleshooting the Intel 2400B Modem 2                   �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

MODEM ANSWERS ONLY AT 300 BAUD

This problem affects only two Intel modems: the 2400EX MNP and the 2400

Classic MNP modems.  If either of these modems is set to auto-answer

and a call comes in at 300 baud, all subsequent calls will connect at

300 baud.  To eliminate the condition, you must turn the modem off,

wait a few seconds, then turn it back on.

This problem affects only incoming calls, not outgoing calls.

MODEM DIALS BUT DOES NOT CONNECT

Modem Dials but Does Not Connnect

This problem commonly results when your modem is set to use MNP but the

remote modem doesn't support MNP.  It can also happen if the value in

your modem's  S7 register is too low.  The value in the S7 register

tells the modem how long  to wait for a carrier signal after dialing.

To find out if MNP is causing the problem, enter the command:  AT\N0&W.

If the problem persists, make your modem act like a Hayes modem by

entering the command for your modem:

Intel Modem           Command

�����������������     �����������������������

SatisFAXtion          AT&F\N0\Q0\J1\V0&W

14.4EX/9600EX         AT&f\N0\Q0\J1\V0"H-J0&W

All 2400 Baud MNP     AT&F\N0\Q0\J1\V0&W

To find out if the value in the S7 register is causing the problem,

increace the value from its default of 30 seconds. Run the TESTCOM

program, select the "Terminal emulation mode", then type   ATS7=60&W

and press ENTER. If this doesn't help, try a higher value (the maximum

is 255).

Here are the common symptoms of an MNP problem:

Your modem dials, the remote modem answers, the modem tones begin, then

either:

a.  your modem hangs up, or

b.  your modem does nothing while the tones continue.

If the user still can't connect after disabling MNP, try calling the

system  yourself. If they're using a SatisFAXtion modem, make sure they

have the latest software. If not, they can order 0011 from Faxback.

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4360                       Page 4

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Troubleshooting the Intel 2400B Modem 2                   �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

MODEM MAKES CONNECTION THEN HANGS UP

Modem makes connection and then hangs up

There are two possible causes:

The modem on the other end is not MNP capable and can't deal with the

MNP starup sequence. The solution to this is to issue "AT\N0" to

disable MNP.

Secondly, the "wait time for answer" register may not be set long

enough. Try ATS7=90 (or something longer than what it is set to).

Sometimes international and/or long distance lines take longer than 30

seconds (default) to connect. Keep in mind that there is a similar

setting in most terminal programs when auto-dialing.Make sure that the

"Wait time for answer" in your communication package is set to S7's

value (or higher).

MODEM STUCK IN LEASED-LINE MODE

If you set the modem to Leased-Line mode and Synchronous mode, here's

the only way to reconfigure the modem:

1.  Make a connection with another modem.

2.  Enter +++ to put the modem into command mode.

3.  Enter AT&F to turn off Leased-Line and Synchronous mode.

When in Leased-Line mode, the modem tries to establish a connection as

soon as you turn it on.  The modem doesn't receive commands until after

it establishes a connection.

MODEM SUDDENLY DISCONNECTS

Intel MNP based modem (SatisFAXtion, 9600EX, etc...) have an inactivity

timer which hangs up after a predetermined amount of time with no data

sent or received. The remote modem disconnecting could be caused also

by extremely noisy lines. Try issuing "AT\T0" to disable the inactivity

timer. Try the call again.

Keep in mind most bulletin board systems will disconnect if no data is

sent or received within a predetermined amount of time.

MODEM COMMANDS BUT DOESN'T ECHO CHARS

First, issue "AT&F" to the modem to ensure that echo and result codes

are turned on. If that doesn't work, look for IRQ conflicts and/or COM

port overlaps (with internal modems). The modem tells the computer that

data has come in by using interrupts. If no interrupts are generated,

no characters will be received or echoed.

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4360                       Page 5

�������������������������������������������������������������������������ͻ

�                                                                         �

�               Troubleshooting the Intel 2400B Modem 2                   �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

MODEM WON'T ANSWER THE PHONE

There are two reasons why the modem won't answer:

1.  The modem isn't in Auto-Answer mode.

2.  The modem doesn't see the DTR signal from the computer.

The command ATS0 controls Auto-Answer mode.  An external modem turns on

"AA" light on when Auto-Answer mode is enabled.

ATS0=0    disables Auto-Answer

ATS0=1    answer after 1 ring

The command AT&D controls how the modem responds to the computer's DTR

signal.  An external modem turns on the "TR" light when it sees or is

set to ignore the DTR signal.  If the "TR" light isn't on, the modem

won't answer whether Auto-Answer is enabled or not.

AT&D0     Modem ignores DTR signal

AT&D2     Modem swtches from data to command mode, hangs up

and disables Auto-Answer mode when DTR switches

from ON to OFF.

Use AT&D0 if you serial port doesn't support the DTR signal or if your

modem cable doesn't connect to it. Otherwise, use AT&D2. DTR is on pin

20 of a 25-pin connector and on pin 4 of a 9-pin connector.

���������������������������������������������������������������������������

February 24, 1992         Intel FaxBACK # 4360                       Page 6