�������������������������������������������������������������������������ͻ

�                                                                         �

�        Intel 2400B Modem Technical Installation Instructions            �

�                                                                         �

�������������������������������������������������������������������������ͼ

INTRODUCTION

This document contains all the information an experienced user needs to

install the Intel 2400 BPS modems.  If you're not an experienced user,

you should use the modem's manual for installation instructions or have

your dealer install the modem.

Intel currently makes the following three 2400 BPS modems:

2400B Classic   Internal modem for IBM PC, PC XT, PC AT, COMPAQ 386 and

100% compatible computers.

2400B Modem 2   Internal modem for IBM PS/2 Models 50, 50Z, 60, 70 and

80, and 100% compatible computers.

2400EX          External Modem for any computer with an external RS232

port connector.

Refer to the installation sections that apply to the modem you're

installing.

SETTING SWITCHES

2400B CLASSIC MODEM

Before installing the 2400B Classic Modem into the computer, set its

switches for an address not used by any other serial ports or mouse in

your computer.  If you have just one serial port, COM2 should work

fine.

Port    Address    IRQ

���������ͻ            �������Ŀ    �����-  ��������-  ����-

� � � � � �            �ܳܳܳ߳     COM1     3F8h       4

� 1 2 3 4 �            �������Ĵ

���������ͼ            �߳ܳ߳ܳ     COM2     2F8h       3

����������������������������Ĵ

COM Port �ܳ߳ܳ߳     COM3     3E8h       4

Address  �������Ĵ

�߳߳߳ܳ     COM4     2E8h       3

���������

2400B MODEM 2

The 2400B Modem 2 has no switches.

2400EX EXTERNAL MODEM

The 2400EX external modem has no switches.

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4410                       Page 1

�������������������������������������������������������������������������ͻ

�                                                                         �

�        Intel 2400B Modem Technical Installation Instructions            �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

INSTALLING THE MODEM

BEFORE YOU BEGIN

Before installing any new device (modems included), always verify that

your computer is operating correctly.  Fix any existing problems BEFORE

installing an Intel modem.

Once you know the computer is operating correctly, use the steps

outlined in the appropriate section to install the Intel modem.

2400B MODEM 2

1.  Turn off the power switch on the system unit and any attached

equipment.

2.  Remove the system unit's cover.

3.  Insert the modem into an empty slot inside the computer.

4.  Replace the system unit's cover.

5.  Plug one end of the telephone cable provided with your modem into

the connector labeled LINE and plug the other end into the wall

jack.

6.  To continue using the phone, connect it to the modem by plugging

the free end of the telephone's cord into the connector labeled

PHONE on the modem.

7.  Turn on the computer and boot up using the Reference Disk.

8.  Press Enter when the IBM logo appears.

9.  Type N when asked "Automatically Configure System?".

10. If you haven't already done so, make a backup copy of your original

Reference Diskette.

11. Select Copy an Option Diskette to copy the Intel 2400B Modem 2

Options diskette to your Reference Diskette.

12. Select Set Configuration from the Main Menu.

13. Select Change Configuration from the Set Configuration menu.

14. Select a serial port for the 2400B Modem 2 to use which does not

conflict with other serial ports in your computer.  SERIAL_2

usually works fine.  An asterisk (*) appears when the current

choice conflicts with an existing serial port.

15. Save the configuration information and exit from the program.

Remove the Reference Diskette from drive A and put in a safe place.

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4410                       Page 2

�������������������������������������������������������������������������ͻ

�                                                                         �

�        Intel 2400B Modem Technical Installation Instructions            �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

2400B CLASSIC MODEM

1.  Turn off the power switch on the system unit and any attached

equipment.

2.  Remove the system unit's cover.

3.  Insert the modem into an empty slot inside the computer.

4.  Replace the system unit's cover.

5.  Plug one end of the telephone cable provided with your modem into

the connector labeled LINE and plug the other end into the wall

jack.

6.  To continue using the phone, connect it to the modem by plugging

the free end of the telephone's cord into the connector labeled

PHONE on the modem.

2400EX EXTERNAL MODEM

1.  Obtain a cable which has a 25-pin connector on one end and a

connector on the other end that mates with the serial port on your

computer (usually either 9-pin, 25-pin, or DIN connector).  This

cable is not included with the 2400EX.

2.  Turn off the power switch on the system unit and any attached

equipment.

3.  Connect the 25-pin end of the cable to the connector on the 2400EX

modem's rear panel.  Tighten the connector screws.

4.  Connect the other end of the cable into your computer's serial

port.  Tighten the connector screws.

5.  Connect the modem power supply cable to the round connector on the

back of the 2400EX.

6.  Plug the power supply into a wall outlet.

7.  Plug one end of the telephone cable provided with your modem into

the connector labeled LINE and plug the other end into the wall

jack.

8.  To continue using the phone, connect it to the modem by plugging

the free end of the telephone's cord into the connector labeled

PHONE on the modem.

FINISHING TOUCHES

VERIFYING MODEM OPERATION

Follow these steps to run the TESTCOM program to verify the modem's

operation:

1.  Turn on the modem.

2.  Insert the Intel modem diagnostics diskette into drive A and set

the default drive to A.

3.  Type TESTCOM and press Enter.

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4410                       Page 3

�������������������������������������������������������������������������ͻ

�                                                                         �

�        Intel 2400B Modem Technical Installation Instructions            �

�                                                                         �

������������������������������� (continued) �����������������������������ͼ

USING THE MODEM

You need a communications program (such as Crosstalk, Procomm and

Smartcom) to use the modem.

���������������������������������������������������������������������������

December 13, 1991         Intel FaxBACK # 4410                       Page 4