-----------------------------------------------------------------

Touch Tone Computer Control

TTCC v1.0�

by Jonah Peskin

7/14/92

-----------------------------------------------------------------

DISCLAIMER:  Because this software is offered as Public Domain,

the author  can not be held responsible for ANY damage resulting

from this software.

-----------------------------------------------------------------

IMPORTANT: The two files MODEM.BAK and ID.BAK are the example

configurations that are in these DOCs.  You may find it

helpful to edit these files and rename them to MODEM.CFG

and ID.CFG, the ASCII text files needed to run TTCC.

-----------------------------------------------------------------

What is Touch Tone Computer Control (TTCC)?

When written, the main purpose of Touch Tone Computer

Control was to  prevent a caller from getting through a telephone

line without knowing a pass code entered from a touch tone phone.

It was revised to be capable of running a program depending on

the code entered.  This provides a certain amount of remote

computer control for the user.

What it does:

Although not highly user friendly, TTCC should first

initialize your modem and wait for the phone to ring.  After a

specified number of rings, the modem will answer.  TTCC will send

a tone over the line to notify the caller that the modem has

answered.  The caller will enter a code (of any length) using the

telephone's  numeric keypad.  After entering the code, the caller

should press the pound (#) key, to let the computer know that the

caller has finished entering the code.  The computer will send

another tone to notify the caller that their code was received.

The actions from there on depend on what the user has specified.

Compatibility:

This software has been tested on an Intel 386DX/25 and a US

Robotics Dual Standard modem w/v32bis.  To my knowledge, US

Robotics is the only company which produces modems with built-in

touch tone recognition.  Though this software is written for this

modem in mind, it is possible to change the commands sent to the

modem.  This is explained later in Configuring "MODEM.CFG".

How to configure:

Being the first (Beta) version of this program, it isn't

very user friendly.  You will need to create  ASCII text files

which include slightly cryptic information.  It is not difficult

to enter the information using any ASCII text editor, but it is

VERY important that the you follow the format shown.

NOTE: All given examples are for a HAYES/USR COMPATIBLE modem,

but using a Hayes compatible modem is NOT mandatory.

Configuring MODEM.CFG:

The first of two configuration files is called "MODEM.CFG".

This specifies the following information in the following format:

Line 1:  COM port

Line 2:  Baud rate value

Line 3:  Modem init string 1

Line 4:  Modem init string 2

Line 5:  Modem command: pick up line

Line 6:  Modem command: hang up

Line 7:  Modem command: signal tone

Line 8:  Modem command: code received tone

Line 9:  Modem command: ring tone

Line 10: Modem command: Touch tone recognition mode - IMPORTANT

Line 11: Answer ring number

Line 12: Ring wait

Line 13: Tone detect wait

Line 14: Ring tone frequency

Line 15: Number of rings

Note: These entries are described in detail.  Example

configuration (for a US ROBOTICS DUAL STANDARD modem) is included

at the end for further clarification.  If you don't know what to

enter for data, try using what the example uses, chances are it

will work.

1. COM port:  <number>

This is very straight forward.  If you don't get this, you

probably won't understand the other commands.  Just put the

number of the COM port that your modem is using

-----------------------------------------------------------------

2. Baud rate value: <number code 5-9>

This REQUIRES a CODE number that corresponds to the baud

rate that your modem will have.

Baud rate CODE                      Actual baud rate

5 .................................... 9600

6 .................................... 19200

7 .................................... 38400

8 .................................... 57600

9 .................................... 115200

-----------------------------------------------------------------

3. to 10. are modem commands (i.e. those preceded by "AT"

on a Hayes compatible modem). These will be strings (text),

not numbers.

-----------------------------------------------------------------

3. Init string 1: <modem command>

This string is just to initialize your modem and its

contents are completely up to the user.  However, resetting

the modem is recommended ("ATZ")

-----------------------------------------------------------------

4. Init string 2: <modem command>

This is another initialization string.  This is here is because

the user will probably want to completely reset the modem

("ATZ") in Init. string 1, then place their own init string here.

This may include any initialization (a useful one is to turn

the modem speaker off).

IT IS MANDATORY THAT THIS INIT STRING INCLUDES A COMMAND TO

PREVENT DIAL TONE ERROR CHECKING:  Many modems will give an error

"NO DIAL TONE" when picking up the line to dial.

-----------------------------------------------------------------

5. Pick up line: <modem off hook command>

This command is used to answer the call ("ATH1" may work).

-----------------------------------------------------------------

6. Hang up:  <modem hang up command>

Simply does what it says.  (Many modems will accept "ATH0").

-----------------------------------------------------------------

7. Signal tone: <modem dial command>

This tone is achieved by simply telling the modem to dial

a few numbers (usually with the "ATDT" command).  This will

signal the caller to enter their code.  ex. "ATDT**;" You

may notice the ";" at the end of that command and you also

may not understand it.  This is another MANDATORY COMMAND.

This addition to "ATDT**"  Prevents the modem from hanging

up when it doesn't get a "CARRIER" after "dialing".  THIS

OR A SIMILAR COMMAND MUST BE INCLUDED FOR THE SOFTWARE

TO WORK.

Note:  This tone is also used to signal the caller

that the modem is about to hang up.

-----------------------------------------------------------------

8. Code received tone:  <modem dial command>

This works the same as signal tone, but this tone is sent when

the caller presses pound (#), to let the caller know that their

code has been understood.

-----------------------------------------------------------------

9. Ring tone: <modem dial command>

This signal tone alerts the caller while waiting for the system

operator to answer (after a valid code has been entered).

ex. "ATDT***************;" NOTE THE ";"

(Called ring tone because the caller hears it while waiting for the

system operator to answer as if the modem was acting like a ringing

phone).

-----------------------------------------------------------------

IMPORTANT NOTE: It is HIGHLY RECOMMENDED that ONLY the pound

("#") or star ('*') keys be specified for ANY TONE SENT BY THE

MODEM:  On the off chance that there is a dial tone when the

modem attempts to send a tone, your modem will be dialing

somewhere, possibly long distance if the tone "1" is sent first.

-----------------------------------------------------------------

10. Touch tone recognition: <modem command to recognize tone>

This is obviously the key command.  TTCC revolves around the

ability of YOUR MODEM to recognize touch tone.  This command

should initiate the touch tone recognition process.  On USR

modems it may be "AT%T".

-----------------------------------------------------------------

11. to 15. are personal preference configurations

-----------------------------------------------------------------

11. Answer ring number:  <number>

This variable specifies the number of "RING"s that the modem

should wait for before picking up the line.

-----------------------------------------------------------------

12. Ring wait:  <number>

The variable specifies the number of seconds to wait before

resetting the ring count.  This is necessary if the line rings and

the caller hangs up before the modem answers.  A good value is 10.

If this value is too small, your modem will hang up after

the first ring.

-----------------------------------------------------------------

13. Tone detect wait:  <number>

This variable is the number of seconds before the caller will be

hung up if they have been idle.  Each key press will reset this

timer.

-----------------------------------------------------------------

14. Ring tone frequency: <number>

This variable is the frequency of the tone used to alert the system

operator.  A good value is 400.

-----------------------------------------------------------------

15. Number of rings: <number>

The number of times the system operator will be paged by a ringing

tone. (frequency specified in 14.)

-----------------------------------------------------------------

NOTE: All data that has a "CODE VALUE" must be specified in

"MODEM.CFG" using the CODE VALUE and NOT the actual

value  i.e. for baud rate, use: 7  NOT 38400

-----------------------------------------------------------------

EXAMPLE for "MODEM.CFG":     - DO NOT INCLUDE TEXT IN PARENTHESES

-----------------------------------------------------------------

2 (LINE 1 - COM port 2)

7 (LINE 2 - Baud rate CODE for 38400)

ATZ (LINE 3 - 1st Init. String - resets modem)

ATX5 (LINE 4 - 2nd Init. - sets modem to ignore "NO DIAL TONE")

ATH1 (LINE 5 - Picks up phone line)

ATH0 (LINE 6 - Hangs up phone line)

ATDT**; (LINE 7 - Signal tone)

ATDT#*#; (LINE 8 - Code received tone)

ATDT***************; (LINE 9 - Ring tone)

AT%T (LINE 10 - Enable touch tone recognition)

2 (LINE 11 - Wait two rings before answering)

10 (LINE 12 - Wait 10 sec. to hang up if ring is interrupted)

20 (LINE 13 - Wait 20 sec. to hang up if code interrupted)

400 (LINE 14 - Alert system operator with rings of frequency 400)

4 (LINE 15 - Alert system operator with 4 rings )

-----------------------------------------------------------------

Configuring ID.CFG:

The next and final configuration file is a lot easier.  The

program will run correctly regardless of what is in this file.

*Errors will only occur when one attempts to enter a code.

This one is called "ID.CFG". It can be as long as you want.  This

file decides what to do when certain codes are entered.  Again,

format is important, and an example file is included at the end.

The file is composed of groups of two text lines.

The first line includes an access code and a command.   When a

caller enters their access code (of any length), TTCC will check

the ID.CFG file for the code.  If the code is not in the file, TTCC

will hang up on the caller.  Otherwise, the command right next to

the code will be executed.  The second line is a comment line, and

MUST COME DIRECTLY AFTER THE CODE/COMMAND LINE.  You may place any

text in it to be displayed on the screen (in the "comments" window)

when someone enters the corresponding code.

NOTE:  Even if you don't want a comment, you MUST LEAVE A BLANK

LINE AFTER the code/command line.

NOTE2: You may include any text that you want in ID.CFG as a note

to yourself as long as NONE OF THE TEXT CONTAINS a pound (#) symbol

and that the text is NOT BETWEEN a CODE/COMMAND line and its

COMMENT line.

Since the caller enters their code as follows:  "007123#"

with a pound  symbol ("#") to terminate the code, the file must

contain this exact string (text) as follows:

<code>#    <command anywhere on this line>

<comments>

Commands:

There are three basic commands that TTCC can carry out.  One

of them occurs when ONLY the access code is specified in

"ID.CFG".  This command is an INTERNAL command.  It will not

execute any external programs, but it alerts the system operator

with a ring (mentioned earlier).  This is specified by

the following line of text:

<code>#

<comments>

ex.:

02313#

Talk to me, hoser!

Remember to keep the rest of the command line blank.

The most powerful command is the external command.  TTCC can

execute a program after the caller enters a code.  To use this

command, write the code in the ID file and specify the FULL PATH

AND FILENAME of the file to be executed.  Command line parameters

are not allowed, but running a batch file which executes your

program will work instead.  The path and filename may be anywhere

ON THE SAME LINE as the related access code.

<code>#    <Path and filename>

<comments>

ex.:

008123#      C:\DOS\BOOT.COM

REBOOTING YOUR COMPUTER!

NOTE:  Regardless of whether the file is in your path, you still

must include the path along with the file name and extension.

The third command is simple.  It will exit TTCC.  If this code is

entered, TTCC will terminate, and no further calls will be

answered. It is as follows, also anywhere on the same line as the

access code:

<code>#     HALT

<comments>

ex.:

911#          HALT

It's exiting, mon

Now for an example "ID.CFG" file:

-----------------------------------------------------------------

000#      HALT

Exiting TTCC!

12345#

Answer the phone!

5*5*5*5#    C:\BAT\WOW.BAT

Running WOW.BAT

-----------------------------------------------------------------

NOTE: The blank lines in between entries are unnecessary.  ANY

lines (regardless of whether they contain text) that are

between entries will be ignored AS LONG AS THEY DO NOT HAVE

THE POUND (#) SYMBOL

-----------------------------------------------------------------

That's it for configuration. . .  Finally.  Hopefully, your

computer/modem setup is compatible with TTCC, because if it

isn't, after all that configuring, I'm sure you're going to be

pissed off at me.

-----------------------------------------------------------------

Extra Notes:

To run TTCC, be sure the two configuration files are in the

same directory as the EXE file "TTCC.EXE" , with your modem on,

type  "TTCC" at your DOS prompt.

If the TTCC display screen shows up, but it doesn't look

like anything is  happening, either your system isn't compatible

or something in MODEM.CFG is wrong.  If it looks like the program

is sending init strings to the modem you're in luck!  The

software will probably work correctly from there on.

HINTS:

Modem output MUST be in verbose form.

The star ("*") character may be used as part of an access code.

-----------------------------------------------------------------

Contacting the author:

If you have any questions, comments, complaints, or

suggestions, feel free to get a hold of me.  I'd really like

to hear whether anyone got this to work on their system, and

whether they thought it was worth it after they did.  Remember

THIS IS A BETA VERSION and things are BOUND to go wrong.  If you

notice anything, contact me and I will attempt to fix it.  If you

would simply like to see an updated, IMPROVED version, tell me

about it, and if enough people want to use TTCC, I will improve

it.

I can be reached under the name JONAH PESKIN on the

following BBS:

Ideal Studies BBS

Worcester, MA

[508]-757-1806

1200/2400/9600 HST

-----------------------------------------------------------------