Documentaion for SBDSP - SoundBlaster APT Demodulation Program

This program and documentation are the property of the Dallas Remote Imaging

Group (BBS 214 394 7438).   If it is used on other systems, this copyright

notice must be kept in this documentation.   This program may NOT be sold or

marketed in any form.  All rights are reserved by the Dallas Remote Imaging

Group (1992).

Dallas Remote Imaging Group

PO BOX 117088

Carrollton, Texas  75011-7088

BBS 214-394-7438

FAX 214-492-7747

Rev DRG5193Beta

----------------------------

Introduction:

The SoundBlaster and SoundBlaster Pro audio I/O cards from Creative

Labs, Inc.  are very popular accessories found in many IBM PC compatible

computer systems.  Although the boards are usually used only to provide

music and sound output for computer games, they also support audio input

into the computer.  The standard SoundBlaster supports mono audio I/O

and is available in the $100 price range.  The Pro model supports stereo

and adds a CD/ROM (SCSI) interface.  It costs around $250 dollars.

The boards can digitize sound at high speed and, using one of the

computer's DMA (direct memory access) channels, transfer the sound

samples directly into computer memory.	Once in memory the sound samples

are written to disk.  It is this ability to digitize and record sound

that enables the SoundBlasters to be used for the reception and decoding

of APT satellite signals.

The output of a radio receiver tuned to an APT signal is just a 2400 Hz

audio tone.  As the satellite scans the earth and clouds,  it varies the

amplitude of the tone in proportion the the brightness of the area that

it is currently observing.

-------

What it does:

If you viewed the contents of the .VOC sample file produced by the

SoundBlaster VREC program with your favorite image processing program,

it would not look at all like a weather satellite photo.  The .VOC file

is like an exposed roll of film.  Its contents must first be "developed"

before you can view the image.	The process of "devloping" the .VOC

file contents into a weather satellite image is called DEMODULATION.

Demodulation converts the recorded values of the samples of the 2400 Hz

APT sine wave into their corresponding pixel values.  The desired image

pixel values actually represent the amplitude of each peak in the

APT sine wave signal.

Signal sampling theory states that if you are to do anything meaningful

with a digitally sampled signal then you must sample the signal at least

twice as fast as the highest frequency component of the signal.  The

2400 Hz APT signal needs to be sampled at least 4800 times a second.

Because the APT signal is not a constant tone,	it has freqency

components much beyind 2400 Hz.  Also the laws of thermodynamics (and

life) say that nothing is perfect as theory predicts and you really need

to sample at a higher rate.

If we sample the APT signal at exactly 4 times its 2400 Hz

carrier frequency (i.e at 9600 samples per second), each input sample

will be eactly 90 degrees (360/4) in phase apart from its adjacent

samples.  Using the following trigonometry identities it turns out to be

particularly easy to accurately demodulate the APT samples:

cos(x) = sin(90-x)

sin(x)*sin(x) + cos(x)*cos(x) = 1

By substituting the first equation into the last equation we get:

sin(x)*sin(x) + sin(90-x)*sin(90-x) = 1

Because each recorded sample of the 2400 Hz APT sine wave is 90

degrees apart from the next sample we can calculate the pixel value as:

pixel = sqrt((sample[t]*sample[t]) + (sample[t-1]*sample[t-1]))

The demodulation program just needs to read the .VOC sample file, throw

away the file header information,  adjust the samples for clock/DMA

rate errors and DC offsets, calculate the pixel values from adjacent

pairs of sample values,  scale the results to values from 0 to 255, and

finally write the resulting pixel values to the image file.

This could be a very time consuming process when you consider that a

typical 14 minute capture has about 8 million input samples and that

multiples, adds, and square roots are all operations that take lots

of CPU time.  This is particularly true if floating point arithmetic is

used.

To make the demodulation program run very quickly a table of all 65536

possible combinations of adjacent 8-bit sample values is pre-computed

according to the above formula.  All the demodulation software then has

to do in order to convert the recorded APT samples into their pixel

values is look up the proper table entry.  No time consuming floating

point arithmetic need be used once the demodulator table has been built.

Reading and writing 8 megabyte files is always a time consuming process.

By always reading and writing the sample and image files in multiples of

512 bytes, the demodulation software takes full advantage of the

behavior of the DOS file system.

-------

Problems,  problems,  problems:

Although the SoundBlaster is quite good at adding a little zip to the

odd computer game,  it is far from being a precision signal processing

system.   Using the SoundBlaster card for APT reception presents many

problems.  Fortunately software techniques may be used to overcome

the problems and yield APT images that rival the best commercial

systems.

Using the SoundBlaster for APT demodulation presents two main types

of problems:

1) Sample inaccuacies and offsets

2) Clock rate and missing sample errors

Sampling a perfect sine wave with a perfect board in a perfect world

should yield a series of samples with an average sample value of zero.

Unfortunately the SoundBlaster circuitry usually adds a small DC offset

to the recorded sample values.	This DC offset can cause noise and

"beat" patterns to appear in the demodulated image file.

To remove the effects of any DC offset in the sample values, the

demodulation software calculates the average sample value in each block

of input samples.  This sample bias is then subtracted from each input

sample.

The demodulation program is currently a bit paranoid about things and

re-calculates the sample bias for each input block.  It would be a

bit faster to calculate the value for only the first block.  If the

input level to the SoundBlaster board is properly adjusted, the sample

value offset does not seem to change between blocks.

For easy demodulation of the APT signal we need to sample at 9600

samples per second.  If the resulting image is to be properly

synchronized to the satellite data this needs to be EXACTLY 9600 samples

per second.  PRECISELY EXACTLY 9600 samples per second.  How exact?

A typical fourteen minute satellite pass requires an accuracy of around

one part per 10 million...  9600.0000 samples per second.

Typically a satellite scans one line of pixels every 1/2 second (120

lines per minute).  At a sample rate of 9600 samples per second, a 14

minute satellite pass generates about 8 megabytes of data.  If the

sample clock rate is only accurate to a mere one part per million, the

resulting image will be skewed 8 pixels from top to bottom.  Although 8

pixels of skew across of screen of 4800 pixel lines is only about a .1

percent skew, it is noticeable if you are looking for it or are wanting

to do some quantitative analysis of the satellite orbit from the doppler

bow.

How accurate is the SoundBlaster clock you ask?  It turns out the

closest that you can set the sample rate is around 9620 samples per

second...  an accuracy of around one part in a few hundred...  not even

close.	To make matters worse the clocks on any two SoundBlaster boards

can easily differ by parts per thousand.  Clock errors of this magnitude

cause the image to appear like a TV set that has lost sync.  The image

is an unreadable mess of slanted bars.

The DMA circuitry on the SoundBlaster boards can automatically transfer

blocks of up to 32768 signal samples into the computer's memory without

using any CPU time.  Once a block of samples has been DMAed into memory,

the recording software starts DMAing the next block of samples into

another memory area.  While the second block of samples is being

received, the recording software writes the first block to the

computer's hard disk.  The process alternates in a ping-pong fashion

between reading samples into one buffer while writing the other buffer's

data to disk.

Unfortunately a few signal samples are lost in the time between the end

of one DMA block and the startup of the next DMA block.  To make matters

worse the number of samples lost depends on the computer system's CPU

type and clock rate.  Faster systems loose fewer pixels between blocks.

Also number of pixels lost seems to depend upon whether the block is an

ODD or EVEN numbered block.

Fortunately it is possible to compensate for the errors in the clock

rate and the samples lost between DMA blocks.  The demodulation program

compensates for the pixels lost betwwen DMA blocks by duplicating the

last few samples of each DMA block.  The user can independently specify

the number of samples to add to (or even remove from) the odd and even

numbered DMA blocks with the /O and /E command line parameters.

The sample clock rate error is compensated for in two stages.  The first

stage is the coarse adjustment factor.	This factor causes every n'th

input sample to be removed (or duplicated).  By throwing away

every 601'st sample, the 9620 samples per second that the SoundBlaster

produces is cut down to ABOUT 9600 samples per second.	The coarse

adjustment factor can be changed by using the /C command line parameter.

The second clock rate adjustment factor is the fine tuning factor.  It

is used to zero in the exact timing value.  It adjusts out any

residual skew in the image caused by variations in individual

SoundBlaster board clock generators.  The fine adjustment factor

works just like the coarse adjustment factor.  It causes every n'th

input sample to be removed (or duplicated).  It may be specified with

the /F command line parameter.

-----------------------------

Doing it:

The first step is to connect your satellite receiver to the microphone

input of the SoundBlaster.  You must be very careful to set the

amplitude of the receiver output low enough so that it does not

overdrive the SoundBlaster.  If you overdrive the card its automatic

gain control (AGC) circuitry will attempt to compensate.  This will

cause streaking in the resulting images.

To properly set the receiver level use the VEDIT2 program supplied with

the SoundBlaster card.	Select the SCAN INPUT item from the RECORD menu.

This will show a graphical representation of the input signal.	While

monitoring a good satellite pass adjust the receiver level until none of

the signal peaks touch the top or bottom margins of the graph.	Leave a

little safety margin (1/8 inch) just to be safe.

Once the receiver level has been set,  the next step is to record a

satellite pass to disk.  This is done with the VREC program supplied

with the SoundBlaster card.

For the SoundBlaster Pro use the command:

vrec c:cap /a:mic /m:mono /f:low /s:9620 /l:15 /t:840

where:

c:cap - is the name of the file that you want to write the samples.

Remember the sample file will be quite large.  Around 600 Kb

per minute of recording.  In this case the samples will be

written to the file C:\CAP.VOC

/a:mic  - selects the MICROPHONE input.

/m:mono - selects MONO recording.

/f:low  - sets the LOWPASS recording filter.

/s:9620 - sets the recording sample rate to 9620 samples per second.

Note the if you ask for values less that /S:9619 the VREC

program rounds the value down to around 9500 samples per

second.  /S:9620 is the smallest value that actually

selects a sample clock greater than 9600 Hz.

/l:15   - sets the microphone level to full open.  Use the receiver

output level control to adjust the recording level.

(I don't think this is what this does or is necessary

any more.	It is left over from early attempts to get

rid of hum in the image caused by a bad cable)

/t:840  - says to record for 840 seconds (14 minutes).  This value

can be adjusted to whatever length is desired.

After a satellite pass has been recorded and written to disk as a .VOC

sound sample file,  the next step is to demodulate the .VOC sample file

into an image file.  Use the command:

sbdsp c:cap.voc c:cap.img

This will read the recorded sample file C:CAP.VOC and write the

resulting image to the file C:CAP.IMG  (remember that these files will

be about 8 Megabytes EACH).  If you do not specify the output file name,

the demodulation program will ask if it is OK to write the output image

over the data in the .VOC file.  This is a VERY time consuming process,

but can be useful if you are short on disk space.  Note that if you

choose to overwrite the input data file,  you cannot experiment with the

various timing adjustments or interrupt the demodulation program.

Once the input file has been demodulated you can use your favorite

image processing program (such as IMDISP, APTCAP, or SATVIEW) to view

the resulting image.  The file written by the demodulation program is

raw binary 8-bit gray scale image.  Each line is 4800 pixels wide.  A 14

minute pass has 1680 scan lines. (Because of the timing adjustments, etc

the output image may be short a few lines).

---

Tweaking in the timing:

What you should see is the APT image complete with the normal "doppler

bow" curve.  The curve in the image is due the speed of light.  Since

light travels at 186,000 miles per second, each pixel of image shift in

the doppler bow (1/9600 of a second) represents about 19.4 miles of

satellite movement relative to the receiver.

As the satellite approaches the receiver the radio signal has to travel

less and less distance to reach the receiver.  As the satellite moves

away the radio signal takes longer and longer to reach the receiver.

The point in the doppler bow where the curve changes slope

is the closest point of approach of the satellite pass.

In a properly adjusted image,  the shape of the doppler bow curve

should appear symmetrical about the closest point of satellite approach.

If the image is slanted more towards the left or right, or appears like

a TV image that has lost sync then you will need to adjust one or

more of the timing adjustment factors.

(For further information, contact the software developers at:

Dallas  Remote Imaging Group BBS - 214-394-7438  FAX 214 492 7747

Usually only the fine timing adjustment (/F) needs to be set.  To get an

idea of the magnitude of the adjustment needed, count or estimate the

number of pixels that the image slants.  (Hint:  use the pan and zoom

features of your image display program.  If your display program

re-sizes the image to fit the screen, remember to count pixels in the

FILE, not those on the screen).  Divide the total file size by the

number of pixels that the image slants from top to bottom.

adjustment factor = (file size / pixels of image slant)

The more the severe image slant is, the SMALLER the timing adjustment

factor value.  Subtle image slants are typically corrected with large

valued adjustment factors (say 50,000 - 500,000).  Adjustment factors in

the thousands would be used for images that slant a good portion of the

width of the screen.  Even more severe slants first should be corrected

by tweaking the /C (coarse) timing adjustment.

Images that slant to the left use a positive adjustment count.

Images that slant to the right use a negative adjustment count.

->|      |<- estimate the number of pixels of image slant

|      |

|       ----------	    ----------

|     /	       /	   \	       \

|   /  image   /		     \	 image	 \

| /	   /		       \	   \

| ----------			 -----------

Use /F+value			Use /F-value

The loss of pixels between DMA sample blocks shows up as small

periodic jags in the image of a pixel or so every 6-8 scan lines.

Experiment with the /O and /E command line options may be to minimize

these remaining jaggies.  For these options,  larger values produce

greater effect.  The maximum DMA adjustment factor possible is +/- 255.

Only + values should be needed here.  The software supports - values

just in case...

The image may look OK except for one or more large and sudden shifts or

discontinuities.  These shifts usually occur on systems with slower hard

drives or CPU's.  The cause is usually the excessive time it takes DOS

to extend or otherwise allocate disk space as it writes the .VOC file.

If this occurs in your images, it can usually be fixed by recording the

sample file to a freshly "defragmented" disk.  Use a disk optimizer

program like Norton Utilities or PC Tools to pack the sample file hard

disk drive before running the VREC recording program.

Once the proper timing adjustment factors have been determined for a

particular computer, they should not need to be changed (except possibly

for a drift in the SoundBlaster sample clock rate with temperature).

A batch file can be created that will automatically use the appropriate

timing factors.

--------

Development team at:

Dallas Remote Imaging Group

PO BOX 117088

Carrollton, Texas 75011-7088

FAX 214 492 7747

================================================================================

============================================================================

DALLAS REMOTE IMAGING GROUP BBS    =====>>  214-394-7438

FAX             214-492-7747

VOICE             214-394-7325

DEDICATED TO SATELLITE IMAGERY, SATELLITE TRACKING, NASA,

DIGITAL SIGNAL PROCESSING, IMAGE PROCESSING,

AMATEUR RADIO, AND ELECTRONIC INTELLIGENCE

PCBoard SUPPORTS 6 Lines (USR Dual Standard V.32bis V.42 24 hrs. Daily)

E-MAIL FOR Weather Satellite Report and InterNet UUCP News Groups

ON-LINE HAMCALL APPLICATION - LOOKUP HAM CALLSIGNS

On-Line SATTRAK Satellite Tracking Routines

On-Line SHOP Satellite Resource Center - On-line Ordering

On-Line BALLTRAK  Balloon Launch and Tracking Program

THOUSANDS OF IMAGE FILES OF SATELLITE IMAGE AND NASA VOYAGER IMAGES

SATELLITE TRACKING PROGRAMS, IMAGE PROCESSING, DSP, ATV, SSTV, FAX

============================================================================