WeatherFAX  V3.0

Weather Facsimile for IBM and

Compatible Personal Computers

USER'S GUIDE

Third edition

February 1991

Copyright 1990 Jerry Dahl

6404 Lakerest Court

Raleigh, NC 27612

TABLE OF CONTENTS

�����������������

GENERAL INFORMATION ............................ 3

Changes in Version 3.0

Introduction

Purpose and Goals

License information

The Software

The Author

SUPPORTED CONFIGURATIONS ....................... 5

Hardware

Software

INSTALLATION INSTRUCTIONS ...................... 6

Installing the WeatherFAX card

CONFIG.SYS Modifications

AUTOEXEC.BAT Modifications

Copying the diskette

Starting the Program

GETTING STARTED ................................ 8

Getting help information

Testing the card

Selecting your printer type and display mode

Connecting your computer to your fax receiver

USING WeatherFAX ............................... 11

Adjusting the page alignment

Getting the image

Viewing the image

Printing the image

Saving the image to disk

ADDITIONAL INFORMATION ......................... 20

Calibrating the card

Using Expanded Memory (LIM 4.0)

Using Super VGA (800 X 600 Pixel) mode

Sources for weather facsimile pictures

WeatherFAX program specifications

IF YOU HAVE DIFFICULTY ......................... 25

ADAPTER/SOFTWARE AVAILABILITY .................. 27

TRADEMARKS USED IN THIS GUIDE .................. 27

GENERAL INFORMATION

�������������������

CHANGES IN VERSION 3.0

This version (V 3.0) of FAX.EXE contains many improvements

over the original (V 2.0) and is a direct result of user

feedback. My goal is to make this the best weather-facsimile

package for professional users -- regardless of cost.

I believe that this goal has been accomplished largely with

this version.

The new features in Version 3 are:

VERY HIGH RESOLUTION GRAPHICS. New graphics drivers are

added for 1024 x 768 x 256 color resolution and lower.

This resolution is essential for viewing large Polar and

GOESTAP images.

Graphics cards that use Tseng ET3000/4000 and ATI chip

sets are supported. These cards include Orchid ProDesigner,

Genoa Super VGA, Boca Research SVGA, Sigma VGA/H, STB

VGA Extra, Everex Viewpoint VRAM and the ATI VGAWonder+.

The Everex Viewpoint VRAM card has been fully tested at

1280 x 1024 x 16 color resolution.

Here is a list of the new graphics drivers and the chip

sets supported:

1024  x  768 x 256 colors   (Tseng 4000)

800  x  600 x 256 colors   (Tseng 3000/4000, ATI)

640  x  480 x 256 colors   (Tseng 3000/4000, ATI)

1280  x 1024 x 16 colors    (Tseng 4000)

1024  x  768 x 16 colors    (Tseng 3000/4000, ATI)

Drivers for Paradise and Video 7 chip sets are also included

on an experimental basis. They have not been tested by

the author.

MORE ENHANCEMENT OPTIONS:

All 20 NOAA/NESDIS image-enhancement curves are included

as an aid in analyzing infrared images. Also included

are Log-Black and Log-White curves for analyzing

visible-light images. You can shift and scale the curves

to better fit the curve to the image.

PALETTE SPAN--Interpolation allows you to set a few

color palette levels and have the software fill in the

in-between values.

SKIP LINES--This option reduces the size of HF Fax and

GOESTAP images. Skipped lines are not lost, but are

averaged with saved lines.

AUTOMATIC FILE SAVE--Images are continually saved to

disk using a time stamp as the file name.

SCHEDULE-TIMER FOR UNATTENDED OPERATION--Specify the

starting time and duration to automatically capture

images. This is needed for capture polar-orbiting

satellites that do not transmit start and stop tones.

You can specify the times from either a pull-down menu

or as a command-line option.

GOESTAP MODE--Allows you to capture GOESTAP images.

USER-SPECIFIED SPM--Specify any scan-per-minute value

and capture images using nonstandard SPM rates. You can

now capture the Meteor-satellite images at 20 SPM and

wire photos on C-band that use nonstandard rates.

MIRROR IMAGE--Reverse right-to-left wire photos that are

sent as mirrored images.

BETTER PRINTER/FILE SUPPORT--You can now print and save

images when they are reduced using the 50-percent Zoom

feature. This allows you to print more of the image area

and reduce the image file size.

OTHER MISCELLANEOUS CHANGES:

-  The 256-color modes have finer scrolling steps that allow

you to more-accurately align the image on the display.

This is useful for saving the images and using other

software to "loop" the saved images.

-  RTTY support is removed.

-  The command line /Hxx parameter that specifies the

800 x 600 x 16 color mode is no longer needed and

should not be used if your card uses either the Tseng

or ATI chip set.

-  24-hour time is used in lieu of 12-hour AM/PM time.

-  The screen is cleared as each new image is acquired.

INTRODUCTION

These instructions explain how to install and use the

WeatherFAX weather scan converter and software. You may

print this guide with your printer by typing the

following command from the DOS command line:

COPY FAXGUID3.DOC LPT1

This scan converter and software is used with your IBM or

compatible personal computer. With the proper receiving

equipment and antenna, your computer will be able to

capture, enhance, save and print--not just black and white--

but gray-scale weather-facsimile images received on the HF,

VHF and UHF bands. Users of this Automatic Picture

Transmission (APT) imagery include military and commercial

meteorologists, ships at sea, scientific observers,

educators, and amateurs.

This package was described in a two-part feature article in

QST magazine. See "A Weather-Facsimile Package for the

IBM PC," April and May 1990.

The WeatherFAX software reads and writes GIF and TIFF

format image files and displays them in CGA, EGA, MCGA,

VGA and SVGA graphics modes. Resolutions up to

1024 x 768 x 256 colors and 1280 x 1024 x 16 colors are

supported. You can enhance the imagery and adjust the

color palettes. You can also print up to 16 gray shades

on 9- and 24-pin dot matrix and laser printers. When

used with the WeatherFAX scan converter, the program

allows capture of all APT modes: HF fax, GOESTAP, polar

and GOES satellites. The image size is limited only by

the available memory in your computer: Up to 32 Mb of

EMS memory is supported. Additional enhancements are

being developed that will offer even higher-resolution

graphics, more operating modes and enhancement options.

The system features include:

o Direct weather-satellite imagery, including: geo-

stationary earth-orbiting satellites (GOES APT/WEFAX,

METEOSAT), polar-orbiting satellites (NOAA 9/10 APT

IR/Vis, Soviet Meteor), HF-FM NAFAX, and similar APT

formats.

o Expanded memory support. You can capture images with

a wide variety of resolutions. The size of the

captured image is limited only by the amount of RAM

in your computer. To view images larger than your

monitor can display at one time, you can scroll up,

down, left, and right.

o Incoming pictures can be saved directly to disk.

Either the Tag Image File Format (TIFF) or Graphical

Interchange Format (GIF) can be selected to allow the

images to be used with other graphics-based software.

o Print images with 16-level gray scale on a wide

variety of printers including 9- and 24-pin dot-

matrix printers and laser printers.

o Very high resolution. All standard graphics cards are

supported including CGA, MCGA, EGA, VGA, and

Super VGA(800 x 600) mode. Also for Tseng and ATI

chip sets resolutions up to 1280 x 1024 are

supported.

o Histograms, enhancement, zoom, scroll, gray scale,

256,000+ colors, interpolation, reverse, real-time

acquisition/display. NOAA/NESDIS and logarithmic curves.

o Automatic gain-lock circuitry. The optimum gain

value for WEFAX-signal reception is statistically

calculated. This provides better and more consistent

pictures than either automatic gain control (AGC) or

fixed-gain circuits.

o No need for external test equipment to verify, test and

calibrate the card. The software and special

circuitry on the board allow the computer to automate

the testing and calibration.

o Unattended operation. Image capture can be fully or

partially automatic, including APT start tone detect,

sync, image save to disk, APT stop-tone detect and

cycle repeat.

LICENSE INFORMATION

Your right to use WeatherFAX

This software is copyrighted in order to control its

orderly development. However, you have the right to

use the program for private, noncommercial purposes

without further permission. That is, you can copy it,

use it, and share it so long as you do not charge any

money for it other than the cost of the disk or other

transfer medium, not to exceed $2.

THE SOFTWARE

The development of the program began in early 1986, and has

gone through several revisions. New versions will be

released as features are added and corrections are made

to previously released versions. Ideas for future

enhancement include:

- Higher resolution graphics drivers

- Direct AVHRR temperature calibration for polar and

GOESTAP images.

- Looping of images for animation

- Gridding for polar satellite images

I am also considering modifying the software to support

a very high resolution VAS (Visable and Infrared Spin Scan

Radiometer/Atmospheric Sounder) digital scan converter

for Modes AAA and GVAR format images from GOES satellites.

This system would feature a very high resolution visible channel

and 20 IR channels from the Atmospheric Sounder instrument on board

the spacecraft.

Please let me know what new features are needed for this

program to suit your needs.

THE AUTHOR

The author can be contacted by mail at the following

address:

Jerry Dahl

6404 Lakerest Court

Raleigh, NC 27612

For a mail reply, please enclose a self-addressed,

self-stamped envelope.

Credit must also be given to the many users who have

made suggestions for improving the program. Most of

their suggestions have been incorporated into this

version of WeatherFAX software.

SUPPORTED CONFIGURATIONS

������������������������

MACHINE REQUIREMENTS

WeatherFAX supports most IBM compatible computer systems

from the least to the most powerful.

The ideal system for a professional user would have a

minimum of a 80386 processor and 4 MB of RAM.	The graphics

card would contain at least 1MB of RAM using the Tseng 4000

chipset.  A math co-processor is not required.

A typical system would have a 80286 or better processor,

640 KB of conventional memory and a SVGA graphics card.

A low end system would have an 8088 processor, 512 KB

of conventional memory and CGA or better graphics.

o WeatherFAX Card

o IBM PC or compatible computer that uses the standard

PC-bus architecture.

o RAM (Random Access Memory):

- 512 kbytes or more of memory:

In most configurations, approximately 190 kbytes of

memory is required for DOS, drivers loaded by

CONFIG.SYS and the WeatherFAX program. All of the

remaining memory is available for image storage.

- Expanded Memory (optional)

LIM 4.0 expanded memory allows up to 32 Mbytes for

image storage. Requires both expanded memory and a

LIM 4.0 software driver.

o A graphics monitor and video adapter card including

- Color/Graphics Adapter (CGA)

- Enhanced Graphics Adapter (EGA)

- Multi-Color Graphics Array (MCGA).

- Video Graphics Array (VGA)

- Super VGA (up to 1280 x 1024-pixel resolution)

o One diskette drive (one of the following):

- 5.25-inch drive (360 kbytes)

- 3.5-inch drive (720 kbytes).

o Fixed disk (optional)

o Printer: (optional)

Printer types and print resolution - dots per inch

(dpi)

���������������������������������������������������

- All standard 9-pin dot-matrix printers  (120 dpi)

- All standard 24-pin dot-matrix printers (180 dpi)

- IBM Pageprinter 3812	      (240 dpi)

- PCL command compatible (laser) printers (300 dpi)

o Receiving equipment and antenna for the frequencies

you want to cover.

PROGRAM REQUIREMENTS

To run the WeatherFAX software, you need:

o IBM Personal Computer Disk Operating System (DOS)

Version 3.0 or later

INSTALLATION INSTRUCTIONS

�������������������������

INSTALLING THE WEATHERFAX ADAPTER

Refer to the Guide to Operations for your computer for

instructions for installing cards. You will need to

turn the computer off and remove the cover from the

computer to insert the fax card into an unused card

socket.

CONFIG.SYS MODIFICATIONS

No changes to CONFIG.SYS are normally needed. However,

if you ever get the error message "DRIVE NOT READY" when

retrieving very large files (1 Mbyte or larger), the

reason may be the assignment of an insufficient number

of DOS buffers. Try increasing the number of buffers by

adding the statement BUFFERS=64 to the CONFIG.SYS file.

AUTOEXEC.BAT MODIFICATIONS

NOTE FOR USERS HAVING THE ORIGINAL IBM CGA DISPLAY

ADAPTER AND SOME VERY EARLY CLONE EGA CARDS (does not

apply for most VGA or EGA displays).

In those cards, the graphics adapter ROM does not

include the entire graphics character set. You will need

to load the DOS graphics-character-set driver

GRAPHICS.COM into memory. The driver is available on the

DOS diskette. Add the statement GRAPHICS.COM to your

DOS AUTOEXEC.BAT file to load the driver every time you

start your computer. If GRAPHICS.COM is in a

subdirectory, supply the necessary path information on

the driver statement. For example: C:\DOS\GRAPHICS.COM.

When using this program, make sure your AUTOEXEC.BAT

file is "clean."  Some terminate-stay-resident (TSR)

programs--especially those that control screen functions

(such as screen blanking) may interfere with the proper

operation of the software. If you experience difficulty

when running the fax software, check your AUTOEXEC.BAT

file first.

COPYING THE DISKETTE

Copy the WeatherFAX diskette and save the original

diskette in a safe place. You can copy the diskette

to either a hard disk or to another diskette. Refer

to your DOS manual for instructions for copying

diskettes. The WeatherFAX program is not copy

protected.

STARTING THE PROGRAM

Start the WeatherFAX program by typing FAX.EXE from

the DOS command line.

Optional Command Line Parameters:

�����������������������������������������������������Ŀ

� Syntax: FAX [options]   where options are:	       �

�						       �

� /C	  Use conventional memory.		       �

�						       �

� /E	  Use Expanded (EMS) memory if installed       �

�	   (Requires LIM 4.0 driver)		       �

�						       �

� /FG	  Default to GIF file format		       �

� /FT	  Default to TIFF file format		       �

�						       �

� /Hxx   BIOS display mode for 800 x 600 x 16 mode    �

�	 (do not use for ATI or Tseng chip sets)       �

�						       �

� /Thh:mm,dd  Automatic acquire for polar satellites. �

�	 Go into acquire mode, wait until	       �

�	 hh:mm (24-hour time), acquire image for       �

�	 dd minutes, save file then exit to DOS.       �

�						       �

� /Whh:mm,dd  Automatic acquire for GOES satellites.  �

�	 Go into acquire mode, wait until	       �

�	 hh:mm, then wait for Start tone,	       �

�	 acquire until either Stop tone or dd	       �

�	 minutes after hh:mm, save file, then	       �

�	 exit to DOS				       �

�						       �

� The /T and /W parameters allow you to write DOS     �

� batch files to automatically capture many images    �

� at various times during the day.		       �

�   hh:mm = starting time in 24 hour time	       �

�   dd    = capture duration in minutes	       �

�������������������������������������������������������

NOTE: The program will automatically use EMS memory if

the available EMS memory size is greater than the

conventional memory size.

Examples:

C:>FAX /H52

Enables 800 x 600-pel resolution mode for Oak

SVGA graphics card. See the section:

Using Super VGA (800 X 600 Pixel) mode.

GETTING STARTED

���������������

GETTING HELP INFORMATION

Help information is available anytime you see F1=Help

displayed on the screen. You can access Help by pressing

the F1 key. Press the Esc key to quit viewing Help.

TESTING THE ADAPTER

Test the fax adapter by using the Test option on the

Main Menu. The various tests will indicate whether

the adapter is operating within its correct operating

region. If any of the tests indicate that the adapter

needs calibration, refer to the section: Calibrating

the adapter.

Static Test

This test is used to check for normal operation and

calibration. The nominal parametric values on the

adapter are displayed. All of the marks should be in

the OK region.

Dynamic Test

A test signal is applied to the input of each function

and the output is checked for correct operation. The

Start and Stop Detectors are tested for both ON and

OFF states. The FM and AM Detectors are tested to

verify linear operation.

AM Graph Test

A test signal is applied to the AM Detector at various

amplitudes and the output linearity is displayed. The

graph line should be straight and sloping upward.

FM Graph Test

A test signal is applied to the FM detector at various

frequencies and the output linearity is displayed. The

graph line should be straight, sloping upward, and

centered between 0 and 256.

SELECTING YOUR PRINTER TYPE AND DISPLAY MODE

Go to the fax menu to specify the Display Mode and

Printer type.

You can select a compatible printer type from the list

of supported printers. Check your printer documentation

to determine which of the supported types that it

emulates.

Printers supported

and print resolution

(dots per inch)

����������������������������������������������Ŀ

� Epson compatible 8-pin printers (120 dpi)    �

�	 - Epson FX, MX, RX, LX series		    �

�	 - IBM Graphics or Proprinter		    �

�	 - Okidata 192I+, 193I+, 292, 293	    �

�	 - Okidata in IBM-compatible mode	    �

�	 - Panasonic KX-P1080i, P1091i, P1093	    �

�	 - Cannon PW-1156A			    �

�	 - Citizen MPS-25			    �

�	 - Star Micronics: Gemini 10X		    �

�						    �

� Okidata compatible 8-pin printers (120 dpi)  �

�	 - Okidata 192, 193			    �

�						    �

� Apple ImageWriter II (160 dpi)		    �

�						    �

� Epson  24-pin compatible printers (180 dpi)  �

�	 - Epson LQ series			    �

�	 - IBM Proprinter X24 and XL24		    �

�						    �

� Toshiba 24-pin compatible printers (180 dpi) �

�	 - Toshiba P321, P341, P351		    �

�						    �

� IBM Pageprinter 3812		  (240 dpi) �

�						    �

� PCL command compatible printers	  (300 dpi) �

�	 - HP LaserJet series II		    �

�	 - HP DeskJet				    �

�	 - Apple LaserWriter II 		    �

�	 - IBM LaserPrinter			    �

������������������������������������������������

NOTE:

If you are using a printer attached to the serial port

(COM1), you need to set up the serial port using the

DOS MODE command before starting the WeatherFAX program.

These commands could be placed in your AUTOEXEC.BAT

file.

1) Set the baud rate and other parameters with the DOS

MODE command. (e.g., MODE COM1:96,N,8,1,P)

2) Redirect LPT1 to COM1 with the DOS MODE command

(e.g., MODE LPT1=COM1)

3) Start WeatherFAX program and set the Printer

Destination in the SETUP MENU to LPT1

Serial port printers are usually configured for either

7-bit or 8-bit graphics mode. Be sure that the printer

is configured for 8-bit mode. Otherwise, the printed

sample will have a blank row (space) every 8 lines.

CONNECTING YOUR COMPUTER TO YOUR FAX RECEIVER

Use a two-wire cable to attach the fax adapter to

your fax receiver. The cable should have a standard

3.5-mm phone plug on the end that plugs into the

adapter. Plug the cable into either the receiver

earphone jack or fax output jack.

USING WEATHERFAX

����������������

ADJUSTING THE PAGE ALIGNMENT

You will need to set the page alignment the first time

you use the adapter. The page alignment synchronizes

the timing of the computer with the fax image. The

image will appear slanted across the display screen when

the page alignment needs to be changed. Once set, the

page alignment will not normally need to be changed

again.

The alignment is set while an image is being received.

The procedure is listed below.

1) Begin receiving an image into computer memory. Set

fax parameters and select the Acquire fax action. The

program will automatically capture the next image

into the computer storage. After the adapter has

started to display an image, go to step 2.

2) Use the cursor down (down-arrow) key until the page

alignment function is selected. If the image slants

to the right, use the cursor left (left-arrow) key to

decrease the alignment value. Otherwise, use the

cursor right (right-arrow) key to increase the

alignment value. Continue adjusting until the image

appears perfectly vertical. The alignment value is

automatically stored in the program file on disk so

that the image is perfectly aligned each time you use

the program.

GETTING THE IMAGE

1) SET THE FACSIMILE TYPE.

o ADAPTER MODE. The fax card can capture four types

of weather facsimile images:

- Marine Weather Facsimile. These images are received

on HF radio on shortwave radio frequencies (e.g.,

8080 kHz). The adapter is set to FM mode.

The fax adapter allows the FM Marine fax signal to

be within a wide amplitude range. However, if the

signal output from your receiver is very strong or

very weak, you can compensate by adjusting

attenuation pads on the adapter. You can adjust the

level while in this pull-down menu by pressing the F4

key. Usually, you will keep the attenuation set at

NORMAL. This setting allows operation for input-signal

amplitudes that vary between -20 dBm to +10 dBm.

- WEFAX image. This is the format for geostationary

weather satellites. The receiver is usually tuned

to S-Band 1691 MHz. The adapter is set to AM mode

and Automatic Gain Lock (AGL) is used to automatically

set the correct level for any type of image.

AGL determines the optimum gain setting at the

beginning of every image and fixes the gain level

for the duration of the image. This technique

provides a distortion free high dynamic range image

each time, whether the image capture is started

automatically or manually.

- Polar image. These are images transmitted from

polar-orbiting satellites at VHF (137.xxx MHz). The

adapter is set to AM mode and a fixed gain level is

used.

While this pull-down menu is displayed, you can

press the F4 key allows you to set the gain of the

fax adapter so that the signal level is just right.

Use the cursor keys to increase or decrease the

signal level while an image is active so that the

level covers a wide dynamic range without exceeding

the MAX (clipping) value.

- GOESTAP image. This is the format transmitted over

phone lines and distributed by a private service on

C-Band satellite.

2) SETTING THE FAX PARAMETERS

You can set the configuration parameters for the selected

adapter mode by pressing the F4 key on the Acquire Fax

menu. You can set parameters separately for each adapter

mode. This allows you to preconfigure the parameters for

each mode and quickly switch between modes.

TABLE OF PELS-PER-LINE FOR VARIOUS IMAGE TYPES

����������������������������������������Ŀ

�Image Type � PELS PER LINE � SPM � IOC  �

�		 �  SKIP LINES	 �     �      �

�		 �  NO	 � YES	 �     �      �

����������������������������������������Ĵ

� HF Fax	 � 1800  �  900  � 120 � 576  �

� WEFAX	 �  840  �  420  � 240 � 268  �

� GOESTAP	 � 2048  � 1024  � 120 �      �

� Polar	 � 1800  �  900  � 120 � 288  �

� Wire Photo� 2048  � 1024  �  *  �      �

������������������������������������������

If your computer has a limited amount of RAM, select SKIP

LINES = YES and PELS PER BYTE = 2.

o SCANS PER MINUTE  (SPM)

Selects the number of image lines sampled every minute.

Use the same value as that of the signal that you

receive. Typically GOES WEFAX satellites are 240 and

others are 120. Many wire photos on C-Band use

nonstandard rates. This menu also allows you to set a

user-defined value that allows you to capture images

that use nonstandard rates.

o PELS PER SCAN LINE

Selects the number of samples (pixels) stored for each

image line. You would typically set this value to be at

least as large as the resolution of your graphics

display mode (e.g., 640 for EGA or VGA). This sets the

resolution of the image and affects the amount of

computer storage needed to hold the image.

The fax program uses all of the available memory in

your computer to capture images. The image size in

bytes can be calculated by the formula:

IMAGE_SIZE = PELS_PER_LINE * NUM_LINES / PELS_PER_BYTE

Where:

PELS_PER_LINE = pixels stored for each image line

NUM_LINES     = lines in the image

PELS_PER_BYTE = pels stored per byte, either 1 or 2

NOTE: The greater the number of Pels Per Line the

greater the image resolution. However, the maximum

value may be limited by the speed of your computer. For

earlier PCs having a 4.77-MHz clock speed use a maximum

value of 700 when using 240 Scans Per Minute.

The "squareness" of the displayed image depends upon

the Index of Cooperation (IOC) of the received image,

the Pels Per Line setting, and the aspect ratio of your

display adapter. The relationship for these parameters

is:

PELS_PER_LINE = � * IOC * ASPECT / SKIP

Where:

� = Pi = 3.14

IOC = Index of Cooperation of received signal

GOES (WEFAX) = 268

Polar satellites = 288

HF fax (Marine) = 576

ASPECT = horizontal/vertical pixel density set by

adapter display mode, not the physical size of the

display. Super VGA, VGA = 1.00 SKIP = 1 if Skip Line

option is OFF 2 if Skip Line option is ON.

o PELS PER BYTE controls the number of gray shades stored

for the image. If your computer has limited memory

(less then 640 kbytes), consider selecting 2 pels per

byte and use the Skip Line option. 2 Pels per byte

saves memory by packing two pixels into each memory

storage byte, which allows you to capture images twice

as large. The image is stored with 16 gray shades and

once captured, you cannot enhance the brightness and

contrast.

If you have expanded memory (LIM 4.0), you should

select 1 pel per byte. The image is stored with 256

shades of gray, and you can enhance the brightness and

contrast of the image at any time.

o SKIP LINES

This option reduces the size of images by storing every

other line. The skipped line is not lost but averaged

the previous saved line. This option is very useful for

reducing the size of HF FAX and GOESTAP images. A

normal GOESTAP image is over 3.2 MB with 2048 pels per

line. With the Skip line option you can capture the

same GOESTAP image using only 1024 pels per line and

use 1/4 the amount of memory.

3) SELECT ACQUIRE FAX or SCHEDULE/TIMER ACTION.

To acquire images you must have a fax card installed in

your computer and your receiver must be attached to the

card with a cable. If you use the Schedule/Timer

specify 24-hour time for the starting times. The

program will automatically capture each image at the

specified time. If Automatic File Save is enabled the

images will automatically be saved to disk.

The program will automatically capture each image from

the fax receiver into the computer's memory. If the

first image appears slanted, the page alignment needs

to be adjusted.

The bottom line of the display is the action line. It

displays the current receive status and allows you to

select various actions while receiving the image.

Different actions are displayed using the cursor keys.

�����������������������������������������������������Ŀ

� ACTION LINE SELECTIONS				   �

�							   �

� Manual start	    Press <�� for manual start	   �

� Move/scroll image    <- -> keys scroll left/right   �

� Darker - 	    <- -> to adjust brightness	   �

� Adj contrast	    <- -> to adjust contrast	   �

� Printer: on/off	    <��   to turn printer on/off   �

� Disk   : on/off	    <��   to save to disk on/off   �

� Page alignment	    <- -> adjust timing.	   �

�������������������������������������������������������

Page alignment allows you to adjust the adapter timing

if the image is slanted. Once adjusted, the timing

should not need to be changed again. Use cursor keys to

increase and decrease the timing value. Increase if the

image slants to the left. Decrease if the image slants

to the right.

�����������������������������������������Ŀ

� RECEIVE STATUS			       �

�					       �

� Wait	  : Waiting for start signal   �

� Start	  : Start signal detected      �

� Sampling   : Putting image into buffer  �

� End of mem : Image size exceeds buffer  �

� Stop	  : Stop signal detected       �

�������������������������������������������

4) EXIT ACQUIRE MODE

After image is captured, press the Esc key exit to the

Fax Acquire Menu. Press Esc again to exit to the Fax

Action Menu. From the Fax Action Menu you can view or

print the image.

VIEWING THE IMAGE

You can view an image once it is stored in memory. The

image may be either retrieved from disk, or acquired

from the fax card. You can then view the image with

different display modes, scroll, zoom, and enhance the

image details in several ways.

When viewing the image, you can either:

1) scroll using the cursor keys, or

2) enhance the image using the pull-down menus.

��������������������������������������������������������Ŀ

� The active keys used for scrolling are:		�

�							�

� cursor keys:	 Scrolls the image window up, down,	�

�		 right and left.  A beep is sounded	�

�		 when you reach the edge.		�

�							�

� Shift Cursor:	 Hold the Shift key and press a cursor	�

�		 key for fast scrolling.		�

�							�

� Home		 Go to top of image.			�

�							�

� End		 Go to bottom of image. 		�

�							�

� F1		 Help (this screen).			�

�							�

� Esc		 Used for several purposes		�

�		 - return from help screen		�

�		 - abort when writing screen		�

�		 - return to previous screen		�

�							�

� Alt/F1..F4	Hold the Alt key and press either	�

�		F1, F2, F3 or F4 to select one of the	�

�		user-defined color palette tables.	�

�							�

����������������������������������������������������������

If you press the space bar while viewing the image, an

action line appears. Use the cursor keys to chose an option,

then press the Enter key. The different image enhancement

selections are described below. Press the Esc key to return

to scroll mode.

PALETTE

You can change the colors and gray scales of the image

by changing the look-up tables (LUT) that define the

way the image is displayed. This allows you to emphasize

selected areas of the image.  Four tables can be defined.

For each table, you can adjust the red, green, and blue

colors separately.

The histogram displays a bar chart of the distribution

of intensity levels in the displayed part of the image.

It is used as a visual aid to to help set the palette

table.  The horizontal axis is the intensity level from

0 to 255.

The active keys are:

Up/Down cursor    :  selects 1 of the 5 functions

Right/Left cursor :  change value

Esc		   :  return to previous menu

F1		   :  Help (this screen)

F9		   :  interpolate colors between the last

two colors that have been modified.

F10		   :  reset colors to gray levels.

Functions:

1) Color number  - Indicates the image intensity level.

All areas in the image with the same

intensity level will momentary flash

to help locate areas of interest.

Use cursor Right/Left to change.

2) Red value	  - Indicates the color's Red component.

Use cursor Right/Left keys to change.

3) Green value   - Indicates the Green component

4) Blue value	  - Indicates the Blue component.

5) Table number  - Use Right/Left keys to change palette

tables

HISTOGRAM

The histogram displays a bar chart of the distribution

of intensity levels in the displayed part of the image.

It is used as a visual aid to to help adjust the brightness

and contrast.  The horizontal axis displays the brightness

level from 0 to 255. The vertical axis displays the number of

pixels for each brightness level.

This option is available only if the image was captured with

pels per byte = 1.

For Brightness, the right cursor key makes the image

darker. For Contrast, the right cursor key increases

the contrast.  For the 16-color modes (e.g., VGA) the

display is redrawn with the new Brightness and Contrast

values when you exit the menu using the Esc key.

This changes the display Look Up Table (LUT). The actual

image data is not changed.

The adjustment scale "[   �   ]" indicates both the

contrast range and brightness level. The center mark

"�" indicates the brightness level. You can make the

image darker by moving the "�" to the right using the

right arrow key. The "[ ]" marks indicate the contrast

range. The contrast is increased by moving the "[ ]"

marks closer together.

OVERALL PICTURE ADJUSTMENT

1) Use the histogram to locate the right and left edges

of the curve where the number of pixels decreases.

2) Adjust the brightness so that the "�" mark is

centered between the edges on the histogram.

3) Adjust the contrast range so that the "[  ]" marks

are directly under the right and left edges of the

curve respectively.

ENHANCEMENT

Select one of the image enhancement curves.

Both logarithmic and NOAA/NESDIS curves are available.

Logarithmic curves include both LOG-White and LOG-Black and

are used for enhancement of visible light imagery.  Both

curves are defined in the "Weather Satellite Handbook",

4th Edition, published by the ARRL.

The 20 NOAA/NESDIS curves are primarly used by professional

meteorologists and are defined by the NOAA/NESDIS IR Imagery

Enhancement Program as described in the 1983

GOES User's Guide.  Includes BD, CC, CE, DD, EC, FC, HC,

HD, IB, IC, JF, JG, LA, MB, NA, PC, SA, TA, ZA and ZB.

ZOOM, NEGATIVE and ROTATE

ZOOM:  You can select a Zoom of 50, 100, 200 or 400 percent.

The 50 percent zoom is useful for shrinking the image

to half size to display more of it at one time. The 200

and 400 percent zooms can be used to enlarge the image

to better display small details of interest.

NEGATIVE:  This displays the negative view of the image

similar to a photographic negative.  The negative view is

useful when displaying GOESTAP, IR images and wirephotos.

ROTATE:	Select either Mirror or 180 Degree Rotate.

Mirror is useful for wire photos that are often

sent as mirror images.  Rotate is useful for viewing images

from ascending-node (South-to-North track) polar-

orbiting satellites that send images upside down.

PRINTING THE IMAGE

Before printing for the first time, use the Setup Menu

to select your printer. You can print the image either

to a printer connected to your computer's printer port

or to DOS device name. The device name could be a file

name or the name of a printer attached to a Local Area

Network.

You can print the entire image or just the part

displayed on the screen.

NOTE: Printing does NOT scale the image to fit the page.

The image will always be printed starting at the

TOP AND LEFT SIDE DISPLAYED ON THE SCREEN.  This

allows you to trim the top and align the left-hand

side before printing. Printing will continue across

the line until the page width is reached.

You can print the image in black and white, or with a

choice of various gray-scale dithering patterns.

Dithering is a printing method of using a series of

dots in an area to simulate gray information in the

same way that a newspaper would via the halftone

technique. The dither patterns allow your image to be

printed with up to 16 gray shades.

Because of the lower resolution of 9-pin dot-matrix

printers, you will get good results using either black

and white, or 2- and 4-gray-shade dithering patterns.

For 24-pin dot-matrix and laser printers, you will get

excellent results with all of the dithering patterns

including the 16 gray-shade dithers. The 16 gray-shade

dithers available are:

������������������������������������������������������Ŀ

� BAYER	       � Brings out detail at the expense   �

�		       � of tonal reproduction (contrast).  �

�		       � Used for very detailed images.     �

� FATTING	       � Good compromise between detail     �

�		       � and good tonal reproduction.	    �

� VERTICAL LINE   � Emphasizes horizontal lines.	    �

� HORIZONTAL LINE � Emphasizes vertical lines.	    �

��������������������������������������������������������

Choosing the best dithering pattern for an image is a

matter of both experimentation and experience. The

Fatting dither offers a good compromise between detail

and contrast and is commonly used.

The print head on some printers may overheat when

printing large images. If this happens the printer will

go into a pause mode to cool down. If the wait is longer

than approximately 20 seconds, the program will time out

and quit waiting for the printer to resume.

If the print head overheats:

- wait for a few minutes for the printer to cool before

printing another image.

- configure the printer for Unidirectional printing

instead of Bidirectional printing. This will slow the

printer and allow more time for cooling.

- Use the DOS MODE command to prevent the printer time-

out condition and to allow the printer to resume

printing the image after the print head has cooled down

(e.g.,MODE LPT1:,,P). This DOS command could be placed

in the AUTOEXEC.BAT file so that it is executed every

time the computer is started.

NOTE:

If you're using the graphics driver shipped with DOS 4.0

(or later), the 'Shift/Print Screen' keys on the computer

keyboard can be used to print a very crude representation

of the image. The DOS GRAPHICS.COM driver must first be

loaded into memory. The graphics driver shipped with DOS

4.0 supports VGA graphics mode.

SAVING THE IMAGE TO DISK

You can save either the entire image or just the part

displayed on the screen.

If you select the entire image then the image is saved

starting at the TOP AND LEFT SIDE DISPLAYED ON THE

SCREEN. This allows you to trim the top and align the

left hand side before saving to disk. You can save the

image using either the standard TIFF (Tag Image File

Format) or GIF (Graphics Interchange Format). TIFF has

fast file operations and stores an image as an

uncompressed raster-scan gray-scale image. GIF is the

standard format used for storing images by CompuServe and

most computer bulletin boards systems (BBS). GIF is

slower than TIFF, but uses an efficient compression

algorithm that minimizes file size and saves disk space.

These standard image file formats allow images to be

imported as graphics files into various painting and

Desktop Publishing applications that support either of

the formats.

TIFF files are supported by Aldus PageMaker, IBM Image

Edit, Logitech PaintShow, and WordPerfect 5.0. Image Edit

is excellent for modifying large TIFF image files. GIF

format files are used by many computer bulletin board

systems (BBS) and are supported by Compuserve CSHOW and

several painting programs.

ADDITIONAL INFORMATION

����������������������

CALIBRATING THE ADAPTER

WeatherFAX card physical layout

���������������������������������������������Ŀ

�	����Ŀ	����Ŀ	      ����Ŀ	 ����Ŀ    �����

�	�POT2�	�POT4�	      �POT3�	 �POT1�    ��

�	������	������	      ������	 ������    ��

�	 Stop	 Start		AM	  FM	   ��

� Detector  Detector   Demodulator  Demod.    ����� <Ŀ

�	450 Hz	 300 Hz        2400 Hz	  1800 Hz  �����  phono

�						   ��	  jack

�						   ��

�						   ��

�						   ��

�						   ��

�						   ��

�						   ��

���������Ŀ			       ����ٳ

���������������������������������    �

Calibration is needed if any of the Self-Tests indicate

that the card is operating out of the indicated

operating range. The self tests will indicate which

parameter is out of range and needs adjustment.

Instructions are provided below for making the

adjustment. The procedure is simple and can be performed

without special tools or knowledge. Because adjustments

are made while the computer is powered on it is

recommended the adjustment be done by a qualified

service technician.

A nonmetallic screwdriver should be used for the

adjustment. This will prevent damage to the computer

if the screwdriver should accidentally be dropped into

the computer.

1) Turn power to the computer off.

2) Remove the cover of the computer. Refer to the User's

Guide for your computer for instructions for

removing the cover.

3) Locate the fax card.

4) Turn computer power on and start the WeatherFAX

program.

5) Select Test Adapter, and then Static Test. Adjust the

potentiometers for the parameter out of range. When

you are finished all parameters should be in the OK

region.

6) The final adjustment for the FM demodulator should be

done while viewing the FM Graph Test. The line should

be linear, sloping upward with time, and centered

between 0 and 256. If not centered adjust the FM

potentiometer while viewing the Graph.

USING EXPANDED MEMORY (LIM 4.0)

Expanded memory eliminates the problem of acquiring

images that are too large for the computer's 640 kbytes

of conventional memory. The LIM (Lotus Intel Microsoft)

standard allows up to 32 Mbytes for image storage. It

requires both expanded memory and a LIM 4.0 software

driver. The WeatherFAX program will automatically use

either conventional or expanded memory based upon the

larger available amount of RAM.

If you have an 80386 type computer then EXTended memory

can emulate LIM 4.0 EXPanded memory by using an expanded

memory manager driver. The QEMM 386 driver by Quarterdeck

Office Systems is fast and recommended. The HIMEM.SYS and

EMM386.SYS drivers shipped with Microsoft Windows 3.0 are

slower and may cause interrupts to be lost when capturing

very high resolution images.

Some of the newer 80286-type computers use the NEAT(r)

chip sets that allow the EXTended memory to emulate

Expanded memory. However EXTended memory on other 80286-

type computers without the special chip sets cannot

emulate EXPanded memory. You must use an actual LIM 4.0

capable memory card (e.g. Intel AboveBoard).

USING SUPER VGA FOR CARDS USING TSENG AND ATI CHIP SETS.

These cards include Orchid ProDesigner, Genoa Super VGA,

Boca Research SVGA and Everex Viewpoint VRAM.

The ATI VGAWonder+ uses the ATI chip set.

If you have a Super VGA card, consult your User's Manual

to identify the chip set type and the enhanced graphics

modes. If the chip set is listed in the table below, you

can select all of the resolutions listed. To identify the

chip set type, you can either:

1) refer to the User's Manual

2) visually inspect the SVGA card

3) try to match the video BIOS modes in the manual with

those in the following table.

TABLE OF VIDEO BIOS MODES FOR SUPPORTED RESOLUTIONS

�������������������������������������������������������Ŀ

�		       �	    VIDEO BIOS MODES	     �

�		       �640x480� 800x600 � 1024x768�1280x1024�

� CHIP SET        �  256  � 16 � 256� 16 � 256�  16     �

�������������������������������������������������������Ĵ

� Tseng ET3000    �  2E   � 29 � 30 � 37 � NA �  NA     �

� Tseng ET4000    �  2E   � 29 � 30 � 37 � 38 �  NA     �

� ATI	       �  62   � 54 � 63 � NA � NA �  NA     �

� Paradise        �  5F   � 58 � NA � 5D � NA �  NA     �

� Video Seven     �  67   � 62 � 69 � 65 � NA �  NA     �

� Everex Viewpoint�  30   � 02 � 31 � 20 � 32 �  21     �

�		       �       �    �	 �    �    �	     �

���������������������������������������������������������

NOTES:

1) NA indicates that the mode is not supported.

2) A multifrequency (multisync) display is required for

resolutions greater than 800 x 600.

3) 512 kbytes of video RAM is required on the graphics

card for the 800 x 600 x 256 and 1024 x 768 x 16

color modes. 1 MB is required for the 1024 x 768 x

256 and 1280 x 1024 x 16 color modes.

4) Everex Viewpoint, Genoa Super VGA, Orchid

ProDesigner, Boca Research SVGA, Sigma VGA/H and STB

VGA Extra use the Tseng chip set. ATI VGAWonder+ use

the ATI chip set. AST VGA Plus, Compaq VGA, Paradise

VGA Plus/Prof and Dell VGA use the Paradise chip set.

Video7 VRAM/Fastwrite use the Video Seven chip set.

5) The ATI VGAWonder is not supported at 1024x768-16

colors because the card does not support the BIOS

commands used by the WeatherFAX program for the pull-

down menus.

If your SVGA chip set is not listed above, then VGA and

the 800 x 600 x 16 enhanced graphics mode can be still be

used by specifying the BIOS mode as a command-line

parameter. See section below for instructions.

USING SUPER VGA (800 X 600 PIXEL) MODE

(chip sets other than Tseng/ATI)

If you have a multifrequency (multisync) monitor and a

graphics card that supports the super 800 X 600 x 16

color graphics mode, you can display images in this mode.

Because different cards use different 800 by 600 BIOS

mode numbers, you must specify the mode as command line

parameter when starting FAX.EXE. To specify the graphics

mode:

FAX.EXE /Hxx	 where: 'xx' is the hex mode.

Example:

For Oak the hex mode is 52 	  ->   FAX.EXE /H52

Be sure to check that you have the correct monitor and

graphics card. The software does not verify the monitor

or graphics card. Selected cards and their respective 800

X 600 graphics mode (hex) are listed below. If your

graphics card is not listed, you will have to consult

your card's user's guide to determine the correct mode.

TABLE LISTING 800 x 600x 16-COLOR BIOS MODES

FOR VARIOUS GRAPHICS CARDS

������������������������������������������Ŀ

� CARD		      � CHIP SET � MODE �

������������������������������������������Ĵ

� Chips & Technology     � C&T	 � /H70 �

� Everex		      � Everex	 � /H02 �

� Great Wall of China    � Trident  � /H5B �

� Oak Technology	      � Oak Tech � /H52 �

� Tecmar VGA-AD	      � 	 � /H18 �

� AST VGA Plus	      � 	 � /H58 �

� Compaq VGA	      � 	 � /H58 �

� Dell VGA 	      � 	 � /H58 �

� Genoa Super VGA/VGA    � 	 � /H29 �

� Paradise VGA	      � 	 � /H58 �

� Video7 VRAM/Fastwrite  � 	 � /H62 �

��������������������������������������������

If your card isn't listed above, you should determine the

correct 800 x 600 x 16-color mode from the User's Manual

that came with your card.

SOURCES OF WEATHER-FACSIMILE SIGNALS

There are three direct sources for weather-fax signals:

geostationary satellites; polar-orbiting satellites; and

Marine weather-radio transmitters. The fax card along

with the WeatherFAX software can capture all the above

types of facsimile. Tone detectors on the card will

automatically detect the beginning and ending of each

image.

Geostationary Satellites.

Geostationary satellites include US GOES, European

Space Agency METEOSAT, and Japanese GMS. These

satellites broadcast on S-Band frequencies; GOES at

1691 MHz and Meteosat at 1694.5 MHz. These satellites

are located approximately 22,000 miles above the Earth

and provide WEFAX (Weather Facsimile) transmissions

using the standard Automatic Picture Transmission (APT)

format.

Reception of WEFAX images requires an antenna,

downconverter, and receiver. The antenna can be either

a small parabolic dish or a loop Yagi. The

downconverter  converts the S-Band signal to 137.5 MHz.

An inexpensive  scanner radio can be used for the

receiver. A two-wire cable from the receiver plugs into

the WeatherFAX card.  Configure the software for AM

mode and 240 scans per minute.

Polar-Orbiting Satellites.

These satellites transmit on VHF frequencies between

137 and 139  MHz. The NOAA (TIROS-N) series transmit at

137.50 MHz and 137.62 MHz. The Russian Meteor series

satellites transmit at 137.15, 137.30 and 137.850 MHz.

A general-purpose scanner receiver capable of receiving

the frequencies between 136-138 MHz is required. Many

of the newer Regency, Uniden, and Radio Shack scanners

cover this frequency range and give excellent results.

This receiver can also be used in conjunction with the

geostationary satellite antenna and downconverter.

Configure the software for AM mode and 120 scans per

minute.

Marine weather radio facsimile.

These broadcasts can be received direct from government

radio transmissions. There are over 65 transmitter

sites worldwide which utilize over 200 different radio

frequencies. The information transmitted includes

weather-analysis charts, satellite pictures, and radar

charts. These radio signals are received on short wave

radio frequencies (e.g., 8080 kHz) on Upper Sideband. A

stable general coverage single sideband (SSB) receiver

is required.  The published frequency is normally the

center frequency of the upper sideband. Tune your SSB

receiver approximately 1600-1700 Hz below the published

frequency. For example, the US Navy Fleet Weather

Station at Norfolk (NAM), transmits at 8080 kHz. Tune

your receiver to 8078.3 kHz (USB) to receive its

transmissions. Configure the software for FM mode and

120 scans per minute.

WEATHERFAX SPECIFICATIONS

o Recording Rate : 60, 90, 120, 240 SPM (Scans Per

Minute)

o Index of cooperation IOC

576 - Marine fax

288 - TIROS

267 - WEFAX (GOES)

o FSK input: 1500 Hz "black", 2300 Hz "white"

o FSK input level:  Range selectable by pads.

-10 dBm to +20 dBm   Strong Signal

-20 dBm to +10 dBm   Normal Signal

-30 dBm to   0 dBm   Weak Signal

o AM input:  2400-Hz AM modulation

lower voltage  = "black"

higher voltage = "white"

o AM input level : -20 dBm to +10 dBm.

o Input impedance:  10 kilohms

o Control signals	 Automatic start, frame, and stop

with manual start and frame.

- START : Signal modulated between "black" and

"white" at 300 Hz rate for 5 seconds.

- STOP : Signal modulated between "black" and "white"

at a 450 Hz rate for 5 seconds.

- FRAME : "black" signal interrupted by a short burst

at a "white" signal each scan line for 5 seconds

immediately following start signal.

o Hardware interrupt:  8 (BIOS timer interrupt)

o I/O Addresses:  386H and 387H

o Voltage and current requirements:

�������������������Ŀ

� Voltage � Current �

�������������������Ĵ

�  + 5V	 � 120 mA  �

�  +12V	 �  25 mA  �

�  -12V	 �  15 mA  �

���������������������

IF YOU HAVE DIFFICULTY

����������������������

SYMPTOM	 SOLUTION

������������� ����������������������������������������

ALIGNMENT	    1) Reduce the value of Pels per Line.

WAVY -

CANNOT BE	    2) If you have a 80386-type computer,

SET	       remove the Expanded Memory driver and

restart the computer. IF THE PROBLEM

GOES AWAY THEN THE DRIVER IS TOO SLOW.

If so then use the EMM-386 driver from

Quarterdeck Office Systems.

NO PICTURE      1) Run fax card tests to assure that the

card is operating within the correct

range.

2) Verify that Adapter Mode is set

correctly for the facsimile signal.

3) For Marine fax mode verify that the

receiver signal level is within the

range of the fax card attenuation

pads. If the signal level is very weak

then decrease the attenuation pads

using the F4 key on the adapter mode

menu.

4) Is the fax receiver powered on and the

cable connected to the fax adapter?

DISTORTED	     1) Verify that the Adapter Mode is set

PICTURE		correctly for the received signal.

(e.g., if the image is WEFAX then the

Adapter Mode must be set to AM.

2) Verify that Scans per minute is set

correctly for the received signal.

3) If the image is slanted then the Page

Alignment needs to be adjusted while

an image is received.

4) For FM Marine fax verify that the

receiver is tuned correctly. Marine

fax images are usually transmitted

using Upper Side Band (USB). The

receiver must be tuned approximately

1700 Hz lower then the center

frequency.

Example: For a 8080 kHz HF fax signal,

the receiver should be tuned to 8078.3

kHz. (8080 kHz - 1.7 kHz = 8078.3

kHz). Vary the receiver frequency

slightly to get the best picture.

5) Check to see if any TSR (terminate and

stay resident) programs are in memory.

If they are, remove them. Some TSR

programs may cause problems when

running WeatherFAX.

EXPANDED		Be sure that the Expanded memory

MEMORY NOT 	device driver (LIM 4.0) is loaded

RECOGNIZED:	into memory. If you have an 80386 type

computer, then the EXTended memory can

emulate LIM 4.0 EXPanded memory using

a device driver shipped with DOS 4.0.

For most 80286 computers, EXTended

memory cannot emulate EXPanded

memory.

800 x 600	     1) Verify your graphics card supports

DISPLAY		this mode.

DOESN'T WORK

2) Verify that you have a multifrequency

monitor that supports 800 x 600

resolution.

3) Verify you have set the correct BIOS

display mode.

4) Verify that the correct mode is listed

as a command line parameter.

ADAPTER/SOFTWARE AVAILABILITY

�����������������������������

The WeatherFAX adapter and software are available from:

OFS Software

6404 Lakerest Ct.

Raleigh, NC 27612

o Assembled and tested scan-converter board, software and

instructions ..................................$360.

o Kit including PC board, all parts, software and

assembly instructions .........................$230.

o Software only (Version 3.0) ....................$30.

Terms:

- Specify either 5.25" or 3.5" diskette format for the

software.

- North Carolina residents add 5% sales tax.

- Please add $6.50 for shipping and handling (or add

$12.00 for overseas orders.)

- Check drawn on a US bank in US funds. International

bank or postal money order in US funds. All foreign

orders must be prepaid.

- Please allow 4-6 weeks for US delivery.

TRADEMARKS USED IN THIS GUIDE

�����������������������������

o GIF and Graphics Interchange Format are trademarks of

Compuserve Incorporated

o Turbo C is a trademark of Borland International

o IBM and PS/2 are trademarks of International Business

Machines Corporation

o PageMaker and Aldus are registered trademarks of Aldus

Corporation.

o Apple, ImageWriter are registered trademarks of Apple

Computer Inc.

o LaserJet is a trademark of Hewlett-Packard Company.

o Intel is a trademark of Intel Corporation.

o Microsoft, and MS-DOS are registered trademarks of

Microsoft Corporation.

o WordPerfect is a trademark of WordPerfect Corporation.

#