Here you can find a little patch to remove the communication drop-out

after MS-BASIC 7.x program termination.

This patch was a modified version for MS-BASIC 7.x from the original

for MS-Quick-BASIC 4.5 provided by Kenny Gardner (thanks !) and works

ONLY for program compiled with the 'EXE Requiring BRT Module' switch.

I try to modify also the BCL70EFR.LIB library, but i don't find the

search string !!!

Bye and good work !

Borsalino Paolo. (CIS 100042,1206)

Patch for Basic Professional Development System 7.x

---------------------------------------------------

If you desire that a BASIC program compiled with the BRT70EFR library

doesn't modify del serial parameter (DTR / DSR ...) after terminating,

you may modify the library with the following patch.

BRT70EFR.EXE patch.

-------------------

1) make a backup copy of BRT70EFR.EXE

2) rename the original BRT70EFR.EXE as BRT70EFR.X

3) type DUBUG BRT70EFR.X

4) type :

s cs:0 ffff b0 00 e3 01

debug should show

xxxx:C32E

where xxxx can be any number, and C32E is the location in which debug

finds the above parameter

type u C32E

debug should show something like

xxxx:C32E B000          MOV     AL,00

xxxx:C330 E301          JCXZ    C333

xxxx:C332 40            INC     AX

xxxx:C333 83C204        ADD     DX,+04

xxxx:C336 EE            OUT     DX,AL

xxxx:C337 C747080000    MOV     WORD PTR [BX+08],0000

xxxx:C33C 83EA03        SUB     DX,+03

xxxx:C33F EB00          JMP     C341

xxxx:C341 B80010        MOV     AX,1000

xxxx:C344 EE            OUT     DX,AL

xxxx:C345 4A            DEC     DX

xxxx:C346 80FE03        CMP     DH,03

xxxx:C349 7402          JZ      C34D

xxxx:C34B B408          MOV     AH,08

xxxx:C34D FA            CLI

this is where MS-BASIC 7.x reset the communication ports.

To fix the problem, type:

a C32E

MOV AL,01

[enter]

a C332

NOP

[enter]

a C336

NOP

[enter]

where [enter] is the ENTER or RETURN key !

5) to verify what you have typed, type

type u C32E

debug should show something like

-> xxxx:C32E B001          MOV     AL,01                <-

xxxx:C330 E301          JCXZ    C333

-> xxxx:C332 90            NOP                          <-

xxxx:C333 83C204        ADD     DX,+04

-> xxxx:C336 90            NOP                          <-

xxxx:C337 C747080000    MOV     WORD PTR [BX+08],0000

xxxx:C33C 83EA03        SUB     DX,+03

xxxx:C33F EB00          JMP     C341

xxxx:C341 B80010        MOV     AX,1000

xxxx:C344 EE            OUT     DX,AL

xxxx:C345 4A            DEC     DX

xxxx:C346 80FE03        CMP     DH,03

xxxx:C349 7402          JZ      C34D

xxxx:C34B B408          MOV     AH,08

xxxx:C34D FA            CLI

6) to save the patch, type

W

debug should show something like

Writing xxxxxx bytes

type Q to exit from debug.

7) rename the modified BRT70EFR.X as BRT70EFR.EXE

8) good luck !

MS-BASIC / MS-Quick-BASIC are registered Trademark of Microsoft

Corporation.