Reference Manual for the

AutoLibrary(tm) Program-Playback Tool

Version 1.2

March 1992

Copyright 1991 by

Wayne E. McDaniel

All Rights Reserved

AutoLibrary(tm) is a Trademark of

Avid Software

Avid Software

P.O. Box 1871

Beaverton, OR  97075-1871

(503) 626-6652

CompuServe Mail:    70372,2513

Internet:           70372.2513@compuserve.com

_______

____|__     |               (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

Copyright 1991 Wayne E. McDaniel,  All rights reserved.  Printed

in the United States of America.

The AutoLibrary(tm) Program-Playback tool Shareware diskette,

which contains a copy of this manual, may be freely copied and

shared.

However, multiple copies of this document may not be printed and

printed copies of this document may not be copied in any way

without permission in writing from Avid Software.

Table of Contents

Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   4

Installation  . . . . . . . . . . . . . . . . . . . . . . . .   8

System Requirements . . . . . . . . . . . . . . . . . . . . .   9

Optional Requirements . . . . . . . . . . . . . . . . . . . .   9

Discussion of Requirements  . . . . . . . . . . . . . . . . .   9

Compiling Source  . . . . . . . . . . . . . . . . . . . . . .   9

Compiling Interface Libraries . . . . . . . . . . . . . . . .   9

Compiling Applications  . . . . . . . . . . . . . . . . . . .  10

Getting Started Using Example 1 (ex01.exe)  . . . . . . . . .  11

Getting Started Using Example 2 (ex02.exe)  . . . . . . . . .  14

A Tutorial  . . . . . . . . . . . . . . . . . . . . . . . . .  15

Reference Guide For the AutoLibrary(tm) Program-Playback

tool . . . . . . . . . . . . . . . . . . . . . . . . . .  33

AvidClosePort - Close Communication Port . . . . . . . .  34

AvidConvert - Convert Characters to Strings  . . . . . .  36

AvidDrainPort - Drain Communication Port . . . . . . . .  38

AvidExitAutoLibrary - Exit AutoLibrary . . . . . . . . .  41

AvidGetFromHoldArea - Get Characters from Hold Area  . .  43

AvidInitAutoLibrary - Initialize AutoLibrary . . . . . .  45

AvidLogInfo - Log information to file or screen. . . . .  47

AvidLogOpen - Open a Log File. . . . . . . . . . . . . .  51

AvidOpenPort - Open Communication Port . . . . . . . . .  53

AvidProcessOptions - Parse an Option Parameter List  . .  57

AvidRead - Read a Character  . . . . . . . . . . . . . .  60

AvidRegisterSend - Register a String to Send . . . . . .  63

AvidRegisterWait - Register a Wait String  . . . . . . .  65

AvidSend - Send a String . . . . . . . . . . . . . . . .  67

AvidSendSearch - Send Strings then Search  . . . . . . .  69

AvidVersion - Print Version and Copyright Information  .  76

EveryChar - Called with Every Character Received . . . .  77

Reference Guide for Interface Routines  . . . . . . . . . . .  79

CommInterfaceClose - Close port  . . . . . . . . . . . .  80

CommInterfaceOpen - Open Port  . . . . . . . . . . . . .  82

CommInterfaceRead - Read Character . . . . . . . . . . .  94

CommInterfaceSend - Send Character . . . . . . . . . . . 105

4

Introduction

The AutoLibrary(tm) Program-Playback tool is a C function library

for automation.  Automation is coded into programs then played

back when the program executes.

This tool will not replace any of your existing automation tools.

You will still use your terminal emulator's script language for

automatic access to bulletin boards and online services and you

may still want your Capture-Playback tool to create quick and

convenient regression tests.

Instead of replacing any tool, add this tool to your personal

tool box and use it when it offers the best solution.

To determine if this tool offers you the best solution, consider

the following questions.

1.   Is there a serial connection to the system being

automated?  Or can there be by redirecting I/O or

designing in a serial test port?

2.   Are you automating with ASCII characters?

3.   Do you want C as your programming language, where

automation is hand coded into subroutines?

If your answers to the previous questions were yes, you will

need...

1.   A PC with a serial port.

2.   Turbo C version 2.0

Or you will need to do one or more of the following...

1.   Port this to a different operating system.

2.   Port the interface routines to something other than RS-

232.

3.   Port the interface routines to a different

communications package.

4.   Port to a different C compiler.

The features of this tool combined with the C programming

language, give you the benefit of writing easy and flexible

5

automation programs.  The following list shows some of the

features and benefits you will like.

1.   Your automation programs can be distributed without

script files or interpreters.

2.   Your automation programs will take advantage of the C

programming language for many things like:  file I/O,

flow control, variable declarations, parameterized

subroutines, and so on.  Also, you can take advantage

of C debuggers, profilers, linkers, and other add-on

libraries.  You will be able to create generic

automation libraries using the C librarian for

distribution to other members of your group.

3.   Your automation programs will not have to depend on

play back timing or comparisons to previous runs.

Instead, your automation programs will have the

expected results coded in.

4.   Your automation programs will take advantage of the way

this tool handles timeouts.

There are two types of timeouts: Repeating-Timeouts and

Ending-Timeouts.  Repeating timeouts start over after

each newly received character and ending timeouts

don't.

5.   Your automation programs will handle variations of

play-back input by registering multiple strings to wait

for.

6.   Your automation programs will take advantage of the

options parameter each routine has.  This parameter

allows customized use of the routines.  It will also

allow me to add functionality without adding new

interfaces or changes existing ones.

7.   Your automation programs may take advantage of the

EveryChar routine.  This routine is written by you and

is called with each received character.

8.   Your automation programs may take advantage of your

ability to vary the send delay between each character

sent.

6

The following lists each of the routines in this tool...

The main routines that support automation sequences...

AvidRegisterSend - Register a string to send.

AvidRegisterWait - Register a wait string.

AvidSendSearch - Send strings then search.

The others in alphabetical order...

AvidClosePort - Close communication port.

AvidConvert - Convert characters to strings.

AvidDrainPort - Drain communications port.

AvidExitAutoLibrary - Exit AutoLibrary.

AvidGetFromHoldArea - Get characters from hold area.

AvidInitAutoLibrary - Initialize AutoLibrary.

AvidLogInfo - Log information to file or screen.

AvidLogOpen - Open a log file.

AvidOpenPort - Open communication port.

AvidProcessOptions - Parse an option parameter list.

AvidRead - Read a character.

AvidSend - Send a string.

AvidVersion - Print version and copyright information.

EveryChar - Called with every character received.

Interface routines to your communications library...

CommInterfaceClose - Close port.

CommInterfaceOpen - Open port.

CommInterfaceRead - Read Character.

CommInterfaceSend - Send Character.

7

The following list shows some of the applications this tool would

be good for.  (The items with an asterisk are projects that I

have done using this automation technique.)

-    Automating CompuServe access.

-    Controlling/testing a modem.

-    Controlling/testing an ASCII Terminal

-    Controlling/testing Unix or a program running on Unix.

-    Controlling/testing other mini or mainframe computer

systems.

-    Controlling/testing Bulletin Board Systems

-    Controlling/testing Data Switches.

-  * Network monitoring from users perspective.

-    Controlling/testing Special equipment.  For example,

medical instruments.  Especially instruments that have

serial ports designed in to help support automated

testing.

-  * Controlling/testing Target debug monitors.

-    Controlling/testing Prom programmers.

-    Controlling/testing anything that has a serial

connection or can be redirected through a serial

connection.

8

Installation

For installation instructions see the readme file for installing

this software.

Please see the PACKAGE.LST file for a description of CONSULT.DOC,

SUPPORT.DOC, LICENSE.DOC, and so on.  Within these files are

important information about registering, warranty, support,

license agreements, vendor/sysop information, hiring me as a

contractor/consultant, and so on.

9

System Requirements

a)   Turbo C v2.0

b)   COM 1 or COM 2

c)   IBM PC or compatible

Optional Requirements

a)   A communication package, such as, Greenleaf Comm Library

v2.21  (Should work with v3.0)

Discussion of Requirements

The AutoLibrary Program-Playback Tool communicates via interface

routines to the communication package.  I provide a basic

asynchronous RS-232 communication package with this tool to get

you started.  However, I recommend using a communication package

with additional features and a proven track record.

Interface routines are provided for the Greenleaf Comm Library.

However, you may port the interface routines to any one of the

many communication packages.  This manual contains detailed

instructions for doing this.

It is possible for both registered and unregistered users to

write interface routines for a different communication package.

Also, given registered user receive the source, it is possible to

port this tool to other compilers and operating systems.

Compiling Source

The source code is generated by the batch file genauto.bat.  Make

sure Turbo C is located in \tc or edit the batch files with your

Turbo C path.

Compiling Interface Libraries

The interface libraries are generated by the batch file

genface.bat.  Make sure Turbo C is located in \tc and the

Greenleaf Comm Library is located in \gls or edit the batch files

with your pathnames.

10

Please see the detailed information on how to port the interface

routines to use other communication packages.  The information is

documented in the reference section for CommInterfaceClose,

CommInterfaceOpen, CommInterfaceRead, and CommInterfaceSend.

Compiling Applications

To compile your application, I recommend starting with example

one in the self-extracting file ex01.exe.  Copy this file to a

clean directory, then execute ex01.  Here you will find a

template file ready to compile with genex01.bat.

11

Getting Started Using Example 1 (ex01.exe)

The goal of this section is to provide you with a simple example

to get you started.  Also, to help you get started, read the

tutorial section.  This will give you an overview of how to

automate using the AutoLibrary(tm) Program-Playback tool.  You

may run the Wyse demo if it is convenient, i.e., if you have a

Wyse terminal handy.  Regardless of whether you are able to run

the Wyse demo, I recommend doing a quick code review of the demo.

Start with example 1 and then look at example 2.  The examples

are in self-extracting files ex01.exe and ex02.exe.  Execute

these programs in clean directories to extract the files.

When getting started with something new, it is nice to know the

minimum that is required.  Therefore, I have supplied a template

file (template.c from ex01.exe) that contains the minimum

required for the AutoLibrary(tm) Program-Playback tool.  The

following code is a listing of template.c.

12

/* template.c */

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include "al.h"

/*****/

void EveryChar(c)

int c;

/*****/

{

/* Print each character received. */

printf("%c", c);

}

/*****/

void main(argc, argv)

/*****/

int argc;

char *argv[];

{

int com;

int err, add_info;

argc = argc; /* for compiler. */

argv = argv; /* for compiler. */

/* Initial AutoLibrary. */

AvidInitAutoLibrary(AVID, &add_info, "_B'3'");

/* Open the communication port. */

com = AvidOpenPort(AVID, "com1", &add_info,

".b'9600' .1 .8 .n +S -H");

/* Display AutoLibrary version. */

AvidVersion(AVID, &add_info, "");

/* Ready for automation. */

/* Finished so close the com port. */

AvidClosePort(AVID, com, &add_info, "");

/* Exit from AutoLibrary. */

AvidExitAutoLibrary(AVID, &add_info, "");

}

13

The above template program has been supplied with this product

and can be compiled.  If the following automation sequence was

added to the template file, it would send "Hello World" to

whatever the PC was connected to.  Then it would wait for the

string "Hello Back".

AvidRegisterSend(AVID, com, "Hello World", &add_info, "");

AvidRegisterWait(AVID, com, 1, "Hello Back", &add_info, "");

if ((AvidSendSearch(AVID, com,(long)20, (long)120,

&add_info, "")) == 1)

printf("Found Hello Back.\n");

else

printf("Timeout occurred.\n");

If you add this code and connect your PC to a terminal or a PC

running terminal emulation software, you will see the "Hello

World" string printed.  Then, from the terminal or PC running

terminal emulation software, you can type in the string "Hello

Back", which would result in a successful automation sequence.

This example is small, but it gives you a feel for how automation

works.  A command string is sent out, then one or more strings

are scanned for.

Note that this example is a bit odd.  Normally, automation

programs are connected to programs or equipment that can

communicate on there own.  For this example, you simulated

something being automated when you typed in "Hello Back".

14

Getting Started Using Example 2 (ex02.exe)

To run the wyse terminal demo you will need to connect

communication port one (com1) of a PC to a Wyse 30, Wyse 50, or

Wyse 60.  For a Wyse 60 terminal, invoke the setup and select the

Wyse 50 compatibility mode.  When connecting the PC to the

terminal, a null modem may be required.

The wyse terminal demo is executed from a command line interface

that works much like a Unix utility.  The code in wyse1.c is the

program control code, where as, the code in wyse.c is the

automation routines.

By typing in 'wyse' (without options), a help screen is

displayed.  For example...

C:\EX02\>wyse

Usage: wyse <options>

Where: <options> are...

+a --- Run all tests.

.A --- Abort program if an error occurs. (Default is continue)

-L --- Run long version of tests.

+p --- Run protected mode test.

-a --- Run screen attributes test.

+s --- Run screen location test.

+P --- Run screen print test.

The following will run all of the tests and abort if there is an

error.

C:\EX02\>wyse +a .A

The following will run the Screen Attributes test and the

Protected Mode test.  The long versions of these tests will be

run if they exist.

C:\EX02\>wyse -a +p -L

One of the tests has been coded to purposely report errors.  This

will demonstrate what error messages look like.

15

A Tutorial

The goal of this tutorial is to introduce you to automation using

the AutoLibrary(tm) Program-Playback tool.

This tutorial uses a PC and a Wyse 50 terminal.  The serial

connection between the PC and the Wyse 50 terminal places the PC

in the unusual role of "host computer."  On the other hand,

everything seems normal to the terminal, which displays

characters when received and sends characters when typed in.  The

Wyse 50 does not know it is connected to a PC.

Once the PC and Wyse 50 are connected, we can write automation

routines that send and receive from the terminal.  This tutorial

focuses on a Wyse 50 command that allows host computers to

identify the terminal.  The command has the format "ESC<space>",

and is one of many Wyse 50 escape codes.  When the Wyse 50

receives this command, it returns the string "50\r".  The '\r'

character is the C notation for a return character.  The

following code segment is the first of many that will use the

"ESC<space>" escape command to illustrate automation techniques

using the AutoLibrary(tm) Program-Playback tool.

sprintf(temp_s, "%s ", ESC_CODE);

AvidRegisterSend(AVID, com, temp_s, &add_info, "");

AvidRegisterWait(AVID, com, 1, "50", &add_info, "");

register_value = AvidSendSearch(AVID,

com,(long)5, (long)10, &add_info, "");

The calls to AvidRegisterSend and AvidRegisterWait result in two

linked lists being built for AvidSendSearch: the send-list and

the wait-list.  The call to AvidRegisterSend does not actually

send the string.  Instead, AvidSendSearch sends all of the

strings in the send-list and then begins a search for the strings

in the wait-list.

In this example, AvidSendSearch sends the command "ESC<space>" to

the Wyse 50 terminal, then begins a search for the string "50".

The value returned depends on the outcome of the search.  If the

string "50" is found, the return value will be 1.  If the string

"50" is not found, the value 0 will be returned.  Zero is always

returned when none of the strings in the wait-list are found.

Notice, when the string is found, the returned value depends on

what was registered.  In this example, the string "50" was

registered with 1.  It could have been registered with the value

527, which would cause AvidSendSearch to return 527 when the

string was found.

16

It is possible to register more than one string in the send-

list.  However, this is not done very often, since complicated

send strings can be built using 'sprintf'.  It is different for

the wait-list.  Your programs will have multiple wait strings

many times.  Since the Wyse 30, Wyse 50, and Wyse 60 have

compatibility modes, it makes sense to write the above sample

code segment as follows.

sprintf(temp_s, "%s ", ESC_CODE);

AvidRegisterSend(AVID, com, temp_s, &add_info, "");

AvidRegisterWait(AVID, com, 1, "30", &add_info, "");

AvidRegisterWait(AVID, com, 2, "50", &add_info, "");

AvidRegisterWait(AVID, com, 3, "60", &add_info, "");

register_value = AvidSendSearch(AVID, com,

(long)5, (long)10, &add_info, "");

Nothing is different except for the two additional calls to

AvidRegisterWait.  These calls allow AvidSendSearch to search for

the strings "30", "50", and "60" at the same time.

AvidSendSearch will return 1 if connected to a Wyse 30, 2 if

connected to a Wyse 50, and 3 for a Wyse 60.  ERR_SNF (Zero) is

returned if none of these terminals are connected.

Now, suppose the PC's serial port is disconnected.  Given the

above code segment, AvidSendSearch will timeout and return

ERR_SNF (String-Not-Found).

The AutoLibrary(tm) Program-Playback tool supports two types of

timeouts:  A Repeating Timeout and an Ending Timeout.  Before

defining repeating and ending timeouts, notice that the basic

model of automation requires timeouts.  Automation is

programmatically sending a keystroke sequence, then scanning the

output for a list of expected responses.  When none of the

expected responses are found, a timeout must occur.  The program

will enter an endless loop without a timeout feature.

A repeating timeout starts over after each received character.

Therefore, if a repeating timeout of 10 seconds is specified, the

timeout will not occur unless there is communication silents for

10 seconds.  A timeout will never occur if characters are

continuously received.  An ending timeout, on the other hand, is

not reset with each newly received character.  If an ending

timeout of 20 seconds is specified, a timeout will occur in 20

seconds regardless, assuming none of the expected responses are

received.

17

These two types of timeouts can play an important role in writing

correct automation "scripts."  The repeating timeout can be given

a reasonable timeout value and still be able to handle many

variations in input.  The ending timeout can be given a large

value, which could indicate an error condition.  When

AvidSendSearch returns ERR_SNF due to a timeout, the return

parameter 'add_info' will be set to ERR_ENDING_TIMEOUT or

ERR_REPEAT_TIMEOUT.

A topic that is related to timeouts is draining communication

ports.  Draining a communication port is simply reading in

characters while ignoring them for a specified amount of time.

The time period to drain a communication port can be a Repeating

Drain and an Ending Drain.  A repeating drain starts over with

each newly received character.  There needs to be communication

silents for repeating drains to stop.  An ending drain will stop

after the specified amount of time regardless of communication

activity.

The following is the same sample code segment with the additional

call to AvidDrainPort.

sprintf(temp_s, "%s ", ESC_CODE);

AvidRegisterSend(AVID, com, temp_s, &add_info, "");

AvidRegisterWait(AVID, com, 1, "30", &add_info, "");

AvidRegisterWait(AVID, com, 2, "50", &add_info, "");

AvidRegisterWait(AVID, com, 3, "60", &add_info, "");

register_value = AvidSendSearch(AVID, com,

(long)5, (long)10, &add_info, "");

AvidDrainPort(AVID, com, (long)1, (long)3, &add_info, "");

This drains the communication port of the <cr> character.

Remember that when a Wyse 50 terminal receives the "ESC<space>"

command, it returns "50<cr>".   However, the wait-list contains

only the string "50", which causes the <cr> character not to be

read in.  To guard against the <cr> character effecting the next

automation sequence, it is drained.

Another way to make sure the <cr> is read in, is to include it in

the wait string.  The following example demonstrates this.

sprintf(temp_s, "%s ", ESC_CODE);

AvidRegisterSend(AVID, com, temp_s, &add_info, "");

AvidRegisterWait(AVID, com, 1, "30\r", &add_info, "");

AvidRegisterWait(AVID, com, 2, "50\r", &add_info, "");

18

AvidRegisterWait(AVID, com, 3, "60\r", &add_info, "");

register_value = AvidSendSearch(AVID, com,

(long)5, (long)10, &add_info, "");

You will notice the call to AvidDrainPort is gone and the '\r'

character has been added to the wait strings.  For this example,

adding the '\r' character is better than calling AvidDrainPort,

only because the one second delay in AvidDrainPort is avoided.

The two methods, otherwise, result in the same thing.

In general, draining is needed when there are leftover characters

after an automation sequence has finished.  In the above example,

the '\r' was a leftover character.  You will find draining

communication ports very useful when the leftover characters of

an automation sequence varies.

The following code segment will be familiar by now.  It sends the

"ESC<space>" command to the Wyse 50 terminal then scans for the

string "50".  When finished, the '\r' character is drained.

sprintf(temp_s, "%s ", ESC_CODE);

AvidRegisterSend(AVID, com, temp_s, &add_info, "");

AvidRegisterWait(AVID, com, 1, "50", &add_info, "");

register_value = AvidSendSearch(AVID, com,

(long)5, (long)10, &add_info, "");

AvidDrainPort(AVID, com, (long)1, (long)3, &add_info, "");

The following code segment works the same way as above, but uses

switches instead.

sprintf(temp_s, ".s'%s ' 1w'50' .d'1'", ESC_CODE);

value = AvidSendSearch(AVID, com, (long)5, (long)10,

&add_info, temp_s);

With switches, the same automation sequence can be accomplished

with one call to AvidSendSearch.  The .s'%s ' switch adds the

"ESC<space>" string to the send-list, the 1w'50' switch adds the

string "50" to the wait-list, and the switch .d'1' indicates

there is to be a drain after the automation sequence has

finished.  These switches replace the need to call

AvidRegisterSend, AvidRegisterWait, and AvidDrainPort explicitly.

Here is the same call again with two additional wait strings.

19

sprintf(temp_s, ".s'%s ' 7w'30' 6w'50' 9w'60' .d'1'", ESC_CODE);

value = AvidSendSearch(AVID, com, (long)5, (long)10,

&add_info, temp_s);

In this example, AvidSendSearch will send the "ESC<space>"

command then search for the strings "30", "50", and "60".  If the

string "30" is found, AvidSendSearch returns 7, if "50" is found,

6 is returned and if "60" is found, 9 will be returned.

Notice that the switch .D'3' would also be required to make the

switched version match the unswitched version exactly.  The .d'1'

switch causes a Repeating Drain for one second and the .D'3'

would cause a 3 second Ending Drain.

The "ESC<space>" command would make a nice general purpose

routine that other parts of the automation program could call.

Here is a boolean function that returns TRUE if a Wyse 30, 50, or

60 is connected and FALSE if not.

/****************/

int wyse_terminal(com)

int com;

/****************/

{

char temp_s[85];

int err;

int add_info;

/*

Test to see if connected to a Wyse terminal.

Use the command  ESC<space>

Where:

<space> is a blank space.

The Wyse terminal will return 30\r, 50\r, or 60\r

*/

sprintf(temp_s, ".s'%s ' 1w'30' 2w'50' 3w'60' .d'1'",

ESC_CODE);

if ((err = AvidSendSearch(AVID, com, (long)1, (long)3,

&add_info, temp_s)) <= ERR_SNF)

return(FALSE);

else

return(TRUE);

}

Since all error codes are returned as negative numbers, the test

for less than or equal to ERR_SNF will work fine.  This just

20

means that FALSE will be returned if none of the strings in the

wait-list are found or an error is returned by AvidSendSearch.

TRUE will be returned if AvidSendSearch returns 1, 2, or 3.

Other parts of the program would then be able to make calls like

the following.

/* Before testing begins, make sure we are cable correctly. */

if ( !wyse_terminal(com)) {

AvidLogInfo(AVID, com, "Bad connection to Wyse terminal",

BAD_CONNECTION, &add_info, "");

}

Since the automation routine 'wyse_terminal' is of general use,

it could be put in a function library with other automation

routines written for wyse terminals.  The library could then be

distributed to other members of the programming staff.  Creating

general use automation routines allows automation programs to be

written like regular programs using a modular design.

The following routine is an improvement on the 'wyse_terminal'

routine.  It will return a constant that specifies the type of

wyse that is connected.

21

/****************/

int terminal_type(com)

int com;

/****************/

{

char temp_s[85];

int err;

int add_info;

/*

Test to see if connected to a Wyse terminal.

Use the command  ESC<space>

Where:

<space> is a blank space.

The Wyse terminal will return 30\r, 50\r, or 60\r

*/

sprintf(temp_s, "%s ", ESC_CODE);

AvidRegisterSend(AVID, com, temp_s, &add_info, "");

AvidRegisterWait(AVID, com, WYSE_30, "30", &add_info, "");

AvidRegisterWait(AVID, com, WYSE_50, "50", &add_info, "");

AvidRegisterWait(AVID, com, WYSE_60, "60", &add_info, "");

return(AvidSendSearch(AVID, com, (long)2, (long)5,

&add_info, ".d'1'.D'3'"));

}

Other parts of the program would then be able to make calls like

the following.

/* Before testing Wyse 50 features, make sure this is a Wyse 50.

*/

if ( terminal_type(com) != WYSE_50)

return;

Notice that the 'terminal_type' routine uses AvidRegisterSend and

AvidRegisterWait to build the send-list and wait-list.  This is a

nice way to code this routine because of the constants WYSE_30,

WYSE_50, and WYSE_60.  These constants would be defined in a

global header file and can be given any valid values.  If

switches were used like in previous examples, WYSE_30 would have

to be defined as 1, WYSE_50 as 2, and WYSE_60 as 3.  It is best

that the 'terminal_type' routine not be required to match the

constant definitions.

22

The call to AvidSendSearch uses the .d'1' and .D'3' switches to

drain the communication port.  An explicit call to AvidDrainPort

would have yielded the same result.

The above implementation of 'terminal_type' is a very good way to

code this automation routine.  However, I would like to code it a

different way to demonstrate AvidSendSearch's ability to capture

strings.  By adding the switch -c'2' and the parameters 'actual'

and 'captured' to AvidSendSearch, a string capture is possible,

as the following code segment will show.

/****************/

int terminal_type(com)

int com;

/****************/

{

char temp_s[85];

int err, add_info;

char captured[50];

long actual;

/*

Test to see if connected to a Wyse terminal.

Use the command  ESC<space>

Where:

<space> is a blank space.

The Wyse terminal will return 30\r, 50\r, or 60\r

*/

sprintf(temp_s, ".s'%s ' 1w'\r' .d'1' .D'3' -c'2'",

ESC_CODE);

if ((err = AvidSendSearch(AVID, com,

(long)2, (long)5, &add_info, temp_s,

(long *)&actual, captured)) <= ERR_SNF)

return(err);

return(atoi(captured));

}

The -c'2' switch causes two characters to be captured.  After the

Wyse 50 receives the "ESC<space>" command, it returns 3

characters.  First, the character is '5', then '0', and then the

return character '\r'.  The capture string starts out empty, then

fills up during the automation sequence.  The following list

shows how the capture string fills.

23

""        - String empty.

"5"       - First character received.

"50"      - Next character received.

"50"      - The return character received.

When the return character is received, the automation sequence

stops.  Notice that the wait string ("\r" here) is not part of

the captured string.

Suppose the Wyse 50 responds to the "ESC<space>" command

differently.  Instead of "50\r", say the Wyse 50 terminal

returned "Wyse50\r".  The code above will still work by capturing

only the string "50".  However, the capture string will fill up

differently as shown below.

""        - String empty.

"W"       - First character received.

"Wy"      - Next character received.

"ys"      - Next one.

"se"      - Next one.

"e5"      - Next one.

"50"      - Next one.

"50"      - The return character received.

This illustrates how the capture will start ignoring characters

by letting them drop off of the end.

The actual number of characters in the capture string may be less

than the amount specified in the switch.  This happens when the

automation sequence ends before the capture string is filled.

Also, since NULL characters can be part of the input stream, the

actual number of characters in the captured string is passed back

to you in the parameter 'actual'.  Strlen(captured) may not equal

the actual number of characters in the captured string due the

NULL characters.

There are actually two types of string captures possible from

AvidSendSearch.  The above sample code demonstrated the first

type, which is initiated with the switch -c'x', where x is the

maximum number of characters to capture.  The second type of

capture is invoked with the +c'x' switch, where x, again, is the

number of characters to capture.  However, there is a big

difference between these two types of captures.  The first type

(-c'x') starts pushing the oldest characters out of the capture

string after x characters have been received.  The second type of

capture (+c'x'), on the other hand, stops the automation sequence

when the capture string is full.

24

Before demonstrating the +c'x' capture, a little history.  When I

first wrote the AutoLibrary(tm) Program-Playback tool,

AvidSendSearch did not support capturing strings.  Instead, the

only way to get captured strings were to extract them from a hold

area with the routine AvidGetFromHoldArea.  Using

AvidGetFromHoldArea is still a valid way to get capture strings,

which the following code segment will show.  You will notice this

code segment gives the same results as the code segment above

that uses the -c'2' switch.

/****************/

int terminal_type(com)

int com;

/****************/

{

char temp_s[85];

int err, add_info;

char captured[50];

long actual;

/*

Test to see if connected to a Wyse terminal.

Use the command  ESC<space>

Where:

<space> is a blank space.

The Wyse terminal will return 30\r, 50\r, or 60\r

*/

sprintf(temp_s, ".s'%s ' 1w'\r' .d'1' .D'3'", ESC_CODE);

if ((err = AvidSendSearch(AVID, com, (long)2, (long)5,

&add_info, temp_s)) <= ERR_SNF)

return(err);

actual = AvidGetFromHoldArea(AVID, com, 1, 2,

captured, &add_info, "");

return(atoi(captured));

}

You will find AvidGetFromHoldArea useful in your automation

routines because of its ability to extract from anywhere in the

hold area and be able to span across data that was received from

multiple automation sequences.  AvidGetFromHoldArea is a general

utility for extracting capture strings.  However, I found that

for the vast majority of strings that I wanted to capture,

convenience was more important.  Therefore, I added the capture

switch to AvidSendSearch.  Internally, AvidSendSearch calls

AvidGetFromHoldArea with the correct parameters.  This process of

25

capturing strings seemed very natural to me.  The process where

AvidSendSearch does not worry about the capture string until the

automation sequence is over.  Then it looks backwards into the

hold area to extract out the capture string.  This method of

looking backwards into the hold area led naturally to the second

type of capture.  Namely, a forward looking capture.  One that

would start putting characters into the capture string as they

were received from the communication port.  One where

AvidSendSearch worries about the capture string before the

automation sequence starts.

The forward looking capture is invoked with the switch +c'x'.

This type of capture will be referred to as a Positive Capture.

The backward looking capture is invoked with the switch -c'x' and

is referred to as a Negative Capture.  Just remember the minus

sign '-' as the Negative Capture and the plus character '+' as

the Positive Capture.  Also, remember that the Negative Capture

looks backwards into the hold area after the automation sequence

has finished.  Where as, the Positive Capture looks forward and

builds the capture string as the automation sequence is

occurring.  The Positive Capture will terminate the automation

sequence if the capture string is full.  The Negative Capture

will never end the automation sequence.

It is useful to again code the Wyse 50's host identification

command "ESC<space>" using a Positive Capture.

26

/****************/

int terminal_type(com)

int com;

/****************/

{

char temp_s[85];

int err, add_info;

char captured[50];

long actual;

/*

Test to see if connected to a Wyse terminal.

Use the command  ESC<space>

Where:

<space> is a blank space.

The Wyse terminal will return 30\r, 50\r, or 60\r

*/

sprintf(temp_s, ".s'%s ' .d'1' .D'3' +c'2'", ESC_CODE);

if ((err = AvidSendSearch(AVID, com,

(long)2, (long)5, &add_info, temp_s,

(long *)&actual, captured)) <= ERR_SNF)

return(err);

return(atoi(captured));

}

The +c'2' switch specifies a Positive Capture of two characters.

The following list shows how the capture string is filled.

""        - String empty.

"5"       - First character received.

"50"      - Next one.

"50"      - The automation sequence ends because the

capture string is full.

- The drain switches .d'1' and .D'3' will

drain the '\r' character.

Suppose, as we did with the Negative Capture, that the Wyse 50

returned "Wyse50\r" instead of "50\r".  Remember the examples

showed the resulting capture string was "50" regardless of

whether the Wyse 50 returned "Wyse50\r" or "50\r".  The result is

different, however, for Positive Captures, as the following list

will show.

27

""        - String empty.

"W"       - First character received.

"Wy"      - Next character received.

"Wy"      - The automation sequence ends with this as

the capture string. - The drain switches then

drain the characters "se50\r" (Not what is

desired).

As you can see the result is not what was desired.  However,

since we know the Wyse 50 really will return "50\r", both capture

methods work fine.

Again, suppose the Wyse 50 worked yet another way, where say, all

terminal made before 1983 returned "Wyse50\r" and all terminals

made after 1983 returned "50\r".  To make our automation routine

work with both types of Wyse 50 terminals, the routine that uses

the Negative Capture would be the only correct solution.

I would like to describe the Positive Capture again with a better

suited example.  The previous examples were good, however, they

compared the Positive Capture with the Negative Capture in a

situation where the Negative Capture was the best choice.  Now I

would like to describe an example that is better suited for a

Positive Capture.

One place where the Positive Capture is best suited, is when the

string to key off of is before the information to be captured.

Before describing a new example, the Reference Manual does have a

good example for Positive Captures, which captures information

about the current whether conditions.  In this example, the

string "Temperature" is always displayed before the current

temperature.  First, a call is made to AvidSendSearch that waits

for the string "Temperature", then another AvidSendSearch call is

made to actually capture the temperature.  A complete description

of this example can be found in the reference guide in the

section that describes AvidSendSearch.

Another place where the Positive Capture is best suited is when

the number of characters to capture needs to have an upper limit.

The following code captures the next 80 characters of an input

stream.

AvidSendSearch(AVID, com,

(long)10, (long)60, &add_info,

"+c'80' 1w'\r' 2w'\n'", (long *)&actual, captured);

This piece of code would be used for reading in a "line" of data

much like a line of data would be read from a file.  When 80

characters have been received, AvidSendSearch will return with

28

the 80 characters in the capture string and the automation

sequence would be finished.  If, however, a newline character

('\n') or a return character ('\r') is received, AvidSendSearch

would return with less than 80 characters in the capture string.

Notice there were no send strings.  It is ok (and common) for

AvidSendSearch to be called without a send string.  For

instances, a previous call to AvidSendSearch may have already

started an input stream.  It is also possible, depending on the

type of application or device that is being automated, that an

input stream does not need initiated, i.e., characters will be

coming in as soon as the port is opened.

The following code segment adds to the previous one.  Here,

AvidSendSearch is put in a while loop.  AvidSend is used to send

the command that starts the data flowing.

AvidSend(AVID, com, "facts\r", &add_info, "");

while (( err = AvidSendSearch(AVID, com,

(long)10, (long)60, &add_info,

"+c'80' 1w'\r' 2w'\n'",

(long *)&actual, captured)) >= ERR_SNF) {

/* It is ok for err to equal ERR_SNF if caused by capture

string filling up. */

if ((err == ERR_SNF) && (add_info != ERR_CAPTURE_FULL))

break;

/* Write captured string to a file. */

fprintf(cap_file, "%s\n", captured);

}

Notice that AvidSend was used to send the command 'facts' to the

host computer.  AvidSendSearch could have been used, with

"facts\r" registered in the send-list and nothing registered in

the wait-list.  However, AvidSend is a better choice here.

AvidSendSearch would have been a better choice if any of the

other AvidSendSearch features were needed, such as, draining the

communication port.

This is the first example that uses 'add_info'.  'Add_info' is a

return parameter that allows your automation programs to get more

information on why an error occurred.  This example needs to know

why the ERR_SNF was returned.  If ERR_SNF was returned due to a

timeout, 'add_info' will return with ERR_REPEAT_TIMEOUT or

ERR_ENDING_TIMEOUT.

29

Notice the above example compares 'add_info' to ERR_CAPTURE_FULL.

This is the best way to find out if the automation sequence

should continue.  When ERR_SNF is returned due to the capture

string being full, more characters are out there to capture.

Now to summarize what we have looked at so far.  This tutorial

has introduced you to the following routines.

AvidDrainPort --- Drains a communication port.

AvidGetFromHoldArea --- Gets a string from the hold area.

AvidLogInfo --- Log messages to a log file.

AvidRegisterSend --- Registers a string in the send-list.

AvidRegisterWait --- Registers a string in the wait-list.

AvidSend --- Sends a string.

Here is a list of the routines that I would like to introduce

now.

AvidClosePort --- Closes an AutoLibrary port.

AvidExitAutoLibrary --- Exit from AutoLibrary.

AvidInitAutoLibrary --- Initializes AutoLibrary.

AvidOpenPort --- Opens an AutoLibrary port.

AvidVersion --- Displays the AutoLibrary version.

The following example uses these routines to put it all together.

The example will call the routine 'terminal_type' to determine

the type of terminal connected.  It will then print the answer to

the screen.

30

#include <stdio.h>

#include <stdlib.h>

#include "al.h"

/******/

void main(argc, argv)

/******/

int argc;

char *argv[];

{

int com;

int add_info;

/* Initial AutoLibrary. */

AvidInitAutoLibrary(AVID, &add_info, "_B'3'");

/* Open the communication port. */

com = AvidOpenPort(AVID, "com1", &add_info,

".b'9600' .1 .8 .n +S -H");

/* Display AutoLibrary version. */

AvidVersion(AVID, &add_info, "");

/* Display the type of Wyse Terminal that is out there. */

if ((term_type = terminal_type(com)) = 0)

printf("Not connected to a Wyse 30, 50, or 60\n");

else

printf("Connected to a Wyse %i terminal\n", term_type);

/* Finished so close the com port. */

AvidClosePort(AVID, com, &add_info, "");

/* Finished so exit from AutoLibrary. */

AvidExitAutoLibrary(AVID, &add_info, "");

}

31

/****************/

int terminal_type(com)

int com;

/****************/

{

char temp_s[85];

int err, add_info;

char captured[50];

long actual;

/*

Test to see if connected to a Wyse terminal.

Use the command  ESC<space>

Where:

<space> is a blank space.

The Wyse terminal will return 30\r, 50\r, or 60\r

*/

sprintf(temp_s, ".s'%s ' 1w'\r' .d'1' .D'3' -c'2'",

ESC_CODE);

if ((err = AvidSendSearch(AVID, com,

(long)2, (long)5, &add_info, temp_s,

(long *)&actual, captured)) <= ERR_SNF)

return(err);

return(atoi(captured));

}

Now all of the routines in the AutoLibrary(tm) Program-Playback

tool have been introduced in this tutorial except for the

following.

AvidConvert --- Utility routine that converts characters to

strings.

AvidLogOpen --- Open a log file for AvidLogInfo to use.

AvidProcessOptions --- Used to process the options

parameter.

AvidRead --- Reads one character.

AvidConvert and AvidProcessOptions are utility routines that your

automation programs may use.  AvidConvert converts characters

into strings.  When a character is not printable, AvidConvert

will change it to a printable string.  AvidProcessOptions is the

routine that is used internally to process the option parameter.

Your programs can use this routine if you also want to support an

32

options parameter.  AvidLogOpen opens a log file for AvidLogInfo

to use.

AvidRead is used internally by AvidDrainPort and AvidSendSearch.

Your automation programs should not require this routine.  It was

included only for completeness.  The Reference Guide has a more

information on these routines.

This concludes the tutorial of the AutoLibrary(tm) Program-

Playback tool.  I hope this section has given you a feel for how

automation is done using this tool.

33

Reference Guide For the AutoLibrary(tm) Program-Playback tool

The following is the reference guide for the AutoLibrary(tm)

Program-Playback tool.  Each routine is described along with its

parameters and options.  Examples are supplied for each routine.

34

AvidClosePort - Close Communication Port

err = AvidClosePort(AVID, com, (int *)&add_info, opts)

Parameter Description

int err;       -- Return code.

AVID      -- System parameter that must always be specified.

int com;  -- The communications port to close.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

NONE

Return Codes

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_COMM_PACKAGE    -- This error can occur when the

communication package cannot close the

communication port correctly.  When this

error occurs, the parameter 'add_info' will

contain the actual error returned by the

communication package.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

ERR_OK         -- No error occurred.

<system err>   -- When ERR_COMM_PACKAGE is returned, 'add_info'

contains the actual error returned by the

communication package.

AvidClosePort

35

Description

This routine closes the communication port that was opened by

AvidOpenPort.  All of the memory allocated by AvidOpenPort is

freed.  This routine calls CommInterfaceClose so the

communication port of the comm package can be closed.  This

routine will close the log if one was opened by AvidLogOpen.

Examples

The following example assumes there will not be any problems

closing the port.

AvidClosePort(AVID, com, &add_info, "");

The following example checks for problems when closing the port.

if ((err = AvidClosePort(AVID, com, &add_info, "")) != ERR_OK)

printf("Error = %d, Additional Info = %d\n", err, add_info);

See Also

AvidOpenPort, CommInterfaceClose

AvidClosePort

36

AvidConvert - Convert Characters to Strings

err = AvidConvert(c, s, (int *)&add_info, opts)

Parameter Description

int err;       -- Return code.

int c;         -- Character to be converted.

char *s;       -- Returned string.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

NONE

Return Codes

ERR_OK         -- No error occurred.

Additional Information Return Codes

ERR_OK         -- No error occurred.

Description

This routine converts characters into strings.  Each of the non-

printable characters are converted into a coded string.  For

example, the character '\0' would return as "[NUL]".  This

routine is useful in EveryChar, which was the main reason this

routine was written.  The following lists how each of the non-

printable characters are returned.

0    - [NUL]

1    - [SOH]

2    - [STX]

3    - [ETX]

4    - [EOT]

5    - [ENQ]

6    - [ACK]

7    - [BEL]

8    - [BS]

AvidConvert

37

9    - [HT]

10   - [LF]

11   - [VT]

12   - [FF]

13   - [CR]

14   - [SO]

15   - [SI]

16   - [DLE]

17   - [DC1]

18   - [DC2]

19   - [DC3]

20   - [DC4]

21   - [NAK]

22   - [SYN]

23   - [ETB]

24   - [CAN]

25   - [EM]

26   - [SUB]

27   - [ESC]

28   - [FS]

29   - [GS]

30   - [RS]

31   - [US]

127  - [DEL]

Examples

The following code will print all characters received from the

communication port in a tty like mode.

/*****/

void EveryChar(c)

int c;

/*****/

{

char s[20];

int add_info;

AvidConvert(c, s, &add_info, "");

printf("%s", s);

}

See Also

EveryChar

AvidConvert

38

AvidDrainPort - Drain Communication Port

err = AvidDrainPort(AVID, com, (long)r_drain, (long)e_drain,

(int *)&add_info, opts)

Parameter Description

int err;       -- Return code.

AVID           -- System parameter that must always be specified.

int com;       -- The communications port to drain.

long r_drain;  -- The number of seconds to drain the

communication port.  The drain time will start

over after each character received.  (A repeating

drain)

The parameter constant AVID_NO_TIMEOUT will cause

AvidDrainPort to never stop draining due to a

repeating drain value.

NOTE: AvidDrainPort will drain forever if

AVID_NO_TIMEOUT is used for both 'r_drain' and

'e_drain'.

long e_drain;  -- The number of seconds to drain the

communication port.  The drain time will NOT start

over after each character received.  (An ending

drain)

The parameter constant AVID_NO_TIMEOUT will cause

AvidDrainPort to never stop draining due to an

ending drain value.

NOTE: AvidDrainPort will drain forever if

AVID_NO_TIMEOUT is used for both 'r_drain' and

'e_drain'.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

NONE

AvidDrainPort

39

Return Codes

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_COMM_PACKAGE    -- This error can occur when AvidDrainPort

reads from the communication port.  When this

error occurs, the parameter 'add_info' will

contain the actual error returned by the

communication package.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

ERR_OK              -- No error occurred.

<system err>        -- When ERR_COMM_PACKAGE is returned,

'add_info' contains the actual error returned

by the communication package.

Description

This routine drains a communication port.  The drain time can be

specified as a repeating timeout or as an ending timeout.  This

routine can be called out explicitly or it is possible to drain a

communication line directly from AvidSendSearch by using

switches.

Examples

The following example will drain the communication port until

there is 2 seconds of communications silents.  If there is not

communications silents for 2 seconds, the drain will stop after

20 seconds.

err = AvidDrainPort(AVID, com, (long)2, (long)20, &add_info, "");

The following example will drain the communication port until

there is 4 seconds of communications silents.  If there is never

communications silents this call will never return.

AvidDrainPort

40

err = AvidDrainPort(AVID, com, (long)4, AVID_NO_TIMEOUT,

&add_info, "");

The following example will drain for 5 seconds paying no

attention to the communication activity.

err = AvidDrainPort(AVID, com, AVID_NO_TIMEOUT, (long)5,

&add_info, "");

See Also

AvidSendSearch (Especially .D and .d switches)

AvidDrainPort

41

AvidExitAutoLibrary - Exit AutoLibrary

err = AvidExitAutoLibrary(AVID, (int *)&add_info, opts)

Parameter Description

int err;       -- Return code.

AVID           -- System parameter that must always be specified.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

NONE

Return Codes

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_CANNOT_EXIT     -- All of the AutoLibrary ports need to be

closed first with AvidClosePort.

Additional Information Return Codes

ERR_OK              -- No error occurred.

Description

This routine exits from the AutoLibrary(tm) Program-Playback

tool.  All memory and resources used by this tool are returned.

Once AvidExitAutoLibrary is called, it is possible to call

AvidInitAutoLibrary to again initialize the AutoLibrary(tm)

Program-Playback tool.

Examples

The following call is made after the last opened port is closed

by AvidClosePort.

AvidExitAutoLibrary

42

err = AvidExitAutoLibrary(AVID, &add_info, "");

See Also

AvidInitAutoLibrary

AvidExitAutoLibrary

43

AvidGetFromHoldArea - Get Characters from Hold Area

actual = AvidGetFromHoldArea(AVID, com, (long)pos, (long)size,

s, (int *)&add_info, opts)

Parameter Description

long actual;   -- This is the actual number of characters in s.

This may not match strlen(s) if null characters

were received from the hold area.

AVID           -- System parameter that must always be specified.

int com;       -- The AutoLibrary communications port.

long pos;      -- This is the position in the hold area to start

getting characters that will make up the string

returned.

long size;     -- This is the size of the string to get.

char *s;       -- This is the string that was built by

AvidGetFromHoldArea.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

NONE

Return Codes

<actual>  -- Returns the number of characters in the string.

Zero is returned when there is no characters in the

hold area or if erroneous values where passed to it.

For example, negative seven for the position or a

position that is specified outside the hold area

boundaries.

Additional Information Return Codes

ERR_OK    -- No error occurred.

Description

AvidGetFromHoldArea

44

This routine will get characters from the hold area.  It is also

possible to capture characters directly from the AvidSendSearch

routine by specifying switches.

Examples

The following example will get 5 characters from the hold area

starting at position 3.  If the hold area had the characters

"0123456789", 'capture' would have the string "34567".

actual = AvidGetFromHoldArea(AVID, com, (long)3, (long)5,

capture, &add_info, "");

After the following example executed 'capture' would have the

string "12" given the same hold area as above.

actual = AvidGetFromHoldArea(AVID, com, (long)8, (long)2,

capture, &add_info, "");

See Also

AvidSendSearch (Especially +c and -c switches)

AvidGetFromHoldArea

45

AvidInitAutoLibrary - Initialize AutoLibrary

err = AvidInitAutoLibrary(AVID, (int *)&add_info, opts)

Parameter Description

int err;       -- Return code.

AVID           -- System parameter that must always be specified.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

_b'num'   -- This changes the number of milliseconds to delay

before sending the first character of the string.  The

specified value will replace the default permanently.

(Default is 0).

_B'num'   -- This changes the number of milliseconds to delay

between each character sent.  The specified value will

replace the default permanently. (Default is 0).

+h'num'   -- This option changes the size of the hold area.  This

value will automatically be increased if the chosen

value is less than AvidSendSearch's capture size

(SEND_SEARCH_CAPTURE_MAX).  (Default is 512).

+R        -- This switch is used after your copy of AutoLibrary

is registered.  This will initialize AutoLibrary

without printing "Unregistered Evaluation Copy" to the

screen.

Unregistered users are not allowed to use this switch.

However, if the actual printing of "Unregistered

Evaluation Copy" interferes with your ability to

evaluate AutoLibrary, you may temporarily use this

switch.

Note: The +R switch does not change the functionality

of AutoLibrary.  Both the registered and unregistered

initialization methods result in fully functional

software.

AvidInitAutoLibrary

46

Return Codes

ERR_OK                   -- No error occurred.

ERR_ALREADY_INITIALIZED  -- This routine cannot be called more

than once without first call

AvidExitAutoLibrary.

Additional Information Return Codes

ERR_OK    -- No error occurred.

Description

This routine is used to initialize the AutoLibrary(tm) Program-

Playback tool.  This routine must be the first routine called.

Use AvidExitAutoLibrary to uninitialize.

Examples

The following example initializes AutoLibrary for the

unregistered libraries.  (The unregistered libraries will print

the message "Unregistered Evaluation Copy".

err = AvidInitAutoLibrary(AVID, &add_info, "");

The following example initialize AutoLibrary for the registered

libraries.  The +R switch causes the printf to not occur.

err = AvidInitAutoLibrary(AVID, &add_info,"+R");

See Also

AvidExitAutoLibrary

AvidInitAutoLibrary

47

AvidLogInfo - Log information to file or screen.

val = AvidLogInfo(AVID, com, title, user_code,

(int *)&add_info, opts)

Parameter Description

int val;       -- Return code or if *E switch is specified, the

number of logged errors is returned.

AVID           -- System parameter that must always be specified.

int com;       -- The AutoLibrary communication port.

char *title;   -- A one line string to log.

int user_code; -- An integer value that can be used in any way to

add information to the log entry.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

+p   -- Print the log message to the screen using printf.  This

switch guarantees the message will be printed, regardless of

the current default.  The default is to print, but this can

be changed by AvidLogOpen.

-p   -- Do not print the message to the screen.  This switch

guarantees the message will not be printed, regardless of

the current default.  The default is to print, but this can

be changed by AvidLogOpen.

+E   -- After the error has been logged, 'exit(1)' is called to

abort further processing.  This switch guarantees processing

will abort, regardless of the current default.  The default

is to continue processing, but this can be changed by

AvidLogOpen.  This switch will only have effect on error

messages, i.e., messages that use the *E switch.

-E   -- After message has been logged, continue processing.  This

switch guarantees processing will continue, regardless of

the current default.  The default is to continue processing,

but this can be changed by AvidLogOpen.

AvidLogInfo

48

+L   -- Log the message to the log file.  This switch guarantees

the message will be logged, regardless of the current

default.  The default is to log all messages, but this can

be changed by AvidLogOpen.  Also, this switch is only valid

after AvidLogOpen is called to open the log file.

-L   -- Do not log message in the log file.  This switch

guarantees the message will not be logged, regardless of the

current default.  The default is to log all messages, but

this can be changed by AvidLogOpen.

*E   -- This option directs AvidLogInfo to print the log entry as

an error.  The string "ERROR(int)," is added to the log

message.  AvidLogInfo returns the number of errors logged

when this switch is specified.  The integer that is printed

as part of ERROR(int) is the 'user_code' parameter.

*T   -- This option directs AvidLogInfo to add a date and time

stamp to the log entry.

*L   -- This option directs AvidLogInfo to print information that

identifies where AvidLogInfo was called from in your source

code.  The line number and source file name will be added to

the log message.  With this information, errors can be

traced back to the exact source line they occurred.

Return Codes

<Positive Number>   -- The number of logged errors.  This is

possible only when the *E switch is used.

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_WRITING         -- There was a problem writing to the log

file.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

AvidLogInfo

49

ERR_OK              -- No error occurred.

<system err>        -- When ERR_WRITING is returned, 'add_info'

will contain the system error that was

returned by the clib call to 'fprintf'.

Description

This routine makes it possible to log a message to a file, to the

screen, or to both.  Switches are used to customize the format of

a log message.  The examples below will show some of the ways

this routine can be used.

Examples

The following example will use this routine to log when a test

has started.  The 'user_code' is set to zero and is not used,

since the *E switch was not specified.  The *T switch will add a

time stamp to the logged message, which will document when the

test started.  The output from this call is also shown.

/* Log the type of test and when it started. */

AvidLogInfo(AVID, com, "Executing screen location test",

0, &add_info, "*T");

OUTPUT:

Mon Jun 24 17:36:12 1991,Executing screen location test

The following example will log an error.  Notice the error

returned from AvidSendSearch is specified as the 'user_code'

parameter.  This will help determine the cause of the error.  The

switches *E, *T, and *L add more information to the error

message.  The output from this call is also shown.

AvidLogInfo

50

/* Make sure the cursor is where we placed it with ESC b */

sprintf(temp_buf, ".s'%sb' 1w'C' -c'6'", ESC_CODE);

if ((err = AvidSendSearch(AVID, com,

(long)10, (long)30, &add_info, temp_buf,

(long *)&actual, capture)) <= ERR_SNF) {

num_logged = AvidLogInfo(AVID, com,

"Cursor not placed correctly",

err, &add_info, "*ETL");

}

OUTPUT:

ERROR(-1002),Mon Jun 24 17:36:12 1991,(TST.C,74),Cursor not

placed correctly

The following example does not log the entry to the log file.

Instead, the message is printed to the screen.  The output from

this call is also shown.

/* Print Welcome message. */

AvidLogInfo(AVID, com, "Welcome", 0, &add_info, "-L");

OUTPUT:

Welcome

See Also

AvidLogOpen

AvidLogInfo

51

AvidLogOpen - Open a Log File.

err = AvidLogOpen(AVID, com, filename, (int *)&add_info, opts)

Parameter Description

int err;            -- Return code.

AVID                -- System parameter that must always be

specified.

int com;            -- The AutoLibrary communication port.

char *filename;     -- The file to open as a log file.

int *add_info;      -- Additional return information.

char *opts;         -- This is the input parameter for options.

Options

-p   -- This option changes the default for AvidLogInfo to not

print the log message to the screen.  The normal default

prints messages to the screen using 'printf'.

+E   -- This option changes the default for AvidLogInfo to abort

processing after the error has been logged.  The normal

default continues processing after logging errors.  This

switch will only have effect on error messages, i.e., those

messages that use the *E switch.

Return Codes

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_OPENING         -- There was a problem opening the log file.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

AvidLogOpen

52

ERR_OK              -- No error occurred.

Description

This routine is used to open a log file that AvidLogInfo will

use.  The switches that this routine supports allows AvidLogInfo

to have different defaults.  AvidLogOpen is needed only to open a

log file and is not required before AvidLogInfo is called.

However, if AvidLogOpen is not called, AvidLogInfo is restricted

to logging messages to the screen.

Examples

The following example will open a log file called "logfile".

err = AvidLogOpen(AVID, com, "logfile", &add_info, "");

The following example will again open a log file called

"logfile".  However, it will also change the defaults for

AvidLogInfo to not print log entries to the screen and to abort

processing after logging error messages.

err = AvidLogOpen(AVID, com, "logfile", &add_info, "-p +E");

See Also

AvidLogInfo

AvidLogOpen

53

AvidOpenPort - Open Communication Port

com = AvidOpenPort(AVID, device, (int *)&add_info, opts)

Parameter Description

int com;       -- This is the AutoLibrary port identification

number that is used by all of the AutoLibrary

routines.

AVID           -- System parameter that must always be specified.

char *device;  -- This is the device name of the communication

port to open.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

_b'num'   -- This changes the number of milliseconds to delay

before sending the first character of the string.  The

specified value will replace the default permanently

for this port.  (Default is 0).

_B'num'   -- This changes the number of milliseconds to delay

between each character sent.  The specified value will

replace the default permanently for this port. (Default

is 0).

+h'num'   -- This option changes the size of the hold area for

this port.  This value will automatically be increased

if the chosen value is less than AvidSendSearch's

capture size (SEND_SEARCH_CAPTURE_MAX).  (Default is

512).

Options (Depends on implementation of CommInterfaceOpen)

.b'baud'  -- This will change the baud rate from the default of

9600 to the specified value.  Choose from:  50, 75,

110, 134.5, 150, 300, 600, 1200, 1800, 2000, 2400,

3600, 4800, 7200, 9600, 19200. (Default is 9600).

.1        -- One stop bit will be used. (Default).

AvidOpenPort

54

.2        -- Two stop bits will be used. (Default is 1).

.7        -- Seven bits per word. (Default is 8).

.8        -- Eight bits per word. (Default).

.e        -- Even parity will be used. (Default is odd).

.o        -- Odd parity will be used. (Default).

.n        -- No parity will be used. (Default is odd).

-S        -- This will cause the 7th bit to NOT be stripped.

(Default is to strip).

+S        -- This will cause the 7th bit to be stripped.

(Default).

+X        -- This will cause software (xon-xoff) flow control to

be used. (Default).

-X        -- This will cause software (xon-xoff) flow control to

not be used. (Default is on).

.T        -- This will cause dtr to be asserted. (Default is not

to).

rx'num'   -- Changes the default receive buffer size. (Default is

8192).

tx'num'   -- Changes the default transmit buffer size. (Default

is 512).

The Greenleaf Comm Library

This is a commerical communication package provided by Greenleaf

Software, Inc.  16479 Dallas Parkway, Bent Tree Tower Two, Suite

570, Dallas, TX  75248,  (214)248-2561.

(Note: This is just one of many good communication packages.  It

is easy to write interface routines for the AutoLibrary Program-

Playback Tool, so don't feel you are tied to any one

communication package.)

Options Supported

AvidOpenPort

55

.b'num'  .1  .2  .7  .8  .e  .o  .n  -S  +S  +X  -X  .T  rx'num'

tx'num'

Baud rates supported are:  50, 75, 110, 134.5, 150, 300, 600,

1200, 1800, 2000, 2400, 3600, 4800, 7200, 9600, 19200. (Default

is 9600).

The ifsrtcl.lib Comm Library

This is a basic communication package supplied with this product

that combines the interface routines with asynchronous

communication software.  It is intended to allow you to evaluate

AutoLibrary and make it easy to get started.

ERR_QUEUE_FULL is returned if the send queue is full.

ERR_INVALID_PORT is returned if something other than com1 or com2

is specified.

Options Supported

.b'num' .1  .2  .7  .8  .e  .o  .n

Baud rates supported are:  110, 150, 300, 600, 1200, 2400, 4800,

9600. (Default is 9600).

Also, the default is to use receive Xon/Xoff, and cannot be

changed.  Transmit Xon/Xoff is not supported, i.e., this package

will ignore Xoff characters.  However, the Xon/Xoff the protocol

will be used if characters are received faster than they are

processed.  The restriction works out ok because you can control

how many characters you send but cannot control how many and how

fast characters are received.

Return Codes

<Positive number>   -- No error occurred.  The positive number is

the AutoLibrary port number.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_COMM_PACKAGE    -- This error can occur when the

communication package cannot open the

communication port correctly.  When this

error occurs, the parameter 'add_info' will

AvidOpenPort

56

contain the actual error returned by the

communication package.

ERR_EXCEEDED_COMLIST_SIZE     -- This will occur when an attempt

is made to open more than

COMLIST_SIZE.  The default for this

value is 30, which can be changed

by re-compiling AutoLibrary.

ERR_NO_MEMORY       -- This will occur when there is no more

memory to allocate required data structures.

Additional Information Return Codes

ERR_OK              -- No error occurred.

<system err>        -- When ERR_COMM_PACKAGE is returned,

'add_info' contains the actual error returned

by the communication package.

Description

This routine opens an AutoLibrary communication port.  This

routine creates the required data structures to support

automation and calls CommInterfaceOpen to open the hardware port

of the communication package supplied.

IMPORTANT:  Note: the list of options for this routine are

dependent on how the interface library routine CommInterfaceOpen

was implemented.

Examples

The following example opens the communication port at 9600 baud,

one stop bit, eight bits per word, no parity, strip 7th bit, and

do not use hardware flow control.

com = AvidOpenPort(AVID, "com1", &add_info,

".b'9600' .1 .8 .n +S -H");

See Also

AvidClosePort, CommInterfaceOpen

AvidOpenPort

57

AvidProcessOptions - Parse an Option Parameter List

o_char = AvidProcessOptions(AVID, o_list, target,

(char *)&control, arg_s, (int *)&start)

Parameter Description

char o_char;   -- This is the character part of the option.  This

will have a '\0' (null) value when there are no

more options to process.  If the switch was -

X'good', for example, 'o_char' would return 'X'.

AVID           -- System parameter that must always be specified.

char *o_list;  -- This is the list of options.

char *target;  -- Look only for the option specified as the

target.  This makes it possible to scan for the

target option without using a while loop.

char *control; -- This is the option control character of the

parsed option.  If the switch was -X'good', for

example, 'control' would return '-'.

char *arg_s;   --  This is where the options argument is written.

If the switch does not have an argument, a null

string is returned.  If the switch was -X'good',

for example, 'arg_s' would return "good".

int *start;    -- The location to start parsing in the option

list.  This parameter must be initialized to zero,

then left alone.  It is used internally by

AvidProcessOptions to track which switch to return

next.

Options

NONE

Return Codes

Not applicable.

Description

AvidProcessOptions

58

This is the routine that parses the option parameters that every

AutoLibrary routine has.  AvidProcessOptions is not needed to

support automation, however, it has been documented and made

public because it is a nice routine of general utility.

AvidProcessOptions

59

Examples

AvidProcessOptions can be used two ways.  This example shows the

first way, where the 'target' parameter is given a specific

switch to look for, namely, "-L".  AvidProcessOptions won't

return until it finds this specific switch or it returns NULL.

This method eliminates the need to call AvidProcessOptions in a

loop.

/* Find out if over night tests should be done. */

if (AvidProcessOptions(AVID, arg_str, "-L",

&control, arg, &start) != '\0')

long_version = TRUE;

else

long_version = FALSE;

The second way to use AvidProcessOptions is to call it in a while

loop.  The 'target' parameter is left empty, which tells

AvidProcessOptions to return the next switch.  The 'sc' variable

returns with the switch character and the 'control' variable

returns with the control character.  It is important to

initialize the 'start' parameter to zero, because this variable

is used to determine which switch to return.  If, for example,

the 'start' parameter was set to zero inside the while loop,

AvidProcessOptions would always return the first switch.

/* Find out what kind of batch mode testing to do. */

start=0; /* Required by AvidProcessOptions. */

while ((sc=AvidProcessOptions(AVID, arg_str, "",

&control, arg, &start)) != '\0') {

/* Execute print test if "+P" switch was specified. */

if ((control == '+') && (sc == 'P')) {

initialize_testing(com);

test_scr_print(com, 20, 80);

}

/* Execute screen attributes test if "-a" switch was

specified. */

if ((control == '-') && (sc == 'a')) {

initialize_testing(com);

test_scr_attribs(com);

}

}

AvidProcessOptions

60

AvidRead - Read a Character

c = AvidRead(AVID, com, (long)r_timeout, (long)e_timeout,

(int *)&add_info, opts)

Parameter Description

int c;              -- The character received is returned.

AVID                -- System parameter that must always be

specified.

int com;            -- The AutoLibrary port number.

long r_timeout;     -- The number of seconds to wait for a

character.  The timeout value starts over

after each character received.

long e_timeout;     -- The total number of seconds wait for a

character plus the current number of seconds.

The timeout value does NOT start over after

each character received.

AvidRead is different from the other

AutoLibrary routines that use an ending

timeout.  For AvidRead the current number of

seconds since Jan 1, 1970 must be added to

the delay before making the call.  Use the

following code segment.

time(seconds);

if (e_timeout != AVID_NO_TIMEOUT)

e_timeout = e_timeout + seconds;

( make call to AvidRead here )

int *add_info;      -- Additional return information.

char *opts;         -- This is the input parameter for options.

Options

NONE

Return Codes

AvidRead

61

<character>         -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_COMM_PACKAGE    -- This error can occur when the

communication package cannot read the

communication port correctly.  When this

error occurs, the parameter 'add_info' will

contain the actual error returned by the

communication package.

ERR_EOF             -- There were no more characters to read or a

timeout occurred.  If a timeout occurred,

'add_info' will have the values

ERR_REPEAT_TIMEOUT or ERR_ENDING_TIMEOUT.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

ERR_OK              -- No error occurred.

<system err>        -- When ERR_COMM_PACKAGE is returned,

'add_info' will contain the actual error

returned by the communication package.

ERR_REPEAT_TIMEOUT  -- When ERR_EOF is returned and the cause for

the timeout is a Repeating Timeout, this is

the value returned.

ERR_ENDING_TIMEOUT  -- When ERR_EOF is returned and the cause is

an Ending Timeout, this is the value

returned.

Description

This routine will read one character at a time from the

communication port.  This routine is supplied only for

completeness and is not required for automation.  Internally

AvidSendSearch and AvidDrainPort call this routine to read

characters.  The ending timeout must be fixed up as described

above in the parameter definition of 'e_timeout'.

AvidRead

62

AvidSendSearch should be used to read characters into a buffer,

instead of AvidRead.

Examples

See the source code for AvidSendSearch or AvidDrainPort.

See Also

CommInterfaceRead

AvidRead

63

AvidRegisterSend - Register a String to Send

err = AvidRegisterSend(AVID, com, s_string,

(int *)&add_info, opts)

Parameter Description

int err;            -- Return code.

AVID                -- System parameter that must always be

specified.

int com;            -- The AutoLibrary port number.

char *s_string;     -- This is the string to be sent by

AvidSendSearch.

int *add_info;      -- Additional return information.

char *opts;         -- This is the input parameter for options.

Options

_b'num'   -- This changes the number of milliseconds to delay

before sending the first character of the string.  The

specified value will replace the default only for this

invocation.  Use AvidInitAutoLibrary or AvidOpenPort to

change the defaults permanently. (Default is 0).

_B'num'   -- This changes the number of milliseconds to delay

between each character sent.  The specified value will

replace the default only for this invocation.  Use

AvidInitAutoLibrary or AvidOpenPort to change the

defaults permanently. (Default is 0).

.N'num'   -- This is the number of null characters in the string.

For example, .N'3' would be specified for the string

"\n\r\0\0abc\0".

Return Codes

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

AvidRegisterSend

64

ERR_NO_MEMORY       -- This error will occur when there is not

enough memory to allocate the required data

structures.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

ERR_OK              -- No error occurred.

Description

This routine will register a string to be sent when

AvidSendSearch is called.  AvidRegisterSend does not send the

string, it puts it into a linked list.

Note that calling AvidRegisterSend is optional since send strings

can be registered directly from AvidSendSearch by using switches.

Examples

The following example registers the specified string for

AvidSendSearch to send.

err = AvidRegisterSend(AVID, com, "hi there\r", &add_info, "");

See Also

AvidRegisterWait, AvidSendSearch (Especially the .s switch)

AvidRegisterSend

65

AvidRegisterWait - Register a Wait String

err = AvidRegisterWait(AVID, com, r_num, w_string,

(int *)&add_info, opts)

Parameter Description

int err;            -- Return code.

AVID                -- System parameter that must always be

specified.

int com;            -- The AutoLibrary port number.

int r_num;          -- This is the registered number to be

returned by AvidSendSearch if the string is

found.

Numbers from 20,000 to max integer are

reserved by Avid Software for future use and

internal implementations.

char *w_string;     -- This is the string to add to the wait-

list.  If this string is found by

AvidSendSearch, the registered value will be

returned.

int *add_info;      -- Additional return information.

char *opts;         -- This is the input parameter for options.

Options

.N'num'   -- This is the number of null characters in the string.

For example, .N'3' would be specified for the string

"\n\r\0\0abc\0".

Return Codes

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

AvidRegisterWait

66

ERR_NO_MEMORY       -- This error will occur when there is not

enough memory to allocate the required data

structures.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

ERR_OK              -- No error occurred.

Description

This routine is used to register a string for AvidSendSearch to

scan for.  This routine puts the wait string in a linked list.

This routine can be called as many times as required.  Each time

it is called another wait string gets added to the linked list.

The value that is registered with AvidRegisterWait is the same

value that AvidSendSearch will return if the wait string is

found.  Wait strings can also be specified as a switch to

AvidSendSearch.  This way calling AvidRegisterWait directly is

not required.

Examples

The following example registers the specified string for

AvidSendSearch to wait for.  If the string is found,

AvidSendSearch will return 100.

err = AvidRegisterWait(AVID, com, 100,

"This is a string", &add_info, "");

See Also

AvidRegisterSend, AvidSendSearch (Especially the 1w switch)

AvidRegisterWait

67

AvidSend - Send a String

err = AvidSend(AVID, com, s, (int *)&add_info, opts)

Parameter Description

int err;       -- Return code.

AVID           -- System parameter that must always be specified.

int com;       -- The AutoLibrary port number.

char *s;       -- This is the string to send.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

_b'num'   -- This changes the number of milliseconds to delay

before sending the first character of the string.  The

specified value will replace the default only for this

invocation.  Use AvidInitAutoLibrary or AvidOpenPort to

change the defaults permanently. (Default is 0).

_B'num'   -- This changes the number of milliseconds to delay

between each character sent.  The specified value will

replace the default only for this invocation.  Use

AvidInitAutoLibrary or AvidOpenPort to change the

defaults permanently. (Default is 0).

.N'num'   -- This is the number of null characters in the string.

For example, .N'3' would be specified for the string

"\n\r\0\0abc\0".

Return Codes

ERR_OK              -- No error occurred.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_COMM_PACKAGE    -- This error can occur when the

communication package cannot send through the

AvidSend

68

communication port correctly.  When this

error occurs, the parameter 'add_info' will

contain the actual error returned by the

communication package.

ERR_NO_STRING_SPECIFIED  -- This error occurs when there was no

string specified.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

ERR_OK         -- No error occurred.

<system err>   -- When ERR_COMM_PACKAGE is returned, 'add_info'

contains the actual error returned by the

communication package.

Description

This routine sends the specified string.

Examples

The following example sends the specified string.

err = AvidSend(AVID, com, "Hello\r", &add_info, "");

See Also

AvidRegisterSend, CommInterfaceSend, AvidSendSearch

AvidSend

69

AvidSendSearch - Send Strings then Search

s_found = AvidSendSearch(AVID, com,

(long)r_timeout, (long)e_timeout, (int *)&add_info,

opts, (long *)&actual, cap)

Parameter Description

int s_found;        -- The registered value of the string that

was found.  ERR_SNF, which is equal to zero,

is returned if none of the registered strings

were found.

AVID                -- System parameter that must always be

specified.

int com;            -- The opened communication port.

long r_timeout;     -- The number of seconds to scan for the

strings in the wait-list.  The timeout value

starts over after each character received.

(A repeating timeout)

The parameter constant AVID_NO_TIMEOUT will

cause AvidSendSearch to never timeout due to

a repeating timeout.

long e_timeout;     -- The number of seconds to scan for the

strings in the wait-list.  The timeout value

does NOT start over after each character

received.  (An ending timeout)

The parameter constant AVID_NO_TIMEOUT will

cause AvidSendSearch to never timeout due to

an ending timeout.

int *add_info;      -- Additional return information.

char *opts;         -- This is the input parameter for options.

<long *actual;>     -- This is the actual number of characters

captured.  May not equal strlen(cap) if cap

received null characters as part of the data.

(This parameter is optional see the +c and

-c options below.)

AvidSendSearch

70

<char *cap;>        -- This is the capture string. (This

parameter is optional, see the +c and -c

options below.)

Options

+c'size'       -- This will cause AvidSendSearch to capture each

received character until 'size' number of

characters have been received.  The captured

string will be smaller that 'size' if a timeout

occurs or one of the strings in the wait-list was

found.

-c'size'       -- This will cause AvidSendSearch to capture

received characters until one of the strings in

the wait-list was found or a timeout occurred.

Truncation will occur if more than 'size' number

of characters are received before AvidSendSearch

is finished.

.d'drain_time' -- This is the drain time in seconds.  The drain

time will start over with each new received

character. (Repeating drain)

.D'drain_time' -- This is the drain time in seconds.  The drain

time will NOT start over with each new received

character. (Ending drain)

1w'string'     -- This is a wait string.  AvidSendSearch will

return 1 if the string is found.  Numbers 1

through 9 are valid for this option.  For example,

2w'string' will register 'string' with 2 and

9w'string' will register it with 9.

.s'string'     -- This is a send string.

Return Codes

<registered value>  -- When one of the wait strings is found, the

registered value is returned.

ERR_SNF             -- SNF (String Not Found).  This is returned

when none of the wait strings were found.

The variable 'add_info' can be tested to find

out additional information about why ERR_SNF

was returned.  The possible values for

AvidSendSearch

71

'add_info' are ERR_ENDING_TIMEOUT,

ERR_REPEAT_TIMEOUT, and ERR_CAPTURE_FULL.

ERR_NOT_INITIALIZED -- This error is returned when

AvidInitAutoLibrary is not called before this

routine is called.

ERR_NO_MEMORY       -- This error can be returned if the .s or 1w

switch is used.  It is possible for

AvidRegisterSend and AvidRegisterWait to not

have the required memory to allocate data

structures.

ERR_COMM_PACKAGE    -- This error can occur when the strings in

the send-list are being sent or when

characters are being read looking for the

strings in the wait-list.  When this error

occurs, the parameter 'add_info' will contain

the actual error returned by the

communication package.

ERR_INVALID_PORT    -- This error can occur when the

communication port specified in the 'com'

parameter was not a valid opened port.

Additional Information Return Codes

ERR_OK              -- No error occurred.

<system err>        -- When ERR_COMM_PACKAGE is returned,

'add_info' contains the actual error returned

by the communication package.

ERR_REPEAT_TIMEOUT  -- When ERR_SNF is returned and the cause for

the timeout is a Repeating Timeout, this is

the value returned.

ERR_ENDING_TIMEOUT  -- When ERR_SNF is returned and the cause is

an Ending Timeout, this is the value

returned.

ERR_CAPTURE_FULL    -- When ERR_SNF is returned and the cause is

due to a full capture string, this is the

value returned.

Description

AvidSendSearch

72

This routine is the main routine used for automation.

AvidSendSearch sends all of the strings in the send-list, then

scans for the strings in the wait-list.  If one of the strings in

the wait-list is found, AvidSendSearch returns with that wait

string's registered value.  If none of the wait strings are found

within the specified timeout period, AvidSendSearch returns with

ERR_SNF.  (SNF is String Not Found).

It is possible to capture characters while AvidSendSearch is

scanning for the strings in the wait-list using the switches +c

or -c.  It is also possible to specify drain times with switches.

If specified, draining will occur after one of the strings in the

wait-list is found.

The send-list and wait-list can be built by calling

AvidRegisterSend and AvidRegisterWait, respectively.  The send-

list and wait-list can also be built in AvidSendSearch by using

switches.  Using switches eliminates the need to call

AvidRegisterSend and AvidRegisterWait.

Examples

The following example sends the string "exit\r" then waits for

the string "login:".  Calls to AvidRegisterSend and

AvidRegisterWait are used to add the strings to the linked list

for AvidSendSearch.

/* Register the send string. */

AvidRegisterSend(AVID, com, "exit\r", &add_info, "");

/* Register the wait string. */

AvidRegisterWait(AVID, com, 1, "login:", &add_info, "");

/* Send the exit command, then wait for the login: prompt. */

if (AvidSendSearch(AVID, com, (long)5, (long)45,

&add_info, "") <= ERR_SNF)

printf("Exit failed\n");

The following example is the same as the one above, except the

send string and wait string are registered by using

AvidSendSearch switches.

/* Send the exit command, then wait for the login: prompt. */

if (AvidSendSearch(AVID, com, (long)5, (long)45,

&add_info, ".s'exit\r' 1w'login:'") <= ERR_SNF)

printf("Exit failed\n");

AvidSendSearch

73

Notice in the following example that it is possible to add more

than one wait string using AvidRegisterWait.  AvidSendSearch will

return the registered string value of the string it finds.  In

this example, either 1 or 2.

/* Register the send string. */

AvidRegisterSend(AVID, com, "exit\r", &add_info, "");

/* Register the wait string. */

AvidRegisterWait(AVID, com, 1, "login:", &add_info, "");

/* Register the wait string. */

AvidRegisterWait(AVID, com, 2, "Enter Data Switch Number:",

&add_info, "");

/* Send the exit command, then wait for the login: prompt. */

if (AvidSendSearch(AVID, com, (long)5, (long)45,

&add_info, "") <= ERR_SNF)

printf("Exit failed\n");

The following is the same as the example above except

AvidSendSearch is used to register the wait strings.

/* Register the send string. */

AvidRegisterSend(AVID, com, "exit\r", &add_info, "");

/* Send the exit command, then wait for the login: prompt. */

if (AvidSendSearch(AVID, com, (long)5, (long)45, &add_info,

"1w'login:' 2w'Enter Data Switch Number:'") <= ERR_SNF)

printf("Exit failed\n");

The following example shows how to capture information using the

negative capture switch.  The negative capture is used when the

wait string follows the information that needs to be captured.

Suppose, for example, sending the command 'weather\r' is a host

command that displays todays weather forecast.  A segment of what

this command returns might be...

November 17, 1990

Sunny

Wind gusts to 30 mph

Temperature 43 F

No Rain

or the following segment is also possible...

November 18, 1990

AvidSendSearch

74

Partly cloudy

Mild wind of 2 mph

Temperature 35 F

No Rain

The wind speed needs to be captured.  However, the text before

the wind speed is never consistent.  Therefore, it is impossible

to key off of a string before the value to be captured.  The

string "mph", however, is always displayed after the wind speed.

With this consistency, it is possible to capture the wind speed

with a negative capture string.  The following example will do

it.

if (AvidSendSearch(AVID, com, (long)5, (long)45, &add_info,

".s'weather\r' 1w'mph' -c'3'",

(long *)&actual, captured) <= ERR_SNF)

printf("Failed\n");

The capture string for the first sample segment would have the

string "30 " and for the second sample segment would have the

string " 2 ".  In each case, the capture string is the last 3

characters before the string 'mph'.

A positive capture is used when the string to key off of is

before the information to be captured.  Using the sample segments

from the weather example, notice that the string "Temperature" is

always displayed before the temperature.  Therefore, a positive

capture can be done.  The following code segment will do it.

if (AvidSendSearch(AVID, com, (long)5, (long)45, &add_info,

"1w'Temperature'") <= ERR_SNF)

printf("Failed\n");

if (AvidSendSearch(AVID, com, (long)5, (long)45, &add_info,

"+c'4' 1w'F' .d'1' .D'10'",

(long *)&actual, captured) < ERR_SNF)

printf("Failed\n");

The first call to AvidSendSearch continues from where the wind

speed was captured to the string "Temperature".  The second call

to AvidSendSearch captures the next 4 characters.  In case the

temperature is in the 100's, 4 characters are captured.  The C

function sscanf can be used to filter out the trailing character

if it is not valid.

Notice that the wait switch 1w'F' was also specified.  This will

cause AvidSendSearch to return if the string "F" is found.  Also,

AvidSendSearch

75

notice that the drain switches .d'1' and .D'10' were specified.

These switches will cause the rest of the 'weather' command to be

read in and ignored.  In this example, the drain switches would

cause 'No Rain' to be ignored.

See Also

AvidRegisterSend, AvidRegisterWait, AvidDrainPort

AvidSendSearch

76

AvidVersion - Print Version and Copyright Information

err = AvidVersion(AVID, (int *)&add_info, opts)

Parameter Description

int err;       -- Return code.

AVID           -- System parameter that must always be specified.

int *add_info; -- Additional return information.

char *opts;    -- This is the input parameter for options.

Options

NONE

Return Codes

ERR_OK    -- No error occurred.

Additional Information Return Codes

ERR_OK    -- No error occurred.

Description

This routine prints the version and copyright information.

Examples

The following example will display the version and copyright

information.

err = AvidVersion(AVID, &add_info, "");

AvidVersion

77

EveryChar - Called with Every Character Received

(void) EveryChar(c)

Parameter Description

int c;    -- The character that was received.

Options

Not Applicable

Return Codes

Not Applicable

Description

This routine is different from the other routines: the

AutoLibrary(tm) Program-Playback tool calls this routine with

each received character.  You must supply this routine in your

code.  This will give your applications the opportunity to print

each received character.  If your application does not need this

feature and you do not want the overhead of this routine, simply

remove the call from CommInterfaceRead.

Examples

This example prints each received character to the screen.  By

putting this implementation of EveryChar in your application,

each received character will be printed to the screen.  EveryChar

could also be used to log every character to a file.  Also, see

the example in AvidConvert.

/*****/

void EveryChar(c)

int c;

/*****/

{

printf("%c", c);

}

This example does nothing with each received character, which is

completely valid.  Also, it is possible to remove the call to

EveryChar in CommInterfaceRead, which is only important if you do

not want to overhead of calling EveryChar.

EveryChar

78

/*****/

void EveryChar(c)

int c;

/*****/

{

c = c; /* for compiler. */

}

See Also

AvidConvert, CommInterfaceRead

EveryChar

79

Reference Guide for Interface Routines

This section describes the routines that interface to your

communication package and describes how to create interface

routines for other communication packages.

Note:  The source code discussed in this section can be found in

intface.c.

80

CommInterfaceClose - Close port

err = CommInterfaceClose(com, (int *)&add_info, opts)

Parameter Description

int err;       -- The returned status.

int com;       -- The AutoLibrary communication port.

int *add_info; -- Additional return information.

char *opts;    -- Options.

Options

See AvidClose

Return Codes

ERR_OK              -- No error occurred.

ERR_COMM_PACKAGE    -- This error can occur when the

communication package cannot correctly close

the communication port.  The parameter

'add_info' will contain the actual

communication packages error code.

Additional Information Return Codes

ERR_OK         -- No error occurred.

<system err>   -- When ERR_COMM_PACKAGE is returned, this

variable contains the actual error returned by the

communication package.

Description

This routine is used to close the communication port.  Your code

must not call this routine directly.  This routine will need to

be modified to support your communication package.  See the

System Requirements section for a list of supported communication

packages.

Porting to other communication packages

CommInterfaceClose

81

The following sample source code is written to interface with the

Greenleaf CommLib.  To support a different communication package,

use this code as a template, then code according to the

specifications of the communication package.

The call to 'asiquit' and the literal ASSUCCESS are unique to the

Greenleaf CommLib and would need to change to support a different

communication package.

When this routine works correctly, ERR_OK must be returned in

'add_info' and as the return status.  When this routine fails for

any reason, the return status must be ERR_COMM_PACKAGE.  When

ERR_COMM_PACKAGE is returned, 'add_info' should be set to the

actual error from the communication package.

/********************************************************/

int         /* The return status. */

CommInterfaceClose (com, info, opts)

int com;    /* The AutoLibrary communication port. */

int *info;

char *opts; /* Options. */

/********************************************************/

{

int err;

opts = opts; /* for compiler */

/* Start out with no errors. */

*info = ERR_OK;

err = asiquit (comlist[com]->hw_port);

if (err != ASSUCCESS) {

*info = err;

return(ERR_COMM_PACKAGE);

}

return(ERR_OK);

}

See Also

AvidClosePort, Actual source code.

CommInterfaceClose

82

CommInterfaceOpen - Open Port

err = CommInterfaceOpen(com, device, (int *)&add_info, opts)

Parameter Description

int err;       -- The return status.

int com;       -- The AutoLibrary port number.

char *device;  -- The device name.  (For example, "com1" or

"com2".)

int *add_info; -- Additional return information.

char *opts;    -- Options.

Options

See AvidOpenPort

Return Codes

ERR_OK              -- No error occurred.

ERR_COMM_PACKAGE    -- This error occurs when the communication

package cannot open the communication port

correctly.  When this error occurs, the

parameter 'add_info' will contain the actual

error returned by the communication package.

Additional Information Return Codes

ERR_OK         -- No error occurred.

<system err>   -- When ERR_COMM_PACKAGE is returned, 'add_info'

contains the actual error returned by the

communication package.

Description

This routine is used to open the communication port.  Your code

must not call this routine directly.  This routine will need to

be modified to support your communication package.  See the

System Requirements section for a list of supported communication

packages.

CommInterfaceOpen

83

Porting to other communication packages

The best way to describe how to port CommInterfaceOpen is a piece

at a time.  You will want to refer to the final version of

CommInterfaceOpen at the end of this section as you read about

this routine.  By referring to the final version you will be able

to see where the many code fragments fit in.  The following line

of code saves the information required to interface with the

Greenleaf CommLib.

/* Save the Greenleaf port number. */

comlist[com]->hw_port = port_num;

The Greenleaf port number 'port_num' must be saved for later use

in the routines CommInterfaceClose, CommInterfaceRead, and

CommInterfaceSend, since each of these routines make calls to

Greenleaf routines.  Note that the Greenleaf port number

'port_num' will not always be equal to the AutoLibrary port

number 'com', i.e., com != comlist[com]->hw_port.  The

AutoLibrary port number is used by AutoLibrary routines to get

information from the 'comlist' data structure.  Where as, the

Greenleaf port number is used to identify a port number to

communicate with.

The following code segment shows a call that opens a Greenleaf

communication port.

err=asiopen(comlist[com]->hw_port, mode, 8192,

512, 9600, P_ODD, 1, 8, 0, 0);

if (err != ASSUCCESS) {

*info = err;

return(ERR_COMM_PACKAGE);

}

This call opens a communication port with a receive buffer of

8192 bytes, transmit buffer of 512 bytes, baud rate of 9600, odd

parity, one stop bit, and 8 bits per word.  This hard coded call

to 'asiopen' is valid, since the AutoLibrary(tm) Program-Playback

tool will work as long as a port gets opened.  However, since

hard coded opens are restrictive, switches have been defined to

support opening a communication port in different ways.  For

example, the switch -b'4800' requests that the port be opened at

4800 baud.  The switch .2 specifies 2 stop bits and rx'1024'

requests a receive buffer of 1K.

CommInterfaceOpen

84

It is the responsibility of CommInterfaceOpen to support these

switches.  (It is ok to add switches that are unique to your

communication package.)

Before describing switches, I would like to show the following

implementation of CommInterfaceOpen, which is instructive because

it only shows the required code.  Additional code is recommended,

which will be shown later.

CommInterfaceOpen

85

/********************************************************/

int

CommInterfaceOpen (com, device, info, opts)

int com;      /* The AutoLibrary communication port. */

char *device; /* The device name. */

int *info;

char *opts;   /* Options. */

/********************************************************/

{

int mode, err;

/* Start out with no errors. */

*info = ERR_OK;

/* For Greenleaf COM1 is defined as 0. */

port_num--;

/* Save the port number for closing. */

comlist[com]->hw_port = port_num;

/* No ASCII device names in this comm package. */

comlist[com]->hw_port_name = (char *)NULL;

mode = ASCII | ASINOUT | NORMALRX;

err = asiopen (comlist[com]->hw_port, mode, 8192,

512, 9600, P_ODD, 1, 8, 0, 0);

if (err != ASSUCCESS) {

*info = err;

return(ERR_COMM_PACKAGE);

}

err = asixon(comlist[com]->hw_port, 30, 70, XON, XOFF);

if (err != ASSUCCESS) {

*info = err;

return(ERR_COMM_PACKAGE);

}

return(ERR_OK);

}

The second line of code decrements 'port_num'.  This is required

because the Greenleaf CommLib defines 0 for com1, 1 for com2, and

so on.  Your communication package may or may not have to

decrement 'port_num'.

CommInterfaceOpen

86

We have already looked at the third line of code.  The Greenleaf

port number ('port_num') is saved in the 'comlist' data structure

for later use by the other interface routines.

The forth line of code saves a pointer to the device name in the

'comlist' data structure for later use for the same reasons the

port number is saved.  This example sets this value to NULL

because the Greenleaf CommLib does not use device names.  If your

communication package uses device names, this field would be used

to save a pointer to the name for the interface routines

CommInterfaceClose, CommInterfaceRead, and CommInterfaceSend.

The device name field 'hw_port_name' is important only if your

communication package refers to ports with text strings instead

of numbers.  Just remember that 'comlist[com]->hw_port_name' is a

pointer.  Use 'malloc' to dynamically allocate the memory needed

to store the device name, then in CommInterfaceClose free the

memory.

The remaining code in the above example opens a Greenleaf

communication port with hard coded parameters.  This

implementation of CommInterfaceOpen contains the bare minimum for

the AutoLibrary(tm) Program-Playback tool to work correctly.  If

your application is always to be opened with the same parameters,

an implementation like this may be enough.  However, more code is

required if CommInterfaceOpen is to be flexible.  When opening a

communication port there are many options to choose from, like

baud rate, number of stop bits, parity, and so on.  To support

these different options, we need to add switch processing to

CommInterfaceOpen.  For example, the following call to

AvidOpenPort opens communication port one (com1) at 4800 baud and

even parity.  The defaults are used for the other parameters.

AvidOpenPort(AVID, "1", &info, ".b'4800' .e");

AvidOpenPort does not process open switches, instead, the

switches are passed directly to CommInterfaceOpen.  If we stayed

with the previous implementation of CommInterfaceOpen, the open

switches ".b'4800' .e" would be ignored.  To support open

switches, the option parameter needs to be parsed using

AvidProcessOptions.  Here is an example of how CommInterfaceOpen

would process the baud rate switch.

CommInterfaceOpen

87

baud = 9600;  /* default */

s_num = 0; /* Required for AvidProcessOptions. */

while ((sc = AvidProcessOptions(AVID, opts, "", &control,

temp_buf, &s_num)) != '\0') {

if ((control == '.') && (sc == 'b'))

baud = atoi(temp_buf);

}

First, the variable 'baud' is set to the default of 9600, which

is used if the baud rate switch ( .b'baud' ) is not specified.

The while loop will return each of the switches specified in the

'opts' parameter.  The 'if' statement inside the while loop tests

for the baud rate switch.  When specified, the new baud rate is

set.  The following if statements, when placed inside the while

loop, will set the parity to the desired value.

if ((o_odd[0] == control) && (o_odd[1] == sc))

parity = P_ODD;

if ((o_even[0] == control) && (o_even[1] == sc))

parity = P_EVEN;

The string 'o_odd' is initialized to ".o" and the string 'o_even'

is initialized to ".e".  This way the actual switch definition is

done in one spot.  The previous example hard coded the baud rate

switch, which has the option string 'o_baud'.  The string

'o_baud' will be used in the final version of CommInterfaceOpen.

The constants P_ODD and P_EVEN are defined in the Greenleaf

CommLib.  Since your communication package will be different,

these constants will be different.

The following while loop shows how the other open options are

parsed by AvidProcessOptions.

mode = 0;

baud = 9600;

parity = P_ODD;

stop_bits = 1;

word_len = 8;

dtr = 0;

rts = 0;

rx_length = 8192;

tx_length = 512;

strip = ASCII;

xon = TRUE;

CommInterfaceOpen

88

s_num = 0; /* Required for AvidProcessOptions. */

while ((sc = AvidProcessOptions (AVID, opts, "", &control,

temp_buf, &s_num)) != '\0') {

if ((o_strip_yes[0] == control) && (o_strip_yes[1] == sc))

strip = ASCII;

if ((o_strip_no[0] == control) && (o_strip_no[1] == sc))

strip = BINARY;

if ((o_even[0] == control) && (o_even[1] == sc))

parity = P_EVEN;

if ((o_odd[0] == control) && (o_odd[1] == sc))

parity = P_ODD;

if ((o_none[0] == control) && (o_none[1] == sc))

parity = P_NONE;

if ((o_baud[0] == control) && (o_baud[1] == sc))

baud = atoi (temp_buf);

if ((o_stop_bits_1[0] == control) &&

(o_stop_bits_2[1] == sc))

stop_bits = 1;

if ((o_stop_bits_2[0] == control) &&

(o_stop_bits_2[1] == sc))

stop_bits = 2;

if ((o_word_len_7[0] == control) && (o_word_len_7[1] == sc))

word_len = 7;

if ((o_word_len_8[0] == control) && (o_word_len_8[1] == sc))

word_len = 8;

if ((o_dtr[0] == control) && (o_dtr[1] == sc))

dtr = 1;

if ((o_xon_yes[0] == control) && (o_xon_yes[1] == sc))

xon = TRUE;

if ((o_xon_no[0] == control) && (o_xon_no[1] == sc))

xon = FALSE;

if ((o_rx_len[0] == control) && (o_rx_len[1] == sc)) {

temp_long = atol (temp_buf);

CommInterfaceOpen

89

rx_length = temp_long;

}

if ((o_tx_len[0] == control) && (o_tx_len[1] == sc)) {

temp_long = atol (temp_buf);

tx_length = temp_long;

}

}

Your implementation of this while loop may be very different.

Some switches may not apply, like the receive buffer length

switch (rx) and the send buffer length switch (tx).  These

options were added to CommInterfaceOpen because the Greenleaf

CommLib supports changing these buffer sizes at run time.  Your

communication package may support other unique features.  If so,

make up a switch that can be used to invoke the feature.  Include

the new switch in intface.c.  Try to make the new switch unique

by checking the other switches in al_sys.h.

Once you have chosen a switch, it is possible to reserve it by

contacting Avid Software.  Also, I am interested in supporting as

many communications packages as possible, so send in your ported

code for possible inclusion in the core product.

The 3 major pieces of CommInterfaceOpen have been described.

First, the port number or device name is saved for later use.

Next, the switches are parsed, and finally the call is made to

the communication package to open the port.  However, before

showing a complete listing of CommInterfaceOpen, it is important

to understand what the first part of CommInterfaceOpen does.  The

following 'if' statements allow communication ports to be

specified in 2 different ways.

/* Let valid input for device be com1 or 1, for example. */

if (sscanf (device, "com%d", &port_num) == 0) {

if (sscanf (device, "%d", &port_num) == 0) {

*info = ASINVPORT;

return(ERR_COMM_PACKAGE);

}

}

This makes it possible, for instance, to accept communication

port 1 as "com1" or as "1".

CommInterfaceOpen

90

The following 'if' statement returns an error if the

communication port number is outside the range possible for the

Greenleaf CommLib.

/* Make sure wild number did not make it through. */

if ((port_num < 1) || (port_num > 17)) {

*info = ASINVPORT;

return(ERR_COMM_PACKAGE);

}

Now that everything in CommInterfaceOpen has been described, the

following listing shows a complete version.

CommInterfaceOpen

91

/********************************************************/

int

CommInterfaceOpen (com, device, info, opts)

int com;      /* The AutoLibrary communication port. */

char *device; /* The device name. */

int *info;

char *opts;   /* Options. */

/********************************************************/

{

int mode, word_len, baud, stop_bits, parity, port_num, strip,

dtr, rts;

unsigned int rx_length, tx_length;

char sc, control;

char temp_buf[85];

int s_num, err;

long temp_long;

/* Start out with no errors. */

*info = ERR_OK;

/* Let valid input for device be com1 or 1 (for example) */

if (sscanf (device, "com%d", &port_num) == 0) {

if (sscanf (device, "%d", &port_num) == 0) {

*info = ASINVPORT;

return (ERR_COMM_PACKAGE);

}

}

/* Make sure wild number did not make it through. */

if ((port_num < 1) || (port_num > 17)) {

*info = ASINVPORT;

return (ERR_COMM_PACKAGE);

}

/* For Greenleaf COM1 is defined as 0. */

port_num--;

/* Save the port number for closing. */

comlist[com]->hw_port = port_num;

/* No ASCII device names in this comm package. */

comlist[com]->hw_port_name = (char *)NULL;

mode = 0;

baud = 9600;

CommInterfaceOpen

92

parity = P_ODD;

stop_bits = 1;

word_len = 8;

dtr = 0;

rts = 0;

rx_length = 8192;

tx_length = 512;

strip = ASCII;

xon = TRUE;

s_num = 0; /* Required for AvidProcessOptions. */

while ((sc = AvidProcessOptions (AVID, opts, "", &control,

temp_buf, &s_num)) != '\0') {

if ((o_strip_yes[0] == control) &&

(o_strip_yes[1] == sc))

strip = ASCII;

if ((o_strip_no[0] == control) && (o_strip_no[1] == sc))

strip = BINARY;

if ((o_even[0] == control) && (o_even[1] == sc))

parity = P_EVEN;

if ((o_odd[0] == control) && (o_odd[1] == sc))

parity = P_ODD;

if ((o_none[0] == control) && (o_none[1] == sc))

parity = P_NONE;

if ((o_baud[0] == control) && (o_baud[1] == sc))

baud = atoi (temp_buf);

if ((o_stop_bits_1[0] == control) &&

(o_stop_bits_2[1] == sc))

stop_bits = 1;

if ((o_stop_bits_2[0] == control) &&

(o_stop_bits_2[1] == sc))

stop_bits = 2;

if ((o_word_len_7[0] == control) &&

(o_word_len_7[1] == sc))

word_len = 7;

if ((o_word_len_8[0] == control) &&

(o_word_len_8[1] == sc))

CommInterfaceOpen

93

word_len = 8;

if ((o_dtr[0] == control) && (o_dtr[1] == sc))

dtr = 1;

if ((o_xon_yes[0] == control) && (o_xon_yes[1] == sc))

xon = TRUE;

if ((o_xon_no[0] == control) && (o_xon_no[1] == sc))

xon = FALSE;

if ((o_rx_len[0] == control) && (o_rx_len[1] == sc)) {

temp_long = atol (temp_buf);

rx_length = temp_long;

}

if ((o_tx_len[0] == control) && (o_tx_len[1] == sc)) {

temp_long = atol (temp_buf);

tx_length = temp_long;

}

}

mode = strip | ASINOUT | NORMALRX;

err = asiopen (comlist[com]->hw_port, mode,

rx_length, tx_length, baud, parity, stop_bits,

word_len, dtr, rts);

if (err != ASSUCCESS) {

*info = err;

return(ERR_COMM_PACKAGE);

}

if (xon) {

err = asixon(comlist[com]->hw_port, 30, 70, XON, XOFF);

if (err != ASSUCCESS) {

*info = err;

return(ERR_COMM_PACKAGE);

}

}

return(ERR_OK);

}

See Also

AvidOpenPort, Actual source code.

CommInterfaceOpen

94

CommInterfaceRead - Read Character

c = CommInterfaceRead(com, (long)r_timeout, (long)e_timeout,

(int *)&add_info, opts)

Parameter Description

int c;              -- The character that was read.

int com;            -- The AutoLibrary communication port.

long r_timeout;     -- The repeating timeout.

long e_timeout;     -- The ending timeout.

int *add_info;      -- Additional return information.

char *opts;         -- Options.

Options

See AvidRead

Return Codes

<Received character>     -- No error occurred.  The character is

returned.

ERR_COMM_PACKAGE         -- This error occurs when the

communication package cannot read the

communication port correctly.  When this

error occurs, the parameter 'add_info'

will contain the actual error returned

by the communication package.

Additional Information Return Codes

ERR_OK         -- No error occurred.

<system err>   -- When ERR_COMM_PACKAGE is returned, 'add_info'

contains the actual error returned by the

communication package.

Description

CommInterfaceRead

95

This routine is used to read from the communication port.  Your

code must not call this routine directly.  This routine will need

to be modified to support your communication package.  See the

System Requirements section for a list of supported communication

packages.

Porting to other communication packages

You will want to refer to the final version of CommInterfaceRead

at the end of this section as you read about this routine.  By

referring to the final version you will be able to see where the

many code fragments fit in.

If timeouts were not required, CommInterfaceRead would be easy.

This routine would simply return the received character, or

ERR_EOF when no character was available.  The following code

shows this simple implementation.

CommInterfaceRead

96

/********************************************************/

int

CommInterfaceRead (com, r_timeout, e_timeout, info, opts)

int com;         /* The AutoLibrary communication port. */

long r_timeout;  /* The repeating timeout. */

long e_timeout;  /* The ending timeout. */

int *info;

char *opts;      /* Options. */

/********************************************************/

{

int c;

/* Start out with no errors. */

*info = ERR_OK;

c = asigetc (comlist[com]->hw_port);

if ((c >=0) && (c <= 0xFF)) {

EveryChar(c);

return(c);

}

if (c == ASINVPORT) {

*info = ASINVPORT;

return(ERR_COMM_PACKAGE);

}

if (c == ASNOTSETUP) {

*info = ASNOTSETUP;

return(ERR_COMM_PACKAGE);

}

return(ERR_EOF);

}

The above implementation would work.  However, since timeouts are

required to successfully automate communications, the

AutoLibrary(tm) Program-Playback tool supports 2 types of

timeouts:  Repeating Timeouts and Ending Timeouts.  Let's

implement Repeating Timeouts first.  It is possible to implement

Repeating Timeouts completely from within CommInterfaceRead.  We

will find out later this is not possible with Ending Timeouts.

To implement Repeating Timeouts, a while loop needs to be added.

The while loop will make repeated calls to the communication

package asking for a character until a character is received or

until a timeout occurs.  The following implementation supports

Repeating Timeouts.

CommInterfaceRead

97

/********************************************************/

int

CommInterfaceRead (com, r_timeout, e_timeout, info, opts)

int com;         /* The AutoLibrary communication port. */

long r_timeout;  /* The repeating timeout. */

long e_timeout;  /* The ending timeout. */

int *info;

char *opts;      /* Options. */

/********************************************************/

{

int c;

long seconds;

/* Start out with no errors. */

*info = ERR_OK;

/* Set up for Repeating Timeout. */

time(&seconds);

r_timeout = r_timeout + seconds;

while (1 > 0) {

c = asigetc (comlist[com]->hw_port);

if ((c >=0) && (c <= 0xFF)) {

EveryChar(c);

return(c);

}

if (c == ASBUFREMPTY) {

/* if still no character, check for timeout. */

time(&seconds);

if (seconds >= r_timeout) {

*info = ERR_REPEAT_TIMEOUT;

return(ERR_EOF);

}

}

}

return(ERR_EOF);

}

The call to 'time(&seconds)' gets the total number of seconds

since Jan 1, 1970.  This number is added to the desired timeout

value.  Say, for instance, a Repeating Timeout of 10 seconds was

CommInterfaceRead

98

specified and the current date and time is June 30, 1991 at

18:37:40.  The variable 'r_timeout' would then contain

678,321,470 which is 678,321,460 plus 10, where 678,321,460 is

the number of seconds from Jan 1, 1970 to Jun 30, 1991.  Once

'r_timeout' is initialized this way, it is possible to determine

when a timeout should happen.  Each time 'asigetc' is called,

which results in no received character, the current system time

is compared with 'r_timeout'.  If the system time is greater than

or equal to 'r_timeout', a timeout is returned.

The above implementation needs some minor but important changes.

The first change is in setting up for the Repeating Timeout.

Instead of...

time(&seconds);

r_timeout = r_timeout + seconds;

the following code is correct.

time(&seconds);

if (r_timeout < seconds)

r_timeout = r_timeout + seconds;

The 'if' test is required to support AVID_NO_TIMEOUT, which

indicates that a timeout is to never happen.  AVID_NO_TIMEOUT is

defined as 2,147,483,600, which is the system time 68 years from

now.  Therefore, when 'r_timeout' has this value, incrementing

'r_timeout' by 'seconds' is not needed.

The next change the above implementation needs is to return an

error if 'asigetc' returns with an error.  Therefore, instead

of...

if (c == ASBUFREMPTY) {

/* if still no character, check for timeout. */

time(&seconds);

if (seconds >= r_timeout) {

*info = ERR_REPEAT_TIMEOUT;

return(ERR_EOF);

}

}

the following code is correct.

CommInterfaceRead

99

if (c == ASBUFREMPTY) {

/* if still no character, check for timeout. */

time(&seconds);

if (seconds >= r_timeout) {

*info = ERR_REPEAT_TIMEOUT;

return(ERR_EOF);

}

} else {

/* Error ASINVPORT or ASNOTSETUP occurred. */

*info = c;

return(ERR_COMM_PACKAGE);

}

This way if an error besides ASBUFREMPTY is returned,

ERR_COMM_PACKAGE is returned by CommInterfaceRead.

Finally, the last change is done for speed.  A call is made to

'asigetc' before setting up for a timeout.  If there is a

character, it is returned without the overhead of setting up for

a repeating timeout.  If there is not a character, the time

required to set up for a timeout is ok since the code will enter

a polling loop anyway.  At this point I would like to show the

complete implementation that supports Repeating Timeouts.

(Remember this implementation doesn't support Ending Timeouts).

CommInterfaceRead

100

/********************************************************/

int

CommInterfaceRead (com, r_timeout, e_timeout, info, opts)

int com;         /* The AutoLibrary communication port. */

long r_timeout;  /* The repeating timeout. */

long e_timeout;  /* The ending timeout. */

int *info;

char *opts;      /* Options. */

/********************************************************/

{

int c;

long seconds;

/* Start out with no errors. */

*info = ERR_OK;

/* Try to get a character without setting up

for a timeout. */

c = asigetc (comlist[com]->hw_port);

if ((c >=0) && (c <= 0xFF)) {

EveryChar(c);

return(c);

}

if (c == ASINVPORT) {

*info = ASINVPORT;

return(ERR_COMM_PACKAGE);

}

if (c == ASNOTSETUP) {

*info = ASNOTSETUP;

return(ERR_COMM_PACKAGE);

}

/* A timeout is required so set up for it. */

time(&seconds);

if (r_timeout < seconds)

r_timeout = r_timeout + seconds;

while (1 > 0) {

/* Try to get a character. */

c = asigetc (comlist[com]->hw_port);

if ((c >=0) && (c <= 0xFF)) {

EveryChar(c);

return(c);

}

CommInterfaceRead

101

if (c == ASBUFREMPTY) {

time (&seconds);

if (seconds >= r_timeout) {

*info = ERR_REPEAT_TIMEOUT;

return(ERR_EOF);

}

} else {

/* Error ASINVPORT or ASNOTSETUP occurred. */

*info = c;

return(ERR_COMM_PACKAGE);

}

}

opts = opts; /* for compiler */

/* Nothing should reach here but if it does... */

*info = c;

return(ERR_COMM_PACKAGE);

}

Now, Ending Timeout support needs to be added to

CommInterfaceRead.  This is simple.  The following test needs to

be added just inside the while loop.

/* Do not try to get a character if an ending timeout has

occurred. */

time(&seconds);

if (seconds >= e_timeout) {

*info = ERR_ENDING_TIMEOUT;

return(ERR_EOF);

}

This test also needs to occur before the first call to 'asigetc'

in case characters are continously received when an ending

timeout should occur.

Now the complete and final implementation of CommInterfaceRead

looks like the following.

CommInterfaceRead

102

/********************************************************/

int

CommInterfaceRead (com, r_timeout, e_timeout, info, opts)

int com;         /* The AutoLibrary communication port. */

long r_timeout;  /* The repeating timeout. */

long e_timeout;  /* The ending timeout. */

int *info;

char *opts;      /* Options. */

/********************************************************/

{

int c;

long seconds;

/* Start out with no errors. */

*info = ERR_OK;

/* Have to check for an ending timeout. */

time(&seconds);

if (seconds >= e_timeout) {

*info = ERR_ENDING_TIMEOUT;

return(ERR_EOF);

}

/* Try to get a character without setting up

for a timeout. */

c = asigetc (comlist[com]->hw_port);

if ((c >=0) && (c <= 0xFF)) {

EveryChar(c);

return(c);

}

if (c == ASINVPORT) {

*info = ASINVPORT;

return(ERR_COMM_PACKAGE);

}

if (c == ASNOTSETUP) {

*info = ASNOTSETUP;

return(ERR_COMM_PACKAGE);

}

/* A timeout is required so set up for it. */

time(&seconds);

if (r_timeout < seconds)

r_timeout = r_timeout + seconds;

CommInterfaceRead

103

while (1 > 0) {

/*Do not try to get a character if an ending

timeout has occurred.*/

time (&seconds);

if (seconds >= e_timeout) {

*info = ERR_ENDING_TIMEOUT;

return(ERR_EOF);

}

/* Try to get a character. */

c = asigetc (comlist[com]->hw_port);

if ((c >=0) && (c <= 0xFF)) {

EveryChar(c);

return(c);

}

if (c == ASBUFREMPTY) {

time (&seconds);

if (seconds >= r_timeout) {

*info = ERR_REPEAT_TIMEOUT;

return(ERR_EOF);

}

} else {

/* Error ASINVPORT or ASNOTSETUP occurred. */

*info = c;

return(ERR_COMM_PACKAGE);

}

}

opts = opts; /* for compiler */

/* Nothing should reach here but if it does... */

*info = c;

return(ERR_COMM_PACKAGE);

}

It was easy for CommInterfaceRead to support Ending Timeouts

because it is the responsibility of the calling routine to set up

for it.  CommInterfaceRead simply gets (in the 'e_timeout'

parameter) a system time value in total seconds from Jan 1, 1970.

An Ending Timeout occurs when the current system time is greater

than or equal to 'e_timeout'.

CommInterfaceRead

104

If you would like to know more about the callers responsibility

for Ending Timeouts, take a look at the source code for

AvidSendSearch.  AvidSendSearch makes calls to AvidRead from

inside a while loop.  (AvidRead passes the 'r_timeout' and

'e_timeout' values directly to CommInterfaceRead).  Before the

while loop, the current system time is added to the 'e_timeout'

parameter.  The 'e_timeout' parameter is then constant for each

call to AvidRead, which makes it possible to support Ending

Timeouts.

See Also

AvidRead, Actual source code.

CommInterfaceRead

105

CommInterfaceSend - Send Character

err = CommInterfaceSend(com, bef, bet, s, len,

(int *)&add_info, opts)

Parameter Description

int err;       -- Error code.

int com;       -- The AutoLibrary communication port.

int bef;       -- The number of milliseconds to delay before

sending the first character.

int bet;       -- The number of milliseconds to delay between

each character.

char *s;       -- The string to send.

int len;       -- The length of the string to send.  This value

may not equal strlen(s) because of null characters

in the string.

int *add_info; -- Additional return information.

char *opts;    -- Options.

Options

See AvidSend

Return Codes

ERR_OK              -- No error occurred.

ERR_COMM_PACKAGE    -- This error occurs when the communication

package cannot send to the communication port

correctly.  When this error occurs, the

parameter 'add_info' will contain the actual

error returned by the communication package.

Additional Information Return Codes

ERR_OK         -- No error occurred.

CommInterfaceSend

106

<system err>   -- When ERR_COMM_PACKAGE is returned, 'add_info'

contains the actual error returned by the

communication package.

Description

This routine is used to send characters out the communication

port.  Your code must not call this routine directly.  This

routine will need to be modified to support your communication

package.  See the System Requirements section for a list of

supported communication packages.

Porting to other communication packages

The following sample source code is written to interface with the

Greenleaf CommLib.  To support a different communication package,

use this code as a template, then code according to the

specifications of the communication package.

This routine is easy to port.  A call is made to 'asiputc' inside

a 'for' loop for each character to be sent.  The delay before

value is done outside of the loop, and the delay between is done

inside the 'for' loop.

CommInterfaceSend

107

/********************************************************/

int

CommInterfaceSend (com, bef, bet, s, len, info, opts)

int com; /* The AutoLibrary communication port. */

int bef; /* The number of milliseconds to delay before

sending first character. */

int bet; /* The number of milliseconds to delay between each

character. */

char *s; /* The string to send. */

int len; /* The length of the string. May not equal

strlen(s) because of null characters in the

string. */

int *info;

char *opts; /* Options. */

/********************************************************/

{

int i, status;

opts = opts; /* for compiler */

/* Start out with no errors. */

*info = ERR_OK;

/* The delay before the first character sent. */

delay (bef);

for (i=0; i<len; i++) {

/* The delay before the first character sent. */

delay (bet);

status = asiputc (comlist[com]->hw_port, s[i]);

if (status != ASSUCCESS) {

*info = status;

return(ERR_COMM_PACKAGE);

}

}

return(ERR_OK);

}

See Also

AvidSend, Actual source code.

CommInterfaceSend