ieetst, version 0.0

This software tests the numerical accuracy of floating point

binary <-> decimal string conversion, as done by your C language

library functions printf() and scanf(), for compliance with the

IEEE arithmetic standards ANSI/IEEE Std 754-1985 and ANSI/IEEE

Std 854-1987.  The test covers 32-bit float, 64-bit double, and

80-bit long double precision conversions to and from decimal

ASCII strings.

The test program includes checks for proper implementation of

the following specifications of the standards:

(1) correctly rounded conversions of numbers of the form M *

10^N, where M and N are integers such that, in double precision,

for example, |M| < 10^17, |N| <= 27.

(2) binary -> decimal -> binary conversions to be an identity.

(3) correctly rounded decimal outputs of less than the maximum

number of digits

(4) The maximum observed conversion error of numbers outside the

domain covered by (1) is reported by the test program; it is

not supposed to exceed 0.97 ulp.

There are 10 separate tests.  Tests 1 through 6 use values near

2^n and 10^n.  Test 7 addresses item (1) above.  Test 8 checks

the rounding of exact half-integer numbers. Test 9 is for item

(4) above.  Test 10 checks denormal numbers.  Tests 8 through 10

address item (3) using printf formats that produce outputs of 1,

2, 3, ... digits after the decimal point.  All tests check, when

appropriate, that the binary output of scanf is the same as the

binary input to printf.

Example error messages:

0000 0000 0000 1000 8000 3f80, ->printf-> 5.87748296e-39 ->scanf->

0000 0000 0000 0000 8000 3f6e, is not an identity.

scanf(-9.9999900000000003e-01) -> 0000,4800,085f,ef39,ffff,bffe,

should be 0000 5000 085f ef39 ffff bffe .

printf("%.14e",  6.13592315154256467968352E-3) -> 6.13592315154257e-03

should be       6.13592315154256E-3 .

Binary values are displayed as four-digit hex groups in

little-endian format, with the least significant 16-bit word

first, exponent last.

The design of the test program requires knowing the binary

data structure of the floating point format under test.  For

configuration, check the .h files carefully. All the programs

need to be told via mconf.h if the numeric format is

little-endian (IBMPC) or big-endian (MIEEE).  If your system

supports an 80-bit long double precision data type, define

LDOUBLE 1 in ieetst.c; otherwise define LDOUBLE 0.  Provision for

DEC PDP-11/VAX numbers is implemented (double precision only).

Conversions for other data structures can be added by analogy

to the ifdefs for DEC.

Most of the tests rely on comparison with the results of a

portable reference arithmetic, contained in the file ieee.c.

This is configured for an 80-bit significand, to have enough

precision to satisfy the conversion requirements of IEEE 854 for

the extended double format of IEEE 754.  The reference arithmetic

includes binary <--> ASCII conversion routines and single <-->

double <--> extended double conversions.  A strictly rounded

square root function is given in esqrt.c.  Additional functions

are provided by elog.c, eexp.c, etanh.c, epow.c, all of which

call on ieee.c for their arithmetic.  Some of the ANSI C

functions are supplied in ieee.c; for example, efloor(),

efrexp(), eldexp(). The reference arithmetic is described further

in the book _Methods and Programs for Mathematical Functions_

(Prentice-Hall, 1989).

As an aid in diagnosis, a calculator program, ecalc.c, is

supplied.  It uses ieee.c for its arithmetic. Documentation for

the calculator's user interface is in the file calc100.doc

(calc100 is a fuller featured 100-digit version of ecalc).  The

calculator needs to be told by qcalc.h if addresses are 32 bits

long (define LARGEMEM 1) or 16 bits long (define LARGEMEM 0).

Because the source code of ieee.c is included here, a version

of PARANOIA is also provided; this has been heavily modified by

substituting subroutine calls to ieee.c in place of all

arithmetic operators.  It is important that you use PARANOIA to

check the arithmetic after any modifications you may make to

ieee.c.

Several systems have been tested with the initial version of

ieetst.  Sun 4 (SPARC) passes; DEC VMS C has only a small flaw;

Microsoft flunks; ATT SysVR2 (Motorola) flunks even worse.

-- Steve Moshier

May, 1992