OOBPLUS - CompuServe B protocol objects

Copyright (c) Steve Sneed 1991

All rights reserved

This document and the included source code ("the material") are the

copyrighted work of their author.  The material is provided to TurboPower

Software, and a license is hereby granted to TurboPower Software, for

distribution with their product(s).  Purchasers of TurboPower Software

product(s) that include the material may use the material under the same

purchase agreement rights and restrictions as the TurboPower product(s).

Portions of the material are copyright (c) by CompuServe, Inc. and

are used by permission.  "GIF" and "Graphics Interchange Format" are

Service Marks (sm) of CompuServe, Inc., an H&R Block Company.

---- Other Documentation --------------------------------------------------

The information and code provided here is based on the BPlus spec

document, and the GIF 87a and 89a spec documents.  For anything more than

a direct "plug-n-play" implementation based on these objects you will

definitely need those documents, and both are available for download from

CompuServe.  The BPlus document is available in the IBMCOM Forum's LIBrary

10 (Protocols) as PBDOC.ARC.  The GIF 89a specification, which includes

all of the 87a spec, is available in the PICS Forum's LIBrary 14 (Misc.

Utilities & Code) as GIF89A.DOC. (Other word processor formats, such as

WordPerfect and MSWord, are available for the GIF 89a spec; BROwse the

library with a KEYword of "89A" or "SPEC" to find them.)

The book "Programmer's Guide to PC & PS/2 Video Systems" (Richard

Wilton, Microsoft Press 1988, ISBN 1-55615-103-9) is generally considered

the "bible" of PC video programming.  Highly recommended if you intend to

work with GIF decoding.  I have also found "C Programmer's Guide to Serial

Communications" (Joe Campbell, Howard W. Sams & Company 1987, ISBN

0-672-22584-0) to be extremely helpful in general comm programming and

protocols.

---- Overview -------------------------------------------------------------

The CompuServe Information Service (CIS) is the largest and most popular

system of its type in the world, with over 1,000,000 members in the US and

abroad.  CIS provides several "special" capabilities, such as the ability

to display graphics in real time and the ability to encapsulate the entire

online session within a protocol for improved data integrity and management.

Many of these special capabilities are based on CompuServe's own transport

protocol, the "B" protocol.  Because of the size and number of users of CIS

today, any communications program should provide support for at least the

basic File Transfer Protocol (FTP) portions of the B protocol.  OOBPLUS

makes providing that support simple.

The OOBPLUS unit provides a set of objects for B protocol.  Derived from

Async Professional's AbstractProtocol object, OOBPLUS provides a "plug-in"

interface for both the B series protocol's FTP services, and the special

version for use with CompuServe's GIF graphics image display services.

There are three distinct species of B protocol in use today:

"Classic" B:  The original B protocol.  This version defines a protocol

similar in some ways to the XMODEM protocol but with special consider-

ation for the packet-switcher network CIS uses to connect users to their

hosts.  It uses a 512-byte block at all bps rates, a one-byte arithmetic

checksum, and simple "send block/wait for ACK" flow of control.  It does

allow for flexible block length of less than 512 bytes, and uses a fixed

quoting set (see below).  Classic B provides a much more robust protocol

environment than XMODEM for CIS, but its thruput is only slightly

better than XMODEM.  Classic B was designed in the days of 300baud

modems and early protocol technology.

"Quick" B:  Quick B (QB) is a major enhancement over Classic B.  While

maintaining backwards compatibility with Classic B it adds "send-ahead"

technology (the ability to send more than one packet before receiving an

acknowledgement), a larger 1K packet size for better thruput with higher

speed modems, and 16-bit CRC blockchecking.  QB is designed so the host

can "handshake" with the remote and determine whether Classic B or QB

will be used.  QB was such a marked improvement over Classic B that many

comm programs rushed to support it, and some unfortunately did not

implement the handshake properly; as a result CIS had to add a new

option to all of its protocols menus just for QB.

"BPlus": BPlus (B+) is a superset of QB, or more correctly, QB is a

subset of B+.  QB was more a preliminary version of B+ than a

self-standing version, and only the rapid implementation of QB by

several major comm programs kept it in use.  B+ defines several

extensions and enhancements over QB, such as the "TI" file information

packet type.  B+ also introduced the use of flexible maximum packet

size, to allow achievement of optimum packet size based on baud rate and

other factors.  Most importantly for users, B+ defines a method to

resume an aborted download from the point of abort in the transferred

file rather than forcing the download to restart from the beginning.

CompuServe would like to see QB disappear completely and is encouraging

developers to provide complete B+ support; many major comm programs

(such a ProComm Plus 2.0 and CrossTalk XVI and Mk 4) have done so, but a

large base of comm programs are still in use that only provide QB

support (such as BitCom, which many modem vendors distribute free with

their modems).

B+ is not a file transfer protocol in the typical sense.  It is a

"Transport Level" protocol, and as such can be used to provide protocol

error trapping and data management to an entire online session which may

include file transfer sub-sessions.  The full B+ specification defines

multiple "layers" of protocol support, the lowest being the "Transport

Layer".  The transport layer provides the basic data packetization, flow

control and error handling.  On top of the Transport Layer may be

"Application-Specific" layer(s) which performs preprocessing (such as

decompression of compressed data) or prioritizing/directing of data to

various parts of the overall program. In order to use B+ as a full

transport protocol, the CIS host and the local program must handshake

immediately after the remote logs into the service, both for protocol

support and then for application support.  This requires the remote

software to have intimate knowledge of the host and vice versa, so this

level of B+ is currently only used by software developed by CIS itself.  B

was originally developed for the Vidtex terminals back in the late 70's and

early 80's, but currently the only software in the PC arena that uses

full-session protocol encapsulation is the Host-Micro Interface protocol in

the Compuserve Information Manager products for PCs and Macs.

HMI NOTE: This new version of OOBPLUS now specifically includes the

modifications nessessary to support the special HMI version of B+.  However,

because no documentation has been made publicly available by CompuServe on

the exact format or usage of HMI, what's here has been arrived at by

experimentation and may require modification as more information on HMI

becomes available.

The B+ protocol was designed with more than PCs in mind.  Many types of

computers access the CIS hosts, from Apple ]['s and RadioShack Model III's

to Cray supercomputers.  Also, CIS ties to several different telephone

networks both in the US and abroad, and many of these networks are

decidedly different from the network CIS itself owns and operates

(especially the overseas PADs and networks).  Because many of these

networks use some control characters for their own internal purposes, and

many of the mini and mainframe computers have their own interpretation of

certain characters, B+ "quotes" some characters in a manner similar to

Kermit.  This gives the protocol greater flexibility, but has the negative

effect of increasing packet overhead and slowing overall data thruput.

Classic B defined a fixed quoting set of the first 32 ASCII characters, but

this proved to be excessive for most microcomputers.  B+ allows the quoting

set to be negotiated during protocol session startup, so that the smallest

nessessary set is used to maximize thruput.

One of the most common questions I've heard about B over the years is,

"How do I add it to my PC-based BBS?"  I cannot recommend using B+ anywhere

but on CIS for typical FTP uses.  B+ is optimized for CIS' packet-switched

networks and is the best protocol to use by far in that environment, but on

direct PC-to-PC links the high per-packet overhead of B+ causes thruput

to suffer dramatically.  Also, a B+ "host-side" program can be difficult

to interface to current BBS technology for forking protocol modules,

because the filename prompting and startup process is different from the

common protocols like XMODEM.  For BBS use, you are much better off to use

ZModem.  B+ would, however, make a good base protocol for a transaction

processing system or similar project that used serial links to other

machines (I have used a modified version of it for such a system between a

main VME-based processor and AT workstations.)  Earlier beta versions of

this library provided a host-side object derived from the BPProtoFTP

object, but recent changes in CIS policy would require each person

receiving this library to obtain a (no-cost) license directly from CIS

before receiving the source or working TPU so it has been eliminated.

(I've tried to remove all references to the BPProtoHost object throughout

this document; if I've missed some please forgive me.)

---- Specifics ---------------------------------------------------------------

OOBPLUS implements the B protocols in a layered fashion, just as the CIS

hosts.  An abstract parent object handles the basic flow management and

packetization chores, while child objects implement the specific FTP and

GIF receive services.  A hierarchial diagram:

AbstractProtocol

��� (...other derived APRO protocols...)

��� BPProtocol                      {abstract BPlus parent}

��� BPProtoFTP                  {File Transfer services}

��� BPProtoGIF                  {GIF receive/decode services}

The BPProtocol abstract parent is derived from Async Professional's

AbstractProtocol object to take advantage of some of its variables and

methods.  Like other protocols derived from AbstractProtocol, BPProtoGIF

can be instantiated "on the fly", as it is needed. However, the

BPProtoFTP object needs to be instantiated whenever the main program is

receiving and processing characters from the commport.

Because B is a transport level protocol, the host may send non-protocol

data between protocol packets, or between the protocol support handshake

and the first actual protocol packet.  The term "protocol session" is used

to denote some period of host-remote activity that begins with the

handshake and ends with with a termination packet.  A FTP or GIF-receive

session may be a complete protocol session, or there may be other data

and/or commands issued via protocol packets.  Some of the variables used in

the BPProtocol object and its children are "session dependent" and are

initialized whenever the handshake is performed, while others are

re-initialized each time a FTP or GIF-receive subsession is started.

B+ requires an 8-bit data path.  The CIS hosts default to E71 port

parameters when you connect to the host, so your terminal handler must be

able to switch to N81 "on the fly" for a protocol session, and back to E71

when the session is complete.  Such a flying switch, with data streaming

into the port, is tricky to do without dropping characters.  The SetLine

method in Async Professional's AbstractPort object can be used to make the

switch.

Considerations for protocol handshake and initialization

--------------------------------------------------------

Most conventional protocols, such as XMODEM, are considered "receiver

driven" because whichever end of the protocol link is receiving the primary

data flow manages the protocol.  B+ on the other hand is considered "host

driven", because one end of the link is the "master" or host while the

other is the "slave" or remote, irrespective of the direction of primary

data flow.  On CIS, the host is always the CIS computer, with the local

PC system being the remote.

Starting XMODEM is typically a two-step process: you start XMODEM on the

other end of the link, and then you start it on the local end.  B+ doesn't

work this way; the host always starts the actual protocol processing

session via command to the remote, without user intervention.

B+ handles this as a two-step process.  First the host issues an <ENQ>

char to the remote.  When the remote "sees" an <ENQ> in the incomming

data, it calls the BPProtocol.HandleENQ method.  This method responds with

a specific sequence of characters as a "support/sync handshake":

for Classic B:  <DLE><'0'>

for QB:         <DLE><'+'><DLE><'0'>

for B+:         <DLE><'+'><'+'><DLE><'0'>   (we use this)

This tells the host what level of B is supported by the remote and that

the remote's initial packet sequence number will be 0.  It also initializes

several internal variables that are protocol-session dependent.  Once the

remote has responded to the <ENQ>, the host may at any time (but not

nessessarily immediately or as the next char) send the remote the

two-character sequence <DLE><'B'>, the first two characters of a protocol

packet. The remote software's terminal handler must look for any <DLE>

received and call the protocol handler function bpDLESeen; the bpDLESeen

method verifies the <'B'> char and continues protocol processing or exits

as is appropriate.

This means that the protocol handler object needs to be instantiated

when you start your terminal handler loop (or before), so that the <ENQ>

handshake properly initializes the nessessary vars in the object for later

when the <DLE><'B'> sequence is received.  For FTP and GIF uses, the

<DLE><'B'> will almost always immediately follow the <ENQ> response, but

for non-FTP purposes this may not be the case so we have to handle it that

way.  Also, you can set your user options at the CIS host end so that an

<ENQ> is sent immediately after login (as a handshake) without starting

a protocol session, so the <ENQ> must be handled throughout the online

session.

Packet definition and layout

----------------------------

B Protocol "packets" are like XMODEM "blocks", but the term packet is

used because the actual data within may be self-standing and have no

connection to data in previous or subsequent packets.  Each packet provides

the information nessessary for the packet handler to determine what to do

with the data, and even within a FTP or GIF session there may be packets

that do not contain file data.  For example, in the CompuServe Information

Manager, when performing a FTP some packets are not file data but instead

is data used to update the "tracking" status line at the bottom of the

screen.  During a protocol session, all commands and data in both

directions, other than packet ACKs/NAKs, are sent via packets.

The layout of a B packet:

<DLE><'B'><SeqNum><...quoted data...><ETX><quoted checkvalue>

The <DLE><'B'> identifies this as a B protocol block.  The <SeqNum> field

is an ASCII digit 0 thru 9 (ASCII $30 to $39) incremented each packet and

wrapping back to 0 from 9.  <...quoted data...> is the data block for this

packet, and contains any command info needed by the packet handler as well

as any "raw data" that might be needed in this packet.  The data is quoted

as nessessary to protect volatile characters.  <ETX> is the "end-of-packet"

flag character, and is sent unquoted.  <Quoted checkvalue> may be either a

one-byte arithmetic checksum or two-byte CRC depending on negotiated

settings (see below), and is quoted as nessessary.  Any given packet can be

a maximum of 1024 data bytes in size and can be less (as few as one byte).

The maximum size to expect is defined per handshake, and typically is

chosen based on current baud rate to provide a good balance between thruput

on good packets against turn-around time on flawed packets.  CIS has shown

the best average has proven to be a maximum packet size that gives a packet

transmission time of 4 to 5 seconds at the current bps rate; at 300baud the

packet size should be 128 bytes, at 1200bps it should be 512 bytes, and at

2400bps and above it should be 1K bytes.

Quoting is performed by sending a <DLE> followed by the quoted data

byte. If the byte to be quoted in in the range $00..$1F, it is OR-ed with

$20 before sending, and if in the range $80..$9F it is OR-ed with $60.

When the receiving side sees a <DLE> in the packet other than in the

header, it assumes the next byte is quoted and reverses the sender's

quoting after discarding the <DLE>.  The checksum/CRC calc is performed on

the byte's "natural" value, i.e. the CRC or checksum calc is performed on

the byte before quoting on the sending side and after the byte is un-quoted

on the receiving side. The <DLE> is discarded without inclusion in the

checksum/CRC calculation.

Classic B used a fixed quoting set, all chars in the range $00..$1F.

This proved to be excessive on most systems, increasing overhead

unnessessarily.  In PC-based applications on most networks, only six

characters actually need to be quoted: <DLE>, <ENQ>, <ETX>, <XON>, <XOFF>

and <NAK>.  The first three and <NAK> are used by the protocol itself and

must be protected to prevent the protocol from becomming confused, and the

flow control chars must be quoted to prevent unintended XOFF deadlocks.

Other platforms require different quoting sets, so B+ defines several

"standard" sets, but we only use 2: DQDefault (the minimum 6-char set) and

DQFull (the full set, equal to the Classic B set).  For B+, DQFull is only

used during handshake, since at that point we have not negotiated the

quoting set and protocol level, and must assume the worst.

Each packet from either end must be acknowledged by the receiving end,

although such acknowledgements may be delayed so that multiple packets are

outstanding.  Positive acknowledgements (ACKs) are sent as <DLE><SeqNum>

where <SeqNum> is the sequence number of the packet being ACKed.  Negative

acknowledgements (NAKs) are sent as a single <NAK> character.  Since the

NAK does not specifiy to which packet it applies, the packet handler must

keep track of how many packets are outstanding and assume the NAK refers to

the oldest.  This also means that when a NAK is received, *all* outstanding

packets are discarded in order to keep the sequencing straight.  Obviously,

the higher the number of outstanding packets, the greater the amount of

data that is resent - and the longer the "turn-around" time on a protocol

error.  B+ is currently defined as having a maximum of 2 outstanding

packets.  Tests by CIS have proved that more than two do not noticably

improve thruput while seriously degrading performance on error turn-around.

Packet types

------------

Although all packets follow the above format, packets have different

types.  The first data byte of the packet (sometimes the first 2 bytes)

defines the packet's type and is not part of other data in the packet.  For

FTP and GIF uses, the only types we are concerned with are "T", "N", "F"

and "+" packets.

"T" packets are transfer management packets.  There are several subtypes

of this packet type, used for specific transfer management jobs.  "N"

packets are "Next Data" packets, meaning they contain data that is a

continuation of the previous packet(s).  "F" packets are Failure packets

and mean the protocol session is aborting for some reason.  Again, there

are several possible "F" packet types depending on the reason for the

failure.  The "+" packet is the special BPlus Parameters Handshake packet

used during protocol initialization.

"T" packets fall into the following categories:

"T"   - Transfer Init.  This packet contains the file type (Binary or

ASCII), the transfer direction relative to the remote (Upload

or Download), and the file's name for the remote's storage.

Sometimes called the "TD" packet.

"TI"  - Transfer Information.  This packet can contain several

different pieces of information (see the full B+ spec for a

detailed description of the fields in this packet).  We

currently only use the FileSize field.

"TC"  - Transfer Complete.  Signals the end of the current file.

This packet will never contain file data; the last previous

"N" packet contained the end of the file or GIF data.

"Tr"  - Resume Request.  Special-case packet, sent by remote to

request the host attempt a resume.  The "Tr" packet will

contain the size of the file (in bytes) that resides on the

remote's disk, and a CRC of the file calculated in the same

manner as a packet's CRC checkvalue.  Note that CIS does not

currently support Upload Resume (where the CIS host resumes

a failed transfer of a file sent by the remote.)

"Tf"  - Transfer Resume Failed.  Special-case packet, sent when a

resume attempt request made by the remote failed.  When the

remote requests a resume attempt it sends the length and CRC

of the "piece" of the file it has; the host verifies it's

local file is at least as long and has an equal CRC over the

matching length, and if either test fails the host sends this

packet.

The "N" packet is pretty much self-explanatory; it is a "Next Data

Block" packet and contains only actual file contents data.  Since B packets

can be any length from one byte up to the maximum specified length, only

the exact file length is transmitted without padding.

"F" packets are typically sent for one of three reasons: the remote's

user requested the protocol be aborted via keystroke or other manual

method, the protocol has detected a hard failure such as too many

sequential error packets, or the remote experienced a file I/O error.

The host can issue Failure packets as well, and although they are rare we

must be prepared for them.

Typical Failure packet types:

"FA"  - Failure/Aborted.  Sent when the user requested an abort.

"FN"  - Failure/Not known.  Sent when an unsupported service was

requested by the host.  CIS supports several file types

besides Binary and ASCII (for use on non-PC systems), but we

can't handle them so we send a FN if the file type is not

Binary or ASCII.  Also, transfer direction flags other than

Upload and Download may be used on non-PC systems, so if we

see a direction flag other than "U" or "D" we send a FN

packet.

"FE"  - Failure/Error.  Usually sent when a hardware error occurs,

such as file I/O error.

Only the first two characters in a Failure packet have meaning to the

CIS hosts, the "F" and the failure-type flag.  Traditionally a text string

is also sent in the packet as descriptive info on the failure, but this is

only for use by the remote if it chooses to display the reason string.

OOBPLUS follows this tradition.

The "+" packet type is a special case.  It provides a method for the

host and remote to negotiate what level of B+ capabilities is to be

supported during this session and the quoting set to be used.  The "+"

packet is typically sent between the <ENQ> handshake and the "T" packet.

The host sends its "+" packet first with its capabilities, and the remote

compares this information to its own capabilities and sends back a "+"

packet with what it can support.  The usual "+" packet handshake will set

the number of outstanding packets that can be supported both on send and

receive, the type of checkvalue used (CRC or arithmetic checksum), the

maximum size of a packet in this session, whether the host will support

download and/or upload resume and whether the remote can support it, and

the quoting set to be used.  See the interface section of OOBPLUS and the

ProcessTransportParams method of the BPProtocol object for more detail.

Flow of control

---------------

The overall flow of control of a B protocol session is basically the

same whether the transfer direction is Upload (from remote to host) or

Download (from host to remote).  A diagram of a typical download is the

easiest way of visualizing the flow.  An upload would be the same except

for the direction of the "N" packets and the possible resume request (no

resume on uploads).  Notice that the host always starts the ball rolling.

(The following information is specific to FTP and GIF uses of the BPlus

protocol.  Exact ordering may be different in non-FTP or GIF uses such as

HMI.)

HOST                                           REMOTE

----                                           ------

<ENQ> sent            ----------->              Init vars,

Receive ENQ resp.     <-----------          Send ENQ Response

Set vars to match

... (possible non-protocol data flow between sides) ...

(NOTE 1)

Send "+" packet       ----------->     Compare host's options with ours,

set our options to "best-fit"

the host's settings

Configure to match    <-----------     Send "+" packet with our options

remote's capabilities

... (possible non-protocol data flow between sides) ...

(NOTE 2)

Send "T" packet       ----------->     Process "T" packet info, enter

protocol FTP mode, open file

{!! IF FILE DOES NOT EXIST, SKIP TO "JUMP HERE" !!}

(NOTE 3)

File exists: calc CRC on file,

build "Tr" packet with file's

size and CRC,

Gets "Tr" packet,     <-----------     send "Tr" packet

check size & CRC:

IF AND ONLY IF the

file size and/or CRC

checks fail, send

empty "Tf" packet     ----------->     "Tf" recd, rewind on file (either

overwrite from beginning, abort or

question the user what to do)

ELSE send first

"N" data packet,

contents of which

are assumed to

be appended to

the existing file     ----------->     append data to file, ACK packet

{JUMP HERE}

Send "N" packets      ----------->     write data to file, ACK packet

...(as many times as needed)...

Send "TC" packet      ----------->     Close file, flush pending ACKs,

exit FTP mode

...(back to normal terminal operations)...

Notes:

1) If the host is to send a GIF stream for decode/display, it will send the

"Start GIF Decode" sequence immediately before the "+" packet.  It will

also send an <ENQ> after the start-decode sequence, so we must handle it.

2) If in GIF decode mode, there will be no "T" packet as it is unnessessary

(there is no filename, and the direction and data type are fixed at

Download and Binary).  The first packet after the "+" packet will be the

first "N" packet of the GIF stream's data.

3) No resume is possible on a GIF stream; it always starts from the

beginning.

Performance, thruput and packet size

------------------------------------

When send-ahead is enabled (QB or B+ mode), multiple packets can be

outstanding (meaning no ACK or NAK has been processed for that packet).

The design of B+ is such that it always tracks the number of packets that

are outstanding rather than a specific offset within the file (ala ZModem)

or other criteria.  This simplifies the tracking process, but it has the

negative effect of requiring all outstanding packets to be discarded and

resent when a packet is NAKed for some reason.

Ultimate thruput for any protocol is a function of several factors: the

port speed, percentage of overhead, turnaround delays on packet ACKs, and

turnaround delays caused by defective packets that must be resent.  B+

essentially eliminates ACK turnaround delays, so raw thruput will closely

approach the raw port speed.  On binary transfers, B+'s quoting overhead

will typically reduce data thruput to 92% - 96% of the raw thruput, while

ASCII transfers will seldom be less than 98% of raw thruput.  However, if

one or more packets are defective and must be resent, the turnaround delay

for the defective packet will be several seconds because multiple packets

must be discarded and resent.  Obviously this seriously degrades overall

thruput, although the degredation percentage is a function of the number of

packets in the transfer, the size of a packet and the number of "hits" that

cause turnaround. A substantial amount of testing has been performed by CIS

personnel on this subject, and it has been determined that a send-ahead

window of 2 packets gives the best thruput with minimal turnaround delay,

when used with a packet size that gives a per-packet transmission time of 4

to 5 seconds at the current port bps rate.  Smaller packets improve

turnaround delay substantially, but hurt thruput on "clean" packets;

packets larger than 1024 bytes do not markedly improve clean thruput but

seriously hurt turnaround times.  I have implemented packet size as a

function of port bps rate in the same manner and using the same criteria as

CIS software, but my own experimentation has shown that, at 2400bps, a

512-byte packet size reduces average data thruput by only 2% - 5% while

reducing turnaround delays 50%.  A like reduction of default packet size at

1200bps produces similar results.  For most users the "factory settings"

will be fine, but you may wish to make max packet size settable via

external configuration to provide greater flexability - for example,

providing a config option for "normal lines" or "noisy lines", and reduce

the max packet size setting if the "noisy lines" option is selected.

NOTE: current B+ implementations on CIS do not support re-negotiation of

transport parameters during FTP or GIF sessions once a given protocol

session has started, although it will "drop out" of send-ahead mode and

revert to "send and wait ACK" mode if it sees a high percentage of

defective blocks.  The special version of B+ used by CIS hosts for HMI

does support such re-negotiation.  Adding re-negotiation capability for

standard FTP and GIF sessions would be wonderful, but would break existing

implementations.

Other things

------------

Most developers will implement watching for <ENQ>, <DLE> and GIF Decode

Start signals from the host within some sort of terminal emulation code,

possibly based on Async Professional's OOEMU unit's ANSI emulator.  CIS

supports ANSI/VT100, so this is convenient and works well.  However, be

aware of one point: CIS hosts actually use VT100 terminal command

sequences.  VT100 and DOS ANSI (ANSI.SYS and emulators built to mimmic it)

are both unequal and improper subsets of the full ANSI X3.64 terminal

control standard.  Specifically, DOS ANSI handles defaults on the "J" and

"K" commands differently than VT100, and an emulator that exactly mimmics

DOS ANSI will find itself clearing the whole screen at the wrong times.

Be sure your emulator setup processes the "J" and "K" commands in the

VT100 manner when on CIS.

CIS also can support a subset of the VT52 sequences.  The <ESC><'I'>

interrogation is a VT52 sequence, for example.  You may wish to implement

VT52 processing as well as VT100 to eliminate potential misunderstood

commands.  A child of Async Professional's AnsiEmulator can easily be

derived that handles both command sets within one emulator, and is

recommended.

When you log on to CIS, the node you call defaults to Even parity/7

data bits parameters.  As mentioned above, a proper full B+ implementation

needs to know when to switch to N81 parameters for the actual protocol and

back once the protocol session is complete, but port and/or modem hardware

can cause problems with the switch.  You are usually better off to force

N81 parameters for the entire online session.  To do so you need to ignore

both parity and framing errors from the port handler, and to mask all

incomming non-protocol chars to 7 bits.  CIS has a user configuration

option to select N81 parameters, but this won't become active until you

have logged in to CIS, so be prepared to cope.  The TERMBP example program

demonstrates one method of handling this.

CIS supports other networks besides its own.  Such "suplimental

carriers" often behave differently than CIS' own network.  Tymnet in

particular seems to have occasional problems, though these are usually

specific to the node you call.  Specifically, Tymnet uses a PAD (Packet

Assembler/Disassembler) system that uses the <DLE> char as a signal

itself.  In order for CIS to use the <DLE> char for its own purposes, it

must tell the Tymnet network to consider that char "transparent".  CIS

issues the nessessary command automatically, but some Tymnet nodes are

slow to update themselves.  In this case the <DLE> is seen as a PAD

command and is "swallowed" by the Tymnet node.  Tymnet then sees the <'B'>

as an unknown command and issues a "pad command error" string to the

remote.  Meanwhile CIS thinks it has switched things and has sent the "+"

packet, and is waiting for a response.  A long timeout ensues.  This

doesn't seem to crop up too often on normal FTP transfers, but is not

uncommon on GIF sends.  Thankfully this problem only seems to occur on a

few nodes.

---- GIF Information --------------------------------------------------------

First off, if you intend to implement GIF decoders, either off- or on-

line, you absolutely need the GIF 89a spec document.  It can be obtained on

CompuServe (see the top of this file for info.)  OOBPLUS is primarily a B

protocols unit, with GIF decoding technology provided as demonstration

only; trying to go into detail on GIF itself and the internals of a GIF

decoder is really outside the bounds of this document.  However, the GIF

spec documents provide very little information on actual online decoder

implementation details other than some general rules.  I'll try to fill

those gaps here.

GIF (Graphics Interchange Format (sm)) is a definition for transmission

and display of raster-based graphics image data - in other words,

"bit-mapped" graphics.  GIF supports images of up to 2048 by 2048 pixels

and up to 256 colors.  Extensions allow support for "full-color" images

such as 24-bit Targa.  GIF provides services for anamation and text display

as well.  GIF was developed by CompuServe as a medium for online graphics,

with an eye towards future development in online graphics environments (ala

Prodigy.)  GIF in combination with "Transport Layer" B+ protocol provides a

rich platform for this type of environment.  GIF rapidly gained acceptance

in the file-based imaging world as well, because of its space efficiency,

ease of processing and device independence - so rapidly, in fact, that

online GIF development has been practically non-existant.  CIS early on

set up a "Developer's Group" of graphics specialists and programmers to

help steer the directions of GIF, but only myself and one other member of

that group worked primarily with the online development aspects of the

spec (and that other member has pretty much ceased online development.)

Not surprizingly, only myself and one or two other members of that group

develop primarily in Turbo Pascal; C and assembler are the norm.

My primary purpose in providing both the GIF children to the B+ objects

and the example decoders is to spark interest in GIF development,

especially online.  The horizons opened up by realtime online graphics are

huge; the surface has only been barely scratched.  I am deluged with calls

from other developers and even more from companies and individuals with

some idea for online graphics usage, wanting me to write this or that

program (even whole BBS systems with built-in GIF databases!); I've taken a

couple of these contracts but there's many, many more where they came from.

(Tip: the real estate industry especially seems to be hurting for a good

BBS-type system for Boards Of Realtors or Brokers to run for their agents,

that provides a database of listings with images made from pictures of the

properties.  I get more requests in this area than any other.  I'm not sure

why but Medical Imaging is another popular area, although GIF's standard

color resolution is generally considered insufficent for many medical

imaging needs.)

GIF general info

----------------

GIF is a stream format.  Besides making it simple to implement online

display, this means GIF image data may formally be included in other data

or that multiple GIF streams may easily be made into a "graphics database".

CIS has steadfastly refused to formally define a "file format" for GIF, but

a format for file storage has evolved over the years and is adhered to by

all known decoders whose primary purpose is display of GIF files.  As long

as the first 6 bytes of the file's contents are the GIF signature

("GIF87a" or "GIF89a"), the image data is contiguous and a valid

terminator exists, it can be handled by most any offline decoder.

CIS maintains a "storage class" flag for each file in its forum

libraries. If you BROwse a library's directory, you will see a "/bin" or

"/gif" or "/rle" next to the filename on many entries.  When uploading a

file to CIS, you are prompted for, among other things, the file's type.

Most users assume that "binary" and "ASCII" are all there is, but GIF image

files should be uploaded with a file type of "GIF".  Even if a file

contains a GIF image and has filename extension of ".GIF", CIS will not

allow online display of the file's contents unless the file's storage class

is GIF as well.

GIF image data, with the exception of the header blocks, is binary

raster-based sequential pixel color data, compressed using a modified form

of the LZW compression method.  The image raster data is uncompressed on

the fly as it is being displayed.  It is somewhat difficult for the LZW

decompressor to itself determine when a invalid byte is received - usually

it is not detected until it has corrupted the codes table in the decom-

pressor, often leading to a lockup.  Most decompressors do no or only

minimal error-checking; they assume the data handed to them is valid.

For this reason the GIF spec notes that the image data stream must be

guaranteed error-free.  For disk-based streams this is assumed, but for

online decode the stream must be protected by protocol.  The GIF spec does

not define one particular protocol that is recommended for image streams.

This is important for the bigger world of Bulletin Board Systems; it means

that it is perfectly acceptable to implement an online GIF decoder that

uses ZModem or some other protocol, so that users of BBS' without CIS B+

support can still easily view GIF images in an online environment.  Since

the main thrust of this unit is the B+ protocol I am only detailing

decoding GIF streams from B+ encapsulation, but careful study of the

example code in OLGIF.PAS should make it fairly obvious how to implement a

GIF decoder using the other protocols Async Professional provides.

The GIF spec does define the two protocols CIS computers will use for

transmission of online GIF display streams: raw stream data (assumed to be

protected by a link-level protocol such as MNP or V.42, and not to be used

without such link-level protection) and B+ protocol.  It also defines a

"handshake" system to be used to determine both that GIF is supported for

online display, and what level of protocol and hardware support is

available on the particular remote.  While only CIS uses this handshake

system, it behooves developers to support it for online decoders based on

other protocols as well, as a standard needs to evolve on this particular

point.

The handshake consists of 3 different "private" ANSI command sequences

("private" because only CIS defines them).  These use the standard ANSI

X3.64 introducer sequence (<ESC><'['>) followed by a right pointer bracket,

a numeric code specifier and the command type tailer character. The

sequences are:

<ESC>[>0g   - sent by host to interrogate remote for GIF/protocol/hardware

support.  If the remote supports GIF display, it returns a

string consisting of codes that tell the host to use raw

stream or B+ transmission, what screen resolutions (X by Y,

and number and depth of colors), whether hardcopy output is

supported, and which version of the spec the decoder supports

(87a or 89a).  See the spec document for the various details

of this code scheme.

<ESC>[>1g   - begin online decoding to the local screen.  This will be sent

by the host immediately before the <ENQ> to handshake B+

support. Once this is received, you can safely assume that

the next <ENQ> will be to handshake the start of a B+ session

to transmit the GIF stream.  Non-B+ sessions will see the

next char after the command be the first char of the GIF

signature starting the image data.

<ESC>[>2g   - begin decoding to the local hardcopy device.  Lots of systems

still in use simply do not have the video capabilities to

adequately display GIFs, especially the higher-resolution/

high number of colors images available from many sources.

Also, some areas of the service provide graphics that are of

more use as hardcopy than on the screen (such as the weather

maps, or the stock performance graphs in the TRENDS area).

I don't support hardcopy decode in this set of examples, but

it is fairly easy to implement and a handy capability.  The

handhake sequence above can be used to tell the host we want

hardcopy decode, and for color printers can tell the host

to use provide normal color information.

One important note concerning the "0" code above: it is *not* sent every

time an online GIF decode is requested.  In the forum areas of CIS, it is

usually sent the first time you request a online decode and not sent again

until you leave that forum, either to another area of CIS or back to the

same forum.  Most of the non-forum areas of CIS that support GIF at all

make GIF support their primary purpose, such as the weather maps areas and

TREND; these areas will send the "0" inquiry when you first enter the area

before they display any menu text or prompts.  Your program must be able to

handle the "0" inquiry at any time, just as it must be able to process a B+

<ENQ> or <DLE> at any time.

Until recently there were a few areas of CIS that provided GIF support

but did not support B+ encapsulation (the TREND area was the most commonly

accessed.)  These areas defaulted to raw stream even if the remote reported

its ability to handle B+, so a decoder had to be able to determine if B+

was initializing or not and process incomming data accordingly.  As you can

imagine, this lead to a lot of headaches, and problems for users when the

unprotected stream took a line hit and locked up the system.  To the best

of my knowledge there are no such areas left on CIS; all areas that provide

any support for online GIF decoding support B+ encapsulation, and all areas

support B+ by default unless specifically told otherwise by the "0" code

response string described above.

The flow of control for B+ when processing a GIF stream for display is

very little different from a "normal" FTP session, with a couple of notable

exceptions:

1) As mentioned in the B+ infomation above, there is no "T" packet sent

during a GIF display.  Since many of CIS' GIF streams are built and sent

"on the fly" in real time they have no filename, and obviously the

transfer's direction and data type are fixed at "Download" and "Binary".

This only complicates matters a little bit, and only if you are writing the

image data to a file as well as displaying it (something every online

decoder should do, but that CompuServe's own software products do not.)

If you do write the image to a file, it's your responsibility to determine

a name for the file and manage getting the stream data into the file; see

OLGIF.PAS and TERMBP.PAS for an example.  Such processing really should be

part of the BPProtoGIF object, but I have not included it that way because

the spec does not define the capability; you may wish to derive a child of

BPProtoGIF that does.

2) The data in each "N" packet is yours to do with as you wish.  The

BPProtoGIF object provides a method (bpGetGIFDataBlock) that gets the next

pending "N" packet and hands you the data within it; what you do with it

from there is your business.  This provides an example of possibilities for

non-FTP uses of B+, but it also points out that protocol management is to a

large extent outside the bounds of the BPProtoGIF object - if you don't

call bpGetGIFDataBlock, or fail to call it in a timely fashion, the

protocol will time out and abort.  Also, to give the GIF decoder enough

power to properly manage the protocol flow, packets must be ACK-ed by the

GIF decoder itself (defective packets are handled automatically within the

GetPacket method of the BPProtocol parent of BPProtoGIF).  The same goes

for sending "F" packets in case the user wants to abort the decode or the

decoder itself has a problem.  With power comes responsibility; study the

OLGIF.PAS file for an example of how to manage the flow control properly.

OOBPLUS and GIF

---------------

GIF data may come from a single file, the serial port, as a piece of a

file from database-type storage, etc., and may be sent to the screen, a

printer, or some other output device, so it makes sense to isolate the

process of getting the stream's raw data and outputting the processed image

data from the decoder's inner workings.  Doing so also makes implementing a

GIF decoder thru a protocol much easier.  The demo decoders' kernel

(DEGIF.PAS) names two procedure pointers, one to a function which returns

the next byte of raw data and one that does something with a processed

raster line of display data, and refers only to these proc pointers for

source and destination.  Various "shells" can then be wrapped around the

decoder to implement the routines for a particular decoder's needs.

This allows any protocol that gathers data in packets or blocks to hand

a block of data to the decoder shell and go on about its business; the

shell will dole out the bytes to the decoder.  The downside (at least for

single-thread operating systems like DOS) is that actual decode/display

only takes place between packets, which gives the decode the appearence of

occuring in spurts.  It also makes basing a GIF decoder on true streaming

protocols like ZModem a bit trickier, as the decoder has to get some time

to do its job.

The BPProtoGIF object provides two methods for the decoder.  The first,

the bpDLESeen virtual method, handles the "+" packet reception and

processing.  It is a function, returning True if it is successful and False

otherwise; the decoder should abort if the function returns False.  The

second is the bpGetGIFDataBlock function, which returns True and a packet's

worth of raw GIF stream data if successful, False if the packet reception

failed for some reason.  bpGetGIFDataBlock also returns a Boolean var

parameter showing whether the "TC" packet has been received, to make it

easier to see the end of the protocol session.

When the main program sees the GIF decode startup ANSI sequence, it

should initialize itself and start looking for an <ENQ> from the port to

handshake the B+ protocol session.  Immediately after the <ENQ> should be

the <DLE> starting the host's "+" packet; the decoder calls bpDLESeen at

this point.  From here until we receive the "TC" terminating packet, all

port data will be B+ encapsulated GIF image data, to be processed by the

decoder.  We simply loop calling bpGetGIFDataPaket and handing the received

bytes to the decoder until done.  Review OLGIF for examples.

Video support considerations

----------------------------

There is a multitude of video hardware types available for PCs.  How do

we support 'em all?  Which should we support?

In today's hardware market, SVGA cards are the most common video cards

sold.  SVGA support is a "must" in a GIF decoder.  But...  Any graphics

programmer will tell you: programming for SVGA can be a royal nightmare.

At least a dozen "common" main SVGA chipsets (more all the time) and, once

past the "IBM-standard" EGA/VGA modes, each having different mode

resolutions, different mode select numbers for those resolutions that are

the same, and different coding requirements to manage video memory

bankswitching and plotting.

The TP BGI's provide a high level of device independence, and can be a

reasonable starting point for a GIF decoder.  But GIF's device-independent

nature doesn't keep device dependence from cropping up.  Because GIF

encoders and decoders are implemented on a wide variety of platforms, many

of which have video resolutions different from the common PC video modes, a

sizable percentage of GIF images are not "sized" to fit well on a PC screen

unless special handling is performed.  Also, since GIF is a high-resolution

format, it is commonly used with video hardware for which there is no BGI

support (including Targa, TIGA, IBM's new HGA and others).  Only a few

SVGA BGI drivers are available, and do not cover all common SVGA chipsets.

While decoder speed is not much of an issue in online environments until

the port bps rate reaches 9600 and above, a production offline decoder

that paints an image on the screen at BGI speeds will be considered

totally unacceptable. Finally, the BGI's are pretty topheavy considering

that GIF support only requires 2 video routines to do its job - a method

of setting the video card to the needed graphics mode and back to text

mode when done (a simple video BIOS call), and a routine to plot pixels or

raster lines on the screen.  An intelligently written set of routines done

in assembler that will detect and support all common video types from Herc

mono to SVGA, complete with SVGA chipset autodetect code and bankswitching

code, will take less code and data space than a single BGI driver linked

into the program.  Turbo Pascal 6.0's BASM makes an ideal environment for

the nessessary assembler, though I have stuck with straight TP in the demo

code for the sake of compatibility with earlier versions of TP and to be

more informative.  Although it can be done, it is no trivial job to write

straight TP code that directly manipulated SVGA hardware since most

chipsets require substantial diddling of ports; VGA and lower video types

can be dealt with in straight TP code quite acceptably, but assembler is

still preferred on these cards for best speed.

The easy way out is to use the BIOS services for pixel plotting.  This

is only marginally faster than the BGI's, but easier to deal with and with

substantially less code overhead.  It is a dangerous way to go, though,

especially on CGA and earlier EGA and VGA cards - these older cards'

BIOS routines for plotting typically disable interrupts while plotting the

pixel, which in an online environment leads to dropped characters at the

port and a high percentage of resent data.  And on Hercules monochrome cards

it is unavailable.  I have mixed the best of both worlds in the demo

decoders (see GIFVIDEO.PAS), but a production program should use direct

hardware manipulation as far as possible for best speed and minimum

potential interference with the serial port.

That said, there's a cure on the horizon: VESA.  VESA is a set of

standards for SVGA detection, mode selection and video manipulation that

provides a consistent set of mode resolutions, commands and data access to

eliminate most all the confusion between SVGA chipsets.  VESA is still new

but gaining support by leaps and bounds; all the major video manufacturers

support the VESA standards at least via a TSR.  Some are now bringing to

market cards that support VESA standards "hardwired" on the card,

although this is currently quite difficult due to the severely limited

room in most SVGA video BIOS ROM chips.  There are no VESA-compliant SVGA

BGIs available that I know of, but at least one is currently under

development and looks very good.  In this most recent version I have added

the nessessary VESA support to work with this code; it is in no way a

complete VESA interface, but it does work on all cards/drivers I have

been able to test with.

On the subject of CGA and Hercules mono support:  I have included

support for these hardware types, but really only for their instructive

capability.  CGA and Herc mono simply do not have the size and/or color

resolution to do justice to the vast majority of GIF images today unless

all sorts of sneaky tricks are used.  With CGA and Herc mono rapidly

becomming a thing of the past (a low-end VGA card and mono analog monitor

costs about the same as a Herc card and mono TTL monitor and a *lot*

less than CGA), there's even less reason to expend effort and code space

on their support.  However, if you need such support, you will need to

implement some sort of dithering method to do color reduction.  In GIF

context, "dithering" an image means to perform colors reduction so that

the colors in the original image are mapped to best fit the available

palette on the local system.  There are a number of common color-reduction

dithers in use.  Most all work on the principle of error distribution;

spreading out the color error over surrounding pixels.  Just about all of

them require the decoder to have access to pixel data "in front of" the

current pixel - data that is unavailable to a decoder getting its data

from the serial port unless relatively complex buffer management is used.

Implementing such dithers is left to your skills and imagination, with the

following tip: there is a large base of dither technology available in

LIBrary 14 of the PICS forum on CIS, although practically none of it is in

Turbo Pascal.  If you can wade thru C and assembler code, you'll find lots

of good information with which to get started.  A good dither method to

reduce 256-color maps to 16 colors is important for EGA support as well.

The switch from text mode to graphics mode is a sticky spot for online

GIF.  The header blocks must first be received so the decoder can

determine which mode to use before making the switch, so data will be

typically be streaming into the port when it is time to make the switch.

Most every video BIOS in captivity disables interrupts while it switches

modes, so at least a few bytes will be dropped by the port - causing a

defective packet and a resend.  This doesn't cause the protocol any grief

it can't handle, but it does slow things down unnessessarily.  The cure is

relatively simple: send the host an XOFF and wait for the port to become

"quiet" for some arbitrary length of time before making the mode switch.

Async Professional does not provide a method for waiting for a port to

quiese, so the demo adds a method for doing it; the method employs a sneaky

trick and goes against good OOP programming, so you may want to implement

such a method within the port handler object.  Don't forget to send an XON

after the mode switch is complete!

---- Beyond what's here: where to get help ----------------------------------

The demo decoders are fully compliant with the 87a version of the GIF

spec.  They will also decode GIF streams built to the 89a version of the

spec, but will ignore all 89a extensions.  The DEGIF.OBJ module is

copyright (c)1989 Cyborg Software Systems, Inc., 3119 Cossell Drive,

Indianapolis, IN 46224  USA., and used by permission of its author.

As mentioned before, dealing with the vagaries of SVGA hardware can be

a daunting task.  John Bridges, author of GRASP and PCPaint, is a past

master at it.  John periodically uploads a file to LIBrary 14 of the PICS

forum called VGAKIT.ZIP, that contains SVGA chipset autodetect and

bankswitch code, example plotting code, code to set and use special "not

supported by BIOS" modes on the VGA, and other juicy tidbits.  Be prepared

for MASM code written primarily to interface to MSC, but the conversion to

TASM and TP is fairly straightforward; John and I spent some time last

year working the kinks out so his MASM code requires minimal changes to

port to TP/TASM.

The best allaround source for GIF help is the Developer's Den section

(17) of the PICS forum.  The place is haunted by some of the best and

brightest graphics minds in the world, all ready and willing to share their

expertise.  You'll find specialists in ray-tracing, format conversion,

fractals, high-end video, scanning, etc.

I can be contacted via CIS at User ID# 70007,3574.  I frequent several

forums at least once a day, especially IBMCOM, PCVENB (TurboPower's section

6), BPROGA and PICS.  The TurboPower folks will be happy to help you as

well, but to help keep their support headaches in check I hope you'll

contact me first; they may need to forward questions to me anyway.

US Mail will reach me at:

Ozarks West Software

14150 Gleneagle Dr.

Colorado Springs, CO  80921

{==== Reference Section ======================================================}

The interface section of the OOBPLUS unit is provided below.

{-----------------------------------------------------------------------------}

{The following define is used to specify status updates at more frequent

intervals than normal.  This makes the status display more informative and

causes the "Time to go" field to just about tick like a clock on downloads,

but it can have a negative effect on thruput at higher bps rates and can

cause the CPS field to "jitter" somewhat, especially on uploads.}

{$DEFINE ShowRates}

{The following define enables support of online GIF decoding.  If you are not

going to use GIF support, un-defining this can both save about 400 bytes of

code space and allow you to use the unit even if you don't have TurboPower's

Object Professional or Turbo Professional libraries.  The provided example

program TERMBP.PAS does require Object Professional to compile.}

{$DEFINE SupportGIF}

{$IFNDEF UseOpro}

{$IFDEF SupportGIF}

!!! The options selected are incompatible with this unit !!!

{$ENDIF}

{$ENDIF}

interface

uses

DOS,

{$IFDEF UseOpro}

OpCrt,

{$ELSE}

{$IFDEF UseTpro}

TpCrt,

{$ELSE}

Crt,

{$ENDIF}

{$ENDIF}

ApMisc,

ApTimer,

ApPort,

ApUart,

OOCom,

OOAbsPcl;

const

UnitVers = '1.2f';

UnitDate = '24-Apr-91';

{consts needed here for status, continued from ApMisc}

ecResync    = 9980;

ecWaitACK   = 9981;

ecDropout   = 9982;

ecHostCan   = 9983;

ecFileIO    = 9984;

ecTryResume = 9985;

ecHostResume= 9986;

ecResumeOK  = 9987;

ecResumeBad = 9988;

ecOverwrite = 9989;

ecUnPacket  = 9990;

BP_Timeout_Max = 30;   {max allowed timeout per-char}

BP_Error_Max = 10;     {max sequential errors}

BP_Buffer_Max = 1032;  {largest data block available}

BP_Abort_Max = 3;      {number of abort requests req'd to trigger Override}

BP_SendAhead_Max = 2;  {max number of packets we can send ahead}

{minimum <ESC><'I'> (and GIF support interrogation) response strings}

{$IFDEF SupportGIF}

ESCI_Response : String[80] = '#IB1,SSxx,GF,PB,DT';

{see the GIF87a or 89a spec for explanation of these codes}

GIFReplyEGA = '#87a;1;0,320,200,4,0;0,640,200,2,2;0,640,350,4,2';

GIFReplyCGA = '#87a;1;0,320,200,2,0;0,640,200,1,0';

GIFReplyHGC = '#87a;1;0,720,350,1,0';

GIFReply : String[60] = GIFReplyEGA;

{$ELSE}

ESCI_Response : String[80] = '#IB1,SSxx,PB,DT';

{$ENDIF}

type

{used by GetResumeProc for resume request handling}

ResumeResultType = (xfrResume, xfrOverwrite, xfrRename, xfrAbort);

BufferType = Array[0..BP_Buffer_Max] of Byte;  {a buffer of data}

SABuffType =                                   {windowing buffer:}

record

Seq : Integer;                             {this sequence number}

Num : Integer;                             {this packet's data size}

Buf : BufferType;                          {this packet's data}

end;

SPackets = Array[0..BP_SendAhead_Max] of SABuffType;

QuoteArray = Array[0..7] of Byte;         {for quoting params sets}

const

DQFull    :      {all chars in ranges $00..$1F and $80..$9F}

QuoteArray = ($FF, $FF, $FF, $FF, $FF, $FF, $FF, $FF);

DQDefault :      {ETX ENQ DLE XON XOFF NAK}

QuoteArray = ($14, $00, $D4, $00, $00, $00, $00, $00);

DQExtend  :      {same as DQDefault plus XON & XOFF w/ high bit set}

QuoteArray = ($14, $00, $D4, $00, $00, $00, $50, $00);

DQClassic :      {Classic B set, all chars in range $00..$1F}

QuoteArray = ($FF, $FF, $FF, $FF, $00, $00, $00, $00);

type

ParamsRecord =                            {xfer params record:}

record

WinSend,                              {send window size}

WinRecv,                              {recv window size}

BlkSize,                              {block size (* 128)}

ChkType  : Byte;                      {check type, chksum or CRC}

QuoteSet : QuoteArray;                {chars to quote}

DROpt,                                {DL Recovery option}

UROpt,                                {UL Recovery option}

FIOpt    : Byte;                      {File Info option}

end;

{protocol direction options}

DirectionType = (Upload, Download);

BPProtocolPtr = ^BPProtocol;

GetResumeProc = function(BPP : BPProtocolPtr) : ResumeResultType;

ChkAbortProc = function : Boolean;

BPProtocol =                               {abstract BPlus object:}

object(AbstractProtocol)

Ch        : Integer;                   {curr char sent/recd}

Quoted    : Boolean;                   {true if last ch recd was quoted}

QuoteTable: Array[0..255] of Byte;     {our active quoting table}

Checksum  : Word;                      {may hold CRC}

Direction : DirectionType;             {upload or download}

DefResume : ResumeResultType;          {default resume handling}

GetResume : GetResumeProc;             {determine how to handle resume}

ChkAbort  : ChkAbortProc;              {see if user wants an abort}

HisParams : ParamsRecord;              {host's parameters}

OurParams : ParamsRecord;              {our parameters}

AbortCount: Integer;                   {# of abort requests so far}

ResumeFlag: Boolean;                   {true if resuming an aborted dl}

ResumeOK  : Boolean;                   {true if resume was successful}

Aborting  : Boolean;                   {true if processing abort}

FatalAbort: Boolean;                   {true if OverrideAbort}

ShowStatus: Boolean;                   {False only for GIF}

PacketRecd: Boolean;                   {true if packet recd in SendPacket}

BPlus     : Boolean;                   {true if in full B+ mode}

ClassicB  : Boolean;                   {true for "original" B proto}

UseSQuote : Boolean;                   {true if using special quote set}

SQuoteSet : QuoteArray;                {user's specified quote set}

RSize     : Integer;                   {size of last recd buffer}

BuffeRSize: Integer;                   {current allowed recv size}

RBuffer   : BufferType;                {receive buffer}

SBuffer   : SPackets;                  {sending buffers}

SeqNum    : Integer;                   {current sequence number}

Next2ACK  : Integer;                   {packet pending ACK}

Next2Fill : Integer;                   {packet to load for send}

SAMax     : Integer;                   {highest current sendahead cnt}

SAWaiting : Integer;                   {# of packets outstanding ACKs}

SAErrors  : Integer;                   {keep track of SendAhead errors}

R_Raw     : LongInt;                   {vars for status display}

R_Packets : LongInt;

S_Raw     : LongInt;

S_Packets : LongInt;

{$IFDEF ShowRates}

R_Counter : LongInt;

S_Counter : LongInt;

{$ENDIF}

constructor Init(AP : AbstractPortPtr);

destructor Done; virtual;

{...public methods called by terminal handlers}

procedure bpHandleENQ;

{-handle an <ENQ> from host}

procedure bpHandleESCI;

{-handle <ESC><'I'> (VT52 terminal capabilities inquiry) from host}

function bpDLESeen : Boolean; virtual;

{-called when <DLE> seen from host, starts protocol}

{...other publics}

procedure bpSetResumeProc(RP : GetResumeProc);

{-set our ResumeProc for this instance}

procedure bpSetChkAbortProc(CAP : ChkAbortProc);

{-set our ChkAbortProc for this instance}

procedure bpSendACK;

{-acknowledge receipt of a complete packet}

{...private methods}

procedure UpdateStatus(W : Word);

procedure UpdateQuoteTable(QS : QuoteArray);

procedure QuoteThis(Value : Integer);

procedure apResetProtocol; virtual;

procedure apUpdateBlockCheck(CurByte : Byte); virtual;

function CheckAbort : Boolean;

procedure SendByte(C : Char);

procedure SendQuotedByte(I : Integer);

procedure SendNAK;

procedure SendENQ;

function IncSequence(Value : Integer) : Integer;

function ReadByte : Boolean;

function ReadQuotedByte : Boolean;

procedure SendFailure(Reason : String);

function ReadPacket(LeadInSeen, FromSend : Boolean) : Boolean;

procedure SendData(BNum : Integer);

function IncSA(Value : Integer) : Integer;

function ReSync : Integer;

function GetACK : Boolean;

function SAFlush : Boolean;

function SendPacket(Size : Integer) : Boolean;

function SendTransport : Boolean;

procedure ProcessTransportParams(SendXPortInfo : Boolean);

procedure bpInitVars;

end;

BPProtoFTPPtr = ^BPProtoFTP;

BPProtoFTP =

object(BPProtocol)

constructor Init(AP : AbstractPortPtr;

DefaultResume : ResumeResultType);

destructor Done; virtual;

{...public methods}

function bpDLESeen : Boolean; virtual;

{...virtual methods}

procedure apPrepareReading; virtual;

procedure apFinishReading; virtual;

procedure apFinishWriting; virtual;

procedure apPrepareWriting; virtual;

{...private methods}

procedure SendFile;

procedure RecvFile;

end;

{$IFDEF SupportGIF}

BPProtoGIFPtr = ^BPProtoGIF;

BPProtoGIF =

object(BPProtocol)

constructor Init(AP : AbstractPortPtr);

destructor Done; virtual;

{...Public methods}

function bpDLESeen : Boolean; virtual;

function bpGetGIFDataBlock(var P;

var PSize : Word;

var IsLast : Boolean) : Boolean;

end;

{$ENDIF}

function bpStatusStr(Code : Word) : String;

procedure NoUserStatus(AP : AbstractProtocolPtr; First, Last : Boolean);

{===== Procedure/Function/Method Reference ===================================}

This section provides details on the public and virtual methods of the

various BPlus protocol objects.

{----- BPProtocol object -----------------------------------------------------}

constructor Init(AP : AbstractPortPtr);

Purpose:

Initialize BPlus protocol instance, including all shared vars from

AbstractProtocol and those BPProtocol vars that are instance-dependent.

Like other protocol objects derived from AbstractProtocol, if the constructor

fails the reason code is returned in the AsyncStatus global.

Example:

{...}

Var

OurBPlus : BPProtocol;

{...}

if NOT OurBPlus.Init(OurPortPtr) then

WriteLn('Error ',AsyncStatus,' initializing BPlus protocol object!');

{...}

destructor Done; virtual;

Purpose:

Dispose of BPProtocol instance when processing is complete.

procedure bpHandleENQ;

Purpose:

Perform protocol-dependent processing when a <ENQ> char (ASCII $05) is

received at the serial port.  This method initializes some internal vars

and sends the proper response back to the CIS host.

Example:

{...}

case RecdChar of

#5:

OurBPlus.bpHandleENQ;

else

Write(RecdChar);

{...}

See also:

bpHandleESCI, bpDLESeen

procedure bpHandleESCI;

Purpose:

Handle responding to <ESC><'I'> inquiry from CIS host.  See the detailed

description in the protocol spec document for more information on the

codes that are used to build the response string.

Example:

{...}

if (LastRecdChar = #27) and (RecdChar = 'I') then

OurBPlus.bpHandleESCI;

{...}

See also:

bpHandleENQ, bpDLESeen

function bpDLESeen : Boolean; virtual;

Purpose:

Called when a <DLE> char (ASCII $10) is received from the CIS host.  This

function calls the main protocol processing loop.  It returns True if

protocol processing completed successfully, or False if a error occurs

during protocol handshaking or if the protocol session is aborted for any

reason.  Note that in the BBProtocol object this is only an abstract method.

Example:

{...}

if (RecdChar = cDLE) then

if OurBPlus.bpDLESeen then ;

{...}

WRONG!  In the BPProtocol base object, bpDLESeen is an ABSTRACT method that

will generate a runtime error (211) if called directly.

See also:

bpHandleENQ, bpHandleESCI

procedure bpSetResumeProc(RP : GetResumeProc);

Purpose:

Assign a function to the BPProtocol object's GetResume proc pointer to tell

the protocol which type of handling should be used for resuming an aborted

download.  When a download is started and the protocol determines that a

file of the same name as the file being downloaded already exists, it can

proceed in one of several ways: it can abort in order not to risk

overwriting the existing file, it can overwrite the existing file

regardless, it can rename the new file being downloaded so as not to

conflict, or it can attempt a resume of a previous download.  No one choice

is appropriate for all occasions, so this hook is provided for an outside

function to tell the protocol how to proceed.  The function must be

compiled using the FAR model and cannot be nested within another procedure

or function.  It takes the following form:

function GetResumeFunc(BPP : BPProtocolPtr) : ResumeResultType;

Typically this function prompts the user what to do and returns his/her

choice.  See the definition of the ResumeResultType above.

Example:

{...}

OurBPlus.Init(OurPortPtr);

OurBPlus.bpSetResumeProc(GetResumeFunc);

{...}

See also:

bpSetChkAbortProc, and the type declaration of ResumeResultType above.

procedure bpSetChkAbortProc(CAP : ChkAbortProc);

Purpose:

Assign a function to determine if the user wishes to abort the protocol

session.  Typically this will be set to point to a function that returns

True of the user has requested an abort (by pressing a key, clicking the

mouse on a hotspot, etc.) or False otherwise.  Like bpSetResumeProc, the

function passed to this procedure must be compiled under the FAR model and

not be nested within any other procedure or function.  The function takes

the form:

function ChkAbortFunc : Boolean;

Example:

{...}

OurBPlus.Init(OurPortPtr);

OurBPlus.bpSetChkAbortProc(ChkAbortFunc);

{...}

See also:

bpSetResumeProc

{----- BPProtoFTP object -----------------------------------------------------}

constructor Init(AP : AbstractPortPtr; DefaultResume : ResumeResultType);

Purpose:

Initialize BPProtoFTP object.  The DefaultResume parameter provides the

default handling type for resume attempts in case no GetResumeProc has

been assigned, so totally automated systems such as "CIS navigators" can

continue processing without user intervention.

Example:

{...}

var OurBPFTP : BPProtoFTP;

{...}

OurBPFTP.Init(OurPortPtr,xfrOverWrite);

{...}

See also:

AbstractProtocol.InitCustom, BPProtocol.Init

destructor Done; virtual;

Purpose:

Dispose of BPProtoFTP instance when processing is complete.

procedure apPrepareReading; virtual;

Purpose:

Prepare for transmitting data via the protocol (usually opens the file to

be uploaded).

See also:

The other AbstractProtocol child objects' apPrepareReading methods.

procedure apFinishReading; virtual;

Purpose:

Perform any nessessary cleanup after the upload is finished (usually closes

the file uploaded).

See also:

The other AbstractProtocol child objects' apFinishReading methods.

procedure apPrepareWriting; virtual;

Purpose:

Prepare for receiving data via the protocol (usually opens the file to

be downloaded).  In this instance the method also checks if a resume

attempt is needed.

See also:

The other AbstractProtocol child objects' apPrepareWriting methods.

procedure apFinishWriting; virtual;

Purpose:

Perform any nessessary cleanup after the download is finished (usually

closes the received file).

See also:

The other AbstractProtocol child objects' apFinishWriting methods.

function bpDLESeen : Boolean; virtual;

Purpose:

Called when a <DLE> char (ASCII $10) is received from the CIS host.  This

function calls the main protocol processing loop.  It returns True if

protocol processing completed successfully, or False if a error occurs

during protocol handshaking or if the protocol session is aborted for any

reason.  In this BPProtoFTP instance, bpDLESeen either receives the "+"

packet and initializes the nessessary internal variables, or receives the

initial "TD" packet, verifies that all provided parameters are correct, and

sends or receives the file named in the "TD" packet.

Example:

{...}

if (RecdChar = cDLE) then

if OurBPFTP.bpDLESeen then ;

{...}

See also:

BPProtocol.bpDLESeen

{----- BPProtoGIF object -----------------------------------------------------}

constructor Init(AP : AbstractPortPtr);

Purpose:

Initialize BPProtoGIF object.

Example:

{...}

var OurBPGIF : BPProtoGIF;

{...}

OurBPGIF.Init(OurPortPtr);

{...}

See also:

AbstractProtocol.InitCustom, BPProtocol.Init

destructor Done; virtual;

Purpose:

Dispose of BPProtoGIF instance when processing is complete.

function bpDLESeen : Boolean; virtual;

Purpose:

Called when a <DLE> char (ASCII $10) is received from the CIS host.  This

function calls the main protocol processing loop.  It returns True if

protocol processing completed successfully, or False if a error occurs

during protocol handshaking or if the protocol session is aborted for any

reason.  In this BPProtoGIF instance, bpDLESeen receives the "+" packet and

initializes the nessessary internal variables; all other protocol

processing is handled by the bpGetGIFDataBlock and bpSendACK methods.

Example:

{...}

if (RecdChar = cDLE) then

if OurBPFTP.bpDLESeen then ;

{...}

See also:

BPProtocol.bpDLESeen, BPProtoFTP.bpDLESeen

function bpGetGIFDataBlock(var P;

var PSize : Word;

var IsLast : Boolean) : Boolean;

Purpose:

Return a block data from the protocol handler for decoding.  P is a var

parameter denoting a buffer at least large enough to hold one full packet's

worth of data.  On exit, PSize holds the number of bytes placed in P, and

the IsLast parameter is set True if the packet received was a "TC"

termination packet or False otherwise.

The function returns False if the protocol terminates (a Failure packet is

received or too many consecutive protocol errors occur), otherwise it

returns True.  Defective packets are automatically NAKed within the

protocol, but _your_ code is responsible for sending the ACK for a good

packet (see bpSendACK above).

Example:

{...}

var DecodeBuffer : Array[1..1024] of Byte;

DecodeSize : Word;

IsLastPacket : Boolean;

{...}

if NOT bpGetGIFDataBlock(DecodeBuffer,DecodeSize,IsLastPacket) then

{handle error}

else

{decode/display image data now in DecodeBuffer}

See also:

The example code in the OFLGIF.PAS file included in the package.

(* EOF *)