NITROX.EXE (c) Copyright 09/1992 - Ray Skelton - All rights reserved

NITROX Ver. 1.01

- This program is a general purpose nitrox calculator, that is used to do most

of the nitrox calculations you would need while diving with nitrox. It was

written in the 'C' language and compiled with Mocrosoft 'C' compiler

version 6.0a. The thought behind this program was to create in table form, all

the information needed to make a nitrox dive. This information can then be

printed, put in a loose leaf binder, and carried with you on a dive site. The

need to carry a calculator, remember formulas, and make all of these

calculations by hand is eliminated. This makes for a smother, safer, and easer

nitrox dive, which in turn makes the dive a more enjoyable dive. It is assumed

that you already know the terms and definitions used in this program, so we

will not discuss any terminology used in this program. You are a certified

nitrox diver aren't you??????

The purpose of this program is to encourage safer nitrox diving by providing

most of the information needed without a lot of trouble. Just as we don't have

to compute our dive tables before each dive, we now don't have to compute our

nitrox information before each dive. Simply look up the information needed in

the tables you create with this program.

This program is provided FREE of charge, and is not to be sold to anyone

by anyone. Feel free to distribute this program to qualified nitrox divers,

but keep in mind the safety concerns i have with non qualified nitrox divers

using this program. Please avoid distributing this program to non qualified

nitrox divers. This program is copyright (c) by Ray Skelton, and All Rights

are Reserved by Ray Skelton.

*********************************************************************************

PROGRAM DISCLAIMER

THIS PROGRAM IS NOT INTENDED TO BE A SUBSTITUTE FOR PROPER TRAINING IN

MIXING, AND USING ANY COMBINATION OF GAS MIXTURES. IT IS STRONGLY RECOMMENDED

THAT YOU BECOME PROPERLY TRAINED IN THE THEORETICAL AND PRACTICAL ASPECT OF

MIXED GAS DIVING. DO NOT TAKE CHANCES WITH OXYGEN OR ANY OTHER MIXTURE OF

GASES IF YOU ARE NOT TRAINED AND QUALIFIED TO DO SO. PLEASE HEED THIS WARNING!

The author of this software excepts NO LIABILITY FOR CONSEQUENTIAL  DAMAGES

for the use of, or inability to use, or the inability to properly interpret

the data presented in this program. the author has made all efforts to ensure

that the data is correct and accurate, but makes no warranty either expressed

or implied. This program is provided  'AS IS' without warranty of any kind.

THE ENTIRE RISK AS TO THE RESULTS AND PERFORMANCE OF THIS SOFTWARE IS ASSUMED

BY YOU THE USER.

Ray Skelton, retains all rights to the nitrox.exe program but does grant the

right to make unlimited, unaltered copies of the nitrox program provided there

is no charge to the users.

*********************************************************************************

- After the DISCLAIMER screen and the confirmation screen, the main menu

appears.

This main menu is listed below:

1) Basic Nitrox Calculations

2) Basic Nitrox Calculations in Table Form

3) Create Ideal O2/PO2 Table

4) Select Ideal O2 based on PO2 Input

5) Create Table of OTU'S PO2

X) Exit to Dos

Selection:

- Selection number one prompts for a projected dive depth, and percent of

oxygen mix, and returns to you the following:

Projected                 PERCENT            MAX         MAX     OTU'S/

Depth      ATA      PO2     O2     EAD      DEPTH       DEPTH    MINUTE

-----------------------------------------------------------------------

40      2.2121   0.7079    32      30       111         132      0.485

- This first selection sends the output to the screen, and does not have an

option to send output to screen or file.

- Selection number two prompts for a percent of o2 mixture, and for that

mixture  creates a table of the above information. This table starts at 10

feet and stops when the po2 exceeds 1.6. This option allows you to send the

output to a file and thus be printed. The file name created is 'NITROXxx.TBL'

where 'xx' is the percent of oxygen selected. By creating many of these tables

you can look up different information for different mixtures to aid in planing

a dive. Example of selection number two is listed below:

Projected                 PERCENT            MAX         MAX     OTU'S/

Depth      ATA      PO2     O2     EAD      DEPTH       DEPTH    MINUTE

-----------------------------------------------------------------------

40      2.2121   0.7079    32      30       111         132      0.485

45      2.3636   0.7564    32      34       111         132      0.576

50      2.5152   0.8048    32      38       111         132      0.665

......................................................................

175      6.3030   2.0170    32     146       111         132      2.498

- Selection number three creates an ideal nitrox mix for a po2 of 1.40 & 1.60.

This option also has a prompt to sent the output to the screen or a file. If

sent to a file, the file name created is 'IDEAL_O2.TBL'. An example of this

output is listed below:

Projected                      PERCENT                    PERCENT

Depth        ATA       PO2       O2               PO2       O2

----------------------------------------------------------------

40       2.2121      1.40       63       |     1.60       72

45       2.3636      1.40       59       |     1.60       68

............................................................

ect.

- Selection number four creates a output of percent of oxygen to use based on

PO2 input, and projected depth of dive. This option is good if you want to

select the po2 you want to use. This option only sends this output to

the screen, and an example output is as follows:

Projected                      PERCENT

Depth        ATA       PO2       O2

--------------------------------------

110       4.3333      1.20       28

- Selection number five creates  two files, if output to a file is chosen,

otherwise a OTU table is displayed on the screen. The first file is an otu

chart and the other is a OTU multiplier. The OTU chart is created for a PO2

from 0.05 to 1.6, and is shown below. The file name this chart is created is

is 'OTU.TBL'. An example of this OTU table is as follows:

PO2                 OTU'S / MINUTE

----------------------------------

0.50 ata                 0.000

0.60 ata                 0.265

..............................

1.60 ata                 1.916

- The second file is a multiplier of OTU'S * five minute increments. This was

created to prevent the need of a calculator at a dive site, or doing the math

by hand. When planing the dive simply cross over the OTU'S that match the PO2

and go to the multiplier chart to get the total OTU'S for the dive. An Example

is as follows.

MINUTES -   5       10      15      20      25      30      35      40      45

OTU'S   -----------------------------------------------------------------------

0.27   ||  1.33    2.65    3.98    5.30    6.63    7.95    9.28   10.60   11.93

0.37   ||  1.85    3.70    5.56    7.41    9.26   11.11   12.96   14.81   16.67

....................................................................

ect.

- It is recommended that you make a separate subdirectory to put this program

into sense it creates many file to be stored or printed. It is possible to

create fifty or more tables, and this would be very unorganized if put in a

directory with other programs. To make a sub dirrectory enter 'CD\',

'MD NITROX', 'CD\NITROX' then copy files into this subdirectory.

- As stated earlier, the thought behind this program was to create all the

information needed to make a safe nitrox dive. This means printing the

tables, putting the printed tables in a binder, and carrying all the

information with you on a dive. No formulas, calculators, or any major

calculations are needed. Everything needed can be crossed from a table.

I hope you enjoy this program, and if you have any comments or feedback

please write me at the following address.

Ray Skelton

258 Indian Branch Way

Lawrenceville, Ga. 30243

************************************************************************

*                            Edit History                              *

************************************************************************

* Ver 1.00 - 09/20/92 - Ray Skelton                                    *

*          - Initially  Written                                        *

* Ver 1.01 - 10/17/92 - Ray Skelton                                    *

*          - Converted to Microsoft C Ver. 7.0 Compiler                *

*          - Fixed bug in option 5 while creating the out mult. table  *

*            on multiple entries.                                      *

*          - Made changes in the routines that create the otu          *

*            multiplication tables to improve performance and          *

*            reuseability                                              *

************************************************************************