PROPR V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 1

DESCRIPTION

-----------

PROPR is a command-line utility which allows you to send ANY commands

to your IBM Proprinter compatible printer. PROPR has over 50 built-in

commands, and it also gives you the ability to create and store your

own, using binary, octal, decimal, hex, and text codes. Commands can

be stacked, and they can be sent to any device or file.

REQUIREMENTS

------------

PROPR will run on any PC compatible running DOS 3.0 or better, with an

IBM Proprinter compatible printer (which, while not actually required,

would make the program a bit more useful... ;^). If you wish to add

new commands, an ASCII text editor will also be needed.

DISCLAIMER

----------

PROPR is distributed on an "AS IS" basis without warranty, expressed

or implied. Considerable testing effort has been expended, but the

user is advised to check the program's suitability before relying on

it. The user assumes full risk as to the results of using this

program. In no event shall the author be liable for any incidental or

consequential damages arising from the use, or inability to use this

program.

All product names mentioned in this document are either Copyright or

Trademark of respective owners.

REGISTRATION

------------

PROPR is freely distributable and fully functional, and may be used

for a 30-day trial period. If you plan to continue using PROPR, you

are obligated to pay the modest registration fee of $10, which

entitles you to unlimited free upgrades (to be distributed through

standard Shareware channels). Upgrades obtained directly ($5 shipping)

will have an individual serial number and no registration reminders.

All registration information is contained in the PROPR.REG file. If

that file is missing, register by sending your name, address, and a

check or money order for $10 ($15 for the registered version) in US

dollars to:

Copy Con Incorporated

PO Box 23255

Baltimore MD 21203-5255

MD residents add 5% sales tax. If ordering the registered version,

please specify 3.5" or 5.25" diskette.

Thank you for supporting Shareware.

PROPR V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 2

INSTALLATION

------------

To install, copy both PROPR.EXE and PROPR.DAT into your path. To add

new commands to PROPR, you will also need an ASCII text editor to edit

PROPR.DAT (see CREATING NEW COMMANDS).

OPERATION

---------

PROPR is activated from the command line, using the format

PROPR [/device] command[-] [command[-]]...

By default, PROPR sends commands to the LPT1 device; to specify

another, use the /device switch, specifying a valid device or

filename; for example,

PROPR /lpt2 reset

sends the reset command to the device connected to LPT2.

PROPR has over 50 built-in commands, and they can be sent individually

or combined with other commands. A command directly followed by a -

(minus sign) disables the command, where applicable; for example,

PROPR italic underline-

enables italics and disables underlining.

Commands do not need to be typed out in their entirety; only as much

of the command that distinguishes it from other commands is needed;

for example, when sending subscript and superscript commands,

"PROPR su" would send subscript because it alphabetically precedes

superscript; "PROPR sup" would send superscript. When using PROPR

inside batch files, it is a good idea to use the full command, as

future versions of PROPR may have more built-in commands that may be

similar to existing commands.

BUILT-IN COMMANDS

-----------------

Following are the commands that are built into PROPR, along with their

descriptions. (Some commands are not supported on some printers.)

Command           Description

1/6               Sets paper feed amount to 1/6 inch

1/8[-]            Sets paper feed amount to 1/8 inch

7/72[-]           Sets paper feed amount to 7/72 inch

AutoLF[-]         Automatically send a LF after a CR

BackSpace         Prints buffer and backspaces one space

Bell              Sounds buzzer

BoldPS[-]         Selects Bold PS font

Cancel            Clears all data in the buffer

PROPR V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 3

BUILT-IN COMMANDS (continued)

-----------------------------

Command           Description

Char              Print continuously from All Character Chart

Char1             Print next character from All Character Chart

Compressed[-]     Sets printing to 17 CPI

Courier[-]        Selects Courier font

CR                Prints buffer and starts next line at left margin

DeSelect[-]       Disables printer from receiving data

Double[-]         Sets double printing

Draft[-]          Selects Draft font

Elite[-]          Sets printing to 12 CPI

Emphasized[-]     Sets printing to twice the original dot density

Escape            Sends the escape ($1B) code

FF                Prints buffer and feeds paper to next TOF position

Flush             Prints all data in buffer

HalfLF            Prints buffer and feeds paper 1/12 inch

IBM1              Selects IBM 1 character set

IBM2              Selects IBM 2 character set

Italic[-]         Selects Italic font

LF                Prints buffer and executes a single line feed

Multilingual      Selects Multilingual character set

NLQ[-]            Selects NLQ font

Null              Sends the null ($00) code

Overline[-]       Sets continuous overlining of characters

PaperOut[-]       Enables paper out detector

Pica[-]           Sets printing to 10 CPI

Prestige[-]       Selects Prestige font

Proportional[-]   Sets proportional spacing between characters

Reset             Resets printer to initial state

Roman[-]          Selects Roman font

SansSerif[-]      Selects Sans Serif font

Script[-]         Selects Script font

Select[-]         Enables printer to receive data

Single[-]         Sets single direction printing mode

SubScript[-]      Selects subscript font

SuperScript[-]    Selects superscript font

Tab               Sends the horizontal tab ($09) code

TabClear          Sets all tabs to initial settings

Tall[-]           Sets double height character printing

TOF               Sets top of form

Underline[-]      Sets continuous underlining of characters

USA               Selects USA character set

VTab              Sends the vertical tab ($11) code

Wide[-]           Sets double width character printing

Wide1[-]          Sets double width character printing for one line

PROPR V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 4

CREATING NEW COMMANDS

---------------------

If you want to send a code that is not already built-in, you can

create your own using binary, octal, decimal, hex, and text codes.

Binary codes must be preceded with the % symbol, octal with the &

symbol, + for decimal, $ for hex, and ' for text; for example,

PROPR $1b 'P +1

sends 1B in hex, 'P' in text, and 1 in ASCII decimal. To send an

entire text string, you still use the ' symbol, but you must also

enclose the entire string in quotes; for example,

PROPR bold "'one two three" bold-

prints "one two three" (without the quotes) in bold.

Strings of codes can be assigned a command name and stored in a

separate data file called PROPR.DAT, which must be kept in the same

directory as PROPR.EXE. PROPR.DAT will be searched first in case you

wish to redefine the internal commands. The format of each line of the

data file is as follows:

command_name code [code]... [ - code [code]... ]

The command name must be left-justified (no left margin), and it

cannot contain any spaces. Following the name are the codes that

enable the command. If the command can be disabled, the enabling codes

are followed by a minus sign and the codes that disable the command.

The name, codes, and minus sign must be separated by spaces and/or

tabs. There is no limit to the number of commands, but there can only

be one command per line.

For example, let's create a new command called SkipPerf. The codes to

skip six lines at the bottom of each page are $1b, 'N, and +6. Using

an ASCII text editor, create PROPR.DAT with the first line reading:

SkipPerf $1b 'N +6

Now, if you type "PROPR skip", $1b, 'N, and +6 will be sent. What if

you want to disable perforation skip? First, edit PROPR.DAT to read:

SkipPerf $1b 'N +6 - $1b 'O

Now, if you type "PROPR skip-", $1b and 'O will be sent, which are the

codes to disable perforation skip.

PROPR V1.11 Copyright (c) 1992-1993 Copy Con Incorporated       Page 5

EXAMPLES

--------

PROPR

will show a help screen and a list of built-in commands.

PROPR ff

sends a formfeed to the printer attached to LPT1.

PROPR /lpt2 ff

sends a formfeed to the printer attached to LPT2.

PROPR bold under- italic-

enables bold text, and disables underlining and italics.

PROPR esc 'W +1

sends the escape code, the letter W, and ASCII character 1.

PROPR "'line one" cr lf wide "'line two" wide- cr lf

prints two lines, the second one being of double width.

PROPR /hi.txt tall wide bold under "'hello there" cr lf

sends "hello there" in tall, wide, bold, underlined characters, to

the file HI.TXT, which can be printed by typing "copy hi.txt LPTx",

where x is the number of the printer port to be used.

SUPPORT

-------

If you have questions, comments, suggestions, bug reports, et cetera,

feel free to contact me by one of the following methods:

1: By modem at The Programmer's Corner (301-596-7692 or 410-995-6873).

Send a message to RD Thomas in the General conference, and check

back in a couple of days for a reply.

2: By modem in the RIME Shareware conference (RD Thomas ->1520). If

you do not recieve a reply within a week, it is because there are

still some kinks to be worked out at this node; try contacting me

via methods 1 or 3.

3: By US Mail at the address given on the first page. If you would

like a reply, please include a self-addressed, stamped envelope.

Looking forward to hearing from you.

This document can be printed by typing "copy propr.doc LPTx" at the

DOS prompt, where x is the number of the printer port to be used.