Doc for BENCH12.SCR 12/21/92

How to run this benchmark.---------------------------------------------------

First, completely configure AutoCAD for the driver environment

to be tested.  Use either the driver's configuration program

and/or the AutoCAD video configuration to set this up.  Ensure

that 32-bit display space operation is enabled in the driver, if

it has this option.

NOTE:  This benchmark will not work with any driver that only

supports 16-bit display space, as these drivers are not true ADI

4.2 drivers and do not provide you, as an end-user, with all the

features of AutoCAD R12.

After the driver has been configured, return to DOS.

All of the Autodesk supplied drivers will perform this

benchmark.  Additionally, the ACAD.DWG prototype drawing used

for this trial should be the default drawing shipped by

Autodesk.

Starting from the directory where ACAD.EXE is located, enter

AutoCAD.  You might want to use the STATUS command to note both

the amount of available memory and the amount of memory used by

the driver without a drawing loaded.

Use the SCRIPT command to invoke BENCH12.SCR.  This benchmark

will take anywhere from two minutes up to an hour, depending on

your system speed, amount of memory and the driver used. On a

486/66 DX/2 with 16Mb of memory, Autodesk's standard VGA driver

takes just under five minutes, and requires about 13.5Mb of

memory.

Remember, the amount of memory you have will determine how much

disk access is required during this benchmark. Disk access speed

is tremendously slower than RAM access speed, so more RAM equals

more overall speed.

The files used.--------------------------------------------------

This script file tests driver performance using 32-bit display

list with Release 12.  It must be run from the ACAD directory

(or the main directory where ACAD.EXE is located).  To find the

following files (SAMPLE\PSSHADOW.DWG, SAMPLE\HOUSEPLN.DWG,

SUPPORT\CHROMA.DWG), this benchmark assumes you have followed

the standard AutoCAD installation.

You must also have read/write access to the directory where this

script was started from.  The script file creates a temporary

file, $$$$TEMP.EPS, for the postscript input/output functions.

NOTE: The only place any of the files are referenced is at the

beginning of the script file. This has been done so that you may

easily change path names to enable this benchmark to function on

a network or customized system.

You should have at least 14Mb of free memory for AutoCAD to use

when running this benchmark.  The Autodesk STANDARD VGA 640x480

16 color driver uses about 13.5Mb to run this script. Other

drivers, especially display list, will require more.

The amount of extra memory required by other display drivers

will make a difference in the performance of this benchmark. As

mentioned before, the major differences in speed are due to the

radically different access times encountered while retrieving

information from either the hard disk or the RAM.

NOTE: Retrieving information from the RAM will be faster than

retrieving information from the hard disk.

What this benchmark measures.----------------------------------

This benchmark was developed at request of the many users, users

groups, graphics board manufacturers, and reviewers who needed a

benchmark that demonstrated actual AutoCAD performance as

affected by display list drivers.

Traditional driver benchmarks only show the benefits of faster

zoom and pan operations, but they ignore the side-effects that a

display list driver may bring to the system, even during the

most normal editing operations.

This new benchmark is meant to demonstrate how a display list

driver can affect an AutoCAD system in a working environment --

it is not simply a test structured to show off the few benefits

available from many display list drivers. If overall AutoCAD

performance is not enhanced, then a display list driver is not

providing a benefit for your system.

The main objective of this benchmark is to show the overall,

performance gains or losses when using a display list driver.

Many of the functions performed in this benchmark typically

cause display list drivers to increase their memory usage

dramatically.

With this in mind, the BENCH12 benchmark may expose poor

implementations of display list editing and memory management.

Additionally, the display list operations typically enhanced by

benchmarks (i.e. zooming and panning) are also performed to show

traditional display list gains.

The second objective of this benchmark was to avoid generating a

regen.  Autodesk is promoting the 32-bit display space built

into AutoCAD R12 and its dramatic reduction of regens. This

script will not generate a regen (after the initial Zoom

Extents) on any driver that is fully ADI 4.2 compatible and

supports 32-bit display space.

The third objective of this benchmark was to test the driver

with 32-bit display list objects. This was done to ascertain if

the driver being tested would perform correctly while scaling

and clipping. Without proper handling of these objects, many

display list drivers exhibit erratic behavior up to and

including random system crashes.

About the Script.----------------------------------------------

This script file first inserts PSSHADOW.DWG, uses PSOUT to

create a temporary .EPS file, and imports it with PSIN to get

the shaded image. This portion is not timed since it is

virtually 100 percent CPU and hard disk related.

The drawing is generated by moving the shaded image, importing

the HOUSEPLN.DWG drawing, importing the HOUSEPLN.DWG drawing at

a reduced scale, importing the CHROMA drawing, and importing the

CHROMA drawing at a reduced scale.  The total time for this

section is reported at the end.

A ZOOM E(xtents) is performed (this causes a regen) and the time

is recorded.

All inserted blocks are EXPLODEd and a donut is created in the

corner.  This time is also reported at the end.

A series of ZOOM commands are used to test the classic display

list benefits with the time being reported at the end.

Accelerated boards should benefit from this section and should

have lower times than non-accelerated boards.  About 28 bits of

display list space (out of the 31 available) are used to prevent

regens at very high resolutions and other aspect ratio

considerations.  The donut created in the Zoom E(xtents) section

is used to check polygon clipping.

A number of edits are performed (i.e. ROTATE, UNDO, PEDIT,

PLINE, CIRCLE, DONUT, MOVE and ERASE).  This section contains

redraws after most edits, simulating a typical user cleaning up

the pick blips.  This time is reported at the end.

After the above test sequence, a small drawing is created

tallying the results.

A STATUS command is also executed to show the amount of virtual

memory used by AutoCAD and the driver. This is important because

some drivers require an excessive amount of RAM during editing

sessions. This excessive use of RAM means more hard disk

accessing and slower overall system performance.

What to look for.-----------------------------------------------

If a regen occurs, most likely the driver does not implement the

full 32-bit display space and is an ADI 4.1 driver (Release 11)

pretending to be an ADI 4.2 driver.  These drivers will most

likely stop in the middle of the script, because AutoCAD was

unable to pick an object due to the reduced display space forced

by the driver.

During this benchmark, pay attention to the redraws as they may

be showing previous edits.  These previous edits may appear as

temporary, or "flashing" ghosts.  If so, then the driver is not

performing real-time display list editing.

When objects are moved or erased, you may see "left-over-bits"

or pixels.  A driver that does not properly match highlighted

lines with normal lines will leave these annoying artifacts.

Additionally, if the "Virtual Memory Allocated to Program" (use

STATUS command) grows more then a couple of megabytes from when

the drawing was created (post-ZOOM E command), then the driver

is not releasing or re-using erased display list memory.  Some

drivers may cause the system to access the hard disk excessively

due to the amount of memory required.

If your system hangs, locks, reboots, gets an error message (i.e

Divide By Zero, Abnormal Program Termination, Fatal Error,

etc...) then it is the driver that has caused the problem.

First, run the benchmark with the Autodesk supplied VGA driver

to get a feel for what the display should look like during

execution. Then, run the benchmark with each of the drivers you

are testing to see if they are operating correctly.

The future.---------------------------------------------------

Some of the areas that this script does not address are:

Multiple Viewports, Paper Space, AVE Render, Grid, Command Area

text speed and dialog-box speed. Overall, however, this

benchmark has proven to be a reasonable attempt to reproduce a

working environment that is using the enhanced features provided

by AutoCAD R12.

If you have any questions or comments concerning this benchmark,

please FAX them to us at (512) 250-5811.

-- end of BENCH12.DOC --