Publication Part Number W0001-520

Sheet  1 of 47

Title:  IBM PC, MS DOS

CCPADI

Version 3.0

Publication Part Number W0001-520

Sheet  2 of 47

CONTENTS

Heading                                                  Page

General description...........................................4

Installation..................................................6

Operations....................................................7

Main Screen...................................................7

Info..........................................................8

Communications................................................9

PORT.....................................................9

Network Plotting........................................10

Baud Rate...............................................10

Parity..................................................10

Flow....................................................10

Sync,eob,cksum...............................................11

Nbr of syncs............................................11

Sync code value.........................................11

EOB code value..........................................12

Checksumming............................................12

Plotter......................................................13

MISC options.................................................14

Scale...................................................14

Copies..................................................14

Colors..................................................15

Radix...................................................15

CDCL....................................................15

Loading CCPADI into memory...................................15

OK  .........................................................16

CANCEL.......................................................16

Configuring AutoCAD 11.......................................17

Configuring AutoCAD 10.......................................22

Publication Part Number W0001-520

Sheet  3 of 47

Configuring CalComp Plotters.................................26

1023 Artisan............................................27

1025 Artisan............................................27

1026 Artisan............................................27

1043....................................................30

1044....................................................30

1073,1075,1077..........................................31

2024....................................................32

2026....................................................32

52224...................................................36

52424...................................................36

52236...................................................36

52436...................................................36

67436...................................................36

68436...................................................36

68444...................................................36

57424...................................................38

57436...................................................38

57444...................................................38

58424...................................................38

58436...................................................38

58444...................................................38

5912....................................................40

5913....................................................40

5902A...................................................40

6603 VRC................................................42

6613 VRC................................................42

List of materials............................................46

Systems requirements.........................................46

Default Pen Definitions......................................47

Pen Width Table..............................................47

Publication Part Number W0001-520

Sheet  4 of 47

GENERAL DESCRIPTION

This CalComp ADI (Autodesk  Device Interface) Driver is a memory

resident routine connected to interrupt 78H of AutoCAD that generates

CalComp 907/PCI commands for CalComp plotters.

The CCPADI 3.0 has several advantages over other CalComp plotter

drivers.

- Supports all current CalComp plotters and Vector printers.

- Outputs Serially at 8 different baud rates.

- Can use 7,e,1 or 8,n,1 parity.

- Can use XonXoff, Hardware, or CalComp AckNak flow control

protocols.

- Outputs Centronics Parallel by bypassing the bios which will allow

for at least a 2 to 1 increase in through put.

- Outputs to a user named plot file.

- Can download pen definitions with the plot data to relieve user

from pen mapping on the plotter.

- Can download a copy command to the plotter, sending the plot only

one time and producing up to 63 copies.

- User can tailor CalComp specific data parameters such as sync

codes, EOB codes, and checksumming to match an existing plotter

setup.

- Smaller file size (typically about one-half)

- CDCL setup information can be included with the plot to configure

the plotter.  Calcomp Device Control Language is a way to

re-configure the plotter to match the incoming plot file.

Publication Part Number W0001-520

Sheet  5 of 47

Autocad ADI driver Plot Size Limit.

When the user selects the step size and plot size using Autocad's

"Configure Plotter" function,  the total X or Y steps in a plot

(plot size multiplied by the stepsize) cannot exceed 65535.

For plotters that use 2032 stepsize it would limit them to a 30 X

30" plotting area.  CCPADI 3.0 solves this problem by allowing

the user to set the stepsize in AutoCAD to 1016 and then doubling

the plot data in CCPADI 3.0.

Publication Part Number W0001-520

Sheet  6 of 47

INSTALLATION

Step 1. Check the CalComp shipped diskette.

Insert the CalComp shipped diskette in Drive A or B.    Type

DIR A: or B:.  Check your diskette and compare the files with the

Section "List of Materials".  Also read  "System Requirements"

before you begin.

Step 2. Installing files to hard disk.

An install program is provided for installing CalComp ADI drivers

to your hard-disk.  The program will allow the user to select the

drive letter on which CalComp ADI drivers will be installed.

INSTALL will create a sub-directory called \CALCOMP and copy all

needed files.

To execute INSTALL:

Make the CalComp supplied floppy the current drive by typing:

A:  or  B:  and press enter

INSTALL     and press enter

The install program will display an introduction and ask the user

to press any key to continue.  Press a key.

Next the install program will ask which ADI driver to install.

There are two drivers available.  CCP386 is a protected mode

driver to use with AutoCAD 386 Version 10 or 11.  CCPADI is a

real mode driver to use with any version of AutoCAD.  Use the up

or down arrow keys to highlight the required driver and press

return.

Next the install program will ask the user to select the hard

disk for installation.  If multiply drives are available a list

of the available drive names will be displayed.  Use the up or

down arrow keys to highlight the required hard disk and then

press return.

INSTALL will check for the existence of a sub-directory called

\CALCOMP on the selected hard drive.  If the sub-directory exists

a warning message will be displayed and asks if the user wants to

continue.

INSTALL will copy all programs and supporting files to the hard

drive selected and in a sub-directory called \CALCOMP.

INSTALLATION COMPLETE

Publication Part Number W0001-520

Sheet  7 of 47

CCPADI OPERATIONS

CCPADI must be configured before the driver can be loaded into memory.

NOTE:

CCPCFG is a Graphical User Interface (GUI) program that can be

operated with or without a mouse.  With a mouse you select the

options to execute and 'Click' the left button.  Without a

mouse use the keyboard and press the letter Capitalized within

the Menu or Button required.

To start CCPCFG make the CALCOMP subdirectory the current directory by

typing from the DOS prompt:

cd /CALCOMP     and press return

Then:

CCPCFG          and press return

The main graphical screen will be displayed looking something like

this:

----------------------------------------------------------------------

| date                      TITLE                               time |

----------------------------------------------------------------------

| Info  Communications  Sync,eob,cksum  Plotter  Misc  oK  canceL    |

----------------------------------------------------------------------

|                                                                    |

|                                                                    |

|                                                                    |

|                                                                    |

|                                                                    |

|                                                                    |

|                                                                    |

|                                                                    |

----------------------------------------------------------------------

Publication Part Number W0001-520

Sheet  8 of 47

The second line down is a Horizontal Menu.  There are 7 items listed

on the menu:

Info            Displays information about the program

Communications  Used to configure the communications options

Sync,eob,cksum  Used to set the CalComp specific options

Plotter         Used to define target plotter

Misc            Used to set various miscellaneous options

oK              Used save the options in options file.

canceL          Used to abort the program

INFO

To see information about CCPCFG select the Info menu item with the

mouse and Click the left button or type 'I' on the keyboard.  A pop-up

will appear.  To clear the Information pop-up, click the mouse again

or press a key.

Publication Part Number W0001-520

Sheet  9 of 47

COMMUNICATIONS

To configure communications, select the Communications menu item with

the mouse and click the left button or type 'C' on the keyboard.

Another window will appear.

|--------------------------------------------------------------------|

|                     COMMUNICATIONS OPTIONS                         |

|                                                                    |

|    PORT             BAUD RATE                       PARITY         |

|  --------            -----                          ---------      |

|  | Com1 |            | 1 | 300                      | 7,e,1 |      |

|  --------            -----                          ---------      |

|  | coM2 |            | 2 | 600                      | 8,n,1 |      |

|  --------            -----                          ---------      |

|  | Lpt1 |            | 3 | 1200                                    |

|  --------            -----                          FLOW           |

|  | lPt2 |            | 4 | 2400                    ------------    |

|  --------            -----                         | Xonxoff  |    |

|  | File |            | 5 | 4800                    ------------    |

|  --------            -----                         | Hardware |    |

|                      | 6 | 9600                    ------------    |

|                      -----                         | Acknak   |    |

|                      | 7 | 19200                   ------------    |

|                      -----                                         |

|                      | 8 | 38400                                   |

|                      -----                         --------------- |

|                                                    | oK | canceL | |

|                                                    --------------- |

|--------------------------------------------------------------------|

PORT

This column of options tells CCPADI 3.0 where to send the plot data.

The ports labeled, Com1, Com2, Lpt1, and Lpt2 are the physical ports

attached to the PC operating this driver and AutoCAD.  These settings

will not allow the data to be re-directed.  These ports are controlled

by CCPADI 3.0 directly, and the output does not go through the BIOS.

Select the output port or file with the mouse and click the left

button or for a keyboard type:

'C'     for Com1

'M'     for Com2

'L'     for Lpt1

'P'     for lpt2

'F'     for File

Publication Part Number W0001-520

Sheet 10 of 47

NETWORK PLOTTING

If you wish to re-direct the output from a LPT or COM port to a

network,  select 'File' and when CCPADI 3.0 ask for the output file

name,  type in the name of the BIOS port (COM1, COM2, LPT1, or LPT2).

BAUD RATE

Baud rate is the speed at which CCPADI 3.0 will transmit data to the

plotter.  This options is invalid if LPT1 or LPT2 or File have been

selected in the PORT option.  Select the required baud rate with the

mouse and click the left button or for keyboard operations press:

1     for 300 baud

2     for 600 baud

3     for 1200 baud

4     for 2400 baud

5     for 4800 baud

6     for 9600 baud

7     for 19200 baud

8     for 38400 baud

PARITY

This option is invalid if LPT1 or LPT2 has been selected in the PORT

option.  Another phrase for parity is character framing.  I will not

give you the long definition of parity but instead will tell you that

the two most common character framing options are available.  The

standard character framing used for AutoCAD is 7,e,1.  Select the

parity you require with the mouse and click the left button or press

'E' for 7,e,1 or 'N' for 8,n,1

FLOW

This option is invalid if LPT1 or LPT2 has been selected in the PORT

option.  Flow is the method of controlling the flow if data between

the computer and the plotter.  CalComp plotters will function 1 of

three ways (Xon-Xoff, Hardware, or Acknak).  Select the flow control

you require with the mouse and click the left button or for keyboard

operations press:

'X'      for Xonxoff

'H'      for Hardware

'A'      for Acknak

To Save the options, select the oK button with the mouse and click

the left button or press 'K' on the keyboard.  To quit and revert back

to the last saved settings, select the canceL button with the mouse and

click the left mouse or press 'L' on the keyboard.

Publication Part Number W0001-520

Sheet 11 of 47

SYNC,EOB,CKSUM

CalComp online 907/PCI data is record orientated and each record

begins with one or two 'SYNC' codes.  This sync code can be any ASCII

character with a decimal value between 2 and 127 inclusive.  Each

record ends with an 'EOB/EOM' code.  This EOB code can be any ASCII

control character with a decimal value between 2 and 31 inclusive.  A

special character can be inserted in the data directly in front of the

EOB/EOM code called the Checksumming character.  This character is

used to check for record validity.  Checking record validity will do

you no good using any of the Flow Control methods with the exception

of ACKNAK.  If a transmission error occurs, Xonxoff or Hardware flow

control methods allow for no error recovery.

The choice of flow control is left to your discretion, but the options

are available to assist you in matching other software packages that

do not give you these options.

NBR OF SYNCS

This option will toggle between 1 and 2 every time is selected.

Select Nbr or syncs with the mouse and click the left button or press

'N' on the keyboard.

SYNC CODE VALUE

This option, when selected,  will display a pop-up used to change this

value.

------------------------------

|  Enter Sync Code Value     |

| ------ /\                  |

| |  2 |                     |

| ------ \/                  |

|                            |

| ------------               |

| | ok | esc |               |

| ------------               |

| ---------------------------|

The operations of this pop-up vary depending if you are using a mouse

or keyboard.

If using a mouse select the up or down arrow to increase or decrease

the counter every time you click the left button.  When the correct

number of copies is displayed, select the 'ok' button and click the

left button.  If you wish to quit without making any changes to the

original value, select the 'esc' button and click the left button.

Publication Part Number W0001-520

Sheet 12 of 47

If using the keyboard, you can use the up or down arrow keys to

increase or decrease the value every time you press a key.  Another

way is to simply type in the number of copies using the number keys on

the keyboard.  The left or right arrow keys will position the cursor

to where you can insert or delete an incorrect number.  The backspace

key will work as well.  When the correct number of copies is displayed

press return.  If you wish to quit without making any changes to the

original value,  DO NOT PRESS the escape key.  The ESCAPE key will

terminate the program.  Press return and re-select the Sync,eob,cksum

function by typing 'S' on the keyboard and change as needed.

EOB CODE VALUE

The operations of this option is the save as with SYNC CODE VALUE.

CHECKSUMMING

This option will toggle between enabled and disabled every time is

selected.  Select Checksumming with the mouse and click the left

button or press 'C' on the keyboard.

To Save the options, select the oK button with the mouse and click

the left button or press 'K' on the keyboard.  To Quit and revert back

to the last saved settings, select the canceL button with the mouse and

click the left mouse or press 'L' on the keyboard.

Publication Part Number W0001-520

Sheet 13 of 47

PLOTTER

This menu item will display a window allowing the user to select the

target plotter.

----------------------------------------------------------------------

|                        PLOTTER OPTIONS                             |

|                                                                    |

|-----                 -----                         -----           |

|| A | 1023            | H | 2036                    | O | 5912      |

|-----                 -----                         -----           |

|| B | 1025            | I | 52224/52424/57424       | P | 5913      |

|-----                 -----                         -----           |

|| C | 1026            | J | 52236/52436/57436/67436 | R | 6603 VRC  |

|-----                 -----                         -----           |

|| D | 1043            | K | 57444                   | T | 6613 VRC  |

|-----                 -----                         -----           |

|| E | 1044            | L | 58424                   | U | Plotmaster|

|-----                 -----                         -----           |

|| F | 1073/1075/1077  | M | 58436/68436                             |

|-----                 -----                                         |

|| G | 2024            | N | 58444                                   |

|-----                 -----                         --------------- |

|                                                    | oK | canceL | |

|                                                    --------------- |

----------------------------------------------------------------------

Select your plotter from the list with the mouse and click the left

button, or for keyboard operations press:

'A'       for the 1023

'B'       for the 1025

'C'       for the 1026

'D'       for the 1043

'E'       for the 1044

'F'       for the 1073 or 1075 or 1077

'G'       for the 2024 Pacesetter

'H'       for the 2036 Pacesetter

'I'       for the 52224 or 52424 or 57424

'J'       for the 52236 or 52436 or 57436 or 67436

'K'       for the 57444

'L'       for the 58424

'M'       for the 58436 or 68436

'N'       for the 58444

'O'       for the 5912 Colorview

'P'       for the 5913 Colorview

'R'       for the 6603 VRC

'T'       for the 6613 VRC

'U'       for the 5902A Plotmaster

Publication Part Number W0001-520

Sheet 14 of 47

To Save the options, select the oK button with the mouse and click

the left button or press 'K' on the keyboard.  To Quit and revert back

to the last saved settings, select the canceL button with the mouse and

click the left mouse or press 'L' on the keyboard.

MISC OPTIONS

These options are extra, used to tailor CCPADI 3.0 to the

configuration you are using:

----------------------------------------------------------------------

|                            MISC OPTIONS                            |

|                                                                    |

|   SCALE        COPIES         COLORS          RADIX        CDCL    |

|   -----         -----         -----          -----       -----     |

|   | A | 1.0     | C | NO      | E | NO       | G | 95    | I | NO  |

|   -----         -----         -----          -----       -----     |

|   | B | 2.0     | D | YES     | F | YES      | H | 59    | J | YES |

|   -----         -----         -----          -----       -----     |

|                                                                    |

|                                                    --------------- |

|                                                    | oK | canceL | |

|                                                    --------------- |

----------------------------------------------------------------------

SCALE

This options is used to double the data sent to CCPADI 3.0.  This is a

way to get around the AutoCAD ADI driver Plot Size Limit.  As we

discussed in the GENERAL DESCRIPTIONS use this option to double the

1016 stepsize configured in AutoCAD to 2032 stepsize for the plotter.

Select the scale factor required with the mouse and click the left

button or press 'A' for a scale factor of 1 or 'B' for a scale factor

or 2.

COPIES

CCPADI 3.0 has the capability to download the CCGL copies command to a

compatible plotter.  This option will tell CCPADI 3.0 to prompt the

user at plot time for the number of copies.  If your plotter is not

capable of accepting the copies command, these buttons will be grayed

out and inoperable.  Select the appropriate answer with the mouse and

click the left button or press 'C' for NO or 'D' for YES.

Publication Part Number W0001-520

Sheet 15 of 47

COLORS

CCPADI 3.0 has the capability to download the CCGL pen definitions

command to a compatible plotter.  If selected, CCPADI 3.0 will down

pen color and width commands to the plotter or to a file at the start

of every plot.  The colors and widths are listed in section titled

DEFAULT PEN DEFINITIONS.  Select the appropriate answer with the mouse

and click the left button or press 'E' for NO or 'F' for YES.  If your

plotter is not capable of accepting pen definition commands, these

buttons will be grayed out and inoperable.

RADIX

This options will modify the plot data in order for the created plot

files to be upload to an EBCDIC computer for plotting on an attached

plotter.  The translation of ASCII to EBCDIC can vary between

computers and lowering the RADIX to 59 solves this problem.  If you

will not be uploading the plot files, always leave this option at 95.

Select the appropriate answer with the mouse and click the left button

or press 'G' for 95 or 'H' for 59.

CDCL

CCPADI 3.0 has the capability to download the CalComp Device Control

Language (CDCL) to a compatible plotter in order to configure the

plotter for the upcoming plot data.  Select the appropriate answer

with the mouse and click the left button or press 'I' for NO or 'J'

for YES.  If your plotter is not capable of accepting CDCL commands,

these buttons will be grayed out and inoperable.

To Save the options, select the oK button with the mouse and click

the left button or press 'K' on the keyboard.  To Quit and revert back

to the last saved settings, select the canceL button with the mouse and

click the left mouse or press 'L' on the keyboard.

LOADING CCPADI INTO MEMORY

Since CCPADI is a memory resident, it must be memory resident before

AutoCAD is executed.  After execution of CCPCFG to configure CCPADI,

the driver can be loaded into memory.  Make the CALCOMP sub-directory

the current directory by typing at the DOS prompt:

cd \CALCOMP    then press enter

CCPADI         then press enter

CCPADI 3.0 will display a banner message stating it is loaded.

Publication Part Number W0001-520

Sheet 16 of 47

OK

In order to SAVE all the options configured with CCPCFG, you must exit

the program by selecting the 'oK' menu item with the mouse and

clicking the left button or pressing 'K' on the keyboard.

OK

If you wish to exit the program and revert all options back to the

last saved settings, select the canceL menu item with the mouse and

click the left button or press 'L' on the keyboard.  A prompt will

appear asking if you really want to Quit.  Select Yes or No.

Publication Part Number W0001-520

Sheet 17 of 47

CONFIGURING AUTOCAD RELEASE 11

Start up Autocad and answer each question prompted by AutoCAD as

follows:

NOTE:  <cr> is the same as <return>

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Main Menu

0.  Exit AutoCAD

1.  Begin a NEW drawing

2.  Edit an EXISTING drawing

3.  Plot a drawing

4.  Printer Plot a drawing

5.  Configure AutoCAD

6.  File Utilities

7.  Compile shape/font description file

8.  Convert old drawing file

9.  Recover damaged drawing

Enter selection: 5 <cr>

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Current AutoCAD configuration

Video Display:           IBM Enhanced Graphics Adapter

Digitizer:               Microsoft Serial or Bus Mouse

Plotter:                 Autodesk Device Interface

Printer plotter:         None

Press RETURN to continue:  <cr>

Publication Part Number W0001-520

Sheet 18 of 47

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 5 <cr>

Your current plotter is Autodesk Device Interface

Do you want to select a different one? <N>

If your current plotter is Autocad Device Interface plotter then skip

over available plotter/driver selection.

Y <cr>

Publication Part Number W0001-520

Sheet 19 of 47

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Available plotters:

1.  None

2.  ADI P386 plotter

3.  Alpha Merics (obsolete)

4.  Autodesk Device Interface

5.  CalComp 906/907/PCI

6.  Canon Laser Printer

7.  Hewlett-Packard

8.  Houston Instrument DMP Series

9.  IBM 7300 Series

10.  PostScript device

11.  UNIX Plot device

Select device number of ? to repeat list <1>: 4 <cr>

Select output format:

0.  ASCII file

1.  Binary file

2.  AutoCAD DXB file

3.  Installed ADI driver

Output format, 0 to 3 <0>: 3 cr

Hexadecimal interrupt code (INT 0XXh) <78>: cr

Does your plotter have multiple pens? <N> Y cr

How many pens does it have, 2 to 127 <2>: 127 cr

Does the plotter have multiple dashed line styles? <N> cr

Is the plotter variable speed? <N> cr

Next you will specify the maximum plot size the plotter

can make, and the step size of the plotter.  You may

specify these either in millimeters or inches.  The

next question selects the units.

Specify plot size in millimeters? <N> cr

Maximum horizontal (X) plot size in inches <11.00>: 50 cr

Plotter steps per inch in the horizontal (X) direction <1000>: 1016 cr

Maximum vertical (Y) plot size in inches <8.5> : 36 cr

Plotter steps per inch in the vertical (Y) direction <1000>: 1016 cr

Publication Part Number W0001-520

Sheet 20 of 47

If you have previously measured the lengths of a horizontal

and a vertical line that were plotted to specific scale,

you may use these measurements to calibrate your plotter.

Would you like to calibrate your plotter? <N> N <cr>

Size Units (Inches or Millimeters) <I> I or M <cr>

Plot origin in Inches <0.00,0.00>: 0,0 <cr>

Standard values for plotting size

Size    Width   Height

A       10.50     8.00

B       16.00    10.00

C       21.00    16.00

D       33.00    21.00

E       43.00    33.00

MAX     50.00    36.00

Enter the Size of Width,Height (in Inches) <MAX>: E <cr>

Rotate plot 0/90/180/270 <0>: 0 <cr>

Pen width in Inches <0.010>: <cr>

Adjust area fill boundaries for pen width? <N> <cr>

Remove Hidden lines? <N> <cr>

Specify scale by entering:

Plotted Inches=Drawing Units or Fit or ? <F>: <cr>

Publication Part Number W0001-520

Sheet 21 of 47

A U T O C A D (R)

Copyright (c) 1982-91   Autodesk, Inc.  All Rights Reserved

Release R11 c2 (4/5/91) 386 DOS Extender

Serial Number:  xxx-xxxxxxxx

Licensed to:    You the customer

Obtained from:  Your ACAD dealer

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 0 <cr>

If you answer N to the following question, all configuration

changed you have just make will be discarded.

Keep configuration changes? <Y> Y<cr>

FINISHED  The CalComp CCPADI driver is now configured.

Publication Part Number W0001-520

Sheet 22 of 47

CONFIGURING AUTOCAD RELEASE 10

Start up Autocad and answer each question prompted by AutoCAD as

follows:

NOTE:  <cr> is the same as <return>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Main Menu

0.  Exit AutoCAD

1.  Begin a NEW drawing

2.  Edit an EXISTING drawing

3.  Plot a drawing

4.  Printer Plot a drawing

5.  Configure AutoCAD

6.  File Utilities

7.  Compile shape/font description file

8.  Convert old drawing file

Enter selection: 5 <cr>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Configure AutoCAD.

Current AutoCAD configuration

Video Display:              IBM Enhanced Graphics Adapter

Digitizer:                  Microsoft Serial or Bus Mouse

Plotter:                    Autodesk Device Interface

Printer plotter:            None

Press RETURN to continue:  <cr>

Publication Part Number W0001-520

Sheet 23 of 47

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 5 <cr>

Your current plotter is Autodesk Device Interface

Do you want to select a different one? <N>

If your current plotter is Autodesk Device Interface, then skip

over available plotter/driver selection.

Y <cr>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Available plotters:

1.  None

2.  ADI P386 plotter

3.  Alpha Merics (obsolete)

4.  Autodesk Device Interface

5.  CalComp 906/907/PCI

6.  Hewlett-Packard

7.  Houston Instrument DMP Series

8.  IBM 7300 Series

9.  PostScript device

10.  UNIX Plot device

Select device number of ? to repeat list <1>: 4 <cr>

Publication Part Number W0001-520

Sheet 24 of 47

Select output format:

0.  ASCII file

1.  Binary file

2.  AutoCAD DXB file

3.  Installed ADI driver

Output format, 0 to 3 <0>: 3 cr

Hexadecimal interrupt code (INT 0XXh) <78>: cr

Does your plotter have multiple pens? <N> Y cr

How many pens does it have, 2 to 127 <2>: 127 cr

Does the plotter have multiple dashed line styles? <N> cr

Is the plotter variable speed? <N> cr

Next you will specify the maximum plot size the plotter

can make, and the step size of the plotter.  You may

specify these either in millimeters or inches.  The

next question selects the units.

Specify plot size in millimeters? <N> cr

Maximum horizontal (X) plot size in inches <11.00>: 50 cr

Plotter steps per inch in the horizontal (X) direction <1000>: 1016 cr

Maximum vertical (Y) plot size in inches <8.5> : 36 cr

Plotter steps per inch in the vertical (Y) direction <1000>: 1016 cr

If you have previously measured the lengths of a horizontal

and a vertical line that were plotted to specific scale,

you may use these measurements to calibrate your plotter.

Would you like to calibrate your plotter? <N> N <cr>

Write the plot to a file? Y or N <cr>

Size Units (Inches or Millimeters) <I> I or M <cr>

Plot origin in Inches <0.00,0.00>: 0,0 <cr>

Standard values for plotting size

Size    Width   Height

A       10.50     8.00

B       16.00    10.00

C       21.00    16.00

D       33.00    21.00

MAX    163.80    23.50

Enter the Size of Width,Height (in Inches) <MAX>: D <cr>

Publication Part Number W0001-520

Sheet 25 of 47

Rotate 2D plots 90 degrees clockwise? <N>: N <cr>

Pen width <0.010>: <cr>

Adjust area fill boundaries for pen width? <N> <cr>

Remove Hidden lines? <N> <cr>

Specify scale by entering:

Plotted Inches=Drawing Units or Fit or ? <F>: <cr>

A U T O C A D

Copyright (C) 1982,83,84,85,86,87,88,89 AutoDesk, Inc

Release 10 c10 (12/21/89) DOS Extender

Serial Number xxx-xxxxxxxxxx

Configuration menu

0.  Exit to Main Menu

1.  Show current configuration

2.  Allow detailed configuration

3.  Configure video display

4.  Configure digitizer

5.  Configure plotter

6.  Configure printer plotter

7.  Configure system console

8.  Configure operating parameters

Enter selection <0>: 0 <cr>

If you answer N to the following question, all configuration

changed you have just make will be discarded.

Keep configuration changes? <Y> Y<cr>

FINISHED  The CalComp CCPADI driver is now configured.

Publication Part Number W0001-520

Sheet 26 of 47

CONFIGURING CALCOMP PLOTTERS

You have the option of configuring CCPADI 3.0 to match an existing

plotter setup or you can take the defaults of CCPADI 3.0 and configure

the plotter to match.

Publication Part Number W0001-520

Sheet 27 of 47

Configuring the 1023, 1025, or 1026 (ARTISAN) plotter

In the following prompts there will be several lines of instructions

for each question.  The ones preceded *D are the CCPADI 3.0 default

answers.  The ones preceded with *U are user definable and should be

written down so that CCPADI 3.0 can be configured to match.  If you

are going to configure CCPADI 3.0 to match the plotter setups,  do not

execute the *D instructions.

*D    CCPADI 3.0 defaults

*U    User defined

Follow the instructions in Chapter 1 of the ARTISAN User's Guide to

load media.  Select the 'Manual' softkey.  When the plotter finishes

sizing the media,  the display will show:

User#1     OrgRtrn

Online      Replot

Press the 'Next' key twice.  The display will show:

Set-up        Test

Size          Time

Press the 'Set-up' softkey.  The display will show:

Scale      PenParm

Host    Form Align

Press the 'Host' softkey.  The display will show:

Format        Baud

Data     Handshake

Press the 'Format' softkey.

*D  Use the lower left softkey to step through the list of data

Formats.  Stop when the display shows:

*U        Data format

[PCI]

Press the 'Exit key to return to Host menu.

Format        Baud

Data     Handshake

Publication Part Number W0001-520

Sheet 28 of 47

Press the 'Baud' softkey.

*D  Press the lower right softkey to step through the list of baud

rates.  Stop when the display shows:

*U        Baud Rate

[19.2K]

Press the 'Exit' key to return to Host menu.

Format        Baud

Data     Handshake

Press the 'Data' softkey.

*D  Press the upper left softkey to change the number of data bits.

Stop when the number changes to [7 bit]

*D  Press the upper right softkey to change the number of stop bits.

Stop when the number changes to [1 stop]

*D  Press the lower left softkey to change the parity type.  Stop when

the parity reads [Even Parity].

*U        [7 bit]   [1 stop]

[Even Parity]

Press the 'Exit' key to return to Host menu.

Format        Baud

Data     Handshake

Press the 'Handshake' softkey.

*D  Press the lower left softkey to step through the list of handshake

protocols.  Stop when the display shows:

*U        PCI Handshake

[DirCntrl]

Press the 'Exit' key to return to Host menu.

Format        Baud

Data     Handshake

Press the 'Next' key.  The display will show the Sync menu:

Sync        End Msg

Chksum

Publication Part Number W0001-520

Sheet 29 of 47

Press the 'Sync' softkey.

*D  Press the upper left key to change the # of sync to 2

*D  Press the '2' key twice to change the Sync Char to 22.

The display will show:

*U          # of sync [2]

Sync char.     22

Press the 'Exit' key to return to the Sync menu.

Sync        End Msg

Chksum

Press the 'End Msg' softkey.

*D  Press the '1' key and then the '3' key to change the End of Msg

char to 13.

The display will show:

*U   End of Msg Char

13

Press the 'Exit' key to return to the Sync menu.

Sync        End Msg

Chksum

Press the 'Chksum' softkey.

*D  Press the lower left softkey until PCI mode chksum changes to

ENABLED.  The display will show:

*U        PCI mode chksum

[ENABLED]

If you have changed any of the above options, press the 'Exit' key 3

times.  The display will show:

Save as User n?

n is current user number

Yes             No

Press the lower left softkey to save configurations.

If you were getting the current setups press the 'Exit' key 4 times to

return to the original menu.  The display will show:

User#1     OrgRtrn

Online      Replot

Press the 'Online' softkey to plot.

Publication Part Number W0001-520

Sheet 30 of 47

Configuring the 1043 or 1044 Pen Plotter

Set the switch on the back of the 1043/1044 plotter:

Bit 1 2 3 4 5 6 7 8

0 1 0 1 0 0 0 0   For Hardware handshaking but will work with

any protocol.

The Serial cable from the PC serial port should be attached to 'TO DTE

HOST' of the 1043/1044 plotter.

Power the plotter on.  Press the 'Setup' key, then the number 7 and

then press the 'Enter Key'.  The display will show 'Host set up

number'.  Press 'Next', 'Enter', make changes if configuring the

plotter to match CCPADI 3.0 defaults, otherwise step through the Host

setups and write down what you find in order to Configure CCPADI 3.0

to match.  Repeat the process for 8 HOST set up modes ('Next' and

'Enter').

For CCPADI 3.0 Defaults change to the following:

Host Set Up Mode      Top              bottom

0-Parity & bits/char  2 Even Parity    7 Bits/char

1-Stop bits & clock   1 Stop Bit       0 Int clock

2-Interface           0 Serial        00 IEEE Address

3-Host Baud           19200            0 PCI Mode

4-Muting & Checksum   NO Term muting   YES checksum

5-Isochronous & EOM   NO Isochronous   13 End of Mess Char

6-Direct Cntrl-Xon    YES Direct Cntrl NO XON/XOFF

7-Term Baud & Duplex  19200            0 Full

8-Sync Code & Value   2 Sync Code      22 Sync Code Value

Press 'Save', Enter user number and press 'Enter' to save any changes

you might of made.

Publication Part Number W0001-520

Sheet 31 of 47

Configuring the 1073, 1075, 1077 pen plotters.

There are two types of controllers available for these plotters, the

PCI and the PLOT MANAGER.  It is best if Field Service is called in

order to determine what controller is present and how to set the

switches, jumpers or thumb wheel switches.  If you wish to attempt the

setup you will need the appropriate manual.

Publication Part Number W0001-520

Sheet 32 of 47

Configuring the 2024 or 2036 Pacesetter

The Pacesetter plotters are unique in that they are the only CalComp

pen plotter capable of accepting data both serially or parallel.  The

setup procedures for the two are similar in most respects.

The following paragraphs contain the procedures for changing the

communications parameters.  Each procedure is self-contained.

To Change parameters, proceed as follows:

1.  Power-on the plotter

2.  Press the Menu key.

NOTE

The Port Type menu display only appears when the optional

Centronics interface is installed.  Skip this step and

proceed to step 4 if the plotter only has a RS-232C (serial)

port.

3.  Press the up or down arrow keys until the display shows:

Port Type:

Use the left or right arrow keys to select Port Type:

If serial                             If Parallel

Port Type:                            Port Type:

RS-232C                               Centronics

4.  This step is only for serial communications.

Press the down arrow key until the display shows:

Protocol:

Use the left or right arrow keys to select:

Protocol:

PCI

If using parallel communications, skip to step 8.

Publication Part Number W0001-520

Sheet 33 of 47

5.  Use the down arrow key to display:

Baud Rate:

Use the left or right arrow keys to select the correct baud

rate configured in AutoCAD.  CCPADI 3.0 defaults to:

Baud Rate:

19200

6.  Use the down arrow key to display:

# of bits-parity

Use the left or right arrow keys to select the correct number

of bits per character and the type of parity configured in

AutoCAD.  CCPADI 3.0 defaults to:

# of bits-parity

7-even

7.  Use the down arrow key to display:

Handshake:

Use the left or right arrow keys to select the correct

handshaking configured in AutoCAD.  CCPADI 3.0 defaults to:

Handshake:

Hardware

8.  Use the down arrow key to select:

Sync # EOB CHK

There are three pre defined configurations for this options

labeled A, B, or C.  Use the left or right arrow keys to display

each.  If one of the three selections match the configurations

stored in Autocad, display it and proceed to step 14.  CCPADI 3.0

defaults to:

Sync # EOB CHK

A:  22   2   13   ON

Publication Part Number W0001-520

Sheet 34 of 47

9.  If none of the pre-defined configurations match your needs

select:

Sync # EOB CHK

Special Format-1 or 2

10. Then press the down arrow key.  The following will be

displayed:

PCI-# of sync

2

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

11. Then press the down arrow key.  The following will be

displayed:

PCI-Sync Char

22

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

12. Then press the down arrow key.  The following will be

displayed:

PCI-EOM Char

13

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

13. Then press the down arrow key.  The following will be

displayed:

PCI-Checksum

On

Use the left or right arrow keys to change this value to match

the value configured in AutoCAD.

Publication Part Number W0001-520

Sheet 35 of 47

14. Then press the down arrow key until the following is

displayed:

Save as User:

NO

Use the left or right arrow keys to select the user number

under which these value will be stored.

15. Then press online.  The plotter is now ready for operation.

Publication Part Number W0001-520

Sheet 36 of 47

Configuring the 52224, 52424, 52236, 52436, 67436, 68436, or 68444

These setup instructions assume that a communications terminal is

attached to the TERM port located on the back of the plotter.

Press <CTRL-P> and enter user PIN or press return.

If you only wish to determine the present configuration of the plotter

use the DISPLAY command to display them.

Determine which port is connected to your computer by tracing the

cable.  The 52224, 52236, 52424, and the 52436 have 1 parallel port

called 'PAR' and three serial ports called 'COM_1  COM_2  COM_3'.  The

67436, 68436 and the 68444 have two parallel and two serial ports

called PAR_1, PAR_2, COM_1, and COM_2 respectfully.

If your PC is connected to a Parallel port type:

DISP PAR on the 52224, 52236, 52424, 52436 to display the parallel

parameters or DISP PAR_n, where n is the parallel port number, on the

67436, 68436, or the 68444.

To changed the Parallel parameters, TYPE:

PAR  or PAR_n

Input Monitor Dump                NO

Host to Plotter CDCL              CENT

Dead Job Time Out                 30

Time Out Action                   Force

Plot Command Language             907

Steps Per Inch                    200 for the 52224 or 52236

400 for the 52424, 52436,

67436, 68436, 68444

Beginning of Record Sync Char    $16 hex

Number of Sync Chars              2

End of Record char                $D

Checksum                          Y

Print                             N

Publication Part Number W0001-520

Sheet 37 of 47

If your PC is connected to COM_1, COM_2, or COM3 type

COM_n                            n is either 1, 2, or 3

Input Monitor dump                NO

Baud Rate                         19200

Character Framing                 7,e,1

Host to plotter CDCL              CTS

Plotter to Host CDCL              CTS

Dead Job Time Out                 30

Time out action                   Force

Plot Command Language             907

Steps per inch                    200 for the 52224 or 52236

400 for the 52424 or 52436

67436, 68436, 68444

Plot Data Protocol                CTS

Beginning of Record Sync Char    $16

Number of sync Chars              2

End of Record Char                $D

Checksum                          Y

To save the setups type:

SAVE n ALL    n is the user number.   The power on default user

number is 1.

To retrieve a user setup type:

USER n all    n is the user number.

If the configurations are not saved they will be lost when the plotter

is powered off.

Publication Part Number W0001-520

Sheet 38 of 47

Configuring the 57424, 57436, 57444, 58424, 58436, or 58444

An example of configuring the 908K or XP board thru OPCOM.

Refer to CalComp plotter users guide OPCOM (Operator

Communication) operations with the optional attached terminal.

These setup instructions assume that a communications terminal is

attached to the TERM port located on the back of the plotter.

PARALLEL COMMUNICATIONS:

(Framing: bits, parity and stop bits are not applicable for

Centronics interface).

<CTRL-P>

To change the communications parameters type:

COM

MODE                             CENT

CSUM                             Y

FRAME                             8,n,1

EOB                              $D  or  13

TACODE                            0

CLOCKING                          ASYNC

SYNC                             $16,$16  or  22,22

To save the parameters type:

SAVE n ALL   n is a user number

To view the existing communications setups type:

DISP COM

Publication Part Number W0001-520

Sheet 39 of 47

If OPCOM is used,  these switches may be ignored.  This is provided as

reference for CalComp Customer Service and Field Analyst.

An example of 908K Board Switch setting with CCPADI 3.0.

Bits

Switches  8 7 6 5 4 3 2 1

S3      0 0 0 1 1 1 0 1  (bit 8    diagnostic mode disabled

bit 7    units = inches

bit 6-4  plotter width = 36 inches

bit 3-2  resolution = 400 dpi

bit 1    indicates color)

S4      1 1 0 0 0 0 0 0  (bit 8-7  disk type: 70 megabyte

bit 6    single disk installed

bit 5    tape bits/character: 8 bits

bit 4    tape density: 1600 bpi

bit 3-2  most significant strip

bit 1    pen plotter limits emulation

disabled)

S5      0 0 1 0 0 0 1 0   bit 8-5  DIRECT CONTROL MODE

0 0 1 1 0 0 1 0            CENTRONICS

bit 4-1  AutoCAD stepsize:  400/inch

S6      0 1 1 0 1 1 0 1  (bit 8-4  End of buffer char = '13 $D'

bit 3-1  character framing: 8 bit

no parity, 1 stop bit

S7      0 1 0 0 0 0 0 0  (bit 8    terminal muting disable

bit 7    checksum enable

bit 6    X16 clocking

bit 5-1  half duplex turn around

character not used

S8      1 0 0 1 0 1 1 0  (bit 8    double sync character

bit 7-1  sync character = '22 $16'

- Set Switch S1 to NORMAL and LOCAL

- Set rotary switches S10 and S11 according to the host baud rate

and terminal baud rate, respectively.

Publication Part Number W0001-520

Sheet 40 of 47

Configuring the 5912, 5913 ColorView, or the 5902A Plotmaster

Enclosed on the floppy with CCPADI 3.0 is a program called SETUP.EXE.

This program is used to set the communication parameters.

SETUP

CALCOMP Color Thermal Plotter Configuration

Copyright (C) Calcomp 1988 Rev. x.x

Use the left/right arrow keys to scroll thru the options

Use the up/down arrow keys to scroll thru the menu

When the correct information is displayed use the <enter> key

Model Type ...............  Model 5912

Interface Mode ...........  PCI HCBS

F5 key exits the program and returns to DOS

ESC key restarts the program

HOME key moves the cursor to the top selection on current screen

END key moves the cursor to the last selection on current screen

When New: is displayed enter the data with the numeric keys

The initial screen will ask for either the 5912, 5913, or the

Plotmaster.  Use the arrow keys to select.  The interface mode is for

either PCI HCBS or PCI AUTOCAD.  If CCPADI 3.0 defaults are to be

accepted select PCI AUTOCAD.

Publication Part Number W0001-520

Sheet 41 of 47

Model 5912 Configurable Parameters

Port connected to                COM1  (use arrow key for

other ports)

Baud Type                        19200

Parity Enable                    Enabled

Parity Type                      Even

Data Bits                        7

Stop Bits                        1

PCI DATA

Number of Sync characters        2

Sync character code              22

Checksum enable                  Enabled

Handshake type                   Direct Control

End of Message Character         13

Turnaround Code                  0

Scale factor numerator           1

Scale factor denominator         1

X axis Origin Offset             0

Y axis Origin Offset             0

Print Timeout Value              300

Plotmaster emulation             NO

Change Screen with PgUp or PgDn

Send Data (F2), Default (F4), System (F5), Restart <ESC>

Change the parameters to look like the above if you are using

CCPADI 3.0 defaults.

F2 will send the parameters to the Colorview and save them in EEPROM.

F5 will exit the program.

Publication Part Number W0001-520

Sheet 42 of 47

Configuring the 6603 or 6613 Colormaster Plus VRC

The setups on the VRC unit us accomplished from the control panel.

Which after initialization should look something like this:

ONLINE         Reset

Setups         Tests

Take the printer OFFLINE by placing the cursor on ONLINE and pressing

ENTER.  The following menu is displayed:

OFFLINE        Reset

Setups         Tests

Position the cursor on Setups and press ENTER.  The following is

displayed:

SETUPS

User          Factory

Position the cursor on User and press enter.  The following is

displayed:

USER        DataFormats

Printer           Ports

Position the cursor on DataFormats and press enter.  The following is

displayed:

DATA FORMAT

CCGL     VDI   Epson

Position the cursor on CCGL and press enter.  The following is

displayed:

CCGL

Numerator  : 02430

Use the cursor to select each digit and change this value to read

00001.  Press enter when completed.  The following will be displayed:

CCGL

Denomintr  : 70633

Publication Part Number W0001-520

Sheet 43 of 47

As with the Numerator used the arrow keys to select each digit and

change this value to read 00001.  Press enter when completed.  The

following will be displayed:

CCGL

X Offset  :+00000

Press enter and the following will be displayed:

CCGL

Y Offset  :+00000

Press enter and the following will be displayed:

CCGL

# of Syncs    :1

Use the arrow keys to change this value to 2 if using CCPADI 3.0

defaults.  Press enter when finished and the following will be

displayed:

CCGL

Sync Code      :2

Use the arrow keys to change this value to 22 if using CCPADI 3.0

defaults.  Press enter when finished and the following will be

displayed:

CCGL

End Code       :3

Use the arrow keys to change this value to 13 if using CCPADI 3.0

defaults.  Press enter when finished and the following will be

displayed:

CCGL

Checksum:   Disable

Use the arrow keys to change this value to enable if using CCPADI 3.0

defaults.  Press enter when finished and the following will be

displayed:

CCGL

TimeOut:    0010 S

Use the arrow keys to change this value to 300 if using CCPADI 3.0

defaults.  Press enter when finished and the following will be

displayed:

CCGL

5902A Pens:  Yes

Publication Part Number W0001-520

Sheet 44 of 47

Use the arrow keys to change this value to NO if using CCPADI 3.0

defaults.  Press enter when finished and the following will be

displayed:

CCGL

Param Set: HCBS

You have now set all the CCGL Setup parameters.  To go to the USER

submenu, press CLEAR.

USER        DataFormats

Printer           Ports

Select 'Ports' and press enter.  The following will be displayed:

SERIAL PORT

Baud Rate: 9600

Use the arrow keys to change this setting to 19200 if you are using

CCPADI 3.0 defaults.  Press enter when finished and the following will

be displayed:

SERIAL PORT

Data Bits: 8

Use the arrow keys to change this setting to 7 if you are using CCPADI

3.0 defaults.  Press enter when finished and the following will be

displayed:

SERIAL PORT

Stop Bits

Use the arrow keys to change this setting to 1 if you are using CCPADI

3.0 defaults.  Press enter when finished and the following will be

displayed:

SERIAL PORT

Parity:  None

Use the arrow keys to change this setting to even if you are using

CCPADI 3.0 defaults.  Press enter when finished and the following will

be displayed:

SERIAL PORT

Handshk:  CCGL

Press enter when finished and the following will be displayed:

SERIAL PORT

Handshk:  XON/XOFF

Publication Part Number W0001-520

Sheet 45 of 47

Use the arrow keys to change this setting to DTR/RTS if you are using

CCPADI 3.0 defaults.  Press enter when finished and the following will

be displayed:

USER        DataFormats

Printer           Ports

Press the Clear button until the following is displayed:

ONLINE         Reset

Setups         Tests

Publication Part Number W0001-520

Sheet 46 of 47

LIST OF MATERIALS

The CalComp Supplied floppy should contain the following files:

BATCHPLT.EXE                    Program

AN_D_SRH.DT2                    Batchplt support file

AN_E_SRH.DT2                    Batchplt support file

AN_D_PLT.DT2                    Batchplt support file

AN_E_PLT.DT2                    Batchplt support file

AR_D_PLT.DT2                    Batchplt support file

AR_E_PLT.DT2                    Batchplt support file

IS_D_PLT.DT2                    Batchplt support file

IS_E_PLT.DT2                    Batchplt support file

BATCHPLT.DOC                    Batchplt documentation

INSTALL.EXE                     Installation program

CCP386.EXP                      Protected Mode ADI driver

CCP386.DOC                      Protected Mode Documentation

CCPADI.EXE                      Real Mode ADI driver

CCPCFG.EXE                      Real Mode Driver config program

CCPADI.DOC                      Real Mode documentation

SETUP.EXE                       Thermal Transfer Setup program

SYSTEM REQUIREMENTS

IBM PC or Compatible

EGA or VGA card and monitor

DOS 2.0 or later

AutoCAD 10 286/386  or  11 286/386

Publication Part Number W0001-520

Sheet 47 of 47

DEFAULT PEN DEFINITIONS

PEN      COLOR         WIDTH in inches

1       RED            0.01

2       YELLOW         0.01

3       GREEN          0.01

4       CYAN           0.01

5       BLUE           0.01

6       MAGENTA        0.01

7       BLACK          0.01

8       ORANGE         0.01

9       LIGHT RED      0.01

10       BROWN          0.01

11       LIGHT GREEN    0.01

12       LIGHT CYAN     0.01

13       LIGHT BLUE     0.01

14       LIGHT MAGENTA  0.01

15       1/2 BLACK      0.01

PEN WIDTH TABLE

PLOTTER RESOLUTION

WIDTH     200 DPI        300 DPI        400 DPI

IN   MM        IN    MM       IN   MM

1   .005   .13    .00375   .10    .0025   .06

2   .010   .25    .00750   .19    .0050   .13

3   .015   .38    .01125   .29    .0075   .19

4   .020   .51    .01500   .38    .0100   .25

5   .025   .64    .01875   .48    .0125   .32

6   .030   .76    .02250   .57    .0150   .38

7   .035   .89    .02625   .67    .0175   .44

8   .040  1.02    .03000   .76    .0200   .51

9   .045  1.14    .03375   .86    .0225   .57

10   .050  1.27    .03750   .95    .0250   .64

11   .055  1.40    .04125  1.05    .0275   .70

12   .060  1.52    .04500  1.14    .0300   .76

13   .065  1.65    .04875  1.24    .0325   .83

14   .070  1.78    .05250  1.33    .0350   .89

15   .075  1.91    .05625  1.43    .0375   .95

16   .080  2.03    .06000  1.52    .0400  1.02