Tune.Doc for NIC Bridge/Async Version 1.1i

Nov 12 1992

Copyright (c) Eighth Layer Systems 1990-1992.

All rights reserved.

Warning:  The possibility of file corruption exists when using any software.

Please take whatever steps are required to protect vital files.

This documentation is formatted with 23 line pages. It contains

IBM PC type block graphics characters.

��������Ŀ

� TUNING �

����������

���������

General

���������

Local Area Network protocols often require tuning in order to be used

over low speed links. This is due to the fact that they often incorporate

default timeout values which, while suitable for the high speeds of Ethernet

and Token Ring, are unsuitable for the low speeds of serial ports.

Testing with MS/IBM NetBEUI has shown that it can be tuned for used at

speeds as low as 300 baud. 3Com NBP has been tuned to be successfully used at

speeds as low as 2400 baud. MS TCP/IP has been used at speeds as low as 9600

baud with no Lanman.Ini or Protocol.Ini tuning.

The following sections detail tuning considerations for the MS/IBM

NetBEUI protocol and the 3Com NBP protocol.

���������������������������������

NetBEUI - Lanman.Ini/IBMLan.Ini

���������������������������������

The tuning option of NicSetup modifies the Lanman.Ini/IBMLan.Ini

Workstation/Requester parameters for slow links. These modifications have been

compiled from those recommended by Microsoft for slow links and used by MS LM

Remote Access Service.

Testing with MS NetBEUI over relatively error free links has shown

some of these modifications to be detrimental to performance when copying

large files. Using default Lanman.Ini parameters can increase the performance

of large file copies. Most other operations, however, will not benefit.

Lanman.Ini/IBMLan.Ini [workstation]/[requester] parameters as modified

by NicSetup for DOS enhanced workstations:

- WRKHEURISTICS:    bit  8 = 0 for Opportunistic Locking off

bit  9 = 0 for Open-Read SMB off

bit 11 = 9 for 36 second beep window

bit 14 = 1 for Raw I/O off

Lanman.Ini/IBMLan.Ini [workstation]/[requester] parameters desirable

for DOS enhanced workstations:

- SIZWORKBUF:       sizworkbuf = 4096 (or higher)

- WRKHEURISTICS:    bit  8 = 0 for Opportunistic Locking off

bit  9 = 1 for Open-Read SMB on

bit 11 = 9 for 36 second beep window

bit 14 = 0 for Raw I/O on

Lanman.Ini/IBMLan.Ini [workstation]/[requester] parameters as modified

by NicSetup for OS/2 workstations:

- WRKHEURISTICS:    bit  0 = 0 for Opportunistic Locking off

bit  6 = 0 for Open-Read SMBs off

bit  8 = 0 for NCB Chain Sends off

bit  9 = 0 for Buffer Small R/W Requests off

bit 11 = 0 for Raw R/W SMBs off

- SESSTIMEOUT:      sesstimeout = 60000

Lanman.Ini/IBMLan.Ini [workstation]/[requester] parameters desirable

for OS/2 workstations:

- SIZWORKBUF:       sizworkbuf = 4096 (or higher)

- WRKHEURISTICS:    bit  0 = 0 for Opportunistic Locking off

bit  6 = 1 for Open-Read SMBs on

bit  8 = 1 for NCB Chain Sends on

bit  9 = 1 for Buffer Small R/W Requests on

bit 11 = 1 for Raw R/W SMBs on

- SESSTIMEOUT:      sesstimeout = 60000

������������������������

NetBEUI - Protocol.Ini

������������������������

The tuning option of NicSetup modifies the NetBEUI Protocol.Ini

parameters for speeds as low as 2400 baud. Specifically, the following

parameters are set:

NETBIOSRETRIES = 2

NETBIOSTIMEOUT = 2000

ADAPTRATE      = 0

MAXIN          = 1

MAXOUT         = 1

PIGGYBACKACKS  = 0

T1             = 12000

TI             = 60000

The T1 (T One) timeout value may be modified according to the minimum

expected baudrate for remote links. Recommended values are shown below.

Minimum BaudRate:   300    1200   2400   9600  57600  115200

T1 Value to use:  65535   24000  12000   3000    500     500

NicSetup sets the NetBEUI MAXOUT parameter to 1. This is the number

of packets NetBEUI will send before waiting for an ACK or acknowledgement

packet. During this waiting time, no useful data is being transferred on the

link. This decreases performance, particularly through bridges and/or at

higher link speeds.

Setting MAXOUT to 4-8 provides much better performance. One problem

with this however is timeouts. For an example, if NetBEUI sends MAXOUT = 4

packets before waiting for an ACK packet, it must wait for the 4 packets to

be transmitted over the serial link, received and acknowledged. This could

take 4 times longer than waiting for 1 packet. Thus the recommended T1 (T One)

values shown below must be multiplied by the MAXOUT value.

T1 (T One) timeout value recommended if MAXOUT = 1:

Minimum BaudRate:   300    1200   2400   9600  57600  115200

T1 Value to use:  65535   24000  12000   3000    500     500

If MAXOUT = 4, the following values for T1 may be used:

Minimum BaudRate:   300    1200   2400   9600  57600  115200

T1 Value to use:  -----   -----  48000  12000   2000    2000

One potential problem with raising T1 concerns error prone links.

When an error occurs on the link, retransmission of the data will not take

place until the T1 timer has expired for the packet in error. During the

time that T1 is expiring, there is generally no data transmission occuring.

Thus, if MAXOUT = 1 at 2400 baud and T1 = 12000, a single transmission error

can cause the link to remain idle for 12 seconds. If MAXOUT = 4 and T1 = 48000

this idle time increases to 48 seconds.

Thus, raising T1 indiscriminately so as to raise MAXOUT is not a good

idea on error prone links. An error correcting modem and use of 16550 buffered

serial ports at higher speeds will both help to greatly reduce this problem.

Also note that the T1 value on a server will affect local users as well as

remote users.

One last potential problem with raising MAXOUT above 1 regards remote

bridge buffering. When NetBEUI on a server transmits MAXOUT packets and then

waits, those 4 packets will be received by the bridge in quick succession at

local network speeds. A remote bridge must buffer those packets for

transmission over the slow link. When all of the bridges' buffers are in use,

any further packets received will be dropped. This will cause timeouts and

lower the efficiency of the link.

By default the NBridge driver can buffer 16 packets. Theoretically,

this should allow up to 4 workstations to transfer files through the bridge

from a server set with MAXOUT = 4. Practically, some bridge buffers should

always be reserved for broadcasts.

If NAsync is installed on a server and bound directly to NetBEUI

a "trick" may be used to limit the T1 multiplier to 2. This requires setting

the following NetBEUI parameters in Protocol.Ini:

mintransmits = 1

maxtransmits = 2

This causes packets to be queued internally in NetBEUI rather than

in NAsync and postpones the start of the T1 timeout. The MAXOUT parameter

may then be raised as high as 8 or 16, while only doubling the T1 parameter

from its' MAXOUT = 1 value.

������������������

NBP - Lanman.Ini

������������������

Lanman.Ini [workstation] parameters required for DOS enhanced NBP

workstations:

- SIZWORKBUF:       sizworkbuf = 1024

- WRKHEURISTICS:    bit  8 = 0 for Opportunistic Locking off

bit  9 = 0 for Open-Read SMB off

bit 11 = 9 for 36 second beep window

bit 14 = 1 for Raw I/O off

Lanman.Ini [workstation] parameters required for OS/2 NBP

workstations:

- SIZWORKBUF:       sizworkbuf = 1024

- WRKHEURISTICS:    bit  0 = 0 for Opportunistic Locking off

bit  6 = 0 for Open-Read SMBs off

bit  8 = 0 for NCB Chain Sends off

bit  9 = 0 for Buffer Small R/W Requests off

bit 11 = 0 for Raw R/W SMBs off

- SESSTIMEOUT:      sesstimeout = 60000

Lanman.Ini [server] parameters required for OS/2 NBP servers:

- SIZREQBUF:        sizreqbuf = 1024

- NUMBIGBUF:        numbigbuf = 0

- SRVHEURISTICS:    bit 15 = 9 for Opportunistic Locking off

��������������������

NBP - Protocol.Ini

��������������������

The following Protocol.Ini NBP parameters should be set for low speed

links:

PROBETIMEOUT    = 1000

MAXPROBETIMEOUT = 8000

The latest and possibly last version of 3Com NBP, version 2.0D, also

provides a RESCALE parameter. When present in Protocol.Ini it allows NBP to

more easily adapt to timeouts and may thus be useful on low speed links.

It should be noted that the use of the MAXPROBETIMOUT and RESCALE

parameters for NBP is not supported by 3Com.

��������������

NAsync Driver

��������������

In cases where a protocol cannot be tuned sufficiently for low speed

operation the NAsync Protocol.Ini parameter MAC_MAXFRAME may be useful. This

parameter sets the maximum framesize that NAsync reports it is capable of to

protocols. By default, MAC_MAXFRAME = 1514, the maximum framesize for an

Ethernet adapter. It may be set as low as 128 by adding to the [NASYNC_NIF]

section of Protocol.Ini the line "MAC_MAXFRAME = 128".

By setting the maximum framesize to 128 the maximum effective timeout

of a protocol may be increased more than 6 times, relative to the default.

Performance, however, will be adversely affected. If this parameter is

added, it should be added at both the client and host side.

The compression and encryption features of NAsync can both consume

considerable amounts of CPU power. In some situations however, the overhead

of compression may be able to provide an increase in throughput for remote

users and this may be more important than CPU consumption.

The NAsync Protocol.Ini parameter CHARS_FILTERBC can be set to "YES"

on servers to prevent the transmission of broadcast packets. This should only

be used on servers without NIC Bridge as the NBridge driver has its' own

parameter to filter broadcasts.

Due to the broadcast of an ARP request by a server running MS TCP/IP

during session setup, broadcast filtering cannot be used with MS TCP/IP. As

the most common server broadcast is the server announce broadcast (which

allows active servers to be determined with NET VIEW) it may be better to

to increase the interval of this broadcast. This can be done by increasing

the Lanman.Ini/IBMLan.Ini [server] SRVANNOUNCE parameter.

The retransmission of network packets caused by serial overrun errors

has a disastrous effect on performance. It has been observed that the MS/IBM

NetBEUI protocol driver can contribute to the likelihood of these errors,

particularly when not tuned for low speed use. This is one more reason to

ensure that NetBEUI is properly tuned for low speed use.

The use of serial ports with the 16550A UART can greatly reduce the

likelihood of serial overrun errors. When this UART is used NAsync will detect

it and enable support for it's special features. Some of these features

include the ability to dramatically reduce the CPU load required to support

the serial port.

The Protocol.Ini COMM_RXTRIGGER parameter is provided specifically for

the 16550A. This sets the number of characters, in a buffer of 16 characters,

which will normally be received before an interrupt is triggered. It can be

set to 0, 1, 4, 8 or 14. The value 0 will set the trigger level to a value

appropriate for the baudrate and is the default for OS/2. The value 1 provides

the greatest protection from serial overrun errors and is the default for DOS.

The value 14 provides the least protection but also provides the lowest

consumption of CPU power for a given baudrate. The tradeoff between these

factors is best determined by the given configuration.

�������������������������

NBridge & SRTok Drivers

�������������������������

The Nbridge driver allows filtering broadcasts with the Protocol.Ini

FILTERBC_MAC parameter. This can greatly decrease overhead on slow links

experiencing significant broadcast traffic. As noted in the NAsync Driver

section however, this does not allow the proper operation of MS TCP/IP. In

addition, broadcast filtering tends to be more useful for remote access,

rather than for remote bridging where it can block needed broadcasts.

In some situations the custom filtering available via the Protocol.Ini

CUSTOMFILTER parameter may be more appropriate. This can, for example, be used

to block broadcasts from certain network source addresses.

The SRTok driver provides a Protocol.Ini SRBC parameter which, when

set to "NO" will block single route broadcasts from being received on the

Token Ring adapter. This can be very effective for eliminating broadcasts

to remote PCs on a Token Ring network.

������������������Ŀ

� ACKNOWLEDGEMENTS �

��������������������

All trademarks are the property of their owners.

This includes, but is not limited to, the trademarks

of 3Com, IBM, Microsoft, and Hayes.

;