NAsync.Doc for NIC Async Version 1.1i

Nov 12 1992

Copyright (c) Eighth Layer Systems 1990-1992.

All rights reserved.

Warning:  The possibility of file corruption exists when using any software.

Please take whatever steps are required to protect vital files.

This documentation is formatted with 23 line pages. It contains

IBM PC type block graphics characters.

����������Ŀ

� CONTENTS �

������������

RELEASE NOTES..................................................  4

INTRODUCTION................Definitions........................  5

Files Included.....................  7

NAsync Drivers.....................  9

NaComA Comport Agent............... 10

Na Control Utility................. 11

CONSIDERATIONS..............NIC Bridge......................... 13

Protocols.......................... 14

Serial Ports....................... 16

Modems............................. 21

SETUP/MAIN CONFIGURATION....MS Lan Manager 2.x................. 29

IBM Lan Server..................... 37

OPERATION...................................................... 39

TROUBLESHOOTING.........Traps.................................. 42

Lan Manager Problems................... 43

Modem Problems......................... 44

NASYNC PARAMETERS.............................................. 46

ACKNOWLEDGEMENTS............................................... 57

���������������Ŀ

� RELEASE NOTES �

�����������������

- The Install.Exe utility can be used to upgrade recent releases

of the software by installing over a previous version.

- Version 1.1i of the NAsync drivers incorporate some changes

which require that host and client PCs both be updated to

this new version.

- A number of error and exception conditions are now detected

and reported by the Na.Exe utility.

- A number of previously undocumented parameters are now documented.

See the NASYNC PARAMETERS section for details.

��������������Ŀ

� INTRODUCTION �

����������������

�������������

Definitions

�������������

The term "LanRoot" denotes the directory under which all network

software is installed. For MS Lan Manager this is \LANMAN.DOS for DOS or

\LANMAN for OS/2. For IBM Lan Server this is \IBMLAN for OS/2.

NIC is a common network industry acronym for Network Interface Card.

This is the Ethernet, Token Ring or other adapter which connects PCs to a

network. NIC Async allows an asynchronous serial port to be used as a NIC.

MAC is an acronym for Medium Access Control. Network adapter drivers

are often referred to as MAC drivers.

NDIS is an acronym for the Microsoft/3Com Network Driver Interface

Specification. This is a software interface designed to allow multiple network

protocols to control network adapter drivers. NIC Async adheres to the NDIS

version 2.01 specifications.

NIC Async is symmetrical. Any IBM compatible PC, whether DOS or OS/2,

standalone or networked, workstation or server may initiate a call to, or

answer a call from, another NIC Async equipped PC. This creates the potential

for confusion about which machine is the "server". This document uses the term

"host" to denote the PC which has answered a call. The term "client" denotes

the PC which has initiated a call.

Note that a PC may be setup to allow both incoming and outgoing calls.

A connection between two PCs, however, will always be initiated by one of them

and answered by the other. Thus, the designation of each PC may depend only on

which PC initiated the call.

����������������

Files Included

����������������

The following files are included on the Install Disk:

\Readme  .Doc       NIC Bridge/Async introductory document

\NAsync  .Doc       This NIC Async document

\Tune    .Doc       Tuning document

\Readme  .Bat       DOS batch file for NicDoc.Exe

\Readme  .Cmd       OS/2 command file for NicDoc.Exe

\NicDoc  .Exe       NIC Bridge/Async DOS & OS/2 Documentation Viewer

\Install .Exe       NIC Bridge/Async DOS & OS/2 Install Utility

\NicSetup.Exe       NIC Bridge/Async DOS & OS/2 Setup Utility

\NicCfg  .Exe       NIC Bridge/Async DOS & OS/2 Configuration Utility

\NAsync  .Nif       NIC Async "NIF" file for IBM LS installation

\NAsync  .DOS       DOS  NIC Async Driver for 386 & up

\NAsync  .D86       DOS  NIC Async Driver for 8086 & up

\NAsync  .OS2       OS/2 NIC Async Driver for 386 & up on OS/2 1.2 & up

\NAsync  .O86       OS/2 NIC Async Driver for 286 & up or OS/2 1.1

\NAComA  .Exe       OS/2 ComPort Agent

\Na      .Exe       DOS & OS/2 Command Line Utility

\MSLM\Protocol.AD           DOS  Protocol.Ini for NIC Async

\MSLM\Protocol.AO           OS/2 Protocol.Ini for NIC Async

\MSLM\NAsync  .ND           DOS   386 NAsync.Nif

\MSLM\NAsync  .ND8          DOS  8086 NAsync.Nif

\MSLM\NAsync  .NO           OS/2  386 NAsync.Nif

\MSLM\NAsync  .NO8          OS/2 8086 NAsync.Nif

����������������

NAsync Drivers

����������������

The DOS and OS/2 drivers NAsync.DOS and NAsync.OS2 are the "engine"

of NIC Async. The vast majority of work is done within these drivers. They

appear to NDIS based protocols as regular Ethernet or Token Ring drivers.

The drivers are optimized for and require an Intel 80386 or higher

based PC. Support for PCs based on 80286 or lesser CPUs is provided by the

drivers NAsync.D86 for DOS and NAsync.O86 for OS/2. NAsync.O86 is also

required on OS/2 versions prior to version 1.2.

The NAsync drivers support two modes under OS/2: "Direct Port Mode"

and "OS/2 Filesystem Mode". Direct Port mode allows the driver to directly

control the serial port and is required under DOS. It supports standard IBM

AT type ports COM1 to COM4 on ISA PCs or IBM PS/2 type ports COM1 to COM8

on MCA PCs. Interrupt sharing on MCA PCs is supported only under OS/2.

OS/2 Filesystem mode allows the driver to access ports via OS/2 Com drivers

such as COM01.Sys, COM02.Sys or COM.Sys and uses the NaComA Comport Agent.

Up to nine instances of the NAsync driver may be loaded in Config.Sys.

Each instance will support one serial port.

����������������������

NaComA Comport Agent

����������������������

The OS/2 ComPort Agent NaComA.Exe does the work of communicating with

a serial port through the file system in OS/2. The use of this Agent is

optional under OS/2.

If you use a serial board such as those in the Digiboard line, the

board should come with drivers for OS/2. The agent is required for special

boards such as these.

The command "NACOMA *" will start up to nine instances of the OS/2

ComPort Agent in the background. This will service the maximum of nine NAsync

driver instances.

��������������������

Na Control Utility

��������������������

The control utility Na.Exe is a command line utility for the control

of NIC Async drivers. It provides options to dial, hangup, set parameters and

view statistics. It runs under both DOS & OS/2.

The command "NA" with no options provides the option listing shown

on the following page.

Note that a number from 1 to 9 may precede any Na option to specify

the driver number under control. For example, "NA 2 D" performs dial out on

the second NAsync driver loaded in Config.Sys, while "NA D" performs the same

function on the first driver.

Na control utility for NIC Async v1.1i.

Copyright (c) Eighth Layer Systems 1990-1992. All rights reserved.

Na may be invoked with the following options:

S                to view Statistics

S+               to view detailed Statistics

D                to Dial

H                to Hangup

E+               to enable Encryption

E-               to disable Encryption

C+               to enable Compression

C-               to disable Compression

N   number       to set a new default phone number

P   password     to set a new default password

T                to Test datalink

M                to view Miscellaneous values

?                to view these options

?+               to view rarely needed options

A NIC Async driver number may precede any option.

The range is 1 to 9. The default is 1.

����������������Ŀ

� CONSIDERATIONS �

������������������

������������

NIC Bridge

������������

NIC Async may be used alone or in combination with NIC Bridge. Remote

user PCs always use NIC Async alone. When used alone on a server, it allows

remote access to that server only. A remote user will be able to access all of

the resources on that server but will have no access to other resources on the

network.

When installed in combination with NIC Bridge a remote user or remote

network will be able to access all resources on the local network. Further

information on NIC Bridge can be found in the NBridge.Doc file.

�����������

Protocols

�����������

The major difference between a real network connection and NIC Async

is the difference in speed. The low speeds used by modems can cause problems

for some protocols and applications. Some protocols, such as MS/IBM NetBEUI

can be tuned for low speed use. Other protocols, such as 3Com NBP, were not

specifically designed to be used at low speeds, but may be tunable for them.

Protocols which can be manually tuned, or that automatically tune

themselves for low speed operation are recommended. The list of such protocols

includes MS/IBM NetBEUI and various implementations of TCP/IP and XNS.

Some network applications, and particularly those that communicate

via NetBIOS, will not operate well or possibly at all, at low speeds.

NIC Async drivers are installed on servers and workstations as network

adapter drivers. Multiple copies of the NIC Async drivers may be loaded, one

for each serial port providing network access. Each driver loaded appears as a

seperate network adapter driver.

In many cases this imposes a requirement for mechanisms to deal with

multiple adapters. Both MS Lan Manager and IBM Lan Server provide support for

multiple network adapters. This support must also be provided by the network

protocols used by the network operating system.

Some NDIS protocols, such as MS/IBM NetBEUI, provide support for

"binding" to multiple adapter drivers. Some protocols, such as 3Com NBP,

support only one adapter driver but allow multiple copies of the protocol

driver to be loaded. Both of these protocol types can be handled by NIC Async.

Other protocols, such as MS TCP/IP, support only one adapter driver

and can be loaded only once. This type of protocol restricts the options

available. If this type of protocol is needed on a local network adapter it

cannot be made available simultaneously to a NIC Async adapter driver. This

type of protocol requires the use of NIC Async with NIC Bridge, bridging via

a dedicated network adapter.

��������������

Serial Ports

��������������

In "Direct Port" mode NAsync allows the use of COM1 - COM8. The

default values for I/O address and IRQ level for these ports are shown below.

The MCA values shown are the IBM standards for PS/2s. The ISA values shown

are the IBM standards for PCs, XTs and ATs for COM1-COM2 while COM3-COM4 are

defacto industry standards.

Note that the sharing of interrupts is only supported on MCA machines

running under OS/2.

ComPort:           COM1  COM2  COM3  COM4  COM5  COM6  COM7  COM8

ISA I/O address     3F8   2F8   3E8   2E8   1E8   1E0   280   288

IRQ level         4     3     4     3     2     5     2     5

MCA I/O address     3F8   2F8  3220  3228  4220  4228  5220  5228

IRQ level         4     3     3     3     3     3     3     3

Non-standard serial ports may be accommodated in "Direct Port" mode by

adding the following parameters to the [NASYNC_NIF] section of Protocol.Ini

AFTER the COMM_PORTNAME section:

; Range: 0x0 - 0xFFF8 where low 3 bits = 0

COMM_IOADDRESS  = 0x3F8

; Range: 2 - 0x0F

COMM_INTERRUPT  = 4

The baudrate values accepted by the NAsync driver are:

300, 600, 1200, 2400, 4800, 9600, 19200, 28800, 38400, 57600 and 115200.

All of these baudrates are valid in the "Direct Port" mode of operation.

Use of the NaComa OS/2 Comport Agent however, limits valid baudrates

to those supported by the OS/2 Com driver. The standard OS/2 Com drivers

usually limit the maximum baudrate to 19200 baud. The standard OS/2 2.0 Com

driver allows a maximum baudrate of 57600 baud on 16550A equipped serial ports

only. Regular serial ports are still limited to 19200 baud.

The use of serial ports which use the 16550A buffered UART chip is

highly recommended, particularly at baudrates of 9600 and up in the DOS or

Windows environment. These buffered UART chips can greatly reduce or eliminate

the loss of characters due to "serial overrun".

Serial overrun occurs when interrupts are disabled for too long and

a new character of data is received before the previous character has been

processed. The MS Windows environment, EMM386.Exe, disk caches, TSRs, network

software, BIOS routines and DOS itself all contribute to the problem by

disabling interrupts for long periods of time. The use of high baudrates

aggravates this problem. At 9600 baud even the short period of 1 milli-seconds

of disabled interrupts can cause problems.

When characters are lost due to serial overrun, network packets have

to be re-transmitted. This causes greatly reduced performance at best or

network timeout errors and disrupted operations at worst.

The 16550A UART chip provides a 16 character receive buffer to reduce

the incidence of this problem. It effectively allows interrupts to be disabled

for 16 times as long as the more common 8250 and 16450 UART chips before

losing characters to serial overrun.

In addition to reducing the incidence of serial overrun errors, the

16550A UART can reduce the CPU load caused by high speed communications. The

chip can be configured to generate an interrupt when 1, 4, 8, or 14 characters

are waiting in the receive buffer. This allows a tradeoff to be made between

safety from serial overrun errors and the average overhead to receive each

character. The 16550A chip also provides a 16 byte transmit buffer which

allows up to 16 bytes to be transmitted during a single interrupt, rather than

16 seperate interrupts.

One potential problem exists when using the 16550A on PC and AT type

PCs. Most BIOS's will not be able to detect a serial port which uses this chip

when it's buffers are enabled. This problem is most evident on AT type PCs

running OS/2 1.x where on a warm boot the COM01.Sys driver is unable to detect

the existence of the port. One solution for this under OS/2 requires the use

of the NaComA.Exe OS/2 Comport Agent. When a "NA R" command is issued for a

NIC Async port, the buffers are disabled before the Agent terminates. This

command prior to a warm boot resolves the problem for ports used by NIC Async.

For other ports the command "MODE COMx:1200,,,,Buffer=Off" is equivalent.

Most PS/2's are factory equipped with the 16550A chip. Some early

models used the earlier bug-ridden 16550 which should be avoided.

The 16550A chip can be easily retro-fitted on most serial ports which

provide a 40 pin socket for the UART. It is available from a number of vendors

including Jameco Electronics and JDR Microdevices, for prices in the range of

$10.00 - $30.00. Both of these vendor also offer serial adapters which can be

fitted with these chips.

��������

Modems

��������

NIC Async is designed to be modem independent, but it does require

some modem parameters to be configured. This configuration should be done in

the NIC Async modem initialization string.

The modem initialization string is set with the Protocol.Ini parameter

MODEM_INIT in the [NASYNC_NIF] section. This string is sent to the modem

during the NDIS NetBIND and each time an outgoing call is terminated.

The default of MODEM_INIT = "AT&FQ0V1&C1&D2&W" is generic and should

be accepted by virtually any Hayes compatible modem. Non-compressing modems of

2400 baud and lower should work with this init string. It should however be

customized for most modern modems. Note that any added parameters should be

inserted before the final "&W" option.

The following pages explain the contents of the modem init string,

NIC Async requirements and recommended modifications to the init string.

MODEM_INIT = "AT&FQ0V1&C1&D2&W"

Q0    - Turns on modem result codes.

V1    - Enables verbose mode of modem result reporting.

&C1   - Configures modem to assert "Carrier Detect" (CD)

signal only when carrier is present.

&D2   - Configures the modem to hangup when "Data Terminal Ready"

(DTR) signal is dropped.

&W    - Writes the new parameters to the modem.

Recommend option additions to the default modem init string:

- Turn on RTS/CTS flow control.

Turn off all other forms of flow control.

- Turn off all error control and compression.

Note: NIC Async has been successfully tested with NetBEUI

using V.42 error control. V.42bis compression however,

has caused timing problems for NetBEUI.

For initial setup with any protocol it is recommended

that all error control and compression be disabled.

- Disable baudrate switching / Enable baudrate locking.

Note: Some modems, usually high speed modems, always

maintain a constant baudrate from PC to modem.

Other modems, usually low speed Hayes compatible

modems, always change the PC to modem baudrate to

match the carrier speed.

To accommodate both modem types the parameter

MODEM_LOCKBAUD is provided. The default value

"YES" is for the former, while "NO" for the latter.

Further, when connecting, NIC Async enters a

"Verifying Data Link" state. It transmits test

packets and waits for a response from the remote.

If no response is received, the host and client

modify their MODEM_LOCKBAUD state at intervals

in an attempt to acheive a verified data link.

An extended "Verifying Data Link" state indicates

that MODEM_LOCKBAUD should be changed on the host,

client or both.

Some representative modem init strings are shown below.

Refer to your modem documentation for definitive information on

the options your modem supports.

; Hayes Compatible w/ Compression or Telebit

; &K3 enables RTS/CTS flow control

; &Q0 disables error control and compression

MODEM_INIT = "AT&FQ0V1&C1&D2 &K3&Q0 &W"

; Microcom

; \Q3 enables RTS/CTS flow control

; \N0 disables error control and compression

MODEM_INIT = "AT&FQ0V1&C1&D2 \Q3\N0 &W"

; US Robotics V.32

; &B1 enables baudrate locking

; &H1&I0&R2 enables RTS/CTS flow control

; &K0&M0 disables error control and compression

MODEM_INIT = "AT&FQ0V1&C1&D2 &B1 &H1&I0&R2 &K0&M0 &W"

NOTE:   Many modems will not accept a command of greater than

40 characters. The "AT", spaces and the terminating

carriage return are not included in this count.

If, after sending the modem init string, NIC Async detects an active

CTS signal, it will use "Request to Send"/"Clear to Send" (RTS/CTS) flow

control. This is the most widely supported hardware flow control method and is

essential for high speeds. It allows the PC & modem to communicate at high

baudrates even when the modem to modem speed is lower.

When the modem is ready to accept data it should assert CTS. When the

modem buffer is full it should de-assert or turn off CTS. NIC Async controls

RTS in the same way to inform the modem when it is ready to receive data.

If your modem supports RTS/CTS flow control, add the appropriate

options to MODEM_INIT. This can generally be done with "&K3" on Hayes and

Telebit modems. Modems such as those from Microcom may require "\Q3".

US Robotics modems have worked well with "&H1&I0&R2".

NIC Async asserts the signal "Data Terminal Ready" (DTR) when

answering, initiating or maintaining a call. This signal is dropped when

the call is to be disconnected. Modems should be configured to hang up when

DTR is dropped. The default option "&D2" will normally do this.

Detection of either the "Ring Indicator" or "Carrier Detect" signal

(RI or CD) will initiate the answering of a call. NIC Async then sends the

MODEM_ANSWER string, which is by default "ATA". It will then wait for the

"Carrier Detect" signal to become active. Modems should be configured to

assert CD only when carrier is detected. The default option "&C1" will

normally do this.

If, for some reason, NIC Async does not detect incoming calls, set

"S0=1" in MODEM_INIT. The command "NA A" may also be used to manually answer

for troubleshooting.

For initial setup and testing, it is recommended that all modem error

control and compression be disabled. Adding &Q0 to the MODEM_INIT string will

often do this on Hayes and Telebit modems. Microcom modems may require \N0

while US Robotics modems may require &K0&M0.

NIC Async has been successfully tested with NetBEUI using V.42 error

control, but not with V.42bis compression. Compression causes NetBEUI to

suffer from timeouts due to buffering. If your modem supports error control

and/or compression you may wish to enable these features one at a time after

ensuring that your configuration is working.

��������������������������Ŀ

� SETUP/MAIN CONFIGURATION �

����������������������������

��������������������

MS Lan Manager 2.x

��������������������

1)      Install Lan Manager if it is not already installed. If this PC will

not use a local network adapter, do one of the following.

1a)     Lan Manager 2.0:

- Select "OK" upon reaching the "Network Drivers" screen.

Do not select "Add driver". Disregard the warning that

Lan Manager will not operate properly.

1b)     Lan Manager 2.1:

- Specify "No Driver" from the bottom of the list when prompted to

"Select Available Network Adapter Driver".

2)      Install NIC Bridge/Async on the PC. With the NIC Bridge/Async Install

Disk inserted into the A: drive the command "A:\Install" will initiate

the installation procedure. When this is complete the PC should be

rebooted.

3)      Run NicSetup.Exe and ensure that the General Configuration information

displayed is appropriate. Option 2 "Change General Configuration" may

be used to make any changes.

Select option 5 "Create MS Lan Manager Driver Import Disk" of the

NicSetup.Exe utility. This requires a floppy disk with at least

600 KBytes of free space.

4)      Import the NIC Async driver by doing one of the following.

4a)     Lan Manager 2.0:

- Run the Lan Manager Setup.Exe utility. From the main Setup menu

select "Actions", "Import Network Drivers". Insert the Import

Disk into a floppy drive and when prompted select the "NIC Async"

driver.

4b)     Lan Manager 2.1:

- Run the Lan Manager Setup.Exe utility. From the main Setup menu

select "Configuration", "Network Drivers", "Add New Config",

"Other Driver". Insert the Import Disk into a floppy drive and

when prompted select the "NIC Async" driver. Select "Cancel" until

the main Setup menu reappears.

5)      Ensure that all local Ethernet and Token Ring adapters have been

configured before doing one of the following.

5a)     Lan Manager 2.0:

- From the main Setup menu select "Actions", "View/Modify" and

advance to the "Network Drivers" screen. Highlight "NIC Async" in

the "Network adapter cards" box and select "Add driver". Highlight

a protocol in the "Protocols" box and select "Add Protocol" for

all protocols to be used remotely. Repeat the "Add driver" and

"Add Protocol" selections for each serial port to be used. When

finished select "OK" and advance through the remaining screens.

Select "Actions", "Save" from the main Setup menu. Setup may now

be exited with the <F3> key.

The following page shows the "Network Drivers" screen for a

configuration with one Ethernet adapter and two async ports.

Note that local adapter drivers should be listed first in the

"Network configuration" box.

LM 2.0 Setup Network Drivers Screen:

���������������������������� Network Drivers ��������������������������Ŀ

� Network configuration:                                                �

�  ����������������������������������������������������������������Ŀ   �

�  �#3Com Etherlink II Adapter.................NetBEUI              �   �

�  � NIC Async.................................NetBEUI              �   �

�  � NIC Async.................................NetBEUI              �   �

�  �                                                                �   �

�  �                                                                �   �

�  ������������������������������������������������������������������   �

� Network adapter cards:                    Protocols:                  �

�  ���������������������������������������Ŀ ���������������������Ŀ    �

�  �#3Com Etherlink II Adapter             � �#NetBEUI             �    �

�  � IBM Token Ring Adapter                � � NIC Bridge          �    �

�  � NIC Async                             � �                     �    �

�  � SRTok                                 � �                     �    �

�  ����������������������������������������� �����������������������    �

�����������������������������������������������������������������������ĳ

� < OK >  <Add driver>  <Add Protocol>  <Remove>  < Done >  <Cancel>    �

�������������������������������������������������������������������������

5b)     Lan Manager 2.1:

- From the main Setup menu select "Configuration","Network Drivers",

"Add New Config". Highlight "NIC Async" and select "OK". Select

the protocols to be used remotely. Repeat the "Add New Config"

selection for each serial port to be used. When finished select

"OK" and "Save".

Setup may now be exited with the <F3> key.

The following page shows the "Workstation Configuration" screen

for a configuration with one Ethernet adapter and two async ports.

Note that local adapter drivers should be listed first in the

"Installed configurations" box.

LM 2.1 Setup Workstation Configuration Screen:

�������������������� Workstation Configuration ������������������Ŀ

� Installed Configuration(s):                                     �

�  �����������������������������������������������������������Ŀ  �

�  �#3Com Etherlink II Adapter                                 �  �

�  �     Netbeui                                               �  �

�  � NIC Async                                                 �  �

�  �     Netbeui                                               �  �

�  � NIC Async                                                 �  �

�  �     Netbeui                                               �  �

�  �                                                           �  �

�  �                                                           �  �

�  �                                                           �  �

�  �                                                           �  �

�  �������������������������������������������������������������  �

�����������������������������������������������������������������ĳ

�   OK    <Add/Remove Protocols>  <Add New Config>                �

� <Remove Config>  <Cancel>  <Help>                               �

�������������������������������������������������������������������

6)      Before rebooting the PC, the Protocol.Ini file may need modification.

This can be done with the NicCfg.Exe utility. Within NicCfg select

"Config", "Async 1". When each screen of options is appropriate press

the <Enter> key. Repeat for "Async 2" etc. for each serial port to be

used.

The MAC Type option should be set to Ethernet for all cases except

when NIC Async is being used with NIC Bridge in source routing mode.

The Network address value should be unique among all network addresses

in use on the network. Remember to set a DIFFERENT Network address on

the host and all clients.

Other options may be left at default or changed as appropriate for

your environment. Exit NicCfg with the <F3> key.

7)      Run NicSetup.Exe again and select option 4 to tune NetBEUI and

the Lan software for low speed operation.

����������������

IBM Lan Server

����������������

1)      Install Lan Server if it is not already installed.

2)      Install NIC Bridge/Async on the PC. With the NIC Bridge/Async Install

Disk inserted into the A: drive the command "A:\Install" will initiate

the installation procedure. When this is complete the PC should be

rebooted.

3)      Run NicSetup.Exe and ensure that the General Configuration information

displayed is appropriate. Option 2 "Change General Configuration" may

be used to make any changes.

4)      Run the Lan Server LanInst utility. Install the NAsync driver in

C:\NIC11 as you would any other NDIS ethernet driver. The "NIF" file

C:\NIC11\NAsync.Nif can provide Lan Server with the information needed

to set NAsync parameters via the "Configure LAN Tranport" option.

Be sure to enable all adapters for use with Lan Server.

5)      Before rebooting the PC, the Protocol.Ini file may need modification.

This can be done with the NicCfg.Exe utility. Within NicCfg select

"Config", "Async 1". When each screen of options is appropriate press

the <Enter> key. Repeat for "Async 2" etc. for each serial port to be

used.

The MAC Type option should be set to Ethernet for all cases except

when NIC Async is being used with NIC Bridge in source routing mode.

The Network address value should be unique among all network addresses

in use on the network.

Other options may be left at default or changed as appropriate for

your environment. Exit NicCfg with the <F3> key.

6)      Run NicSetup.Exe again and select option 4 to tune NetBEUI and

the Lan software for low speed operation.

�����������Ŀ

� OPERATION �

�������������

The NIC Async host should start and run normally. The Device driver(s)

should load from Config.Sys and display ID messages. Note that should a driver

cause problems booting a PC, installation of the driver may be aborted by

holding down the Alt-Key.

The NIC Async client should also start and run normally. After

the netbind has been done you can dial with "NA D". This should show a final

status of Open. The workstation service may be started and you should now

be able to login and proceed normally. Use "NA H" to terminate the session.

Note that the workstation service may actually be started before the

remote connection is established. MS Lan Manager and IBM Lan Server 3.0 will

also allow an unvalidated logon before dialing.

"NA C+" and "NA C-" enable and disable software compression, while

"NA E+" and "NA E-" enable and disable encryption. Both of these have been

found usable to about 57,600 baud on a 25 Mhz 386 DX, however this DOES use

up virtually all of the CPU power. This may not be a problem for a remote

DOS PC but local users of a network server will probably not appreciate the

loss of response time. If you intend to make use of compression or encryption

it is strongly recommended that you test your intended configuration to ensure

that the results are acceptable to you and your users.

The compression is optimized for "Byte Redundant" data such as text

and typical network control traffic. NIC Async does not offer the compression

ratios you can get with utilities such as PKZip. It is designed to compress

data per packet with as little overhead as possible. If you intend to transfer

already compressed files and compression is currently enabled, you can use

"NA C-" to turn compression off while connected. This sends a message to the

remote NIC Async PC to ensure compression is disabled in both directions.

Modem compression will do nothing for you if software compression

or encryption are turned on. This is due to the fact that both compression

and encryption essentially randomize the data, preventing the modem from

doing any useful compression.

The software compression in NIC Async is "smart" in that it won't

expand data & slow down as happens with MNP. There is, however, a higher load

on the PC attempting to compress. In general, unless encryption is needed, if

modem compression is available, it probably makes more sense to use it.

Encryption performed by NIC Async is basic "Data Encryption Standard"

(DES) in "Electronic Code Book" (ECB) mode. The 8 byte passwords available are

mapped to the 56 bit DES Keys. Basic ECB mode DES seems to be suitable for

most commercial work. Turning compression on with encryption increases the

"randomness" of the data and thus offers further protection.

�����������������Ŀ

� TROUBLESHOOTING �

�������������������

�������

Traps

�������

It is expected that the majority of problems will be with either

protocol tuning or modem setups. Regardless, you may have a problem at some

point with an OS/2 Trap or similiar nasty bug. OS/2 device drivers are much

more likely than applications to bring OS/2 to a halt.

This software has been extensively torture-tested in many different

configurations at 115200 baud and lower. No unexplained traps or mysterious

failures have been observed in quite some time. Given the nature of software,

however, it's quite possible that you will experience something like this.

If you get any traps under OS/2 (or DOS, via Qemm for example) please

write down the register dump you receive. Unlike many OS/2 drivers, a system

trap in NAsync.OS2 will usually identify NAsync.OS2 by name. If a register

dump can be provided, the problem causing it can most likely be fixed.

����������������������

Lan Manager Problems

����������������������

Note that the DOS NetBEUI T1 parameter is reported by MS to be broken

in LM 2.0B through to LM 2.0C patch level 8 in such a way as to make slow

links impossible to use. This hasn't, however, been observed in testing.

Microsoft TCP/IP has been found to be incompatible with the DOS

Lanman.Ini WRKHEURISTICS tuning. Specifically, a DOS workstation running

MS TCP/IP will give "Incorrect Network Response" errors when both raw I/O

and opportunistic locking are disabled (WRKHEURISTICS bit 8=0 and bit14=1).

The solution in this case is to enable raw I/O by setting bit 14 to 0.

����������������

Modem Problems

����������������

The Na.Exe utility offers a VS option to view the NIC Async serial

buffers. You can use this to see the commands NIC Async sends to the modem

and the modem responses.

Sample output from the command "NA VS 48" is shown below. The second

parameter 48 specifies how many buffer bytes to display and may range from 16

to 4096. This example shows the modem initialization string and result.

Transmit Buffer:

2B 2B 2B 41 54 48 0D 41 54 26 46 51 30 56 31 26    +++ATH.AT&FQ0V1&

43 31 26 44 32 26 57 0D 00 00 00 00 00 00 00 00    C1&D2&W.........

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00    ................

Receive Buffer:

2B 2B 2B 41 54 48 0D 0D 0A 4F 4B 0D 0A 41 54 26    +++ATH...OK..AT&

46 51 30 56 31 26 43 31 26 44 32 26 57 0D 0D 0A    FQ0V1&C1&D2&W...

4F 4B 0D 0A 00 00 00 00 00 00 00 00 00 00 00 00    OK..............

Na.Exe also allows changing the modem initialization string from the

command line. If the new modem init string is set while the driver is in the

"Closed" state, the new string will be sent to the modem immediately. The

command to do this is in the form:   NA Q "AT&FQ0V1&C1&D2 &W".

All statistics and both serial buffers can be cleared with "NA VC".

This is useful when changing the modem initialization string to get a clearer

picture of how the modem has responded. For example the sequence:

NA VC

NA Q  "AT&D255"

NA VS 48

Produces the output:

Transmit Buffer:

2B 2B 2B 41 54 48 0D 41 54 26 44 32 35 35 0D 00    +++ATH.AT&D255..

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00    ................

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00    ................

Receive Buffer:

0D 0A 4F 4B 0D 0A 2B 2B 2B 41 54 48 0D 0D 0A 4F    ..OK..+++ATH...O

4B 0D 0A 41 54 26 44 32 35 35 0D 0D 0A 45 52 52    K..AT&D255...ERR

4F 52 0D 0A 00 00 00 00 00 00 00 00 00 00 00 00    OR..............

�������������������Ŀ

� NASYNC PARAMETERS �

���������������������

The following pages detail Protocol.Ini parameters for the NAsync

driver. Defaults are shown. Parameter values shown in quotes are ASCII string

parameters. All other parameters are numeric. Numeric parameters preceded with

"0x" are hexadecimal values while all others are decimal values.

REQUIRED NDIS PARAMETERS:

; First driver is "NASYNC$", second is "NASYNC2$", etc.

; Range: NASYNC$ - NASYNC9$

; Only required parameter

DRIVERNAME      = "NASYNC$"

MAC PARAMETERS:

; Sets network address of simulated adapter

; MUST be different for each driver on each PC

; Range: "000000000000" - "FFFFFFFFFFFF"

; MUST be exactly 12 hex digits

MAC_NETADDRESS  = "123456789ABC"

; Sets Ethernet or Token Ring emulation

; Range: "DIX+802.3" or "802.5"

MAC_TYPE        = "DIX+802.3"

; Maximum number of queued transmit requests

; Range: 1 - 50

MAC_MAXXMITS    = 8

; Changes reported IRQ Level

; Included only for 3Com XNS which intercepts this IRQ

;  and can cause overruns; set to 0xFF for 3Com XNS

; Range: 0x00 - 0xFF

MAC_REPORTIRQ   = Interrupt for this port (4 for COM1)

COMM PARAMETERS:

; Communications port name

; Range: 4 - 64 characters, "COM1"-"COM8" in Direct Port Mode

COMM_PORTNAME   = "COM1"

; Communications port baudrate

; Range: 300/600/1200/2400/4800/9600/19200/38400/57600/115200

COMM_BAUDRATE   = 9600

; Communications port I/O Address for non-standard ports

; MUST appear after COMM_PORTNAME if present

; Range: 0x0 - 0xFFF8 where low 3 bits = 0

COMM_IOADDRESS  = 0x3F8 (For COM1)

; Communications port IRQ Level for non-standard ports

; MUST appear after COMM_PORTNAME if present

; Range: 2 - 0x0F

COMM_INTERRUPT  = 4     (For COM1)

; Modifies 16550A Uart detection

; Not normally needed

; Range: "AUTO", "YES" or "NO"

COMM_16550A     = "AUTO"

; Controls removal of BIOS information about serial port

; Range: "YES" or "NO"

COMM_RBIOSINFO  = "YES"

; Sets 16550A Rx Trigger Level

; High values minimize CPU usage,

;  low values minimize serial overrun errors

; Range: 1, 4, 8, 14 or 0 for AUTO

COMM_RXTRIGGER  = 0 (For OS/2)

COMM_RXTRIGGER  = 1 (For DOS)

; Enables OS/2 File System mode

; Range: "YES" or "NO"

COMM_FSMODE     = "NO"

MODEM PARAMETERS:

; Sets Hayes or Null modem type

; Range: "HAYES" or "NULL"

MODEM_TYPE      = "HAYES"

; Modem initialization string

; Range: 0 - 50 characters

MODEM_INIT      = "AT&FQ0V1&C1&D2&W"

; Modem dial string

; Range: 0 - 50 characters

MODEM_DIAL      = "ATD"

; Modem answer string

; Range: 0 - 50 characters

MODEM_ANSWER    = "ATA"

; Modem connect detection string

; Range: 0 - 50 characters

MODEM_CONNECT   = "CONNECT"

; Modem command terminator, 0x0D = Carriage Return

; Range: 0x00 - 0xFF

MODEM_TERM      = 0x0D

; Sets baudrate locking (YES) or baudrate switching (NO)

; Range: "YES" or "NO"

MODEM_LOCKBAUD  = "YES"

TIMING PARAMETERS:

; Inactivity hangup timeout

; Range: 0 - 30 Minutes, default 0 = never

TIME_INACTTMO   = 0

; Time to wait for Carrier after dialing

; Range: 0 - 2000 Seconds

TIME_WTCARROUT  = 60

; Time to wait for Carrier when answering

; Range: 0 - 2000 Seconds

TIME_WTCARRIN   = 60

CHARACTERISTICS PARAMETERS:

; Enables automatic dial/redial on packet transmission

; Range: "YES" or "NO"

CHARS_AUTOCNCT  = "NO"

; Enables encryption as initial state

; Range: "YES" or "NO"

CHARS_ENCRYPT   = "NO"

; Enables compression as initial state

; Range: "YES" or "NO"

CHARS_COMPRESS  = "NO"

; Enables answer on dialin

; Range: "YES" or "NO"

CHARS_DIALIN    = "YES"

; Password caller must supply when answering

; Range: 1 - 8 chars

CHARS_PWORDIN   = "PASSWORD"

; Default outgoing password when calling

; Range: 1 - 8 chars

CHARS_PWORDOUT  = "PASSWORD"

; Encryption/Decryption Key

; Range: 1 - 8 chars

CHARS_ENCRKEY   = "PASSWORD"

; Enables password check when answering

; Range: "YES" or "NO"

CHARS_CHECKPW   = "YES"

; Default phone number to dial

; Range: 0 - 50 chars

CHARS_PHONE     = "P555-1212"

; Enables filtering of transmitted broadcasts

; Use only on servers without NIC Bridge

; Range: "YES" or "NO"

; Note: "YES" does not allow correct operation of MS TCP/IP

CHARS_FILTERBC  = "NO"

������������������Ŀ

� ACKNOWLEDGEMENTS �

��������������������

All trademarks are the property of their owners.

This includes, but is not limited to, the trademarks

of 3Com, IBM, Microsoft, and Hayes.

;