JETSET OR 747WORLD MANUAL

The documentaion which follows was originally published in the

November, 1982 issue (v7,n11) pp. 272 of BYTE Magazine.  It is

reproduced here (with slight modification to account for figures not

reproducible here) so that those of you who no longer can locate this

issue may enjoy this excellent program... Russ McCallister, P.O.Box

79, River Forest, Illinois 60305 - September 1983 747 or JETSET as it

was named by the author offers the adventure of piloting a jet

aircraft minus the jet lag and the risk.  The program name JETSET  is

an acronym for the Jet Simulator  Electronic Trainer. You will

manoeuver an aircraft through the three stages of flight--take-off,

cruising, and landing--in less than ideal conditions. The program

originally written for the TRS-80 Model II, uses the keyboard and

screen to make a personal computer version of a commercial flight

simulator.  To make JETSET or 747WORLD a realistic simulation,

everything the pilot does in this program must be coordinated with an

instrument panel displayed on the computer screen.  In addition, the

pilot must follow the actual procedures required when flying in

near-zero visibility.  A plane flown in such inclement weather must

proceed according to Instrument Flight Rules (IFR) established by

government, and the pilot must be specially trained and certified to

fly ON INSTRUMENTS.  This information is incorporated into the JETSET

or 747WORLD program.

NOTE: The following modifications have been made by Jean-Pierre

Bernier in 1988, 1990 and 1992 and are included in 747WORLD:

-Integration of take-off and cruise.

-Possibility to fly all over the world except near the poles.

-Take-off from many airports.

-Possibility of changing altitude, latitude and longitude any

time by typing C except during take-off.

-After missed approach resume flying.

-Random speeds of winds at take-off, cruise and landing.

-Stall conditions in cruise.

-After touch-down use of the brakes (after Rev. Thrust).

-Informations on winds.

-Above 10000 feet decrease of the rate of climb.

-It is not necessary to reset the clock at 7:59 am. This was

provided because the basic fonction TIMER resets the elapsed

time in seconds at midnight.

With a PC running at 4.77 MHz it is very difficult to enjoy flying

with the 747WORLD.BAS program.  The uptake delay is 4 seconds. The

compiled version 747WORLD.EXE is necessary and the uptake delay is

less than 1 second. The following text is a modification of the

original one with additions and some changes.

COMPUTER SIMULATED FLIGHT

The JETSET or 747WORLD Program lets the pilot activate the control

surfaces of the jet aircraft, adjust engine thrust, and tune

navigational radio equipment by pressing a set of keys. (See Table 1.)

The program responds to the keypress commands by adjusting aircraft

attitude to match the control surfaces and updating the instrument

panel display in less than one second as the trajectory of the

jetliner is tracked through space by the computer.

The jet instrument panel gives the pilot all the flight information he

needs to take off, navigate, and land an aircraft using standard

flight procedures and the radio facilities established for modern-day

flying.  The panel functions reveal what the aircraft is doing and

where it is located, so that after a short period of training the

pilot knows instinctively how to scan and interpret the panel data.

Position tracking, a vital ingredient in the simulation, is performed

in real time to keep the flight situation up to date.  Although the

pilot completely controls the motion of the jet, wind forces that vary

with altitude can influence the flight.  The program uses an

analytical combination of jet and wind motion to solve the "wind

triangle" that is formed whenever an aircraft is aloft and moving

through layers of air.  The wind-triangle solution yields the "true"

motion of the jet relative to the earth's surface.

When the simulation begins, the jetliner is poised for take-off on the

runway of the airport choosen on the starting menu.  The geographic

coordinates of the VOR of the choosen airport mark the starting point

of flight.  The computer fixes this initial position in memory and

cranks out a new longitude and latitude about 60 times a minute.  The

pilot controls the path of the jet during the take-off roll down the

runway because there is a wind drift when the speed of the jetliner is

above 50 knots.  If everything is done correctly in the cockpit, this

path will lead to a take-off with room to spare.

Once airborne, the jet is tracked against a grid of meridians and

parallels, an involved computation that requires the program to used

spherical trigonometry because of the earth's curved surfgace. Because

the geographic coordinates of airports and radio beacons are stored in

the computer's memory, a comparison of positions yields the

information needed to update the insrument panel the pilot uses to

navigate.

An instrument landing, the trickiest part of any actual flight, is

also the most complex operation for the computer to simulate.  This

type of landing requires a programmed geometry to simulate the

Instrument Landing System (ILS) pattern formed by special radio beams.

These beams, which converge at the landing end of a runway, deflect an

indicator on the instrument panel of the landing jet and give the

pilot an exact path to follow during the final approach to the

airport.

Because JETSET or 747WORLD knows precisely where the pilot is telling

the plane to go, the program will continue to run until the jet lands

safely and rolls to a halt or until the flight ends in disaster. When

the simulation has ended, for whatever reason, JETSET or 747WORLD

provides a complete report of the pilot's performance. The report

includes the landing location of the plane-whether on or off the

runway-to the nearest foot, and in case of pilot error a description

of the error and the likey damage to the aircraft.

TABLE 1

KEYBOARD KEYS, DEFINITIONS AND FONCTIONS

----------------------------------------------------------------------

KEY     DEFINITION                FUNCTION

----------------------------------------------------------------------

F    THRUST INCREASE*    INCREASES POWER TO JET ENGINES.

S    THRUST DECREASE*    DECREASES POWER TO JET ENGINES.

Q    THRUST REVERSE      REVERSES ENGINE THRUST AFTER TOUCH-DOWN.

D    PITCH DOWN*         LOWERS NOSE OF AIRCRAFT (DEGREES).

U    PITCH UP*           LIFTS NOSE OF AIRCRAFT (DEGREES).

\    PITCH CANCEL        IN CRUISE SETS NOSE TO LEVEL FLIGHT.

\    FLARE UP            IN ILS:TO BE USED WHEN RADAR READS 50 FEET.

, <  RUDDER LEFT*        INCREASES RUDDER LEFT BY ONE INCREMENT.

. >  RUDDER RIGHT*       INCREASES RUDDER RIGHT BY ONE INCREMENT.

/    RUDDER CANCEL       IN CRUISE RETURNS RUDDER TO CENTER POSITION.

L    FLAPS               RAISES AND LOWERS WING FLAPS.

W    WHEELS              RAISES AND LOWERS LANDING GEAR.

B    BREAKS              RELEASES WHEEL BRAKES FOR TAKE-OFF.

AFTER LANDING TO APPLY OR RELEASE BRAKES TO

REDUCE THE SPEED ON THE RUNWAY.

M    MISSED APPROACH     SIGNALS AN ABORTED ILS LANDING ATTEMPT.

THE PROGRAM RETURNS TO VOR. TO CONTINUE IT

IS NECESSARY TO APPLY POWER AND TO CLIMB.

V    VOR FREQUENCY TUNE  INPUTS A FREQUENCY TO VOR RECEIVER.

R    VOR RADIAL SELECT   SELECTS A RADIAL VALUE FOR NAVIGATING .

A    VOR AUTO SELECT     AUTOMATICALLY ROTATES RADIAL SELECTOR DIAL.

C    IN CRUISE CHANGES   ALTITUDE, LATITUDE AND LONGITUDE.

NOTES:

1.�The CAPS LOCK key must be engaged throughout the simulation.

2.�Keys < and > are in fact , and . without SHIFT.

3.�During take-off keys L B F < > U W S are used.

4. During take-off < > control the nose wheel when the airspeed

exceeds 50 knots. Key / has no effect.

5. An asterisk (*) identifies keys that may be typed additional

times to increase their control function.

6.�In ILS, after touch down, it is necessary to apply reverse

power ( Q key ) to reduce the speed. The brake ( B key ) can

also be used.

TABLE 2

INSTRUMENT PANEL LEGEND

------------------------------------------------------------------------

INSTRUMENT    UNITS              FUNCTION

------------------------------------------------------------------------

FUEL        pounds, %      fuel aboard (in pounds and percentage full).

VHF         MHz            communications channel.

THRUST                     position of engine thrust levers.

PITCH                      attitude of aircraft.

DEG         degrees        angle of pitch, measured from horizontal.

COMPASS     degrees        compass heading of aircraft (direction of

nose).

AIRSPEED    knots          aircraft velocity through the air.

VERT SPEED  feet/minute    rate of climb or descent.

ALTITUDE    feet           altitude above the ground.

CLOCK       hr.min.sec     time of day (local).

VLF OMEGA   degrees,min    aircraft position (latitude and longitude).

RUD                        rudder angle.

FLAPS                      flaps position.

WHEELS                     landing gear position.

BRAKE                      position of wheel brakes.

VOR         MHz            frequency to which VOR receiver is tuned.

RANGE                      status of VOR receiver.

RADIAL      degrees        value of selected radial (needle moves along

window directly above radial).

DME         nautical miles distance to VOR ground station.

RADAR ALT   feet           aircraft elevation during final approach.

MARKER                     turns on when flying directly over the ILS

outer and middle marker beacons.

ILS                        pair of needles that deflect according to

aircraft position in ILS radio cone.

STALL       indicator      on with sound on a stall condition in cruise,

also in ILS when the plane touches the

runway. It signals the time to apply reverse

thrust with the Q key (this turns the

indicator and sound off).

NOTES:

-When in ILS it is not possible to return to VOR unless we do

a missed approach (any time in ILS).

-The program permits stall conditions with the flaps down. In

a stall the the airspeed is too low, then the nose drops and

the plane looses altitude. The best action is to increase

power with the F key and to level up. If with the nose down

the plane gains enough airspeed, levelling up may be

satisfactory.

TAKING OFF

When you call JETSET or 747WORLD a list of airports is displayed. After

choosing the airport, a message authorizing a take-off is displayed.

The pilot aknowledges by pressing F9. Random numbers are generated for

the speeds of winds on the runway, in altitudes and later during

landing. Let choose PHILADELPHIA to take off. The screen will then

display the upper section of the jet instrument panel and a

perspective view of the runway as it would appear from the cockpit. At

this point the jet is parked in the take-off position with its engines

idling, ready to go when its brakes are released.

To prepare for take-off, lower the flaps (L key) and check the panel

FLAP indicator.  A down position shows that the wing flaps are now

extended. The flaps provide the vital extra lift needed during landing

and take-off, when the jet airspeed is marginal.  Next, release the

wheel brakes (B key). The jet will begin to move slowly because the

engines are idling at only a fraction of their rated power or thrust.

To apply full take-off power, press the "F" key and watch the THRUST

lever indicator move to its maximum forward position.  The program

will now apply acceleration to gradually bring the jet up to its rated

take-off speed, 150 knots (173 mph).

As momentum builds, the AIRSPEED  indicator begins to register. The

jet begins its take-off roll down the 10,500 foot runway.  Soon

afterward, the COMPASS  indicator begins to deflect from its 075

degree reading as the jet is hammered by gusts of wind sweeping across

the runway.  This is a busy time in the cockpit because you must

carefully steer the jet along the 200-foot-wide runway strip as you

come up to take-off speed.  A sliding arrow at the base of the runway

graphic shows how far the jet is wandering from the runway centerline.

Use the rudder keys (< and >) to steer the jet via its nose wheel

whenever this arrow veers away from the center position.  The arrow

will shift left or right whenever the compass reading deviates from

the 075 degree direction of the runway.  Careful steering, then is an

exercise in coordinating both keys with the compass reading and the

runway graphic (each press of a rudder key alters the direction of

travel by one degree).

Assuming that the jet doesn't veer off the runway (which would end the

flight), you must be ready to execute the lift-off manoeuver when the

airspeed indicator reaches 150 knots, at which point you press the "U"

key once, and once only, to tilt the nose up 10 degrees.  The jet will

lift off just before the end of the runway moves to the bottom of the

screen with the horizon line.

Immediately following the lift-off, you must execute a three-step

sequence to gain altitude promptly:

1. raise the landng gear (W key) to reduce "drag" (air friction).

2. retract the wing flaps (L key).

3. reduce the thrust (S key) to attenuate engine noise- in

accordance with federal antinoise regulations-as the jet passes

over metropolitan Philadelphia.

You must perform this sequence in the above order because the three

keys are software-interlocked.  In addition, you must complete the

three steps before the ALTITUDE indicator reads 1200 feet.  If you do

everything correctly, the screen will erase to indicate a successful

take-off and a display of the complete instrument panel will appear.

Takeoff Mishaps

JETSET or 747WORLD doesn't introduce random flight emergencies, but the

simulation will abort with a grim message if you mishandle the jet.

Using the built-in program specifications of a Boeing 747, the

equations of motion dictate that it takes ABOUT 63 seconds to reach

take-off velocity (150 knots) after full engine thrust is applied.

During this interval, the accelerating jet uses up about 78 percent of

the two mile runway.

This equation of motion establishes the safe take-off envelope for the

simulation. You must use the "U" key promptly once when the airspeed

reaches 150 knots.  If you hesitate for another ten seconds, it will

be too late, the jet will simply charge down the runway at 172 knots,

plunge into the marchlands beyond, and...you get the picture.

The anxious pilot who pulls the nose up too sharply at lift-off time

(by pressing the "U" key more than once) also comes to grief.  The

abort message will point out that the tailend of the fuselage has

struck the runway; the aft end of a 747 will clear the ground by only

a few feet during normal take-off. Most important, as pilot you must

always remember to lower the wing flaps before you attempt to take off

in a 400-ton jet, even in a simultation.

MANOEUVERING

Following the take-off, the jet slowly gains altitude as it passes

over central New Jersey and heads toward the Atlantic coast.  None of

this geography is visible, of course, because of the blanket of clouds

below. At this point, you must navigate the jetliner entirely on

instruments until it's just a few hundred feet from the point of

landing at the destination airport, wherever that may be.

This lesson will give you a "feel" for the controls and show you how

they relate to the instrument panel functions. (See table 2 for

controls list.) The PITCH indicator shows that the nose is tilted

upward (positive pitch) at an angle of 10 degrees.  With the current

position of the THRUST lever, the jet is gaining altitude at the rate

of about 6330 feet per minute (VERTICAL SPEED). Press "\" or the "D"

key twice to level the nose to a zero-degree pitch. The AIRSPEED will

now increase. VERTICAL SPEED will become zero, and the ALTITUDE will

remain constant.

The "U" and "D" keys, which correspond to motion of the pilot's

control stick, are used to climb or descend to a new altitude.  Each

press of the "D" key pushes the nose down another 5 degrees (with

flaps up) , causing a rapid loss of altitude as both airspeed and

vertical speed build up. Regardless of the manoeuver--climbing or

diving--you should always use the "\" key to quickly level off the jet

when the ALTITUDE readout reaches the desired value.

You can steer the jet to a new COMPASS course by pressing the keys

that control rudder angle.  Press the "," (<) key once to begin a slow

turn to the left and watch both the COMMPASS and the rudder-angle

indicator (RUD). Each additional push of the rudder key will make the

angle more acute, causing the COMPASS to swing faster as the rate of

turn increases.  Always use the rudder-cancel key (/) to stop further

turning as soon as the COMPASS indicates the desired course.

You can adjust AIRSPEED by moving the thrust level forward or backward

(F and S keys) one step at a time.  Each tap of the key shifts the

position of the arrow displayed on the THRUST indicator and alters the

AIRSPEED reading. The 747 normally cruises at 540 knots, and for a

given thrust setting the AIRSPEED indicator will drop back during a

climb and increase during descent.

With the 747WORLD.BAS version, since the instrument response time is 4

seconds, you must delay consecutive applications of the stick or

rudder keys until the panel instrument readings catch up. With the

compiled version the delay is less than 1 second.  The jet will

automatically level off when it reaches an altitude of 45,000 feet; a

dive to ground level while cruising however, will abort the flight

with a simulated crash.

In a plane, the VLF OMEGA indicator is part of an electronic subsystem

that receives and correlates specially phased, very low-frequency

radio waves. These waves, which propagate over great distances, are

processed in the airborne receiver to give the pilot a continuous

display of the changing position of the aircraft.  The JETSET or

747WORLD simulator tracks aircraft motion as the sum of two vectors:

aircraft movement relative to the wind (compass heading and airspeed)

and wind movement relative to the earth's surface. As a result of this

tracking, the longitude and latitude displayed by the OMEGA readout

can fix the exact geographic position of the jet as it is manoeuvered

through computer-simulated winds.  This process results in an

effective real-time simulation of the actual OMEGA system.

Although the longitude and latitude displayed on the OMEGA indicator

may be used along with any chart or road map to check the progress of

the simulated flight, the actual OMEGA system is normally used for

flying between continents. For short-range and cross-country flights,

most aircraft-and the JETSET or 747WORLD simulator-rely on a more

convenient system known as VOR (VHF Omni-directional Ranges).

NAVIGATING

Most aircraft navigates from point to point using VOR radio

facilities.  A ground station transmits radio beams that radiate

horizontally outward in all directions like the spokes of a wheel.

Each spoke or radial (there are 360) is fixed in direction and can be

used to provide an accurate and unvarying path to its source, the VOR

station transmitter.

In practice, the pilot first tunes the VOR receiver to a ground

station located at or near the destination.  Each station is assigned

a unique frequency.  Next the pilot adjust the receiver's radial

selector dial to match the particular radial intended for use as a

path (this dial is calibrated in one degree step, from 000 to 359

degrees).  The pilot then flies while watching the needle of a

sensitive meter connected to the VOR receiver. When the needle moves

to its center position, the aircraft has intercepted the selected

radial.  By altering the course to keep the VOR needle centered, the

pilot will be able to guide the plane directly along the radial in a

straight line toward the VOR transmitter.

Let say that we want to go to Buffalo, New York after departing from

Philadelphia. First tune the VOR receiver to 116.4 MHz (the frequency

assigned to the Buffalo VOR station) and select a radial to intercept

let say the radial 115 degrees "FROM".  Rotate the radial dial until

its points to 295 degrees, the reciprocal value of 115 (115 + 180 =

295).  The reciprocal value is always used when setting the selector

dial to match the chosen "FROM" radial because we want to go in the

direction of the radial "TO" , 295 degrees in this example.  Rotate

the radial dial until it points to 295 degrees.  . This process gives

the VOR receiver proper internal orientation. In fact it is preferable

to say that we want to intercept the 295 degress radial "TO".

The simulator VOR receiver is tuned and adjusted from the keyboard. To

tune to a station, first press the V key. then type in the station

frequency. The typed characters will echo on the screen; to correct

them, use the Backspace key.  Finally, press Enter to terminate the

input.  To tune in the Buffalo station, type the sequence "V116.4"

followed by the Enter key.

A similar procedure sets the VOR  receiver to any selected radial

except that you type "R" first rather than "V".  To adjust the

receiver for the flight to Buffalo, type "R295" followed by the Enter

key.

Once tuning is completed, you turn left to fly in an approximate

northerly (COMPASS indicating 0 degree) direction and watch the

movement of you VOR panel indicator. Initially the needle will be

"pegged" to the right side of its travel, but it will slowly begin to

move toward the center as the plane nears the 295 degrees radial "TO".

Once the needle is at center, alter your course to 295 degrees by

compass and swing the nose of your jet toward Buffalo.  Now you must

make minor steering corrections, using the rudder to keep the VOR

needle centered.

This needle, rather than the compass reading, provides the guidance

for the remainder of the trip.  Upper air winds will generally deflect

the heading (compass course) of the jet from its actual track over the

earth's surface, but if the plane is flown with the needle centered,

the path of travel will remain  exactly on the  295  degrees  radial

"TO" . The compass reading may differ by a dozen or more degrees when

you are flying at upper altitudes in the presence of high velocity jet

streams.

The process of adjusting the steering to keep the VOR needle on center

is called "chasing the needle."  If the needle (which represents the

radial), begins moving to the left, you must apply some left rudder

until the needle returns to center.  For needle deflection to the

right, steer to the right. After a minute or two you should be able to

establish a compass heading that keeps the VOR needle centered until

the jet arrives in Buffalo.

The VOR system carried aboard a jetliner includes a very useful and

important device know as the DME (Distance-Measuring Equipment).  Once

the VOR receiver is tuned to a station, the DME indicator continuously

displays the distance in nautical miles (NM) to the station.  In a

flight to Buffalo, from Philadelphia airport for example, the DME

would read about 180 NM when the northward-flying jet first intercepts

the 295  degrees radial "TO". From then on, as the pilot steered

toward Buffalo the DME value would progressively decrease in step with

the aircraft's position until the reading reached zero.  A zero

reading would indicate that the jet had flown over the VOR station.

The DME readout would then slowly begin to increase as the pilot

passed by Buffalo.

The RANGE window of the VOR receiver displays OUT whenever the

receiver is not tuned to any station or whenever it is tuned to an

incorrect frequency. An OUT also appears if the receiver is tuned to a

VOR station whose distance exceeds 300 NM, the maximum range of the

VOR signals.

PRACTICING VOR

Several practice flights to Buffalo on the JETSET or 747WORLD simulator

will acquaint you with the simple principle of VOR navigation.

Although it isn't necessary, a chart or group of road maps that

encompass the Buffalo-Philadelphia area would help you visualize the

progress of the jet.

Begin by taking off from Philadelphia, climbing to about 10,000 feet,

and leveling off.  Then apply the left rudder until the compass reads

000, give or take a few degrees.  While you're on this northerly

course, adjust the thrust (F and S keys) for an airspeed of 540 knots.

Tune to the Buffalo VOR station by typing "V116.4" and the Enter key.

Set the receiver for the reciprocal of the 115 degrees radial "FROM"

by typing "R295" followed by Enter.  This completes the tuning

procedure.  The VOR needle, which is located directly above the RADIAL

window on the display, will now remain pegged to the rightmost

position for about seven minutes as the jet flies north.  Once the VOR

needle begins moving toward the center of the graphic slot, prepare to

alter course.  When the needle reaches center, apply the left rudder

(< key) and bring the jet on a compass course of 295 degrees.  Remain

on this course for about a minute and watch the motion of the VOR

needle.  Now you can begin chasing the needle by applying the rudder

corrections needed to center the needle and keep it there.  You may

need to make an occasional steering adjustment if the needle begins to

wander, but as long as it remains within one dot of center (each dot

represents one degree), your course will be reasonably accurate.

When the Buffalo radial is first intercepted, the DME indicator should

read approximately 180 NM, and it should take about 20 minutes for the

540 knot jet to reach its destination.  The exact flying time, of

course, will depend on the strength and direction of the prevailing

winds, but the DME readout will always show the exact remaining

distance.  If you use a map to keep tabs on the practice flight,

remember that DME distances are nautical (not statute) miles.  A DME

reading of 100 NM corresponds to 115.2 statute miles.

As the jet moves along the radial the RANGE window of the VOR panel

will display TO, indicating orientation toward the VOR station.  As

soon as the DME reaches zero, note the reading of the OMEGA display.

Because the jet is passing directly over the ground station, the

display should read 42 degrees 55.7 minutes North, 78 degrees 38.8

minutes West, equal to the geographic coordinates of the VOR station.

This reading confirms that the navigation was accurately performed by

the VOR system.  If you have maintained the course, a FROM will appear

in the RANGE window as the jet proceeds in a westerly direction away

from Buffalo, New York.

FLYING ALONG AIRWAYS

Although I used the 115 degree radial "FROM" for the practice flight

to Buffalo, I could just as will have chosen other radials for

guidance.  For example, a map shows that the 140 degree radial "FROM"

passes directly through Philadelphia and would therefore reduce the

flying time if it had been used as a path. I selected 115 degrees

instead because it is designated as a jet route by the FAA (Federal

Aviation Administration). The FAA has established a network of special

radials that high altitude jets must use when flying on instruments.

An aviation chart reveals that radial 115 from Buffalo corresponds to

jet route J-95 when the radial direction is adjusted for the earth's

magnetism (the JETSET or 747WORLD program works with true, not magnetic

directions).

In order to comply with regulations, an actual high altitude flight

from Philadelphia to Buffalo might require the pilot to proceed as

follows:

-fly toward Philipsburg, Pennsylvania along jet route J-60 alter

course at Philipsburg to pick up jet route J-61 which leads

directly to Buffalo.

During the first leg of the trip, the pilot would tune the VOR

receiver to 115.5 MMz, the frequency of the Philipsburg ground

station, and fly along the J-60 radial (278 degrees).  Just before the

pilot reached Philipsburg (as shown by DME indicator), he would retune

the receiver for Buffalo (116.4 MHz) and adjust it to the radial that

corresponded to jet route J-61 (346 degrees).  The pilot would then

alter his course, chasing the needle to follow radial 346 until he

arrived at Buffalo.

Numerous VOR stations scattered throughout the country enable a pilot

to fly extended distances simply by hopping from one station to the

next, retuning the receiver to locate the designated jet routes.

JETSET or 747WORLD , however needs only a handful of VOR stations to

establish a network for instrument flight simulation.  Table 3 shows

some frequencies and locations of the VOR stations for practice

flights. Fourth maps of U.S.A. and one map of the world are available

in the following files, MAPNE.DAT, MAPNW.DAT, MAPSE.DAT,MAPSW.DAT and

WORLDMAP.DAT with graphics, VOR informations and ILS informations. You

can send them to the printer with the DOS command TYPE MAPNE.DAT>PRN.

You may use any of these VOR stations for practice flights to the

given cities or as stepping stones for navigating from city to city.

(Remember that a tuned-in VOR station must be within 300 miles to

activate the airborne VOR receiver).

The VOR receiver in the JETSET or 747WORLD simulator is as versatile as

its real life counterpart.  When a pilot is lost or disoriented the

receiver can be tuned to a VOR station and the radial selector dial

rotated until the needle of the VOR meter centers.  The reading shown

on the radial dial then represents the direction from the VOR station.

Combining this with the distance read on the DME indicator results in

an exact position "fix".

In the JETSET or 747WORLD simulator a press of a "A" key results in an

exact position fix. The program automatically rotates the invisible

radial selector dial for the pilot and quickly displays the direction

from (the radial "FROM") the tuned-in station in the RADIAL window.

TABLE 3

LOCATIONS AND FREQUENCIES OF SOME VOR GROUND STATIONS

-----------------------------------------------------------------

ILS

LOCATION        FREQUENCY  LATITUDE  LONGITUDE   RADIAL   RUNWAY

APPROACH  HEADING

-----------------------------------------------------------------

Philadelphia, PA  113.8     39 51.1   75 14.7      80       75

Philipsburg, PA   ll5.5     40 55.0   77 59.0     119      164

JFK, NY           115.9     40 38.0   73 46.4      58       40

Boston, MA        112.7     42 21.5   70 59.6      50       40

Buffalo, NY       116.4     42 55.7   78 38.8     346       42

Detroit, Mi       115.7     42 24.6   83 00.6

Chicago, Il       113.9     41 51.0   87 45.0     325      320

Kansas City, MO   112.6     39 18.0   94 34.0     342      328

----------------------------------------------------------------

INSTRUMENT LANDING

Using the VOR receiver as a guide a pilot can navigate accurately from

one city to another without any view of the earth below.  VOR radials

are suitable for point-to-point navigation, but when a pilot arrives

at his destination he needs another system of guidance to get to the

airport runway itself.  In this case the pilot must revert to a radio

aid, the Instrument Landing System (ILS), a facility designed to make

blind landings possible.  A trained pilot flying an aircraft equipped

with an ILS receiver can locate an airport and safely land on a runway

that may not be visible until a minute or so before the actual

touch-down.

An ILS installation consists of a group of radio transmitters arranged

in the vicinity of the airport where ILS landings are to take place.

These transmitters radiate highly directional radio beams that

converge at about 1250 feet from the foot of the runway (this will be

call the origin of the glideslope), forming a cone-shaped pattern like

the rays of a searchlight.  The pilot first manoeuvers the plane into

this invisible cone, then uses the ILS receiver to follow the radio

waves down until the aircraft is just a few hundred feet above the

ground.  At this low altitude the runway should be visible, so the

actual landing can be completed in the usual way.

The airborne instruments used to locate and follow the cone of radio

waves are a marker lamp, an ILS indicator, and a radar altimeter.  On

the JETSET or 747WORLD simulator panel these three components are

identified as the MARKER, ILS, and RADAR ALT respectively.  The panel

MARKER lamp flashes on when the aircraft flies over a point called the

"outer marker" telling the pilot that the plane has just entered the

ILS cone.  The crosshairs (horizontal and vertical needles) of the

panel ILS meter will now begin to deflect, and the pilot must

manoeuver the plane to keep the needles centered in order to follow

the path of the ILS radio cone.  As the aircraft descends along this

narrow path, the radio altimeter (RADAR ALT) gives a continuous

display of the exact elevation from the ground (in feet).The radio

altimeter is much more sensitive than the conventional altimeter, so

it is always used for precision landings.

During the time the aircraft has entered the ILS cone and is heading

toward the runway, when the pilot is making the final approach, the

plane flies in a direction known as the "localizer" direction of the

ILS radio beams. The angle that the radio cone makes with the ground

is called the "glidescope" angle, and the descending plane is said to

be flying within the ILS "glidepath". The two moving needles of the

ILS indicator correspond to the localizer and glideslope axes during

the final approach.  The pilot chases the vertical needle (which moves

left and right) to remain aligned with the localizer direction.  The

horizontal needle (which deflects up and down) must be chased using

the elevator controls to keep the plane within the glidepath.

Once the descending aircraft reaches the ILS "middle marker", the

panel MARKER lamp will flash again, alerting the pilot that the plane

is just a fraction of a mile from the runway.  This critical location

is called the "decision height" of the final approach because the

pilot must now decide whether he can safely complete the landing.  If

the runway appears in view directly ahead, the pilot can make a visual

landing.  If, however, the plane is not properly lined up with the

runway (because the ILS needles were not kept centered), the pilot

must abort the landing attempt at once by climbing out of the

glidepath.  This situation is known as a "missed approach".  When a

pilot misses the approach, he flies a safe distance away from airport

traffic and then returns to the OM point for another try.

Specifications:

-ideal glideslope angle..................... 3 degrees .

-maximum semi angle in horizontal plane......2.5 degrees.

-ILS displayed........  from 1250 feet to between 34490 to

38000 feet from origin of glideslope.

-outer marker flashes...from 34490 to 38000 feet from origin

of the glideslope. The best altitude

is about 1900 feet.

-inner marker flashes...at about 5000 from the origin of

glideslope at an altitude of about 300

feet.

-the runway is displayed from 12000 feet from the origin of

the glideslope and each mark is 10

feet.

Every ILS equipped airport uses an arrangement which places the VOR

station away from the airport in such a way that the plane will cross

the ILS cone near the outer marker.  The exact ILS arrangement

(localizer direction and glidescope angle) for any given airport is

published in a manual of approach diagrams (one for each airport),

which the pilot studies well in advance of his instrument landing.

Obviously, an instrument landing is a tricky procedure that airline

pilots must practice in large-scale simulators to perfect.  The

routines that simulate landing are an important part of the JETSET or

747WORLD program; they closely follow the sequences that develop when a

plane flies into the ILS pattern. You may have to make several

attempts at a simulated landing before you can consider yourself

qualified to handle a jetliner under bad weather conditions.

PRACTICING ILS

Preparing for an instrument landing, even aboard the JETSET or 747WORLD

simulator, begins when the plane is still many miles away from the

airport. Because all ILS landing procedures follow a standard pattern,

the John F. Kennedy (JFK) International Airport, conveniently located

with respect to Philadelphia, can serve as a practice landing site.  A

simulated flight from Philadelphia to JFK lasts about 20 minutes from

take-off until the jet rolls to a stop on the runway.

Every airline flight must be conducted in accordance with a flight

plan, a document that specifies the routes the pilot will fly until he

arrives at the destination.  An actual flight takes place at standard

altitude levels and under close supervision of air traffic

controllers, but the flight plan prepared for the practice run to JFK

International tells the JETSET or 747WORLD pilot exactly how to proceed.

Using the Philadelphia-JFK flight plan as a guide, execute the

take-off procedure and climb to 5000 feet while maintaining a compass

course of 075 degrees. During the climb, tune your VOR to the JFK

ground station (115.9 MHz) and input the radial value of 058 degrees.

Level off at an altitude of approximately 6000 feet.  Use the "<" key

for the left rudder to alter the compass course to approximately 030

degrees. Hold this course until the VOR needle nears its center

position.  Now steer to 058 degrees and begin chasing the VOR needle.

The jet will head directly for JFK as long as you keep the VOR needle

centered, the 058 degree radial is used because it's the "initial

approach" radial defined for JFK airport. It will lead to an intercept

with the runway outer marker (OM), a prerequisite for the instrument

landing.

As soon as the DME indicator reads 38, you must prepare for landing.

To begin a descent, adjust the elevators for a pitch of -10 degrees

(press "D" twice) and level off at an altitude of about 1900 feet.

Start the "initial approach trim" procedure for the jetliner when the

DME distance is 20 NM. First reduce your airspeed to 300 knots (S

key), lower the landing gear (W key), and lower the wing flaps (L

key).  The airspeed will automatically drop back to 120 knots as soon

as the flaps are lowered, as required for a proper landing.  Complete

the trim procedure by adjusting altitude until the ALTITUDE indicator

reads between 1850 and 1950 feet.

You must execute this procedure quickly so that the aircraft is in

proper "profile" or flight configuration as it approaches the OM along

the initial approach radial.  You will reach the OM when the DME reads

exactly 12 NM, so the jet should be in its trim profile and steered to

keep the VOR needle centered (to within two graphic dots) as the OM

point nears.

NOTE:When the DME reads 12 NM, the jet is at 12 NM from the VOR.

For the program the jet is about 5.5 to 6.2 NM from the

runway.

If you've done these steps carefully, the panel MARKER lamp will flash

when the DME indicator reads 12 NM.  This is a signal that the

aircraft has just intercepted the ILS radio cone and must be promptly

steered to align with the localizer direction (040 degrees) at JFK

airport.

Press the left rudder (< key) quickly when the MARKER lamp flashes.

It's imperative that you swing the jet to a compass course of 040

degrees before it flies out of the narrow area of the radio cone (this

would occur in less than 1 minute after the MARKER lamp turns on and

is always the case if the DME becomes less than 10 NM ). A compass

reading of 040 degrees (give or take one degree) before the MARKER

lamp goes off will ensure that you completed the turn in time for the

jetliner to enter the ILS radio cone.  Both the ILS indicator and the

RADAR ALT meter should be activated.  If not, the turn took too long

to complete and you need more practice in making a fast turn. For

another attempt, you can stop the simulation program and begin again

or raise the flaps and wheels and circle back to pick up the initial

approach radial for another attempt.

The rapid updating of the ILS indicators ( on the left side of the

panel ) means the jet is now beginning its crucial final approach. You

have very little margin for error. With flaps down the sensitivity of

the elevator is changed, each press of the elevator key varies the

pitch by one degree. Quickly press the "D" key three times to pitch

the nose down 3 degrees. In the ILS the mode of action of the rudder

keys are automatically modified, and the course changes by one degree

each time a rudder key is pressed. Now turn your attention to the ILS

display.

You must use the rudder keys (< or > 1, 2 or 3 times and wait for the

response...) to chase the vertical needle of the ILS indicator as the

jet loses altitude (as shown by the RADAR ALT reading). If the ILS

horizontal needle moves from center, chase by using the elevator keys.

Crosswinds blowing across the airport will tend to deflect the jet

(and the vertical ILS needle), so you must make every effort to keep

the two ILS needles where they belong, exactly on center.

The RADAR ALT indicator, a meter that activates when the final

approach begins, shows the elevation of the descending jet (feet above

ground level).  At an elevation of about 600 feet and a distance of

about 12000 feet from the origin of the glideslope , JETSET or 747WORLD

will display the approaching runway under the ILS meter to simulate

that the ground is now visible. The arrow appearing at the foot of the

graphic screen shows the exact alignment of the jet in relation to the

approach end of the airport runway.  You mut now use this visual

reference instead of the ILS vertical indicator to quickly correct any

course errors.  For example, if the arrow extends too far to the left,

beyond the runway base, apply right rudder to realign the jet's path.

The distance between each mark on the runway is 10 feet

After a few more seconds (at an altitude of about 300 feet) the MARKER

lamp should flash again to announce that the plane has just reached

the middle marker point along the approach path, the decision-height

location.  Now a quick decision is vital.  If the arrow of the runway

graphic extends too far left or right, beyond the runway base, the jet

is not properly lined up for a safe landing and you must press the "M"

key immediately to signal a missed approach to the computer.  JETSET

or 747WORLD will aknowledges and you can apply power, gain altitude, try

again or go to another airport.

If however, the runway arrow shows that the jetliner is safely aligned

for a landing, and that the altitude is acceptable, you must bring it

down as follows. Remember that over the end of the runway the ILS

indicators should vanish:

1. At an elevation of 100 feet (RADAR ALT reading), press the

"S" key once. This command will "chop the throttle" (abruptly

reduce the engine thrust to idle).

2. At 50 feet, press the "\" key once to "flare up" the nose of

the jet. This manoeuver automatically tilts the aircraft

upward slightly to a positive pitch (+1 degree) , causing a

controlled stall.  The jet will now sink gently down to

ground level as it loses aerodynamic lift.

3. At 0 feet the jet has landed and is rolling along the runway.

Quickly press the "Q" key to apply reverse thrust to the

engines. Reverse thrust decelerates the aircraft gradually

until the AIRSPEED readout reaches zero. You can reduce the

airspeed by pressing the "B" key to apply the breaks . The

"B" acts like a toggle key.

Your JETSET flight concludes with a display of the landing information

that  tells you how well you handled the jet. This information

specifies where ground contact occurred and where the jet finally

rolled to a halt. If you made a mistake after the middle marker, the

landing report will print out the consequences.

This is only a small part of the capabilities of the JETSET or 747WORLD

simulator. There are about 30 airports built in, 17 in North America

including one without name for ILS practice and 13 for the rest of the

world. In USA there are also many VOR stations for navigating.

Remember though, this simulator flies in real time. If it takes 6

hours to fly from New York to San Fransico in a real aircraft, it will

take the same 6 hours flying the simulator. See the files containing

the maps and the relevant informations.(VOR frequencies, runway

headings etc have been chosen arbitrarily...).

TAKE-OFF PROCEDURE

1.  Lower flaps (L key).

2.  Release breaks (B key).

3.  Apply full throttle (F key).

4.  Steer along the 075-degree runway using the left/right rudder

keys , (<) and . (>) without SHIFT.  Coordinate steering with

the COMPASS reading and the position of the arrow located at

the base of the runway graphic.

5.  As soon as the AIRSPEED indicates 150 knots, press the U key

once to gently lift the jet off the runway.

6.  After the  horizon line drops below the screen, press the W

key to raise the landing gear before an altitude of 1200 feet.

7.  Retract the flaps (L key).

8.  Throttle back the engines (S key).

9.  Sit back and relax for a minute or so as the jet gains

altitude.

FLIGHT PLAN - PHILADELPHIA, PA TO BUFFALO, NY

1.  After take-off, continue climbing to 3000 feet on course 075

degrees.

2.  At 3000 feet alter course to 000 degrees and continue climbing.

Adjust thrust for airspeed 440 knots, tune VOR to Philipsburg

station (115.5 MHz), and set radial to 278 degrees. Level off

at 40000 feet.

3.  When the 278 radial is intercepted, steer along 278 radial and

proceed to Philipsburg at 540 knots. Adjust heading to take

wind into consideration.

4.  At DME = 20 NM, retune VOR to Buffalo (116.4 MHz) and set

radial to 346 degrees.

5.  Upon intercepting 346-degree radial, alter course to follow

radial to Buffalo. Adjust heading for wind.

6.  At DME = 73 NM, reduce speed and begin descent to 1900 feet

(descend at between 8000 to 9000 feet per minute).

7.  Level off at 1900 feet.  Remain aligned with radial.

8.  Begin initial approach trim when DME=20 NM as follows:

A. Reduce airspeed to 300 knots (S key).

B. Drop landing gear (W key).

C. Lower the flaps (L key).

D. Adjust altitude to between 1700 and 1900 feet (elevator

keys).

E. Keep the VOR needle centered (rudder keys) to stay on

the initial approach radial.

9.  Be alert for the flash of the MARKER lamp (which occurs when

the DME=12). At this signal the jet must be manoeuvered for the

final approach:

A. Quickly swing the nose until the compass shows 042

degrees.

B. Use  rudder and elevator keys to keep the ILS indicator

needles centered as the jet descends along the

glidepath.

C. As soon as the runway graphic appears on the screen, use

the graphic arrow as a guide to apply rudder

corrections.

10.  When the MARKER lamp flashes again to announce arrival at the

decision-height point, check the runway alignment using the

graphic displayed on the screen. If necessary, press the M

(Missed Approach) key to abort the landing attempt. Otherwise,

if the plane is lined up safely, take all cues from the RADAR

ALT from here on in:

A. At 100 feet, idle the engines (S key).

B. At  50 feet, flare up the nose (\ key).

C. At   0 feet, the jet is on the runway. Slow it down by

applying reverse thrust to the engines (Q key) and

brakes (B).

FLIGHT PLAN - PHILADELPHIA, PA TO JFK INTERNATIONAL, NY

1.  After take-off, continue climbing to 6000 feet on course 075

degrees.  While climbing, tune VOR to JFK station (115.9 MHz)

and set radial to 058 degrees.

2.  Level off at 6000 feet.  Steer left to intercept radial, align

with it, and proceed toward Long Island, NY at 380 knots.

3.  At DME = 38 NM, begin descent to 1900 feet (descend at

approximately 3442 feet per minute).

4.  Level off at 1900 feet.  Remain aligned with radial.

5.  Begin initial approach trim when DME = 20 NM.

6.  Execute ILS final approach procedures.  When MARKER lamp

flashes, turn to runway heading 040 degrees.

===== END ======