ANTONOV AN-225 MRIYA

Written and modelled by Tim Vasquez, CIS 71611,2267

The An-225 Mriya (Dream), the largest flying aircraft in the world

today, boasts a wingspan of nearly 100 yards and a maximum takeoff weight

of 1.3 million pounds!  It first flew on December 21, 1988, from Kiev,

Ukraine (then USSR), and a series of payload records and performances were

set in 1989.  There have even been exhibits at international airshows

touting a possible commercial version for sale which could seat 800

passengers or carry multiple decks of freight.

HISTORY

In 1985 there was a need to transport the Soviet space shuttle, along

with rocket components and equipment for the petroleum, chemical, and

electrical industries, to remote portions of the country.  Antonov was

assigned the project, and they based their designs on the already-existing

An-124, an aircraft slightly larger and similar to Lockheed's C-5 Galaxy.

The wing was extended, engines changed from 4 to 6, and the vertical fins

were placed away from the center to eliminate aerodynamic wake effects from

piggybacked cargo.

There are only two An-225s in existence; the aircraft is highly

expensive and its development was a major burden on the Soviet economy

and industry.  It is unlikely any more will be built unless financed by

private companies.

PERFORMANCE DATA

Due to its large size, the An-225 suffers from tremendous parasitic

drag.  This is offset by its six high-bypass turbofan engines, which come

in at a respectable 51,000 pounds of thrust each.  Its rather wide wingspan

and a low aspect ratio contribute to a high lift capability, however, this

and its huge thickness also amounts to a slow cruising speed (in the Mach

.65 to .7 range).

Although I have been unable to find numbers on the An-225's performance,

talk with experienced airline pilots along with technical references suggest

the following data, which has been modelled into the simulation aircraft.

TAKEOFF CHARACTERISTICS, Antonov An-225

V1, takeoff decision speed........................ 115 kts

Vr, rotation speed................................ 121 kts

V2................................................ 135 kts

Takeoff run....................................... 7,500 feet

Takeoff roll

-Flaps 10

-Max power

-Rotate at Vr, aircraft will lift at V2

-Gear up on positive rate of climb

Climbout below 10,000 feet

-Maintain pitch of 4 deg, slow acceleration to flap retraction (148 kts)

-Trim to gradually accelerate to and maintain 250 kts

Climbout 10,000 feet and above

-Maintain pitch of 4 deg, slow acceleration to 400 kts TAS

-Reduce power as needed

Cruise

-Maintain pitch of 0 deg, use power as needed according to load/speed

LANDING CHARACTERISTICS, Antonov An-225

Vref, basic minimum operating speed, full flaps... 105 kts

Vref, flaps 10.................................... 118 kts

Vref, flaps 5..................................... 125 kts

Vref, no flaps.................................... 132 kts

Final approach speed, flaps down.................. 115 kts

Landing run....................................... 5,000 feet

Approach to terminal

-Apply flaps and apply power using flap schedule Vref + 20 knots:

Apply 5 deg flaps upon deceleration through 155 kts

Apply 10 deg flaps upon deceleration through 135 kts

Apply full flaps upon deceleration through 125 kts

Final Approach

-Maintain pitch of 12 deg and 40% power to maintain 115 knots,

adjust power and pitch as required for glidepath