BETA RELEASE OF OS/2 COMMUNICATION DRIVERS

COM.SYS          08-10-92

VCOM.SYS       08-07-92

COMM.DRV      06-11-92

MODE.COM      08-06-92

IBM DISCLAIMS ALL WARRANTIES, WHETHER EXPRESSED OR IMPLIED, INCLUDING

WITHOUT LIMITATION, WARRANTIES OF FITNESS AND MERCHANTABILITY WITH

RESPECT TO THE INFORMATION IN THIS DOCUMENT AND INCLUDED SOFTWARE

PRODUCTS.  . BY FURNISHING THIS MATERIAL, IBM GRANTS NO LICENSES TO ANY

RELATED PATENTS OR COPYRIGHTS.

Copyright IBM Corporation, 1992, all rights reserved.

1.0       INSTALLATION PROCEDURE

a.      The following files will be replaced:

x:\OS2\COM.SYS

x:\OS2\MODE.COM

x:\OS2\MDOS\VCOM.SYS

x:\OS2\MDOS\WINOS2\SYSTEM\COMM.DRV

These files will be backed up with the extension *.392.  If

files with this extension already exist, then NO BACKUP will

take place.  The installation utility is assuming that you are

using IBM OS/2 2.0 GA release drivers.

b.    Switch to the drive and directory where the BETA files are

located.

c.     Enter "cominst  x:" at the command line where

x: is the drive letter of the OS/2 Partition.

d.     To DE-Install the BETA drivers communication driver, Enter

DEINSTL x: where x: is the drive letter of the OS/2 Partition.

2.0      DESCRIPTION OF DRIVERS

The following is a brief description of the BETA

communication drivers for OS/2 2.0.  There are new

parameters for the COM.SYS which is located in the

CONFIG.SYS file.

2.1      COM.SYS

COM.SYS now has new parameters to indicate the COM port

number, the I/O address, the UNIQUE IRQ level and  how to

handle spurious interrupts.   An example follows:

DEVICE=C:\OS2\COM.SYS (n,xxxx,ii,s) [(n,xxxx,ii,s)]...

where the last parameter is optional.

n         :   COM port number (usually 3 and 4 but it is

possible to configure any communication port to a

differrent IRQ or I/O Address

xxxx   :  COM port address

ii         :  IRQ level

s         :  Spurious interrupt handling switch

values:

D or d   to deinstall COM driver if

more than 1000 consecutive

spurious interrupts occur.

I or i     to ignore spurious interrupt.

P or p  to post error if more than 1000

consecutive interrupts occur.   When

posting an  error, some DOS applications

can hang.

COM.SYS also has the following fixes:

1. COM.SYS now uses a different algorithm to test for he presence of a

communications port at the given address.  There were certain cases

where a port was not recognized.

2. Change from DevHlp EOI to writing port 0x20 to

reduce interrupt pass.  (Possible performance

improvement).

3. Better checking of FIFO.

4. COM.SYS does not detect Hayes ESP card so that Hayes

ESP card can be used as a regular dual asynch card.

5. New DOS_SETTING Propertys :  COM_DIRECT_ACCESS removes the IO hook of

VCOM.SYS after check the access control.  If the application only

works  when  VCOM.SYS is not loaded, COM_DIRECT_ACCESS will help the

problem.  When COM_DIRECT_ACCESS is on, any application which

requires the presence of VCOM.SYS will not work in the DOS session.

This VCOM.SYS requires the new COM.SYS.  The COM_SELECT setting will

"hide" communication ports from certain DOS applications which try to

control all the communication ports (See Below).

2.2      VCOM.SYS

No parameters or settings required.  This version of VCOM.SYS should

work better with X00.SYS (and other similar applications) when

COM_DIRECT_ACCESS is ON.  COMDD.SYS (which is required by older DOS

applications such as BASIC) should also perform better.  COMMDD.SYS

is loaded into the DOS_DEVICE setting for a Virtual Dos Machine.

2.2.1    COM_DIRECT_ACCESS DOS property

When COM_DIRECT_ACCESS is ON, VCOM.SYS will allow a DOS application

to access the communication ports directly.  This DOS property makes

LapLink III, FaskLynx, FSDUAT, AS/400 Asynch Router, MS WORD work in

VDM session.  However, since the buffers in COM.SYS cannot be used,

characters may be lost and and some applications may suffer from the

lack of buffering.  With most DOS applications, COM_DIRECT_ACCESS

should be set to OFF as its default setting.

2.2.2    COM_SELECT DOS Property

COM_SELECT allows the DOS session to select only one communication

port to be used by the session.  The communication ports which are

not selected will be hidden from the DOS session.  There are some

DOS applications which take over every available communication port.

This DOS property is effective in preventing those DOS applications

from taking over all the communication ports.  An example of a DOS

application which attempts to control all the communcication ports

is LapLink Pro.  If LapLink Pro and another application which

accesses a communication port are executed at the same time, it is

necessary to set COM_SELECT.  The default setting is ALL.

2.3      COMM.DRV

COMM.DRV contains the support for COM3 and COM4 in WinOS2.  This

COMM.DRV requires the new VCOM.SYS.