WORDPERFECT VERSION 5.1

CHAPTER CONTENTS

WORDPERFECT CORPORATION........................................................2

INSTALLATION...................................................................3

Selecting the Correct Printer Driver......................................3

Installing a Sheet Feeder.................................................4

Installing Accessory Typefaces and Fonts..................................5

Installing Font Cartridges................................................6

Installing Bitmapped Soft Fonts...........................................6

GENERAL INFORMATION............................................................8

Printer Definition........................................................8

Helps and Hints...........................................................8

COMMON TASKS...................................................................8

Selecting Fonts...........................................................8

Underlining...............................................................9

Existing Text.............................................................9

Printing Special Characters...............................................9

Letter-Size Paper in Landscape Orientation................................9

Legal-Size Paper.........................................................10

Com-10 Envelopes.........................................................10

Monarch Envelopes........................................................11

Formatting and Printing Labels...........................................13

Creating New Forms.......................................................13

Embedding Printer Commands...............................................15

COMMON PROBLEMS...............................................................16

Computer Beeps When Printing.............................................16

New Font Cartridge Does Not Appear on Menu...............................16

Units of Measure.........................................................16

Incorrect Font...........................................................16

MP LOAD `paper size'.....................................................17

Missing Forms............................................................17

Calling for Help.........................................................18

________________________________________________________________________Page 1

WORDPERFECT CORPORATION

Customer Support:

WordPerfect 5.1 Installation:..........(800) 533-9605

Printer Support:.......................(800) 541-5170

Feature Support:.......................(800) 541-5096

Network Support:.......................(800) 321-3389

Graphics Support:......................(800) 321-3383

Hours:...........................7:00am to 6:00pm MST

Charge:..........................................None

Required Identification:...............License Number

Ordering Printer Disk Updates:..............(800) 222-9409

The information and procedures contained in this document apply to

WordPerfect version 5.1.

To access all features of the HP LaserJet IIP plus, your printer definition file

(PRS) must be dated on or after 8/22/90.

Follow these steps to find the date of your PRS file:

1.  Enter WordPerfect and press [Shift F7].

2.  Press [S] Select Printer. Highlight the

HP LaserJet IIP plus, and then press [H] Help.

3   This will take you to the Printer Helps and Hints screen. The date on the

upper left corner of the screen is the date of your PRS file. (It should be

8/22/90 or later.)

If your PRS file is dated prior to 8/22/90, you must obtain an updated printer

drivers disk. You can order updated printer drivers for a nominal charge by

calling WordPerfect Corporation at (800)222-9409.  You can also build a driver

for your fonts using the AutoFont Support disks included with each HP font

product.

________________________________________________________________________Page 2

INSTALLATION

Installation of WordPerfect version 5.1 for your LaserJet IIP plus printer

involves one or all of the following:

Selecting the correct printer driver.

Installing a sheet feeder.

Installing accessory fonts and typefaces.

If you have not installed WordPerfect, you must do so before proceeding.

Refer to the Installation Instruction card that came with your WordPerfect

software.

SELECTING THE CORRECT PRINTER DRIVER

The driver file WPHP1.ALL is included with WordPerfect 5.1 on Printer Disks 1-4.

This file contains drivers for most HP LaserJet printer models. If your release

of WordPerfect 5.1 does not yet contain a driver for your specific printer,

select a driver for a Hewlett-Packard printer with similar features (the

LaserJet IIP, for instance) until you can obtain the correct driver.

To select a printer driver within WordPerfect 5.1, perform the following steps:

1.  From your document screen, press [Shift F7] Printing Menu, then [S] Select

Printers.

2.  If the printer you want to select is listed on the display, highlight that

printer and press [S] Select.

If your printer is not listed on the screen, press [A] Additional Printers. Then

highlight the correct printer and press [S] Select.  If your printer is not

listed, you will need to run the WordPerfect "Install" Program to add the

printer.  If you need assistance with using WordPerfect's "Install" program, you

should call WordPerfect's customer support.

________________________________________________________________________Page 3

INSTALLING A SHEET FEEDER

To enable WordPerfect 5.1 to utilize all of your printer's paper feed

options, you must first install a sheet feeder. Complete the following

steps for sheet feeder installation:

1.  Type [Shift F7] Print Menu, [S] Select Printer, [E] Edit.

2.  Choose [S] Sheet Feeder. A screen similar to the following will appear.

________________________________________________________________________________

Select Printer: Sheet feeder

BDT LaserFeeder 890 (SF)

BDT LF-880 Plus / 2E Plus (SF)

BDT LF-890 Plus / 3E Plus (SF)

BDT LF-895 Plus / 5E Plus (SF)

BDT MF 830 6 Bin (SF)

BDT MF 850 3 Bin (SF)

Build Your Own (SF)

HP LaserJet 2000 (SF)

HP LaserJet 500+ (SF)

HP LaserJet IID (SF)

HP LaserJet IIID (SF)

HP LaserJet IIIP(SF)

HP LaserJet IIISi (SF)

HP LaserJet IIP (SF)

HP  LaserJet IIP plus (SF)

Rutishauser TowerFeed 444  (ILC) (SF)

Rutishauser TowerFeed 444 (SF)

Ziyad PaperJet 400 (SF)

1 Select; 2 None; 3 Help; N Name search: 1

________________________________________________________________________________

3.  Highlight the HP LaserJet IIP plus printer, then press [S] Select. (If your

printer model is not listed, select an HP LaserJet printer model with

similar features.)

4.  Read the Sheet Feeder Helps and Hints screen, then press [F7] Exit to return

to the Select Printer: Edit menu.

5.  Press [F7] Exit until you return to the document screen.

SHEET FEEDER BINS

When you select a sheet feeder, WordPerfect assigns a sheet feeder

bin number to each paper feed option available on the printer.

Complete the following steps to change your printer's paper

feed location:

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift] [F8], choose [P] Page, then [S] Paper Size and Type.

________________________________________________________________________Page 4

3.  Select [E] Edit, then [L] Location.

4.  Select [B] Bin Number.  Type the number of the bin you want to use for your

form (document type), press Enter twice, then press [F7] Exit twice to

return to your document screen.

The following chart shows the bin numbers that WordPerfect assigns for the HP

LaserJet IIP plus.

Bin 1.    MP Tray (Upper)

Bin 2.    Optional Lower Cassette

Bin 3.    Manual Paper  Feed

Bin 4.    Manual Envelope Feed

If the LaserJet IIP plus printer control panel TRAYS selection is set to TRAYS

LC ONLY, and there is paper in the MP tray, the printer will priority feed from

the MP tray rather than the lower cassette tray.

INSTALLING ACCESSORY  TYPEFACES AND FONTS

Bitmapped fonts are designed in fixed point sizes. They cannot be

made smaller or larger, and are available in two forms:

Bitmapped font cartridges

Bitmapped soft fonts

WordPerfect 5.1 includes printer disks that contain drivers for many

HP bitmapped font products. These font products are listed in the table below.

______________________________________________________________________________

Bitmapped font cartridges:

WordPerfect C2053A #C01

Microsoft C2053A  #C02

ProCollection92286PC

Global Text C2053A #C08

TextEquations C2053A #C07

Pol. Worksheets C2053A #C03

Persuasive Pres. C2053A #C04

Forms, Etc. C2053A #C05

Bar Codes & More C2053A #C06

International Collection C92286IC

Great Start C2055A #C01

All HP font cartridges 92286:...A, B, C, D, E, F, G, H, J, K, L, M, N, P, PC, Q,

R, T, U, V,W,WP, W1, X, Y, Z1, Z1A

All HP font cartridges 92290:...S1, S2

Bitmapped Soft Fonts 33412...AA, AB, AC, AD, AE, AF, DA,  EA, RA, RB, SA, SB,

TA, TB,UA, UB

Bitmapped Super Fonts*...33412 (AG),33412 (DC)

(PC-8 Symbol Set

________________________________________________________________________________

________________________________________________________________________Page 5

If you are using bitmapped fonts that are not listed in this table (such as the

HP Pretty Faces font cartridge), you must follow the instructions in the

Software Application Notes for AutoFont Support that are packaged with your font

product. Those notes will tell you how to build a driver for your new fonts

through AutoFont Support.

* If you plan to use the Bitmapped Super Fonts, you must make the fonts in the

PC-8 symbol set.  The PC-8 symbol set is the only symbol set supported by

WordPerfect's drivers for the AG and DC fonts.

INSTALLING FONT CARTRIDGES

To make WordPerfect recognize the presence of your font cartridge, complete the

following steps:

1.  Enter WordPerfect 5.1.

2.  Press [Shift F7] Print Menu, [S] Select Printers.  Highlight your HP

printer, then press [E] Edit.

3.  Press [C] to change the Cartridge and Font information.

4.  Highlight Cartridges and press [Enter].  A screen similar to the following

will appear:

________________________________________________________________________________

Select Printer: Cartridges                   Quantity

Total :    2

Available Quantity:    2

Quantity Used

HP #359 Greek..................................1

HP #374 Cyrillic...............................1

HP B Tms Proportional 1........................1

HP Bar Codes & More............................1

HP C International 1...........................1

HP D Prestige Elite............................1

Mark:  * Present when print job begins       Press Exit to save

Press Cancel to cancel

________________________________________________________________________________

5.  Move the bar again until you highlight the name of the HP font cartridge you

are installing, and press the asterisk (*) to mark the name of the

cartridge.

6.  Press [F7] repeatedly to return to your document screen.

INSTALLING BITMAPPED SOFT FONTS

You must first have installed the soft fonts onto your hard drive. If you have

not already done so, follow the installation instructions for soft fonts

included in your font documentation.

1.  Enter WordPerfect 5.1.

________________________________________________________________________Page 6

2.  Press [Shift F7] Print Menu, [S] Select.  Highlight your HP printer, then

press [E] Edit.

3.  Press [C] to change the Cartridge and Font information. Move the highlight

bar with your arrow keys to the selection Soft Fonts and press [Enter]. You

will see a screen similar to the following:

Select Printer: Soft Fonts

________________________________________________________________________________

Font Groups:

HP AC Times Roman/Helvetica US (4)

HP AD Times Roman/Helvetica R8 (4)

HP AE Times Roman/Helvetica US (4)

HP AF Times Roman/Helvetica R8 (4)

HP AG Helvetica Headlines PC-8 (4)

HP DA L.Gothic/Presentation (4)

1   Select; N Name search: 1

________________________________________________________________________________

5.  Move the bar again to highlight the name of the soft font group and press

[Enter].

6.  Mark all the soft fonts that you would  like to use within WordPerfect. Soft

fonts can be marked or unmarked with an asterisk (*) or a plus (+) symbol,

depending on the way you choose to access them in WordPerfect. (Read the

section on Marking Cartridges and Fonts in the WordPerfect 5.1 reference

manual for details.)

7.  Press [F7] Exit until you return to Select Printer: Edit screen.

8.  Choose [D] Path for Downloadable Fonts and specify the path where you have

installed your LaserJet bitmapped soft font files.

Example. If your soft font files are installed in the C:\HPFONTS directory,

specify C:\HPFONTS and press [Enter].

9.  Press [F7] to exit the Select Printer: Edit  screen.

10. Choose [S] Select to select the printer, then [F7] Exit to return to your

document screen.

________________________________________________________________________________

WordPerfect 5.1 presets the memory available for fonts on the HP LaserJet IIP

plus printer to 350K. If you have an HP LaserJet IIP plus printer and optional

memory, press [Shift F7], [S] Select Printer, [E] Edit, [C] Cartridges and

Fonts. Highlight Soft Fonts and press [Q] Change Quantity. Specify the available

memory:

1350 (for 1 MB of optional memory)

2350 (for 2 MB of optional memory)

3350 (for 3 MB of optional memory)

4350 (for 4 MB of optional memory)

________________________________________________________________________________

________________________________________________________________________Page 7

GENERAL INFORMATION

Printer Definition

The following paragraphs summarize the Printer Helps and Hints associated with

WordPerfect's printer definition.

HELPS AND HINTS

Initializing the printer will delete all soft fonts in printer memory and those

fonts marked with an asterisk (*) will be downloaded.

Line draw is not supported with proportionally spaced fonts.

Since the LaserJet IIP plus printer can rotate fonts, only portrait soft fonts

are used in this printer definition, but the printer will print in either

orientation.

Memory requirements necessary to rotate fonts may cause a 20 ERROR message. If

this happens, a printer memory expansion board is recommended.

COMMON TASKS

Selecting Fonts

The font and font attributes are selected using WordPerfect's Font feature,

[Ctrl] [F8].

The HP LaserJet IIP plus printer automatically rotates fonts from portrait to

landscape orientation. All fonts will be listed in your font selection menu,

regardless of orientation.

BASE FONT

The base font is the font in which normal text is printed.  All other

font attributes (for example, bold) are variations of the base font. To change

the base font:

1.  Position the cursor where you want to change fonts.

2.  Press [Ctrl] [ F8] to display the Font menu.

3.  Select [F] Base Font to display a list of available fonts.

4.  Highlight the font of your choice, then press [S] Select to choose the font.

APPEARANCE

To change a font's appearance (for example, bold, or italic):

1.  Press [Ctrl] [F8] to display the Font menu.

2.  Select [A] Appearance.

3.  Select the desired attribute.

________________________________________________________________________Page 8

4.  Type the text to be printed using the specified attribute.

5.  Press the right arrow key or press [Ctrl] [F8] and select [N] Normal to turn

off all attributes.

UNDERLINING

Place the cursor where you would like underlining to begin. Press [F8] to turn

the underline function on, then type your text.  Press [F8] again to turn

underlining off.

EXISTING TEXT

To change the attributes of existing text, block the text using WordPerfect's

Block feature, [Alt] [F4] then follow steps 1-3 in the section, Appearance.

PRINTING SPECIAL CHARACTERS

The HP LaserJet IIP plus printer will print any of the symbols in the character

sets shown in the WordPerfect User's Manual.  Those characters that you create

with the [Ctrl] [V] Compose function will be sent to the printer as graphics

figures.

To print the special character within a document:

1.  Position the cursor where the character is to be printed.

2.  If  necessary, change the current font (symbol set).

3.  Press [Ctrl] [V] Compose. The prompt Key = will appear at the bottom of the

screen.

4.  Enter the WordPerfect character set number (0-12), a comma, the character

number, and then press [Enter].

Example:

The copyright symbol is found in character set 4. It is character number 23.

To access this character, press [Ctrl] [V], [4], [,], [23],  [Enter] to select

this character.

LETTER-SIZE PAPER IN LANDSCAPE ORIENTATION

Follow these steps to print on letter-size paper in landscape orientation.

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift] [F8], choose [P] Page, then [S] Paper Size and Type.

3.  Highlight Standard - Wide, and press [S] Select.

4.  Press [Enter] twice to return to your document.

The HP LaserJet IIP plus printer will automatically rotate your fonts to the

landscape orientation.

________________________________________________________________________Page 9

LEGAL-SIZE PAPER

Follow these steps to print on legal-size paper.

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift] [F8], choose [P] Page, then [S] Paper Size and Type.

3.  Highlight the appropriate legal form, and press [S] Select.

4.  Press [Enter] twice to return to your document.  If your computer beeps when

the document is printed, press [Shift] [F7], choose [C] Control Printer, and

press [G]o.  (IIP plus users must click the [ON LINE] key to resume printing

when using manual feed.)

To print on legal paper with manual paper feed, you must change the location of

your legal form to manual in WordPerfect's Paper Size/Type menu.

COM-10 ENVELOPES

(Standard Business Envelopes with Preprinted Return Address)

Follow these steps to print on Com-10 (4-1/8" x 9-1/2") envelopes which have a

preprinted return address.  (When printing envelopes on the LaserJet IIP plus

printer, be sure to use the front output tray.)

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift] [F8], choose [P] Page, then [S] Paper Size/Type.

3.  If you want to print with manual feed, highlight the form called Envelope -

Wide and press [S] Select. (This is a form that is already included in the

WordPerfectSoftware.  It tells the printer that you want to print wide

business envelopes and use manual feed.)

NOTE:  To use  the Optional Lower Cassette on the LaserJet IIP plus, you must

first create a new form for your envelope (see Creating Forms).

4.  Press [M] Margins. Specify 2" for the top margin and 1" for the bottom

margin.  Press [Enter] to go to the Format menu.

5.  Press [L] Line, then [M] Margins.  Specify a left margin of 4" and a 1"

right margin.

6.  Press [Enter] and [F7] Exit  to return to your document.  Type the

addresses, separating each address with a page break, [Ctrl] [Enter]. When

you are ready to print, press [Shift] [F7] and choose [F] Full Document.

If your computer beeps when you send a document to the printer, it is set up to

print manual feed. To tell it to print the envelope, press [Shift] [F7], then

choose [C] Control Printer, and press [G]o.

(If you are using a LaserJet IIP plus printer with manual feed, you must click

the printer's [ON LINE] key to resume printing.)

________________________________________________________________________Page 10

COM-10 ENVELOPES

(Standard  Business Envelopes without Preprinted Return Address)

Follow these steps to print on Com-10 (4-1/8" x 9-1/2") envelopes which do not

have a preprinted return address.

(When printing envelopes on the LaserJet IIP plus printer, be sure to use the

front output tray.)

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift] [F8], choose [P] Page, then [S] Paper Size/Type.

3.  If you want to print with manual feed, highlight the form called Envelope -

Wide and press [S] Select.  (This is a form that is already included in the

WordPerfect Software.  It tells the printer you want to print wide business

envelopes and use manual feed.)

NOTE:  To use the Optional Lower Cassette on the LaserJet IIP plus, you must

first create a new form for your envelope (see Creating New Forms). Highlight

the correct envelope form and press [S] Select.

4.  Press [M] Margins. Specify 0.5" for the top margin and 1" for the bottom

margin. Press [Enter] to go to the Format menu.

5.  Press [L] Line, then [M] Margins.  Specify a left margin of 0.25" and a 1"

right margin.

6.  Select [T] Tab Set. Press [Ctrl End], type 4" and press [Enter].

7.  Press [F7] Exit  twice to return to your document.  Format the addresses as

shown in the following example.  Separate each envelope with a page break,

[CtrlEnter].

________________________________________________________________________________

RETURN ADDRESS:

press [Enter] until the cursor is

positioned approximately 2" from the top

press [Tab]....................................MAILING ADDRESS

________________________________________________________________________________

If your computer beeps when the document is printed, press [Shift] [F7], choose

[C] Control Printer, and press [G]o. (On the LaserJet IIP plus, you must click

the printer's [ON LINE] key to resume printing.)

MONARCH ENVELOPES

(Small Business Envelopes with Preprinted Return Address)

Follow these steps to print on Monarch (3-7/8" x 7-1/2") envelopes which have a

preprinted return address. (When printing envelopes on the LaserJet IIP plus

printer, be sure use the front output tray.)

To print on Monarch envelopes, you must first define a form (see Creating New

Forms). Proceed with the following steps once the appropriate form has been

created.

________________________________________________________________________Page 11

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift] [F8], choose [P] Page, then [S] Paper Size/Type.

3.  Highlight the correct envelope form and press [S]Select.

4.  Press [M] Margins. Specify 2" for the top margin and 1" for the bottom

margin. Press [Enter] to go to the Format menu.

5.  Press [L] Line, then [M] Margins.  Specify a left margin of 3" and a 1"

right margin.

6.  Press [F7] Exit  to return to your document.  Type the addresses, separating

each address with a page break, [Ctrl] [Enter]. When you are ready to print,

press [Shift] [F7] and choose [F] Full Document.

If your computer beeps when the document is printed, press [Shift] [F7], choose

[C] Control Printer, and press [G]o. (LaserJet IIP plus users must click the [ON

LINE] key to resume printing when using manual feed.)

MONARCH ENVELOPES

(Small Business Envelopes without Preprinted Return Address)

Follow these steps to print on Monarch (3-7/8" x 7-1/2") envelopes which do not

have a preprinted return address.

(When printing envelopes on the LaserJet IIP plus printer, be sure to use the

front output tray.)

To print on Monarch envelopes, you must first define a form (see Creating New

Forms). Proceed with the following steps once the appropriate form has been

created.

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift] [F8], choose [P] Page, then [S] Paper Size/Type.

3.  Highlight the correct envelope form and press [S] Select.

4.  Press [M] Margins. Specify 0.5" for the top margin and 1" for the bottom

margin. Press [Enter] to go to the Format menu.

5.  Press [L] Line, then [M] Margins.  Specify a left margin of 0.25" and a 1"

right margin.

6.  Select [T] Tab Set. Press [Ctrl End], type 3" and press [Enter].

7.  Press [F7] Exit  twice to return to your document.

Type the addresses, separating each address with a page break, [Ctrl] [Enter].

When you are ready to print, press [Shift] [F7] and choose [F] Full Document.

Format the addresses as follows, separating each envelope with a pagebreak,

[Ctrl] [Enter].

________________________________________________________________________Page 12

________________________________________________________________________________

RETURN ADDRESS

press [Enter] until the cursor is

positioned approximately 2" from the top

press [Tab]              MAILING ADDRESS

press [Tab]              MAILING ADDRESS

________________________________________________________________________________

If your computer beeps when the document is printing, press [Shift] [F7], choose

[P] Printer Control, and press [G]o. (LaserJet IIP plus users must click the [ON

LINE] key to resume printing when using manual feed.)

FORMATTING AND PRINTING LABELS

WordPerfect 5.1 now has an advanced label formatting feature (be sure to use the

front output tray.)

Follow these steps to print your labels.

1.  Press the [Home] key three times and the up arrow key once to position the

cursor at the start of your document.

2.  Press [Shift F8], then [P] Page to select the Format: Page menu.

3.  Press [S] Paper Size, then [A] Add.

4.  Select [L] Labels.

5.  Select [A] Labels at the Format: Edit Paper Definition menu. Type [Y] for

Yes.

6.  The Format: Labels screen will appear. Enter the measurements for your

labels. You can define the label size, the number of labels on the sheet,

the top left corner, the distance between labels, and the label margins.

7.  When you have finished, press [F7] Exit twice to return to the Format: Paper

Size/Type screen. Press [S] Select.

8.  Press [Enter] twice to return to your document.  Begin typing your labels.

If your computer beeps when the document is printing, press [Shift] [F7], choose

[P] Printer Control, and press [G]o. (LaserJet IIP plus users must click the [ON

LINE] key to resume printing when using manual feed.)

CREATING NEW FORMS

To accommodate envelopes and odd sizes of paper, WordPerfect allows you to

create your own custom forms. When you select a form, you are telling the

printer what size and shape your paper is, and what method you want to use to

feed paper into the printer.

Use the following steps to define a form:

1.  Press [Shift] [F8] Format.

2.  Press [P] Page.

________________________________________________________________________Page 13

3.  Press [S] Paper Size/Type

4.  Press [A] Add.

5.  Select [O] Other. Enter the name of your form.

6.  Select [S] Paper Size.

7.  Select [O] Other. Type the width (in inches) and press [Enter] .  Specify

the height and press [Enter] again to return to the Format: Edit Paper

Definition menu.  Continue to define your form. (If you do not know what

size to make your forms, refer to the tables following these instructions.)

8.  Press [F7] Exit to save your changes. Highlight your new form and press [1]

to select it.

9.  Press [F7] to return to your document.

HP LASERJET IIP PLUS FORM SETTINGS

________________________________________________________________________________

Form Type           Paper Size      Paper      Font     Prompt  Location  Double

Type       Type                        Sided

[9] Other              [1]           [2]       [3]       [4]       [5]      [6]

________________________________________________________________________________

Legal Paper           Legal         Legal      Both        No     Bin 1       No

(MP Tray)        8.5" W x 14" L     (MP)

Legal Paper           Legal         Legal      Both        No     Bin 2       No

(LC Tray)        8.5" W x 14" L     (LC)

COM-10 Env.       #5 Envelope       COM 10   Landscape     No     Bin 1       No

(MP Tray)        9.5" W x 4" H      (MP)

COM-10 Env.       #5 Envelope       COM 10   Landscape     No     Bin 2       No

(LE Tray)        9.5" W x 4" H      (LE)

Monarch  Env.        Other         Monarch   Landscape     No     Bin 1       No

(MP Tray)      7.5" W x 3.875" H    (MP)

Monarch  Env.        Other         Monarch   Landscape     No     Bin 2       No

(LE Tray)      7.5" W x 3.875" H    (LE)

________________________________________________________________________________

________________________________________________________________________Page 14

EMBEDDING PRINTER COMMANDS

For features not supported by WordPerfect's printer definitions, printer

commands can be embedded into documents. The steps are as follows:

1.  Press [Shift] [F8], choose [O] Other, then [P] Printer Functions.

2.  Next, select [P] Printer Command, then [C] Command.

3.  Enter the printer command. To enter the escape control character, Ec, type

<27>.  Then enter the remainder of the printer command in its ASCII form.

There should be no spaces in the command!

Example:

Ec&l5X would be entered as <27>&l5X

4.  Press [Enter], then [F7], to return to your document.

________________________________________________________________________Page 15

COMMON PROBLEMS

This section provides a troubleshooting approach to solving many of the common

problems associated with using WordPerfect and the LaserJet IIP plus printer. In

most cases, these problems can be avoidedif you follow the guidelines presented

in the General Information and Common Tasks sections of this application note.

COMPUTER BEEPS WHEN PRINTING

Problem: The computer beeps and WordPerfect's Printer Control screen  [Shift]

[F7], [4]  displays the message Insert paper, press G to continue.  Cause: The

Prompt To Load setting for the Paper Size/Type you have Selected is set to Yes.

Solution: Change the Prompt To Load setting for the Selected Paper Size/Type to

No.

NEW FONT CARTRIDGE DOES NOT APPEAR ON MENU.

Problem: After you have used the PTR program to install a new font cartridge

with AutoFont support, the new cartridge does not appear on your printer's

Cartridges menu.

Solution: In the Select Printer menu, delete the name of your printer, then add

it again. Edit the printer definition and check your Cartridges menu. Your new

cartridge should be listed.  If it is not, redo the instructions in the Software

Application Notes that are packaged with your font product.

UNITS OF MEASURE

Problem: Some of the procedures (for example, creating forms and formatting

envelopes) listed in this application note do not work.

Cause: Your default units of measure may not be set to inches.

Solutions: 1) Include the quote mark (type 4" rather than 4) when entering the

margin and form size specifications listed in this application note. 2) Set your

default units of measure to inches.  Press [Shift] [F1], choose [E]Environment,

then [U] Units of Measure, press  [D]Display and Entry of Number, specify ["],

then press [F7] Exit.

INCORRECT FONT

Problem: Your document, or a portion of it, does not print in the correct font.

Causes:

1)  Did you install the correct printer definition and fonts?

2)  When selecting fonts, are you specifying the desired base font, or simply

specifying the size (e.g., large or very large)?

3)  If you are using a font cartridge, is the font cartridge properly inserted?

________________________________________________________________________Page 16

4)  If the soft font has been marked *, has the printer been initialized?

Solutions: 1) Repeat the installation procedures described earlier in this

document. 2) Rather than specifying size (e.g.,large or very large), press

[Ctrl] [F8], choose [F] Base Font, and select the desired font. 3) If the

font is a cartridge font, take the printer off line and insert the font

cartridge firmly you should hear and feel the cartridge click.  Place the

printer on line and try printing again.

MP LOAD `paper size'

Problem: Printer Displays MP LOAD in the first line of the display when sending

a print job to the printer.

Cause: The Job size (page size) command being sent to the printer does not match

MP SIZE on the front panel menu of the LaserJet IIP plus printer.

Solution: Change MP SIZE on the front panel menu of the LaserJet IIP plus

printer to match the Job size command being sent to the printer.

MISSING FORMS

Problem: At the Format: Paper Size/Type screen you selected [8] Other, but no

forms, such as Legal Paper (Tray),are listed.

Cause: The currently selected printer definition does not contain user-defined

forms.

Solutions:

1)   Press [Shift] [F7] and confirm that the proper printer definition has been

selected  press [S] to select a different printer.

2)  If no forms have been created, or if you reinstalled your printer, create

the necessary forms.  Refer to Creating Forms in the Common Tasks section

of this application note.

OTHER PROBLEMS

If you are having other types of problems or if the suggestions provided in this

section have not resolved the problem:

1.  Look at your document in the View mode and compare it to your hard copy.

2.  Read the General Information section of this application note.

3.  Try printing similar documents. Can the problem be isolated to one document?

If so, use WordPerfect's Reveal Codes function [Alt] [F3] and Search

function [F2] to find and delete invalid or misplaced format selections.

4.  Repeat the installation procedures. This will help ensure proper

installation of the correct printer definition.

________________________________________________________________________Page 17

CALLING FOR HELP

In case of further difficulty, refer to your WordPerfect Manual or call

WordPerfect Customer Support if you need assistance using WordPerfect.

If you still need assistance, call Hewlett-Packard's Personal Peripherals

Assist  Line.

Telephone:....................(208) 323-2551

Hours (Mon, Tue, Thu, Fri):...7:00am to 6:00pm MST

Wednesday:....................7:00am to 4:00pm MST

When you call Hewlett-Packard, please call from a phone next to your computer

and LaserJet printer. Assisting you often requires access to your printer and

WordPerfect application.  You should also have a current copy of the self-test

available.  This will allow you to quickly identify current  printer settings

for the support technician.

________________________________________________________________________Page 18