MICROSOFT WORD version 5.0

Chapter Contents

Microsoft Word Corporation.............................................2

Supported Fonts...................................................2

Unsupported Fonts.................................................2

Installation...........................................................3

Internal, Cartridge, and

Soft Font PRD Installation........................................3

Soft Font Installation............................................4

General Information....................................................4

Four Steps For LaserJet Printing..................................4

Identifying (Selecting) the Correct Printer

Description (PRD).................................................5

Identifying Supported Fonts.......................................7

Specifying the Correct Print Options..............................7

Common Tasks...........................................................9

Selecting Fonts...................................................9

Setting Page Length..............................................10

Landscape........................................................11

Legal Paper......................................................11

Printing Envelopes...............................................12

Printing Special Characters......................................13

Formatting And Printing Labels...................................14

Drawing Lines And Boxes..........................................15

Embedding Printer Commands.......................................15

Common Problems.......................................................16

Creeping Text....................................................16

PC Load Exec or PC Load A4 Message...............................16

PC Load C5.......................................................16

Incorrect Font...................................................16

Error 21 or Text Printing On One Line............................17

MP LOAD Page size Message........................................17

Other Problems...................................................17

_______________________________________________________________________Page 1

MICROSOFT CORPORATION

Customer Support Number                                (206) 454-2030

Hours                                       6:00am to 6:00pm PST

Charge                                                      None

Required Identification                                     None

Product Upgrade Policies and Procedures - SUPPLEMENTAL PRINTER

DISKETTES can be requested by calling Microsoft's Customer Service

Number or by calling a local dealer. The disks will be free if obtained

from a Microsoft dealer or will be $15.00 if ordered directly from

Microsoft Corporation.

Customer Service Number   (800) 426-9400

Supported Fonts

Cartridge Fonts 92286...

A, B, C, D, E, F, G, H, IC, J, K, L, M, N, P, PC, Q, R, T, U, V, W,

X, Y, Z, Z1A

Cartridge Fonts 92290...

S1, S2

Cartridge Fonts C2053A...

#C01, #C02, #C03, #C04, #C05, #C06, #C07, #C08

Cartridge Fonts C2055A...

#C01

Soft Fonts 33412...

AC, AD, AE, AF, DA, EA, RA, RB, SA,SB,

TA,TB, UA, UB

Super Fonts 33412...

AG, DC

Type Director...

All Typeface Products

Unsupported Fonts

Cartridge Fonts C2050B...

#C80,#C90

Cartridge Fonts C2050C...

#C80,#C90

Cartridge Fonts C2050D...

#C80,#C90

Advanced users can use Microsoft Word's MAKEPRD program

to edit or create printer description files for unsupported fonts.

Microsoft's MERGEPRD program can be used to combine printer

description files. Refer to Microsoft Word's documentation or call

Microsoft Customer Support if you need assistance.

_______________________________________________________________________Page 2

INSTALLATION

Using MS Word 5.0 with your LaserJet IIP plus printer requires

that you install the appropriate Printer Descriptions (PRDs). Microsoft

provides LaserJet printer PRDs which support the following:

Internal and Cartridge Fonts

Hewlett-Packard Soft Fonts

The paragraphs which follow outline the installation of MS Word's

internal font, cartridge font, and soft font PRDs. If you have not

installed MS Word, you must do so before proceeding  see

the Using Microsoft Word manual.

What is Required

Diskettes: PRINTER DISK 1,  PRINTER DISK 2, and UTILITIES/PRINTERS

disk along with the Additional HP LaserJet Support disks

Manuals: Using Microsoft Word and Microsoft Word Printer Information

The steps below will install all of the LaserJet printer PRD files

located on each disk. If you are familiar with MS Word and DOS, you

may want to install only those PRDs that are required for your particular

LaserJet printer configuration.

Internal, Cartridge, and Soft Font PRD

Installation

To install MS Word's internal, font cartridge, and soft font printer

description files:

1.Insert PRINTER DISK 1 in drive A.

2.At your DOS prompt, type:

COPY A:HPDWN*.*C:\WORD5 and press [Enter].2

3.  Insert the UTILITIES/PRINTERS disk in drive A.

4. At your DOS prompt, type:

COPY A:HP*.*C:\WORD5 and press [Enter].

5.  Insert PRINTER DISK 2 in drive A.

6.  At your DOS prompt, type:

COPY A:HP*.*C:\WORD5 and press [Enter]

7.  Insert SUPPLEMENTAL PRINTER DISK in drive A.

8.  At your DOS prompt, type:

COPY A:HP*.*C:\WORD5 and press [Enter].

_______________________________________________________________________Page 3

Microsoft Word 5.0 does not need to use the AutoFont Support

disks included with your new HP fonts. Instead, Microsoft has created

a printer driver (.PRD file) for each HP font product (bitmapped cartridges

and soft fonts, scalable cartridges and soft fonts). The supplemental

printer disks that contain these drivers are available from your local

software dealer. You can also order the supplemental printer disks

by calling Microsoft Customer Services at (800) 426-9400 or Hewlett-Packard

Driver Distribution at (303) 353-7650.

Soft Font Installation

Microsoft Word's printer descriptions for HP soft fonts automatically

download soft fonts. If you plan to use soft fonts with MS Word, follow

the steps below to install your soft fonts.

1. Insert HP soft font disk #1 in drive A.

2. At your DOS prompt, type:

COPY A:*.*C:\WORD5 and press [Enter].

3. Repeat steps 1 and 2 for each HP soft font disk.

You can use one of Hewlett-Packard's utilities (e.g., FontLoad,

Type Director) to download your soft fonts. Type Director may be used

to download your scalable soft fonts directly to the printer. You

can then instruct Microsoft Word 5.0 to skip the downloading procedure

when you print your documents.

If you plan to use Hewlett-Packard's soft fonts and want to

use another subdirectory to manage your fonts more effectively than

C:\WORD5, you must insert a path command in your autoexec.bat

file. This path statement (for example, path c:\fonts) must specify where

your fonts are located to allow Microsoft Word to find your font files.

GENERAL INFORMATION

The information which follows is applicable to most of the documents

you are likely to print using Microsoft Word 5.0 with your LaserJet

IIP plus printer. Read this information carefully as it will help you

avoid many of the common problems discussed later in this application note.

For printer feature support (fonts, page sizes, orientation,

etc.) do not use the printer's front panel. Microsoft Word will override

any front panel setting.

EXCEPTION: When using the MP Tray on the LaserJet IIP plus

change MP SIZE on the front panel menu to match the Page

size command being sent from the software.

Four Steps For LaserJet Printing

There are four basic steps to follow when printing Microsoft Word

documents on an HP LaserJet IIP plus printer:

Identify the Correct Printer Description

Plan Your Fonts

Create the Document

Specify the Correct Print Options

_______________________________________________________________________Page 4

These steps are briefly explained below. Follow each step carefully

and you will be able to print most documents quickly and easily.

Step 1

The first step requires that you identify the appropriate Printer

Description (PRD). Refer to Identifying The Correct Printer Description

section of this note to identify the correct PRD for any internal

fonts, cartridge fonts, or soft fonts you plan to use in your document.

Step 2

After you identify the correct PRD, you must plan which fonts you will use

in your document. In doing so, make sure the PRD you selected supports all of

the fonts you plan to use. The Identifying Supported Fonts section of this

note tells you how to determine which fonts are supported by a particular PRD.

Step 3

As you create your document, you will most likely change fonts, format

paragraphs, etc. The Common Tasks section of this note will

assist you with these tasks.

Step 4

Prior to printing your document, verify that you have the correct

print options specified. Refer to the section Specifying The

Correct Print Options of this note for the correct options.

Identifying The PRD ( Printer Description)

Use the tables below to  identify the correct printer description.

(Printer descriptions are contained in special files with a .PRD filename

extension. They are referred to in this text as the PRD.) For cartridge fonts,

compare part number on the cartridge with the part number in the table below.

Example:

For a LaserJet IIP plus printer, use the HPLASER1 printer

description. The fonts supported by this PRD most closely match the

internal fonts offered by this printer.

A PRD selected for the portrait orientation will also work

for landscape orientation, since the LaserJet IIP plus printer

can automatically rotate fonts to the desired orientation.

PRDs for Internal Fonts

Model                Portrait/Landscape PRD's

LaserJet IIP plus           HPLASER1

PRDs for Cartridge Fonts

HP Cartridge Part Number Portrait/Landscape PRD's

C2053A#C01     HPWP*

C2053A#C02     HPLASMS*

C2053A#C03     HPWSHEET*

C2053A#C04     HPRESENT*

C2053A#C05     HPFORMS*

C2053A#C06     HPBAR*

C2053A#C07     HPEQUATE*

C2053A#C08     HPGLOBAL*

C2055A#C01     HPGSTART*

_______________________________________________________________________Page 5

*This PRD is contained on Microsoft's SUPPLEMENTAL PRINTER DISKS.

Call Microsoft to order these disks.

PRDs for Cartridge Fonts

Cartridge Part Number    PRD Portrait/Landscape

92286A    HPLASER1

92286B    HPLASPS

or        HPLASER1

92286C    HPLASER1

92286D    HPLASER1

92286E    HPLASER1

92286F    HPLASRMN

or        HPLASER2

92286G    HPLASER1

92286H    HPLASER1

92286IC   HPINTL*

92286J    HPLASER1

or        HPLASER3

92286K    HPLASER2

92286L    HPLASER1

92286M    HPLASER1

92286N    HPLASER1

92286P    HPLASER2

92286PC   HPPRO

or        HPPROL

92286Q    HPLASER1

92286R    HPLASER2

or        HPLASER3

92286T    HPLASTAX

92286U    HPLASER2

92286V    HPLASLAN

92286W    HPLASER1

92286X    HPLASER1

92286Y    HPLASER

92286Z    HPLASMS

or        HPLASMSA*

or        HPLASER3

92286Z1A  HPLASMS2

92290S1   HPLAS2S1

92290S2   HPLAS2S2

*This PRD is contained on Microsoft's SUPPLEMENTAL PRINTER DISKS.

Call Microsoft to order these disks.

PRDs for Soft Fonts

Soft Font Part Number    PRD Portrait/Landscape

33412AC   HPDWNACP

33412AD   HPDWNADP

33412AE   HPDWNSFP*

33412AF   HPDWNR8P*

33412AG   HPDWNHHP*

33412DA   HPDWNLGP*

33412EA   HPDWNPRP*

33412RA   HPDWNGAP*

33412SA   HPDWNCNP*

33412TA   HPDWNZHP*

33412UA   HPDWNHLP*

_______________________________________________________________________Page 6

*This PRD is contained on Microsoft's SUPPLEMENTAL PRINTER DISKS.

Call Microsoft to order these disks.

Identifying Supported Fonts

This section explains how to determine which fonts are supported by

Microsoft's printer descriptions.

Information about supported fonts is available from two sources:

The Microsoft Word Printer Information Manual

The Microsoft Word Format Character Command Area

Your Microsoft Word Printer Information manual lists the internal,

cartridge, and HP soft fonts supported by each PRD. If you received additional

printer diskettes, refer to the documentation included with each disk.

You can also use the MS Word's Format Character command area

to determine which font names and sizes are supported.

1. Press [Esc][P]rint[O]ptions followed by [F1], highlight the desired

printer description (PRD), and press [Tab]. Highlight the Model field,

press [F1], and select the LaserJet IIP plus or LaserJet IIP option. Press

[Enter] and then press [Esc] to exit the Print command area.

If LaserJet IIP plus is not a selection in the[P]rint[O]ptions

Model field, select  the  LaserJet IIP option.  Selecting

LaserJet IIP will allow you to access the fonts and features of

the HP LaserJet IIP plus printer.

2. Press [Esc][F]ormat[C]haracter.

3. Tab to the Font Name field.

4. Press [F1] to see which fonts are supported

by the PRD selected in step 1.

5. To see what point sizes are supported, tab to the

Font Size field and press [F1].

6. MS Word will display the supported point sizes for

the font selected in step 4.

Specifying The Correct Print Options

Proper printing is dependent upon your specifying the correct print options.

To access MS Word's Print Options menu, press [Esc] [P]rint

[O]ptions. A screen similar to the following will appear.

PRINT OPTIONS printer:HPLASER1                              setup: LPT1:

model: LaserJet IIP                         graphics resolution: 150 dpi

copies: 1                                                 draft: Yes(No)

hidden text: Yes(No)                              summary sheet: Yes(No)

range:(All)Selection Pages                                 page numbers:

widow/orphan control:(Yes)No                             queued: Yes(No)

paper feed: Continuous                                   duplex: Yes(No)

_______________________________________________________________________Page 7

A brief discussion and the correct selections for some of these options

follows. Refer to the Microsoft Word Reference manual for a detailed

explanation of each option.

Printer

Specifies the name of the printer description (PRD) file. Press [F1] to

display a list of available printer names. Select the correct PRD for your

LaserJet printersee Identifying The Correct Printer Description earlier in

this section or refer to the Microsoft Word Printer Information manual.

Setup

Designates the port where the printer is attached. If the proper DOS

MODE commands have been entered into your AUTOEXEC.BAT file, selection

LPT1: will work regardless of which port has the printer connected to it.

Model

Designates the model of LaserJet printer being used with Microsoft

Word 5.0. If the LaserJet model you have is not listed, choose a LaserJet

model (i.e.,  LaserJet IIP or LaserJet Series II) with similar features.

Manual Feed

MS Word supports the following paper feed options:

LaserJet IIP plus:

Continuous  Multi-Purpose paper tray on the

LaserJet IIP plus printer or the optional Lower Cassette, if installed.

Bin 1:  Multi-Purpose paper tray on the LaserJet IIP plus printer.

Bin 2:  optional Lower Cassette on the LaserJet IIP plus printer, if installed.

Mixed:  prints the first page (e.g., letterhead) from the Multi-Purpose

paper tray; following pages from the optional Lower Cassette, if

installed.

Manual: allows manual feed of the paper through the Multi-Purpose tray.

Envelope:  used with envelope macro.

Paper Trays

If you have an optional Lower Cassette installed, the printer will

default to TRAYS=LC ONLY *.  With LC ONLY* selected,

if there is any paper in the MP tray, the printer will select that

paper until the MP Tray is empty.  The printer will then take paper

from the Lower Cassette.  To use the MP tray and the optional Lower

Cassette together, you will need to change the TRAYS setting

in the Printer Menu to one of the other settings. TRAYS=LC TRAY*

is usually best, but TRAYS=MP TRAY * and TRAYS=BOTH * also work.

If you are printing using the MP tray and send a printer command to select

a page size different from that on the front panel, the message MP LOAD  will

appear in the display. You must either change the MP SIZE item in the

printer menu or hold down the [Alt] key and click the [Continue] key.

_______________________________________________________________________Page 8

COMMON TASKS

This section will assist you with many of the tasks you are likely

to use with your LaserJet IIP plus printer.

For printer feature support (fonts, page sizes, orientation,

etc.) do not use the printer front panel selection. Microsoft Word

will override any front panel setting.

EXCEPTION: When using the LaserJet IIP plus MP Tray change

MP SIZE on the front panel menu to match the page size command

being sent from the software.

Selecting Fonts

To select a font, you must know whether MS Word supports it and which

PRD is used to access the font. This information can be found in the

General Information section of this note and in the Microsoft

Printer Information manual.

Fonts made by Type Director for MS Word have letter codes

added to the end of the font name that identify symbol set and font

class.  For example, CG_Times_US identifies the ASCII symbol set for

a standard bitmap font.  Compressed bitmaps are identified with _c

and scalable fonts are identified with an _s.  See Appendix A of the

Type Director User's Guide for more symbol set information.

Once you determine that the font is supported and which PRD to use,

follow these steps:

1. If you have not already done so, go to MS Word's

[P]rint [O]ptions command area and specify the appropriate

printer PRD. If the necessary PRD is not listed, install it according

to the Installation section of this note.

2. Highlight the text you want to format by selecting

[F6] and using the corresponding arrow keys.  For other highlighting

options, refer to your Using Microsoft Word manual for the

keystroke sequences.

3. Press [Esc][F]ormat[C]haracter.  This command area

allows you to choose the desired font attributes.

FORMAT CHARACTER

bold: Yes (No)               italic: Yes (No)         underline: Yes(No)

strikethrough: Yes (No)      uppercase:Yes (No)      small caps:Yes (No)

double underline:Yes (No)    position:(Norm)       Superscript Subscript

font name: Courier           font size: 12              font color:Black

hidden:Yes(No)

Bold, Italics and Underlining

Bold, italics, underlining, and double underlining are selected

by tabbing to the appropriate field and choosing [Y]es.

_______________________________________________________________________Page 9

Typeface

Typefaces (e.g., Courier, TmsRmn, Letter Gothic) are selected by tabbing

to font name and entering the desired font. Pressing [F1]

will display a listing of fonts supported by the PRD selected in step 1.

Point Size

A font's point size (10, 12, 24, etc.) is selected by tabbing

to font size and entering the desired point size. Press [F1]

to see a list of supported point sizes for the specified font name.

4. Press [Enter] to apply the selected format options.

5. If you selected a font with a point size greater than

12, you may need to change the line spacing in MS Word's Format

Paragraph command area. To modify the line spacing,

press [Esc] [F]ormat[P]aragraph, tab to the line

spacing field and enter one of the following:

1         Single line spacing, 12 point fonts

1.5       One and One-Half line spacing, 18 point fonts

2         Double line spacing, 24 point fonts

2.5       Two and One-Half line spacing, 30 point fonts

3         Triple line spacing, 36 point fonts

You may find it easier to specify Auto line spacing by highlighting

the line spacing field and pressing [F1]. This setting will  automatically

adjust line spacing to accommodate the largest font size within each line.

When using fonts that are larger than the default you will

need to set line spacing to auto. To do this press: [Esc]

[F]ormat [P]aragraph, highlight the line spacing

field, press [F1], highlight Auto, and press [ENTER].

Setting Page Length

Press [Esc] [F]ormat [D]ivision [M]argins.  This command area

allows you to choose the desired margins and page length.

FORMAT DIVISION MARGINS

top: 1"                        bottom: 1"

left: 1.25"                  right: 1.25"

page length: 11"              width: 8.5"              gutter margin: 0"

running-head position from      top: 0.5"              from bottom: 0.5"

mirror margins:Yes(No)                           use as default: Yes(No)

When you set the page length in MS Word's Format Division Margins

menu make sure you enter the correct value. Valid page lengths are:

Portrait:

Letter Paper   11 inches

Legal Paper    14 inches

Landscape:

Letter Paper   8.5 inches

Legal Paper    8.5 inches

_______________________________________________________________________Page 10

If you specify an invalid page length, your LaserJet IIP

plus printer may request the improper size of paper.

Landscape

This section will guide you through printing landscape with the LaserJet

IIP plus printer on letter size paper.

1. Press [Shift F10] to select your entire document.

2. Press [Esc][F]ormat[D]ivision[M]argins

and specify the following:

Top Margin      0.50" or greater

Bottom Margin   0.50" or greater

Page Length     8.5"

Page Width     11.0"

3. Press [Enter].

4. Press [Esc][P]rint[O]ptions and select

a PRD that supports the desired fonts.  Highlight the Model

field, press [F1], and select the LaserJet IIP plus

or LaserJet IIP and press [Enter].

5. Press [Esc] to exit the Print command area.

6. To print the document, highlight Printer and press   [Enter].

Legal Paper

The following sections will guide you through printing on legal size

paper in the portrait and landscape orientations.

1. Press [Shift F10] to select your entire document.

2. Press [Esc][F]ormat[D]ivision[M]argins,

specify the following and continue with steps 3 through 7:

Portrait Settings:

Top Margin     0.50" or greater

Bottom Margin  0.50" or greater

Page Length   14.0"

Page Width     8.5"

Landscape Settings:

Top Margin     0.50" or greater

Bottom Margin  0.50" or greater

Page Length    8.5"

Page Width    14.0"

3. Press [Enter] to exit the Margins menu.

4. Press [Esc][P]rint[O]ptions and select

a PRD that supports the fonts you plan to use in your document.

(See pages 2-5 through 2-8.)

_______________________________________________________________________Page 11

5. Press [Tab] to highlight the Model field.  Press

[F1] and select the LaserJet IIP plus or LaserJet IIP.

6. Press [Tab] to highlight the Paper Feed

option. Press [F1] and select one of the following:

Manual Feed....    select Manual

If using Manual Feed on the LaserJet IIP plus, you need to

click the [ON LINE] key to activate printing.

MP Tray or optional Lower Cassette w/Legal Tray...    select Continuous

(If you are using the MP Tray on the LaserJet IIP plus, you must change

the MP SIZE setting on the front panel of the printer to read MP TRAY=LEGAL.)

7. Press [Enter] and print the document.

Printing Envelopes

Microsoft Word 5.0 is able to print business and letter-size envelopes

using the envelope tray in the optional Lower Cassette on the LaserJet

IIP plus printer.  This section will assist you in manually

feeding envelopes, and using the  envelope tray in the optional Lower

Cassette on the LaserJet IIP plus printer. Use the face-up

output tray when printing envelopes.

Manual Feed

1. Type the desired address.

2. Press [Shift F10] to highlight the entire document.

3. Press [Esc] [F]ormat [D]ivision [M]argins and specify the following:

Top Margin     2.25"

Bottom Margin  0.50"

Left Margin    5.65"

Right Margin   0.50"

Page Length    8.5"

Page Width    11.0"

4. Press [Enter].

5. Press [Esc] [P]rint [O]ptions, press [F1], and select the desired PRD.

6. [Tab] to reach the Paper Feed option,

press [F1], and select the Manual option.

7. Press [Enter] and print the envelope,

then click the [ONLINE] key to print the envelope

The message displayed by the front panel or the printer will

be MF FEED LETTER instead of ME FEED COM-10.  The Microsoft Word driver

does not directly support manually feeding envelopes and requires

that the  envelope be created in a letter size format.

_______________________________________________________________________Page 12

LaserJet IIP plus Envelope Tray  (Lower Cassette)

1. Type the desired address.

2. Press [Shift F10] to highlight the entire document.

3. Press [Esc] [F]ormat [D]ivision [M]argins

and specify the following:

Top Margin     2.25"

Bottom Margin  0.50"

Left Margin    3.50"

Right Margin   0.50"

Page Length    4.0"

Page Width     9.5"

4. Press [Enter].

5. Press [Esc] [P]rint [O]ptions, press [F1],

and select the desired PRD.

6. Press [Tab] to highlight the Model field,

press [F1], select the LaserJet IIP plus  or

LaserJet IIP.

7. Press [Tab] to reach the Paper Feed option,

press [F1], and select the Envelope option.

8. Press [Enter] and print the envelope.

Printing Special Characters

Special characters can be printed if your LaserJet internal, cartridge,

or soft font supports the character and if Microsoft Word's PRD fully

supports that font.

Refer to your LaserJet printer manual to determine whether your printer's

internal fonts support a particular character or symbol. If you have

an optional cartridge font or soft font, refer to the documentation

that accompanied the font.

To determine the decimal value to be used to print your character,

the macro.gly file must be installed into Microsoft Word.

Installing Macros

1. Press [Esc] [T]ransfer [G]lossary

[M]erge [F1]. The macro.gly file should appear

on your screen.  Highlight macro.gly and press [Enter].  If

this file is not listed on your screen, insert Word's UTILITIES

2 Disk in drive A, enter A:macro.gly and press [Enter].

2. Press [Esc] [T]ransfer [G]lossary

[S]ave, enter c:\word5\macro.gly, and press [Enter].

This macro installation procedure does not need to be repeated in

the future unless you have trouble accessing the character macro.

_______________________________________________________________________Page 13

Once the macro is installed, do the following:

1. Press [Esc] [I]nsert followed by the key [F1].

Select the macro, character_test.mac, and press [Enter].

2. When prompted by Microsoft Word, select [N] unless you want a printout

of every font character available under the current printer description.

3. Next, select the font desired at the current time to obtain the ASCII

character decimal equivalents. Once the printout is finished, examine

these documents for the decimal value of the character desired.

4. Select [N] to prevent printing another character

decimal equivalent sheet, unless you need decimal equivalents for

another font. If you need a second printout, specify [Y] and

select the next desired font.

5. To obtain the character, hold down the [Alt] key

and enter the decimal value on the numeric key pad.

Formatting And Printing Labels

The steps and table which follow provide instructions and format settings

for adhesive labels available from Hewlett Packard, (800) 538-8787,

or Avery Corporation, (800) 535-3232, ext. 30.

1. Press [Esc][P]rint[O]ptions and select

a PRD that supports the desired fonts. [Tab] to the Paper

Feed option, press [F1], and select Manual.

2. Format your labels document according to the settings

provided in the following table. If you choose to adjust these settings,

make sure your top and bottom margins combined do not exceed one inch

and that you use only one format line.

Label Supplier              Hewlett       Avery        Avery

Packard

Part Number                 92296A         5161         5162

Label Dimensions          (2-5/8" x1")   (4"x1")   (4"x1-1/2")

Margins [Esc]             [F]ormat [D]ivision [M]argins

Top Margin               .60"      .60"       .25"

Bottom Margin            .25"      .25"         0"

Left Margin              .25"      .15"       .25"

Right Margin               0"        0"         0"

Layout [Esc]              [F]ormat [D]ivision [L]ayout

Number of Columns             3         2          2

Space Between Columns       .40"      .40"       .50"

3. Press [Esc] [P]rint [P]rinter to print.

4. For the LaserJet IIP plus you must install

the face up output tray.

5. When prompted by the LaserJet IIP plus, load

the labels, face down in the MP Tray, set the output selection to

face-up output then click the [ON-LINE] key.

_______________________________________________________________________Page 14

4 Line Label

Addresses

For 4 line addresses, use the (2-5/8"x1") labels or the (4"x1") labels.

Separate each address with 2 blank lines.

6 Line Label

Addresses

For 6 line addresses, use the (4"x1-1/2") labels. Separate each address

with 3 blank lines.

Drawing Lines And Boxes

Line and box drawing in Microsoft Word 5.0 is accessed by pressing

[Control F5] and using the cursor keys to draw. These characters

are resident in the LaserJet IIP plus printer's PC-8 symbol

set.  Microsoft Word's line and box drawing feature will work with

the PC-8 symbol set if you use the HPLASER1.PRD and select the font

name CourierPC.

Embedding Printer

Commands

For features not supported by MS Word's PRDs, printer commands can

be embedded in documents. The steps are as follows:

1. To enter the escape control character in Word 5.0,

hold down the [Alt] key and use your computer's numeric keypad

to enter 027. If you make this entry correctly, a left pointing arrow

will appear on your screen when you release the [Alt] key.

2. Immediately following the  symbol, enter

the remainder of the printer command in its ASCII form. There should

be no spaces in the command!

Example

To embed the printer command "Esc(1U" for the legal symbol

set, hold down the [Alt] key and enter 027 on your computer's

numeric keypad. Release the [Alt] key and enter the remaining

ASCII characters, (1U.

_______________________________________________________________________Page 15

COMMON PROBLEMS

This section provides a troubleshooting approach to solving many of

the common problems associated with using Microsoft Word 5.0 and your

LaserJet IIP plus printer. In most cases, these problems can

be avoided if you follow the guidelines presented in the General

Information and Common Tasks sections of this application note.

Creeping Text

Problem: The first page of your document prints properly, but

subsequent pages have larger and larger top margins (i.e., creep down).

Causes: 1) Failure to specify the correct printer. 2) Invalid

page length specification.

Solutions: 1) Press [Esc][P]rint[O]ptions[F1] and select the correct LaserJet

PRD (e.g., HPLASER1) . 2) Press [Esc][F]ormat[D]ivision[M]argins and specify a

valid page length setting. See the Setting Page Length section of this note.

MP Load Exec or MP Load A4 Message

Problem: The LaserJet IIP plus printer displays an "MP

Load Exec or MP Load A4" message when you print your document.

Causes: 1) Invalid page length specification.  2) Improper Form

(lines-per-page) setting on the LaserJet printer control panel.

3) Incorrect LaserJet model chosen.

Solutions: 1) Press [Esc][F]ormat[D]ivision[M]argins and specify a

valid page length setting. Refer to the Setting Page Length section

of this note.2) Take the LaserJet IIP plus off line and, while holding

down the [ALT] key, hold down the [ON-LINE/RESET] key until 09 MENU RESET

displayed.  3) Press [Esc][P]rint[O]ptions, highlight the Model field,

press [F1], and select the LaserJet IIP plus or LaserJet IIP  model.

MP Load C5

Problem: The LaserJet printer displays an "MP Load C5" message

when you print your document.

Cause: Incorrect paper feed choice.

Solution: Press [Esc] [P]rint [O]ptions, highlight

the Paper Feed field, press [F1], and select Continuous.

Incorrect Font

Problem: When you print the document, some or all of the text

is printed in an incorrect font.

Causes: 1) The font cartridge containing the desired font(s) is not firmly

inserted in the printer. 2) You did not select the font(s) properly.

3) The soft font files(s) are not copied to your MS Word subdirectory.

Solutions: 1) Take the printer off line and re-insert the font cartridge. Push

firmly until the cartridge clicks into place. 2) Refer to the Selecting Fonts

section of this note. 3) Refer to the Soft Font Installation section of this

note.

_______________________________________________________________________Page 16

Error 21 or Text Printing On One Line

Problem: The LaserJet printer displays a "21 PRINT OVERRUN"

message or lines of text are printed on top of each other.

Cause: Line spacing specified as 0.

Solution: Press [F10] to highlight the problem paragraph,

then press [Esc] [F]ormat [P]aragraph and specify a valid

line spacing value.

MP LOAD `page size' on the LaserJet IIP plus

Problem: Printer Displays MP LOAD `page size'  when

sending a print job to the printer.

Cause: The page size command being sent to the printer does

not match MP SIZE on the front panel menu of the printer.

Solution: Change MP SIZE on the front panel menu of

the printer to match the page size command being sent to the printer.

Other Problems

If you are having other types of problems, or if these suggestions

have not resolved the problem:

1. Read the General Information and Common

Tasks sections of this application note.

2. Try printing similar documents. Is the problem isolated

to one document? If so, find and delete invalid or misplaced format

selections in the problem document.

3. Repeat the installation procedures. This will help

ensure proper installation of the correct Printer Resource Description

(PRD) file.

4. Refer to your Microsoft Word manuals or call Microsoft

Customer Support at (206) 454-2030 if you need assistance in using

MS Word's functions.

5. Call Hewlett-Packard's LaserJet Assist Line.

Telephone                                (208) 323-2551

Hours (Mon, Tue, Thu, Fri)         7:00am to 6:00pm MST

Wednesdays                         7:00am to 4:00pm MST

When you call Hewlett-Packard, call from a phone next to your

computer and your LaserJet printer. You should have a recent self

test printout available.  This will allow you to quickly identify

your panel settings for the Hewlett-Packard technician assisting you.

Assisting you may also require access to your Microsoft Word documentation.

_______________________________________________________________________Page 17