PORT FINDER Version 2.20 by mcTRONic Systems

PORT  FINDER is a very versatile program.  It  will  display your  com-

puter   type,  BIOS  date, DOS version and display adapter  type.   It  will

detect  and  display  COM1 to COM4 and LPT1 to LPT3 ports that are  respond-

ing with the actual HEX address.  Because PORT FINDER displays the HEX  port

addresses  you can also use PORT FINDER as a diagnostic tool.  You can  swap

COM  or LPT ports addresses instead of swapping cables.   If you  are  using

DOS  3.3 or higher  and have COM3  and/or COM4, PORT FINDER  will   activate

both   COM3   and/or  COM4.   DOS  will then recognize these ports  and  you

will  be able to  use the  DOS  MODE command.  Most system information  type

programs  such as the Norton Utilities System Information (SI) program  will

correctly  indicate the number of serial ports that you have  including  the

COM3 and/or COM4 ports after you run PORT FINDER.   Any other  programs that

operate   through  DOS should also accept  COM3 and/or  COM4 after  you  run

PORT  FINDER (if the program does not allow you to enter COM3 or  COM4  then

you  will need to swap COM3 or COM4 with COM1 or COM2 before you run such  a

program then select either COM1 or COM2 in said program).  If you have a new

IBM PS/2 computer COM3 and/or  COM4 will already be activated .  You can use

PORT  FINDER  to reset both COM and LPT port addresses in  case  you  forget

what addresses have been swapped.  PORT  FINDER can also be used in a  BATCH

file   to  detect which COM ports  are present.    An  Errorlevel   is   set

for  all possible  combinations.   An  example  batch file  COMMTEST.BAT  is

included.  PORT FINDER was listed as the more versatile of two programs that

where mentioned in the Advisor Column in the Oct. 17 1989 issue of PC  Maga-

zine (The best PC Magazine that you can take).

Any  program  that directly addresses the COM or  LPT  ports will   not

be  affected by PORT FINDER.  Many communication programs like ProComm  Plus

do  directly  address the hardware and do not need PORT FINDER to  make  the

COM3 or COM4 port work, nor will Procomm Plus be affected in any way by Port

Finder.   PORT  FINDER can still be used to install the COM3  or  COM4  port

address  so  that DOS does know it is there and so your port  count  is  set

properly.   Any program that uses  DOS to  address  the  COM or  LPT  ports,

such  as  any basic or compiled basic program and many mouse  drivers,  will

work  with  PORT FINDER, allowing  you to  install COM3 or COM4 and swap any

port addresses that you like.  Activation of COM3 or  COM4 port does require

DOS  3.3 or higher  and a  COM3 or COM4 port.  Swapping COM  port  addresses

would  be  required with any program that does go through DOS but  who  will

allow you to only enter COM1 or COM2.

PORT  FINDER  requires  no run time parameters, to  display  responding

COM  and  LPT ports  on all  systems,  or to activate alternate COM ports on

PC/XT/AT  systems.  DOS 3.3 and higher only supports  access  to a   maximum

of  4   COM  and  3 LPT ports. The optional  PORT  FINDER   parameters   for

parm1  and/or  parm2  with descriptions are:

d:>PF [parm1] [parm2]

H   - Display this help screen       C23 - Swap COM ports 2 and 3

RC  - Reset COM port addresses       C24 - Swap COM ports 2 and 4

RL  - Reset LPT port addresses       C34 - Swap COM ports 3 and 4

C12 - Swap COM ports 1 and 2         L12 - Swap LPT ports 1 and 2

C13 - Swap COM ports 1 and 3         L13 - Swap LPT ports 1 and 3

C14 - Swap COM ports 1 and 4         L23 - Swap LPT ports 2 and 3

NOTES:

All  parameter  letters  can be upper or lower case.  The COM and   LPT

port  addresses displayed   always  indicate  the  final location after  all

port   swaps  and resets  have been executed.  Enter all port  numbers  with

low first then high.

I highly recommend that you not leave any address holes  for either the

COM  or  LPT ports.  As an example do not have only COM1 and  COM3  with  no

COM2.   Do not have a LPT2 and LPT3 with no LPT1.  If you have one  port  it

should  be 1,  two ports should be 1 and 2 and  so forth.  Swap  only  ports

that you have.  While nothing  is wrong with testing out a port address  you

do not have by swapping an  address  you  have to it,  you and  your  system

may  become confused.   Use  the RC or RL functions to restore  all   stand-

ard port  address  locations.

TECHNICAL DATA:

DOS assigns COM and LPT numbers, from LOW to HIGH, to all responding

I/O   ports  in  order  as  they  are  found  and non- responding I/O  ports

addresses are skipped.

MACHINE         COMx             I/O            INT.        MEM

ALL             COM1            3F8 HEX         IRQ4       40:00

ALL             COM2            2F8 HEX         IRQ3       40:02

PC/XT/AT        COM3            3E8 HEX         IRQ4       40:04

PC/XT/AT        COM4            2E8 HEX         IRQ3       40:06

PC/XT/AT        COM5            2E0 HEX         IRQ4       NONE

PC/XT/AT        COM6            3E0 HEX         IRQ3       NONE

PS/2            COM3           3220 HEX         IRQ3       40:04

PS/2            COM4           3228 HEX         IRQ3       40:06

PS/2            COM5           4220 HEX         IRQ3       NONE

PS/2            COM6           4228 HEX         IRQ3       NONE

PS/2            COM7           5220 HEX         IRQ3       NONE

PS/2            COM8           5228 HEX         IRQ3       NONE

SERIAL PORT COUNT IS AT MEM 40:11 BITS 2, 3, AND 4.

ErrorLevel  COM1  COM2  COM3  COM4     ErrorLevel COM1  COM2  COM3  COM4

0        0     0     0     0           8        0     0     0     X

1        X     0     0     0           9        X     0     0     X

2        0     X     0     0          10        0     X     0     X

3        X     X     0     0          11        X     X     0     X

4        0     0     X     0          12        0     0     X     X

5        X     0     X     0          13        X     0     X     X

6        0     X     X     0          14        0     X     X     X

7        X     X     X     0          15        X     X     X     X

(X means COMx is there)

PORT ADDRESS     MEM

LPT1   3BC HEX       40:08 (may be included on MDA or other card)

LPT2   378 HEX       40:0A (may be addressed as LPT1 on PC,XT,AT)

LPT3   278 HEX       40:0C (may be addressed as LPT2 on PC,XT,AT)

Normal Installation:

If  you are using Port Finder to activate COM3 or COM4 then you  should

copy  Port Finder to your boot disk in the root directory or  any  directory

that is in your path statement if you run Port Finder after your path state-

ment.   Add the command "PF" to your AUTOEXEC.BAT file after the path if  it

is required or add a path in front of the PF name if you prefer.  If you are

using  Port Finder to only swap ports in batch files and the like  then  put

Port  Finder in your DOS or UTILITY sub-directory of your hard disk or  just

on your boot disk if you have a floppy based system.  Port Finder should  be

in your path statement if you have one.

Ordering Information:

PORT   FINDER  is guaranteed for 90 days to  registered  users.    PORT

FINDER  costs just $10 (plus $2.50 shipping in the US or $7.50  outside  the

US, US funds only please!) sent to you First Class Mail.  So order your copy

today!  Send your check or money order to my address listed below with  your

mailing address and which product that you are ordering.

Thank You,

James D. McDaniel

Owner/Programmer

mcTRONic Systems

7426 Cornwall Bridge Ln.

Houston, TX  77041-1709

(713) 462-7687

Hours: 5-10 PM M-F CST

*   If  you  need  a good mail list  program  then  send  for  my  shareware

program MAILALL.   It is the best and  least expensive mail list program you

can  buy  for a small company or personal use.  Also each  copy  of  MAILALL

includes a copy of PORT FINDER!

MAILALL will maintain the names of up to 20 address files (per address  name

file as you can have more than one) each with  4000 addresses on a hard disk

or 1440 addresses per  floppy disk.   There are 12 data fields and you   can

sort the  address file by any two that you wish.  MAILALL will print  Labels

(up   to four across), Reports, and Envelopes.  You can edit or  print   all

functions   based  on address search matches from one to   all   12  fields.

MAILALL  can import or export WORDSTAR  mailmerge  data files and can export

MULTIMATE  ADVANTAGE II mailmerge files.  MAILALL is network compatible  and

supports  any Microsoft compatible mouse.  The user can select  any  printer

port that he would like and the program colors are also user selectable.  It

cost  just $27.50 shipped FIRST CLASS MAIL to you.  Send  for  it today!

PORT FINDER, COPYRIGHT (C) 1988, 1989 by mcTRONic Systems

MAILALL, COPYRIGHT (C) 1987, 1988, 1989 by mcTRONic Systems

DOCUMENT DATE:  September 27, 1989

##################################################################################################