CONTENTS

1.  DESCRIPTION..........................................................   2

2.  EXIT STATUS..........................................................   4

3.  COMMANDS.............................................................   5

3.1   List of Commands...............................................   6

3.2   Interrupt Keys.................................................  11

4.  NUMERIC PARAMETERS...................................................  12

5.  ENVIRONMENT VARIABLES................................................  15

6.  EXAMPLES.............................................................  18

7.  IN CASE OF DIFFICULTY................................................  19

7.1   LOCKUPS........................................................  19

7.1.1   Insufficient Memory  19

7.1.2   Environment TOO BIG  19

7.1.3   Bad COMM Port Configuration  20

7.2   Incorrect Batch Files..........................................  20

7.3   Cheap Internal Modems..........................................  20

7.4   Commands Entered in Upper Case.................................  21

7.5   No Carrier Detect..............................................  21

7.6   PKARC 3.6......................................................  21

7.7   DOS 3.x........................................................  21

7.8   Perstor Controllers............................................  21

7.9   IBM PS/2 Model 50Z.............................................  22

7.10  Brain Damaged UARTS............................................  22

7.10.1  Everex Internal Modems  22

7.11  Weird Iron.....................................................  23

7.12  Will the real YMODEM please stand up?..........................  23

7.13  286 and 386 Extended Memory....................................  23

8.  BACKGROUND OPERATION.................................................  24

9.  CALLING DSZ FROM COMM PROGRAMS.......................................  26

9.0.1   Ms-Kermit 3.00  26

9.1   GT POWER 13.00/14.00...........................................  26

9.2   PROCOMM PLUS...................................................  26

9.3   Qmodem.........................................................  27

9.4   PROCOMM........................................................  28

9.5   BULLETIN BOARDS................................................  28

10.  FLOW CONTROL.........................................................  29

11.  ERROR MESSAGES.......................................................  30

11.1  Got ZRPOS......................................................  31

11.2  Serial Input Error: Line Status Register xx ...................  32

11.3  COMMAND NOT FOUND..............................................  33

11.4  Out of Environment.............................................  34

12.  SLOW TRANSFERS.......................................................  35

13.  ASSOCIATED DOCUMENTS.................................................  36

13.1  ZMODEM Developer's Collection..................................  36

14.  FILES................................................................  37

15.  CHANGES..............................................................  38

16.  Other Omen Technology Products.......................................  44

17.  FAIR PLAY............................................................  45

17.1  Sharing DSZ Files..............................................  45

17.2  REGISTRATION...................................................  47

17.3  Files on Registration Disk.....................................  48

17.4  Bulletin Board Registration....................................  49

18.  ACKNOWLEDGEMENTS.....................................................  50

Chuck Forsberg

Omen Technology Inc

ABSTRACT

ZMODEM-90(TM) - Not just another XMODEM hack

DSZ is one of many available programs that implement the ZMODEM file transfer

protocol.

DSZ's ZMODEM provides advanced file management features including two levels

of Intelligent Crash Recovery(TM), and flexible control of selective file

transfers.  Omen Technology's Professional-YAM Communications Tools also

provide security verified command downloading.

The ZMODEM file transfer protocol provides reliable file and command transfers

with complete END-TO-END data integrity between application programs.

ZMODEM's 32 bit CRC protects against errors that sneak by "error free" modems

and even the most advanced networks.

ZMODEM safeguards all data and supervisory information with effective error

detection.  (XMODEM and many recently introduced protocols do not protect

vital supervisory data.)

ZMODEM rapidly transfers files, particularly with buffered (error correcting)

modems, timesharing systems, satellite relays, and wide area packet switched

networks.  ZMODEM's streaming operation eliminates delays caused by

Kermit/XMODEM/YMODEM/JMODEM block acknowledgements.  ZMODEM eliminates the

classic XMODEM/YMODEM/JMODEM PACKET LENGTH compromises between transfer

efficiency and error recovery.  ZMODEM's packet length is the entire file.

User Friendliness is an important ZMODEM feature.  ZMODEM AutoDownloadm

(Automatic file Download initiated without user intervention) greatly

simplifies file transfers compared to traditional protocols.

DSZ - a ZMODEM-90(TM), True YMODEM(TM), XMODEM File Transfer Program

Omen Technology Inc

Omen Technology Incorporated

The High Reliability Software

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

TeleGodzilla BBS: 503-621-3746 Speed 1200,2400,19200

FAX: 503-621-3735

CompuServe:70007,2304  GEnie:CAF

UUCP: ...!tektronix!reed!omen!caf

*************************************************************************

There's something special about DSZ registration.  With most programs,

registration applies only to the current version.  DSZ isn't like that.

DSZ registration bypasses the shareware announcement and unlocks the advanced

features in all past and future versions of DSZ.  You don't have to pay and

pay again for new versions.  Omen's Demand Upgrade(TM) technology allows you

to download and activate future DSZ versions without further registration!

*************************************************************************

Chapter 1 Pg 2           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

1.  DESCRIPTION

DSZ is a "protocol driver" that uses the ZMODEM-90(TM), True YMODEM(TM), or

XMODEM protocol to transfer files over a serial port with a variety of

programs.

ZMODEM Transfers feature 32 bit CRC, advanced file management, and Crash

Recovery, ZMODEM Crash Recovery can be used to complete interrupted file

transfers started with any protocol.[1]

DSZ has two main commands for ZMODEM transfers: "sz" sends file(s), and "rz"

receives them.

When calling bulletin boards to download files with ZMODEM you can avoid the

bother of starting DSZ for each transfer by calling DSZ first, before asking

the BBS for the download.

dsz t

The "t" command allows you to keyboard the file request directly to the BBS.

(The "t" stands for "terminal emulation".) Just give the "dsz t" command from

DOS or your comms program's DOS GATEWAY.  DSZ's ZMODEM AutoDownload then

receives the file automatically, using the filename provided by the BBS,

without any wasted keystrokes or delay.  Try it, you'll like it!  (The "t"

command is described below.)

YMODEM downloads are almost as easy.  Use the "t" command as shown above, and

hit the F3 key after telling the board to download a file with YMODEM.

ZMODEM and True YMODEM(TM) transfers preserve exact file length and

modification time.

DSZ supports XMODEM-1k (XMODEM with 1024 byte blocks, sometimes mislabeled as

ymodem) and CRC-16 with the sx -k and rc commands.

DSZ is derived from Omen Technology's "Professional-YAM" high reliability

communications tools (also known as PowerCom(TM)) and Omen Technology's ZCOMM

User Supported Communications Software.  DSZ supports a subset of Pro-YAM's

file transfer related commands.  This documentation makes extensive references

to contemporary editions of the Pro-YAM or ZCOMM manual, and is incomplete

without it.  (Registered copies include an alternate, copyrighted file.)

DSZ is usually called from a communications or bulletin board program.

DSZ uses its own modem I/O.  The original interrupt vector, UART parity, and

__________

1. Provided the other protocol did not write false data to the file.

Chapter 1 Pg 3           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

interrupt configuration are restored on exit.  The original state of the 8259

interrupt mask bit for this vector is also restored on exit.

DSZ uses DOS standard output (stdout) for messages.  DSZ never writes directly

to the screen.  DSZ expects its standard output to point to the console, a

logging file, or NUL, but never the modem port.  If dsz's standard output is

redirected to the modem (as with ProComm's Host mode DOS shell), the

combination will lock up the computer.  When necessary, dsz's standard output

can be redirected back to the console device with the CON command.

Users of pcAnywhere must use dsz's A command to disable pcAnywhere host mode

during file transfers.

Chapter 2 Pg 4           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

2.  EXIT STATUS

DSZ exits with 0 status for successful transfers, or the value of the Q

parameter (default 1) if a file transfer failed, or if no files were

transferred.  Many languages and "door" programs do not return this

information to the calling program.  Some languages and "door" programs freak

out (hang) on certain non-zero exit values.

A batch file that calls DSZ directly may be used to detect the DSZ exit status

with the DOS "errorlevel" construct.  Please refer to the Q numeric parameter

below for more information.

DSZ's log file entries may also be examined to determine the success or

failure of a file transfer.

Chapter 3 Pg 5           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

3.  COMMANDS

DSZ commands are an improper subset of Pro-YAM/PowerCom/ZCOMM commands.  The

file transfer commands are fully described in ZCOMMDOC.ARC, the ZCOMM User's

Manual.  Relevant portions of the ZCOMM User's Manual are provided in

DSZBG.ZOO, a Copyrighted file included with the DSZ registration disk.  You

may refer to one of those documents for details on the following commands.

Chapter 3 Pg 6           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

***********************************************************

NOTE WELL

Except as noted, all commands and options must be typed in lower case.

DO NOT USE UPPER CASE!!

Case is significant!

***********************************************************

Optional commands or arguments are shown in square brackets [].

When used (in any combination), the A, CON, port, speed, handshake, estimate,

and d commands must be used in exactly this order.

dsz [A] [CON] [port N] [speed S] [handshake XX] [D] [d] command

3.1  List of Commands

A     Suspend pcAnywhere operation during file transfers.  The A command is

used to avoid lockups caused by pcAnywhere host mode.  This command

should be given to the DSZ running under pcAnywhere host mode.

EXAMPLE: C>dsz A sz *.foo Sends files from a pcAnywhere hosted machine

to you.

Fm: Al de la Torre

1 sending from host to remote

type: "dsz A sz filename.ext" ENTER

shell to dos on remote machine (ctrl-Rshift)

type: "dsz A rz" ENTER

2 sending from remote to host

type: "dsz A rz" ENTER

shell to dos on remote machine (ctrl-Rshift)

type: "dsz A sz filename.ext" ENTER

CON   Redirect message output to the DOS "CON"  (console)  device,  overriding

any previous redirections.  This is necessary to avoid lockups caused by

ProComm host mode.  Most  programs  do  not  require  this  hack.   This

command is unique to DSZ.

port N select port N (n = 1,2,3,4,5,6,7,8, 10,11,12,13,14,15,16,17,18) Default

is  port  1  (COM1)  or  read from the DSZPORT dos environment variable.

COM11 to COM18 refer to the COM1 to COM8 ports on the PS/2.

*****************************************************************

NOTE

Only COM1  and  COM2  are  standard  on  all  PC  compatible  computers.

Multiple  standards  exist  for COM3 and above, so use only COM1 or COM2

Chapter 3 Pg 7           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

when first installing DSZ.

*****************************************************************

DSZ COM3 uses 3E8 and IRQ4.  DSZ COM4 uses 2E8 and IRQ3.

DSZ COM7 uses 3E8 and IRQ5.  DSZ COM8 uses 2E8 and IRQ5.

If you wish to use COM3 or COM4 and your computer is not wired according

to  one  of  these  choices, you MUST use the "portx" command to specify

base address and interrupt vector.

EXAMPLE: to use COM2:

dsz port 2 rz

In multitasking operation, accessing a  port  used  by  another  running

program will hang the machine.

portx base,irq To select an arbitrary port address and interrupt  vector,  the

portx command must be used instead of the port command.  Please refer to

DSZBG.DOC (provided on the Registration Diskette) or  the  Professional-

YAM or ZCOMM (ZCOMMDOC.ARC) manuals for details on this command.

EXAMPLE: dsz portx 3e8,4 speed 2400 rz

speed S Set the specified speed.  This command is rarely needed because DSZ

reads the port's current speed by default.  Please refer to the FLOW

CONTROL, MAIN COMMANDS and SOFTWARE COEXISTENCE chapters in DSZBG.ZOO or

the ZCOMM or Pro-YAM manual for important information about the speed

command.

Please study all the chapters of this document when using speeds above

2400 bps.

Officially, DSZ supports a maximum speed of 38400.  However, you are

welcome to experiment with higher speeds up to 115200.

handshake XX Enable one of several choices of hardware handshake.  (The

default is XOFF/XON software handshake).  This command is documented in

the ZCOMM or Pro-YAM manual, and in DSZBG.DOC.  (The ZCOMM manual is

available on all bulletin boards registered for DSZ as ZCOMMDOC.ARC.) On

Chapter 3 Pg 8           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

DSZ the "handshake" command may be abbreviated to "ha".

D     Disables the modem (drops Data Terminal Ready, DTR) if carrier is lost.

This may be used to prevent spurious events (phone ringing) from

interfering with the calling bulletin board's recovery from carrier

loss.

d     Disables carrier dropout monitoring, for modems that do not properly

drive the "Carrier Detect" input on the PC.  If you see a No Carrier

Detect message, you need to give the d command.  Before using the d

command, make sure you have the proper port selected!!!

estimate length [speed] Use the specified length and optional specified

effective transfer speed for the next receive file to be created.  This

command must come after any port, speed, or handshake commands.  Please

refer to DSZBG.ZOO or the ZCOMM User's manual for details.

The following makes DSZ calculate file transfer times on the basis of

290 character per second aggregate transfer rate:

EXAMPLE: dsz port 2 speed 9600 estimate 0 2900 rz

EXAMPLE: dsz port 2 speed 9600 estimate 0 2900 sz bimama.zip

pxN   Set a numeric parameter x to N.  These parameters are mostly a subset of

the Pro-YAM/ZCOMM numeric parameters, and are listed below.  They assume

their defaults each time DSZ is called.  Without an argument, p displays

the numeric parameters and their values.

EXAMPLE: C>dsz p

z pxN Set zmodem numeric parameter x to N.  These parameters correspond

to the Pro-YAM or ZCOMM "zmodem" parameters and rarely need changing.

These parameters assume their defaults each time DSZ is called.  Without

an argument, z p displays the zmodem numeric parameters and their

values.

restrict Restrict pathnames to the current disk and directory tree, and

disallow overwriting of existing files.  When DSZ is restricted, it will

refuse to transfer files containing the string autoexec.bat and

command.com in upper or lower case.  This provides some defense from

malicious uploaded files.

Chapter 3 Pg 9           DSZ Reference Rev Jan-28-90           Typeset 1-28-90

t     [-T] [-8g] Talk with ZMODEM AutoDownload.

The optional -T forces DSZ to exit after it has recognized a ZMODEM

AutoDownload(TM) request.

The optional -8g (available on registered copies only) enables reception

of "BBS (ANSI) graphics" displays.  (A video device driver such as

ANSI.SYS must be loaded to support some types of graphics.)

A small subset of ZCOMM/Pro-YAM term function subcommands are

recognized:

Keyboard F1 or ALT-X to exit.

F3 receives file(s) with True YMODEM(TM),

ALT-P toggles parity between the 8n default and 8g (enables ANSI

"graphics").

ALT-H toggles two local echo ("Half Duplex") modes; the second displays

a newline (CR/LF) when Enter is keyboarded.

Keyboarded ASCII characters are sent to the modem.

ALT-B sends a break.

Characters from the modem print on the screen.  A device driver such as

ANSI.SYS may be used to recognize escape sequences.

DSZ's ZMODEM AutoDownload(TM) detects the start of a ZMODEM file

download and accepts the file(s) without operator intervention.

rz    [-abeEmMnpPrt+YZy] [dir] [file ...]     (ZMODEM-90 receive) **

rz    [-abenprt+Yy] [dir] [file ...]          (ZMODEM receive) **

rb    [-abt+y] [dir] [file ...]     (YMODEM receive) **

rb -g [-abt+y] [dir] [file ...]     (YMODEM-g receive) **

rc    [-abt+y] file1                (XMODEM/CRC receive)

ro    [-abt+y] file1                (XMODEM OverThruster receive)

rx    [-abt+y] file1                (XMODEM receive)

rx -g [-abt+y] file1                (Qmodem-G receive) **

** Download directory specification and -g ignored w/o registration

Chapter 3 Pg 10          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

sz    [-abefnprtFYQS+yZ] [-w[N]]  [PREFIX=p | ONAME=n] file ...(ZMODEM send)

sz    [-abeEfmMnpPrtFYQS+yZ] [-w[N]]  [PREFIX=p | ONAME=n] file ...(ZMODEM-90

send)

Pathnames are separated by spaces.  Wildcards are OK.

EXAMPLE: dsz port 2 sz -r /tmp/file1 /tmp/file3 *.c

The "sz" and "sb" commands exploit DOS's tree structured file system.  A

directory or disk specifier expands to all files in that directory.

EXAMPLE: dsz port 2 sz -r C:srcdir

If you use PREFIX= or ONAME= with the "sb" or "sz" commands, these

pathnames should be entered in lower case unless the underlying file

system supports upper AND lower case.

EXAMPLE: dsz port 2 sz -r ONAME=newsrc.c temp.c

sb    [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM or YMODEM-g send)

sb -k [-ft] [PREFIX=p | ONAME=n] file ...     (YMODEM-1k or YMODEM-g send)

sx    [-t] file1                    (XMODEM, XMODEM/CRC, Qmodem-G send)

sx -k [-t] file1                    (XMODEM-1k send)

v     Verify the serial number by writing a log entry (see DSZLOG).

v      1     0 bps    0 cps   0 errors     0  128 s/n 666

The letter "v" indicates a verification entry.  The first numeric field

is 1 if YMODEM-g is supported.  The last field is the serial number.

Unregistered copies show a s/n of 0.

o     Turn off Data Terminal Ready (DTR), terminating ("offing") the

connection if the modem responds to DTR.  Useful with the "t" command to

disconnect afterwards.

EXAMPLE: dsz t o

Chapter 3 Pg 11          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

LARG  Convert ARG to lower case, then parse arg.

The meanings of optional arguments to the file transfer commands are described

in DSZBG.DOC and the ZCOMM/Pro-YAM manual.

The s (SlugBait) option prevents a receiver from receiving a file without

accounting for it.  Attempts to do so are flagged with a Q status in the log

file.  This option can cause a transfer that legitimately fails near the very

end of the file to be reported as Questionable rather than ERROR.

Send files may be specified indirectly:

dsz sz @C:files.lst

sends the files in C:files.lst.  This provides another means of bypassing the

DOS command line limit.  The first letter must be "@" and the third letter

must be ":" to distinguish this from an ordinary pathname.  Pathnames in the

indirection file may be separated by any combination of space(s), tab(s), CR

and/or LF.  No spaces may appear inside a pathname; "FOO     .BAR" won't work.

3.2  Interrupt Keys

Ctrl-Break Terminates the transfer in progress.

ALT-N Abruptly terminates (NUKES) the transfer in progress.

Ctrl-X Typing a dozen Ctrl-X characters to another ZMODEM program terminates

ZMODEM operations.

ALT-F Skips the current file in ZMODEM transfers.

ALT-B Cancels the current ZMODEM session.

Chapter 4 Pg 12          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

4.  NUMERIC PARAMETERS

DSZ uses the following main numeric parameters, settable with the pxN command.

EXAMPLE: C>dsz pQ0

In addition, DSZ uses the Pro-YAM/ZCOMM zmodem numeric parameters described in

the Pro-YAM/ZCOMM reference manual and in DSZBG.DOC.

B   Set the size of buffer used for disk reads and writes.  The default is

1024, 4096 maximum.  (16384 max on DSZ.EXE) If your copy of DSZ is running

on a floppy disk system or a very slow turkey hard disk, a larger value

may give better results.  Too large a value may exhaust memory, induce

receiver timeouts, cause disconnects(!), or generate spurious CRC errors.

If performance logging doesn't work, this parameter is too big.

EXAMPLE: C>dsz pB4096 sz A:floppy.fil

Setting the buffer size may lock out the performance log, DSZFILES, and/or

indirect pathname list in the "tiny model" DSZ flavor.

IF YOU AREN'T BLOODY SURE YOU NEED A BIGGER BUFFER, DON'T USE ONE.

O   This parameter controls XMODEM OverThruster(TM).  It is described in

dszbg.doc.  This file is part of the software package sent to paid

registrants.

P   After a "speed" command, giving a p command (display parameters), the P

parameter value will indicate the type of serial I/O UART chip detected.

DSZ automatically detects and enables the FIFO buffer on the NS16550A and

Intel 82510 chips.

EXAMPLE: dsz port 1 speed 19200 p

0  Indicates a standard 8250 or 16450 UART with no special features.

1  Indicates an NS16550A with 16 deep receive buffering.  This is the

preferred chip.

2  Indicates an Intel 82510 with 3 deep receive buffering.

The sequence "port 1 pP1 speed 19200" bypasses DSZ's verification checks

normally performed on the FIFO configuration and may allow defective FIFO

logic on certain chips to be used.  This incantation is not required for

the NS16550A.

Chapter 4 Pg 13          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

Q   Error code returned to DOS (default 1).  (Success always returns 0.) Some

programs toss cookies when confronted with certain exit codes.

EXAMPLE: dsz pQ-1 sz foo.bar

DSZ reports an error if no files have been transferred, even if the

command used wasn't supposed to transfer any file(s).

S   Timeout in seconds (default 15) waiting for restraint release (XON and/or

handshake).

W   Wait for Enter key before exiting after an error.  Effective only on

registered copies.  Useful when calling DSZ from DESQview, allows the

window to close unless there is an error message that should remain

visible.

EXAMPLE: C>dsz pW1 sz foo.bar

a   Calibrates certain protocol timeouts used with XMODEM and YMODEM transfers

(Cybernetic Data Recovery(TM)).  This parameter does not affect ZMODEM

transfers.

The a numeric parameter is automatically set to 2000 for a 4.77 mHz PC or

XT, 4000 for a PS2/30, 6000 for an 8 mHz 1ws AT, and 12000 for a PS2/80.

If your computer has a speedup card or otherwise runs unusually fast (or

slow!) for its type, you may set the a numeric parameter to a value that

reflects its speed.

EXAMPLE: dsz port 2 pa4000 rb

NOTE: Setting this parameter does not suppress the "Unknown Machine Type"

message which is displayed before the command line is scanned.

d   A non zero value (the default) causes incoming files received with ZMODEM

and full YMODEM transfers to have their Date set to that sent in the file

header.  ZMODEM's file management features depend on the transmission of

each file's modification date.  Files will have the same creation date,

even if the two machines are in different time zones.  When transferring

files to/from a Unix system, creation dates are interpreted according to

GMT or Universal Coordinated Time.  File dating may be disabled with a pd0

command.

NOTE: Backdating file creation times may confuse some backup programs.

v   Verbose level (debugging output) Negative values inhibit some routine

messages.  Setting v to 1 or greater causes serialized copies to ring the

bell when a transfer is done.

Chapter 4 Pg 14          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

z   Timezone in minutes behind GMT, used for ZMODEM and True YMODEM file

transfers to coordinate the actual date of the file with computers in

other time zones.  Effective on registered copies.  PDT or MST corresponds

to 420.  This may be preset with the ZONE DOS environment variable.

Chapter 5 Pg 15          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

5.  ENVIRONMENT VARIABLES

The following DOS environment variables may be set to modify DSZ operation.

DOS environment variables are set by giving a command to DOS.

EXAMPLE: set DSZPORT=2

These DOS "set" commands may be placed in your AUTOEXEC.BAT file or given just

before you run your communications program.

IMPORTANT! Do not add spaces to these commands!!

set DSZPORT  =  2 WILL NOT WORK!

DIRRX (Registered copies only.) Specifies the directory to receive (download)

files.  If the sting in DIRRX ends with "/", directory and/or disk

designators in the incoming pathname are stripped, and only the stem (name

and extension) is used.  Otherwise, if an incoming file has an absolute

pathname, the incoming pathname is used.

EXAMPLE: set DIRRX=C:/rxdir/

dsz rz -r

EXAMPLE: set DIRRX=B:/

The DOS "set" command defining the receive directory may be placed in your

AUTOEXEC.BAT file.  Make certain there is enough environment space to hold

this value.  Confirm this by typing "set" to the DOS prompt and verifying

the DIRRX value is listed correctly.

DSZFILES May contain a list of up to 9999 pathnames to transmit, with entries

separated by spaces.  If present, DSZFILES is used in place of a normal

command line list of files to send with the sb and sz commands.

EXAMPLE: C>set DSZFILES=file1 file2 file3 *.c

C>dsz port 2 handshake on sz -rr

In practice, this string would be built up by a calling program and passed

to DSZ with a "spawnvpe", "execvpe", or similar subroutine call.

SEE ALSO: Pathname list specification @C:files.lst

DSZLOG log file to receive entries for files sent and received.  (Default

null).  Sample entries:

Z  46532 38400 bps 3324 cps   0 errors    66 1024 DSZ.COM 1177

Z 124087 19200 bps 1880 cps   0 errors     6 1024 MXY.TMP 1177

Chapter 5 Pg 16          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

Transfer times are based on the DOS time of day clock and have up to two

seconds' uncertainty.  As with all protocol throughput reports generated

by Omen Technology programs (and contrary to the false claims of some

critics), DSZ's throughput figures for ZMODEM, Kermit, and True YMODEM(TM)

transfers do not take "credit" for pathname and/or protocol overhead

characters.  DSZ's throughput reports do not include the time required to

send the pathname.  This understates the true time required to transfer

files, but the time required to send the pathname often depends on the

alacrity of the operator, and thus does not always reflect the performance

of the protocol itself.

Transfers measuring less than one second are reported as having 9999

characters per second throughput.

The unlabeled numeric field after the error (retry) count is the number of

flow control stoppages encountered sending the file.  This number gives an

indication of flow control restrictions (overloaded networks, etc.)

encountered during the transfer.

The last numeric field before the file name is the YMODEM block size or

ZMODEM subpacket length used for the last block in the file.

The last field is the serial number of the other program, or -1 if this

information is not available.

The code letter indicates the type of transfer, Z (ZMODEM), S (X/YMODEM),

R (X/YMODEM), E (error), or L (transfer terminated by loss of data

carrier).

WARNING: If DSZ encounters a loss of carrier or other fatal error

condition before a file COMMENCES transmission, a log entry may not be

made.

EXAMPLE: C>set DSZLOG=C:/yam/dszlog

Chapter 5 Pg 17          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

DSZOPT (Registered copies only) String containing default options for DSZ file

transfers.  Options are optional modifiers to the DSZ sz, rz, sx, rx, sb,

rb commands.

A T may be appended to the string to force DSZ to exit after it has

recognized a ZMODEM AutoDownload(TM) request.

An 8g may be appended to the string to make BBS (ANSI) graphics the

default for DSZ's t command.  The ability to preset these options is

especially helpful for controlling ZMODEM automatic downloads from DSZ's t

command.

EXAMPLE: C>set DSZOPT=r

Enables DSZ's Intelligent Crash Recovery(TM) as a default.

Let's say you wish ZMODEM crash recovery, and BBS "graphics" for the DSZ

term function:

C>set DSZOPT=r8g

WARNING: Do NOT include "ena" or any other commands in the DSZOPT string.

With the exception of the "8g" mode, only **options** may be placed in the

DSZOPT string!  (Options are the letters that modify the sz, rz, sb, etc.

commands.  In "rz -r", "r" is an option.)

MAXBYTES Maximum number of bytes allowed for transfer.  If this number is

exceeded, DSZ skips the offending files (ZMODEM) or aborts remaining files

in the batch (YMODEM).

DSZPORT Default communications port override, default 1.

EXAMPLE: C>set DSZPORT=2

IMPORTANT! NO SPACES!!

ZONE sets the z numeric parameter to the number of minutes the current local

time lags GMT time.  PDT or MST corresponds to 420.  PST corresponds to

480.

EXAMPLE:

C>set ZONE=420

Chapter 6 Pg 18          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

6.  EXAMPLES

dsz sz c:foo.arc              (send 1 file with ZMODEM)

dsz sz -m c:*.zip             (send all .ZIP files with MobyTurbo(TM))

dsz sz f1.c f2.c f3.c         (send 3 files with ZMODEM)

dsz sz c:foo.* b:*.c          (send multiple files with ZMODEM)

dsz sz -Z *.lst               (send text files with ZMODEM compression)

dsz sz -r bigone.arc          (CRASH RECOVERY: resume interrupted transfer)

dsz rz -r                     (CRASH RECOVERY: resume interrupted transfer)

dsz rz -rr                    (CRASH RECOVERY with file comparison)

dsz handshake both sz yzmodem.arc       (Hardware+Software handshake)

dsz sz -n *.*                 (send files that are newer or rx has no copy)

dsz sz -Yn *.*                (Only send files that are newer than receiver's

copy)

(Do not send files that do not exist on

receiver's disk)

dsz rz C:/UPLOADS             (receive files to specified directory #)

dsz port 2 speed 38400 handshake both restrict rz

(Restrict provides protection for BBS's)

dsz rz  -y prog.arc           (ZMODEM, single file pathname override)

dsz z pr1 rx foo.dat          (XMODEM receive, remove file if error)

dsz pO60 ro foo.dat           (XMODEM/OverThruster receive, GEnie MNP #)

DSZ LRC L-Y NEWPROG.ARC       (generate lower case "rc -y" command)

dsz sz foo >debug.dmp         (Debugging data dump)

dsz sz @C:/bbs/files.lst      (Indirect file list)

# Registered copies only

Chapter 7 Pg 19          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

7.  IN CASE OF DIFFICULTY

If you have difficulty getting DSZ to work, please follow the procedure given

below.

Start with the following command to DSZ, given directly from the DOS prompt,

without running any TSR or other COMM program:

dsz port 1 speed 2400 d t

Type it exactly as it appears above; do NOT use upper case.

(Change the port number and speed as necessary.  Use only COM1 or COM2 for

testing.)

This connects you to the modem.  Dial a bulletin board by typing AT commands

to the modem ("ATDP621-3746 <enter>") to make it dial the BBS.  Log in as

usual.  Then, request a ZMODEM download of a file you don't already have.  The

file will download automatically, you don't have to do anything.  This

procedure verifies the correct operation of DSZ.

7.1  LOCKUPS

7.1.1  Insufficient Memory

DSZ complains when it detects insufficient memory to operate.  When calling

DSZ from another program, there may be ALMOST enough memory for proper

operation, but DSZ locks up during a transfer or the calling program freezes

after DSZ displays the "DSZ Finished" message.

Sometimes there is a message about insufficient memory, sometimes just a

lockup and no message.  To verify this problem, call CHKDSK from within your

comms program (not from DOS!) by defining it as an external protocol.  If you

use an interface program (Hotkeyz, Prodoor, etc.), you must call CHKDSK from

that program, as those programs also reduce the memory available to DSZ.

CHKDSK should report at least 64k free memory.  If less memory is reported,

reduce the size of the COMMS program's capture buffer, or reboot the system

with fewer/smaller device drivers and TSR programs.  Calling DSZ directly from

the comms program instead of from an interface also saves memory.

7.1.2  Environment TOO BIG

A large environment may cause DSZ.COM to lockup.  The size of the environment

(the amount of text you get when you give the DOS "set" command) is subtracted

from the data space available to 8086 C programs.  This can be a problem with

DSZ.COM, whose 8086 tiny model limits code, data, stack, and environment to 64

kb TOTAL.  If DSZ.COM complains about the environment being too big, you

should correct the problem.  Use DSZ.EXE (compiled with 8086 small model) to

alleviate this problem.

Chapter 7 Pg 20          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

7.1.3  Bad COMM Port Configuration

DSZ is fastidious about which interrupt vectors it takes over.  Unlike some

programs, DSZ uses only the correct interrupt vector for its interrupt service

routine.  (This allows multitasking.)

DSZ obtains and releases this vector with standard DOS function calls.  This

allows multi line operation under DESQview and other multitasking systems.

Incorrectly configured serial ports that work with programs that pig out both

IRQ3 and IRQ4 will lock up with DSZ.  If DSZ locks up the machine as soon as

it tries to send or receive data, this is usually the fault.  Make sure the

comm port is strapped correctly; COM1 uses IRQ4, COM2 uses IRQ3.  DSZ will

lock up if the comm port is not properly set up.

In addition, DSZ will not work properly if two devices are connected to the

same IRQ line, even when some other programs (that don't use interrupts as

intensively) do work normally on the same machine.

COM3 and above are not widely standardized.  If you have problems with COM3,

chances are your "COM3" is non standard.

7.2  Incorrect Batch Files

Incorrect batch files are the number one source of problems using DSZ for

uploads and downloads.  Bulletin board programs with convoluted batch file

mechanisms exacerbate the problem.

DSZ's command line echo was added to assist in correcting defective batch

files.  You should see a display like:

Command line = 'rz '

or

Command line = 'port 2 speed 9600 sz foo.baz '

In general, the Command line display should look like one of the examples in

this document or those displayed by DSZ itself.  (The Command line display

does not include the name of the DSZ program itself.)

Try running DSZ by itself, without any batch files or TSR programs, calling it

directly from DOS as described above.

7.3  Cheap Internal Modems

Numerous trouble reports have been received lately caused by cheap internal

modems causing errors on ZMODEM transfers but not XMODEM.  A similar problem

was experienced in the mid 80's during beta testing of some 2400 bps modems.

Perhaps defective LSI chips left over from those days were cloned or recovered

from the trash bin.

Chapter 7 Pg 21          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

7.4  Commands Entered in Upper Case

Most DSZ commands and options must be entered in lower case (small letters,

not big letters).  Where the documentation or help screen indicates "rz", you

must use "rz" and not "RZ".

7.5  No Carrier Detect

If you see a No Carrier Detect message, and you haven't lost the connection,

you should fix your modem cable or modem settings to allow proper operation of

the Carrier Detect signal (CD).

A poorer alternative is to give the d command:

dsz port 1 speed 2400 d t

7.6  PKARC 3.6

It has been reported that PK36 grabs some interrupts and doesn't restore them

on exit in an attempt to prevent hacks of the program.  Since eliminating that

particular version and cleaning up memory, high speed xfers even with 8250

UARTS work properly again.

The problem with PK 3.6 appears to be most severe if a BBS operator has

doorware, etc that accesses the PK programs from a shell of some sort.  This

info may be helpful to users who are having otherwise inexplicable file xfer

problems.

Phil Katz has written me that this problem has been corrected in subsequent

versions.

7.7  DOS 3.x

DOS 3.x systems may need to have a "STACKS = 8,256" line in CONFIG.SYS to

prevent antisocial behavior. This is required on the NEC Multispeed, or if

certain TSR's are used.

7.8  Perstor Controllers

Several reports have been received indicating Perstor disk controllers

interfere with data communications.  Perstor is reportedly working on a fix.

Chapter 7 Pg 22          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

7.9  IBM PS/2 Model 50Z

Early samples of this machine have a design error on the motherboard that

causes comms programs to lock up the machine.  Replacing the motherboard with

a correctly functioning unit from current production corrects the problem.

The part number of the replacement PS2/50 Z motherboard is 35F5928.

(Information provided by John M. Choma 73047,3566) U.S. IBM has stonewalled on

this issue, but in December 1988 agreed to replace the defective motherboards

under warranty.

7.10  Brain Damaged UARTS

Omen Technology has received reports of problems with buggy 8250 type UART

integrated circuits in Leading Edge modem boards, serial port interfaces, and

computers.  The defective chip logic affects high performance software.

Replacing the buggy chip with a newer chip (16450 or NS16550AN) corrects the

problem.

The UM82450 chip in the Zoom 2400HC also appears to cause problems.

The NS16550AN is a pin compatible plug in replacement for 8250 and 16450

serial chips.  Omen Technology software enables the FIFO buffer in the

NS16550AN to prevents loss of data from poorly written device drivers, TSR

programs, etc.

The NS16550AN has better electrical characteristics than the older UART chips.

386 and fast 286 machines should use the NS16550AN for best results.  Omen

Technology software has been written to allow time for the older UART chips to

operate. However, there is a limit to how much the software can rearrange chip

accesses without resorting to performance robbing wait loops.  Some other

software is not as forgiving of slow chips.

Jameco Electronics at 415-592-8097 sells National NS16550AN chips mail order

(credit card) in small quantities.  Another source is Arrow Electronics at

800-932-7769 (516-467-1000).  Be sure to ask for the NS16550AN.  It's

important to get the "FAN" or "AN" part, other versions won't do.

7.10.1  Everex Internal Modems

Numerous reports have been received concerning difficulties with Everex modem

cards.  Some of these cards contain buggy 8250 chips which are not up to the

demands of full duplex streaming protocols such as ZMODEM.  Replacing the

buggy 8250 chip with a 16450 (or better yet NS16550AN) chip corrects these

problems.

Chapter 7 Pg 23          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

7.11  Weird Iron

A few machines, some with V20 CPUs, don't like DSZ.COM.  We have not been able

to repeat this; TeleGodzilla (using a V20) runs DSZ.COM without a hitch.

Switching to DSZ.EXE has cleared up the problems on other machines.

7.12  Will the real YMODEM please stand up?

Some XMODEM programs claiming to support YMODEM neither send nor accept the

YMODEM pathname block.  1024 byte XMODEM is XMODEM-1k, not the YMODEM defined

by Ward Christensen in 1985 when he invented the term YMODEM.  1987 and later

versions of YMODEM.DOC remind the reader of the official YMODEM definition,

which has not changed since YMODEM's coinage in 1985.

7.13  286 and 386 Extended Memory

Virtual memory disk caches and ramdisks using 80286 or 80386 extended memory

require a kludge hardware reset of the CPU chip to return to the normal "real

mode" after accessing extended memory.  This interference to the interrupt

response time causes lost characters on comm ports, and may be minimized by

specifying shorter memory transfers.

Switching to a National NS16550AN UART chip helps tremendously, allowing

transfers at 115200 bps instead of 9600 bps with a standard 8250.  Please

refer to the subchapter on "Brain Damaged UARTs".

Chapter 8 Pg 24          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

8.  BACKGROUND OPERATION

Three DESQview description files are provided to demonstrate background

operation with modest (66 kb window) memory requirements.  DSZ uses tight

C/ASM code, table driven CRC generators, and buffered receive and transmit for

good performance in the background without hogging CPU cycles.  On a QIC Labs

8 mHz PC-AT clone, DSZ can download files in the background (3 background

clock ticks, 9 foreground clock ticks) with a throughput better than 9600 bps.

2S   Prompts for file name(s) and sends them with ZMODEM protocol.  File: 2S-

PIF.DVP

2T   Talks to the modem and allows manual YMODEM (F3) or automatic ZMODEM

downloads.  File: 2T-PIF.DVP

2A   Like 2T, but using Quarterdeck's dvansi.com memory resident ANSI CRT

device driver to process ANSI screen codes.  File: 2A-PIF.DVP DS.BAT is

the batch file used by 2A.

These files are written for a port 2 local connection and \TMP directory, but

may be modified to suit your requirements using the DESQview CP command.  The

W numeric allows DESQview to close the window after DSZ finishes, providing a

pleasant user interface.

Other DVPs, Scripts, and Batch files are available on the DESQView conference

on the Sound of Music as DVDSZ20Z.ARC.

Please consult the DESQview documentation for instructions on installation

procedure for the DVP files.

Operation at high speed (above 9600 bps) may require the DESQview "Optimize

communications" performance Advanced Setup option.

Newer versions of DESQview may require more "common memory" allocation (6-8 k

bytes) to operate with DSZ.  Some versions of DESQview do not operate properly

with DSZ; DESQview 2.0 is known to work, and DESQview 2.25 is reported to have

fixed bugs in intermediate versions.

On 286 and 386 machines, some DESQview operations involve swapping memory

contents with buffers in extended memory.  As with extended memory ramdisks

and disk caches, this causes errors with high speed communications.  The

considerations discussed in the "286 and 386 Extended Memory" subchapter apply

here.  Since this DESQview extended memory swapping is asynchronous with

respect to DSZ's file transfers, "handshake slow" doesn't help here.  The

solution is to force DESQview's memory swaps to use smaller blocks (with

interrupts allowed between blocks) and/or use NS16550AN chips for the serial

interface.

***********************************************************

WARNING

When running DSZ under a multi tasking system such as DESQview, do not allow

Chapter 8 Pg 25          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

two programs to access the same comm port at the same time.  At best, such

actions will disrupt communications.  At worst, your computer will attempt to

open a subspace channel to V'ger.

When running a comm program under DESQview or any related multitasking

executive, DSZ must be called directly from the comm program, in the same

window.  DO NOT open a separate window for DSZ unless you were about to power

down the computer anyway.

***********************************************************

With some communications programs, you can make the call, log in, and then

exit (closing the window) without disconnecting the modem.  At this time you

can fire up DSZ with the "t" command to allow YMODEM or ZMODEM downloads in a

small window.

Example File: 2A-PIF.DVP

Alternatively, you can start DSZ from the DOS prompt, then type "AT" commands

to the modem to call the bulletin board.  DESQview's "learn" facility may be

helpful here.

Chapter 9 Pg 26          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

9.  CALLING DSZ FROM COMM PROGRAMS

It may be necessary to give a

SET COMSPEC=A:\COMMAND.COM

(change as needed) in your AUTOEXEC.BAT file to allow programs to call DSZ.

Note: NO SPACES in the SET command!

9.0.1  Ms-Kermit 3.00

The "push" command (^[p) in MS-Kermit 3.00 sends an XOFF character to the

modem before calling DOS.

Using this to activate DSZ's term mode with a "dsz t" command causes most

remote hosts to freeze.  The remote host can be unfrozen by keyboarding XON

(Ctrl-Q) from DSZ's terminal mode.

9.1  GT POWER 13.00/14.00

GT Power version 13/14 includes a YMRX.BAT (YMODEM-g receive) file with an

"rz" command.  You guessed it, "rz" is for ZMODEM protocol (the z means

ZMODEM!).  Please change the file to use the correct "rb -g" for YMODEM-g.

The files ZMTX.ARC and GT1300.NOT available on many bulletin boards contain

useful information for GT Power users.

9.2  PROCOMM PLUS

DSZ may not operate properly when called from ProComm if ProComm has files

open at the time.  COM3 and COM4 on Procomm are non standard; in case of

dififculty use COM1 or COM2 or read this document carefully.

Here are some handy settings for ProComm Plus.

First, set the DOS environment as necessary before running ProComm.  Give

these commands to DOS before running ProComm.

Note: NO SPACES in the SET command!

SET DSZPORT=2                           (Only needed for COM2)

SET DSZLOG=DSZ.LOG            (Only if you wish a log file)

Then set ProComm's external protocol strings to:

A- External protocol 1 upload filename:  ZMODEMU

B- External protocol 1 download filename: ZMODEMD

C- External protocol 2 download filename: ZMODEMMD

The following batch files are included in the DSZ archive:

ZMODEMU.BAT contains:         DSZ sz %1 %2 %3 %4 %5 %6 %7 %8 %9

ZMODEMD.BAT contains:   DSZ rz

Chapter 9 Pg 27          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

ZMODEMDR.BAT contains:   DSZ rz -r

ZMODEMMD.BAT contains:   DSZ rz -mr

ZMODEMAD.BAT contains:        DSZ t

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

Once you've got the hang of things the "ProComm way", try the "dsz t" command

described above to give you automatic ZMODEM downloads (Look Ma, no

keystrokes!!!)..

9.3  Qmodem

The following is based on a message from Mike Cody, sysop of HomePlate 313-

385-9969 3/12/24/96 HST

Go into Qinstall and go to frame 3 Directories. Then select External protocols

#3 I think. Now you will see a definition area to tell Qmodem the

Letter,batchfile combo to use.

Now put Z in the letter definition, Zup.bat in the u/l batch zdown.bat in the

d/l batch, and Y in the batch protocol. Now save it at the menu with f10 and

S.

Then go to DOS and create the following batch files in your Qmodem

directory...

zdown.bat: DSZ port %2 speed %1 rz -rr

zmdown.bat: DSZ port %2 speed %1 rz -mrr((FOR MOBYTURBO!!))

zup.bat: DSZ port %2 speed %1 sz %3

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

These are the batch files that will be called by Qmodem.  If you did

everything right when you hit PGDN or PGUP you will see an external protocol

called Z) Zmodem in the box. For an easier time do a search for either

External.arc or maybe the string "Qmodem" in the file database as there is a

file with several external protocols and batch files available in the Mahoney

file section (EXEC BBS).

The following batch files for Qmodem are included in the DSZ archive:

ZDOWN.BAT contains:           DSZ port %2 speed %1 rz -rr

ZUP.BAT contains:             DSZ port %2 speed %1 sz %3

!!!!IMPORTANT USE lower case (small letters) EXCEPT WHERE SHOWN!!!!

Chapter 9 Pg 28          DSZ Reference Rev Jan-28-90           Typeset 1-28-90

9.4  PROCOMM

DSZ may not operate properly when called from ProComm if ProComm has files

open at the time.

A number of reports have been received of problems running DSZ from ProComm

Plus because of insufficient memory.  Please refer to the subchapter on

LOCKUPS.

When running under ProComm's Host mode, failure to use the DSZ CON command may

cause ProComm to lock up the computer, or at least interfere with file

transfers, etc.

EXAMPLE: DSZ CON sz %1

A number of third party programs have been developed to provide a congenial

interface to DSZ if you have enough memory to spare.  The POE, PCALL, DSZ_ZAP,

and PROZ programs have been popular in interfacing DSZ to ProComm's editor

gateway.  These programs are available on many bulletin boards.

Hint: the DOS PATH environment variable must include ProComm's directory to

allow POE to locate all the POE files,.  The POE files should be stored in the

ProComm directory.  Setting the "PROCOMM" environment variable is

insufficient.

9.5  BULLETIN BOARDS

When installing DSZ on a bulletin board or host system, be sure to check for

proper operation when the caller hangs up while a file is being sent or

received.

Currently, Only the TurboC version supports file sharing (shared read access

on files sent by  dsz).

Some bulletin board interface routines do not call DSZ to send a file until

they have verified the file's presence by opening the file.  If this checking

is done in DOS's default exclusive open mode, dsz's file sharing will be

defeated.

Chapter 10 Pg 29         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

10.  FLOW CONTROL

If you plan to send files over Telenet (PC-Pursuit), you must refer to the

Pro-YAM or ZCOMM Reference manual (in ZCOMMDOC.ARC) FLOW CONTROL chapter for

vital information on flow control considerations necessary for proper

operation of ZMODEM and other streaming protocols.

Chapter 11 Pg 30         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

11.  ERROR MESSAGES

COM2 1200 bps Carrier Detect enabled Handshake slow

Command line = (commands)

This is a normal message displayed to help resolve problems with incorrect

batch files.  The port number, speed, carrier detect ("enabled" or "DISABLED")

and "handshake" indicate the selected mode of operation.

DSZ FINISHED! exit code = 0

This is DSZ's normal exit when at least one file has been transferred and no

unrecoverable errors have been detected.  If an error such as a failed file

transfer had been detected, the exit code would be 1, or the value set in the

Q numeric parameter.

As the message implies, DSZ has done all of its processing, and has return the

communications port and vector to its original value before displaying this

message.  If the computer locks up after this message is displayed, there's

something wrong with the program that called DSZ, not with DSZ itself.

Unknown Machine Type XX, 'a' parameter preset to 6000 indicates a ROM BIOS

that does not follow the IBM specification for identifying the machine type.

If your computer causes DSZ to display this message, you may set the a numeric

parameter described elsewhere in this document.  This message is generated

before any commands are read, so do not be alarmed when you "discover" that

setting the a parameter doesn't make this message go away.  The a parameter

does not affect ZMODEM transfers.

Please send us a postcard or letter indicating the reported hex value for

machine Type, name of the computer, and its speed so we can include that

machine type in a future release.

Need more memory to run DSZ or Abnormal program termination is displayed by

the TurboC startup routine if DSZ is run in too small a memory partition.  If

you see this message, use a larger memory partition for DSZ or remove some

memory resident programs.  This message may also appear if the calling program

fails to free memory before calling DSZ.

No Carrier Detect is displayed when DSZ recognizes a lack of carrier detect

signal from the modem.  Most support calls concerning "No Carrier Detect" are

caused by the user failing to tell DSZ to use the correct port.  DSZ does not

magically divine the COM port number; if the default COM1 is incorrect, DSZ

must be told what port to use with the port command or the DPORT environment

variable.

No Carrier Detect may also be caused by a line disconnect, defective cable, or

incorrect modem strapping.  It may be disabled with the d command.  Sometimes

this is caused by setting the B parameter to too large a value.

Garbage Count Exceeded Indicates ZMODEM has not detected a valid start of

header in a reasonable number of characters.

Chapter 11 Pg 31         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

If DSZ becomes "stuck" on emitting this messages, it is possible the sending

program may not be able to recognize and act upon transmission requests (ZROPS

headers).  The sending program should be corrected.  In the meantime, setting

a ZMODEM packet length with the "zmodem pl4096" ("l" is the small letter l)

command will force the sender to wait for the receivers' progress reports.

Permission denied comes from the Microsoft C Library.  This is caused by

attempts to modify a read only file or directory, or to create a file in a

nonexistent directory.

A furtive flashing appearance of XOFF or FLOW during ZMODEM or YMODEM-g file

sending signifies DSZ is responding to flow control restraint.  This is normal

operation when buffered modems or packet switched networks are used.  ZMODEM's

robustness and reliability allow creative users to transfer files over unusual

multiple computer lash-ups, causing modem data lights and computer status

displays to dance in unfamiliar rhythms.

Output FLow Control Restraint RELEASED Indcates DSZ has not received a

positive flow control signal (XON and/or CTS as specified by the handshake

command) within the timeout specified by the S numeric parameter.  This

message may appear as the result of line noise.  If it appears sonsistiently,

it may indicate improper flow control settings, defective modem and/or cable,

or (rarely) too small a value for the S numeric parameter.

Not allowed to overwrite %s By default, DSZ will not overwrite an existing

file.  Use the -y or -n or -r option to overwrite/update existing files.

11.1  Got ZRPOS

Got ZRPOS indicates the receiving program has detected a transmission error

and has requested retransmission.  In other words, ZMODEM is doing its job of

detecting and correcting missing and garbled data.  If this happens more often

than observed line errors would indicate is reasonable, there may be a flow

control problem in one of four areas:

+ Between the sending program and its modem.  If using an interface speed

higher than the transmission speed, or modem to modem error correction

(MNP, LAPB, PEP, etc.), check that the modem and the sending program are

configured for the same flow control methods.  Omen Technology's software

is faster than most other programs, and may expose flow control problems

not seen with slower software.

+ Between the sending modem and the network.  Correcting this requires

"handshake both" to allow the sending program to honor both hardware (CTS)

flow control from the modem and software flow control (XON/XOFF) from the

network.

+ Between parts of the network.

Chapter 11 Pg 32         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

+ Between the receiving modem and the receiving program.  This can happen at

high transmission speeds when the receiver has specified a large I/O

buffer size with the B numeric parameter.

Excessive interrupt latency from extended memory ramdisks or disk caches,

and poorly written hard disk drivers also wreak havoc on streaming

protocols but do not affect start-stop protocols (XMODEM, etc.).  Problems

with extended memory disk caches may cause errors at regular intervals;

check the receiver's error messages.

To work around these problems, give the "handshake slow" command to the

receiver.

Byte NNNN retry M: <error message> These messages appear when the protocol has

detected an error and is attempting to retransmit the garbled data.  These are

normal messages.

11.2  Serial Input Error: Line Status Register xx

Serial Input Error: Line Status Register HH indicates a hardware problem, slow

device driver, TSR program, or other software is disabling interrupts too long

for the program to accept incoming data, and some characters are lost.  A slow

response to folow control also triggers this message.

The value of the line status register is displayed in HEX.  The following

error conditions are OR'ed into the Line Status Register.

01   The Interrupt level circular buffer has been overrun.  The remote did not

respond to DSZ's XOFF or hardware flow control, and continued to send

characters after being told not to.  This problem is alleviated by

correct modem flow control configuration.

02   An 02 value for the LSR indicates Data Overrun.  "Data Overrun" means the

computer was not able to respond to an incoming character from the UART

(modem) in time to make room for the next incoming character.  Data

Overruns are caused by poorly designed software (or ROM BIOS "firmware")

locking out interrupts for excessive periods.  These lockouts prevent the

communications program from responding to the incoming characters quickly

enough.

To correct Data Overruns, correct the offending condition(s) to allow the

communications program to operate properly.

Common causes of excessively long interrupt lockouts ("critical code

regions") are disk drivers (BIOS), window managers, disk caches, TSR

programs, EGA/VGA board auto-select.

Some BIOS programs are written without regard to their effect on high

Chapter 11 Pg 33         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

speed communications.  Sometimes better versions of the offending BIOS

programs are made available if enough users complain about poor

performance.

Interrupt latency caused by EGA/VGA board auto-select can be often

corrected by disabling the board's Auto-Select.

Other chapters in this document give suggestions for working around

excessive interrupt latency, primarily the handshake slow command and use

of the NS16550AN UART chip.  Please refer to the "Brain Damaged UARTS"

subchapter.

Please try all the suggestions mentioned here before contacting Omen

Technology about data overruns.

08   Framing Error generally caused by line noise or an incorrect transmission

speed ("baud rate") or format (number of bits).  Also caused by some

cheap internal modems.  Most protocols require 8 bits no parity (-8n) for

proper operation.  This is set automatically by Omen Technology programs,

but other programs may not be so smart.

10   Break Interrupt generally caused by line noise or an incorrect

transmission speed ("baud rate") or format (number of bits).  Also caused

by some cheap internal modems.

18   Framing Error and Break Interrupt (see above) in combination.

This error may also be caused by defective 8250 UART chips.  Certain

internal modem cards have slow 8250 chips that do not operate properly

with computers faster than 4.77 mHz.  Replacing the 8250 with a NS16550AN

is recommended.

Please refer to the In Case of Difficulty chapter of this document for

solutions.

Less commonly encountered DSZ error messages are explained in the Pro-YAM

manual.

11.3  COMMAND NOT FOUND

If you see this message, make sure DSZ is in the DOS PATH.  Sometimes this

error is caused by having too many environment variable strings.

Chapter 11 Pg 34         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

11.4  Out of Environment

means there is not enough room in the program environment to accept more data.

DOS prints this message when it cannot allocate more memory to accommodate a

"set" command, such as "set dirrx=c:/downloads/".

Try moving the "set" commands to the batch file that calls your comms program,

or to your AUTOEXEC.BAT file.

To increase environment, use the /E switch. The default is 160 bytes.

An example for your CONFIG.SIS file: SHELL=C:COMMAND.COM/E:256

NOTE: Large environments may cause DSZ.COM to lockup.  This is less of a

problem with DSZ.EXE.

Chapter 12 Pg 35         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

12.  SLOW TRANSFERS

Some users report slower than expected file transfers even when few or no

errors are reported.  Some possible causes are:

If "handshake slow" is used, throughput may suffer.  Try one of the other

solutions to the "Line Status Register = 02" errors discussed above.  When

using "handshake slow" it may be possible to increase the modem's interface

speed, say from 9600 to 19200 bps.

Bad phone lines will silently slow MNP modems, sometimes without any error

indications.  If the modems disconnect fairly often you should suspect

marginal phone line conditions that are being hidden by MNP error correction.

One of the programs may have set a parameter or option that acts to slow the

transfer under certain conditions.

Chapter 13 Pg 36         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

13.  ASSOCIATED DOCUMENTS

Consult the Professional-YAM, PowerCom, or ZCOMM manual chapter "OPTIONS FOR

FILE TRANSFERS" for descriptions of the file transfer options.  The "FILE

TRANSFER BASICS" chapter explains the relative merits of XMODEM, YMODEM,

ZMODEM, and other protocols and gives tips on their use.  The "MAIN COMMANDS"

chapter describes the rb, rx, rc, rz, sb, and sz commands.  The "MODEM

INTERFACING" and "FLOW CONTROL" chapters contain important information on the

use of buffered modems with DSZ.

Information on the ZMODEM Inter Application File Transfer Protocol may be

found in ZMODEM.DOC, part of YZMODEM.ARC (available on CompuServe (IBMCOM) and

GEnie (IBM Roundtable)).

C Source code for XMODEM, YMODEM, and ZMODEM transfers is included in

RZSZ.ZOO.  This file includes a Pro-YAM/ZCOMM script to upload a bootstrap

file to a Unix system, force its compilation, and (if the compilation is

successful) upload the rz/sz source files.

The C source code now includes VAX/VMS support.

13.1  ZMODEM Developer's Collection

Omen Technology sells the "ZMODEM Developer's Collection" for $89.00.  This

diskette includes the most recent public domain versions of YMODEM.DOC,

ZMODEM.DOC and the Unix/VMS rz/sz C source code for YMODEM and ZMODEM.  A set

of public domain PC-DOS executable programs are included for convenience.  For

many developers, this collection affords a significant labor savings in

obtaining these materials.

Chapter 14 Pg 37         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

14.  FILES

DSZ.COM, DSZ.DOC, 2A-PIF.DVP, 2S-PIF.DVP, 2T-PIF.DVP, BBS_REG.TXT, DS.BAT,

FIX.BAT, MAILER, ZMODEMAD.BAT, ZMODEMD.BAT, ZMODEMDR.BAT, and ZMODEMU.BAT

constitute Omen Technology's DSZ.ARC.  These same files are in DSZ.ZOO on the

distribution diskette mailed to registered users.[1]

2S-PIF.DVP, 2A-PIF.DVP, DS.BAT, and 2T-PIF.DVP are used with Quarterdeck

Office Systems' DESQview multitasking executive for background operation.

Their use is described in a previous chapter of this document.

ZCOMMDOC.ARC (manual), ZCOMMEXE.ARC (programs), ZCOMMHLP.ARC (help processor

and database), make up the ZCOMM communications tools shareware distribution.

The ZCOMM files are available on finer bulletin boards.

RZSZ.ZOO contains ZMODEM "C" source code for COMPLETE VAX VMS and Unix/Xenix

programs licensed for use transferring files to/from licensed Omen Technology

products.

DSZ.EXE compiled with the Xenix to DOS XC provides moderately higher maximum

throughput.  DSZ.EXE supports ZMODEM over 7 bit paths (E and P options), and

specified control character quoting (-Q<string> option).  The Xenix XC version

requires somewhat more memory than DSZ.COM.  DSZ.EXE does not support file

sharing due to brain damage in the Microsoft stream I/O library.  The maximum

DSZ.EXE I/O buffer size available with the B numeric parameter is 16384.

DSZ.EXE is available in DSZEXE.ZOO.

ZCOMM and Pro-YAM also run faster than DSZ.COM.

DSZBG.ZOO contains DSZBG.DOC.  Part of the DSZ registration disk (and not for

public distribution), this file provides background information on the

ZCOMM/Pro-YAM commands provided in DSZ.  For non-registered users, the same

information is available in ZCOMMDOC.ARC.

The ZOO archive format is a machine independent improvement on the PC-DOS

specific .ARC format. The ZOO format has been precisely defined, and stable

ZOO archivers/dearchivers are available on a wide variety of systems.

LOOZ.EXE unpacks .ZOO archives.  LOOZ.DOC describes LOOZ.EXE.  Be certain to

type the arguments to LOOZ in lower case, UPPER CASE WILL NOT WORK.

__________

1. DSZ.ZOO is supplied to registrants instead of DSZ.ARC because not all

registrants have copies of unarchiving programs.  The registration disk

includes a copy of LOOZ, a simple dearchiver for ZOO files.

Chapter 15 Pg 38         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

15.  CHANGES

Be sure to save your serial number/password (SNP) card and the putsnp program

you received from Omen to use with future versions of DSZ you may download!

The current DSZ version is maintained on GEnie (IBM Roundtable), EXEC-BBS,

CompuServe (IBMCOM and CONSULT SIGs) as well as TeleGodzilla.

A problem in the 1/17/89 and 2/2/89 versions corrupts files under unusual

circumstances.  Please delete all instances of the 1/17/89 and 2-02-89

version.

A reminder: the -Z option is effective only when it is given to the sending

program unless both are Omen Tecnology products revised Jan 22 1990 or later.

If you use PREFIX= or ONAME= with the "sb" or "sz" commands, these pathnames

must be entered in lower case unless the underlying file system supports upper

AND lower case.

Jan-28-1990: Doco improvements, especially to the "In Case of Difficulty"

chapter.  BBS registration clarified (I hope).  Qmodem batch files fixed.  No

change to the DSZ software.

Jan-22-1990: ZMODEM-90(TM) extensions:

Receiver specified window size: The -Wn option operates similarly to the -wn

option.  The -Wn option is specified to the receiver, which sends this

information to the sender.  Pro-YAM, ZCOMM and DSZ versions beginning with Jan

22 support this.

Specified extra escaped characters: The "zmtxesc" and "zmrxesc" allow extra

characters to be escaped when sending (zmtxesc) or receiving (zmrxesc).

Zmtxesc is effective sending to any ZMODEM program.  The sz commands in Omen

products after Jan 22 1990 except for DSZ.COM recognize incoming "zmrxesc"

requests.  TThe zmrxesc and zmtxesc string parameters consist of one or more

printing characters corresponding to the control characters to be escaped.  To

escape NULL:  set zmrxesc "@"

Since DSZ does not support string parameters, a -Q<string> option unique to

DSZ performs the same function as zmrxesc and zmtxesc.  If the Q<string>

option is used in DSZOPT, it must be last.  To escape NULL: dsz sz -Q@

file.ext

MobyTurbo(TM):

MobyTurbo optimizes throughput on ZIP, ARC, and similar files over paths that

support ZMODEM and do not "eat" or otherwise respond to control characters.

This includes all paths that support YMODEM-g (or would except for random

errors).

Activate MobyTurbo with the -m option to one or more of: sz, rz, or adlopts.

Chapter 15 Pg 39         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

LOCK OUT MobyTurbo with the -M option.  -M is effective until the next "call"

command.  Use -M in situations when the sender has -m turned on and YAM's

check fails to discover path conditions that preclude MobyTurbo.

ZMODEM over 7 bit paths:  (Not supported in DSZ.COM)

A pair of ZMODEM-90 programs with 7-bit support operating over a 7 bit path

should detect that fact and switch to one of two 7-bit path compatible

encodings.  For testing, this can be forced with a "-E" option.

Default 7-bit mode is RLE compression and 8th bit quoting similar to Kermit

but more efficient because not so many control characters are quoted.  Use

this default for text files.  Rubout and all control characters can be escaped

with the -e option.  Individual control characters can be escaped with

"zmtxesc" and "zmrxesc".

Pack-7 instead of quoting is activated with the -P option.  (-P has no effect

on 8 bit paths.) Pack-7 packs 4 bytes into 5 printing characters.  This is

more efficient than quoting for files where 25% more 8th bits appear than the

number of characters saved by RLE compression.  (This statement applies to

GIF, ARC, ZOO, and ZIP files.  It may or may not apply to other binary file

formats, depending on the data.) Pack-7 will match or beat standard Kermit on

ZIP files, even when Kermit has use of all 8 bits.

SUPK412.ZIP 75136 bytes sent at 2400 bps direct connect from an

Everec 1800 AT clone 8mHz/0ws to an 18 mHz Intel 386 motherboard.

8-Bit line

cps     Protocol/command

156     MS Kermit 3.00 to MS Kermit 3.00  (defaults)

182     MS Kermit 3.00 to MS Kermit 3.00  (windows=2 length=2000)

184     MS Kermit 3.00 to Pro-YAM 17.51 (windows=2 length=2000)

231     Pro-YAM 17.51 ZMODEM (defaults)

237     Pro-YAM 17.51 ZMODEM-90(TM) MobyTurbo(TM) (sz -m)

7-Bit line

cps     Protocol/command

156     MS Kermit 3.00 to MS Kermit 3.00  (windows=4 length=2000)

190     Pro-YAM 17.51 ZMODEM-90 PACK-7 (sz -EP)

Kermit transfers used 8 bit CRC, ZMODEM used 32 bit CRC.

FLAG DAYZ: I doubt MobyTurbo will change much except maybe the name.  A number

of issues remain to be tested on the 7-bit business.

*****************************************************************************

*****************************************************************************

WHAT TO LOOK FOR:

Chapter 15 Pg 40         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

Performance      difference      with      MobyTurbo,      possible       side

effects/incompatibilities.   MobyTurbo  logic  tests for software flow control

incompatibilities: when does this test fail?

Automatic detection of 7-bit paths.  Spurous  detection  of  7-bit  paths  and

failure to detect 7-bit paths.  (DSZ.EXE only)

12-13-89  The Virus check was removed  becuase  of  user  confusion  and  slow

startup  under  certain  conditions.   Code  has  been added to prevent and/or

recover from "line status register 18" errors.

10-28-89 A T may be appended to DSZOPT or "dsz t -T" may be used to force  DSZ

to exit after it has recognized a ZMODEM AutoDownload(TM) request.

10-24-89  BBS registered copies (serial numbers 8000-8999 and 20000-23999) are

now fully featured.

The BBS registration form has been modified.  Please read this form  carefully

before  applying  a BBS serial number.  Use of BBS serial numbers (4000, 20000

series) acknowledges acceptance of this contract.

The IMP/KMD  compatibility  file  length  information  in  the  YMODEM  header

generated  by  the  sb command now uses the correct byte ordering.  Changes to

the XMODEM/YMODEM sz/sb commands were made to accommodate programs that ise CK

or  CKL.   (As  predicted  in  1985,  networks  sometimes interfere with these

sequences.)

Files passed over because of creation errors at  the  receiving  end  are  now

counted  by  the "sz" command as errors.  The receiving program must be a 9/89

or later version for this to be effective.

The specified directory form of the rz and rb commands  has  been  enanced  to

accommodate certain forms of erroneous syntax.

6-19-89 A documentation error (concerning DSZPORT) was corrected  in  the  DOC

file.

5-25-89  Corrects a protocol negotiation problem encountered when the rz or sz

-e option is used.

Host programs can now specify an alternate CANCEL sequence (not used  for  DOS

and Unix).

The ZMODEM "Slow Send" mode inserts a brief pause after  each  subpacket  when

invoked  with  the  -F  option, or automatically when the "sz" command detects

failing flow control.  The zmodem F numeric parameter  (default  1)  sets  the

pause in tenths of seconds.

4-24-89 The -rr option has been improved.  Long file compares are done in  two

Chapter 15 Pg 41         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

stages to save time when the first portions do not match.  Diagnostic code and

undocumented commands have been removed to decrease the program size.

DSZ now defaults to "handshake both" if the speed is greater than 2400 and the

clear  to  send  signal is high when DSZ selects the comm port (was "handshake

on" in 3-19).

The "IN CASE OF DIFFICULTY" chapter has been expanded.

The 4-18 version introduces ZMODEM variable length headers of up to  16  bytes

information  field  instead  of  the  previous fixed 4.  Selection of variable

length header capability  is  automatic  and  downward  compatible.   Variable

length headers allow future extensions to ZMODEM's capabilities.

ZMODEM Crash Recovery now understands the -rr option to compare the CRC-32  of

the assumed common file segment before continuing an incomplete file transfer.

The "zmodem R" numeric parameter may be set to limit the number  of  bytes  in

the  files  that are compared; the default R value of 0 allows all of the file

to be compared, an R value up to 32000 (up to 2 billion  on  32  bit  flavors)

limits  the  comparison to the first R bytes of the two files.  The ZMODEM -rr

option is supported by Omen Tech versions April 18 1989 and later.

When ZMODEM Crash Recovery detects incompatible source  and  destination  file

contents,  the  destination  file  is  now  truncated  to  zero  length before

proceeding with the transfer.  Crash Recovery requires the destination file to

be no longer than the source file.

Error reporting on unregistered copies has been enhanced to point the user  to

solutions described in the relevant chapters in DSZ.DOC.

3-18-89 In order to work around a bug in the new TCOMM bbs, DSZ  now  defaults

to  "handshake  on"  if  the  speed is greater than 2400 and the clear to send

signal is high when DSZ selects the comm port.

The "o" command has been added  to  disconnect  the  modem  after  a  terminal

session:  "dsz  t  o"  disconnects  the  modem  when  the  terminal session is

completed.

The "P" (capital P) numeric parameter (q.v.) has been  added  to  display  and

control the UART fifo on some clone chips.

BBS registrations postmarked after Feb 28 1989 must be accompanied by  a  paid

DSZ  registration.   The  free  program expired Feb 28 1989.  Outside help has

been hired to process and verify applications received under that program.

2-23-89 The term function now sends  BREAK  in  response  to  the  ALT-B  key.

Handling  of  the Ctrl-Break vector has been changed so an untimely keyboarded

Ctrl-C or Ctrl-Break will not hang DOS.

2-15-89  DIRRX is enabled for BBS registrations  (8xxx  numbers).   Logic  has

been added to simulate transmit interrupts on chips that have problems in this

Chapter 15 Pg 42         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

area.  The "A" command does not require registration.  An interaction  between

ONAME=  or  PREFIX=  and  the  file count display has been removed.  Time/date

conversion routines have been enhanced to deal with files with missing  dates.

The  "v"  command  has  been  added to allow bbs programs to interrogate DSZ's

serial number.

2-06-89 A spurious error message at the completion of YMODEM downloads with  a

download directory specified on the command line has been eliminated.

2-05-89 DSZ now returns a non 0 status unless  at  least  one  file  has  been

transferred.   An  inappropriate  pathname/disk  specifier  check on specified

download directories has been removed.

Previous versions:

Added the "A" command to prevent pcAnywhere host mode  from  locking  up  DSZ.

SlugBait  logic  has been simplified, reporting any failed file transfer where

any of the last 1K of the file was sent with Q status.

Introduced ZMODEM RLE compression with the -Z option.  (registered DSZ  copies

also  accept  "Z"  in the DSZOPT environment variable.) ZMODEM RLE compression

packs runs of  the  same  character  in  two  or  three  character  sequences.

Compression  is  useful  for  screen  dumps, uncompressed workspaces, compiler

listings, and other files rich with strings of repeated  characters,  but  not

.ARC,  .ZOO and other compressed files.  Compression percentages range from -1

per cent for already  compressed  files  to  more  than  1000  per  cent  (10x

speedup!)  for the Personal Computing text file benchmark(!).  When sending to

programs that do not support ZMODEM compression, the  -Z  option  is  ignored.

The -Z option is effective when specified to the sending program only.

Good news for  tired  frog  stompers!   The  ZMODEM  rz/sz  programs  are  now

available  in  a  VMS C flavor.  The VAX/VMS source files are contained in the

RZSZ.ARC (RZSZ.ZOO) archive file.

"Handshake slow" has been added to disable the RTS signal to the  modem  while

writing  to  disk,  even  if  the interrupt buffer is not chock full of bytes.

This should help systems with excessive interrupt latency on disk writes.

Setting the v (verbose) parameter to 1 or greater causes serialized copies  to

ring the bell when a transfer is done.

DSZ.DOC has been extensively rewritten and reorganized.  DSZ now supports  the

advanced  buffering of the National NS16550AN and Intel 82510 serial interface

devices.  Please refer to the subchapter on "Brain Damaged UARTs".

Chapter 15 Pg 43         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

Pirated/hacked versions of DSZ have cost users and DSZ's  developer  thousands

of  dollars  in  lost  time  and  other  expenses, in addition to injuring the

reputation of Omen Technology and its products.  Regrettably, Omen  Technology

cannot support prior versions of DSZ because of this piracy.

Name             Length                           Date        Time   CRC

dsz.com          51422  Crunched   15%     43722  22 Jan 90   0:13a  7ab9

dsz.exe          61719  Crunched   19%     50384  22 Jan 90   0:58a  ce41

PKZIP -vt 32 bit CRC: 49FE057E   51422 dsz.com

PKZIP -vt 32 bit CRC: D6E797D6   61719 dsz.exe

Your copy of DSZ should show this information as provided by

arc v dsz

or

pkxarc -v dsz

lharc v dsz

ARC, and LHARC use a 16 bit CRC.

ZIP uses a 32 bit CRC compatible with the  Professional-YAM  and  ZCOMM  "crc"

command.

pkzip -vt dsz

The following Pro-YAM/ZCOMM command gives the 32 bit CRC for DSZ.COM:

EXAMPLE: >>>c: crc dsz.com

If your copy shows a different CRC or file length, insist on a legitimate copy

and report the source of the bogus/pirated copy to Omen Technology Inc.  Omen

Technology has received reports of serious problems caused by unauthorized

patches to DSZ, and cannot assume liability for the performance of the

resultant code.

Chapter 16 Pg 44         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

16.  Other Omen Technology Products

Omen Technology makes Professional-YAM, a comprehensive comms program with

unmatched power.  Flavors are available for PC-DOS, Unix, Xenix, and OS/2

Protected mode.

ZCOMM is a full featured shareware version with a $40.00 basic registration.

Registered DSZ users upgrading to ZCOMM get a free binder!

Chapter 17 Pg 45         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

17.  FAIR PLAY

Unlike snazzy menu interfaces, reliable high speed transfer protocols take

many hours of testing, often over expensive phone connections, to develop.

Two decades of communications experience and protocol testing are distilled in

DSZ.

DSZ is a user supported program developed and supported by Chuck Forsberg,

owner of Omen Technology.  Countless hours of work and expen$ive resources

went to develop and support DSZ for the myriad environments users use it in.

The fact that most of the problems people encounter with DSZ are caused by

other software, hardware problems, failure to read the supplied documentation,

or pirating has not kept users from seeking and getting help, even if it means

calling the author in the middle of his sleep.

Beyond this, the world of communications is constantly evolving, and it

requires a significant ongoing effort to keep DSZ and its documentation up to

date with the current state of the art.  Individuals with little knowledge of

the dynamic nature of telecommunications (or hidden agendas) have

misrepresented the frequency of enhancements to DSZ as a sign that DSZ is

"unstable" and unworthy of "serious consideration".  If you think such sniping

is fair play, you are more than welcome to use one of the many other protocol

programs available, each of which claim to offer speed and reliability

comparable to DSZ.

Non registered users are welcome to use DSZ for a reasonable evaluation

period.  "Reasonable" means "until you've decided whether the program is worth

$20.00".  It does not mean "until some competing program gets around to adding

ZMODEM or True Ymodem(TM)".  Once you've evaluated DSZ, ethical use of DSZ

requires registering the copy.

DSZ is not and never has been Public Domain.  Every version of DSZ has

included a Copyright Notice and a prohibition against sale or modification.

There is no usage counter or other Trojan Horse in DSZ, but if you use this

program, you have a moral and ethical obligation to register it.

Paid registration enables OverThruster(TM), YMODEM-g, and other advanced

features that are present in DSZ.ARC in encrypted form.  The registration

package serializes and decodes all past and future versions of DSZ.COM and

DSZ.EXE.  You can get a fully featured upgrade of DSZ simply by downloading a

newer version from TeleGodzilla, GEnie (IBM Roundtable), CompuServe, and

trusted bulletin boards.

17.1  Sharing DSZ Files

You are welcome to share unmodified copies of DSZ.ARC or DSZ.ZOO with your

friends and associates for their evaluation.  This documentation file and the

MAILER file must be provided with all copies of DSZ except by prior written

permission from Omen Technology Inc.  Because of piracy problems, Omen

Technology must INSIST that DSZ be distributed only as part of the DSZ.ARC

file EXACTLY as provided by Omen Technology Inc, or in an archive which

Chapter 17 Pg 46         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

displays a CRC value for DSZ.COM equal to that shown in the Changes chapter.

If DSZ.ARC is "zipped" or otherwise repacked, no files may be added to or

removed from the archive.

If you see an improperly packed or otherwise modified DSZ archive or parts

thereof in a bulletin board, please ask the board's operator to replace such

files with DSZ.ARC exactly as provided by Omen Technology Inc.  Flame them

until they behave ethically.

No version of DSZ may be sold without the prior written permission of Omen

Technology Inc.

DSZ is not and never has been Copy Protected.  "DSZ unprotect" files have no

function whatsover except to aid and encourage the theft of Omen Technology's

Intellectual Property.

Distribution or use of programs or procedures that modify, patch, disassemble,

decompile, or otherwise reverse engineer DSZ without the prior written

permission by Omen Technology Inc is a bloody theft of intellectual property

and services and is a violation of Federal Copyright Law.  The same applies to

debug scripts and documentation written for the same purposes.  These perform

the same function as pirate satellite and/or cable descramblers, to obtain

benefit from encrypted communications without proper payment.  Piracy has been

investigated by the FBI and prosecuted under federal statutes.  Applicable law

provides for damages and recovery of attorney's fees.  Omen Technology has

received reports of serious problems caused by unauthorized patches to DSZ,

and cannot assume liability for the performance of the resultant code.  So

far, none of the slime producing such bilge have seen fit to sign their real

name and address to their cretinous attempts at ego gratification.

Distribution of DSZ in any medium without this documentation file is an

unauthorized modification to DSZ that deprives the rightful author of

registration revenues.  That is a theft of intellectual property.  Such

distribution damages the author by forcing the user and author to waste time

and phone bills scrounging for the documentation, and answering questions that

are fully covered in the documentation.

If any version of DSZ is distributed with any other product, software or

hardware, the product documentation must clearly and prominently state that

the customer's continued use of DSZ requires registration directly with Omen

Technology or its duly authorized agent(s).  When unregistered copies of DSZ

are distributed on disk, a contemporary copy of ZCOMMDOC.ARC should be

included on the same disk.

Thank you for your support.

Chapter 17 Pg 47         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

17.2  REGISTRATION

After a short evaluation period, ethical users must register their copy of DSZ

with a check for $20.00 in U.S. funds sent to:

Omen Technology Inc.

P.O. Box 4681

Portland, OR 97208

The address on your check is sufficient, no need to fill out forms.

For VISA/MC Registrations, you can also:

Call VOICE: 503-621-3406

Or register online: 503-621-3746  300-1200-2400 bps

Or FAX the INSTANT MAILER to: 503-621-3735

Don't bother scrounging for an envelope, just print the INSTANT MAILER

included in this archive!

Overseas registrations are sent out International Air Mail.  Users, especially

those overseas, may choose to register their copy of DSZ for $25.00 U.S.  by

providing their VISA or Mastercard card number with expiration date and their

name as it appears on their credit card.

Purchase orders for less than $150 must be prepaid.  Omen Technology Inc

Federal TAX ID #: 930858776 Oregon TAX ID #: 536565

*************************************************************************

The user registration package includes a printed serial number password (SNP)

card.  The registration "care package" diskette contains the latest versions

of the DSZ programs, additional user documentation, and the installation

program which uses your personal SNP to enable advanced features and bypass

the opening advertisement.

The registration package unlocks the extra value in all past and future

versions of DSZ.COM and DSZ.EXE.

*************************************************************************

The unique serial number on the registration card is registered to the user,

and is acceptable proof of registration to corporate auditors.

The putsnp program inserts your unique DSZ Serial Number Password (SNP) into

you copy of DSZ with the following command:

putsnp dsz.com

Putsnp accepts your SNP and asks appropriate questions before inserting the

SNP to enable the extra value in DSZ.

Some TSR programs have been known to interfere with putsnp.  If you have a

lockup running putsnp, run it without any TSR programs.

Chapter 17 Pg 48         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

The only legitimate way to disable the opening advertisement in DSZ is to

insert an authorized SNP obtained from Omen Technology into DSZ with the

putsnp program.  Any other suppression or modification of the opening screen

is an illegal, unethical theft of services.  Omen Technology has received

reports of serious problems caused by unauthorized patches to DSZ, and cannot

assume liability for the performance of the resultant code.

Piracy is stealing.

17.3  Files on Registration Disk

The following files are currently included on the Registration Disk sent to

paid registrants.

LOOZ.EXE       Program to unpack .ZOO archives.  Please note that commands to

LOOZ must be given in lower case.  "looz x dsz" works, "LOOZ X

DSZ" fails.  LOOZ.DOC describes LOOZ.EXE.

DSZEXE.ZOO     Contains DSZ.EXE.

To dearchive with looz: looz x dszexe (Note: x must be lower

case, X won't work!)

Type "looz" without arguments to see the help message.

DSZ.ZOO        Contains DSZ.COM, this document, DESQview description files,

and an instant no-effort registration mailer.

To dearchive with looz: looz x yzmodem (Note: x must be lower

case, X won't work!)

RZSZ.ZOO       ZMODEM C source code, porting base for several dozen ZMODEM

implementations

To dearchive with looz: looz x rzsz (Note: x must be lower

case, X won't work!)

PUTSNP.EXE     Installs your unique registered serial number to enable the

extra value in DSZ.

DSZBG.DOC (in DSZBG.ZOO) Background information for DSZ, consisting of

relevant parts of the ZCOMM User's Manual.  DSZBG.DOC is for

paid registered users only; registered bulletin boards and

unregistered users are welcome to read the same information

contained in the ZCOMM manual (contained in ZCOMMDOC.ARC).

Chapter 17 Pg 49         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

To dearchive with looz: looz x dszbg (Note: x must be lower

case, X won't work!)

17.4  Bulletin Board Registration

Use and/or copying of DSZ by bulletin board sysops and/or systems is allowed

according to the conditions stated in the accompanying bbs_reg.txt form,

including but not limited to the display of the ZCOMM files.

(For this BBS registration, a multi node bulletin board operating from a

single, common file system counts as one machine.)

The purpose of bulletin board registrations is to support and promote ZCOMM

and Professional-YAM by popularizing the ZMODEM protocol and featuring the

ZCOMM shareware files available for downloading.

BBS registrations must be submitted on the bbs_reg form with signature and

date, and must accompany a paid DSZ or ZCOMM registration.  The paid

registration that must accompany your BBS registration can be one submitted by

one of your board's users.  Most boards have many unregistered DSZ users, so

getting one of them to register their DSZ should pose no difficulty.  Since

the paid registration may be from one of your board's users, you can get your

BBS registered for free by getting one of your users to register his/her DSZ

at the same time.

Chapter 18 Pg 50         DSZ Reference Rev Jan-28-90           Typeset 1-28-90

18.  ACKNOWLEDGEMENTS

32 bit CRC code courtesy Gary S. Brown.

ZMODEM-90(TM), MobyTurbo(TM), The High Reliability Software(TM), Demand

Upgrade(TM), TurboLearn Script Writer(TM), Cybernetic Data Recovery(TM),

ZMODEM AutoDownload(TM), Intelligent Crash Recovery(TM), Error

Containment(TM), Full Time Capture(TM), True YMODEM(TM), OverThruster(TM),

Password Guardian(TM), CryptoScript(TM), and TurboDial(TM) are Omen Technology

trademarks.

PowerCom was trademarked in 1985 by POLYTRON Corporation, Hillsboro Oregon.

True YMODEM(TM) is an Omen Technology Trademark signifying YMODEM protocol

support that meets the full YMODEM specification.

DSZ and this documentation are Copyright 1990 by Omen Technology Inc, all

rights reserved.  DSZ is not and never has been Public Domain.  Every version

of DSZ has included a Copyright Notice and a prohibition against sale or

modification.  This prohibition applies to any theft of the intellectual

property and trade secrets contained in DSZ by decompilation, disassembly, or

any other form of reverse engineering including but not limited to profiling,

tracing, data analysis, or monitoring the operation of DSZ.

Omen Technology copyright applies to its expression of the encoding and

compression ideas expressed as code and byte streams used in the ZMODEM-90

enhancements.

DSZ is not and never has been Copy Protected.  "DSZ unprotect" files have no

function whatsover except to aid and encourage the theft of Omen Technology's

Intellectual Property.

Omen Technology Incorporated

The High Reliability Software

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

Modem: 503-621-3746 Speed 1200,2400,19200

FAX: 503-621-3735

CompuServe:70007,2304  GEnie:CAF