----------------------------------------------------------

Note

----------------------------------------------------------

Final Approach is not (yet) shareware or a commercial program

and will probably never be freeware.

I'm making this evaluation version publicly available because

at this stage of development I need some feedback from real FS

enthusiasts (and you probably are one of those). Please give

Final Approach a test run and tell me if it could be of any

concrete use for you while flying FS or ATP.

Depending on the kind of feedback I'll receive, I will continue

working on the project, despite the fact that my leisure time

is getting pretty close to zero point zero at the time being.

If you tell me that these charts are hardly usable for a

"serious" approach and that you do absolutely need the real thing,

Final Approach will still continue to grow, but certainly at a

much slower pace.

So thanks for any comment!

Georges Lorsche, Luxembourg  (CIS: 100041,211)

----------------------------------------------------------

About Final Approach

----------------------------------------------------------

Most modern flight simulators have reached a level of realism

that makes the use of real-world approach charts a necessity.

Unfortunately only few charts are generally included with the

documentation of these programs. While ATP has got a whole

bunch of them, they are as rare as hen's teeth within the docs

of FS4 or the Scenery Disks.

Because buying the original charts is hardly affordable for

most of us, I had the idea of a program, that would replicate

those charts as realistically as possible on screen. If the

data was compressed to a reasonable size, the program could

include virtually hundreds of charts at the prize of a dozen

originals.

For a long time I worked on a DOS program, that would comply

to this specs. But the intended software should also generate

printouts in the best possible quality and this is where I

got stuck. I simply found no way to write printer drivers for

dozens of printers I had no access to.

When Visual Basic was released, I took my old Pascal code and

adapted it to the new language. With Windows taking care of

the print job, I could now concentrate on designing the charts

and came up with a workable program in less than 300 hours.

The data required to display a whole chart (plan view, profile

and missed approach instructions) now takes up as little as

350 byte, so that up to 1000 (!) charts will fit on a single

360K disk.

This evaluation version includes just 20 of them, but that

should be largely enough to give you an idea of what the

final product could be. Above all I need your feedback. I had

to simplify the charts to fit into a given database structure,

did I go too far? Are these charts usable for a "serious"

approach, or do you absolutely need the real thing?? What else

would you like included in such a program?

Thank you for reviewing an unfinished program and for helping

me develop and fine-tune a product, that should, as I hope,

some day meet the needs of many of you.

----------------------------------------------------------

Setting up Final Approach

----------------------------------------------------------

A. Minimum configuration

Final Approach requires a 286 or higher processor, 2 Mb of

RAM, an EGA or higher adapter and any printer supported by

Windows.

The Visual Basic runtime library VBRUN100.DLL must be present

in the Windows directory. You can download it from almost any BBS.

B. Installation

Copy the files included in FINALAPPA.ZIP to any directory of your

hard disk. They will take up less than 300K.

C. Running Final Approach

From within the Windows program manager, choose File - Run...,

then type the path and name of FINALAPP.EXE into the dialog

box. Click OK.

To install Final Approach permanently into one of your program

groups, choose File - New - Program Item and then enter a

description and path to FINALAPP.EXE. From that moment on, you

can run the program by double-clicking on the Final Approach

icon.

----------------------------------------------------------

A short guide to Final Approach

----------------------------------------------------------

A. Loading a file & selecting a chart

The data for the charts is contained in files with the

extension *.cht. Before selecting an approach chart, you must

tell Final Approach which of these data files to use.

- Select File/Open...

This evaluation version includes a single chart file

("default.cht"), so your choices are somewhat limited. After

opening the file, a list of all available approaches is

displayed.

- Select any chart from the list

Final Approach returns you to the main menu. The silhouette of

a landing 747 indicates the presence of a chart in your

computer s memory. Below the graphic the current approach is

identified.

B. Displaying a chart on the screen

Because the resolution of our current generation of monitors

would hardly allow for a complete chart to be displayed and

remain easily readable, I split each chart into its two major

components.

- Plan View shows the geographic approach course, including

Navaids, intersections, radials and required headings. The

holding can optionally be included.

- Profile View displays the vertical flight path from

initial altitude to the missed approach point. It includes

glidepath and timing information, rates of descent and the

describing text for the missed approach procedure.

While both views cannot be displayed together on the screen,

they can be printed out combined into a single approach plate.

C. Switching to another chart

You can easily switch to another chart from within the same

chart file.

- Select File/Other Chart

D. Printing charts

There are several ways to print a chart. From within the Print

menu, Plan View and Profile View may be printed separately or

combined into a single chart. Try the latter:

- Select Print/Both

Depending on your installed memory and type of printer, the

printout will take more or less time. While this will give you

some leisure time (typically 1.5 to 3 minutes), the resulting

vector quality should be worth the wait.

An alternate way to print a chart is available while the chart

is displayed on the screen. You might have noticed the related

buttons to the right of each chart.

- Print Fine prints the current view in vector quality

- Print Screen generates a hardcopy directly from the

screen s contents (limited to the map items of course).

While the latter is generally faster, the quality depends on

the resolution of your graphics adapter. Consider this option

only if the vector printout does not work on your hardware for

some reason or another (with Windows you never really know!).

E. Wind Correction Angles

Before you have a look at the simple navigation calculator,

you should enter some wind data:

- Select Wind/Enter data

Change the wind settings by typing into the appropriate boxes

or by moving the horizontal scroll bars with the mouse.

Click Save to make those data the default settings or click OK

to keep them in use for the current session only.

- Select Display/Correction Angles

The appearing table shows you exactly, how the current wind is

going to act on the selected approach and what to do about it.

Column 1 lists all headings you might be required to fly

within this approach (course, backcourse, procedure turn,

holding).

Column 2 gives you the wind correction angle for each course.

Column 3 shows the corrected heading, i.e. the direction where

the nose of your aircraft should point to. This heading will

assure proper compensation for undesirable wind drift.

Column 4 computes the wind influence on your ground speed and

indicates the real speed above ground for each segment. This

information is crucial for a well timed approach.

The graph illustrates the situation for the final segment of

the approach.

The blue arrow symbolizes the required approach course, the

red arrow corresponds to the wind direction.

On top of the blue arrow, a smaller, black arrow gives you a

visual cue as to how much you will have to turn your aircraft

towards the wind to remain on the horizontal approach course.

F. Selecting another aircraft

If you want to switch to a different aircraft, you can do so

from the main menu.

- Select Aircraft then click on any aircraft in the list

If you reopen the wind correction chart, Final Approach will

have adapted the correction angles and ground speeds to match

the default speeds of the new aircraft.

G. Options

- Select Options/Settings

Several features can be activated/deactivated from here. Just

check or uncheck the approapriate box.

<Maximize Maps> automatically enlarges the selected approach

charts to full display size.

<Display Holding> and <Print Holdding> include the holding

into the displayed or printed chart. Not using this feature

makes the chart somewhat easier to read and will speed-up the

printout.

<Ask: Printer ready?> makes Final Approach display a dialog

box asking the user to turn the printer online before

beginning a print job. This feature makes sense in case you

don t use the Windows Print Manager.

The combo boxes on the right side allow setting the type of

font Final Approach should use for displaying and printing the

charts. The default values will probably be your best choice,

but if you installed a Type Manager like ATM or FaceLift you

might want to experiment a bit to find the best-suited

combination of screen and printer fonts for your system.

If you select a font that does not seem appropriate, Final

Approach will warn you, but of course you may go ahead and see

for yourself. If the font does not work for you, just go back

and select another one.

When you are done with the settings, you may save them to make

them automatically current on the next startup.

- Now select Options/Aircraft

Final Approach will manage 10 aircraft for you. You decide

about their names and default speeds.

Enter a name and the desired speeds into the dialog boxes and

click Save or OK. Your new aircraft will immediately appear in

the aircraft menu and can then be selected for use in the wind

correction module.

If you want to return to the default options, you may of

course retype them into the option menus (if you remember

them that is!). An easier way will be to erase the file

FINALAPP.CFG from your disk. Final Approach will then

automatically reset all options to their default settings.

Have fun!! (And don't be shy on giving some feedback!)

Georges Lorsche, CIS: 100041,211