----------------------------------------------------------------------

----------------------------------------------------------------------

DOCUMENT:

TSR DMA SOUND FILE PLAYBACK ROUTINE

Version 1.12

JUNE 7, 1991

Programming and Documentation by Ryan Hanlon

Copyright (c) 1991,  Covox, Inc.  All Rights Reserved

*************************** BETA VERSION *****************************

----------------------------------------------------------------------

PROGRAM NAMES : tsrplay.exe

----------------------------------------------------------------------

GENERAL DESCRIPTION

The program TSRPLAY.EXE was designed to run as a background task and play

back a sound file(s) a variable number of times.

TSRPLAY.EXE will accept up to 20 files for playback (although that many

files will probably not fit on the command line).  Each file can be given

a rate or the rate will be read from the header.  If the file does not have

a Covox 16 byte header and no rate is specified on the command line then

it will be assumed that the file is 8 bit and was recorded at the default

rate of 132 (i.e., 9622 KHz).

Each file that is specified on the command line will be played back once

and TSRPLAY.EXE will then 'sleep' until another file is specified.  If the

'/N' option is used the file(s) can be played back more than once.  If the

'/N-1' option is used the file will be played back indefinately until the

program is explicilty removed from memory using the 'Q' option.  At any time

during play back the program can be removed from memory using the '/Q'

option.

Future releases of this software will ...

1. ... support expanded and extended memory.

2. ... be controlled via a script file.

3. ... be callable from another program through an interrupt (using the

above script).

4. ... have the abliity to add files to the list of files currently

playing.

If you have suggestions, comments or problems with this software please

leave a message on the BBS or contact Ryan Hanlon directly at Covox.

----------------------------------------------------------------------

SYNTAX : TSRPLAY file1 [file2...fileN] [OPTIONS]

----------------------------------------------------------------------

OPTIONS:

/Rxxxxx - Rate in hertz to play back file (4679-44191). 8000 is the default.

/Bxx    - Number of playback buffers (2-5). 3 is the default.

/Kxx    - Size in kilobytes of each of the buffers (1-65). 32 is the

default.

/Nxxx   - Number of times to playback each file. 1 is default. Use -1 for

continuous repeat of file(s).

/Z      - Pause output.

/W      - Restart paused output.

/F      - Flush all files being played back.

/Q      - Uninstall TSRPLAY.EXE program.

/S      - Install/Uninstall quietly (no screen output).

The following Voice Master and Sound Master II DMA settings can be selected.

/Jx     - Select jumper setting (1-4).  0 (default) for sequential search.

/Cx     - Select DMA channel (1 or 3).  0 (default) checks 1 then 3.

/Ix     - Select IRQ (3 or 7).  0 (default) checks 7 then 3.

----------------------------------------------------------------------

EXAMPLES :

- Install TSRPLAY.EXE to play back 2 sound files.

C:\TSRPLAY>TSRPLAY c:\songs.v8 c:\voice.v3s

- Install TSRPLAY8.EXE to play back 1 sound file at a 9622 KHz.

C:\TSRPLAY>TSRPLAY c:\8bitfile.v8  /R9622

- Install TSRPLAY.EXE to play back 1 sound files. Set the buffer

size to 16 kilobytes.

C:\TSRPLAY>TSRPLAY c:\songs.v8 /K16

- Install TSRPLAY.EXE to play back 1 sound files. Set the number

of buffers to 5.

C:\TSRPLAY>TSRPLAY c:\songs.v8  /B5

- Install TSRPLAY.EXE to play back 3 sound files. Each file is

specified with a playback rate.

C:\TSRPLAY>TSRPLAY c:\f1.v8 /R8000 c:\f2.v3s /R10000 c:\f3.v4s /R12000

----------------------------------------------------------------------

VERSION RELEASE CHANGES :

Changes from Version 1.11 to 1.12

---------------------------------------------------------------------------

- The previous versions of this program played back the sound file(s)

repeatedly until TSRPLAY.EXE was uninstalled with '/Q' on the command line.

This version will only play back the file(s) once unless otherwise

specified (See OPTIONS above).  After the file(s) have completed

TSRPLAY.EXE will 'sleep' but the resident code will remain in memory.

Another request to TSRPLAY.EXE will no longer result in an error message.

- TSRPLAY.EXE can now be paused and unpaused in the middle of playback.

- Files can be be flushed from the playback queue at any time with use of

the '/F' flag on the command line.