HELP.DOC

~~~~~~~~

ANSI PIT TERM ver 3.53 (c) Copyright 1991,92 by Midas Touch Software

(FREEWARE VERSION)    (c) Copyright 1991,92 by James R. Berry

FORMAT: PTERM <COM PORT> <BAUD RATE>

SAMPLE: PTERM 1 2400

Term runs on COM PORT 1 at 2400 Baud

Options -   /IRQ?     = Set IRQ

/ADDR???? = Set Port Address in HEX

/FLOW     = Force CTS/RTS flow control on

(Required for HIGH SPEED MODEMS!)

/NOBAUD   = Makes the term ignore the baud rate settings. The

baud rate defaults to that previously set by the

com package.

/SHAREIRQ = Force SHARED IRQ's ON

EXAMPLE: PTERM 3 2400 /IRQ5 /ADDR03E8

Term runs on COM 3 at 2400 baud on IRQ 5 - Address 03E8 Hex

EXAMPLE: PTERM 2 19200 /FLOW

Term runs on COM 2 at 19200 baud with FLOW CONTROL.

The /FLOW parameter is required to make HIGH SPEED

modems work properly.

EXAMPLE: PTERM 1 19200 /NOBAUD

This parameter allows HIGH SPEED modems with ASL or AUTO SPEED

LEVEL adjustment (New HST Duals) to work properly. These modems

continually adjust their baud rate and any attempt to set a baud

rate by our game can cause lots of "LINE NOISE" errors.

EXAMPLE: PTERM 1 2400 /SHAREIRQ

Shared IRQ's are required if you need to share them

to make your mouse work in the TERMINAL. A lockup

will occur if you are sharing IRQ's but the software

is not set to handle it. COM 1 & COM 3 Share IRQ 3.

COM 2 & COM 4 share IRQ 4. So if you had your mouse

on COM 1 and modem on COM 3 they will conflict if

SHARE IRQ's is not turned on.

NOTE: Never SHARE IRQ's when in DESQVIEW.