PHONEV4.COM -  Version 4 of PHONE!!!!!!!!  (8088 compatible)    July 21, 1990

Here's the source code and ready to run .COM file for my Phone line monitor

program...........

I have revised the PHONE program in several ways from previous releases. The

previous releases were having difficulty running on 8088 based machines. This

was due to some trickery in interrupt vector table.

The 80286/80386 BIOS initializes unused interrupts to point to a IRET statement,

the 88 BIOS does not. Therefore, when my program called an interrupt to check

for previous installation, the 8088 machines would crash. I have discontinued

using this method of checking for previous installation, PHONEV4 will run on any

IBM PC or compatible.

new syntax:

phonev4 /p## /r## [/s] [/u]

/p - port number 1 thru 4

/r - # of rings 1 thru 99

/s - sound on or off, (tone when phone is ringing)

/u - uninstall

Remember that the system will reboot when the number of unanswered rings

EXCEEDS the trigger-point you have specified.

Example: to monitor COM1 and reboot on unanswered ring number 7, use the

following command line.

PHONEV4 /p1 /r6

Just run it before the BBS..... It goes TSR, and if the phone line rings more

times than the number you specified, without having seen a carrier in between,

then the computer executes a reset, the same LOW-LEVEL reboot that //REBOOT

performs (WWIV SysOp command to reboot the system)........ Keep in mind that

this will only be helpful IF your BBS is booted from your AUTOEXEC.BAT file.

Here's an example of an AUTOEXEC.BAT file which will work with PHONEV4.COM:

Echo Off

Path=C:\;C:\DOS

Prompt $P$G

PHONEV4 /p2 /r6

cd \BBS

BBS

<End of File>

The preceding AUTOEXEC.BAT sets the PHONEV4 program to monitor COM2 and reboot

on unanswered ring number 7.

I have not yet tested it on COM1, but it works fine on COM2.....

Unanswered rings are cumulative...... the counter is zeroed out when a carrier

is detected....So, for example, suppose 9 rings was specified as the

trigger-point :

Caller A lets it ring 4 times, the gives up..

Caller B lets it ring 4 more times then gives up.

Caller C will reboot the system on his SECOND ring (4+4+2=10 which EXCEEDS 9)..

If either Caller A or Caller B had received a carrier, the ring counter

would have been reset, and the system would have to wait for 9 unanswered rings

to occur......

The amount of time elapsed between the three calls is irrelevent. The improtant

factors are:

1) whether or not ANY carrier had been detected (incoming OR outgoing calls)

2) whether or not the machine had been rebooted by some other mechanism

3) trigchg has been run (remember, trigchg is only for use on 286/386 machines)

Those are the only three conditions that will cause the program to re-set its

count of unanswered rings (i.e. start counting from zero again).

I have not yet tested it out on a PS\2 yet either, but I don't see why it

wouldn't work.. I don't do anything strange in the BIOS.... I just use regular

COMPORT addressing..

I figure if WWIV will run on a PS\2, then this will also.....

NOTE:

The com ports MUST follow standard convention. This means that:

COM1 == 03f8h

COM2 == 02f8h

COM3 == 03e8h

COM4 == 02e8h

If you are using COM3, but have it configured with a base port address as

02e8h, just LIE to my program , and tell it COM4. (I don't know anybody who

uses nonstandard com port base addressing, but I am sure they're out there).

If you use some OTHER base port address, and you have a copy of my source code,

just replace one of the above listed comport base addresses with the address

you use, re-assemble/link the program, and tell my program the COM number of

the COMport whose address you changed.

SPECIAL USAGE -

PHONEV4 /s

When PHONEV4 is already installed, the above command line will cause it to

toggle the "sound activity" (i.e. if its on, turn it off... if its off, turn it

on).

Scott........

Computer from Hell   1@13121 WWIVLink

������������������   The Depths of Hell BBS

(301) 638-0211