============================================================================

There were no .DOC files provided with this program by Intel, so I've put

together some simple instruction that should help ease things a little.

It's all pretty straight-forward and easy to run.

============================================================================

TESTCOM.EXE is a diagnostic utility for Intel modems and FAX/modems.  To

run it, turn on your modem, type TESTCOM, and press <Enter>.  Help is

available at most times by pressing <F1>.  The status line at the bottom of

the screen tells you what global commands are available to you at the time.

============================================================================

The first screen you will see is a title and copyright screen.

Press any key to continue.

---------------------

The first part of TESTCOM is a serial port integrity test.  Tests performed

are Serial Port Presence, Data Integrity, Modem Presence, Port Interrupt,

and Modem Determination.  Use the up/down arrow keys to select the port

that your modem is connected to and press <Enter>.  Standard serial ports

1 through 4 are supported.

1.  Com 1

2.  Com 2

3.  Com 3

4.  Com 4

Below is what you'll see when the Serial Port Integrity Test is running.

The words TESTING, PASSED, and/or FAILED will appear as appropriate in the

boxes as the test runs.  You will also be informed of approximately how

long it will take to run the tests.

���������Ŀ

Serial Port Presence test   �         �

���������Ĵ

Serial Port Data Integrity test   �         �

���������Ĵ

Modem Presence test   �         �

���������Ĵ

Serial Port Interrupt test   �         �

���������Ĵ

Modem Type Determination test   �         �

�����������

When the Serial Port Integrity Test is successfully completed, you will

go to the Main Menu.

---------------------

Below is the Main Menu.  A brief description of each selection follows.

Selection 1, Modem Diagnostics, is covered in more detail after the menu

selection's descriptions.

Use the up/down arrow keys to select the test to run and press <Enter>.

1.  Modem Diagnostics

2.  Check Phone line

3.  Dial Intel BBS

4.  Change COM port

5.  System and COM Status

6.  Terminal emulation mode

7.  Exit

1. MODEM DIAGNOSTICS: Tests whether the modem is working properly.  This

test can take about 1 minute to run successfully on a 14.4kbps modem.

Approximate test run-times are displayed on-screen.

2. CHECK PHONE LINE: Checks the phone line for the presence of a dial

tone.  It also checks the modem's internal connections to a telephone

that may be connected to the modem.

3. DIAL INTEL BBS: Dials, logs on, and displays Intel BBS info.  This

is a call to area code 503, so be aware of that when you see your phone

bill.  This will allow you to peruse the BBS, but not download files.

4. CHANGE COM PORT: Lets you change serial (COM) ports.

5. SYSTEM AND COM STATUS: Displays minimal serial port/modem/computer info.

6. TERMINAL EMULATION MODE: Terminal emulation mode.  TTY only.  No parity,

8 data bits, 1 stop bit (N81).  Port speeds to 57600 supported.  This

allows you to get online using the AT command set directly.  Does not

support file transfers.

7. EXIT: Returns to the DOS command prompt.  Resets the modem to the

settings it had prior to running TESTCOM.

---------------------

Modem Diagnostics

After selecting Modem Diagnostics from the Main Menu, you will be asked

how many times to run the test.  Once is usually sufficient for an initial

test of a new modem.  If you're having an intermittent problem, more cycles

may be necessary to catch it.

How many times do you wish to run the Diagnostics? (1-50)

Type in any number from 1 to 50 and press <Enter>.

Below is the Main Diagnostics Screen.  The words TESTING, PASSED, and/or

FAILED will appear as appropriate in the boxes as the test proceeds.

Note that if TESTCOM encounters problems, it will re-try a particular test

until it is sure that the result it got is not going to change.  This can

add a considerable amount of time to the test.  If a FAILED test occurs,

TESTCOM will beep and stop further testing.

���������Ŀ                           ���������Ŀ

Digital test  �         �              NV Ram test  �         �

���������Ĵ                           ���������Ĵ

Analog test  �         �      Carrier Detect test  �         �

���������Ĵ                           ���������Ĵ

ROM Checksum test  �         �      Data Set Ready test  �         �

�����������                           �����������

������������������������������ͻ    ������������������������������ͻ

� MODEM COMMAND/DATA SENT:     �    � MODEM RESPONSE/DATA REC:     �

� #                            �    � #                            �

������������������������������ͼ    ������������������������������ͼ

After the Modem Diagnostics Test runs successfully, you will be sent back

to the Main Menu, where you can make further menu selections, or exit to

DOS.