STS-61 PRESS KIT

FOR IMMEDIATE RELEASE

PUBLIC AFFAIRS CONTACTS

For Information on the Space Shuttle

Ed Campion	Policy/Management

Headquarters, Wash., D.C.

James Hartsfield	Mission Operations/EVA

Johnson Space Center, 	Astronauts

Houston

Bruce Buckingham	Launch Processing

Kennedy Space Center, Fla.	KSC Landing Information

June Malone	External Tank/SRBs/SSMEs

Marshall Space Flight

Center, Huntsville, Ala.

Nancy Lovato	DFRF Landing Information

Dryden Flight Research Facility,

Edwards, Calif.

For Information on STS-61 Payloads

Sarah Keegan	HST Program/Science

Headquarters, Wash., D.C.

Debra Rahn

Headquarters, Wash., D.C.	HST International Elements

Jim Elliott	HST Project/Science

Goddard Space Flight Center	STOCC Operations

Greenbelt, Md.

Michael Finneran	HST Project/Science

Goddard Space Flight Center	STOCC Operations

Greenbelt, Md.

Bob MacMillin	Wide Field/Planetary Camera-II

Jet Propulsion Laboratory

Pasadena, Calif.

Jean Paul Paille	European Space Agency

ESA, Hq., Paris

Ray Villard	HST Science, COSTAR

Space Telescope Science Institute

Baltimore, Md.

CONTENTS

GENERAL BACKGROUND MATERIAL

General Release	3

Media Services Information	4

Quick-Look Facts	5

Shuttle Abort Modes	7

Vechicle And Payload Weights  8

Summary Timeline	10

Orbital Events Summary	12

CARGO BAY PAYLOADS & ACTIVITIES

HST Servicing Mission-01 (HST/SM-01) Overview

History of HST	14

Mission Objectives And Success	................15

First Corrected Image Availability	15

Science Accomplishments	16

ESA Role in HST Program	17

Servicing Mission Orbital Verification	17

Key Hubble Scientific Goals Following the First Servicing Mission	19

Hubble Space Telescope Rendezvous and Retrieval	19

Commands to Hubble	20

STS-61 Extravehicular Activities	20

Replacement Hardware and Instruments	21

Primary Servicing Tasks	25

Secondary Servicing Tasks	35

HST Tools and Crew Aids	35

Imax Camera 	43

IN-CABIN PAYLOADS

Imax Camera	43

Air Force Maui Optical System (AMOS)	44

DTO-667 Pilot Inflight Landing Operations Trainer (PILOT) 	44

STS-61 CREW BIOGRAPHIES

Richard O. Covey, Commander (CDR)	45

Kenneth D. Bowersox, Pilot (PLT)	45

Tom Akers, Mission Specialist 5 (MS5)	45

Jeffrey A. Hoffman, Mission Specialist 3 (MS3)	45

Kathryn C. Thornton,  Mission Specialist 1 (MS1)	46

Claude Nicollier, Mission Specialist 2 (MS2)	46

F. Story Musgrave, Mission Specialist 4 (MS4)	46

ACRONYMS AND ABBREVIATIONS	47

STS-61 General Release

FIVE SPACEWALKS TO SERVICE HUBBLE SPACE TELESCOPE HIGHLIGHTS

SHUTTLE MISSION STS-61

RELEASE:  93-204

The December flight of Endeavour on Space Shuttle Mission STS-61 will see

the first in a series of planned visits to the orbiting Hubble Space Telescope

(HST).  The 11-day mission has been designed to accommodate a record five

spacewalks with the capability for an additional two if needed.

The first HST servicing mission has three primary objectives: restoring

the planned scientific capabilities, restoring reliability of HST systems and

validating the HST on-orbit servicing concept.  These objectives will be

accomplished in a variety of tasks performed by the astronauts in Endeaour's

cargo bay.

Replacement of the spacecraftUs solar arrays Q HSTUs source of electrical

power Q tops the primary servicing task list.  This is because solar array

jitter, or excessive flexing which happens when the telescope passes from cold

darkness into warm daylight, may be compromising the structural integrity of

the arrays.

The objective to restore the HSTUs science capabilities will be

accomplished with the installation of the Wide Field/Planetary Camera-II and

the Corrective Optics Space Telescope Axial Replacement, both of which will

compensate for the spherical aberration of the primary mirror.

The installation of new gyroscopes, which are required to point and track

HST, along with fuse plugs and electronic units will increase the reliability

of the HST system.

Leading the seven-person STS-61 crew will be Mission Commander Dick Covey.

Pilot for the mission is Ken Bowersox. The mission specialists for the flight

are Kathy Thornton, Claude Nicollier, Jeff Hoffman, Story Musgrave and Tom

Akers. Working in pairs, Hoffman and Musgrave and Thorton and Akers, all of

whom have prevthe five spacewalks scheduled for flight days 4-8.

Launch of Endeavour on the STS-61 mission is currently scheduled for no

earlier than Dec. 1, 1993 at 4:57 a.m.  EST. The planned mission duration is 10

days, 22 hours and 36 minutes.  An on-time launch on Dec. 1 would produce a

3:33 a.m.  EST landing on Dec. 12 at Kennedy Space Center's Shuttle Landing

Facility.

STS-61 will be the 5th flight of Space Shuttle Endeavour and the 59th

flight of the Space Shuttle system.  The Hubble Space Telescope is an

international cooperative program between NASA and the European Space Agency.

- end-

STS-61 MEDIA SERVICES INFORMATION

NASA Select Television Transmission

NASA Select television is available on Satcom F-2R, Transponder 13,

located at 72 degrees west longitude; frequency 3960.0 MHz, audio 6.8 MHz.

The schedule for television transmissions from the orbiter and for mission

briefings will be available during the mission at Kennedy Space Center, Fla;

Marshall Space Flight Center, Huntsville, Ala.; Ames-Dryden Flight Research

Facility, Edwards, Calif.; Johnson Space Center, Houston and NASA Headquarters,

Washington, D.C. The television schedule will be updated to reflect changes

dictated by mission operations.

Television schedules also may be obtained by calling COMSTOR 713/483-

5817.  COMSTOR is a computer data base service requiring the use of a telephone

modem.  A voice update of the television schedule is updated daily at noon

Eastern time.

Status Reports

Status reports on countdown and mission progress, on-orbit activities and

landing operations will be produced by the appropriate NASA newscenter.

Briefings

A mission press briefing schedule will be issued prior to launch.  During

the mission, status briefings by a Flight Director or Mission Operations

representative and when appropriate, representatives from the science team,

will occur at least once per day.  The updated NASA Select television schedule

will indicate when mission briefings are planned.

STS-61 QUICK LOOK

Launch Date/Site:	Dec. 1, 1993/Kennedy Space Center, Fla., Pad 39B

Launch Time:		4:57 a.m. EST (approximate)

Orbiter:			Endeavour (OV-105) 5th Flight

Orbit/Inclination:	320 nautical miles/28.45 degrees

Mission Duration:	10 days, 22 hours, 36 minutes (approximate)

Landing Time/Date:	3:33 a.m. EST (approximate)/Dec. 12, 1993

Primary Landing Site:	Kennedy Space Center, Fla.

Abort Landing Sites:	Return to Launch Site - KSC, Fla.

TransAtlantic Abort Landing -

Banjul, The Gambia

Moron, Spain

Ben Guerir, Morocco

Abort Once Around - Edwards AFB, Calif.

Crew: Dick Covey, Commander (CDR)

Ken Bowersox, Pilot (PLT)

Kathy Thornton,  Mission Specialist 1 (MS1/EV3)

Claude Nicollier, Mission Specialist 2  (MS2)

Jeff Hoffman, Mission Specialist 3 (MS3/EV1)

Story Musgrave, Mission Specialist 4 (MS4/EV2)

Tom Akers, Mission Specialist 5 (MS5/EV4)

Cargo Bay Payloads:	HST SM-01 (Hubble First Servicing Mission)

HST Replacements:	SA (Solar Arrays)

WF/PC-II (Wide Field/Planetary Camera-II)

RSU-1, 2 & 3 (Rate Sensor Units 1, 2 and 3)

ECU-1 & 3 (Electronic Control Units 1 and 3)

MSS-1 & 2 (Magnetic Sensing Systems 1 and 2)

COSTAR (Corrective Optics Space Telescope

Axial Replacement)

SADE (Solar Array Drive Electronics)

Cargo Bay Equip:  HST FSS (HST Flight Support System)

ORUC (Orbital Replacement Unit Carrier)

SAC (Solar Array Carrier)

SIPE (Scientific Instrument Protective Enclosures)

ICBC (IMAX Cargo Bay Camera)

In-Cabin Payloads:IMAX (IMAX In-Cabin Camera)

Other:		  AMOS (Air Force Maui Optical Site)

DTOs/DSOs:	  DTO 648:  Electronic Still Camera

DTO 656:  PGSC Upset Monitoring

DTO 700-2:  Handheld Laser Ranging Device

DTO 700-8:  Global Positioning System Flight Test

DTO 1211:  Water Dumps at 10.2 psi Cabin

DSO 326:  Window Impact Observation

DSO 469:  Inflight Radiation Dose/Distribution

DSO 483:  Back Pain in Microgravity

DSO 485:  Inter-Mars Tissue Equivalent Counter

DSO 487:  Immunological Assessment of Crew

DSO 489:  EVA Dosimetry Evaluation

DSO 604:  Visual-Vestibular/Function of Adaptation

DSO 617:  Skeletal Muscle Performance

DSO 624:  Cardiovascular Response to Exercise

DSO 901:  Documentary Television

DSO 902:  Documentary Motion Picture

DSO 903:  Documentary Still Photography

STS-61 SPACE SHUTTLE ABORT MODES

Space Shuttle launch abort philosophy aims toward safe and intact

recovery of the flight crew, orbiter and its payload.  Abort modes include:

* Abort-To-Orbit (ATO) - Partial loss of main engine thrust late enough to

permit reaching a minimal 105-nautical mile orbit with orbital maneuvering

system engines.

* Abort-Once-Around (AOA) - Earlier main engine shutdown with the capability to

allow one orbit around before landing at Edwards Air Force Base, Calif.

* TransAtlantic Abort Landing (TAL) - Loss of one or more main engines midway

through powered flight would force a landing at either Banjul, The Gambia,

Moron, Spain, or Ben Guerir, Morocco.

* Return-To-Launch-Site (RTLS) - Early shutdown of one or more engines, and

without enough energy to reach Banjul, would result in a pitch around and

thrust back toward KSC until within gliding distance of the Shuttle Landing

Facility.

STS-61 contingency landing sites are the Kennedy Space Center, Edwards Air

Force Base, Banjul, Moron or Ben Guerir.

STS-61 VEHICLE AND PAYLOAD WEIGHTS

Vehicle/Payload				              Pounds

Orbiter (Endeavour) empty and 3 SSMEs	              173,014

Flight Support System (FSS)		              4,200

Imax Cargo Bay Camera (ICBC)		              608

Imax (in cabin)				              329

Orbital Replacement Unit Carrier (ORUC)	              6369

Solar Array Carrier (SAC)		              3829

Solar Array II				              702

Rate Sensor Units (3)			              73

Wide Field/Planetary Camera II		              613

Corrective Optics Space Telescope Axial Replacement   660

Electronic Control Units (2)			      35

Magnetic Sensing System (2)			      30

Co-processor					      140

Goddard High Resolution Spectrograph Redundancy Kit   7

Changeout complement total (launch)		      2300

High-Speed Photometer				      603

WF/PC I						      624

Solar Array I					      668

RSU (3)						      73

ECU (2)						      35

Changeout complement total (Earth return)	      2135

DSOs/DTOs					      104

Total Vehicle at SRB Ignition			      4,511,115

Orbiter Landing Weight				      209,383

STS-61 SUMMARY TIMELINE*

Flight Day One*

Ascent

OMS-2 burn (310 n.m. x 297 n.m.)

ICBC activation

NC-1 burn (310 n.m. x 302 n.m.)

Flight Day Two*

Remote Manipulator System checkout

Cabin pressurization to 10.2 psi

Space Support Equipment checkout/survey

Configure Flight Servicing Structure

NPC burn (310 n.m. x 302 n.m.)

NSR burn (310 n.m. x 304 n.m.)

Extravehicular Mobility Unit checkout

NC-2 burn (317 n.m x 305 n.m.)

Flight Day Three*

HST rendezvous operations

NH burn (320 n.m. x 305 n.m.)

NC-3 burn (320 n.m x 310 n.m.)

NCC burn (320 n.m. x 310 n.m.)

TI burn (320 n.m. x 312 n.m.)

HST grapple (320 n.m x 313 n.m.)

HST berth

HST survey

Group B powerdown

Flight Day Four*

HST Extravehicular Activity 1

(Hoffman and Musgrave:  Two Rate Sensor Units/Secondaries)

Flight Day Five*

HST Extravehicular Activity 2

(Thornton and Akers:  Solar Arrays)

Flight Day Six*

HST Extravehicular Activity 3

(Hoffman and Musgrave:  Wide Field/Planetary Camera; Two Magnetic

Sensing Systems)

Flight Day Seven*

HST Extravehicular Activity 4

(Thornton and Akers:  Corrective Optics Space Telescope Axial

Replacement/Secondaries)

Flight Day Eight*

HST reboost burns (320 n.m. x 313 n.m.)

HST Extravehicular Activity 5

(Hoffman and Musgrave:  Solar Array Drive Electronics/Secondaries)

Flight Day Nine*

Group B power up

HST grapple

HST unberth

HST release (320 n.m. x 313 n.m.)

Separation burns 1, 2 and 3 (320 n.m. x 311 n.m.)

Group B power down

Flight Day Ten*

Cabin pressurization to 14.7 psi

Off-duty half day (MS1, MS3, MS4, MS5)

Off-duty half day (CDR, PLT, MS2)

Flight Day Eleven*

Group B power up

Flight Control Systems checkout

Reaction Control System hot fire

Cabin stow

Flight Day Twelve*

Space Support Equipment power down

Deorbit preparations

Deorbit burn

Entry

Landing

*SPECIAL NOTE ON STS-61 SUMMARY TIMELINE

On every Shuttle mission, some day-to-day replanning takes place to adjust

crew and event timelines according to unforeseen developments or simply to

optimize the use of time in orbit.

During STS-61, the bulk of the daily replanning will be undertaken, while

the crew sleeps, by the planning shift in mission control.  During the EVA

days, this team will play a crucial role in making the most of the astronauts

time in Endeavour's payload bay.  To maximize crew productivity and to adapt to

any unexpected challenges, the planning team will have the ability to reorder

the sequence of individual tasks within any given spacewalk or to shift tasks

from one day's agenda to another.

Each day's replanning effort will produce an execute plan defining the

approach for the next day's activities in space and on the ground.

STS-61 ORBITAL EVENTS SUMMARY

EVENT		START TIME	VELOCITY CHANGE	      ORBIT

(dd/hh:mm:ss)   (feet per second)	      (n.m.)

OMS-2		00/00:45:00		461 	     310 x 297

NC-1		00/05:33:00		8 	     310 x 302

(adjusts the rate at which Endeavour is closing on HST)

NPC 		00/23:12:00		.5 	     310 x 302

(fine-tunes Endeavour's ground track to be exactly in line with HST track)

NSR		01/03:57:00		5.5 	     310 x 304

(adjusts Endeavour's closing rate on HST)

NC-2		01/04:32:00		12 	     317 x 305

(adjusts Endeavour's closing rate on HST)

NH		01/17:22:00		4	     320 x 305

(adjusts altitude of Endeavour's orbital high point, fine-tunes course to

arrive at a point 40 nautical miles behind HST at same altitude)

NC-3		01/18:10:00		10 	     320 x 310

(fired at 40 n.m. behind HST, begins closing in on HST; initiates closing

rate of about 16 n.m. per orbit aimed to arrive at a point 8 n.m. behind HST,

at same altitude as HST, two orbits later)

NCC 		01/20:23:00		TBD	     320 x 310

(first burn calculated by onboard computers using onboard navigation

derived from orbiter star tracker sightings of HST; fired while orbiter is

about closing on point 8 n.m. behind HST to fine-tune course)

TI		01/21:23:00		3 	     320 x 312

(fired upon arrival at point 8 n.m. behind HST; begins terminal

interception of HST)

MC1-MC4		TBD			TBD	     TBD

(mid-course corrections; calculated by onboard computers, double-

checked by ground; fine-tune final course toward HST, may or may required)

MANUAL		01/22:57:00		TBD			TBD

(Begins about 45 minutes prior to HST grapple, less than 1 nautical mile

from HST.  Commander takes manual control of orbiter flight, fires braking

maneuvers to align and slow final approach)

GRAPPLE		01/23:42:00		n/a			n/a

(HST captured with mechanical arm)

HST REBOOST	06/17:45:00		TBD			TBD

(Performed only if amount of available propellant allows, lifts Endeavour's

orbit to reboost HST's orbit while HST is in cargo bay)

HST REBOOST	06/18:33:00		TBD			TBD

(Performed only if amount of available propellant allows, lifts Endeavour's

orbit to reboost HST's orbit while HST is in cargo bay)

HST RELEASE	08/00:43:00		n/a			n/a

(HST is released from Endeavour's mechanical arm)

SEP-1		08/00:44:00		1 			320 x 313

(Begins a slow separation of Endeavour from vicinity of HST)

SEP-2		08/01:08:00		2 			320 x 313

(Increases rate at which Endeavour is departing vicinity of HST)

SEP-3		08/01:32:00		3 			320 x 311

(Puts Endeavour on final course departing vicinity of HST)

DEORBIT		10/20:31:00		508

LANDING		10/21:45:00

NOTE: All planned burns are recalculated in real time once the flight is

underway using the latest information for the position of HST and will likely

change slightly.  Depending on how accurate the orbiter's navigation and course

is at certain times, some smaller burns listed above may not be required.

However, the times for major burns and events are unlikely to change by more

than a few minutes.

HUBBLE SPACE TELESCOPE/SERVICING MISSION-01

(HST-SM-01)

OVERVIEW OF MISSION

History

Launched on April 24, 1990, NASAUs Hubble Space Telescope was designed to

be the most powerful astronomical observatory ever built.  And indeed, HST far

surpasses the capabilities of ground-based optical telescopes for many types of

research.  The keys to HubbleUs power are its operation in space, far above the

interference of the EarthUs atmosphere, and to the unique instruments it

carries as it orbits the planet.  In addition HST was the first observatory

designed for extensive on-orbit maintenance and refurbishment.

While the launch on the Space Shuttle Discovery more than 3 years ago was

flawless, Hubble was not.  Two months after HST was deployed into orbit 370

miles (595.5 km) high, Hubble produced a disquieting discovery not about space,

but about itself.  The curvature of its primary mirror was slightly Q but

significantly Q incorrect.  Near the edge, the mirror is too flat by an amount

equal to 1/50th the width of a human hair.

A NASA investigative board later determined that the flaw was caused by

the incorrect adjustment of a testing device used in building the mirror.  The

device, called a Rnull corrector,S was used to check the mirror curvature

during manufacture.

The result is a focusing defect or spherical aberration.  Instead of being

focused into a sharp point, light collected by the mirror is spread over a

larger area in a fuzzy halo.  Images of extended objects, such as stars,

planets and galaxies, are blurred.

NASA has been coping with HubbleUs fuzzy vision with computer processing

to sharpen images.  For bright objects, this technique has yielded breathtaking

detail never seen from the ground.  NASA also has been concentrating on the

analysis of ultraviolet light, which ground-based telescopes cannot see because

of the EarthUs intervening atmosphere.

To realize the full potential of HST, however, the spacecraft must be

serviced.  The telescope mirror itself cannot be fixed or changed.  However,

corrective optics can be applied to the HST instruments to compensate for the

aberration, much the same as glasses or contact lenses correct human sight.

The new optics should allow Hubble to accomplish most, if not all, of it's

originally planned objectives.

The mission, though, will accomplish much more than improved vision.

Hubble was designed to spend 15 years in space.  Even before the spherical

aberration was known, several servicing missions, including one in 1993, had

been planned so that failed parts could be replaced and others improved with

better technology.  This mission will perform that type of servicing in

addition to installing corrective optics.

Endeavour will carry some 16,000 pounds (7,257 kilograms) of servicing

hardware into space.  During nearly 2 weeks in orbit around the Earth,

astronauts will use the Shuttle as a kind of orbiting service station from

which they will venture to work on the 12.5-ton (11.34-metric ton) telescope as

it hurtles around the planet at 18,000 miles (28,968 km) an hour.

The crew will spend some 30 hours in space during at least five separate

spacewalk periods, undertaking a series of tasks more complex than any ever

attempted in orbit, to ensure that Hubble remains a viable and productive

national resource throughout its planned 15-year lifetime.

Mission Objectives and Success

The three objectives of the first Hubble servicing mission are to restore

the planned capabilities of the telescope by correcting the optics, to restore

reliability of the spacecraft and to validate that the concept of Hubble on-

orbit servicing is viable.

The top priorities are installation of the replacement solar arrays; two

rate sensing units, one with an electronics control unit; the Wide

Field/Planetary Camera II (WF/PC-II) and fuses; the Corrective Optics Space

Telescope Axial Replacement (COSTAR); at least one new magnetometer; and a new

Solar Array Drive Electronics unit.

For the first servicing mission to be considered fully successful, these

top priority items must be accomplished.  In addition, other tasks may be

performed on a time-available basis.  The minimum criteria for mission success

are to leave Hubble with three newer-design gyroscope systems and either an

operational WF/PC-II or COSTAR.

First Corrected Image Availability

The first fully corrected Hubble images are estimated to be available 6 to

8 weeks after the servicing mission.  This time is necessary for adjustments to

ensure telescope stability and the best possible focus.  During this period,

telescope operators on the ground will remotely calibrate the gyros, which help

keep the HST fixed on its targets, and position the corrective mirrors in the

Corrective Optics Space Telescope Axial Replacement (COSTAR) and the Wide

Field/Planetary Camera 2 (WF/PC-II).

COSTAR is being installed to remedy the blurred vision of three observing

instruments on HST. The WF/PC-II is a replacement camera that has its own

corrective optics.

More information on activities after STS-61 necessary to produce a fully

corrected Hubble image can be found in the section on Servicing Mission Orbital

Verification.

Science Accomplishments

Despite the flaw in the primary mirror, the bus-size Hubble still has

been able to gather a wealth of scientific data, most of which would have been

impossible to collect if the telescope did not exist.  In the last 3 years, HST

has conducted a variety of scientific investigations that have rapidly expanded

knowledge of what lies beyond the Earth, from the relatively nearby planets in

Earth's solar system to the most distant reaches of the universe.

HubbleUs studies have ranged from Earth's neighbor Mars, to the

evolution of stars from birth to death, to the characteristics of galaxies

beyond, and finally to a field known as cosmology, which probes the fundamental

nature of the universe itself.

The following is a small sampler of some of HubbleUs discoveries and

work in progress:

% The Planets

Even prior to the servicing mission, Hubble conducted and continues to

conduct long-term observations of global weather changes on Mars. Hubble has

observed the development of a rare, planet-wide storm on Saturn. The telescope

also resolved, as two distinct objects, the most distant planet in the solar

system, Pluto and its moon Charon. Previously, no telescope had been able to

separate clearly the two bodies.

HST also has been studying long-term weather changes on Jupiter and its

auroral activity.  Hubble also has been measuring the extent of the atmosphere

of the Jovian moon Io and also has looked for changes in the satelliteUs

surface.

% Stellar Evolution

Hubble uncovered the strongest evidence yet that many stars form

planetary systems.  This evidence was the discovery of disks of dust around 15

newly formed stars in the Orion Nebula, a starbirth region 1,500 light- years

away.  Such disks are considered a prerequisite for the formation of solar

systems like Earth's. The HST images confirm more than two centuries of

speculation, conjecture and theory about the genesis of a solar system.

% Star Clusters

HST discovered young globular star clusters at the core of a peculiar

galaxy.  The discovery of these stars early in their evolution was the

equivalent of finding a RJurassic ParkS in space.

The space telescope found Rblue stragglerS stars in the core of

globular cluster 47 Tucanae, providing evidence that some stars RcaptureS

others and merge with them.

% Gallaxies

HST uncovered circumstantial evidence for the presence of a massive

black hole in the core of the neighboring galaxy M32 as well as the giant

elliptical galaxy M87. Both galaxies have a central concentration of starlight

that probably is shaped by the gravitational field of the black hole.  This

implies that massive black holes may be quite common among RnormalS galaxies,

perhaps even Earth's.

Hubble yielded direct evidence for galaxy evolution by resolving the

shapes of galaxies that existed long ago.  HST revealed that many ancient

spiral galaxies have since disappeared, possibly through fading or collisions

and mergers with other galaxies.

% Cosmology

The space telescope allowed astronomers to take a major first step in

determining the rate at which the universe is expanding.  HST detected 27 stars

called Cepheid variables.  These stars are Rstandard candlesS for estimating

distances to galaxies.  The expansion rate, known as the Hubble Constant, is

one of two critical numbers needed for making a precise determination of the

size, age and fate of the universe.

HST discovered boron, the fifth lightest element, in a very ancient

star.  This star would have been one of the earliest formed after the Big Bang

explosion that most scientists believe began the universe.  If boron was

produced in the first few minutes of the birth of the universe, it implies that

the Big Bang was not a uniform explosion.

Hubble precisely determined the ratio of deuterium to hydrogen in

interstellar gas clouds.  This value shows that the universe has only 6 percent

of the observable matter required to prevent itself from expanding forever.

European Space Agency (ESA) Role in HST

The Hubble Space Telescope is a program of joint cooperation between NASA

and ESA. ESA provided Hubble's deployable solar arrays, the major source of

electrical power, which collects energy from the sun to recharge the

spacecraft's six nickel-hydrogen batteries.  ESA's second contribution was the

Faint Object Camera (FOC), which was intended for imaging of the faintest

objects in the visible and ultraviolet spectral regions at very high spatial

resolution.  These elements are discussed further in the section addressing

replacement hardware and instruments.

Claude Nicollier, a mission specialist on this flight, is an ESA

astronaut.

SERVICING MISSION ORBITAL VERIFICATION (SMOV)

The purpose of SMOV is to "recommission" HST so that it can begin science

operations as soon as possible following the first servicing mission.  This

involves a thorough engineering checkout of all serviced subsystems; optical

alignment and initial calibration of all science instruments; and the phasing

in of astronomical observations.  SMOV begins when HST is released from the

Shuttle and is expected to last approximately 13 weeks.

Key Activities During SMOV

% Activation and engineering checkout of the science instruments.

% Optical alignment and focusing of WF/PC-II and COSTAR.

% Initial calibration of WF/PC-II and the COSTAR-corrected science

instruments.

% Early science observations.

Engineering Checkout Activities

% Decontaminate the WFPC II detectors (charge-coupled devices or CCDs) of any

foreign substances by heating the detectors to "drive-off" contaminants.

% Establish proper operating temperature of WFPC II CCDs by monitoring

ultraviolet (UV) light from a calibration star.

% Monitor pressure drop (due to outgassing) until it is safe to turn on high

voltage to the COSTAR-corrected science instruments.

% Determine the effects of the servicing mission on the basic (pre-COSTAR)

optical performance of the science instruments.

Steps in Focusing the Science Instruments

% Check out the first generation instruments and conduct prefocusing tests.

% Adjust the secondary mirror in HST's Optical Telescope Assembly to set

focus for WF/PC-II and correct for residual coma in the Optical Telescope

Assembly.

% Deploy COSTAR arms.

% Adjust COSTAR and WF/PC-II optics and mirrors, including mirror tilt,

coarse adjustment, fine alignment and focus.

Science Instruments Calibration

% A series of tests and measurements to establish the actual performance of

the science instruments in the areas of sensitivity, resolution and detector

response characteristics.

KEY HST SCIENTIFIC GOALS FOLLOWING THE FIRST SERVICING MISSION

% Hubble will determine, precisely, the expansion rate of the universe by

measuring the light curve of Cepheid Variable stars in galaxies out to the

distance of at least 50 million light-years.

Cepheids are pulsating stars that become alternately brighter and fainter

with periods (duration of the states of brightness or faintness) ranging from

10 to 50 days.  Astronomers have known for over 50 years that the periods of

these stars precisely predict their total luminous power, which allows their

distance to be measured.

In the expanding universe, the Hubble Constant (Ho) is the ratio of the

recession velocities of galaxies to their distance. (Recession velocity is the

speed at which the galaxy is moving away from Earth.) The age of the universe

can be estimated from the Hubble Constant. The age currently is estimated to be

between 10 and 20 billion years, but a more precise measurement of the Hubble

Constant is required to narrow this range to an accuracy of 10 percent.

% HST will look for the gravitational signature of massive black holes in the

cores of normal and active galaxies.  A black hole is a theoretical object that

is so compact and dense, nothing can escape its gravitational field.  The HST

spectrographs will measure precisely the velocities of gas and stars orbiting

the center of a galaxy.  If the stellar velocities increase rapidly toward the

galaxy center, it would be the signature of a massive, compact central object.

% Hubble will be able to determine the shapes of galaxies that are very

distant.  Because remote objects also are relics of the early universe, HST

will be able to study how galaxies have evolved since the beginning of the

universe.  Nearby galaxies have spiral, elliptical and irregular shapes,

however, these shapes should have changed over time because the universe is

evolving.

% Hubble will be able to precisely measure the ages of globular clusters by

observing the faintest stars in the clusters.  Globular clusters are considered

to be the oldest objects in the universe, and their ages provide insights into

how stars evolve and also provide an independent estimate of the age of the

universe.

HUBBLE SPACE TELESCOPE RENDEZVOUS AND RETRIEVAL

The rendezvous and retrieval operaTelescope will be similar to those

conducted on previous missions requiring capture of a free-flying satellite in

orbit.

For the HST mission, Endeavour's crew will perform many orbit adjust burns

to catch up with and retrieve the telescope on flight day three of the mission

using the Shuttle's robot arm.

Once the Shuttle is safely in orbit and the payload bay doors opened, the

space support equipment activation is performed.  This includes activating the

flight support system and orbital replacement unit carrier heaters.  Other

pre-rendezvous activities will include checkout of the robot arm, the orbiter

Ku-band dish antenna used as a radar system during rendezvous and the ground

command system.

The terminal initiation burn occurs about 2 hours prior to capture at a

distance of approximately 40,000 feet (12,192 m) in front of the telescope.

Several small mid-course correction burns follow before the Commander takes

over manual control of the Shuttle about 1,200 feet (366 m) below and 500 feet

(152 m) behind the telescope.

The orbiter approaches Hubble from underneath, just after orbital sunset.

This approach technique is designed to minimize potential contamination from

the Shuttle's thruster firings.

Prior to capture, a ground-commanded maneuver of the telescope will be

performed to align the grapple fixture on the HST with Endeavour's robot arm.

The size of the telescope maneuver will depend on the angle to the Sun and

ranges from about 70 degrees to 180 degrees.

When the telescope is grappled, using the robot arm's end effector, it

will be lowered into the payload bay and berthed in the flight support system,

a turntable likened to a lazy susan for its rotation and tilt ability to assist

in the servicing tasks.  An electrical cable is remotely attached to provide

orbiter power to the telescope.

COMMANDS TO HUBBLE

Commands to HST are issued from the Space Telescope Operations Center

(STOCC) at Goddard Space Flight Center, Greenbelt, Md., which manages the

orbiting observatory.  The STOCC has been the nerve center for Hubble

operations since the telescope was launched.  Commands to Hubble are issued

from the STOCC and data gathered by the spacecraft arrive there first.

The STOCC is responsible for most commanding of the HST during STS-61,

although the crew can send a limited number of commands from Endeavour. The

STOCC will send commands configuring the space telescope for retrieval by the

orbiter; integrate commands with crew activities during extravehicular

activispacecraft hardware and perform hardware checkouts and send commands to

configure the space telescope for deployment from Endeavour.

STS-61 EXTRAVEHICULAR ACTIVITIES

A total of five spacewalks are planned on STS-61 to service the HST.

Unlike past Shuttle repair work performed on satellites such as Intelsat on

STS-49, HST was designed with the objective of servicing it in orbit through

Shuttle spacewalks.  As such, it has two grapple fixtures for the Shuttle's

mechanical arm, many handholds for spacewalkers and bolts and electrical

connections designed to be serviced by a spacewalker.

However, the amount of work to be performed on STS-61 has increased above

what originally was projected for the first servicing flight to the telescope

due to deficiences and equipment problems that have occurred or been discovered

since HST was launched.  Since there is such a large amount of work to be

accomplished on STS-61, the various tasks have been prioritized by the HST

program officials.

The primary tasks on STS-61 will be to install two Rate Sensor Units, one

with an Electronics Control Unit, the Solar Arrays, the Solar Array Drive

Electronics, the Wide Field/Planetary Camera and four instrument fuse plugs,

the Corrective Optics Space Telescope Axial Replacement and one Magnetic

Sensing System.

Secondary tasks that may be performed during the spacewalks if time

permits include installing the Goddard High Resolution Spectrograph Redundancy

Kit, a 386 Coprocessor, a second Magnetic Sensing System, four gyro Fuse Plugs

and one Electronic Control Unit. A third Rate Sensor Unit is being carried in

the payload bay for use if needed.

The spacewalks will be peformed by STS-61 extravehicular crew members Jeff

Hoffman, Story Musgrave, Kathy Thornton and Tom Akers. Each spacewalk will be

performed by two crew members, one of whom will be in a foot restraint mounted

at the end of Endeavour's mechanical arm.  During all EVAs, the crew member

mounted at the end of the arm will be referred to as Extravehicular Crew Member

2, or EV2, while the other spacewalker will be designated EV1.

The EVA crew can be distinguished by markings on the legs of their

spacesuits.  Hoffman will have a solid red stripe around the legs of his suit;

Musgrave will have no stripes on the legs of his suit; Thornton will have a

dashed red stripe around the legs of her suit; and Akers will have a diagonal,

broken red stripe around the legs of his suit.

In planning for the mission, the EVAs have been designed to take into

account the possibility that crew members may encounter unforeseen difficulties

either in tasks or equipment that could cause the preplanned schedule for

installation of various equipment to change.  All four EV crew members have

cross-trained so that any one is capable of performing any given task.

For all of the various tasks, the Flight Support Structure in Endeavour's

cargo bay on which HST will be mounted, once it is retrieved, will be rotated

so the area being worked on faces forward to allow better visibility.  Those

specific tasks and the EVA work required to complete them are described in the

following sections.

REPLACEMENT HARDWARE AND INSTRUMENTS

While the servicing mission is complex, steps have been taken to make the

spacecraft as straightforward to work on as possible.  Since HST was designed

for servicing in space from it's inception, many of its subsystems are modular,

standardized and accessible.  Hubble has 49 different modular subsystems

designed for servicing, ranging from small fuses to large scientific

instruments.  The space telescope, which is 43.5 feet (13.25 meters) long, also

has 225 feet (69 meters) of handrails and 31 footholds to aid astronauts in

servicing tasks.  And more than 200 tools Q from screwdrivers to hardware

designed specifically for HST servicing Q are available for use on this

mission.

NASAUs Goddard Space Flight Center, Greenbelt, Md., is responsible for the

components that will be serviced or replaced on Hubble. The components make up

a primary servicing task list that will be carried out during the mission,

followed by a secondary task list to be undertaken if time and conditions

allow.

The missionUs primary objective is to restore the HSTUs science

capabilities with the Wide Field/Planetary Camera-II and the Corrective Optics

Space Telescope Axial Replacement, both of which will compensate for the

spherical aberration of the primary mirror.

However, the replacement of the spacecraftUs solar arrays Q HSTUs major

source of electrical power Q tops the primary servicing task list.  This is

because solar array jitter, or excessive flexing, may be compromising the

structural integrity of the arrays.  By replacing the arrays first, the

observatory still will be able to perform science even if an emergency causes

the mission to be called off and forces astronauts to release the telescope

from the Space Shuttle before installing the optics packages

Likewise the replacement of one gyro pair is second on the task list,

because if more gyros fail, the pointing of the spacecraft at science targets

cannot be accurately controlled.

The primary servicing task list includes:

%  Solar Array II (SA II).

%  Gyro Pair 2.

%  Wide Field Planetary Camera 2 (WFPC2) and four instrument fuse plugs.

%  Corrective Optics Space Telescope Axial Replacement (COSTAR)

%  Magnetometer System 1.

%  Gyro Pair 3 with Electronics Control Unit (ECU).

%  Solar Array Drive Electronics 1 (SADE).

The secondary servicing task list includes:

%  A redundancy kit for the Goddard High Resolution Spectrograph (GHRS).

%  The 386 co-processor on the spacecraftUs primary computer, called the

DF-224.

%  Magnetometer System  2

%  Four gyro fuse plugs.

%  Electronics Control Unit for Gyro Pair 1.

PRIMARY SERVICING TASKS

Gyroscope Pairs (Rate Sensing Units) and Electronics Control Unit

Three gyroscopes (or gyros) are required to point and track HST. Three

more gyros are onboard as backups.  The total of six gyros are packaged in

pairs of two, called Rate Sensing Units (RSU).  One gyro failed in December

1990; a second failed in June 1991 and a third in November 1992.

Two of these three gyros, one located in pair 2 and the other in pair 3,

contain hybrid electronics that are suspected of causing the failures.  Gyro

pairs 1 and 3 also have experienced a failure in one channel of their

Electronics Control Unit (ECU), the cause of which is suspected to be a random

electronic part failure.  While these failures have not affected HSTUs

performance, replacing the failed hardware will increase system reliability.

The Rate Sensor Units are inside the housing of HST. To begin the

replacement work, the spacewalking astronaut, standing in a foot restraint

mounted on the end of the Shuttle's mechanical arm, will first back out several

bolts to open doors covering the star tracker near the base of HST. One of the

four bolts holding the doors, called the star tracker seal, must be completely

removed to unlatch the doors.

A programmable power wrench will be used to loosen and tighten all bolts

during the RSU replacement.  While the arm-mounted astronaut unlatches the

doors, his fellow spacewalker, mounted on a foot restraint attached to a

support structure in the Shuttle's bay, will assist.  Once the doors are

unlatched, they will be swung open to provide access to the bay area.

The Rate Sensor Units are located behind the cone-shaped star tracker

shades in the bay area of the telescope.  To replace an RSU, these shades may

have to be removed.  If so, three bolts must be loosened and the shade pulled

off.  The shade then can be temporarily stowed outside the work area.  The

shades are reinstalled by pushing them back into place and then retightening

the three bolts.

To remove an RSU, the spacewalker, standing in an adjustable foot

restraint attached to the telescope, will loosen three bolts and disconnect two

electrical plugs.  The RSU then may be removed by holding a handrail located at

the top of the unit.

During these activities, the astronaut standing on the end of the arm will

assist his partner from behind.  The new RSU, carried aloft in an Orbital

Replacement Unit Carrier (ORUC) in the Shuttle's bay, will be installed by

sliding it into place and then retightening the three bolts and hooking up the

two electrical plugs.

The time required to set up, remove two RSUs, install two RSUs and then

clean up the area during training has been around 3.5 hours, including the

possible removal and reinstallation of the star tracker shades.

Electronic Control Units

The Electronic Control Units are the electrical brains for the Rate

Sensor Units and are located in a service bay on the Hubble Space Telescope.

Once the compartment door is opened, two of the three ECUs will be replaced by

removing four bolts and disconnecting each one's electrical cable.  The new

units, which are retrieved from a protective container in the payload bay, will

be installed in a similar fashion.

Solar Array II

The Solar Arrays were built in Europe under the auspices of ESA. The solar

arrays provide the telescope and its instruments with 5kW of electrical power

at the start of their lifetime.  They constitute the spacecraft's two "wings"

and consist of 50,000 silicon photoelectrical cells, covering a surface area of

84 square yards/6.6 x 39.6 x 9.24 ft (70 square meters/2 x 12 x 2.8 m).

The arrays are the flexible "roll-out and roll-up" type and are made of

huge sheets of plastic (fiberglass-reinforced Teflon) held in place by

horizontal metal struts, which also unroll.  Each wing weighs about 352 pds

(160 kg).

But the arrays create a jitter that interferes with spacecraft stability

and affects its pointing capability.  The arrays now on Hubble were supposed to

accommodate the expansion and contraction caused by heating and cooling as the

space telescope moves in and out of daylight during its 96-minute orbits.

However, a compensation device that allows for the expansion and contraction of

the solar array blankets does not expand and contract as smoothly as expected.

As a temporary fix, engineers created software that commanded Hubble's

pointing system to compensate for the jitter automatically.  This procedure

occupies a large portion of the on-board computer memory, though, and to truly

fix the problem, the solar arrays must be replaced.

ESA redesigned and provided to NASA a set of spare solar arrays to reduce

the jitter to an acceptable level.  This set will be installed on the HST after

the existing arrays are removed; the arrays now on the spacecraft will be

returned to Earth. To significantly reduce jitter, the new arrays have thermal

insulation sleeves on the array supports, called bi-stems, to minimize heating

and cooling of the support during each orbit.  Springs that work like shock

absorbers also will take up tension at the array ends.

To replace the HST Solar Arrays, first the old arrays are retracted by

commands from the Space Telescope Operations Control Center. Once they have

been retracted and stowed, the arm-mounted spacewalker will release three

latching points on the first array to be removed.  Once the three latches have

been released, the array can be removed and handled using a transfer handhold

mounted to the array.

The array then is carried by having the mechanical arm moved to position

the astronaut within reach of a temporary stowage bracket for the old arrays,

mounted on the right-hand side of the Solar Array Carrier in Endeavour's cargo

bay.  While being moved from place to place at the end of the arm, the crew

member also may evaluate the handling characteristics of the array to prepare

for carrying the new array up to its installation position.  Thoughout the

removal operation, the arm-mounted astronaut will be assisted by his

counterpart, who will be moving about via handholds on the telescope and

Shuttle.

The new solar array is removed from the SAC by disconnecting a power and a

data connection and then unlatching three latch points exactly like the latch

points on the telescope.  A temporary transfer handle allows the arm-mounted

astronaut to carry the new array up to the installation location.  His

counterpart will assist with the installation by standing in a foot restraint

mounted to the telescope near the work area.

While the array is being transported, the power and data connectors are

secured in temporary holding brackets on the array.  To install the new array,

first it is moved into position and once seated, the three latch points are

locked in place and the connectors plugged in.  The second array is removed and

its replacement installed in exactly the same fashion.

During training, the time required to perform a changeout of the solar

arrays was about 5 hours, and one full spacewalk is dedicated to this task.

The new arrays are not planned to be deployed during the spacewalk performed to

install them.

Wide Field/Planetary Camera-II (WF/PC-II)

The current WF/PC has been used to study bright, high-contrast objects,

such as major solar system planets and nearby star clusters and galaxies.

Spherical aberration, however, has hampered the ability of the camera to

provide high-resolution images of the very faintest objects, or objects in a

field crowded with other objects.

The WF/PC-II is a spare instrument developed, beginning in 1985, by the

Jet Propulsion Laboratory (JPL) team, Pasadena, Calif., that built the first

WFPC. When HubbleUs mirror was found to be flawed, NASA and the WFPC science

team immediately began working on an optical correction that could be built

into WFPC2. The new design incorporates an optical correction by the refiguring

of relay mirrors already in the optical train of the cameras.  Each relay

mirror is polished to a new RprescriptionS that will compensate for the

incorrect figure on HSTUs primary mirror.  Small actuators will fine- tune the

positioning of these mirrors on orbit, ensuring the very precise alignment that

is required.

Through a servicing bay door built into the side of HST, astronauts will

slide out the 610-pound (277-kilogram), wedge-shaped WFPC, as they would a

giant drawer, and replace it with WFPC2. The removed WFPC will be returned to

Earth.

The WFPC2 will have three wide-field cameras and one planetary camera

instead of the original total of eight.  The WFPC2 team chose to reduce the

number of cameras to four in order to develop a system to align the corrective

relay mirrors on-orbit.  Improved Charged Coupled Devices (CCDs) are

incorporated into WFPC2 to improve its sensitivity, particularly in the

ultraviolet.

To remove and replace the Wide Field/Planetary Camera (WF/PC), the doors

to the service bay at the base of the telescope are opened and specially

designed guide rails are installed to assist with removal of the instrument.

A temporary handhold then is installed on the WF/PC. Using this handhold,

the arm-mounted spacewalker pulls the WF/PC out of its installed position while

his counterpart watches the alignment of the WF/PC on the rails and ensures it

is level as it is removed.  Once removed from the telescope cavity, the

arm-mounted astronaut is positioned within reach of a temporary parking fixture

for the old WF/PC in Endeavour's cargo bay, where it is stowed.

A temporary handhold is installed on the new unit, WF/PC-II, in its

Scientific Instrument Protective Enclosure (SIPE) in Endeavour's cargo bay.

The WF/PC-II is pulled from the SIPE by the arm-mounted crew member, who is

later moved to the installation site by the arm operator.  Meanwhile, the SIPE

door is temporarily latched by his crew mate.

Before sliding the WF/PC-II into place inside the telescope, a cover over

its mirror is removed.  Then, the arm-mounted spacewalker slides it into the

telescope slot while his fellow spacewalker checks to ensure that it is aligned

on the guide rails.  Once inserted in place, the handhold is removed from the

instrument and the guide rails are detached.

Finally, the old WF/PC is removed from its temporarily stowed postion in

the parking fixture and inserted into the SIPE that carried WF/PC-II, where it

is secured for Endeavour's trip home.

Removal of the old WF/PC and the installation of WF/PC-II has taken about

4 hours and 15 minutes during training.

Coorective Optics Space Telescope Axial Replacement (COSTAR)

COSTAR is designed to optically correct the effects of the primary mirror

aberration on three instruments besides the WFPC: the Faint Object Camera

(FOC), the Faint Object Spectrograph (FOS) and the Goddard High Resolution

Spectrograph (GHRS).

The FOC, provided by ESA, is designed to detect very low-luminosity

celestial bodies and to provide the most detailed images on HST. It consists of

an electronic conventional scanning camera (of the television type), whose

front part is a powerful image intensifier tube.  Its performance has been

degraded by the spherical aberration, but the sharp image cores still allow the

camera to detect details not seen by ground-based telescopes.

The FOS analyses the light from very faint objects in the visible and

ultraviolet spectral regions.  While the faintest objects now cannot be

reached, observations of brighter sources are only moderately degraded.

The GHRS is intended for very detailed analysis of ultraviolet radiation.

The instrument now loses spectral resolution on the faintest objects, but

observations of brighter sources are only moderately degraded.

COSTAR was invented by the Hubble Space Telescope Strategy Panel, a group

of scientists and engineers brought together at the Space Telescope Science

Institute, Baltimore, Md., in the fall of 1990 to consider how to fix HST.

Built by Ball Aerospace, Boulder, Colo., under contract to NASA, COSTAR has no

detectors or cameras.  It will use precisely shaped mirrors, ranging from about

the size of a dime to a quarter, to correct for the spherical aberration.

Through a servicing bay door, astronauts will pull out the 487-pound

(221-kilogram), phone booth-size High Speed Photometer (HSP) and install in its

place the identically sized COSTAR. Once in place, COSTAR will deploy a set of

mechanical arms, no longer than a human hand, that will place corrective

mirrors in front of the openings that admit light into the Faint Object Camera,

the Faint Object Spectrograph and the Goddard High Resolution Spectrograph.

COSTARUs corrective mirrors will refocus light relayed by HSTUs primary mirror

before it enters these three instruments.  COSTAR will restore the optical

performance of these instruments very close to the original expectations.

To install COSTAR, the spacewalkers first will open doors to the bay, that

enclosed the HSP, by loosening several bolts.  Once the doors are open, latches

that hold the HSP in place will be loosened and then four electrical connectors

and a ground strap will be disconnected from the instrument.

Then the HSP is lowered from its position to guide rails for the unit.

The arm-mounted spacewalker and his crew mate, standing in a foot restraint

attached to the telescope, will coordinate efforts to remove the device.  The

arm-mounted crew member will slide HSP out while his fellow spacewalker ensures

that it is aligned with the guide rails.

Once removed, the HSP is held by the crew member standing on the end of

the arm while the arm is positioned so the HSP can be placed in a temporary

parking fixture mounted in the cargo bay.  After it is temporarily stowed, a

handhold is attached to COSTAR and it is lifted from its protective enclosure.

A ground strap is disconnected and, while the arm-mounted astronaut is lifting

COSTAR out, his crew mate is assisting by ensuring COSTAR is squarely aligned

with the enclosure as it is extracted.

The arm-mounted crew member then is positioned up to the installation area

while his fellow spacewalker moves to the site via handrails on the telescope.

COSTAR then is aligned with the guide rails, with the arm- mounted spacewalker

watching the alignment of a rail at the upper left-hand corner of COSTAR and

his counterpart checking the alignment of a rail at the lower right corner.

Once COSTAR slides into place, the four electrical connections,

disconnected from HSP, are connected to COSTAR along with the grounding strap,

and the latches are tightened to hold COSTAR in place.

In training, removal of the HSP and installation of COSTAR has taken about

3 hours and 15 minutes.

Solar Array Drive Electronics 1 (SADE)

Each solar array wing has an electronics control assembly that includes a

drive electronics unit.  These units transmit positioning commands to the wing

assembly.  One of these Solar Array Drive Electronics units has failed due to

transistor overheating.  A replacement SADE, provided by ESA, will restore that

lost capability and provide better heat protection for the transistors.

The Solar Array Drive Electronics are mounted on the inner side of one of

the doors to an HST service bay.  Two electronics boxes are mounted on the

inside of the door, but only one is being replaced.  Once the door is opened,

the two spacewalkers - one mounted on the arm and one holding handrails on the

telescope - will loosen six bolts to free the old SADE unit and disconnect

electrical connectors attached to the unit.  The new SADE unit is installed in

the reverse of this process.

Magnetometer System 1

The HSTUs two magnetometers (also known as magnetic sensing systems)

measure the spacecraftUs relative orientation with respect to the EarthUs

magnetic field.  Neither magnetometer is functioning at full capability.  Both

replacements have improved electronics and thermal blankets added.  The

replacement magnetometers will be snapped into place over the existing

magnetometers, which will not be removed from the HST.

Both of the magnetic sensing systems are located near the top of the

telescope near the aperture door.  The new units will be installed using four

rotating knob connectors and will be attached directly on top of the old units

by removing some insulation and removing and reinstalling the electrical cable.

These units are used to help measure the observatory's position relative to

Earth's magnetic field.

Fuse Plugs

Fuses for both the gyros and instruments will be replaced to correct

sizing and wiring discrepancies.

The fuses that will be replaced on the HST are located on the inside of a

compartment door.  Eight of the fuse plugs will be removed and replaced by the

spacewalking astronauts.  Once all have been replaced, checks will be made to

ensure they are working properly.

SECONDARY SERVICING TASKS

Co-Processor

The DF-224 is the HSTUs flight systems computer.  One of the computerUs

six memory units has failed and another has partially failed.  Hubble requires

only three memory units to fully function, so the failures have not affected

telescope operations.  However, to restore the memory redundancy and augment

the telescopeUs memory capacity and speed, a co- processor, based on

386-computer architecture, will be integrated into the flight systems computer,

which will increase both flight computer memory and the speed of some

operations.

The DF-224 co-processor will be installed on top of the HST computer

located in a compartment on the telescope.  The memory upgrade is installed

using handles on the computer and attaching four bolts using a power tool.

Goddard High Resolution Spectrograph Redundancy Kit

The GHRS has two detector systems.  Because of the anomalous behavior of

a low-voltage power supply, the side-one detector no longer is used.  The

redundancy kit consists of an externally mounted relay box that enhances system

redundancy so that the side-one detector can be used and the side- two

detectors will not be compromised if the anomaly recurs.

Made up of four cables and a relay box, the Goddard High Resolution

Spectrograph redundancy kit is designed to bypass an erratic detector system on

the science instrument located in an instrument bay on the lower portion of the

HST. The relay box is installed first using a power tool similar to an electric

drill.  This is followed by attachment of the four cables.

HUBBLE SPACE TELESCOPE TOOLS AND CREW AIDS

The crew of STS-61 has more than 200 tools and crew aids with them for the

servicing of the Hubble Space Telescope. The tools and crew aids, known as

Space Support Equipment (SSE) hardware, range from a simple bag for carrying

some of the smaller tools to sophisticated, battery operated power equipment.

These tools will be used by the EVA crew members servicing the spacecraft.

Crew Aids

Crew aids are defined as those that are fixed in place and those that are

portable equipment items but not hand tools, used to assist crew members in

accomplishing servicing mission tasks.  SSE equipment crew aids permit the crew

members to maneuver safely or to restrain themselves, transfer Orbital

Replacement Units (ORUs) and other portable items, protect equipment and

themselves during changeout activities and temporarily stow or tether equipment

during EVAs.

Examples of crew aids are handrails, handholds, translation devices,

transfer equipment, protective covers, tethering devices, grapple fixtures,

foot restraint sockets and stowage and parking fixtures.

Tools

Tools are hand-operated or manipulated devices that allow the EVA

astronauts to increase the efficiency of performing intricate, labor-intensive

tasks.

Stowage

The tools and crew aids will be stowed on or in the Solar Array Carrier

(SAC), Orbital Replacement Unit Carrier (ORUC), Flight Support System (FSS),

HST Tool Box, Sidewall-mounted adapter plates, Provisions Stowage Assembly

(PSA), an Adaptive Payload Carrier (APC), middeck lockers, aft flight-deck and

airlock.  Tools and crew aids are provided by Johnson Space Center, Houston,

and Goddard Space Flight Center, Greenbelt, Md.

Uses

Tools and crew aids considered Rgeneral,S with a wide variety of uses,

include the Power Ratchet Tool (PRT), Multi-setting Torque Limiter (MTL),

adjustable extension with 7/16th-inch sockets, ingress aids, portable work-

light receptacle and a locking connector tool.  More specific, but still

considered general, items are a low-gain antenna (LGA) cover, umbilical

connector covers, a flight support system (FSS), berthing and positioning

system (BAPS), support post and a multi-layer insulation (MLI) repair kit.

To be used on the changeout of the Wide Field Planetary Camera 2 (WFPC2)

are the WFPC handholds, WFPC guide studs, quick-release zip nuts, WFPC pick-off

mirror cover, forward fixture, aft fixture and the HST radial bay cover.

For the High Speed Photometer (HSP) replacement with the Corrective Optics

Space Telescope Axial Replacement (COSTAR), tools and aids to be used will be

the COSTAR contamination cover, a COSTAR handling aid, an HSP handling aid,

forward fixture, aft fixture and an axial Science Instrument Protective

Enclosure (SIPE) safety bar.

For the solar array replacement, the astronauts will use articulating foot

restraints, solar array primary drive mechanism handles, solar array temporary

stowage brackets (TSBs), solar array transfer handles, solar array jettison

handle, solar array spines, Portable Flight Release Grapple Fixture (PFRGF) and

a Marmon clamp.

For the changeout of the gyro rate sensor units, crew members will use a

Portable Foot Restraint (PFR) socket converter (90-degree), Fixed-Head Star

Tracker (FHST) light shade covers and a FHST delta plate cover.

Portable Foot Restraint

There are two Hubble Space Telescope portable foot restraints built for

use on the STS-61 mission.  These restraints are used by the spacewalking

astronauts during the five extravehicular activities to provide a stable

platform from which to work.  Both restraints are stowed in the payload bay,

one on the left side and the other on the flight support system.

Tool Box

The HST tool box is designed to stow individual tools, tool boards and tool

caddies that will be used throughout the mission.  The box is mounted on the

right side of the payload bay.  Each tool inside the box is stowed in a

specific location with markings to assist the astronauts in the retrieval and

stowage.

Power Ratchet Tool (PRT)

The Goddard-provided power ratchet tool (PRT) is powered by a 28-volt

battery.  Made of titanium and aluminum, the 17-inch (43-centimeter) tool will

be used for tasks requiring controlled torque, speed or turns and can be used

where right-angle access is required.  It will provide 25 foot-pounds of

pressure in the motorized mode and 75 foot-pounds of pressure in the manual

mode.  It has a speed of 10 to 30 revolutions per minute.  A spare power

ratchet will be carried on the mission, as will spares for all other tools to

be used by the astronauts.

HST Power Tool

This tool is a modified, battery-operated power tool with torque and rpm

control.  The design includes a 3/8-inch drive fitting, forward and reverse

drive rotation, torque ranges from 50 to 300 inch-pounds and a bracket for

mounting the tool to the spacesuit.

Mini-Power Tool

The mini-power tool is a battery-operated screwdriver intended for use

when a larger power tool is not required and when work space is limited.  It

can be used as a power tool or with the power off, the output shaft is locked

automatically for use as a manual driver.

Multisetting Torque Limiter

This tool is provided to prevent damage to hardware due to the application

of torque which may exceed the design limits.  Multisetting torque limiters are

used in conjunction with the power tools or hand tools that interface with

bolts and latches on the telescope.

Adjustable Extensions

Several extensions were designed to be adjustable to ease the movement of

the astronauts while reducing the time required for tool changeouts.  The

adjustable extensions replace several fixed length extensions by providing

adjustment from 12 to 16.5 inches.  Another adjustable extension provides

lengths from 15 to 24 inches.  When retracted, these extensions reduce the

potential for damage to other hardware.

IMAX

THE IMAX project is a collaboration between NASA and the Smithsonian

Institution's National Air and Space Museum to document significant space

activities using the IMAX film medium.  This system, developed by IMAX Systems,

Corp., Toronto, Canada, uses specially designed 70mm film cameras and

projectors to record and display very high definition large- screen pictures.

An IMAX camera system will be flown on Shuttle Mission STS-61 and will be

used by Endeavour's crew to collect material for upcoming IMAX productions.

IMAX cameras have flown on several Shuttle missions and film from those

missions was used to form the IMAX productions The Dream is Alive and The Blue

Planet.

In-Cabin IMAX Camera Equipment

The IMAX system consists of a camera, lenses, rolls of film, two magazines

with film, an emergency speed control, a Sony recorder and associated

equipment, two photographic lights, supporting hardware in the form of mounting

brackets to accommodate the mode of use, two cables and various supplemental

equipment.

The IMAX and supporting equipment are stowed in the middeck for in- cabin

use.  The IMAX uses two film magazines which can be interchanged as part of the

operation.  Each magazine runs for approximately 3 minutes.  When both

magazines are consumed, reloading of the magazines from the stowed supply of

film is required.  Lenses are interchanged based on scene requirements.  The

IMAX will be installed in the orbiter middeck approximately 7 days prior to

launch.

IMAX Cargo Bay Camera (ICBC)

During Shuttle Mission STS-61, an IMAX Cargo Bay Camera (ICBC) will be

carried in the payload bay of Endeavour and used to document activities

associated with the servicing of the Hubble Space Telescope. The camera is

mounted in a pressure sealed container with a viewing window.  The window has a

sliding door which opens when the camera is in operation.  The camera is

controlled from the aft-flight deck, exposing the film through a 30mm fisheye

lens.

STS-61 IN-CABIN PAYLOADS

AIR FORCE MAUI OPTICAL SYSTEM

The Air Force Maui Optical System (AMOS) is an electrical-optical facility

on the Hawaiian island of Maui. No hardware is required aboard Discovery to

support the experimental observations.  The AMOS facility tracks the orbiter as

it flies over the area and records signatures from thruster firings, water

dumps or the phenomena of "Shuttle glow," a well-documented fluorescent effect

created as the Shuttle interacts with atomic oxygen in Earth orbit.  The

information obtained by AMOS is used to calibrate the infrared and optical

sensors at the facility.

DTO-667: PILOT INFLIGHT LANDING OPERATIONS TRAINER (PILOT)

One of the challenges to flying long duration Shuttle missions is the

issue of orbiter landing tasks requiring a high level of skill and proficiency

yet data showing that a pilot's landing skills degrade after an extended

absence from a landing trainer such as the Shuttle Training Aircraft. During

Shuttle Mission STS-61, a portable scientific workstation designed to aid the

Shuttle commander and pilot in maintaining those landing skills will be

demonstrated for the second time.

The PILOT system hardware, which flew on Shuttle Mission STS-58 in October

1993, consists of a portable scientific workstation, a high resolution color

display and a hand controller with orbiter look and feel.  The software used in

the system was transferred from the Shuttle Engineering Simulator software

which is used to validate Shuttle flight software.  This provides PILOT with

orbiter handling and guidance characteristics.

The PILOT system is stowed in lockers on the flight deck and middeck areas

of the Space Shuttle. When a member of the crew wants to use the system, the

workstation is mounted on a counsole directly in front of the pilot's seat on

the flight deck and the PILOT system hand controller is attached to the

orbiter's hand controller.

In addition to evaluating the ability to maintain landing skills of a

Shuttle crew in Earth-orbit, the PILOT system may be integrated into the

standard training activities of all Shuttle crews at the Johnson Space Center

in Houston.

STS-61 CREW BIOGRAPHIES

Richard O. Covey, 47, Col., USAF, is Commander (CDR) of STS-61. Selected

as an astronaut in January 1978, Covey considers Fort Walton Beach, Fla., his

hometown and will be making his fourth space flight.

Covey graduated from Choctawhatchee High School, Shalimar, Fla., in 1964;

received a bachelor of science degree in engineering sciences with a major in

astronautical engineering from the U.S. Air Force Academy in 1968, and a master

of science degree in aeronautics and astronautics from Purdue University in

1969.

Covey first flew as Pilot for Shuttle mission STS 51-I in August 1985.  He

next flew as Pilot on STS-26 in September 1988.  On his most recent flight, he

was Commander for STS-38 in November 1990.  Covey has logged over 385 hours in

space.

Kenneth D. Bowersox, 37, Cmdr., USN, will serve as Pilot (PLT).  Selected

as an astronaut in June 1987, Bowersox was born in Portsmouth, Va., but

considers Bedford, Ind., his hometown and will be making his second space

flight.

Bowersox graduated from Bedford High School, Bedford, Ind.; received a

bachelor's degree in aerospace engineering from the Naval Academy in 1978 and a

master of science degree in mechanical engineering from Columbia University in

1979.

Bowersox first flew as Pilot for Shuttle mission STS-50 in June 1992.  He

has logged over 331 hours in space.

Tom Akers, 42, Lt. Col., USAF, will serve as Mission Specialist 5 (MS5)

and as one of the extravehicluar activity crew members.  Selected as an

astronaut in June 1987, Akers was born in St. Louis, Mo., but considers

Eminence, Mo., his hometown and will be making his third space flight.

He graduated from Eminence High School and received bachelor and master of

science degrees in applied mathematics from the University of Missouri-Rolla in

1973 and 1975, respectively.

Akers served as a mission specialist on STS-41 in October 1990.  His next

flight was as a mission specialist on STS-49 in May 1992.  Akers was one of the

EVA crew members of a three-person spacewalking team that successfully captured

the stranded International Telecommunications Satellite. He also performed a

second EVA on STS-49 to evaluate space station construction techniques.  He has

logged over 311 hours of space flight.

Jeffrey A. Hoffman, 49, will be Mission Specialist 3 (MS3) and serve as

one of the extravehicular activity crew members.  Selected as an astronaut in

January 1978, Hoffman considers Scarsdale, N.Y., his hometown and will be

making his fourth space flight.

Hoffman graduated from Scarsdale High School, received a bachelor's degree

in astronomy from Amherst College, received a doctorate in astrophysics from

Harvard University and received a master's degree in materials science from

Rice University.

Hoffman first flew on STS-51D in April 1985, a mission during which he

performed a spacewalk in an attempt to rescue a malfunctioning satellite.  He

next flew on STS-35 in December 1990.  Hoffman made his third space flight as

Payload Commander and mission specialist on STS-46 in July 1992.

F.  Story Musgrave, 58, will be Mission Specialist 4 (MS4).  He also will

serve as Payload Commander and as a member of the extravehicular activity team.

Selected as an astronaut in August 1967, Musgrave considers Lexington, Ky., his

hometown and will be making his fifth space flight.

Musgrave graduated from St. Mark's School, Southborough, Mass., in 1953;

received a bachelor's degree in mathematics and statistics from Syracuse

University in 1958; received a master's degree in operations analysis and

computer programming from the University of California at Los Angeles in 1959;

received a bachelor's degree in chemistry from Marietta College in 1960;

received a doctorate in medicine from Columbia University in 1964; received a

master's degree in physiology and biophysics from the University of Kentucky in

1966 and received a master's degree in literature from the University of

Houston in 1987.

Musgrave flew as a mission specialist on STS-6 in April 1983, on

Spacelab-2 in August 1985 and on STS-33 in November 1989.  More recently, he

served aboard Space Shuttle Atlantis, STS-44 in November 1991.  He has logged

more than 598 hours in space.

Claude Nicollier, 49, will be Mission Specialist 2 (MS2).  Under an

agreement with the European Space Agency and NASA, he was selected as an

astronaut in 1980.  Nicollier was born in Vevey, Switzerland, and will be

making his second space flight.

Nicollier graduated from Gymnase de Lausanne, Lausanne, Switzerland,

received a bachelor's degree in physics from the University of Lausanne and

received a master's degree in astrophysics from the University of Geneva.

Nicollier first flew as a mission specialist on STS-46 in July 1992 and has

logged more than 191 hours in space.

Kathryn C. Thornton, 41, will serve as Mission Specialist 1 (MS1) and as

one of the extravehicular activity crew members.  Selected as an astronaut in

May 1984, Thornton was born in Montgomery, Ala., and will be making her third

space flight.

Thornton received a bachelor of science degree in physics from Auburn

University and a master of science degree and a doctorate in physics from the

University of Virginia. Thornton was awarded a NATO postdoctoral fellowship to

continue her research at the Max Planck Institute of Nuclear Physics in

Heidelberg, Germany.

Thornton first flew as a mission specialist on STS-33 in November 1989.

On her second flight, she served on the crew of STS-49 in May 1992.  On STS-49,

Thornton performed an extravehicular activity to evaluate space station

assembly techniques.  She has logged over 333 hours in space.

STS-61 ACRONYMS AND ABBREVIATIONS

A-h		ampere-hour

ac		alternating current

ACE		actuator control electronics

ACP		astronaut control panel

AD		aperture door

AFD		aft flight deck

APS		antenna pointing system

AS		aft shroud

ATM		Apollo Telescope Mount

BAPS         	berthing and positioning system

BCU		bus coupler unit

BOD		bright object detector

BOL		beginning of life

BOT		beginning of travel

BPRC          	battery protection and reconditioning circuit

bps		bits per second

BPRC           	battery protection and reconditioning circuit

BPSK	        biphase shift keyed

BREC	        body rate error check

C&DH	        communication and data handling

C/C		configuration control

CC		cargo control

CCC		charge current controller

CCTV           	closed-circuit television

CDI		command and data interface

CDU		command detector unit

CFRP           	carbon-fiber-reinforced plastic

CGG		contingency gravity gradient

CIF		computer interface

CMD		command module

COM		communications module

CORU	        candidate orbital replacement unit

CPM		central processor module

CPU		central processing unit

CSM		cargo systems manual

CSS		coarse Sun sensor

CU/SDF	        control unit/science data formatter

D/A		digital to analog

DAK		double-aluminized Kapton

DBA		diode box assembly

dc		direct current

DCE		deployment control electronics

DIH		discrete input high

DISP		display

DIU		data interface unit

DIUI		DIU interface module

DMS		data management subsystem

DMU		data management unit

DPC		direct power converter

ECA		electronics control assembly

ECU		electronics control unit

ED		engineering data

EDB		external data bus

EMI		electromagnetic interference

EOL		end of life

EOT		end of travel

EP/TCE	        electrical power/thermal control electronics

EPS		electrical power subsystem

ES		equipment section

ESA		European Space Agency

ESTR	        engineering/science tape recorders

EU		electronics unit or expander unit

EVA		extravehicular activity

FEP		fluorinated ethylene-propylene

FGE		fine guidance electronics

FGS		fine guidance sensor

FHST	        fixed head star tracker

FHSTI	        FHST interface

FIFO		first in first out

FMDM	        flexible multiplexer/demultiplexer

FOC		faint object camera

FOD		Flight Operations Directorate

FOS		faint object spectrograph

FOV		field-of-view

F/S		filter/shaper

FPA		focal plane assembly

FPDA	        focal plane deck assembly

FPSA		focal plane structure assembly

FS		forward shell

FSS		flight support system

FWH           	flexible wire harness

GEA		gimbal electronics assembly

GG		gravity gradient

GGM		GG mode

GPC		general purpose computer

GSE		ground support equipment

GSFC           	Goddard Space Flight Center

GSTDN         	ground spaceflight tracking and data network

HGA		high-gain antenna

HLD		high-level discrete

HOSC	        Huntsville Operations Support Center

HRS		high resolution spectrograph

HSP		high-speed photometer

HST		Edwin P. Hubble Space Telescope

HWL		hardware load

I&C		instrumentation and communication

ICAPC          	increased capacity adaptive payload carrier

ICU		instrumentation control unit

IDB		internal data bus

IMU		inertial measurement unit

IOU		input/output unit

IPCU		interface power control  unit

IR		infrared

JSC		Lyndon B. Johnson Space Center

KA		keep alive

kbps		kilo bits per second

KSC		Kennedy Space Center

LGA		low-gain antenna

LLD		low-level discrete

LMU		logical memory unit

LS		light shield

LMSC	        Lockheed Missiles and Space Company

MA		multiaccess

Mbps           	megabits per second

MCC		Mission Control Center

MCC-H          	MCC - Houston

MCE		monitor and control electronics

MCU		mechanism control unit

MDB		multiplexed data bus

MET		mission elapsed time

MLD		manual locking device

MLI		multilayer insulation

MMS	        multimission modular spacecraft

MOD		Mission Operations Directorate

MR		main ring

MRA		main ring assembly

MSB		most significant bit

MSFC           	Marshall Space Flight Center

MSS		magnetic sensing system

MT		magnetic torquer

MTE		magnetic torquer electronics

MTP		master timing pulse

MTS		magnetic torquing system

MU		memory unit

N.m		Newton meter

NASA	        National Aeronautics and Space Administration

NASCOM         	NASA communications

NGT		NASCOM ground terminal

nm		nanometer

NSSC-I	        NASA Standard Spacecraft Computer, Model I

OAO		Orbiting Astronomical Society

OCE		optical control subsystem

OCS		optical control subsystem

OCXO	        ovencontrolled crystal oscillator

OLD		offload device

OPI		Orbiter payload interrogator

ORU		orbital replaceable unit

ORUC	        ORU carrier

OSO		Orbiting Solar Observatory

OTA		optical telescope assembly

OV		orbital verification

P-E		Perkin-Elmer Corporation

PC		planetary camera

PCM		pulse-code modulation

PCS		pointing control subsystem

pointing control system

PCU		power control unit

PDI		payload data interleaver

PDM		primary deployment mechanism

PDU		power distribution unit

PI		payload interrogator

PI/KUSP	        PI/Ku-band signal processor

PIT		process interface table

PLBD	        payload bay door

PLR		payload recorder

PM		primary mirror

PMA		primary mirror assembly

PMB		primary mirror baffle

PMT		photomultiplier tube

PMU		physical memory unit

PN		pseudorandom noise

POCC	        Payload Operations Control Center

PRCS	        primary react ion control system

PRI		primary

PRIPL	        unregulated Orbiter power

PRLA           	payload retention latch assembly

PROM           	programmable read-only memory

PSEA	        pointing and safe-mode electronics assembly

psi		pounds per square inch

psia		pounds per square inch absolute

PSP		payload signal processor

PSP/PI	        PSP/payload interrogator

QD		quick disconnect

RBM		radial bay module

RED		redundant

RF		radio frequency

RFI		radio frequency interface

RGA		rate gyro assembly

RGAI		RGA interface

RIG		rate integrating gyro

RIU		remote interface unit

RMGA	        retrieval mode gyro assembly

RM		remote module

RMS		remote manipulator system

ROM		read-only memory

RPA		reaction plate assembly

RSOC	        Rockwell Shuttle Operations Company

RSU		rate sensor unit

RTC		real-time command

RWA		reaction wheel assembly

SA		solar array

SAD		SA drive

SADA	        SA drive adapter

SADE	        SA drive electronics

SADM	        SA drive mechanism

SD		science data or scientific data

SDM		secondary deployment mechanism

SI		scientific instrument

SIC&DH	        scientific instrument control and data handling

SIP		standard interface panel

SIPE		scientific instrument protective enclosure

SISS		SI support structure

S/M		safe mode

SM		secondary mirror

SMA		secondary mirror assembly

SMB		secondary mirror baffle

SMC		safemode computer

SMCH	        standard mixed cargo harness

SMSA           	secondary mirror subassembly

SM4		safe-mode utility

SOC		state-of-charge

SPA		solar panel assembly

SPC		stored program command

SPG		single-point ground

SQPSK         	staggered quadiphased, shift-keyed

SS		safing system

SSA		S-band single-access

SSC		Science Support Center

SSE		space support equipment

SSM		support systems module

SSM-ES	        SSM equipment shelf

SSP		standard switch panel

SSS		star selector servo

STACC          	standard telemetry and command components

STDN          	space flight tracking and data network

STINT	        standard interface

STOCC	        Space Telescope Operations Control Center

STS		Space Transportation System

STSCI	        Space Telescope Science Institute

TAG		two-axis gimbal

TBD		to  be determined

TCE		thermal control electronics

TCS		thermal control subsystem

TCXO           	temperature controlled crystal oscillator

TDRS           	tracking and data relay satellite

TDRSS        	TDRS system

TFC		TLM format control

TIM		timing interface module

TLM		telemetry

TRI		tape recorder interface

TTL		transistor-to-transistor logic

UDM		umbilical disconnect mechanism

ULE		ultralow expansion

URM		umbilical retraction mechanism

UV		ultraviolet

VATA           	Vehicle Assembly and Test Area

VCO		voltage controlled oscillator

VSS		vehicle support software

VSWR	        voltage standing wave radio

WF/PC	        wide field/planetary camera

WFC		wide field camera

WSGT	        White Sands Ground Terminal

- end -