THIS PACKAGE IS A USEFUL ANTENNA ANALYSIS PROGRAM FOR THE RADIO

AMATEUR, PROFESSIONAL, TECHNICIAN AND STUDENT INTERESTED IN ANTENNA DESIGN.

IT IS STILL UNDER DEVELOPMENT AND YOUR COMMENTS WILL BE USEFUL

TO THE AUTHORS. TO USE THESE PROGRAMS YOU MUST:

1. Add GWBASIC to the disk.

2. You should compile the programs you wish to use or else they are too

slow although still useful.

3. The manual describes a menu system but this feature is not included

in this package. Programs can be loaded separately an run.

The manual is lacking in a few line diagrams that are included

with the original printed and bound version.

Any comments can be addressed to:

A. J. Ferraro

RM 225 EEE

University Park, Pa.

16802

INTRODUCTION

Concepts and ideas can be more easily understood when one

can feel and visualize what is being discussed.  This is ever

more so in the teaching of electromagnetic theory where there is

so much abstraction.  The instructor has the arduous task of

formulating his ideas and presenting it to the students who have

no other alternative but to try to understand as much as they

can.  An application of the electromagnetic theory is antenna

engineering.  This is an area where many discussions are centered

around the "invisible".  Describing an antenna pattern is not an

easy job and when there are many cases to study, drawing many

patterns is no fun either.  This is where this courseware

package comes in.

This package was designed in the Electrical Engineering

Department at Pennsylvania State University.  It is aimed at

both the students and faculty.  To the undergraduate students,

the approach to this package is to treat it as an antenna

engineering laboratory.  Instead of hooking up wires and

equipment, students can carry out experiments on the computer.

This saves time and effort since changing several parameters can

be done quickly and results are immediately displayed.  Graduate

students who are preparing for the candidacy examinations can

use it as a review or to get some information in a limited time. The

faculty can use it to try new ideas and also to formulate exam

questions.

Basically, this courseware covers courses EE361, EE411,

EE432, EE438 and EE538 offered by the Electrical Engineering

Department at Penn State.  Presently, only topics which are

related to antenna engineering are covered.  The package can be

divided roughly into six areas.  These are Transmission line

problems, wire antennas, arrays, broadband antenna, aperture

antenna and antenna synthesis.

Transmission line problems can be used by students taking

courses EE361, EE432 and EE438.  It involves simple matching

methods.  Wire antennas includes the straight wire dipole, the

folded dipole and the Yagi-Uda antenna.  This module can be used

to study the radiation patterns, self impedance of a single

element and the mutual impedance between two elements.  This

module is particularly relevant to students in EE361 and EE438.

The third module is Arrays.  Mutual impedance between

several elements are studied.  Array characteristics such as the

driving point impedance, efficiency and directivity can be found

for various linear array configuration.  Feed line designs for

the array are also included.  The radiation pattern of

rectangular and circular planar arrays can be computed for

different array configuration.  Students taking EE432, EE438 and EE

538 should find this module useful.

The next module is Broadband antennas.  The design of the

Log periodic dipole array is made easy using this module.  Other

topics include linear, rectangular and circular aperture, the

parabolic antenna and the horn antenna.  These antenna are

discussed in EE438 and EE538.

The study of antenna engineering is incomplete without

knowing antenna synthesis.  In normal antenna design, the

radiation pattern and array factor are calculated after the

antenna configuration is established.  However, in antenna

synthesis, the desired pattern are first specified.  This is then

followed by a procedure to find the best antenna configuration

that meets the pattern requirement.  The classes of synthesis

method which are covered are the Woodward-Lawson line source and

array factor, the Chebychev synthesis and the Taylor line source

method.  This last module is fully covered in the graduate course

EE538.

03

ANTENNA ENGlNEERlNG COURSEWARE

by

AHMAD FAlZAL MOHD. ZAlN and DR. ANTHONY J. FERRARO

ELECTRICAL ENGlNEERlNG DEPARTMENT

PENNSYLVANlA STATE UNIVERSITY

The authors have made every conceivable effort to ensure the correctness and

accuracy of the programs.  However, no implied warranty of guarantee of any

kind

is made with respect to the accuracy, completeness and effectiveness of

these

programs.

Under no circumstances will the authors be liable for incidental or

otherwise

consequential damages in connection with the furnishing of, or the

performance

of, or as a result of the use of any of these programs.

USING THE COURSEWARE

1.  Switch on the computer.

2.  You will be presented with an option to type in the date and

time.  Press return twice if you do not wish to do so.

3.  A disclaimer notice will then be displayed.  Please read it

carefully.

4.  After striking any key, a menu screen would the come up.

This is similar to the map in Fig. 1.

5.  Select your choice by pressing the appropriate function

keys labelled Fl to F10 on the left side of the keyboard.

6.  Pressing F10 will return you to the previous menu.

GETTING HARDCOPIES

If you would like to get a hardcopy of the plots or the screen,

just press the keys <SHIFT> and <PrtSc> together.  Make sure the

graphics printer is turned on!

SELF IMPEDANCE OF A DIPOLE

The self impedance of a dipole antenna depends on its

length and radius of wire used.  This program will calculate the

self impedance of a straight wire dipole given its length and

radius using the approximate induced emf method (refer to Jordan

and Balmain).  The radiation resistance is referred to the loop

current.  The resistance referred to the base is R(loop)/sin H.

Thus for antenna length which are a multiple of half-wavelength, the

input resistance, R(base), would theoretically be infinite.  Thus we

need to use a different method.

For those cases, you are referred to a program called

MININEC in the book 'MICROCOMPUTER TOOLS FOR COMMUNICATIONS

ENGINEERING  by Shing Ted Li et al.  It is published by Artech House.

The program uses the moment method to compute the current

distributions, impedance and patterns of any antenna composed of

wires arbitrary oriented in free space or over perfect ground.

PROGRAM NAME : SELFIMP

VARIABLE LIST:

H       : Length of dipole in terms of lambda

A       : Radius of dipole in terms of lambda

RIN     : Real part of self-impedance

XIN     : Imaginary part of self-impedance

PROCEDURE:

Choose F2 (WIRE ANTENNAS) from main menu.  Then choose Fl

(DIPOLE (STRAIGHT WIRE)).  Finally choose F2 (Self-impedance of

dipole).

EXPERlMENT

1.  For a half wavelength dipole with radius 0.0035 lambda, find

the self impedance.

2.  Try the same experiment with a smaller radius, say .00014

lambda.

3.  For both the antennas above, what is the resonant length of

each?  What can you conclude?  Which antenna is better for

broadband use, a thicker or a thinner one?

4.  What is the self impedance of a monopole of half the height

of the dipole?

MUTUAL IMPEDANCE BETWEEN TWO DlPOLES

OF ANY LENGTH AND SEPARATION

The mutual impedance of two dipoles changes with distance

of separation between them.  This program allows one to change

both the distance of separation and length of two identical

dipoles and computes the mutual impedance.  This is useful for

finding the driving point impedance of a linear array of

dipoles.

PROGRAM NAME : MUTUAL

VARIABLE LIST:

H       : Length of dipole in terms of lambda

D       : Separation between the dipoles in terms of lambda

R21     : Real part of mutual impedance

X21     : Imaginary part of mutual dipole

PROCEDURE:

Choose F2 (WIRE ANTENNAS) from main menu.  Then choose Fl

(DIPOLE STRAIGHT WIRE)).  Finally choose F3 (Mutual-impedance of

2 dipole).

EXPERIMENT

1.  Find the mutual impedance between two half wavelength

dipoles separated by .25, .5 and 1 lambda.

2.  What happens if the separation is 0 lambda?

3.  Plot a graph of the mutual resistance and reactance with

respect to separation for dipoles of half wavelength in a side

by side configuration.  Do the same for a collinear

configuration.  Which one gave more reactance?  Why?

4.  For a side by side configuration, plot the mutual resistance

and impedance with respect to separation for dipoles of length

0.4, 0.45 and 0.55 lambda.  What happens to the mutual impedances

as the length is increased?

DRIVING POINT IMPEDANCE

This program will compute the driving point impedance of an

array of parallel side-by-side evenly spaced half-wavelength

dipoles.  It will also calculate the power radiated, directivity

and efficiency of the array.  Feedline design using quarter

wavelength line is also included.

PROGRAM NAME : DRIVPT

VARIABLE LIST:

N         : Number of elements

LAMBDA    : Element spacing

I(J)      : Excitation currents

A(J)      : Phase of excitation currents

ZDR(ROW)  : Real part of driving point impedance

ZDX(ROW)  : Imaginary part of driving point impedance

WATT(ROW) : Power radiated by each element

WATTRAD   : Total power radiated by array

DIRECTDB  : Directivity in dB

Z0        : Characteristic impedance of main line

Z0(I)     : Characteristic impedance of feed-lines

RIN(I)    : Input impedance of feed-lines

FREQ      : Working frequency

ECD       : Efficiency of array

PROCEDURE :

Choose F3 (ARRAYS) from main menu.  Then choose Fl (LINEAR

ARRAY).  Finally choose Fl (Driving-point impedance).

EXPERIMENT

1.  Find the driving point impedance of a 2-element array of

parallel side-by-side lambda-by-two dipoles spaced half

wavelength apart and fed with currents of equal amplitude and

zero phase shift between them.

2.  Try with a phase shift of 90 and 180 degrees.  Which has

better directivity?

3.  Change the second current to twice the first with zero phase

shift.  What happens?

4.  Design a 3-element array of three half-wavelength dipoles

separated .65 lambda apart.  The currents are I1=1, I2=2 and

I3=1.  All have zero phase shift.  The transmitter is rated at

10 kW at 500 MHz and is to be matched to a 75 ohm coaxial main

feed line.

NOTE:  The feedlines of quarter wavelength long are all

connected at one point.  The quarter wavelength lines can be

extended by half wavelength lines if they are too short due to

physical constraint.

RADIATION PATTERN PLOT OF DIPOLE ANTENNA

This program will plot the radiation pattern of a dipole of

any length.

PROGRAM NAME : DIPOLE

VARIABLE LIST:

L         : Length of dipole in lambda

PROCEDURE:

Choose F2 (WIRE ANTENNAS) from main menu.  Then choose Fl

(DIPOLE (STRAIGHT WlRE)).  Finally choose Fl (Pattern of any

length).

EXPERIMENT

1.  For a half-wavelength dipole, plot the radiation pattern.

What is the HPBW and directivity?

2.  Repeat 1) with dipoles of wavelengths 0.25, 0.75, 1, 1.5 and

2 lambda.  When do you get multiple lobes?

NOTE:  The dipole is oriented vertically in the polar plot.

TRANSMISSION LINE

This program calculates the input impedance of a lossless

transmission line, given the characteristic impedance, the load

impedance and its length.  The reflection coefficient and SWR is

also computed.

PROGRAM NAME : TXNLINE

VARIABLE LIST:

FREQ      : Working frequency

Z0        : Characteristic impedance

RL        : Real part of load impedance

XL        : Imaginary part of load impedance

L         : Length of line in meters

REAL      : Real part of input impedance

IMAG      : Imaginary part of input impedance

GAMMA     : Magnitude of reflection coefficient

ANGAMMA   : Angle of reflection coefficient

S         : SWR

PROCEDURE :

Choose Fl (TRANSMISSION LINE PROBLEM) from main menu.  Then

choose Fl (Input impedance, load , etc.)

EXPERIMENTS

1.  For an open circuit line of .2 m at 300 MHz, what is the

input impedance?  What is the input impedance for a line of .6 m

at the same frequency?  What about a .25 m line?  Can you plot a

9raph of the input impedance versus length at 300 MHz?

2.  Try the same experiment for a short circuit line.

3.  When are the input impedance of an open circuit and a short

circuit line the same?

4.  What does a quarter wavelength do to the load?

5.  What is the input impedance of a half wavelength line?  Try

the experiment using an open circuit, a short circuit and with a

load of 30+j80 ohms at 300 MHz.

6.  What is the reflection coefficient and SWR of an open and

short circuit line?  What about a matched load?

7.  What happens to the reflection coefficient for a resistive

load which is greater than the characteristic impedance?  What

about a smaller load?

8.  Given a transmission line of half a meter, with a load of

25+j75 ohm, what is the input impedance of the line at 300 MHz?

9.  What is the reflection coefficient and SWR?

10.  What happens to the input impedance when the length is

changed to a quarter meter?

SINGLE STUB MATCHING

This program will compute the length of the short circuit

stub and its distance from the load needed for matching.  Two

solutions are given.  The program requires the operating

frequency, the characteristic impedance and the load as input.

PROGRAM NAME : STUBMATC

VARIABLE LIST:

FREQ     : Working frequency

Z0       : Characteristic impedance of line

RL       : Real part of load impedance

XL       : Imaginary part of load impedance

Sl,S2    : Stub lengths

Dl,D2    : Stub distance from load

B        : Stub reactance

PROCEDURE:

Choose Fl (TRANSMISSION LINE PROBLEM) from main menu.  Then

choose F2 (Single stub matching).

EXPERIMENT

1.  A 50 ohm transmission line is connected to a load of

impedance 25-j75 ohm.  Find the position and length of a short

circuited stub needed to match the line.

2.  Using the transmission line program, what is the SWR before

and after matching?

3.  Try the experiment with a load impedance of 50 ohm.

4.  Can you match a load of 100+j100 ohm to a 50 ohm line?

LOG PERIODIC DIPOLE ARRAY

The Log-periodic dipole array (LPDA) is a very broadband

array.  This program computes the number of elements, lengths

and spacings for a certain frequency bandwidth determined by the

desired maximum and minimum frequencies that the LPDA is to

operate over.

PROGRAM NAME : LPDA

VARIABLE LIST:

FMIN     : Minimum frequency (MHz)

FMAX     : Maximum frequency (MHz)

TAU      : Scale factor

SIGMA    : Spacing factor

ALPHA    : Vertex angle

N        : Number of elements

L(I)     : Length of elements

X        : Length of array

PROCEDURE:

Choose F4 (BROADBAND ANTENNAS) from the main menu.  Then choose

Fl (Log-periodic dipole array).

EXPERIMENT

1.  Design a LPDA to operate from 54-216 MHz.

For details on the geometry and definition of the parameters

sigma, tau and alpha, see Antenna Theory and Design by Stutzman

and Thiele, John Wiley and Sons, 1981, page 287.

YAGI-UDA ARRAY

The Yagi-Uda array is a very directive and high gain array.

In this program, the three-element Yagi is studied.  The

reflector and driver is of half-wavelength.  The length of the

director can be varied by changing its reactance.  The distance

between the reflector and driver and the director and driver is

set with the provision of changing the director spacing during

the program.  This permits the front-back ratio to be determined

for various setting of director lengths and spacings.

PROGRAM NAME : YAGI

VARIABLE LIST:

SR       : Spacing between driver and reflector

SD       : Spacing between driver and director

XZ(3,3)  : Director reactance

ZR(0)    : Director resistance

ZDR      : Driving point impedance of YAGI (real part)

ZDX      : Driving point impedance of YAGI (imaginary part)

MAGEFTOB : Front/Back ratio

D        : F/B Directivity

DBACK    : B/F Directivity

PROCEDURE:

Choose F2 (WIRE ANTENNAS) from main menu.  Then choose F3 (Yagi

Uda antenna).

EXPERIMENT

1.  Find the optimum F/B ratio for a three-element Yagi with a

spacing of 0.1 lambda between the elements.  What is the driving

point impedance and directivity at that value?  (Tabulate the

F/B ratio for different director reactance).

2.  For a fixed length of elements and a distance of 0.1 lambda

between the reflector and driver, find the distance between

the director and driver which gives the optimum F/B ratio.  What

is the driving point impedance and directivity of the array?  (Tabulate the

F/B ratio for different director spacing).

3.  When does the reflector functions as a director?

4.  What is the advantage of using a Yagi array?

UNIFORM LINEAR PHASED ARRAY

Linear array will plot out the array factor for an

equispaced, isotropic sources excited with equal magnitude of

current with progressive phase shift.

PROGRAM NAME: LINPHASE

VARIABLE LIST:

ALP  : Progressive phase shift in degrees

D    : Element spacing in lambda

N    : Number of elements

THE0 : Main beam direction

PROCEDURE:

From the main menu choose F3 (ARRAYS).  Then choose F2 (ARRAY

FACTOR) from the next menu.  Finally choose Fl (Uniform linear

phased array).

EXPERIMENT

1.  Plot the array factor of a 2-element array with .5 lambda

spacing and zero phase shift.  Where is the main beam?

2.  Now increase the phase shift by 180 degrees.  (The phase

shift can be incremented or decremented by 10 degrees using the

up or down arrow cursor keys.  However, you are recommended to

quit and rerun program if the change in phase shift required is

large).  Where is the main beam?  Is the directivity the same as

before?

3.  Tabulate the HPBW and directivity for an array with 5, 10,

20, 50 and 100 elements.

4.  Do the same as above, but this time steering the beam by 45,

90 and 180 degrees?  Does the HPBW and directivity stay the

same?  If not, why?

5.  What is the advantage of increasing the number of elements?

What is the better trade-off, HPBW or directivity?

NOTE:  The isotropic elements are placed horizontally in the

polar plot.

LINEAR ARRAY (BEAM STEERING)

Linear array will plot out the array factor for an

equispaced, isotropic sources excited with equal magnitude of

current with progressive phase shift.  The user can specify the

main beam direction and it will compute the required phase

shifts required and also plot the array factor.

PROGRAM NAME: LINSCAN

VARIABLE LIST:

ALP   : Progressive phase shift in degrees

D     : Element spacing in lambda

N     : Number of elements

THE0  : Main beam direction

PROCEDURE:

From the main menu choose F3 (ARRAYS).  Then choose F2 (ARRAY

FACTOR) from the next menu.  Finally choose F2 (Linear array

(beam steering)).

EXPERlMENT

1.  plot the array factor of a 5-element array with .5 lambda

spacing and main beam directed at 0.  What is the phase shift

required?

2.  Now steer the beam to ten degree.  What is the required

phase shift?  ls the directivity the same as before?

3.  Tabulate the HPBW and directivity for an array with 5, 10,

20, 50 and 100 elements.

4.  Do the same as above, but this time steering the beam by 45,

90 and 180 degrees?  Does the HPBW and directivity stay the

same?  If not, why?

5.  What is the advantage of increasing the number of of elements?  What is

the better trade-off, HPBW or directivity?

NOTE:  The isotropic elements are placed horizontally in the

polar plot.

PLANAR RECTANGULAR ARRAY

Given the direction of the main beam required (both theta and

phi), this program will compute the progressive phase shift for a

rectangular planar array.  The user can also specify the phase

shifts, and it will calculate the main beam direction.  It will also

plot the array factor in both the xz-plane and yz-plane.

PROGRAM NAME : RECTPLAN

VARIABLE LIST:

THE0      :Main beam direction in THETA

PHI0      :Main beam direction in PHI

M         :Number  of elements in x-direction

N         :Number of elements in y-direction

DX        :Element spacing in x-direction

DY        :Element spacing in y-direction

ALPX      :Progressive phase shift in x-direction

ALPY      :Progressive phase shift in y-direction

PROCEDURE:

From the main menu, choose F3 (ARRAYS).  Then choose F2 (ARRAY

FACTOR) from that menu.  Finally choose F3 (Planar rectangular

array).

EXPERIMENT

1.  Design a 10 x 8 (10 elements in x-direction and 8 elements

in y-direction) element uniform planar array so that the main

beam maximum is oriented along  theta=10 and  phi=90.  For a spacing of

dx=dy= 1/8 between the elements, find the

a) progressive phase shift between the elements in the x and y

direction.

b)  directivity of the array.

c)  half-power beamwidths (in the perpendicular planes) of the

array.

2.  Repeat 1) such that  theta=80 and  phi=90.

Why does the directivity and half-power beamwidth change?

PLANAR CIRCULAR ARRAY

Given the direction of the main beam required, this program

will compute the phase excitation for a uniform circular planar

array.  It will also plot the array factor in both the xz-plane

and yz-plane.

PROGRAM NAME : CIRCULAR

VARIABLE LIST:

THE0      :Main beam direction in THETA

PHI0      :Main beam direction in PHI

N         :Number of elements

A         :Radius of array

ALPN      :Phase excitation

PROCEDURE:

From the main menu, choose F3 (ARRAYS).  Then choose F2 (ARRAY

FACTOR) from that menu.  Finally choose F4 (Circular Planar

array).

EXPERIMENT

1.  For a 10 element circular array with main beam in the  theta=0

and  phi=90, find the phase excitation required.  The radius of the

array is 1.591 lambda.  Plot the array factor in both the xz and

yz plane.  Calculate the directivity.

2.  Using the same number of elements and main beam direction,

increase the radius to 2,2.5, 5, 10 lambda.  What is the

directivity?

THE WOODWARD-LAWSON SYNTHESIS METHOD

The Woodward-Lawson method allows one to input a desired

pattern.  It will then plot the pattern and the current

distribution.  WOODLS is used to synthesize the line source and

WOODAF is used in array synthesis.

PROGRAM NAME : WOODLS and WOODAF

VARIABLE LIST:

WOODLS                                WOODAF

L     : Line source length           NE    : Number of elements

W(I   : Sampling point               D     : Element spacing

A(I)  : Desired pattern              A(I)  :Desired pattern

I(I)  : Current magnitude            I(I)  :Current magnitude

PHI(I):Phase shift

PROCEDURE:

Choose F6 (ANTENNA SYNTHESIS) from the main menu.  The choose F3

(Woodward-Lawson line source) or F4 (Woodward-Lawson linear

array).

EXPERIMENT

USING F3 OPTION

1.  The desired pattern is unity from w=-0.5 to 0.5 and zero

elsewhere.  It is to be synthesized with a ten-wavelength line

source using the Woodward-Lawson method.

a) Plot the pattern in linear rectangular form.

b) Plot the current distribution.

Repeat the synthesis using a) f(theta)=0.5 at w=-0.5 and w=0.5 and b)

f(theta)=0 at w=-0.5 and w=0.5.

Which choice is closer to the desired pattern? Can you explain

why?

USING F4 OPTION

2.  The sector pattern above with f(theta)=0.5 at w=0.5 is to be synthesized

with a 10 element, half-wavelength spaced array.  Determine the element

position, current excitation and phase.  Plot the synthesized pattern.

Repeat the above using increasing number of elements (say 15,

20, 50, 100, etc.).  What is a reasonable number of elements you

would use to achieve the desired pattern?

3.  A collinear array of 18 half-wave dipole antennas is to be

used to synthesize a sector pattern with a main beam sector over

the region 70 <theta< 110, that is f(theta)=1 over this region and zero

elsewhere.

a) For 0.65 lambda spacings determine the input currents

required for Woodward-Lawson synthesis of the complete pattern.

Account for the element factor.

b)  Plot the total array pattern.

4.  Repeat experiment 3 for a cosecant desired pattern where

f(theta ) is 1 for 80 <  theta < 90, cos 80/cos(theta)   for 0 <  theta <

80,

and zero elsewhere.

DOLPH-CHEBYCHEV SYNTHESIS METHOD

The Dolph-Chebychev method give a compromise between narrow

beamwidth and low side lobe levels.  This program will generate

the excitation currents and plot the array factor for a broadside,

equispaced linear array.

PROGRAM NAME : CHEBY

VARIABLE LIST:

NE       : Number of elements

D        : Element spacing

RR       : Side-lobe level (-dB)

HP       : Half-power beamwidth

A(N)     : Excitation currents

PROCEDURE:

Choose F6 (ANTENNA SYNTHESIS) from the main menu.  Then choose

F2 (Dolph-Chebychev array).

EXPERlMENT

1.  For a five-element, broadside, -20 dB sidelobe, half-

wavelength spaced Dolph-Chebychev array,

a)  Obtain the pattern plot

b)  Verify the sidelobe level and beamwidth from your pattern.

2.  Design a Dolph-Chebychev broadside array of five, half

wavelength spaced elements for a -30 dB sidelobes.

a)  Obtain the current distribution.

b)  Compute the directivity.

3.  Design a broadside Dolph-Chebychev array with six, 0.6

lambda spaced elements for -25 dB sidelobes.

a)  Obtain the element currents.

b)  Plot the pattern.

TAYLOR LINE SOURCE SYNTHESIS

The Taylor line source synthesis method allows for narrow

main beam with the first few sidelobes having nearly equal level

and decreasing far-out sidelobes.

PROGRAM NAME : TAYLOR

VARIABLE LIST :

NB       : Number of equal side-lobes

RR       : Side-lobe level (-dB)

L        : Aperture length

A(J)     : Sampling currents

W(J)     : Sampling points

HPWI     : Half-power beamwidth w.r.t wi

HP       : Half-power beamwidth

PROCEDURE:

Choose F6 (ANTENNA SYNTHESIS) from the main menu.  The choose Fl

(Taylor line source).

EXPERIMENT

1.  Design an eight-wavelength Taylor line source (nbar=7) with

-30 dB sidelobes.

a) Obtain and tabulate the sample values and locations.

b) Plot the pattern in rectangular logarithmic form.

UNIFORM RECTANGULAR APERTURE

Given the width of the aperture in both the x and y

direction, this program will compute the half-power beamwidths

in the xz and yz-plane.  The directivity is also calculated.  The programs

the

plots the radiation patterns in both the principal planes.

PROGRAM NAME : RECTAPER

VARIABLE LIST:

LX           : Width in x-direction in terms of lambda

LY           : Width in y-direction in terms of lambda

HPX          : Half-power beamwidth in xz-plane

HPY          : Half-power beamwidth in yz-plane

D            : Directivity

PROCEDURE:

From the main menu, choose F5 (APERTURE ANTENNAS).  Then choose

Fl (Uniform rectangular aperture).

EXPERIMENT

1.  Find the half-power beamwidths for a uniform rectangular

aperture which has Lx=20 lambda and Ly=10 lambda.  Plot the

pattern.  What is its directivity?

2.  Do the same thing for an aperture with Lx=3 lambda and Ly=2

lambda.

REFERENCES

1.  Antenna Theory and Design by Warren L. Stutzman and Gary A.

Thiele

- John Wiley & Sons, 1981

2.  Antenna Theory by Constantine A. Balanis

- Harper & Row, 1982

3.  Antenna Design using Personal Computers by Pozar

- Artech House

4.  Microcomputer Tools for Communication Engineering by

Shing Ted Li et. al.

- Artech House, 1983

EVALUATION FORM

Contrary to popular beliefs, this evaluation form will be

fully scrutinized.  So please fill it with care.  This will not

only help us to understand your needs, but it will also allow us

to modify and further improve the courseware.

IF YOU=STUDENT THEN FILL SECTION A

ELSE GOTO SECTION B

SECTION A

1.  Circle one :  FRESHMAN SOPHOMORE JUNIOR SENIOR

MASTERS PRE-CANDIDACY POST-CANDIDACY

2.  Circle the courses that you have taken :

Write the grade you obtained beside the courses you circled.

EE361    EE411    EE432    EE438    EE538

3.  Circle the courses that you are currently registered :

EE361    EE411    EE432    EE438    EE538

4.  Circle the courses you plan to register in future :

EE361    EE411    EE432    EE438    EE538

If you had not used this courseware, would you still had

plan to registered for these courses?

EE361 Y/N EE411 Y/N EE432 Y/N EE438 Y/N EE538 Y/N

5.  Would you choose Antenna Engineering as a career? Y/N

SECTION B

1.  Circle the programs that you used :

TXNLINE STUBMATC DIPOLE SELFIMP MUTUAL YAGI

DRIVPT LINPHASE LINSCAN RECTPLAN CIRCULAR LPDA

RECTAPER TAYLOR CHEBY WOODLS WOODAF

2.  On the scale of :  1 - POOR

2 - FAIR

3 - GOOD

4 - VERY GOOD

5 - EXCELLENT

how would you rate each program?  Write your rating beside

each of the programs below.

TXNLINE   STUBMATC  DIPOLE    SELFIMP   MUTUAL    YAGI

DRIVPT    LINPHASE  LINSCAN   RECTPLAN  CIRCULAR  LPDA

RECTAPER  TAYLOR    CHEBY     WOODLS    WOODAF

3.  Circle your ratings below:

a.  Increased understanding of

Antenna Engineering                 1  2  3  4  5

b.  Easiness of understanding

concepts                            1  2  3  4  5

c.  Visualization of the

concepts presented                  l  2  3  4  5

d.  Expectation                         1 2 3 4 5

e.  Learnt something new                1 2 3 4 5

f.  Overall performance                 1 2 3 4 5

g.  Documentation                       1 2 3 4 5

h.  Ease of use                         1 2 3 4 5

i.  Value                               1 2 3 4 5

4.   Would you recommend others to use it? Y/N

5.  Please write your remarks and comments below

Thank you.

8/14/86