Contents

µIntroduction	2

Source file organisation	2

Writing a video card detection routine	3

The video mode translation tables	3

The video chipset revision ID definitions	4

The video chipset name table	4

The find() card detection routine	4

Helper Functions	5

The _TABLES.C file	6

Writing the low level support routines	7

The bank16,bank256 routines	7

The rbank16,rbank256 routines	7

The page routines	7

The 6bit and 8bit DAC routines	8

The setup and exit routines	8

The init routines	8

The _TABLES.ASM file	9

Tying it all together	10

# Introduction

This document is intended for the developer or video card vendor who wishes to enhance the support provided by the Universal VESA VBE for a particular video card. It describes the internal details of the Univesal VESA VBE, explaining how to modify existing routines, or to add support for a completely new video card.

The intent of this document is to provide the necessary information to allow the Universal VESA VBE to be extended and modified with a minimum of fuss. The easier it is for this to be done, the more likely support for particular video cards will be enhanced or added.

# Source file organisation

This new version of the Universal VESA VBE has been organised with maintainability and extensibility in mind. All the code that is specific to a particular SuperVGA chipset is located in two files - a .C file providing the detection routines, and a .ASM file providing the low level support routines required when the TSR is installed. Thus all of the code for the Tseng Labs SuperVGA chipsets is located in the files _TSENG.C and _TSENG.ASM. Notice that the files are preprended with an underscore - this is to enable all the SuperVGA chipset specific files to be quickly identified. Any file that does not have an underscore prepended to it should not need to be modified to enhance or add support for a particular SuperVGA chipset.

Thus if you are interested in a particular SuperVGA chipset, you need only look in the two source files that relate to that particular video card. In fact, you should not even need to know the internal details of the rest of the source code in order to fix a problem or add support for a new video card. If you are adding support for a non-supported video card, you will also need to add a couple of lines to the DETECT.C and

VESAVBE.ASM source files.

If you are about to try and add support for a SuperVGA chipset that is not already supported, you may wish to peruse the .C and .ASM files for a video chipset that is already support in order to get an idea of how things have been laid out.

All of the code in the package has been written using Borland C++ 3.1 and TASM 3.1. It cannot be compiled and assembled using Microsoft C and MASM.

# Writing a video card detection routine

The video card detection routines are located in the .C files specific to each video card. Thus the detection routines for the Tseng Labs SuperVGA chipsets is located in the _TSENG.C source file. This source file

contains a mode translation table for translating internal VESA VBE modes numbers into the appropriate vendor specific mode numbers to initialise a particular video mode. It also contains numeric and textual definitions of the different chipset revisions, along with the routine that does the video card detection.

Although some of the detection routines already present in the library make use of BIOS calls to detect specific video cards and their configuration, this is not recommended. Routines that call the video BIOS will not be portable to a protected mode environment and will thus make the code specific to the DOS real mode environment. A better approach is to check for unique characteristics of each particular chipset, such as special extended registers that are unique to each chipset. One of the more robust methods of

doing this is to check for the existence of unique extended registers with access to these register first disabled, then enabled. Thus for a new SpiffyVGA video chipset, you might use the following code

sequence to determine if the video card is installed:

modinx(0xXXX,0xXX,0xXX,0xXX);    /* Disable extended registers   */

if (tstinx(0xXXX,0xXX,0xXX))     /* Check for locked Spiffy bank */

goto NoSpiffy;               /*  register 1                  */

modinx(0xXXX,0xXX,0xXX,0xXX);    /* Enable extended registers    */

if (!tstinx(0xXXX,0xXX,0xXX))    /* Check for unlocked Spiffy    */

goto NoSpiffy;               /*  bank register 1             */

/* We have a SpiffyVGA SuperVGA chipset!! */

Once you have determined that the chipset is present, you will then need to determine the currently installed configuration, such as the amount of video memory on board, and the chipset revision (newer revisions may provide enhanced features not found on older ones). Once again this should be done by directly accessing the chipsets extended registers.

If possible, I would like all of the video card detection routines to be re-written not to use the video BIOS, but to identify the video environment directly from the chipsets extended register specifications. The intent here is that this code can then be used to provide support for SuperVGA cards in protected mode environments that have no connection to MSDOS, such as Unix, Windows NT and OS/2 2.x.

## The video mode translation tables

In order to be able to initialise each of the available video modes supported by the SuperVGA chipset, you need to provide a video mode translation table for each chipset. This translation table will be used to obtain the values to load into the AX and BX registers in order to initialise a particular video mode via the

INT 10h BIOS interface. The translation table should be initialised to contain all of the possible video modes supported by the SuperVGA chipset to date. If a particular video mode is not supported by the installed chipset (maybe it is an older revision), support for a particular video mode should then be removed with the unsupportedMode() helper function (see below).

The format of the table is simple. It provides two values for each video modes supported by the Universal VESA VBE in a single dimensional array. The first value is the value to be loaded into the AX register, and the second value is the value to be loaded into the BX register before the INT 10h BIOS call is executed. Thus for a card that has VESA VBE compliant BIOS (such as the S3 based SuperVGA's), the first few entries in the table would look like:

short S3Modes[] = {0x4F02, 0x102,     /* 800x600 16 color    */

0x4F02, 0x104,     /* 1024x768 16 color   */

...

0, 0,              /* 1280x1024 16m color */

};

Thus to initialise the 800x600 16 color SuperVGA mode on this chipset, we load the AX register with the value 0x4F02 and the BX register with the value 0x102 and then execute an INT 10h instruction. Thus this would translate to the following assembly language sequence:

mov   ax,4F02h

mov   bx,102h

int   10h

Notice that the last entry in the table contained zeros. This most is not supported by S3 SuperVGA chipsets (at least not at the time of this writing), so the entries for this mode are set to zero. Thus is a mode is not supported by a particular chipset it's video mode translation table should contain zeros in the appropriate entries.

## The video chipset revision ID definitions

In order to be able to correctly install the low level support code, the detection routine should also determine the current chipset revision. This value is defined in a small ENUM definition in the .C source file, and as a number of macro definitions in the .ASM source file. A sample definition might be:

typedef enum {

grSPIFFY_REV1,            /* Spiffy VGA revision 1    */

grSPIFFY_REV2,            /* Spiffy VGA revision 2    */

} Spiffy_chipsets;

The naming conventions that you use should follow the normal conventions used throughout the code. If the video card id is grSVGA_SPIFFY (defined in the CHIPSETS.H file) for examples, you would then name each revision grSPIFFY_xx where xx is the name for that revision (like V1 or something).

## The video chipset name table

In order to be able to inform the user of the program what type of video card has been detected, a lookup table describing each individual chipset revision is also required, even if only one chipset revision is supported. This table is an array of character pointers that is initialised at compile time. A sample table might look like the following (assuming the above numerical chipset id's):

char *SPIFFYNames[] = {

"Revision 1",

"Revision 2",

};

## The find() card detection routine

All the actual code to detect a particular SuperVGA chipset is defined in the find() routine. Assuming you are attempting to detect our hypothetical SpiffyVGA SuperVGA card mentioned earlier, the routine would be defined as follows (note: the name of the detection routine must always be prepended with 'find'):

bool findSpiffy(int *superVGA,int *chipID,int *memory,int *dac,

int *pageFlip)

{

/* Perform video card detection here */

*superVGA = grSVGA_SPIFFY;

*chipID = grSPIFFY_REV1;

*memory = 2048;

*pageFlip = true;

return true;

NoSpiffy:

return false;

}

The values passed to the detection routine should be modified by the detection code in order to accurately describe the capabilities of the currently installed SuperVGA. If the video card was succesfully detected, the routine should return true, otherwise it should return false and the detection process will continue. The following describes each parameter in detail:

superVGA	The value in this variable should be set to the id for the SuperVGA chipset that was detected (the definitions for these values are stored in the CHIPSETS.H header file, which you will need to modify if you add support for a new SuperVGA).

chipID	The value is this variable should be set to the internal SuperVGA chipset revision id depending on the chip revision that was detected. The values stored in this variable are defined in the enum definition in the same source file.

memory	This variable should be set to the amount of installed video memory on the SuperVGA card. If the memory cannot reliably be detected, this value should be set to zero and the user will be forced to specify the memory size from the command line.

dac	This variable will already be defined for you when this routine is called. The Universal VESA VBE will perform a general video DAC detect routine to determine the type of installed DAC (8 bit, 15 bit, 16 bit or 24 bit). However you may override this value if the video card provides a more reliable way of detecting the installed DAC (such as setting bits in an extended register).

pageFlip	This variable should be set to true if the SuperVGA chipset supports extended CRT addressing that can be used to implement virtual screens and double buffered animation (assuming enough memory is installed).

## Helper Functions

In order to simplify the process of writing the video card detection routines, a number of helper functions are provide which perform the bulk of the work that is required. You simply need to string together calls to these helper functions to detect the appropriate video card and configuration.

int rdinx(int port,int index);

Read a value from the specified index register accessible via the specified port address. The value read is a single byte, but is returned as a signed integer in the range 0-0xFF.

void wrinx(int port,int index,int val);

Write the specified value to the index register accessible via the specified port address. The value written is a single byte in the range 0-0xFF.

void modinx(int port,int index,int mask,int value);

Modify the specified bits stored in the index register accessible via the specified port address. The mask defines all the bits that will be modified (a 1 indicates it should be modified, a 0 indicates it should be retained). The value parameter is then used to modify the specified bits. Effectively the operation being peformed is: reg = reg | (value & mask).

bool tstreg(int port,int mask);

This routine is used to determine if a particular register is responding correctly or not. The mask value is used to define which bits of the register should be modifiable. If all of the bits specified in the mask can be correctly modified the register is present and the routine returns true. Otherwise is returns false.

bool tstinx(int port,int index,int mask);

This routine is similar to the tstreg() routine above, but works for a indexed register rather than a single I/O port.

void unsupportedMode(int VBEMode,short *modeTable);

This routine is used to remove support for a particular video mode from the mode tables (it should be used to remove support when installing for older chipset revisions). It takes a number representing the internal VESA VBE mode number (defined in the CHIPSETS.H file) and a pointer to the video card's mode translation table.

void setBytesPerLine(int VBEMode,int bytesPerLine);

This routine is used to change the default bytes per line value for a particular video mode. This is required for some video modes (such as 24 bit video modes) in order to simplify the programming requirements.

saveMode(void);

restoreMode(void);

The saveMode() and restoreMode() functions can be used to save the current video mode and restore it later on. This should be used if the video detection routines need to initialise a specific video mode in order to determine information about the video environment. Ideally you should not need to do this.

## The _TABLES.C file

The _TABLES.C file is used to tie together all of the individual mode tables and chipset name tables defined for each of the supported video cards. It contains a number of lookup tables indexed on the SuperVGA card's id to find the appropriate video mode table etc. You will need to add entries to each of these tables if you are adding support for a new video card.

# Writing the low level support routines

The low level support routines are required by the resident portion of the Universal VESA VBE to perform the bank switching, extended CRT addressing and other required low level functions. Each routine is described in detail in the next section.

Please note that all the routines except the init routine are required to be completely relocatable so that they may be relocated into a small buffer area when the TSR goes resident. The bank switching and extended CRT addressing routines may also be copied by an application into the applications code space to be run in either 16 or 32 protected mode (for 32 bit protected mode the retf instruction will need to be patched by the user program). Hence these routines may not access any global variables of any kind.

## The bank16,bank256 routines

The bank16 and bank256 routines are required to change video banks in 16 color modes and in 256 color modes (the 256 color version should also work in 15,16 and 24 bit modes). These routines will change both of the read and write banks to the same value for video cards that support separate read/write banks. If bank switching is performed in the same way in all video modes, you may simply provide one bank switching routine.

The low level assembly language interface to these routines is defined as follows (note that these routines always take a bank number in 64k increments, even though the underlying hardware may provide a better granularity such as 16k or 4k; this is to provide compatability between different bank switching schemes):

Entry:	AL	- Bank number to set (0-255) in 64k increments

Exit:	AL	- Bank number that was set

Registers:	None, _ALL_ registers _MUST_ be preserved across this call. The

state of the interrupt flag may _NOT_ be modified during this

call.

## The rbank16,rbank256 routines

The rbank16 and rbank256 routines are provided for video cards that support separate read/write banks, one for each of the 16 and 256 color video modes. These routines will change only the read bank to the specified value. If your video card does not support separate read write banks, these routines may be left out. Note that in order to set up separate read/write banks, an application must first call the standard bank routine defined above to set both the read./write banks to the same value (most common operation) and then call the read bank routine to change the read bank to a different value.

The low level assembly language interface to these routines is defined as follows (note that these routines always take a bank number in 64k increments, even though the underlying hardware may provide a better granularity such as 16k or 4k; this is to provide compatability between different bank switching schemes):

Entry:     AL    - Read bank number to set (0-255) in 64k increments

Exit:      AL    - Read bank number that was set

Registers: None, _ALL_ registers _MUST_ be preserved across this call. The

state of the interrupt flag may _NOT_ be modified during this

call.

## The page routines

The page routines are provided to change the extended CRT starting address for video cards that support this feature. The extended CRT start address can then be used to view any section of video memory for implementing virtual screens and for implementing double buffered animation.

The low level assembly language interface to these routines is defined as follows (note that this routine

sets both the standard VGA CRT values as well as the extended CRT address values - a macro is provided

to set the standard VGA CRT values first, and to move the extended CRT bits into the BX register):

Entry:     BL    - Index of start address low register

BH    - Bits 7-0 of new start address

CL    - Index of start address high register

CH    - Bits 15-8 of new start address

SI    - Bits 16+ of new start address

Exit:      Nothing.

Registers: AX,BX,CX,DX,SI - Register values need not be preserved.

## The 6bit and 8bit DAC routines

The 6bit and 8bit DAC routines are used to kick the video card's palette into an 8 bit mode if supported, and to return the DAC back to the normal 6 bit VGA compatible palette. The routine takes no parameters and simply performs the necessary register manipulations to do this.

Currently this has not been implemented for any of the supported SuperVGA's due to lack of information (and lack of hardware support in most SuperVGA's).

## The setup and exit routines

The setup routines are called after the SuperVGA video mode has been initialised. These routines should enable access to extended registers that must be enabled during normal operation, and may also modify certain register values to get the video card to work in dual banking mode or to enable the extended CRT addressing routines.

The exit routine is called prior to resetting a non graphics mode and should perform the reverse of the setup routines, such as disabling extended registers and turning off dual banking mode. Both routines take no parameters and are simply called upon to peform the appropriate function.

## The init routines

One init routine is provided for each video card type, and is the last procedure in the _*.ASM file. This routine will be called to initialise the appropriate vectors to point to the correct bank switching, CRT addressing, setup and exit code for the installed SuperVGA chipset. If different chip revisions require different bank switching routines etc, the you will need to correctly install the vectors depending on the type of chipset installed. The vectors that you will need to modify are:

WriteBank16           - Address of 16 color write bank routine

WriteBank16Len        - Reloctable length of this routine

ReadBank16            - Address of 16 color read bank routine

ReadBank16Len         - Relocatable length of this routine

WriteBank256          - Address of 256+ color write bank routine

WriteBank256Len       - Reloctable length of this routine

ReadBank256           - Address of 256+ color read bank routine

ReadBank256Len        - Relocatable length of this routine

NewPage               - Address of extended CRT address routine

NewPageLen            - Reloctable length of this routine

SetupSVGA             - Address of SuperVGA setup routine

SetupSVGALen          - Reloctable length of this routine

ExitSVGA              - Address of SuperVGA exit routine

ExitSVGALen           - Relocatable length of this routine

Set8BitDac            - Address of routine to force 8 bit DAC mode

Set8BitDacLen         - Relocatable length of this routine

Set6BitDac            - Address of routine to force 6 bit DAC mode

Set6BitDacLen         - Relocatable length of this routine

Note that you do not need to set all the vectors. If the card does not provide separate read/write banks or extended CRT addressing, then you can simply leave the vectors in the default state (which will disable the appropriate functions).

There are also a number of global variables that will have been initialised before this routine is called, so that you can determine the current SuperVGA environment. These are:

CntSuperVGA           - ID of currently installed SuperVGA

CntChipID             - Internal chip revision id for chipset

CntMemory             - Amount of memory installed on card

CntDAC                - Type of video DAC installed on card

## The _TABLES.ASM file

The _TABLES.ASM file contains a single lookup table for finding the correct video card init routine given the video cards id value. You will need to add a new entry into this table if you add support for a new video card to the program.

# Tying it all together

In order to complete the installation of support for a new video card, you will need to modify the DETECT.C, CHIPSETS.H, CHIPSETS.EQU and VESAVBE.ASM source files as well. You will need to add a definition for the SuperVGA card that you have installed into the CHIPSETS.H and CHIPSETS.EQU files. You will also need to modify the DETECT.C to include the detection code for the video card and to call the detection code from the detect() routine. Finally you will need to modify the VESAVBE.ASM file to include the low level support code for the SuperVGA card.

So to continue with our previous examples, adding support for the SpiffyVGA card you would need to do the following:

CHIPSETS.H

typedef enum {

...

grSVGA_SPIFFY,       /* SpiffyVGA SuperVGA card       */

grSVGA_NONE    = -1

};

#define LAST_SVGA    grSVGA_SPIFFY

CHIPSETS.EQU

...

grSVGA_SPIFFY      = 18     ; SpiffyVGA SuperVGA card

grSVGA_NONE        = -1

DETECT.C

...

#include "_spiffy.c"

#include "_tables.c"

...

check(Oak);

check(Spiffy);

*superVGA = grSVGA_NONE;

return;

VESAVBE.ASM

...

include "_spiffy.asm"

include "_tables.asm"

You should now be able to make the executable program with support for the new SpiffyVGA SuperVGA card.

NOTE:	The order that the findXX() SuperVGA detection routines is called in is extremely important. Some video card answer to the same detection test that is performed to identify another video card (Cirrus card will answer to the AcuMos test, as will the Paradise cards). Thus certain tests must be performed before other tests to ensure the correct video card is detected.

Universal VESA VBE

Implementors Guide

Copyright ã 1993 Kendall Bennett

All Rights Reserved