___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: Scheck and Stacker Drive Space Reporting

STACKER NOTE TEC011 - 2/20/92                          STAC FAX Index #2602

___________________________________________________________________________

Background.

Use Stacker's SCHECK utility to see exactly how the space has been used

in the Stacker drive.

Here is an example of an SCHECK report:

SCHECK - 2.0, (c) Copyright 1990-92 Stac Electronics, Carlsbad, CA

Volume in drive C is STACVOL_DSK

No errors found

Stacker Drive Statistics:

Stacker Drive             STACVOL File

Drive C:                  D:\STACVOL.DSK

-------------------       --------------------

Total Bytes:  4,109,312                   2,054,656

Bytes Used:   3,735,552 ( 91.2%)          1,846,272 ( 89.9%)

Bytes Free:     373,760 (  8.8%)            208,384 ( 10.1%)

Stacker Drive Compression Ratio = 2.0:1

Projected Bytes Free            = 373,760

What does it all mean?

The Left Hand Column:  This column displays "logical" data in the clusters

on the Stacker drive.  In this example, it tells us that there are enough

allocation units (clusters) for 4 MB of data.  There are enough clusters

left to acomodate 373,760 bytes of data.  The DOS CHKDSK program

reports these numbers because it looks at allocation units.

The Right Hand Column:  This column shows the usage of actual physical

hard disk space inside the STACVOL.DSK file.  The Stacvol file contains

all of the data and free space for the Stacker drive C:.  It's name and

location are shown in the column title.  In our example, D:\STACVOL.DSK is

2 MB in size and has used all but 208,384 bytes of that 2 MB.  Therefore,

there are 208,384 bytes of physical free space left in D:\Stacvol.dsk.

Compression ratio.

The Stacker drive compression ratio indicates the average compression of

the files in that drive.  The compressibility of the files is generally

dependent on their contents.  In the above example, the 2.0:1 ratio tells

us that the files in this Stacker drive are taking up half the space they

would take up on a "Non-Stacked" drive. (A 2.0:1 ratio is typical for an

average mixture of files.)  A drive full of graphics files may achieve an

average compression of 8.0:1 or more.  An 8.0:1 ratio would mean that the

files are taking up 1/8 the space they would on a standard drive.

Conversely, some files are already compressed, such as "zipped" files.

Stacker will not attempt to compress previously compressed files, so they

will bring down the overall compression ratio of the Stacked drive in

which they reside.

NOTE: The drive compression ratio does not display if the drive is empty.

Projected bytes free.

This number is Stacker's estimate of the available space left on the

Stacked drive.  It is determined by multiplying the physical free space

(right side) by the compression ratio, OR by the number of clusters left

unallocated (left side).  The projected bytes free is limited to the

smaller of these two values.

In our example, physical space free multiplied by the compression ratio

would be: 208,386 x 2.0 or 416,772 bytes. However, we only have 373,760

bytes worth of unallocated clusters.  Since the projected bytes free is

the smaller of these two numbers, it is 373,760.  Fortunately, you can

increase the number of allocation units by "growing" the drive.  To grow

the Stacker drive, run the Grow & Shrink utility from the STAC command

assistant.  "Growing" the drive in this example would increase the

projected free space to 416,772.

Notes:  When the number for physical bytes free goes to zero, the drive is

full.  You cannot "grow" the drive to create more physical space.  Also,

if the drive is less than 12% full, the projected bytes free will equal

the "logical" bytes free number (left column).

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712