___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: DR DOS and Stacker

STACKER NOTE TEC012 - 2/20/92                          STAC FAX Index #2301

___________________________________________________________________________

Background.

Stacker is compatible with Digital Research DOS (DR DOS) versions 5.0 and

6.0.  However, there are some special considerations when using either

version of this operating system.

DR DOS 6.0.

1. DR DOS 6 is bundled with another file compression program.  There

is no reason to use this program if you have chosen to use Stacker.

Stacker gives you better speed, better compression, a smaller

device driver, and more advanced features than the compression program

that comes with DR DOS.

2. Installing DR DOS 6.0 after Stacker is installed may require a special

procedure.  See STACKER NOTE Tec023 (STAC FAX index #2303)

"Upgrading to DR DOS 6.0 with Stacker Installed" for details.

DR DOS 5.0 and 6.0.

If you configure Stacker to use EMS for its cache, or if you load the

Stacker device driver into upper memory, the following message will be

displayed by the Stacker device driver when you start your computer:

NOTE: "In DR DOS, loading Stacker high (or using /EMS)

may slow disk access.  Please see the Stacker README

file for help."

This condition can be remedied by using the /ND parameter on the

STACKER.COM device line in Config.sys.  Use the /ND parameter if your

hard disk controller is NOT a bus-master controller.  (If you are

unsure of  whether your disk controller is a bus-master, it probably

isn't.)  If you DO use a bus-master disk controller and it does not

support virtual DMA, DO NOT use the /ND switch to increase disk

performance.

If you do not use the /ND parameter and wish to suppress the warning

message, place the /NW (no warning) parameter on the Stacker DEVICE

statement in the CONFIG.SYS file.

DR DOS 5.0.

Under DR DOS 5.0,  the SSWAP option does not operate properly with

Stacker versions 1.0-1.11.  It is recommended that you upgrade to Stacker

version 2.0 in order to SSWAP the Stacker drives.

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712