This is an actual working small version of HYDRONET 3.0 where your own data

can be used to verify proper functioning of the program.

To start this small version ...

- print this document file

- insert the diskette in drive A:

- move to that diskette drive by typing " A: " then press RETURN

- type " HN " and press RETURN.

HYDRONET will then load.

A letter to interested parties...

Dear Engineer,

On this disk or residing in this directory is a small version of

Engineering Software's HYDRONET 3.0 along with instructions and 5

example problems.  HYDRONET performs the functions of a hydraulic

analysis tool for systems conveying fluids through complex network

systems.  It runs on the IBM PC and the related family of PC/MS-DOS

microcomputers.

The design engineer essentially works in a worksheet type

environment, analyzing hydraulic elements and observing the bottom

line effects.  HYDRONET's extensive flexibility starts with the fact

that calculations can be performed in English or metric units and

use any type of fluid.  All of Engineering Software's packages have

this basic capability.  The full featured version includes the

UTILITY functions where with a single command the entire system's

diameter or friction can be changed to a new value.

Cavitation calculations are made and reports detailing the system's

head losses for various flows are provided.  This small version is a

working version in that your own hydraulic calculations can be performed

and verified.  It is a good way to evaluate HYDRONET risk free.  Full

screen graphics of the systems hydraulic grade lines and pump curves are

accessible along with the full functions in the REPORT option.

The full featured version costs $495 and includes a User's Guide

with numerous examples and a hydraulic reference handbook.  Tele-

phone support is available at no cost to registered users.

ORDER FORM

Name: ________________________________  Title: ____________________

Firm: ________________________________  Phone: ____________________

Address: __________________________________________________________

City, St, Zip: ____________________________________________________

Both 5 1/4" and 3 1/2" diskettes are included in all packages.

Quantity      Product                                                                                              Total

____________  HYDRONET 3.0  ($495 each US)      _________

____________  HYDROFLO 1.2  ($195 each US)      _________

____________  H-CALC 1.1    ($ 95 each US)      _________

____________  All Three     ($645 total US)     _________

Calif. residents add 6% sales tax _________

If desired, add $15 Express (2nd Day) Service  _________

Outside USA, Canada and Mexico add $25 airmail _________

Total Enclosed _________

Call us!  We honor VISA/MasterCard orders.

Check  or  M.O.  enclosed _____      or    COD  delivery _____

Make checks payable and mail to:

ENGINEERING SOFTWARE

P. O.  Box 8128

Truckee, CA 95737

(916)  582 - 1525

This small version of HYDRONET 3.0 has the following limitations...

21 pipes, 20 nodes, 2 pumps and 8 fittings and valves.

The maximum element or node number is 100.

The following documentation contains a condensed

version of the full featured HYDRONET 3.0 manual.

INTRODUCTION

HYDRONET is a hydraulic network analysis tool based on the popular Hardy-Cross

method.  A hydraulic network is defined as a closed pressurized fluid

distribution system made up of pipes, fittings, valves, pumps and reservoirs.

Examples of network systems are industrial fluid conveyance systems,

municipal water supply systems, fire main systems or petroleum transport

systems.  In a network, many interconnected conveyance lines have to deliver

liquids to numerous nodes simultaneously. The engineer needs to know the flows

and pressures throughout the system so that the needed pipe and pump sizes

and strengths can be specified.  Once a system is defined by describing the

components in detail, various operational strategies can be investigated.

UNDERLYING  THEORIES

HARDY CROSS METHOD

The Hardy-Cross method is based on two principles...

1. The sum of the head losses around a single loop is always zero

(conservation of energy).

2. The sum of flows into and out of a node always equals zero.

(conservation of mass).

HYDRONET adjusts the flows in each loop to meet both the above conditions and

converges toward the steady-state flows in each successive iteration.  The

user provides a flow tolerance at which HYDRONET will exit the flow adjustment

process.  The original work on the Hardy-Cross method is published in "Hardy

Cross, Analysis of Flow in Networks of Conduits or Conductors", Univ Ill.

Bull. 286, Nov. 1936.

NODES

Nodes are the discrete physical points at either the junction of two

elements (pipes, valves, fittings or pumps), at the end of a fixed flow

line (branch) or at the location of the inlet or exit of a reservoir

(unknown inflow or outflow supply of fluid).

There are two type of nodes used in the Hardy-Cross method.  Nodes where the

either the flow (demand) or the head (pressure) is fixed.  You cannot fixed

both the flow and head.  Nodes at an inlet or exit of a reservoir of unknown

flow but known head (surface level) are termed "fixed head" nodes.  Nodes in

branch lines must be the "fixed flow" type.  HYDRONET will determine the

head at which these lines will provide the set flow amount.  Nodes between

elements at which the pressure is to be found, are also of the "fixed flow"

type.

Discharges to the atmosphere are "fixed head" conditions.  The atmospheric

discharge amount may also be fixed by setting the demand to a specific flow

or including it as the end node in a branch and HYDRONET will determine the

resulting head (pressure) at that node.

PSEUDO-LOOPS

Loops consist of an alternating node - element - node - element - node

sequence.  HYDRONET uses two types of loops, pseudo-loops and regular loops.

Pseudo-loops are loop sequences that start and end with "fixed head" nodes.

The head difference between the starting and ending "fixed head" nodes

maintain a constant head loss or gain in the calculation of total head losses

around the loop.  A loop connecting two fixed pressure mains or reservoirs

(tanks) would be a pseudo-loop.  All "fixed head" nodes must start or end

at least one pseudo-loop. Regular loops (sometimes called closed) start and

end at the same "fixed flow" node.

BRANCH LINES

Branches are fixed flow lines that take flow from a loop.  HYDRONET performs

simple head loss calculations on these lines.  All nodes in branches must

be "fixed flow" nodes and no pumps are allowed to be in these lines. Nodes in

the branch may each have fixed demands and the last node will have the flow

out of the line entered as its demand.

PIPES

Both the Hazen-Williams and Darcy-Wiesbach equations are available for

determining the major friction losses in pipes.

The empirical Hazen-Williams equation is available for water systems under

turbulent flow conditions and at 60 degrees F. (or for fluids with kinematic

viscosity around 1.2x10-5 ft2/s).  To use this equation, Hazen-Williams

coefficients are needed for all pipes.

The fundamentally based Darcy-Wiesbach equation can be used for fluids of

various viscosities.  The kinematic viscosity of the fluid must be known as

well as the piping materials specific roughness (sometimes called absolute

roughness).  HYDRONET uses the Newton-Raphson method to solve the

Colebrook-White equation which determines the Moody friction factor f.

FITTINGS

HYDRONET has a large selection of fitting resistance coefficients available

for use in water systems.  Minor friction losses through fittings (which

includes inlets and exits) in water systems are modeled by multiplying the

velocity head by the appropriate resistance coefficient, K.  HYDRONET will

automatically use the proper friction coefficient upon menu selection of the

type and diameter.  Custom K values can also be input for fittings or valves

not listed.

VALVES

A large selection of valves are available for use in water system design.

Check valves allow flow in only one direction.  Custom valve coefficients

can be input by pressing F2 when the valve menu is displayed.

PUMPS

HYDRONET allows pumps to be used in the Hardy-Cross method by incorporating

their head additions.  Four points off the pump curve are needed so that

HYDRONET can fit a cubic equation to the curve.  A continuous function for

head versus flow will then be available for use.  Pump manufacturers provide

plots of their pump performance curves.

START-UP

To start the small version of HYDRONET, go to the disk drive that the diskette

has been placed in (ie, type A: or B:, then RETURN) then, at the DOS prompt

type " HN " and press RETURN.

HYDRONET takes a few seconds to load.  The opening screen displays information

on the full featured version.  Press RETURN and the main HYDRONET screen

appears.  The top line of the main screen displays the HYDRONET copyright

notice on the left and the current data filename on the right with the word

"Balanced" if the system has been solved.  The bottom line of the screen has

the computer time and date on the left and the right has an error space

reserved for when errors occur during system operation.  The various

keystrokes available as responses for any HYDRONET prompt are displayed on

the line just above the bottom line of the screen.

HYDRONET uses the following Lotus-style menus and keystroke syntax...

To select an option from a displayed menu:

- Use the space bar or cursor arrows to hi-light the option and press RETURN.

- or press the first character of the menu option name.

To exit from a menu or operation:

- Press escape (Esc key).  (Note:  pressing escape repeatedly will eventually

bring you back to the main menu but never let you exit to DOS.  To fully

exit, you must press "Y" at QUIT).

An on-line help facility is available by pressing the F1 (function) key

anywhere during program execution.

When entering characters or numbers, a toggle between insert mode and overtype

mode is available by pressing the insert (Ins) key.

MENU OPTIONS

FILE

The FILE option allows the retrieval from disk of datafiles. The HYDRONET data

file names are appended with the extension of ".NET" automatically.  The user

need not type it in and cannot override this extension.

Selecting the RETRIEVE sub-option will display a screen of HYDRONET data

filenames.  Use the space bar or cursor arrows to hi-light the desired

filename and then press RETURN to retrieve the data in the file.  The user

may also press function key F2 and key in a custom path and filename (ie,

B:\NET_DATA).  The data will be read by HYDRONET and the main screen will

return with the current filename displayed in the upper right corner. This

small version does not allow your work to be saved.

SETUP

The SETUP option is used to set the default data file directory (where *.NET

datafiles will reside), head loss equation, flow tolerance, units systems,

fluid properties and default friction coefficients.  The up or down arrow keys

are used to move between the various input fields (RETURN acts like down

arrow).  Pressing escape (or moving off the end of the fields) will lead to

the prompt:

Reset current system to new values?     NO     YES

If the new parameter values aren't desired, press the escape (Esc) key

or "N" to return to the main menu.

PARAMETER DESCRIPTIONS

The DATA FILE DIRECTORY is the path where HYDRONET will look for data files

(shown in the FILE - RETRIEVE option).

Pressing RETURN (or down arrow) hi-lights the HEAD CALCULATION EQUATION field.

The user chooses either the Hazen-Williams or the Darcy-Wiesbach equation by

pressing the left or right arrow keys and then pressing RETURN or the arrow

down key to select it.

The UNIT SYSTEM field will now be hi-lighted.  The UNIT SYSTEM and FLOW UNITS

are chosen, as above, by pressing arrow right or left then pressing RETURN or

arrow down to select.

The FLOW TOLERANCE is the flow amount at which HYDRONET will exit the

iterative flow adjustment process.  The iterative process can always be

exited by pressing escape.

HEAD UNITS are selected like flow units above.  If using units of psi or

Kpascals, be sure to set the proper liquid specific gravity.

The DEFAULT SPECIFIC ROUGHNESS or DEFAULT H-W COEFFICIENT can be set so that

the user has to only press RETURN when performing pipe data input.

The SPECIFIC GRAVITY value is needed for the conversion of head units to psi

or Kpascals.

The KINEMATIC VISCOSITY is required when using the Darcy-Wiesbach equation.

DEFINE

To enter individual element data, use the DEFINE main menu option.  The

sub-menu of LOOPS  BRANCHES allows either loop (pseudo-loop or regular loop)

or branch information to be entered.  Select LOOPS by hi-lighting the word

and pressing RETURN or simply press the letter "L".  The data for loop #1

will be entered first.  You will be prompted for the first node number if a

new system is to be entered (or be able to view the first page (screen) of

element and node data).  The starting node will be a "fixed head" node if

inputing a pseudo-loop or a "fixed flow" node if inputing a regular loop.

You enter data for nodes and elements in the same sequence that they occur

in the loop of your design (ie, node - element - node - element - node,

etc.).  The default value in the element or node number field will be the

lowest number that has not been used in the current design.  You can overtype

this value with any number desired.  Pressing RETURN, the node data fields

appear.  Once the first nodes data is input, an element menu will appear and

allow the next element type in the sequence to be specified.  Once the element

type is selected by hi-lighting the type and pressing RETURN, an input field

for the element number will appear.

To input the second and remaining loops, press the F8 key.  Respond

"Y" to the prompt for creating another loop and the pointer will go to

the first node of the next loop.  If the next loop exists, the

first screen of the next loop will appear.  The current loop or branch number

out of the total number in the design appears on the bottom left of the

display.  The current screen (page) out of the total number available for

the current loop or branch is also displayed.  If some elements and nodes

in the current loop have been previously input in another loop, you simply

have to press RETURN at the data input fields to retain existing data.

The next section details the type of nodes and elements available for defining.

NODES

Node data includes elevation, demand and head (pressure).  There are two type

of nodes.  Nodes with a "fixed head" and those with a "fixed flow".

A node with a set pressure and a flow to be determined, is a termed a "fixed

head" node.  Examples are any supply or outlet fluid level (reservoir) or an

unrestricted supply or drain of fluid at a specific elevation and pressure.

For a fluid reservoir at a specific surface height, the surface elevation

would be input in the elevation field, a "?" (question mark) would be input

in the demand field (this is to be solved for), and zero would be entered

in the head field.  A supply point (hydrant) would have its inlet elevation

entered, a "?" in the demand field and the supply pressure entered.

A node with a head to be determined, is a "fixed flow" node, even if the

flow or demand at the node is zero.  The elevation would be the ground surface

elevation and the demand would be input as zero or a negative amount

signifying flow out of the node.  The head field requires a "?" to be entered

so HYDRONET will solve for the resulting pressure at the node.

PIPES

The data entered for pipes include diameter, length and friction factor.  The

friction factor that appears will be the default coefficient specified in the

SETUP menu option.

VALVES

Like using fittings, once valves are selected, a menu lists numerous types.

Use the cursor arrow keys to hi-light the desired valve type and press RETURN

to select it. You will then be prompted for the fitting diameter.

BRANCH LINES

HYDRONET allows fixed flow outlet lines to be included in a network design

(these lines not being a part of a true "network").  Simple head loss

calculations are performed to find the heads at branch nodes.  The same input

procedure applies to branches as it does to loops with minor exceptions...

The flow direction in branches is always out from the node that starts the

branch (the node connected to a loop).  On the screen this flow appears

left to right. The branch input still goes from node to element then node

and will always end on a node.  The last node will have the flow through the

branch entered in its demand field.  Other nodes in a branch may have a

demand value entered also.

EDITING...

This is a list of the keystrokes available for use in editing and moving

through the loop and branch data within the DEFINE option.

Right and Left Arrow keys

Use these keys to move the position pointer to the previous or next node or

element in the sequence.

(F2)  Edit

Use the cursor arrow keys (right or left) to move the pointer to an existing

element or node and press F2 (Function key 2) to edit its data.  The input

screen will be displayed with the current data.

(F3)  Delete node and element

When positioned at a defined element or node, pressing the F3 key prompts the

user for deletion of an element and node.  If the element and node are not the

ones to be deleted, press escape (Esc key) and move to the correct position.

(Ctrl-F3) Delete loop or branch

Pressing Ctrl-F3 (hold down the Ctrl key and press the F3 key) prompts the

user for deletion of the entire current loop or branch.  If you did not intend

to delete the current loop or branch, press escape (Esc key).

(F4)  Insert node and element

When positioned at a defined element or node, pressing the F4 key allows the

user to insert an element and node just before the current position.

(F5)  Plot pump curves

(F6)  Print pump curves & HGL plots

When positioned at a pump, pressing F5 plots the curve on a graphics screen.

(F7)  Go to previous sequence (loop or branch)

(F8)  Go to (or create) next sequence (loop branch)

Pressing F7 places the position pointer to the first node of the previous

sequence.  Pressing F8 will place the position pointer at the first node of

the next loop or branch.  If you are positioned anywhere within the last loop

or branch, HYDRONET would ask...

Create another loop (branch)?  NO   YES

and if you answer "Y" then you would be positioned to enter data for the first

node of a new sequence.

(F9)  Go to element

(F10)  Go to node

Pressing F9 prompts...    Go to element _____

Pressing F10 prompts...   Go to node _____

and when a number is entered, the position pointer is placed at an occurrence

of that element or node.  If the position pointer is not moved and the F9 or

F10 key is pressed again and the same element or node number is entered, the

position pointer will be placed at the next occurrence of that element or node

in the system.  This way you can locate all occurrences of a particular

element or node.

(Pg-Up)  Go to prev page (screen)

(Pg-Dn)  Go to next page (screen)

These keys allow fast movement within the screens (pages) of each sequence.

The current loop or branch number out of the total number of sequences and the

current screen number out of the total for the sequence is displayed on the

lower left of the screen.

REPORT

The REPORT option offers reports of elements, nodes and loops, along with

summaries of flows, head losses, pressures and hydraulic grade line plots.

The user is asked to specify whether output is to go to the SCREEN, FILE or

PRINTER.

SUMMARY

The first screen of the REPORT - SUMMARY option lists the pipe head loss

equation along with the flow tolerance and fluid properties on the top of

the first screen.  The bottom of the screen displays a summary of node,

element and loop totals.

The following screens list pipe, fitting, valve, pump and node data by group

and are sorted by element or node number. The letter "B" is placed behind the

pipe, fitting or valve numbers if the element is in a branch line.  The node

demands and heads that had a "?" entered in their fields will display "tbs"

meaning "to be solved".  If the system is balanced, then the element data

will display the flows, velocities and head losses.  Node data that was "tbs"

will now be hi-lighted to differentiate results from initially fixed data.

DETAIL

The REPORT - DETAIL option provides integrated element data listed by element

number.  All pipe, valve, fitting and pump data is provided together.  The

node data portion of this report is the same as in the REPORT - SUMMARY option.

If the current system is balanced, the velocities, flows and head losses will

be displayed.

CRITICAL

The REPORT - CRITICAL option provides a list of the 5 highest and 5 lowest

pressure nodes in the current balanced system.  Also, if any low suction

heads for pumps exist, they are displayed.  The suction head plus the

atmospheric pressure is a close approximation for the NPSHA (net positive

suction head available) and can identify potential cavitation problems.

LOOPS

The REPORT - LOOPS option lists all loops and branches in the current system

using the  node - element - node - element - node - element - node  format.

HGL

The REPORT - HGL option provides graphics of a sequences hydraulic grade and

elevation line.  The Y-axis is the head and elevation in feet (or meters).

The X-axis is the distance along the loop or branch.  Thus non-length elements

such as fittings, valves and pumps are displayed as part of a vertical line

that represent their head losses or gains.  Major pipe numbers are displayed

just above the X-axis.

UTILITIES

The DIAMETER option allows the user to change ALL pipe, fitting and valve

diameters to a specified value.  The FRICTION option allows the user to

change ALL of the current systems Hazen-William pipe roughness coefficients

or Darcy-Wiesbach specific roughnesses.  The ERASE option clears HYDRONET

memory of the current systems data.

ANALYZE

The ANALYZE sub-option analyzes the current system for data inconsistencies.

If HYDRONET is unable to plot a graph, balance a system or outputs strange

results, ANALYZE may find the problem.  The warnings that this option provides

are strictly informational.  ANALYZE should be run prior to balancing a

system so that processing time is not wasted.

The BALANCE sub-option solves the system for its steady-state flows and

pressures.  Each iterations total flow errors are added and displayed on the

screen.  The user can press escape and abort the process if the total

adjustments to flows does not tend to converge toward zero or if the set flow

tolerance might be too small to allow a solution within a reasonable amount of

time.  In this case, reset the flow tolerance and rerun BALANCE.

QUIT

This option is used to exit HYDRONET and return to DOS.

DESIGN METHOD

1.  Label all nodes with sequential numbers and note those with "fixed heads".

Nodes exist before and after any element (pipe, fittings, valves, fittings &

pumps).

2.  Label all elements with sequential numbers.  Obtain pump curve data, pipe

roughnesses, valve types and other needed information.

3.  Create loops making sure all "fixed head" nodes are connected to at least

one other "fixed head" node (w/dotted lines, graphically) forming

pseudo-loops.  Number all loops.  Identify fixed outflow branch lines that

are not to be a part of the loop system.

4.  Enter the STATUS option and set the head loss equation, units, fluid

parameters and flow tolerance.  If these conditions are going to be used for

most designs, you may wish to place them in the default parameter file for

automatic start-up.

5.  Use the DEFINE option to enter the system data.  Enter any branches using

the DEFINE - BRANCHES option.

6.  When data is being entered, use the  FILE - SAVE option regularly to save

your work.

7.  Run the ANALYZE option to identify any incomplete or inaccurate data that

might cause problems.

8.  Run BALANCE to obtain a solution for the system.

9.  Use the REPORT - CRITICAL option to obtain a summary of highest and lowest

pressure nodes and possible pump problems and observe the graphs of hydraulic

grade lines (REPORT - HGL).  Use the REPORT - DETAIL option to observe

velocities in pipes and excessive head losses.  Go back to step 5 and using F9

and F10, edit node and element data.

SAMPLE PROBLEMS

Call (916) 582-1525 or write for furthur documentation and diagrams of these

example problems.

To review the sample problem data, use the  FILE - RETRIEVE  option to

retrieve the data, then use the  DEFINE  and  REPORT option to display and

edit the data.

SAMPLE PROBLEM 1

This problem demonstrates the modeling of water towers, wells and check valves

in designing a fire protection system.  The current simulation has nodes 10

and 12 providing a flow of 500 gpm at approximately 40 psi.  It is desired

that the pump (element #2) is to provide flow to the delivery nodes and still

be slightly filling the tank at the same time.  The foot valve (element #1)

maintains the pumps prime and prevents the network from draining.

Entering the SETUP option, we set the Hazen-Williams equation for use by

pressing RETURN until we arrive at the HEAD CALCULATION EQUATION field then

press right or left arrow so that "Hazen-Williams" is displayed.  Press RETURN

to accept this and go on to set the desired units in the same manner.  At the

end of the data fields we respond "Y" to setting these new parameter values.

We enter the DEFINE option and specify LOOPS.  The first loop will be a

pseudo-loop starting with "fixed head" node #1.  Enter the groundwater surface

elevation for the node and enter a "?" for the demand telling HYDRONET that

this node has a "fixed head" and the flow into or out of this node is to be

determined.  The head at this node will be entered as zero since this is a

free surface at atmospheric pressure.  The next element (#1) is a foot valve

so when presented with the element menu, we press arrow down until "Valve" is

hi-lighted and then press RETURN.  The valve menu appears and we use the arrow

keys to hi-light "Other check valves" and press RETURN.  A sub-menu appears

and we select "foot valve-strainer hinge disc".  When we enter the valves

diameter, the proper resistance coefficient, K will be used.  Node #2's data

is then entered with a zero demand and a "?" entered for its head which tells

HYDRONET to find the pressure head at this node.  When the element menu

appears next, we select "Pump" and press RETURN.  Four points off the pump

curve are needed so we enter the increment of flow between the four points and

then the actual four points starting with the shutoff head.  The rest of the

data for this loop is entered ending with "fixed head" node #6.

We continue entering the remaining data in regular closed loops #2, 3 and 4.

The demands at nodes 10 and 12 will be entered as -500 gpm so that it will be

recognized that the flow is out of the nodes.  When all data is entered, enter

the ANALYZE option and select the ANALYZE sub-option to test for data

integrity.  If everything looks good, select the BALANCE option and the system

will be solved for steady-state flows and pressures.  Use the REPORT option to

obtain the results.  Additional modeling scenarios might include the pump

being on and only filling the tank to make sure that the pump operation

(choice of characteristic curve) is adequate and efficient just for that

purpose.

SAMPLE PROBLEM 2

This example problem demonstrates the use of the Darcy-Wiesbach equation to

calculate pipe head losses in an oil conveyance system, using metric units

and the equivalent length method.

The system is to convey crude oil from one vented tank to three other vented

tanks.  The fluid used is .855 (SG) crude at 120 degrees Fahrenheit with a

kinematic viscosity of 4.1 centistokes (4.1e-6 m2/sec).  The pipes are steel

with a specific roughness of .03 mm.  A pump size and pipe diameters are

desired so that oil flows into all tanks.

The equivalent length method will be used for fitting and valve losses since

the coefficients used by HYDRONET are strictly for water or fluids with

properties close to that of water at 60 degrees F.  This puts the resistance

of fittings and valves in terms of equivalent lengths of pipe.

The inlet from the tank at node #1 is represented by pipe #1.  Using the

Cameron hydraulic data book, we find the Borda Entrance on the left hand scale

and the inside diameter of the pipe on the right hand scale.  Drawing a line

between these two points gives the equivalent length of pipe to be used to

represent the friction in the entrance.

Intermediate results show that the pump at element #3 has a very low suction

head.  To remedy this situation, we would move the pump closer to the inlet

at node #1 or increase the diameter of pipe #2.  Both these actions serve to

reduce the suction dynamic head that develops when the fluid starts to flow.