=======================================================================

Stealth 24                                                   README.DOC

=======================================================================

Release Notes

-------------

This is v1.03 of the Stealth 24 Install/Utilities/Drivers disk set.

Installation Notes

------------------

Use the GO.EXE file to install DOS/CAD drivers, Windows 3.1 drivers,

and Utility files.

STLMODE should be installed for all first-time operations.  This program

controls the frequencies sent to the monitor and will allow the high

resolution and high color modes to syncronize correctly on your screen.

Installation of MicroStation drivers requires placing the Stealth 24

drivers in the DRIVERS subdirectory of the installed MicroStation

application.  For example, if MicroStation is installed on C:\USTATION

install the Stealth 24 drivers to C:\USTATION\DRIVERS then follow the

remaining instructions outlined in the Stealth 24 User's Manual.

Utilities

---------

STLMODE.EXE allows selection of text modes, video emulation modes, and

monitor mode frequencies.  Monitor timing information is saved to an

on-board EEPROM and is automatically loaded on powerup or reset.

Users with monitors not conforming to the industry preset monitors

provided can select the CUSTOM MONITOR option which allows selection

of frequencies, sync polarities, and screen centering for each video

resolution and color depth.  Please consult your monitor manual for

the exact capabilities of your monitor before selecting monitor

timing.  We strongly urge that selection of monitor timings in CUSTOM

be made for all resolutions and color depths.

Windows 3.1

-----------

The following Windows 3.1 drivers are included:

Resolution     Colors Displayed

640x480      16  256  65,536  16,777,216

800x600      16  256  65,536

1024x768     16  256

1280x1024    16

Changes made from v1.00:

*  Updated STLMODE to include more monitor presets

*  Revised all DOS/CAD drivers with updated S3 release files

*  Windows drivers revamped:

New code base established

High application performance

800x600 65,536 support added for 1MB memory

Driver Notes:

*  640x480 24-bit has not been revised since v1.00

*  No 1280x960 drivers are included; there are no plans to include them

*  16-color drivers may cause a GPF in PageMaker 4 when manipulating

a color bitmap image.  This is a limitation of the application

but will be corrected in a future driver release from Diamond.

Contacting DIAMOND

------------------

Diamond can be reached through a number of ways:

Postal Service  : Diamond Computer Systems, Inc.

1130 East Arques Avenue

Sunnyvale, CA  94086

Voice (Main)    : (408) 736-2000

Voice (TechSppt): (408) 736-2000 [M-F 6-7; Sat 8-4 Pacific]

[plus 24hr automated help]

FAX (Main)      : (408) 730-5750

FAX (Support)   : (408) 773-8000

BBS (to 2400)   : (408) 730-1100

BBS (9600+)     : (408) 524-9301

CompuServe      : GO DMNDONLINE

America Online  : GO DIAMOND

All product names listed are trademarks or copyrights of their

respective owners.

=======================================================================

Diamond Computer Systems, Inc.                                 (c) 1993

(408) 736-2000                                      All Rights Reserved

=======================================================================