Desqview 2.6x - High Speed Communications & Various Wildcat! Issues

-------------------------------------------------------------------

Table of Contents

Topic                                           Section

-----                                           -------

Introductory Info

-----------------

Preamble .......................................  1.0

Target Audience ................................  1.1

Desqview & Qemm Issues

----------------------

Desqview and High Speed Communications .........  2.0

Desqview Beyond COM1 & COM2 ....................  2.1

Desqview Factory Default Settings ..............  2.2

Desqview Related Key DOS Level Settings ........  2.3

Desqview Settings ..............................  2.4

Desqview & Shared Interrupts ...................  2.5

Desqview & Port Addressess .....................  2.6

Desqview Windows using Serial Ports ............  2.7

High Speed Communications

-------------------------

FIFO Transmit and Receive Buffers ..............  3.0

16450 vs 16550A UARTS ..........................  4.0

Hayes Products

--------------

Hayes ESP Card .................................  5.0

Desqview & Hayes ESP Card ......................  5.1

DTE Issues

----------

DTE Locked Baud Rates Selections................  6.0

Modem Profiles NRAM ............................  7.0

Wildcat Setup

-------------

Wildcat Multi-Line Setup .......................  8.0

Wildcat Multi-Line Setup Batch Files ...........  8.1

Wildcat .MDM File Settings .....................  8.2

File Transfer Problems & Wildcat!

---------------------------------

Wildcat Upload and Download Failures............  9.0

Poor Quality Phone Lines........................  9.1

File Transfer Failure Hardware Factors..........  9.2

File Transfer Failure Software Factors..........  9.3

BIOS Problems with 16550 UART...................  9.4

Non-Standard DOS................................  9.5

End User Terminal Emulation.....................  9.6

Summary

-------

Summary ........................................ 10.0

Contacting the Author .......................... 11.0

-------------------------- oooOooo ----------------------------

1.0 - Preamble

--------------

This paper focuses on Desqview, Qemm, BBS Software, Hayes V.FAST Modems and

the ESP technolgy and various General WC! BBS Issues.

It should be noted from the outset that Quarterdeck Office Systems, the OEM

of Desqview, have produced some very helpful .TEC documents which can be

found on CIS, Quarterdecks BBS or my Host BBS.

I have NO formal association with the OEM's mentioned below and enphasize

that the material that follows reflects my own personal view and

experience.

1.1 - Target Audience

---------------------

Both BBS operators and terminal emulation users will benefit from the

information in this document.

Desqview is used as the multi-tasker for the following BBS Packages:

GAP               REMOTEACCESS      TINYHOST          WWIV

MEGAHOST          ROBOBOARD/FX      TRIBBS

OPUS-CBCS         SEARCHLIGHT       ULTRABBS

PCBOARD           SPITFIRE          VIRTUAL BBS

POWERBOARD        SYNCRONET         WILDCAT

2.0 - Can Desqview Handle High Speed Serial Communications?

-----------------------------------------------------------

Yes!.  Despite the fact that the Desqview manual only mentions 2400 to 9600

bps and implies that 9600 bps is HIGH SPEED, Desqview is able to handle the

new V.FAST Class modems at full throughput.

2.1 - Can Desqview Address more than COM1 and COM2?

---------------------------------------------------

Yes!.  The Desqview documentation mentions COM1 & COM2 but omits to mentions

COM3 onwards.  Nevertheless any Desqview window can use the full range of

COM1 to COM8.  Desqview also indirectly supports FOSSIL drivers.

2.2 - Are the Desqview 'Factory Default' settings suitable for High Speed Links?

--------------------------------------------------------------------------------

No!.  A lot of people have literally given up on Desqview because they do

not have the patience to configure their DOS/Desqview environment to work

with programs that use Serial Ports.

2.3 - What are the Key Settings  -  DOS Level

---------------------------------------------

Firstly, I stronly recommend you use QEMM and not the DOS HIMEM.SYS and

EMM386.EXE drivers.

A typical DOS/QEMM CONFIG.SYS file might look like this in a STANDARD UART

environment with 1 to 4 Modem Lines.

CONFIG.SYS

----------

DEVICE    = C:\QEMM\DOSDATA.SYS

(1) DEVICE    = C:\QEMM\QEMM386.SYS R:2 RAM

(2) DEVICE    = C:\QEMM\DOS-UP.SYS @C:\QEMM\DOS-UP.DAT

DOS       = HIGH

FILES     = 80

(3) BUFFERS   = 30

(4) STACKS    = 0,0

COUNTRY   = 061,437 C:\DOS\COUNTRY.SYS

LASTDRIVE = H

(5) SHELL     = C:\QEMM\LOADHI.COM /R:2 \DOS\COMMAND.COM C:\DOS\ /E:2048 /p

A typical DOS/QEMM AUTOEXEC.BAT file might look like this in a STANDARD UART

environment with 1 to 4 Modem Lines.

AUTOEXEC.BAT

------------

@ECHO OFF

PROMPT=$P$G

(6) PATH=D:\;C:\WC4;\DV;C:\;\DOS;\NU;\QEMM;

\NU\IMAGE C:

CLS

(7) SHARE /F:5120 /L:80

(8) \ESP\ESPCA P280:1 R280:+ P288:1 R288:+ F

(9) \NU\NCACHE2 C: /EXP=4120 /BLOCK=8192 /QUICK=OFF /-I /WRITE=0

Notes:

(1)  The QEMM386.SYS parameters are explained clearly in the QEMM Guide so

I will not touch on them here.

(2)  A handy HINT is to backup all of the QEMM semi-dynamic data files to

avoid lenghty crash recover delays.

(4)  A lot of programs insert a STACKS 9,256 statement in your CONFIG.SYS

file this is of NO value to Desqview which automatically uallocates stack

space when requested by programs.

(5)  A number of Communications programs use DOS environment space for SET

commands such as  SET IRQID=  if insufficent environment space is

available you will loose these commands from the DOS environment table

which may stop your programs from working.  So make sure set aside

extra environment space if using lots of DOS variables

(6)  Program peformance is can be significantly improved if you optimise

your PATH= statement.  Frequently use programs should reside on

directory's closest to the = sign.

(7)  DO NOT load SHARE.EXE High or in your CONFIG.SYS file as an INSTALL

= command.  SHARE will not function correctly in HIGH memory.

(8)  If you using the HAYES ESP card, then configure the card on a single

line, for some reason invoking the program twice on some PC's causes

lock-ups.  (See the ESP Section for More Info)

(9)  Many users have reported problems with file transfers when using the

SMARTDRV program.  If you ensist on using SMARTDRV then use

SMARTDRV /X

to TURN OFF write behind caching.  I have found the Nortons NCACHE2

program to be a better DISK cache for use in the COMMS environment.

You may be able to get away with usingd delayed writes using NCACHE2.

If so the command line would be something like this:

\NU\NCACHE2 C: /EXP=4120 /BLOCK=8192 /MULTI=ON /WRITE=24

The main thing to remember about cache programs is that they can be

guilty of grabing software interrupts at just the WRONG time.

A typical DOS/QEMM CONFIG.SYS file might look like this in a DIGIBOARD or

FOSSIL driver environment with 1 to 8 Modem Lines.

DEVICE     = C:\QEMM\DOSDATA.SYS

(10) DEVICE     = C:\QEMM\QEMM386.SYS X=D000-DFFF RAM ST:M R:2

DEVICE     = C:\QEMM\DOS-UP.SYS @C:\QEMM\DOS-UP.DAT

DOS        = HIGH

FILES      = 200

BUFFERS    = 8

STACKS     = 0,0

COUNTRY    = 061,437 C:\DOS\COUNTRY.SYS

SHELL      = C:\QEMM\LOADHI.COM /R:3 /RES=4800 /SQT=D000-DAFF C:\COMMAND.COM /E:2048 /P

LASTDRIVE  = Z

(11) DEVICE     = C:\DIGI\DOS\XIDOS5.SYS

Notes:

(10) In the DigiBoard environment its is important to EXCLUDE the memory being

used by the Digiboards XIDOS5.SYS driver.

(11) Take Note that the Digiboard driver is Loaded LOW not high.  You will

experience problems if you load the driver HIGH.

2.4 - Desqview Settings

-----------------------

The most important DESQVIEW SETUP window is "P" for PERFORMANCE.

-------------------------------------------

|  Task Processing Time (in Clock Ticks)  |

|                                         |

|          Foreground: 2                  |

|          Background: 2                  |

|                                         |

|   Memory Useage (in K)                  |

|                                         |

|          Common Memory:      20         |

|          DOS Buffer for EMS:  5         |

|                                         |

|   Optimise Communications:   (Y/N)  Y   |

|   Manage printer contention: (Y/N)  N   |

|                                         |

-------------------------------------------

Processing:

It is important for communication programs to get access to

CPU on a frequent basis, therefore the default DV setting of

Foreground ticks:9 and Background ticks:3 is inappropriate.  I would

suggest at a 2 to 2 ratio is a good starting point.  The table below may

help to determine what is best for you:?

CPU     Foreground      Background

----------------------------------

80386       3               3

80486       2               2

Pentium     1               1

Clock tick settings are NOT set in stone.  I know for a fact that 18:18

will work on a Pentium without dropping carriers.

Optimize Communications:

This question asks for a YES or NO ANSWER, but in reality will accept a numeric

answer.  Optimize Commnications deals with IRQ priority's and has nothing to

do with COM port numbers.

If you answer "Y" to this question then DV assigns a prioriy sequence of

IRQ 2 3 4 5 6 7 8 9 10 11 12 13 14 15 0 1

unless a mouse is detected at IRQ 2 then the prority would be

IRQ 3 4 5 6 7 8 9 10 11 12 13 14 15 0 1 2

If you are only using COM port "1" then you should answer this question with

a "4" this will yield the following IRQ priority list:

IRQ 4 5 6 7 8 9 10 11 12 13 14 15 0 1 2 3

The main thing to note here is that if you were to then start using COM2 or

decide to only use COM2 then you would be in big trouble, because COM2 will

now be down the bottom of the priority list.

For BBS operators using COM1 and COM2 at standard IRQ's I suggest you set

Optimize Comminications (Y/M): to "3", ensuring that COM1 and COM2 both get

High priority service.

Non-Standard IRQ's:

You can optimize Non-standard COM port IRQ's in the exact same way as the

above example.  If you have move COM port 1 to IRQ 5 then enter 5 at the

(Y/N): prompt.

If you are using Non-Standard IRQ's you may find the table below helpful

IRQ     Typical Owners

--------------------------------------------------

0     Timer

1     Keyboard

2     Mouse

3     COM2       Numerous Add-On Boards

4     COM1

5     LPT2       Network Cards

6     Floppy Diskette Drive

7     LPT1

8     Realtime clock

9     Reserved

10     Reserved    CD-ROM Cards    Network Cards

11     Reserved

12     Reserved    AUX Pointing Device

13     Coprocessor

14     Fixed Disk Drive

15     Reserved

Suggested Non-Standard IRQ's:

I would suggest using 5 9 11 or 12

2.5 - Can you Share Interrupts Under Desqview?

----------------------------------------------

The answer is a Strong 'NO' for Serial Communications.

A number of multiport cards come configure as per the table below:

COM Port   IRQ     Address

--------------------------

1       4        3F8

2       3        2F8

3       4        3E8

4       3        2E8

From the table you will notice that COM1 & 3 and COM2 & 4 share interrupts,

this will NOT work under Desqview.  If you try to run all four ports sharing

IRQ's at least two of the Modems will respond at startup with:

"Modem Not Responding"  or "Did Not Receive OK"

or a similar message indicating that your modem isn't responding.

Each COM port needs a unique IRQ, which you can most likely be derived from

the earlier typical IRQ table.

2.6 - Desqview and Port Addressess

----------------------------------

Port addresses are of relevance to most communication programs, if you are

forced to use Non-Standard addresses then this table may help:

COM1    COM2    COM3    COM4

----------------------------

2E0     2E8     2F0     2F8

2C0     2C8     2D0     2D8

280     288     290     298

200     208     210     218

120     128     130     138

100     108     110     118

2.7 - Configuring a Desqview Window to use Serial Ports

-------------------------------------------------------

When you ADD a program to Desqview you are asked to complete STANDARD

information and are given the opportunity to customize ADVANCED settings.

Typical setup screens are as follows:

------------------------------------------------------------------------------

Standard Options

Program Name............: Serial Communications

Keys to Use on Open Menu: C1              Memory Size (in K): 460

Program...: COMMS1.BAT

Parameters:

Directory.: C:\COMMS

Options:

Writes text directly to screen.......: [Y]

Displays graphics information........: [N]  (1)

Virtualize text/graphics (Y,N,T).....: [T]  (1)

Uses serial ports (Y,N,1,2)..........: [1]  (2)

Requires floppy diskette.............: [N]

------------------------------------------------------------------------------

Notes:

(1) Both Items marked with a (1) can be important in the area of

memory management.  If you having problems with a program running

under Desqview that does not use graphics, then leaving graphics set OFF

and Virtualize set to T for TEXT can be a good fault finding technique.

(2) Uses Serial Ports is a VERY IMPORTANT question. See the table below

for the best settings:

Port    Set to

--------------

COM1      1

COM2      2

COM3      Y

COM4      Y

-----------------------------------------------------------------------------

Advanced Options

System Memory (in K).......:   0     Maximum Program Memory Size (in K):

Script Buffer Size.......:  1000     Maximum EMS/XMS/VCPI/DPMI (in K):

Text Pages: 4               Graphics Pages: 0         Initial Video Mode:

Window Position:                                                            �

Maximum Height:  25       Starting Height:  25      Starting Row...:   0

Maximum Width.:  80       Starting Width.:  80      Starting Column:   0

Shared Program

Pathname..:

Data......:

*Close on exit (Y,N,blank)......: [N]  Uses its own colors.............: [Y]

Allow Close Window command.....: [Y] *Runs in background (Y,N,blank)..: [Y]

+Uses math coprocessor..........: [N] =Keyboard conflict (0-F).........: [0]

Share CPU when foreground......: [Y]  Share EGA when foreground/zoomed: [Y]

*Can be swapped out (Y,N,blank).: [N]  Protection level (0-3)..........: [0]

-----------------------------------------------------------------------------

* Close on exit:  NO  - If set to yes when running a BBS the corresponding

node will be terminated

* Can be swapped out: NO  - if set to yes a BBS node could ne swapped to disk

and therefore terminated

* Runs in Background:  NO - Multiple BBS Nodes run in Background sharing the

CPU with the Foreground

+ Its ok! to set uses Math coprocesser to ON.  Some Quick Basic and Turbo

Pascal programs require it.

= Explaining Keyboard conflict does beyond the scope of this document,

however it should be noted that a setting of 8 is know to help fix

ill behaved 3270 emulation sessions

3.0 - FIFO Transmit and Receive Trigger Levels

----------------------------------------------

This is relevant for WILDCAT! 4.x and above

If your communications program allows you to customize FIFO transmit

and receive trigger level settings, then you should take advantage of these

facilities:

For Desqview the optimum settings are:

FIFO Receive Trigger Level:  4

FIFO Transmit Trigger Level: 1

These settings are designed to minimise processor activity which will in

turn improve Desqview performance.  If you experience difficulties

with file transfers at 28800 bps you may need to set the Receive Trigger

Level to 1.

4.0 - 16450 vs 16550A UARTS

---------------------------

Most users are now aware of the issues surronding the use of the older 16450

UART's.  The superior buffering of the 16550A goes a long way towards solving

most COM Port overruns.  If your computer does not support replacement UART's

i.e. the UARTS are permantely soldered in place, then perhaps a new serial

interface card may be the best option.

5.0 - Hayes ESP Card

--------------------

I believe the Hayes Communications Accelerator card is a sound product.

However, I have lost count of the number of messages I have seen on the

CIS GO Hayforum threatening to do unspeakable things to the card.  In

my opinion most of the problems are a result of users being far too

impatient.

5.1 - Can you use the ESP Card with Desqview?

---------------------------------------------

Desqview will quite happily support two dual-port ESP cards configured

as COM1-COM4.

My suggest card configuration settings are:

Card 1          Card 2

----------------------

COM1    280

COM2    288

COM3            180

COM4            188

On the diskette supplied with the ESP card is a program called ESPCA

which is the DOS configuration program.  The following ESPCA.EXE command

will configure all ports with:  Port Multiplier: x1,  RTS/CTS flow control

and all ports FIFO enabled:

ESPCA P280:1 P288:1 P180:1 P188:1 R280:+ R288:+ R180:+ R188:+ F

You can ofcourse experiment with various port multipliers.

6.0 - DTE Locked Baud Rates

---------------------------

Locked DTE refers to the baud rate between the HOST PC running the BBS

software and your BBS Modems, that is the (D)ata (T)erminal (E)quipmemt.

The table belows indicates the DTE rates proven to work reliably and those

worth experiementing with depending on how adventureous you are:

Maximum Modem       Proven DTE      Possible DTE

Carrier Speed         Rate             Rate

----------------------------------------------------

2400               2400             2400

9600               9600             19.2

14.4               19.2             38.4

28.8               38.4             57.6, 115.2

7.0 - Modem Profile

-------------------

Accessible via the AT&V command in TERMINAL Mode

The active modem profile is Important.  A typical Hayes Optima 28.8 profile

might look like this:

ACTIVE PROFILE: B0 B76 E0 L2 M1 N1 P Q0 V1 W0 X4 Y0 &A0 &C1 &D2 &G0 &K3 &Q9 &R0

&S0 &T4 &U0 &X0 &Y0     S00:000 S02:043 S03:013 S04:010 S05:008 S06:004 S07:050

S08:004 S09:006 S10:014 S11:090 S12:050 S25:005 S26:001 S36:007 S37:000 S38:020

S46:002 S48:007 S49:064 S50:250 S108:001 S109:4094 S110:002

The Key Settings are:

E0      - Don't ECHO echo characters from the keyboard in command

state

Q0      - Return RESULT CODES

V1      - Display RESULT CODES in words

&C1     - Track presence of CARRIER DETECT signal

&D2     - Monitor DTR

&K3     - Enable RTS/CTS Flow control

&Q9     - Communicate in V.42bis/MNP2-4 error control

S0      - Select RING to auto answer ON

8.0 - Wildcat! Multi-Line Setup?

--------------------------------

The Wildcat! Sysop Guide covers MULTI-LINE operation in some detail so I

don't want to waste too much time on this subject.

There are two key issues:

1. One Node = One DV! Window = One Modem = One Telephone Line

2. Node 0 is the SYSOP Node in Version 4.x not NODE 1

8.1 - Multi-Line Setup Batch Files

----------------------------------

Each DV! window should be linked to a unique CATx.BAT File. i.e. CAT0.BAT,

CAT1.BAT, CAT2.BAT and so on......

Sample Batch Files

CAT0.BAT        CAT1.BAT        CAT2.BAT        CAT3.BAT

@ECHO OFF       @ECHO OFF       @ECHO OFF       @ECHO OFF

:RELOAD         :RELOAD         :RELOAD         :RELOAD

CD\WC4          CD\WC4          CD\WC4          CD\WC4

SET WCNODEID=0  SET WCNODEID=1  SET WCNODEID=2  SET WCNODEID=3

SET WCPORTID=0  SET WCPORTID=1  SET WCPORTID=2  SET WCPORTID=3

WILDCAT         WILDCAT         SET WCMDM=HAYES SET WCIRQ=11

GOTO RELOAD     GOTO RELOAD     WILDCAT         SET WCMDM=USROB1

GOTO RELOAD     WILDCAT

GOTO RELOAD

CAT0.BAT

--------

Note:   WCPORTID=0 - Tells Wildcat! to leave the Modem port alone

WCNODEID=0 - Tells Wildcat! that its the SYSOP Node

CAT1.BAT

--------

Note:   WCPORTID=1 - Tells Wildcat! to use Serial or Digiport 1 (COM1)

WCNODEID=1 - Tells Wildcat! to set the Node Number to 1

CAT2.BAT

--------

Note:   WCPORTID=2 - Tells Wildcat! to use Serial or Digiport 2 (COM2)

WCNODEID=2 - Tells Wildcat! to set the Node Number to 2

WCMDM=HAYES - Tells Wildcat! to look in the \MODEM subdir for

a .MDM File with the name HAYES which has been customize for

the modem being used.

CAT3.BAT

--------

Note:   WCPORTID=3 - Tells Wildcat! to use Serial or Digiport 3 (COM3)

WCNODEID=3 - Tells Wildcat! to set the Node Number to 3

WCMDM=USROB1 - Tells Wildcat! to look in the \MODEM subdir for

a .MDM File with the name USROB1 which has been customize for

the modem being used.

SET WCIRQ=11 - Tells Wildcat! that the default share interupt 4

is NOT to be used (remember we cannot share Interupts) instead

we are using IRQ 11 in this example.

Use the example above to configure your system to your needs.  Try Not to

use non stardard BASE I/O Addressess

8.2 - Wildcat .MDM Files

------------------------

You can modify the .MDM modem profile files that come with Wildcat!

using the MSI supplied WCMODEM program.

A typical .MDM File Looks like this:

������������������������������������������������������������������������������Ŀ

�    wcMODEM Copyright (c) 1991,94 Mustang Software, Inc.  Version 4.01  MP    �

����������������������������� View/Edit MDM Files ��������������������������ͻ��

��  Modem name                              : Hayes Optima 28.8              #��

��  Type of serial port                     : Serial port                    ���

��  Communication port number               : 1                              ���

��  Communication port IRQ                  : 4                              ���

��  Communication port base address         : $03F8                          ���

��  Determine ringing using                 : Ring Detect                    ���

��  Determine baud rate using               : Result Code 1                  ���

��  Initialize port at what baud rate       : 38400                          ���

��  Lock DTE at initialization baud rate    : Y                              ���

��  Use CTS/RTS flow control                : Y                              ���

��  Take modem off hook on exit (busy line) : N                              ���

��  Number of seconds to wait for carrier   : 60                             ���

��  Milliseconds before answering           : 1000                           ���

��  Milliseconds to lower DTR for hangup    : 2000                           ���

��  Milliseconds to delay before prelog     : 1000                           ���

��  Milliseconds to wait for result code    : 3800                           #��

����������������������������������������������������������������������������Ķ��

��   F2 Pick             F3 Load             F4 Save              F10 Done   ���

����������������������������������������������������������������������������ͼ��

��������������������������������������������������������������������������������

��������������������������������������������������������������������������������

Port is CLOSED � Port #: 1  � Base Address: 03F8 � Port Irq: 4  � Speed: 9600

������������������������������������������������������������������������������Ŀ

�    wcMODEM Copyright (c) 1991,94 Mustang Software, Inc.  Version 4.01  MP    �

����������������������������� View/Edit MDM Files ��������������������������ͻ��

��  Milliseconds between command strings    : 2000                           #��

��  Trigger level for receive FIFO          : 4                              ���

��  Trigger level for transmit FIFO         : 1                              ���

��  Verbal code for RING                    : RING                           ���

��  Modem reset string                      : ATZ                            ���

��  Modem error correction strings          : V.42|MNP2-4|REL|ARQ            ���

��  Modem on hook string (ready for calls)  : ATH0                           ���

��  Modem off hook string (busy)            : ATH1                           ���

��  Modem initialization string             : ATE0V1&C1&D2S95=3S0=0S7=60     ���

��  Modem answer string                     : ATA                            ���

��  Modem caller ID string                  :                                ���

����������������������������  Modem Nram settings  �������������������������İ��

��  Modem reset command                     :                                ���

��  Modem setup string # 1                  :                                ���

��  Modem setup string # 2                  :                                ���

��  Modem setup string # 3                  :                                #��

����������������������������������������������������������������������������Ķ��

��   F2 Pick             F3 Load             F4 Save              F10 Done   ���

����������������������������������������������������������������������������ͼ��

��������������������������������������������������������������������������������

��������������������������������������������������������������������������������

Port is CLOSED � Port #: 1  � Base Address: 03F8 � Port Irq: 4  � Speed: 38400

*** PAGE #1 - IMPORTANT SETTINGS ***

Type of serial port                     : Serial port

Communication port number               : 1

Communication port IRQ                  : 4

Communication port base address         : $03F8

Determine ringing using                 : Ring Detect

Determine baud rate using               : Result Code 1

Initialize port at what baud rate       : 38400

Lock DTE at initialization baud rate    : Y

Use CTS/RTS flow control                : Y

******************************

port IRQ  - Should be matched on the physical port or Multi-port I/O Board

base add  - Should be matched on the physical port or Multi-port I/O Board

Determine Ringing Using:  Ring Detect:  Signal From Modem RI Circuit

Ring Result:  Uses the Modem Result Code

Auto Answer:  Modem does all the work via ATS0=0

Determine Baud Rate Using: Result Code 1:  Monitor CD while waiting for Connect String

Result Code 2:  Don't Monitor while waiting

CR:  Use the old style PRESS CR a few times method

DTE  Assume calls is at Locked DTE Rate

*** PAGE #2 - IMPORTANT SETTINGS ***

Trigger level for receive FIFO          : 4

Trigger level for transmit FIFO         : 1

Verbal code for RING                    : RING

Modem reset string                      : ATZ

Modem error correction strings          : V.42|MNP2-4|REL|ARQ

Modem on hook string (ready for calls)  : ATH0

Modem off hook string (busy)            : ATH1

Modem initialization string             : ATE0V1&C1&D2S95=3S0=0S7=60

****************************

FIFO Trigger Levels:            See Section 3.0

Modem Error Correction Strings: Match these with your modem manual

On & OFF Strings:               These are correct for the Hayes 28.8.

v4.00 of Wildcat! stated them incurrectly

Modem Init String:              See Section 7.0

9.0 - Wildcat! Upload and Download Failures

-------------------------------------------

The are alot of users still asking about file transfer failures which manifest

themselves in two main ways:

1.  Excessive CRC errors

2.  Link seems to run ok! for around 120K or so then dies dropping back

and eventually losing carrier.

I have experienced both!

9.1 - Poor Quality Phone Lines

------------------------------

There is NO doubt that a poor quality phone line can cause the two problems

nentioned above to occur.  Ask you local telephone company to check the lines

for you.

Assuming that your problems are being caused by line noise or interference you

might like to get hold of a Noise testing program and run it on your BBS and see

what degree of noise the program picks up.

For a detailed look at Telephone Line Noise See the LNOISHLP.DOC File

9.2 - File Transfer Failure Hardware Factors

--------------------------------------------

Always check the obvious:

1.  Check your installation by using a different brand of modem

2.  Check all phsyical connections.

3.  Check all environmental facts, such as heat, cold or dirty power

supply.

9.3 - File Transfer Failure Software Factors

--------------------------------------------

If you are certain that your Telephone Lines are Fine and that your modem is

fuctioning correctly then the port of last resort is software.

Here are some things to try:

1.  Carefully review your modems NRAM settings for data compression, if

in doubt use AT&F to restore your modem to its factory ready state.

2.  Carefully review your modems NRAM settings for error correction, if

in doubt use AT&F to restore your modem to its factory ready state.

3.  Don't be frightended to play with items 1 or 2.

4.  Remove any vsheild style programs from your boot up files

5.  In MAKEWILD under Memory and Hardware Information set the Following:

Size of Scollback Buffer (in K):       5

Where Should Overlay be Held:          Disk

Extra Memory in K for overlay Buffer:  0

Where should swapped data be held:     EMS

6.  If your PC uses SHADOW memory of any type disable it in CMOS some

users have claimed it can cause problems.

7.  Discontinue the use of Desqview, Qemm, DOS Share, EMM386, Smartdrv

and run Wildcat as a SINGLE LINE DOS System.

Use STARTUP batch files similar to these:

AUTOEXEC.BAT

------------

@ECHO OFF

PROMPT=$P$G

PATH=C:\;\DOS;\WILDCAT;

SET WCPORTID=1

SET WCNODEID=1

CD\WILDCAT

WILDCAT

CONFIG.SYS

----------

DEVICE    = C:\DOS\HIMEM.SYS /TESTMEM:OFF

DOS       = HIGH

FILES     = 80

BUFFERS   = 30

SWITCHES  = /F

NUMLOCK   = OFF

SHELL     = \DOS\COMMAND.COM C:\DOS\ /E:2048 /p

Please Note: /TESTMEM:OFF and SWITCHES= /F are relevant to DOS 6.x ONLY!

============

9.4 - BIOS Problems with 16550 UART

-----------------------------------

Some Sysops have tracked their High Speed Comms problems to incompatibilities

between their PC's BIOS and the 16550 series UART Chips.  In particular users

of AMI BIOS dated 1991, have reported various problems.

This is not an area, I wish to get into in any detail, instead can I suggest

that you might like to check with the OEM of your BIOS to see whether or

not there are any reported problems with serial communications.

9.5 - Non-Standard DOS

----------------------

It is always best to start the troubleshooting process from a 'Known' position.

If you choose to use a version of DOS which has not been tested as a platform

for Desqview, then you except the risk of living on the bleeding edge of

technology.

If you are using IBM DOS or Compaq DOS and so on.., then it may be wise to

boot your PC from a MS-DOS 6.x system diskette and see whether or not your

system works properly under defacto 'Standard DOS'.

During the floppy disk based testing process you might like to put a copy

of the MS-DOS COMMAND.COM file in a directory on your hard disk and use

the COMSPEC command to redirect COMMAND loading from the floppy disk

to the vastly superior speed of your hard disk.  The Syntax would be

SET COMSPEC=C:\dirname\command.com

where dirname is a directory name of your choice.

9.6 - End User Terminal Emulation

---------------------------------

Many WC! Sysops like myself encourage users to to use QMODEM Testdrive to

access our WC! BBS's.  From time to time you will find users who experience

significant CRC errors when using Qmodem.  The most common reason for this

is incorrect Toggle Settings under MODEM in the QINSTALL program.

The most stable MODEM--->TOGGLES settings in QMODEM for High Speed Comms

are:

XON/XOFF:              ON *

CTS/RTS FLOW:          ON *

RTS when writing:      OFF

FIFO During Shell:     OFF

Locked DTE Baud Rate:  ON

High Speed XMIT:       ON

16550 UART Enable:     ON

Init Ignores CD:       OFF

Note: * Normal one or the other, however some External Protocols i.e. Hyperp

require both on to work.

Whilst on the subject of QMODEM, if a user asks 'why their screens colors

don't look right, i.e. your menus etc seem to have mixed colors, its

because under VIDEO--->TOGGLES the user does NOT have

STRICT COLOR:   ON

set to ON.

10.0 - Summary

--------------

This file will be added to on an ongoing basis.  WATCH for updates.

11.0 - Contacting Me

--------------------

I am happy to answer any questions that you may have. I carry MSI SupportNet

so a message left in any of the [E] conferences will reach me.

[SEE THE MYBBS.DOC FILE]

Michael Purdy

(Sysop) The Speed Limit BBS

Melbourne, Australia

Internet:100026.1177@compuserve.com