Telephone Line Noise Help

-------------------------

Preamble

--------

This file looks at possible sources of Telephone Line Noise and was written

by   PATRICK MOORE  the Sysop of the Supra BBS.

This document is a combination of data from engineers at Supra, a telephone

call with an engineer who works for AT&T, electronic mail from other people

involved with the modem industry, and my own knowledge and education

relating to data communication and transmission lines.  Please feel free to

post this document in its original and unedited form.

BRIDGE-TAPS & HALF-TAPS

These are "extra" wires connected to the wire pair coming to your home from

the local telephone office.  Often these are the result of normal service

and repair procedures that occur over the years.  These don't normally

create a problem for VOICE use (although they have been traced to things

like radio stations or buzz-type noises in some cases).  Devices using

complex signaling tones and patterns, however, can be greatly affected by

these "extra" signals on the phone line.  Theses extra wires act like

antennas and pick up noises from a wide range of sources.

LOADS (Step-Up Transformers)

These are commonly used to increase the volume of voices on longer phone

lines.  They cause distortion of the rated 300hz to 3000hz band pass, and

can cause the frequencies to be shifted up or down from their original

values.  The volume of these frequencies is also not increased equally.

In some cases, the line can become poor enough that TOUCH-TONE dialing

may not be supported, and only PULSE dialing is possible.  LOADS are most

commonly used in older, rural settings, but still do exist in urban

installations.  LOADS also serve a means of introducing high levels of

noise onto the line, both by increasing the level of noise on the line

along with the voice level and by picking up environmental noise from the

area directly around the transformer itself (other transformers, power

supplies, high voltage power lines, etc.).

LINE LOSS

This is the composite of all of the types of loss on your phone line which

include the following:

-Pure Loss

This natural loss in the phone line caused by the resistance of the wire

and connections between the local office and your home.

-Return Loss

This is a measurement of the signal loss on the line.  This is affected

by the line itself (see above) and the devices you have connected to it.

It is basically a measurement of the impedance match between the local

office and the line and the devices you have connected (problems in any

part will affect the over-all measurement).

-Frequency Distortion and Attenuation.

This is basically the loss of volume and shift in frequency on the line.

Problems often arise in high-speed data transfers because only part (or

in very poor conditions none) of the original signal gets transferred

faithfully to the receiving end.  In some cases many frequencies have been

"reflected" back on to each other, further damaging the signal.

SOURCES OF SIGNAL DAMAGE

-Quanitazation noise

Digital links have quanitazation noise which is inherent in A/D converters.

-Thermal Noise

This is naturally occurring noise sourced from normal electrical decay and

energy transformation.  Local loops, if properly engineered, have a thermal

noise of 22 dBrn.  But it is not uncommon to find it at higher levels.

(22 dBrn in "reference noise" is equal to -68 dBm.)

-Digital Compression

Compressing the signal to get a higher number of voice signals on one

carrier.  (This is just fine for voice, as the human ear does not notice

the small gaps in the signal this creates, but modems can't deal well with

gaps in the data received.)

-Power line hum

Low frequency noise conducted into the phone line as it passes power lines.

(Also it is possible for power hum to be passed into the phone line from

power sources in the phone system itself.)

-Phase Jitter

The shifting or bouncing of the carrier signal in relation to time.

-Crosstalk

Noise or sounds from other lines in the phone system.  (In very bad

conditions this can be loud enough you can hear other lines ringing and

even hear other voice or data signals.)

-RFI (Radio Frequency Interference)

High frequency electrical signals conducted into the phone lines.  (In very

bad cases radio stations or other radio transmissions can be heard.)  If you

can figure out the source of the signal (like the name of the radio station),

then you should contact them in regard to curing this problem.  In the US,

FCC regulations require the person in charge on the transmission to fix this

problem at his expense. (In the case of a radio station, contact the station

engineer.  He can normally construct a filter to remove the signal from your

phone line.)

Common Tests to ask to have done by the phone company:

-Frequency Test (also called Frequency "Slope" or Frequency "Twist" test)

This test shows both total loss in signal, and the amount of difference in

signal loss at different frequencies.  A 1mW 1Khz signal is measured for

total  loss (in dBm) and then compared to readings taken from signals at

400hz and 2800Hz.  For a VOICE line, the high and low frequency values

should be within a +3dBm to -5dBm range of the value of the center frequency

(a DATA grade line should be closer to the reading of the center value).

In conditions where this problem is quite bad, you will hear the voices as

being "tinny" and it may be more difficult to recognize voices over those

lines.

-Loss and/or Return Loss Test

This will test the loss of a signal transmitted from one end of the line

to the other.  (In the case of a Return Loss Test, it tests the loss of a

signal transmitted into the line and reflected back from the other end.

-45dBm is a reasonable level, -60dBm is not acceptable.

Common Problems, Tests, and Repairs for end users to address:

-Try the modem/fax with ALL other devices removed from the line.

This tests for two common problems:

1. NOISE -from devices like cordless phone and "neon-display" phones most

commonly, but also can be sourced by any telephone line device.

2. Signal Loss -from devices that "pull" power off the phone line.  Phones

and devices that take power from the line when not actively in

use for dialing memory, status lights, and lighted displays

(although most lighted displays are only taking power when the

handset is picked up) will lower the power level on the line,

and will also lower the signal levels being passed on the line.

-Check the condition of the phone wiring in your home.

"High Twist" or "Data Grade" phone cable, rather than standard "low twist"

or non-twisted phone cable, wire is better for high-speed data

transfer.

"Silk" cable (that flat cable that runs between the phone device and the

phone jack) should be kept short (under 8 feet).  It is better to

put in a new phone jack (preferably wired with "High Twist" cable),

then to connect the modem via a long "silk" type extension cable.

Wires should be at least 26 gauge, 22 gauge is better.

Insulation should be in good condition.

Wiring runs should be kept out of direct sunlight and weathering.  (If

wiring must be done outside, use phone wire rated for outdoor use.)

Remove wiring runs no longer in use (this does not mean to rip out any

phone outlet you aren't using, but it is a good idea to remove

wiring that has already been disabled or covered up by remodeling

-many older homes and rentals have phone lines that go nowhere!)

*WARNING* If you find equipment that you don't understand connected

to the phone lines in your home, or "extra" wiring to transformers

or grounding rods/water lines, do not disturb it without talking to

the phone company about it.

*** This document will be updated at random points in time, so please check

the Supra BBS {503-967-2444} to assure the most current information.