ISPFMT   1.1

NOTE:

THIS PROGRAM CAN DESTROY CUSTOMER DATA.  BE SURE TO HAVE ALL

VALUABLE DATA BACKED UP AND VERIFIED BEFORE CONTINUING!

FUNCTIONAL DESCRIPTION:

This is a Low-Level format utility designed to help you

maintain GROWN DEFECTS on your WESTERN DIGITAL Intelligent

Storage Peripheral.

DRIVE SELECTION:

The user must select drive 0 or 1 and the model type.

LOW-LEVEL FORMAT:

Performs a LOW-LEVEL FORMAT, pre-existing defects are

mapped out.  ESC will abort back to the main menu.  If

aborted the user will be warned that previous defects will

not be updated.

QUICK-FORMAT/SURFACE ANALYSIS:

First LOW-LEVEL format is performed then a short SURFACE

analysis writes and read_verifies the entire surface of

the disk with two critical patterns.  This function can be

useful in a production environment.  All pre-existing and

surface analysis defects will be automatically mapped out.

ESC will abort back to the main menu.  If FORMAT is aborted

the user will be warned that previous defects will not be

updated.

SURFACE ANALYSIS:

Ten critical patterns are used to write and read-verify the

entire surface of the disk.  This function is an in-depth

check for GROWN defects.  All pre_existing and surface

analysis defects will be automatically mapped out.  ESC

will cause an abort back to the main menu, defects are not

updated.  IF a previous VERIFY updated the grown map,

SURFACE updates to the map will not occur.  FORMAT or

UPDATE DEFECTS will clear this condition and allow updates

to occur.

VERIFY:

This function performs a read-verify of the entire disk.

Any new defects are presented to the user.  Upon exit of

the defects display the user is asked if the newly found

defects should be added to the defect list.  Yes will

append these defects to the GROWN defects map on the drive.

A subsequent VERIFY will not allow more defects to be marked

until FORMAT is run.  A subsequent FORMAT or UPDATE DEFECTS

will then map out these defects along with any previous

defects.  No will exit without updating the defect list.

ESC will cause an abort back to the main menu.

UPDATE DEFECTS:

This function will update all defects on the drive selected.

The factory and grown defect lists will be used to update

defects on the drive.  This will clear a previous VERIFY

update to the grown defect map and allow SURFACE ANALYSIS

and VERIFY to update the maps.

EXIT:

Any drive in CMOS mother-board setup will be re-established.

If drive 0 or drive 1 is attached and powered up it will be

PARKED.  Some systems and drives will not respond after

this operation.  The user is cautioned to WARM or COLD boot

the system to GUARANTEE proper operation.

ERROR LOG DESCRIPTION:

Drive, Model number and the Function that was executed is

shown.  Each physical location of a GROWN defect is shown

once with cylinder, head, sector, status, error, and

pattern/window information.  The status and error display

information is from the first error occurance.  The pattern

and window information are shown from left to right in order

of occurance.  Each pattern has a letter designation:

a  d6b5ad6b5a ----------------- Isolated ones WD

b  c6319c6319                   Isolated ones IBM

c  cf3cf3cf3cf3 --------------- Isolated ones WD & IBM

d  492492492492                 HI frequency IBM all ones

e  0101010101 ----------------- Isolated zero's WD

f  136b31136bb3                 Mixed all frequency WD

g  c086043021800c086043021800   WD worst case pattern

h  5ab6b5ab6b                   Max jitter

i  3333333333 ----------------- Low frequency

j  0000000000                   Hi frequency

The window information has a letter designation:

E   Early  read

L   Late   read

N   Normal read

W   WRITE  only

Four pattern/window entries for a designated sector would

be displayed:

PATTERN/WINDOW

aEaLcEcL

Patterns a and c failed during read-verify with both EARLY

and LATE window.

DRIVE DESCRIPTION:

CENTAUR:

model  93042A/95042A (20 meg)

(782 cyl, 2 heads, 27 sec PHYSICAL)

(615 cyl, 4 heads, 17 sec TRANSLATION type 2)

model  93044A/95044A (40 meg)

(782 cyl, 4 heads, 27 sec PHYSICAL)

(977 cyl, 5 heads, 17 sec TRANSLATION type 17)

SINGLE DRIVE  J8 no jumpers

MASTER  J8 5-6 jumper.

SLAVE  J8 3-4 jumper.

J8 1-2 for slave to original CONNER CP342, CP3022.

----------------------------END ISPFMT HELP--------------------------