With knowledge gained partly through advising the construction of

Olympic facilities in Atlanta, the U.S. Department of Justice has

now published a guide to design of accessible stadiums.

Jamal Mazrui

National Council on Disability

Email: 74444.1076@compuserve.com

----------

U.S. Department of Justice

Civil Rights Division

Disability Rights Section

Accessible Stadiums

The Americans with Disabilities Act (ADA) requires new stadiums

to be accessible to people with disabilities so they, their

families, and friends can enjoy equal access to entertainment,

recreation, and leisure.

This document highlights key accessibility requirements of the

ADA that apply to new stadiums.  Other accessibility

requirements, such as those for parking lots, entrances, and rest

rooms, also apply but these are the same as for other buildings.

Compliance with all the accessibility requirements is essential

to provide a basic level of access for people with disabilities.

To obtain a copy of the requirements for new stadiums and other

facilities, contact the Department of Justice ADA Information

Line at (800) 514-0301 voice or (800) 514-0383 TDD.

Key Features of Accessible Stadiums

Seating

Wheelchair accessible seating is required.  At least one

percent of the seating must be wheelchair seating locations.

Each wheelchair seating location is an open, level space that

accommodates one person using a wheelchair and has a smooth,

stable, and slip-resistant surface.

Accessible seating must be an integral part of the seating

plan so that people using wheelchairs are not isolated from other

spectators or their friends or family.

A companion seat must be provided next to each wheelchair

seating location.  The companion seat is a conventional seat that

accommodates a friend or companion.

Wheelchair seating locations must be provided in all areas

including sky boxes and specialty areas.

Removable or folding seats can be provided in wheelchair

seating locations for use by persons who do not use wheelchairs

so the facility does not lose revenue when not all wheelchair

seating locations are ticketed to persons who use wheelchairs.

Whenever more than 300 seats are provided, wheelchair

seating locations must be provided in more than one location.

This is known as dispersed seating.  Wheelchair seating locations

must be dispersed throughout all seating areas and provide a

choice of admission prices and views comparable to those for the

general public.

Wheelchair seating locations must be on an accessible route

that provides access from parking and transportation areas and

that connects to all public areas, including concessions,

restaurants, rest rooms, public telephones, and exits.

Wheelchair seating locations must provide lines of sight

comparable to those provided to other spectators.  In stadiums

where spectators can be expected to stand during the show or

event (for example, football, baseball, basketball games, or rock

concerts), all or substantially all of the wheelchair seating

locations must provide a line of sight over standing spectators.

A comparable line of sight, as illustrated in the figure below,

allows a person using a wheelchair to see the playing surface

between the heads and over the shoulders of the persons standing

in the row immediately in front and over the heads of the persons

standing two rows in front.

Line drawing showing a side view of people seated in

assembly-type seating and a person using a wheelchair seated in a

wheelchair seating location behind the spectators.  A dashed line

illustrates that line of sight for the spectator using the

wheelchair and other lines show the line of sight for seated and

standing spectators.  All spectators can see between the heads of

the person in the row directly in front of them and over the

heads of the people two rows in front.

Figure Showing Comparable Line of Sight for Wheelchair Seating

Location

In addition to wheelchair seating locations, at least one

percent of all fixed seats in all seating areas must be aisle

seats with no armrest, or with a removable or folding armrest, on

the aisle side.  These seats accommodate people who have a

mobility disability but who wish to use a seat that is not a

wheelchair seating location.

An accessible route must connect the wheelchair seating

locations with the stage(s), performing areas, arena or stadium

floor, dressing or locker rooms, and other spaces used by

performers.

Concessions

All concessions, including food service areas, restaurants,

and souvenir stands, must be accessible.  For example, lowered

counters must be provided where goods are provided and where cash

registers are located.  Condiments and self-serve food items must

be provided within reach of a person using a wheelchair.

Access to playing fields, lockers, and spaces used by players and

performers

An accessible route must provide access to all public and

common use areas including the playing field, locker rooms,

dugouts, stages, swimming pools, and warm-up areas.  The

accessible route provides access for the public, employees, and

athletes using the facility.

Assistive Listening Systems

When audible communications are integral to the use of a stadium,

assistive listening systems are required for people who are hard

of hearing.  These systems amplify sound and deliver it to a

special receiver that is worn by the spectator, or to the

spectator_s hearing aid, depending on the type of system that is

used.

The stadium must provide receivers for the assistive

listening system.  The number of available receivers must equal

four percent of the total number of seats.

Signs must be provided to notify spectators of the

availability of receivers for the assistive listening system.

Other Accessible Features

Accessible Parking Spaces

When parking spaces are provided, accessible parking spaces

for cars and accessible parking spaces for vans are required.

Accessible parking spaces must be the closest parking spaces to

the accessible entrances and must be on an accessible route to

the entrances.

Accessible Drop-Off and Pick-Up Areas

If passenger drop-off areas are provided, they must be

accessible and an accessible route must connect each accessible

drop-off area with the accessible entrance(s).  Curb ramps must

be provided if the drop-off area is next to a curb.

Accessible Entrances

At least fifty percent of the entrances must be accessible.

Those that are not accessible must have signs that direct the

public to the nearest accessible entrance.

Accessible entrances that have turnstiles must provide an

accessible gate or door.

Rest Rooms

Each public and common use (including employee) rest room

must be accessible.  This includes rest rooms in work areas and

rest rooms located in sky boxes and suites.

Public Telephones

Each bank of public telephones must have one or more

wheelchair accessible telephones and these and other public

telephones must have the ability to amplify the volume at the

handset.  A sign must identify telephones equipped with

amplification.

At least one public TDD (telecommunications device for

persons who are deaf or who have speech impairments) must be

provided.  Signs must identify the location of the TDD and

provide direction from other telephone banks.

For each bank of public telephones with three or more units,

at least one telephone must be equipped with a shelf and

electrical outlet to permit a person to use a portable TDD.

Water Coolers or Drinking Fountains

Drinking fountains must accommodate people who use

wheelchairs and people who stand but have difficulty bending or

stooping.  Half of the units must be wheelchair accessible and

the others must accommodate standing users.

Visual Alarms

Where audible fire alarms or emergency notification is

provided, flashing lights are required in public and common use

areas, including toilet and bath rooms, locker rooms, and along

public corridors.

Signs

Signs that identify permanent rooms and spaces, such as

those identifying rest rooms, exits or room numbers, must have

Braille and raised letters or numbers so that they may be read

visually or tactually (by feeling the characters with one_s

fingers).  They must also meet specific requirements for mounting

location, color contrast, and non-glare surface.  Signs that

provide direction to, or information, about functional spaces

must only comply with requirements for character proportion,

character height, and finish and contrast between the characters

and background.

ADA Information Line

For more information about the ADA_s design and construction

requirements, contact the Department of Justice_s toll-free ADA

Information Line at 800-514-0301 (Voice) and 800- 514-0383 (TDD).

Detailed requirements can be found in the ADA Standards for

Accessible Design.  The ADA Standards and other useful technical

assistance documents are available from the ADA Information Line.