# Modem Assist PlusÒ

# Version 4

# Reviewer’s Guide

Synergy Solutions, Inc.

2150 South Country Club Drive

Suite 1

Mesa, Arizona 85210

TEL (602) 545-9797

FAX (602) 545-9827

BBS (602) 545-0232

February 27, 1995

# TABLE OF CONTENTS

1.  TECHNOLOGY BRIEFING

How the Technology is Used

Who Uses It?

Why Is It Used?

2.  PRODUCT ENHANCEMENTS

Windows Installation

Client DLL for Windows

Redirector Enhancements

Modem View

Background Queuing

INHOST for Windows

Port Status & Setup

New Screens

Faster Client & Server Initialization (IPX and NetBIOS)

Table Building Routine for IPX

3.  FEATURE AND SPECIFICATION CHART

4.  COMPETITIVE ANALYSIS

Strengths

Weaknesses

5.  REVIEW METHODOLOGY

Installation

Dialing Out with the New Client DLL

What’s happening at the Comm Server?

Background Queuing

Comm Port Control

Dialing In

6.  IDEAS FOR REVIEW

Infobahn Usage Becoming More Common in the Small Office

Using Stand-Alone Communication Services on a Network

7.  HOW MODEM ASSIST WORKS

The Basics of Modem Sharing

INT 14

NASI

Windows COM Port Redirection

The Modem Assist Communication Server

SERVER.CON Parameter File

Types of Connections

Comm Server Port Allocation

Running Windows on the Communication Server

The Modem Assist Communication Client

CLIENT.CON Parameter File

Dialing Out With the DOS Client

Dialing Out With the Windows Client (MCLIENT.DLL)

How Port Redirection Works (MACOMM.DRV)

How the Client Makes a Connection

Dialing In

Handling Incoming Calls

The INHOST Program

Managing Modem Assist Plus

8.  SUMMARY

# TECHNOLOGY BRIEFING

## How the Technology is Used

Modem Assist Plus is a software solution for creating dedicated or non-dedicated asynchronous communication servers on your network.  With Modem Assist  you can share modems or other asynchronous devices across the network.  This capability enables you to make efficient use of the hardware and phone lines you already have for both inbound and outbound communications.

Modem Assist Plus consists of two primary components, the communication Server and communication Clients.   A communication Server is a workstation, functioning in either a non-dedicated or dedicated fashion, sharing serial ports among Clients.  Communication Clients  are workstations connected to the communication Server’s ports through the network.

Historically, communication programs expected to interface to a local serial port. Since the serial ports exist at the Server, rather than the Client, different communication interfaces were developed to accommodate this situation.  Modem Assist supports the standard Interrupt 14 interface, an extended Interrupt 14 interface, the Novell NASI interface, and the Windows communications interface. Any communications program that you use with Modem Assist Plus must support one of these interfaces.

## Who Uses It?

LAN users without modems attached directly to their PCs can use the modems attached to Modem Assist communication Servers for FAX and data transmissions.  LAN users can also use Modem Assist Plus to make direct connections to mainframe computers.  Remote users can dial into the communication Server and be routed to a machine for remote access to a system.  Modem Assist Plus operates on IPX/SPX and NetBIOS (NetBEUI) networks.

## Why Is It Used?

Network modem sharing is implemented because of the cost and management economies that can be achieved.  Instead of installing twenty phone lines and twenty modems on twenty workstations, two or three modems and phone lines can be installed on a single communication Server and shared by all users on the network.  All communication equipment can be managed in one place instead of across the network.

There are also cost economies in using outdated workstations to create high performance communication servers.  By adding a multi-port serial card and a few modems to the average workstation and controlling those resources with the Modem Assist Plus software solution, expenditures on costly communication server hardware solutions can be avoided.

# PRODUCT ENHANCEMENTS

## Windows Installation

New to version 4 is a Windows installation.  Although Modem Assist Plus 3.0 had windows compatibility,  version 4 provides native Windows features which can be implemented through an easy Windows Installation. DOS installation is also included for DOS-only users.

The Windows installation also includes UN-Install capabilities to ensure restoration of all affected files (.INI, .bat) and removal of all Modem Assist Plus program files.

## Client DLL for Windows

NEW to version 4 is the Client DLL.  Previous versions of Modem Assist PLUS required the use of the MCLIENT.EXE program that could only be loaded as a TSR or Windows Virtual Device Driver (VxD).  Version 4 now supports seamless access to the Server's modems from Windows communications programs. The Client DLL is designed to be used with communications programs that use the standard Windows communications interface. You can still load the MCLIENT program if you need support for DOS communications programs or if you want to run a Windows program that uses the INT 14 or NASI interface.

## Redirector Enhancements

The Redirector for Windows allows the Client to intercept and redirect Windows communications calls to the Server. You choose which comm ports are to be redirected to the Modem Assist Server.  The Modem Assist PLUS Redirector for Windows does not replace Windows communications drivers.  Instead, if a port is not a redirected Modem Assist PLUS port, Synergy’s driver pass-through technology sends the request to the communication driver that you were using when you installed Modem Assist.  This feature allows you to continue using replacement drivers for printing and servicing local ports.

New to version 4 is the ability to assign groups and lines to individual COM ports 1 through 9.  This gives users the flexibility to assign different communication programs to different modems.  By allowing each user to have their own Modem Assist .INI file,  COM port selections can be unique for each user.

## Modem View

To make it easier to see what is happening at the communications Server, modem lights now pop-up when a connection to a modem is established.  Even as you change COM ports - Modem View will change to monitor those ports using its automatic DETECT-PORT feature.   When the communication program is closed, Modem View minimizes.

## Background Queuing

The queuing mechanism has been enhanced for Windows users by allowing the queued communications program to minimize while you continue working in Windows.  When the modem becomes available, Modem Assist pops up the minimized communications program indicating that a port is open and a connection is established.

## INHOST for Windows

Also new with Version 4 is the ability to launch a Windows communications program to service inbound calls.  The INHOST program is now provided for use in both DOS and Windows. You can use the Windows INHOST program to establish an inbound service at any Modem Assist Client.  INHOST can then be minimized to allow you to continue work on your machine while you wait for an inbound call. INHOST launches a specified communications program when an inbound call is received.

## Port Status & Setup

The Port Status & Setup window allows users to see the detailed status of Servers and the redirected COM ports on the Client.  From this window, COM port assignments can be made to specific modems or groups of modems.

Windows COM ports are assigned by dragging particular groups and lines over to the “Virtual COM Port Assignments” list and dropping them on the desired port.  LAN users can double click on a particular Windows COM port to turn off redirection of that COM port.

Also, from within the Port Status & Setup window, easy access is provided to Client configuration files for customization of communication services, and the Client Redirector can be reinstalled if needed.  Here are some of the things the Port Status & Setup window displays:

- available lines at all communication servers

- available groups (modem pools)

- COM port assignments (1-9)

- client in use (DLL or TSR)

- default inbound service name

- redirection services available (NASI or INT 14)

- buffer size allocated

- queuing status (on / off)

- call retries (on / off)

## New Screens

In version 4, all of the DOS .exe programs will be displaying new screens.  These screens have an updated look and feel in comparison to prior versions of Modem Assist Plus.

## Faster Client & Server Initialization (IPX and NetBIOS)

In version 4, the initialization time for the Client and Server has been reduced.  In version 3.0, initialization  on a NetBIOS network took 30-40 seconds - now users can expect to wait approximately 5 seconds to find communication servers.

## Table Building Routine for IPX

New to version 4, the Server and Client now build a table that contains routing information for IPX networks. The new table building routine allows for immediate loading of the CLIENT (after the initial table is built upon first use ).  This table can be edited to allow network managers to add or delete router table information and restrict access to communication servers from certain segments/networks.  This is especially helpful in a WAN environment.

# FEATURE AND SPECIFICATION CHART

NOS Compatibility			IPX/SPX

NetBIOS

NetBEUI

Operating Environments		PC or MS DOS 3.3 or greater

Windows 3.1

Comm Protocols Supported	INT 14

Modem Assist Plus Extended INT 14

NASI

Windows Communication Interface

Required RAM			Server TSR for DOS operation:  50K (can load high)

Client TSR for DOS operation:  40K (can load high)

Client DLL for Windows operation:

Required Hard Disk Space 	Approximately 2MB for all files

Lines Supported at Server	2 port

4 port

8 port

16 ports @ single server in Unlimited port version

Supports non-intelligent multi-port serial cards

Maximum Line Speed		115,200 kbps each port

Background Queuing		Windows users minimize communication application for 						automatic connection when line becomes available

Automatic Retry			DOS users get periodic retry of connection attempt when all 						lines are busy

Automatic time-out		Lines reset after defined periods of  inactivity

Modem Status lights		Modem View status indicator lights reveal activity at distant 						communication Server

Security				Password and service identifier on inbound calls

Conference Calling		Unlimited users can view a single communication session

Inbound/Outbound 		Dynamic in and outbound non-proprietary support in both 						Windows and DOS

Modem Pooling			Flexible support of service groups or specific lines

Flow Control			Supports both Software and Hardware

# COMPETITIVE ANALYSIS

## Strengths

- Transparent connections with the option to display available lines (you don’t have to bother with selecting your modem before each communication session if you don’t want to).

- Does not replace Windows COMM driver

- Easy Port Status & Setup window for port configuration and control

- Flexible pooling using multiple groups

- Dynamic INT 14, NASI Support

- Supports IPX/SPX, NetBIOS and NetBEUI networks

- Supports Data, FAX and SLIP/PPP communication packages

- Windows inbound support

- Non-dedicated modem Server

- Includes programs for comm testing (Terminal), Server management (Monitor), and Parameter editing (Paredit)

- This is the version 4 release and has a polished operation and interface

- Easy Windows installation

- Excellent documentation

## Weaknesses

- No Windows Server

In a total Windows environment (like Windows for Workgroups) there is still a requirement for a DOS based communication Server which must start NetBEUI at DOS.

- Large TSR sizes

Consider, however, that there are not different TSRs for INT 14, NASI and Windows communication interfaces.  What you buy with the larger TSR is an unmatched feature set and the flexibility to use all communication interfaces .

- Non dedicated Server must run both Server and Client TSRs

The Client DLL will not function concurrently with the Server TSR for access to local modems or other shared modems on the network.  In a Windows environment, a user sharing a modem must utilize the Client TSR to access local modems or modems elsewhere on the network.  Although the Modem Assist TSRs can be loaded high and can be unloaded, this is certainly a management consideration for those who would share a modem.

- Must reboot after changes are made in the Port Status & Setup

The capability to change COM port configuration for redirection to specific communication lines or groups is not an “on-the-fly” operation..  The port setup facility could have been designed to load the necessary program elements before each communication session and unload afterwards so that port configuration modifications could be recognized in each new communication session.  Such a design would not, however, perform as well as the current design.

It was determined after consulting with our beta sites that they were not frequently making changes to their COM port configurations.  With the current design, the necessary program elements load once when Windows is started and each request for communication is serviced immediately without waiting for program elements to load in Windows.

The real strength of the COM port setup facility is that each user can easily customize the way they access network communication services with their varying communication applications.

# REVIEW METHODOLOGY

The purpose of this section of the Reviewer’s Guide is to give you a working overview of the newest features in Modem Assist Plus.  Here we will provide the steps to help you quickly see what Modem Assist does.  For an in-depth look at how Modem Assist works, see the section “How Modem Assist Works”.

## Installation

1.  Go to the workstation that has the modem or modems and run INSTALL from DOS or Windows.

2.  Select the appropriate operating system along with the option to install the Server. All other questions should be answered appropriately depending on your setup.

3.  After installation of the Server is completed, restart the workstation and load the MSERVER.EXE program from the directory you installed to (If during install you asked for this command to be added to the autoexec.bat, MSERVER will already be loaded).

4.  On the communication Server workstation, run the monitor program (MONITOR.EXE). This program allows you to see detailed connection information about all clients on the network.  The Monitor program also provides capabilities to control individual lines and connections (toggle in and out of service) with out downing the communication Server.

5.  Go to a workstation on the network without modems and run INSTALL from Windows.  You can install at each Client from the network drive with a standard File - Run - Browse. You can also install from DOS but installing from Windows will allow you to test all of the new Windows features in Modem Assist.

6.  Select the appropriate operating system along with the option to install the Client. For initial review, select COM 4 for the redirected port (this will be used in further examples below).  Later, you will be able to change the COM port to suit your needs (New feature).

7.  Restart Windows so that Modem Assist can begin servicing COM 4.

## Dialing Out with the New Client DLL

On workstations where you have installed the Modem Assist Client you will notice a Windows TERMINAL Icon in the Modem Assist Group.  Modem Assist has set TERMINAL to address COM 4. Click on the ICON.

Immediately you should notice that you are at a blinking cursor in Terminal, Type "ATZ" and press enter, the modem responds with "OK". You now have a modem to dial out.  This connection to a modem at the communication Server is established with the new MCLIENT.DLL

NOTE: If you have any other Windows communication programs (including FAX programs) simply select COM 4 in the program’s port setup area and dial out just as you have done with Windows Terminal.

## What’s happening at the Comm Server?

Another new feature of Modem Assist version 4 is the Modem View program. This program allows you to see the status of the modem as you are using it i.e. Send light, Receive light, etc. When this program is active, it will automatically detect communication activity and will pop into view on your desktop.  When you are finished with your communication session Modem View will “hibernate”.

1.  To use Modem View, click on the Modem View Icon in the Modem Assist Group.  The program will immediately go to a minimized icon and wait to activate.

2.  Click on the TERMINAL Icon in the Modem Assist Group. Notice that as soon as TERMINAL comes up the Modem View status lights appear on your screen as shown below. Type "ATZ" and press Enter.  You will notice that as you type and interact with the modem, the lights flash. Feel free to drag the modem to any location on your desktop.

3.  Modem View also provides the ability to display or not display the caption bar showing which COM port you are attached to.  Simply double click on the face of the modem and the Modem View Options screen will appear as shown below.  This setup screen provides you with the capability to monitor a specific port or to detect communication activity on all COM ports.  You can also shut down Modem View in this window.

## Background Queuing

Another new feature of Modem Assist version  4 is the Windows background queuing feature. This feature allows you to place yourself in a queue for the next available modem but continue to work in Windows. When you select the option to be queued, Modem Assist will automatically minimize your communication program so you can go on to something else. When the modem becomes available, Modem Assist automatically maximizes your communication program and you can begin where you left off.

To exercise this feature:

1.  Assure that only one modem is in service at the Modem Assist Server.  Also see that you have the Modem Assist Client installed on at least two workstations.

2.  At one of the Client workstations, click on the TERMINAL Icon in the Modem Assist Group to obtain a modem.

3.  Go to another Modem Assist Client workstation on your network and click on the TERMINAL Icon in the Modem Assist Group. You will receive a message stating that all the modems are busy, and asked if you would like to be queued.  Click OK. Terminal is now minimized on your desktop and you can do something else.

4.  Go back to the first CLIENT (currently connected to the modem) and EXIT Terminal. As soon as you exit, you will notice that the second Client has Terminal on the screen and is ready to dial out.

## Comm Port Control

In version 3.0 of Modem Assist, Synergy introduced a COM port Redirector allowing a port to be redirected to a modem on the network.  Although it was functional, the first generation Redirector did not offer the LAN user a great deal of flexibility in redirecting multiple COM ports to lines or groups of lines. New to Modem Assist Plus version 4 is a second generation Redirector which is easily controlled and customized with the Port Status and Setup program.

The Port Status and Setup program allows you to drag and drop lines or groups and assign them to COM ports 1 through 9. This gives each user the flexibility to determine which communication lines or groups they would like to access and then configure their communications applications to address those ports.  For example, COM 4 might be redirected to a FAX modem for use by WinFax PRO and COM 3 might be redirected to a high speed modem for access to a BBS with ProComm.

The Port Status & Setup window also provides an interface to the LAN user which allows them to see the status of each communication Server, and each modem.

To use Port Status and Setup:

1.  Click on the Port Status and Setup Icon in the Modem Assist Group.   The Port Status & Setup screen is presented as shown below.

2.  Using your mouse, select a line from the "Available Lines / State" box and hold down the left mouse button. Now drag that line to a COM port in the "Virtual COM Port Assignments" box and drop it on the COM port you would like to have assigned  to this line. The groups work the same way.

3.  While your in the Port Status and Setup you can also see which communication Servers are available by clicking on the "List Server" Icon.   The list will be displayed as shown below.

4.  You can also get a detailed view of your current Client settings by clicking on the "More Detail" Icon.  The Detail screen will be displayed as shown below.

5.  Information on all of the items in Port Status and Setup can be brought up by selecting HELP from the menu bar.

6.  When you have finished changing your settings, simply EXIT the Port Status and Setup. You will be prompted to save your changes and restart Windows. When Windows is restarted you are ready to use the newly assigned redirected COM ports.  Test your changes by setting your communication software to the appropriate port and starting a communication session.  Your session will be redirected to the Line or group you assigned to that port.

## Dialing In

In past versions of Modem Assist, the inbound capabilities where limited to launching only DOS based remote control and host communication programs. With version 4 of Modem Assist we have now included a Windows interface to our already familiar INHOST program. When this program is used in DOS it takes on a DOS appearance. When launched from Windows it takes on a Windows appearance and allows for the launching of Windows based remote control and host communication programs such as pcANYWHERE for Windows and ProComm Plus for Windows running in the Host mode.

If you want to dial in for a remote control session on the network you will need a remote control program like pcANYWHERE.  Instructions on the operation of various third party remote control packages can be found  in a file called 3RDPARTY.DOC in the subdirectory where you installed Modem Assist.   The following steps describe using the INHOST program and Windows Terminal to conduct an inbound test.

1.  While in Windows, click on the INBOUND SERVICE Icon.  The Inbound Service setup screen is displayed as shown below.

2.  Click on the Browse button.

3.  Click on the program name "TERMINAL.EXE"

Note: Make sure that you have previously set Terminal to use a redirected Modem Assist COM port.

4.  Enter "TERMINAL" or some other suitable description in the "Program Description" field (Optional).

5.  You can leave the "Input Parameters" field blank or, if you have created a TERMINAL settings file you can place that file name (something like: TEST.TRM) in this field.

6.  Enter a "Service Name" in the field provided (Optional). If you do not supply a service name, Modem Assist takes the default name specified in the CLIENT.CON file. This is the

name displayed in the list of services displayed when you call into the Server.

7.  Select the "Reset after Connection Termination" box.

8.  Select the "OPTIONS" button and check the box labeled "Terminate program on carrier drop. Do not select the other option box.

9.  Select the OK button. INHOST is now ready to accept an inbound call.

10.  From another location that has a modem, call into the Modem Assist Server. You will see a menu that displays the names of workstations that are ready to accept an inbound call. Select the appropriate line number and press enter.

11.  You are now “talking” to TERMINAL on the workstation that is running INHOST. Type various characters on your screen and you will see them displayed on the other screen.

Note: This is the most basic form of dialing in. Typically you would want to use a remote control package so that you can actually control the workstation.

12.  Now hang up the call. You will see that the INHOST workstation is now ready to accept another call and the modem Server is ready to accept and inbound or outbound call.

# IDEAS FOR REVIEW

## Infobahn Usage Becoming More Common in the Small Office

Owners of small businesses know that knowledge is power .  Awareness, however, of the best methods to access information in a timely fashion to gain advantages over the competition has been lacking.  Now there is a growing awareness of information services like Lexis/Nexis (legal), Medline (medical), CompuServe and so forth.  With this increased awareness, service and internet providers are signing up small businesses at an accelerated pace .

The luxury of these information services or access to the internet used to be reserved to larger organizations with more sophisticated communication capabilities and big budgets.  Small businesses are waking up to the value of these services and are setting up their smaller, less sophisticated networks to access the desired services by modem.

Small business owners remain budget conscious however.  To provide key employees with access to the desired information, it is often thought that a modem and a leased phone line for each user is needed.  Sharing the modem and phone lines across the network optimizes the usage of the resources and  saves money.

## Using Stand-Alone Communication Services on a Network

As companies such as United Parcel Service and FedEx develop software to service their customers, they often develop with stand alone systems in mind.  Modem Assist Plus enables LAN users to take advantage of the functionality of  the software and access network  communication resources from a stand-alone Windows communication interface.  The following quote is an example:

“Using Modem Assist Plus, ten of our users have seamless access to two shared modems with Windows UPS and FedEx on-line tracking software” states Michael McCormick, Chief of Information Systems at Centercom, Inc.  Based in  Milwaukee, the company replicates Video, CD and CDI media and ships daily to locations all over the world. “Without the cost of additional phone lines and modems,  Modem Assist Plus lets our sales and service people provide immediate shipment status to the customer without bothering our shipping people to do the tracking,” said McCormick.

# HOW MODEM ASSIST WORKS

In this section of the Reviewer’s Guide,  we will discuss the technical underpinnings of Modem Assist Plus -- the program components that enable Modem Assist Plus to do what it does.  For a foundation, we will discuss some of the basic concepts of network modem sharing.  Next we will address the operation of Modem Assist Plus’ main components, the Communication Server and the Communication Client and how they function for outbound and inbound service.  Lastly we will discuss some of the built-in management capabilities of Modem Assist Plus.

## The Basics of Modem Sharing

The heart of all asynchronous network communications is the Asynchronous Communication Server (ACS).  The ACS can resolve many obstacles associated with  network communications and give network users the flexibility they are accustomed to having with stand alone PC's.

An ACS is a device, usually a PC or other type of device that has the intelligence of a PC (CPU), that is connected to the LAN and serves as a communication server or gateway.  An ACS may provide both inbound and outbound communication capabilities and may operate in either a dedicated or non-dedicated mode.  An ACS should be able to share modems and may even share direct connections to other asynchronous computers.

Sharing modems or asynchronous devices across the LAN is accomplished by using the INT 14 redirection protocol, the NASI (INT 6B) interface, or other redirection capabilities. Network communication software provides the option to configure your COM port destination as INT 14 or NASI instead of COM 1, 2, 3 or 4.  Both the INT 14 and INT 6B communication interfaces have become accepted standards for vendors of ACS products.  New developments, like Synergy’s Redirector for Windows, add a new dimension of flexibility to redirecting communication sessions to shared modems on a network from standard windows COM ports.

### INT 14

INT 14 is a software interrupt initially developed by IBM corporation allowing communication devices to be shared across a network.  The standard IBM ROM BIOS software INT 14 allows software developers to develop communication programs that re-direct asynchronous communications over the network.  The actual re-directing comes from third party developed software that monitors this interrupt.  When the software recognizes an INT 14 call, it takes the data and sends it to the appropriate communications port on the ACS.  By its original design, the INT 14 interface is rather slow because the re-direction is accomplished one character at a time.  Because computing speed has become so critical, many communication developers have written INT 14 extensions which improve performance by sending blocks of characters.  The problem with the extensions is that each extension is proprietary to each specific manufacturer.  For more specific information on the INT 14 interface please see the Personal System/2 and Personal Computer BIOS Technical Reference, Document 68X2341, published by IBM Corporation in May, 1988.

### NASI

The NASI interface was developed to provide enhanced speed for sharing communication devices across the network.  The initial INT 14 specification, without the addition of extensions, is limited to only 9600 baud.  The NASI (Novell Asynchronous Services Interface) uses the software INT 6B and is capable of supporting speeds up to 115,200.  NASI allows software developers to develop communications programs that will re-direct asynchronous communications over a network.   NASI, or INT 6B works just like INT 14 except the third party developed software watches for calls to INT 6B.  When the third party developed software recognizes an INT 6B call it sends the command to the appropriate communications port on the ACS.  The NASI interface was primarily designed to be used with Novell's Asynchronous Communication Server (NACS) for dialing out from a Novell network.  Because of the better speed and performance offered by the NASI (INT 6B) re-direction, many third party software and hardware vendors began to develop products that took advantage of this interrupt.  Development of the NASI interface has been a joint effort of several vendors in the network communications area and has become an accepted standard among all network communication developers.

In order for an ACS to be effective on a network, the ACS must support either INT 14, NASI (INT 6B) or both of these redirection options or have other redirection capabilities.

### Windows COM Port Redirection

The capability to access shared network modems with any communication or FAX package with a standard Windows communication interface will be discussed in the context of the Modem Assist Communication Client (see Using the Windows Client and How Port Redirection Works).

## The Modem Assist Communication Server

The communication Server (MSERVER.EXE) is a DOS program that manages the serial ports on any PC connected to an IPX or NetBIOS LAN. Its function is to allow the ports to be used by other stations on the LAN.

The communication Server is a terminate-and-stay-resident (TSR) program. Because it is a TSR, the Server PC does not have to be dedicated to the Server function.  Other DOS programs can run concurrently with the communication Server.

As many as 16 ports can be managed by a Modem Assist PLUS communication Server.  Some versions of Modem Assist support fewer than 16 ports on the Server.  The ports can be any combination of the standard built-in ports and those added using non-intelligent multi-port adapter cards, such as the Synergy SS-554.  The ports can be connected to communication lines, with or without modems, or can be directly attached to other serial devices or computers.

### SERVER.CON Parameter File

The Server parameter file describes both the hardware facilities for the shared ports and the software attributes to be used during connections.  Hardware facilities include the port addresses of each line, the interrupt level being used, and the type of equipment connected to the line (e.g., presence of a modem).  Software attributes include names for the lines, names for groups of lines, names for services, and default values to use for line parameters when not specified in a connection request.

A basic parameter file for both the Server and Client is built during the installation process. These can be used without making any changes if the line parameters are suitable, or they can serve as models for more extensive files.  A complete description of the structure and content of Server parameter files and details on the use of the Modem Assist Plus Server are found in Chapter 4 of the User’s Guide.

### Types of Connections

In order for a station on the LAN to use a line on the Server, a connection must be established between the station and the line. The communication Server provides for two types of connections.

#### Outbound

An outbound connection is one that is initiated by a request from a Client to the Server. If the request can be satisfied, the connection is established by the Server and the Client then has exclusive use of the line until it disconnects. This facility is called Outbound Service.

#### Inbound

An inbound connection is one that is initiated by a call from an off-LAN device (e.g., a PC calling a dial-in telephone number). If the Server can locate a Client on the LAN running the INHOST program, the connection is established and the off-LAN caller can conduct a remote control session on the network.

The LAN Client has exclusive use of the line until it disconnects.  This facility is called Inbound Service.

Requests for outbound connections ask for a line by a name, or use a group name to ask for any line from a group of lines.  Requests for inbound connections ask for a service, not a line, again by naming the service.  The communication Server establishes service names when the INHOST program is loaded on a Client workstation.

### Comm Server Port Allocation

After the Server is loaded, it controls all direct access to the ports being shared. This means that you must not run any program that attempts to directly access the same ports assigned to the Server.  Accessing the shared ports must be done using the Modem Assist Client (or Local Client).  Attempting to directly access a port that is being controlled by the Server will likely hang the system.  However, if you are using a COM1, COM2, COM3, or COM4 port with the Server, and you want to run a program that uses one of these ports, you can remove the port from Modem Assist PLUS service using the Monitor Program (see Chapter 8 Managing Server Connections in the User’s Guide).  When a port is removed from service, the port can be used directly by a program.  This procedure will not work for multi-port boards because multiple port addresses are sharing a single interrupt level.

### Running Windows on the Communication Server

To run the Server on a machine that is also running Windows, the following restrictions and precautions apply:

1.	You must load MSERVER before loading Windows.

2.	If you are using a COM1, COM2, COM3, or COM4 port with the Server, the Server modifies a system control block to keep Windows from directly accessing the ports used by the Server.  However, if the [386Enh] section of your Windows SYSTEM.INI file contains port addresses (COM1Base=, COM2Base=, etc.), this mechanism can be circumvented and allow a Windows program to directly access the port while it is being used by the Server.  You should remove any COMxBase= statements from the SYSTEM.INI file.

3.	The ability of the Server to service multiple, high-speed lines can be greatly reduced by the presence of Windows.  The impact depends on factors, such as the number and kinds of windows programs that you run.  One area of special concern is running any process that generates interrupts of a higher priority than the interrupt level used for the communication ports at the Server. An example of a process with a higher interrupt priority could be a NIC (Network Interface Card) driver.

## The Modem Assist Communication Client

The setup of the Client program depends on which communications interfaces are supported by your communication programs and whether they operate in a Windows or DOS environment or both.  For an in-depth discussion of the various options and methods available for setting up a modem Client, refer to pages 5.14 through 5.19 in the Modem Assist Plus User’s Guide.

### CLIENT.CON Parameter File

As is the case with the communication Server, the communication Client uses a parameter file to provide information required for outbound and inbound connection requests. The information is similar to that contained in the Server parameter file, although much less extensive.

NOTE:	Any information provided in a Client parameter file takes precedence over information provided in a Server parameter file. In a sense, the Server values are used as default values in cases where a Client does not provide all the required values.

A basic communication Client parameter file is supplied with Modem Assist.  It can be used without making any changes if the line parameters are suitable, or it can serve as a model for a more extensive file.  A complete description of the structure and possible content of a communication Client parameter file as well as details on the use of the Modem Assist Plus Communication Client are found in Chapter 5 of the User’s Guide.

### Dialing Out With the DOS Client

The Modem Assist communication Client (MCLIENT) and the Modem Assist Local Client (MLCLIENT) are DOS programs which run on any network PC to access a communications Server.  The Local Client is run on the communications Server station to allow access to locally shared modems or to other communication Servers.  The Client and Local Client function identically with a few minor exceptions.  Both MCLIENT and MLCLIENT provide access to the shared modems for a communications program that uses one of the following interfaces:

•	The Windows communications interface

•	The basic INT 14 IBM ROM BIOS interface

•	The Modem Assist extended INT 14 interface

•	The Novell NASI/NCSI interface

### Dialing Out With the Windows Client (MCLIENT.DLL)

For Windows communications programs, the Modem Assist Client DLL provides seamless access to modems at communication Servers.  The Client DLL is automatically loaded by the Redirector if the Redirector determines that the MCLIENT.EXE program has not already been loaded.  Therefore, you need take no specific action to use the DLL other than not loading the MCLIENT.EXE program.

NOTE:	The Local Client function is not provided in the Client DLL. If you want to use the Local Client, you must use the MLCLIENT.EXE program.

Using the Windows communication Client is easy due to the fact that during installation, everything is set up to use the Modem Assist shared modems.  During installation, you chose a COM port for redirection under Windows.  To use a Windows communication or FAX program, simply select the Modem Assist redirected port in the Windows communication program you are using.  For example, if you selected COM4 as your redirected port during installation , simply set your communication program to COM4 and proceed with dialing out as if you had a locally attached modem.

You can change your COM port settings or assign additional COM ports to be redirected by using the Port Status and Setup program.  This feature is useful when specific modems or groups of modems are preferred for use with certain communication programs.

### How Port Redirection Works (MACOMM.DRV)

Windows COM port redirection is made possible by the Modem Assist MACOMM.DRV driver.  This driver allows the Client to intercept Windows communications calls for one or more COM ports (that you select) and redirect the request to the communications Server. The Modem Assist PLUS Redirector for Windows does not replace Windows COM drivers.  Instead, if a port is not a redirected Modem Assist PLUS port, requests are passed to the standard communication drivers (see diagram below).  This feature allows you to use any existing drivers for enhanced print support and local port access.

During installation, you select a COM port that is to be redirected to the communications Server.  All requests by Windows communications programs to the redirected port are serviced by Modem Assist.  Requests to COM ports that are not redirected are passed to the communications driver that was being used before you installed Modem Assist.

To establish the Redirector under Windows, the installation makes the following changes:

1.	The comm.drv= parameter in the [boot] section of the Windows SYSTEM.INI file is changed to point to the Modem Assist Redirector (MACOMM.DRV).

2.	A device=MCLIENT.EXE statement is added to the [386enh] section of the Windows SYSTEM.INI file. This change is not made for the Local Client since it is not needed on a Server station.

3.	Information on the redirected COM ports and the communications driver to use for non-redirected ports is placed in the MCLIENT.INI file.

Use of the Windows communication port Redirector is easily managed with the Port Status and Setup program.

### How the Client Makes a Connection

In order to establish a connection, a Client sends a connection request to a Server using a line or group name. The line or group name used can come from one of the following places:

1.	The communications program you are using. Communications programs that use the NASI interface usually supply the name of the line or group desired.

2.	The value you supplied in a COMxDest= parameter in the MCLIENT.INI file for Windows communications programs.

3.	You can directly request a connection with a line or group prior to using communications programs that support the INT 14 interface. You make a direct request using a parameter on the MCLIENT command or by using the MENU program.

If the line or group name is not supplied using one of the three methods shown above, the Client uses the default value supplied in the DEST= parameter of the CLIENT.CON file.

The request also contains some specification of the desired line and data transfer parameters.  The parameters supplied by the communications program are used for the connection.  The Client uses default values from the CLIENT.CON file for any values not supplied by the communications program.

#### Queuing

The Modem Assist Queuing feature allows you to "camp-on" a line that can satisfy the connection request but is in use when the request is made.  If a Server queues a Client request, you will see the following message:

Connection request queued by a Server

You are then given a chance to cancel or accept the queuing.  If you accept the queued request and you are running a DOS communication program, you must wait until the Server allocates a line to your communications program before you can resume work at your machine.

Queued requests for Windows users are processed by minimizing the Window in which the communications program is running.  This allows you to perform other work while you wait for the connection to be established.  When a connection is established, the communication program will maximize on the screen.

#### Retry

The Modem Assist Retry feature allows you to automatically retry requests that cannot be satisfied by a Server when they are made. If you are using the Retry feature and no Server can satisfy your request, you will see the following message:

All Servers are busy or unavailable...

You are then given the option to cancel or retry the request.

## Dialing In

Establishing an inbound connection is essentially the same as an outbound connection, except that the roles of Client and Server are reversed.  The Server initiates the connection request, and any Client that responds to the request will control the connection.

### Handling Incoming Calls

The two basic methods used to receive an inbound communication session on your LAN are known as “Inbound Service” and “Outbound Answer Mode”.

#### Inbound Service

The Inbound Service method assumes that a Client or Local Client on the LAN has the INHOST program loaded. When the communication Server receives an inbound call, it initiates a conversation with the caller to determine what service is desired. When the caller has entered the requested service name, the communication Server broadcasts a request to all communication Clients to see if one of them is willing to accept the service request. If a Client accepts the request, a connection is established and the Client takes control of the interaction with the caller.

#### Outbound Answer Mode

The second method, “Outbound Answer Mode” assumes that a Client has established a connection with the Server using a communication package and is waiting in the Host mode to receive a call.  The following table contrasts Inbound Service and Outbound Answer Mode.

Inbound Service		Outbound Answer Mode

Requires a Client to 	Requires a Client to

participate in the call 	participate in the call

Server answers call	Communications program

and passes to INHOST	takes control of line and

Client			is responsible to answer call

Does not dedicate a line 	Dedicates a line

to call reception unless 	and modem

you choose this mode

Asks the caller to try 	The phone is not answered

later if a Client is not 	if a Client is not

ready for the call		ready for the call

The line can be used 	The line is dedicated

for multiple services	to a single service

The Inbound Service method is preferable to Outbound Answer Mode in many situations because of the flexibility it offers.  With Inbound Service, lines are not dedicated to inbound calls, nor are they dedicated to Host sessions at a single workstation.  Inbound Service also provides the off-LAN caller with a helpful status message when a service is not available.

### The INHOST Program

To condition a Client or Local Client station to receive an inbound call, you use the INHOST program that is provided with Modem Assist.  The INHOST program has the following primary functions:

1.  To associate a service name (that you select) with a communications program (that you also select). In order for the Modem Assist inbound service to be useful to an off-LAN caller, you would normally choose a remote control communications package or at least a communications package that offers a "host" mode.

2.  To notify the Server of the service name that you selected. The Server keeps a list of all service names and provides a list of these names to an off-LAN caller.

3.  To accept a connection request from a Server based on the service name.

4.  To load the selected communications program when a connection is established.

5.  To reestablish the Client as an inbound service provider when the off-LAN call has completed.

The INHOST program is packaged as a dual environment program. If you load INHOST under DOS, INHOST functions as a DOS program and will invoke a DOS communications program to service an inbound connection.  If you load INHOST from the Modem Assist group under Windows, INHOST runs as a Windows program and will load a Windows communications program to service inbound calls.

## Managing Modem Assist Plus

Modem Assist includes a communication monitor program (MONITOR.EXE), that displays information about port usage when the Server is loaded.  Communication server activity can be logged for billing purposes.  The Communication Monitor is described in Chapter 8 of the User’s Guide.

A special parameter editor called PAREDIT is provided with Modem Assist Plus to facilitate changing and updating the Client and Server configuration files.  PAREDIT is menu driven and includes many standard options so that the network administrator does not manually have to code changes to configuration files  PAREDIT is covered in detail in Chapter 6 of the User’s Guide.

# SUMMARY

Modem Assist Plus is a flexible, feature-rich solution for outbound and inbound network modem sharing.  The new version 4 represents significant enhancements for use in a Windows environment and improvements to overall performance and usability.  Modem Assist is very competitive to other available solutions in terms of its price and feature set.  By  installing and trying some of the new features you will see that Modem Assist Plus is both easy to use and powerful.  Many companies have made Modem Assist Plus an integral part of their daily operations and have realized the economies of implementing a software asynchronous communication server on their networks.  First released in 1989, Modem Assist Plus,  is a stable, easy-to-manage modem sharing solution.

Modem Assist Plus and INSYNC are registered trademarks and SS-554, Connection Manager, and NPPC are trademarks of Synergy Solutions, Inc.  Other product names or services are marks of their respective owners, and are used for explanation only.